| Safety Problem | Problem Items | Cause of Items | Priority (High/ low/ no importance) | Docs. | Possible Technical Solution GRE-OVIG-Working Paper No. 2007-06 (1st session of the GRE Informal Group on Operating Voltage, 19-20 June 2007) (as applic. to vehicles) | Possible Technical Solution (as applic. to devices/) | Veh. Cats. | Item addressed by group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Governments | Governments / IWG | IWG | IWG | IWG | OICA | CLEPA/IEC | IWG | IWG |
| Road users glared by oncoming vehicle in reduced ambient lighting conditions | Discomfort/disability glare from dipped beam headlamp Reported excessive light output in-use | Overdriving of headlamp device with voltage higher than that for which it is designed and tested/approved |  | German data collection. SAE-830567 (1983) | Some vehicle manufacturers use voltage management (e.g. voltage limitation, etc.), including PWM. Large commercial vehicle manufacturers use reduced voltage to solve problem | Can offer plug in voltage stabilisers and voltage control devices for all vehicles | M N | Yes |
|  |  |  |  |  | HID and LED lamps already have voltage control |  |  |  |
|  |  | Mis-aim of headlamp in-use |  | UK survey on vehicles <3yrs indicates most vehicles outside OEM spec but within COP spec | Calibration of set up/checking equip in line regulation requirements ? (not in IG mandate) |  |  | No |
|  |  | # Light source OE |  |  | # COP | # COP |  | No |
|  |  | # Light source After market/illegal bulbs |  |  |  | # COP/tolerance |  | No |
|  |  | # Incorrect insertion of the light source into the lamp |  |  | # Condition for simple replacement # PTI | # Socket tolerance |  |  |
|  |  | Dirt on headlamp lens – |  |  | No solution – advise user to keep clean in owners manual ? (not in IG mandate) |  |  |  |
|  |  | Dirt on headlamp lens – |  |  | Lamps over 2000l have auto clean in line with R-45. Assume all >2000l (not in IG mandate) |  |  |  |
| Diminished presence / conspicuity of vehicle | Light out Increased incidence of light source failure over and above the cycle of light source replacement expected from currently available light sources | Driving of light sources with voltage higher than that for which it is designed and tested/approved, reduces rated light source lifetime |  |  | Some vehicle manufacturers use voltage management (e.g. voltage limitation, etc.), including PWM. Large commercial vehicle manufacturers use reduced voltage to solve problem | Good knowledge of usage conditions. Design of new light sources. | M, N, O |  |
|  | Diminished light source luminance | Driving of headlamp/signalling device with voltage lower than that for which it is designed and tested/approved |  |  |  | Can offer plug in voltage stabilisers and voltage control devices for all devices |  |  |
| Diminished presence / conspicuity of vehicles | Signalling light sources | As per headlamp |  |  | As per headlamp |  |  |  |
|  |  | Installation of low quality approved light sources |  |  | Manuf to add suitable advise in owners manual for replacing light sources |  |  |  |
|  |  | No standard available for vehicle testing that matches vehicle voltage condition in-use |  |  | Create a definition of the vehicle voltage specifications during normal operating conditions (must be harmonised with R-48) Definition to be used as test criteria for COP and checking vehicles in-use |  | M1 N1 N3? |  |
|  | EMC immunity |  | 0 | UNECE R10 | Compliance with UNECE R10 must be assured |  |  | No |

| Safety Problem | Problem Items | Cause of Items | Priority (High/ low/ no importance) | Docs. | Possible Technical Solution (as applic. to vehicles) | Possible Technical Solution (as applic. to devices/) | Veh. Cats. | Item addressed by group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Governments | Governments / IWG | IWG | IWG | IWG | OICA | CLEPA/IEC | IWG | IWG |
| Malfunction of vehicle electrical system | EMC radiation |  | 0 | UNECE R10 | Compliance with UNECE R10 must be assured |  |  | No |
| Malfunction of electrical systems in vehicle environment |  |  |  |  |  |  |  |  |
|  | variance between vehicle type approval and in-service operating voltage | Installation regulation not harmonised with real world conditions (current fleet) |  |  |  |  | M, N | Yes |
|  | variance between component type approval voltage and vehicle type approval voltage, with said lighting devices installed | Inconsistency between installation regulation and component regulations |  |  |  |  | M, N | Yes |