| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRPE/2007/5 31 October 2006 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Fifty-third session

Geneva, 9-12 January 2007

Item 1.2. of the provisional agenda

PROPOSAL FOR THE 05 SERIES OF AMENDMENTS TO REGULATION No. 49

(Emissions of C.I. and P.I. (NG and LPG) engines)

(Revision 4)

Submitted by the expert from the European Commission (EC)

Note:		The text reproduced below was prepared by the expert from the EC to insert into the Regulation new additional requirements on:

(a)	durability of exhaust after-treatment systems,

(b)	in-service conformity,

(c)	on-board diagnostic (OBD),

(d)	ensuring correct operation of NOx control measures.

Due to the complex restructuring of the text, it was agreed to introduce these requirements as part of a new consolidated version (Revision 4) of Regulation No. 49.

UNIFORM PROVISIONS CONCERNING the measures to be taken against the emission of gaseous and particulate pollutants from compression-ignition engines for use in vehicles, and the emission of gaseous pollutants from positive-ignition engines fuelled with natural gas or liquefied petroleum gas for use in vehicles

CONTENTS

REGULATION	Page

1.	Scope 	4

2.	Definitions	5

3.	Application for approval	15

4.	Approval	17

5.	Specifications and tests	26

6.	Installation on the vehicle	43

7.	Engine family	43

8.	Conformity of production	45

9.	Conformity of in-service vehicles/engines	49

10.	Penalties for non-conformity of production	49

11.	Modification and extension of approval of the approved type	50

12.	Production definitely discontinued	50

13.	Transitory provisions	50

14.	Names and addresses of technical services responsible for conducting approval tests and of administrative departments	52

Appendix 1 - 	Procedure for production conformity testing when standard deviation is satisfactory	53

Appendix 2 - 	Procedure for production conformity testing when standard deviation is unsatisfactory or unavailable	55

Appendix 3 - 	Procedure for production conformity testing at manufacturer's request	58

Appendix 4 - 	Determination of system equivalence	60

ANNEXES

Annex 1 -	Essential characteristics of the engine and information concerning the conduct of tests	62

Appendix 1 -	Essential characteristics of the engine family	71

Appendix 2 -	Essential characteristics of the engine type within the family	73

Appendix 3 -	Characteristics of the engine-related vehicle parts	79

Appendix 4 -	OBD-related information	80

Annex 2A -	Communication concerning the approval or extension or refusal or withdrawal of approval or production definitely discontinued of a compression-ignition (C.I.) or NG engine type or a positive-ignition (P.I.) LPG-fuelled engine type as a separate technical unit with regard to the emission of pollutants pursuant to Regulation No. 49	82

Appendix 1 -	OBD related information	85

Annex 2B -	Communication concerning the approval or extension or refusal or withdrawal of approval or production definitely discontinued of a vehicle type with regard to the emission of gaseous and particulate pollutants pursuant to Regulation No. 49	86

Annex 3 -	Arrangements of approval marks	89

Annex 4A -	Test procedure	92

Appendix 1 -	ESC and ELR test cycles	97

Appendix 2 -	ETC test cycle	116

Appendix 3 -	ETC engine dynamometer schedule	136

Appendix 4 -	Measurement and sampling procedure	173

Appendix 5 -	Calibration procedure 	184

Appendix 6 –	Carbon flow check	203

Appendix 7 -	Analytical and sampling systems	206

Annex 5 -	Technical characteristics of reference fuel for C.I. engines prescribed for approval tests and to verify the conformity of production	241

Annex 6 -	Example of calculation procedure	248

Annex 7 -	Procedures for conducting the test for durability of emission control systems	270

Annex 8 -	Conformity of in-service vehicles/engines	278

Annex 9A -	On-board diagnostic systems (OBD)	287

Appendix 1 -	On-board diagnostic (OBD) system approval tests	301

1.	SCOPE

1.1.	This Regulation applies to vehicles of categories M and N 1/ and their engines, as shown in table A, with regard to the tests foreseen for these engines in table B.

Table A: APPLICABILITY

| Vehicle category 1/ | Maximum mass | Positive-ignition engines |  |  | Compression-ignition engines |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Petrol | NG (a) | LPG (b) | Diesel | Ethanol |
| M1 | ≤ 3.5 t | - | - | - | - | - |
|  | > 3.5 t | - | R49 | R49 | R49 | R49 |
| M2 | - | - | R49 | R49 | R49 or R83 (c) | R49 |
| M3 | - | - | R49 | R49 | R49 | R49 |
| N1 | - | - | R49 or R83 | R49 or R83 | R49 or R83 | R49 |
| N2 | - | - | R49 | R49 | R49 or R83 (c) | R49 |
| N3 | - | - | R49 | R49 | R49 | R49 |

(a)	Natural Gas.

(b)	Liquefied Petroleum Gas.

(c)	Regulation No. 83 applies only for vehicles with a reference mass ≤  2,840 kg as an extension of an approval given for an engine used in vehicles of categories M1 or N1. /

"R49 or R83" means that manufacturers can obtain type approval according to this Regulation or to Regulation No. 83, see paragraph 1.2.

Table B: REQUIREMENTS

|  | Positive-ignition engines |  |  | Compression-ignition engines |  |
| --- | --- | --- | --- | --- | --- |
|  | Petrol | NG | LPG | Diesel | Ethanol |
| Gaseous pollutants | - | Yes | Yes | Yes | Yes |
| Particulates | - | - | - | Yes | - |
| Durability | - | Yes | Yes | Yes | Yes |
| In-service-conformity | - | Yes | Yes | Yes | Yes |
| OBD | - | - (a) | - (b) | Yes | Yes |

(a)	Foreseen for 2008 at the earliest.

(b)	Foreseen for 2008 at the earliest.

1.2.	Equivalent approvals

The following do not need to be approved according to this Regulation, if they are part of a vehicle approved according to Regulation No. 83:

(a)	compression-ignition engines to be mounted in vehicles of categories N1, N2 and M2 1/ fuelled with diesel;

(b)	positive-ignition engines fuelled with natural gas (NG) or liquefied petroleum gas (LPG) to be mounted in vehicles of category N1. 1/

2.	DEFINITIONS

2.1.	For the purposes of this Regulation, the following definitions shall apply:

"approval of an engine (engine family)" means the approval of an engine type (engine family) with regard to the level of the emission of gaseous and particulate pollutants;

"auxiliary emission control strategy (AECS)" means an emission control strategy that becomes active or that modifies the base emission control strategy for a specific purpose or purposes and in response to a specific set of ambient and/or operating conditions, e.g. vehicle speed, engine speed, gear used, intake temperature, or intake pressure;

"base emission control strategy (BECS)" means an emission control strategy that is active throughout the speed and load operating range of the engine unless an AECS is activated.  Examples for BECS are, but are not limited to:

- (a)	engine timing map;

- (b)	EGR map;

- (c)	SCR catalyst reagent dosing map;

"combined deNOx- particulate filter" means an exhaust aftertreatment system designed to concurrently reduce emissions of oxides of nitrogen (NOx) and particulate pollutants (PT);

"continuous regeneration" means the regeneration process of an exhaust aftertreatment system that occurs either permanently or at least once per ETC test.  Such a regeneration process will not require a special test procedure;

"control area" means the area between the engine speeds A and C and between 25 to 100 per cent load;

"declared maximum power (Pmax)" means the maximum power in EC kW (net power) as declared by the manufacturer in his application for approval;

"defeat strategy" means:

- (a)	an AECS that reduces the effectiveness of the emission control relative to the BECS under conditions that may reasonably be expected to be encountered in normal vehicle operation and use;

- (b)	a BECS that discriminates between operation on a standardized approval test and other operations and provides a lesser level of emission control under conditions not substantially included in the applicable approval test procedures, or;

- (c)	an OBD or an emission control monitoring strategy that discriminates between operation on a standardized approval test and other operations and provides a lower level of monitoring capability (timely and accurately) under conditions not substantially included in the applicable approval test procedures;

"deNOx system" means an exhaust aftertreatment system designed to reduce emissions of oxides of nitrogen (NOx) (e.g. there are presently passive and active lean NOx catalysts, NOx adsorbers and selective catalytic reduction (SCR) systems);

"delay time" means the time between the change of the component to be measured at the reference point and a system response of 10 per cent of the final reading (t10).  For the gaseous components, this is basically the transport time of the measured component from the sampling probe to the detector.  For the delay time, the sampling probe is defined as the reference point;

"diesel engine" means an engine which works on the compression-ignition principle;

"ELR test" means a test cycle consisting of a sequence of load steps at constant engine speeds to be applied in accordance with paragraph 5.2.;

"ESC test" means a test cycle consisting of 13 steady state modes to be applied in accordance with paragraph 5.2.;

"ETC test" means a test cycle consisting of 1800 second-by-second transient modes to be applied in accordance with paragraph 5.2.;

"element of design" means in respect of a vehicle or engine,

- (a)	any control system, including computer software, electronic control systems and computer logic;

- (b)	any control system calibrations;

- (c)	the result of systems interaction, or;

- (d)	any hardware items;

"emissions-related defect" means a deficiency or deviation from normal production tolerances in design, materials or workmanship in a device, system or assembly that affects any parameter, specification or component belonging to the emission control system.  A missing component may be considered to be an "emissions-related defect";

"emission control strategy (ECS)" means an element or set of elements of design that is incorporated into the overall design of an engine system or vehicle for the purposes of controlling exhaust emissions that includes one BECS and one set of AECS;

"emission control system" means the exhaust aftertreatment system, the electronic management controller(s) of the engine system and any emission-related component of the engine system in the exhaust which supplies an input to or receives an output from this(these) controller(s), and when applicable the communication interface (hardware and messages) between the engine system electronic control unit(s) (EECU) and any other power train or vehicle control unit with respect to emissions management;

"emission control monitoring system" means the system that ensures correct operation of the NOx control measures implemented in the engine system according to the requirements of paragraph 5.5.

"emission default mode" means an AECS activated in the case of a malfunction of the ECS detected by the OBD system that results in the MI being activated and that does not require an input from the failed component or system;

"engine-aftertreatment system family" means, for testing over a service accumulation schedule to establish deterioration factors according to Annex 7 to this Regulation and for checking the conformity of in-service vehicles/engines according to Annex 8 to this Regulation, a manufacturer's grouping of engines that comply with the definition of engine family but which are further grouped into engines utilising a similar exhaust after-treatment system;

"engine system" means the engine, the emission control system and the communication interface (hardware and messages) between the engine system electronic control unit(s) (EECU) and any other powertrain or vehicle control unit;

"engine family" means a manufacturer's grouping of engine systems which, through their design as defined in paragraph 7. of this Regulation, have similar exhaust emission characteristics; all members of the family shall comply with the applicable emission limit values;

"engine operating speed range" means the engine speed range, most frequently used during engine field operation, which lies between the low and high speeds, as set out in Appendix 1 to Annex 4A to this Regulation;

"engine speeds A, B and C" means the test speeds within the engine operating speed range to be used for the ESC test and the ELR test, as set out in Appendix 1 to Annex 4A to this Regulation;

"engine setting" means a specific engine/vehicle configuration that includes the emission control strategy (ECS), one single engine performance rating (the approved full-load curve) and, if used, one set of torque limiters;

"engine type" means a category of engines which do not differ in such essential respects as engine characteristics as described in Annex 1 to this Regulation;

"exhaust aftertreatment system" means a catalyst (oxidation or 3-way), particulate filter, deNOx system, combined deNOx-particulate filter or any other emission-reducing device that is installed downstream of the engine.  This definition excludes exhaust gas recirculation, which, where fitted, is considered an integral part of the engine system;

"gas engine" means a positive-ignition engine which is fuelled with natural gas (NG) or liquefied petroleum gas (LPG);

"gaseous pollutants" means carbon monoxide, hydrocarbons (assuming a ratio of CH1.85 for diesel, CH2.525 for LPG and CH2.93 for NG (NMHC) and an assumed molecule CH3O0.5 for ethanol-fuelled diesel engines), methane (assuming a ratio of CH4 for NG) and oxides of nitrogen, the last-named being expressed in nitrogen dioxide (NO2) equivalent;

"high speed (nhi)" means the highest engine speed where 70 per cent of the declared maximum power occurs;

"low speed (nlo)" means the lowest engine speed where 50 per cent of the declared maximum power occurs;

"major functional failure" / means a permanent or temporary malfunction of any exhaust aftertreatment system that is expected to result in an immediate or delayed increase of the gaseous or particulate emissions of the engine system and which cannot be properly estimated by the OBD system;

"malfunction" means:

- (a)	any deterioration or failure, including electrical failures, of the emission control system, that would result in emissions exceeding the OBD threshold limits or, when applicable, in failing to reach the range of functional performance of the exhaust aftertreatment system where the emission of any regulated pollutant would exceed the OBD threshold limits;

- (b)	any case where the OBD system is not able to fulfil the monitoring requirements of this Regulation.

A manufacturer may nevertheless consider a deterioration or failure that would result in emissions not exceeding the OBD threshold limits as a malfunction;

"malfunction indicator (MI)" means a visual indicator that clearly informs the driver of the vehicle in the event of a malfunction in the sense of this Regulation;

"multi-setting engine" means an engine containing more than one engine setting;

"NG gas range" means one of the H or L range as defined in European Standard EN 437, dated November 1993;

"net power" means the power in kW obtained on the test bench at the end of the crankshaft, or its equivalent, measured in accordance with the method of measuring power as set out in Regulation No. 85;

"OBD" means an on-board diagnostic system for emission control, which has the capability of detecting the occurrence of a malfunction and of identifying the likely area of malfunction by means of fault codes stored in computer memory;

"OBD-engine family" means, for approval of the OBD system according to the requirements of Annex 9A to this Regulation, a manufacturer's grouping of engine systems having common OBD system design parameters according to paragraph 7.3. of this Regulation;

"opacimeter" means an instrument designed to measure the opacity of smoke particles by means of the light extinction principle;

"parent engine" means an engine selected from an engine family in such a way that its emissions characteristics will be representative for that engine family;

"particulate aftertreatment device" means an exhaust aftertreatment system designed to reduce emissions of particulate pollutants (PT) through a mechanical, aerodynamic, diffusional or inertial separation;

"particulate pollutants" means any material collected on a specified filter medium after diluting the exhaust with clean filtered air so that the temperature does not exceed 325 K (52 °C);

"per cent load" means the fraction of the maximum available torque at an engine speed;

"periodic regeneration" means the regeneration process of an emission control device that occurs periodically in less than 100 hours of normal engine operation.  During cycles where regeneration occurs, emission standards can be exceeded.

"power take-off unit" means an engine-driven output device for the purposes of powering auxiliary, vehicle mounted, equipment;

"reagent" means any medium that is stored on-board the vehicle in a tank and provided to the exhaust aftertreatment system (if required) upon request of the emission control system;

"recalibration" means a fine tuning of an NG engine in order to provide the same performance (power, fuel consumption) in a different range of natural gas;

"reference speed (nref)" means the 100 per cent speed value to be used for denormalizing the relative speed values of the ETC test, as set out in Appendix 2 to Annex 4A to this Regulation;

"response time" means the difference in time between a rapid change of the component to be measured at the reference point and the appropriate change in the response of the measuring system whereby the change of the measured component is at least 60 per cent FS and takes place in less than 0.1 second.  The system response time (t90) consists of the delay time to the system and of the rise time of the system (see also ISO 16183);

"rise time" means the time between the 10 per cent and 90 per cent response of the final reading (t90 – t10).  This is the instrument response after the component to be measured has reached the instrument.  For the rise time, the sampling probe is defined as the reference point;

"self adaptability" means any engine device allowing the air/fuel ratio to be kept constant;

"smoke" means particles suspended in the exhaust stream of a diesel engine which absorb, reflect, or refract light;

"test cycle" means a sequence of test points each with a defined speed and torque to be followed by the engine under steady state (ESC test) or transient operating conditions (ETC, ELR test);

"torque limiter" means a device that temporarily limits the maximum torque of the engine;

"transformation time" means the time between the change of the component to be measured at the sampling probe and a system response of 50 per cent of the final reading (t50).  The transformation time is used for the signal alignment of different measurement instruments;

"useful life" means, for vehicles and engines that are approved to either row B1, row B2 or row C of the table given in paragraph 5.2.1. of this Regulation, the relevant period of distance and/or time that is defined in paragraph 5.3. (durability of emission control systems) of this Regulation over which compliance with the relevant gaseous, particulate and smoke emission limits has to be assured as part of the approval;

"Wobbe index (lower Wl; or upper Wu)" means the ratio of the corresponding calorific value of a gas per unit volume and the square root of its relative density under the same reference conditions:

W=Hgas × ρair/ρgas

"λ-shift factor (Sλ)" means an expression that describes the required flexibility of the engine management system regarding a change of the excess-air ratio λ if the engine is fuelled with a gas composition different from pure methane (see Annex 7 for the calculation of Sλ).

2.2.	Symbols, abbreviations and international standards

2.2.1.	Symbols for test parameters:

| Symbol | Unit | Term |
| --- | --- | --- |
| Ap | m2 | Cross sectional area of the isokinetic sampling probe |
| Ae | m2 | Cross sectional area of the exhaust pipe |
| c | ppm/vol. per cent | Concentration |
| Cd | — | Discharge coefficient of SSV-CVS |
| C1 | — | Carbon 1 equivalent hydrocarbon |
| d | m | Diameter |
| D0 | m3/s | Intercept of PDP calibration function |
| D | — | Dilution factor |
| D | — | Bessel function constant |
| E | — | Bessel function constant |
| EE | — | Ethane efficiency |
| EM | — | Methane efficiency |
| EZ | g/kWh | Interpolated NOx emission of the control point |
| f | 1/s | Frequency |
| fa | — | Laboratory atmospheric factor |
| fc | s−1 | Bessel filter cut-off frequency |
| Fs | — | Stoichiometric factor |
| H | MJ/m3 | Calorific value |
| Ha | g/kg | Absolute humidity of the intake air |
| Hd | g/kg | Absolute humidity of the dilution air |
| i | — | Subscript denoting an individual mode or instantaneous measurement |
| K | — | Bessel constant |
| k | m−1 | Light absorption coefficient |
| kf |  | Fuel specific factor for dry to wet correction |
| kh,D | — | Humidity correction factor for NOx for diesel engines |
| kh,G | — | Humidity correction factor for NOx for gas engines |
| KV |  | CFV calibration function |
| kW,a | — | Dry to wet correction factor for the intake air |
| kW,d | — | Dry to wet correction factor for the dilution air |
| kW,e | — | Dry to wet correction factor for the diluted exhaust gas |
| kW,r | — | Dry to wet correction factor for the raw exhaust gas |
| L | per cent | Percent torque related to the maximum torque for the test engine |
| La | m | Effective optical path length |
| Mra | g/mol | Molecular mass of the intake air |
| Mre | g/mol | Molecular mass of the exhaust |
| md | kg | Mass of the dilution air sample passed through the particulate sampling filters |
| med | kg | Total diluted exhaust mass over the cycle |
| medf | kg | Mass of equivalent diluted exhaust over the cycle |
| mew | kg | Total exhaust mass over the cycle |
| mf | mg | Particulate sample mass collected |
| mf,d | mg | Particulate sample mass of the dilution air collected |
| mgas | g/h or g | Gaseous emissions mass flow (rate) |
| mse | kg | Sample mass over the cycle |
| msep | kg | Mass of the diluted exhaust sample passed through the particulate sampling filters |
| mset | kg | Mass of the double diluted exhaust sample passed through the particulate sampling filters |
| mssd | kg | Mass of secondary dilution air |
| N | per cent | Opacity |
| NP | — | Total revolutions of PDP over the cycle |
| NP,i | — | Revolutions of PDP during a time interval |
| n | min−1 | Engine speed |
| np | s−1 | PDP speed |
| nhi | min−1 | High engine speed |
| nlo | min−1 | Low engine speed |
| nref | min−1 | Reference engine speed for ETC test |
| pa | kPa | Saturation vapour pressure of the engine intake air |
| pb | kPa | Total atmospheric pressure |
| pd | kPa | Saturation vapour pressure of the dilution air |
| pp | kPa | Absolute pressure |
| pr | kPa | Water vapour pressure after cooling bath |
| ps | kPa | Dry atmospheric pressure |
| p1 | kPa | Pressure depression at pump inlet |
| P(a) | kW | Power absorbed by auxiliaries to be fitted for test |
| P(b) | kW | Power absorbed by auxiliaries to be removed for test |
| P(n) | kW | Net power non-corrected |
| P(m) | kW | Power measured on test bed |
| qmaw | kg/h or kg/s | Intake air mass flow rate on wet basis |
| qmad | kg/h or kg/s | Intake air mass flow rate on dry basis |
| qmdw | kg/h or kg/s | Dilution air mass flow rate on wet basis |
| qmdew | kg/h or kg/s | Diluted exhaust gas mass flow rate on wet basis |
| qmdew,i | kg/s | Instantaneous CVS flow rate mass on wet basis |
| qmedf | kg/h or kg/s | Equivalent diluted exhaust gas mass flow rate on wet basis |
| qmew | kg/h or kg/s | Exhaust gas mass flow rate on wet basis |
| qmf | kg/h or kg/s | Fuel mass flow rate |
| qmp | kg/h or kg/s | Particulate sample mass flow rate |
| qvs | dm³/min | Sample flow rate into analyser bench |
| qvt | cm³/min | Tracer gas flow rate |
| Ω | — | Bessel constant |
| Qs | m3/s | PDP/CFV-CVS volume flow rate |
| QSSV | m3/s | SSV-CVS volume flow rate |
| ra | — | Ratio of cross sectional areas of isokinetic probe and exhaust pipe |
| rd | — | Dilution ratio |
| rD | — | Diameter ratio of SSV-CVS |
| rp | — | Pressure ratio of SSV-CVS |
| rs | — | Sample ratio |
| Rf | — | FID response factor |
| ρ | kg/m3 | density |
| S | kW | Dynamometer setting |
| S i | m−1 | Instantaneous smoke value |
| Sλ | — | λ-shift factor |
| T | K | Absolute temperature |
| Ta | K | Absolute temperature of the intake air |
| t | s | Measuring time |
| te | s | Electrical response time |
| tf | s | Filter response time for Bessel function |
| tp | s | Physical response time |
| Δt | s | Time interval between successive smoke data (= 1/sampling rate) |
| Δti | s | Time interval for instantaneous CVS flow |
| τ | per cent | Smoke transmittance |
| u | - | Ratio between densities of gas component and exhaust gas |
| V0 | m3/rev | PDP gas volume pumped per revolution |
| Vs | l | System volume of analyser bench |
| W | — | Wobbe index |
| Wact | kWh | Actual cycle work of ETC |
| Wref | kWh | Reference cycle work of ETC |
| Wf | — | Weighting factor |
| Wfe | — | Effective weighting factor |
| X0 | m3/rev | Calibration function of PDP volume flow rate |
| Yi | m−1 | 1 s Bessel averaged smoke value |

2.2.2.	Symbols for chemical components

| CH4 | Methane |
| --- | --- |
| C2H6 | Ethane |
| C2H5OH | Ethanol |
| C3H8 | Propane |
| CO | Carbon monoxide |
| DOP | Di-octylphtalate |
| CO2 | Carbon dioxide |
| HC | Hydrocarbons |
| NMHC | Non-methane hydrocarbons |
| NOx | Oxides of nitrogen |
| NO | Nitric oxide |
| NO2 | Nitrogen dioxide |
| PT | Particulates. |

2.2.3.	Abbreviations

| CFV | Critical flow venturi |
| --- | --- |
| CLD | Chemiluminescent detector |
| ELR | European load response test |
| ESC | European steady state cycle |
| ETC | European transient cycle |
| FID | Flame ionisation detector |
| GC | Gas chromatograph |
| HCLD | Heated chemiluminescent detector |
| HFID | Heated flame ionisation detector |
| LPG | Liquefied petroleum gas |
| NDIR | Non-dispersive infrared analyser |
| NG | Natural gas |
| NMC | Non-methane cutter |

2.2.4.	Symbols for the fuel composition

| wALF | hydrogen content of fuel, per cent mass |
| --- | --- |
| wBET | carbon content of fuel, per cent mass |
| wGAM | sulphur content of fuel, per cent mass |
| wDEL | nitrogen content of fuel, per cent mass |
| wEPS | oxygen content of fuel, per cent mass |
|  | molar hydrogen ratio (H/C) |
|  | molar carbon ratio (C/C) |
|  | molar sulphur ratio (S/C) |
|  | molar nitrogen ratio (N/C) |
|  | molar oxygen ratio (O/C) |
| referring to a fuel C H O N S  = 1 for carbon based fuels,  = 0 for hydrogen fuel |  |

2.2.5.	Standards referenced by this Regulation

| ISO 15031-1 | ISO 15031-1: 2001 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 1: General information. |
| --- | --- |
| ISO 15031-2 | ISO/PRF TR 15031-2: 2004 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 2: Terms, definitions, abbreviations and acronyms. |
| ISO 15031-3 | ISO 15031-3: 2004 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 3: Diagnostic connector and related electrical circuits, specification and use. |
| SAE J1939-13 | SAE J1939-13: Off-Board Diagnostic Connector. |
| ISO 15031-4 | ISO DIS 15031-4.3: 2004 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 4: External test equipment. |
| SAE J1939-73 | SAE J1939-73: Application Layer – Diagnostics. |
| ISO 15031-5 | ISO DIS 15031-5.4: 2004 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 5: Emissions-related diagnostic services. |
| ISO 15031-6 | ISO DIS 15031-6.4: 2004 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 6: Diagnostic trouble code definitions. |
| SAE J2012 | SAE J2012: Diagnostic Trouble Code Definitions Equivalent to ISO/DIS 15031-6, April 30, 2002. |
| ISO 15031-7 | ISO 15031-7: 2001 Road vehicles - Communication between vehicle and external equipment for emissions related diagnostics - Part 7: Data link security. |
| SAE J2186 | SAE J2186: E/E Data Link Security, dated October 1996. |
| ISO 15765-4 | ISO 15765-4: 2001 Road vehicles - Diagnostics on Controller Area Network (CAN) - Part 4: Requirements for emissions-related systems. |
| SAE J1939 | SAE J1939: Recommended Practice for a Serial Control and Communications Vehicle Network. |
| ISO 16185 | ISO 16185: 2000 Road vehicles – engine family for homologation. |
| ISO 2575 | ISO 2575: 2000 Road vehicles – Symbols for controls, indicators and tell-tales. |
| ISO 16183 | ISO 16183: 2002 Heavy duty engines - Measurement of gaseous emissions from raw exhaust gas and of particulate emissions using partial flow dilution systems under transient test conditions. |

3.	APPLICATION FOR APPROVAL

3.1.	Application for approval for a type of engine or engine family as a separate technical unit

3.1.1.	The application for approval of an engine type or engine family with regard to the level of the emission of gaseous and particulate pollutants for diesel engines and with regard to the level of the emission of gaseous pollutants for gas engines as well as the useful life and on-board diagnostic (OBD) system shall be submitted by the engine manufacturer or by a duly accredited representative.

Should the application concern an engine equipped with an on-board diagnostic (OBD) system, the requirements of paragraph 3.4. shall be fulfilled.

3.1.2.	It shall be accompanied by the undermentioned documents in triplicate and the following particulars:

3.1.2.1.	A description of the engine type or engine family, if applicable, comprising the particulars referred to in Annex 1 to this Regulation.

3.1.3.	An engine conforming to the "engine type" or "parent engine" characteristics described in Annex 1 shall be submitted to the technical service responsible for conducting the approval tests defined in paragraph 5.

3.2.	Application for approval for a vehicle type in respect of its engine

3.2.1.	The application for approval of a vehicle with regard to emission of gaseous and particulate pollutants by its diesel engine or diesel engine family and with regard to the level of the emission of gaseous pollutants by its gas engine or gas engine family as well as the useful life and on-board diagnostic (OBD) system shall be submitted by the vehicle manufacturer or by a duly accredited representative.

Should the application concern an engine equipped with an on-board diagnostic (OBD) system, the requirements of paragraph 3.4. shall be fulfilled.

3.2.2.	It shall be accompanied by the below-mentioned documents in triplicate and the following particulars:

3.2.2.1.	A description of the vehicle type, of the engine-related vehicle parts and of the engine type or engine family, if applicable, comprising the particulars referred to in Annex 1 to this Regulation.

3.2.3.	The manufacturer shall provide a description of the malfunction indicator (MI) used by the OBD system to signal the presence of a fault to a driver of the vehicle.

The manufacturer shall provide a description of the indicator and warning mode used to signal the lack of required reagent to a driver of the vehicle.

3.3.	Application for approval for a vehicle type with an approved engine

3.3.1.	The application for approval of a vehicle with regard to emission of gaseous and particulate pollutants by its approved diesel engine or diesel engine family and with regard to the level of the emission of gaseous pollutants by its approved gas engine or gas engine family as well as the useful life and on-board diagnostic (OBD) system shall be submitted by the vehicle manufacturer or by a duly accredited representative.

3.3.2.	It shall be accompanied by the undermentioned documents in triplicate and the following particulars:

3.3.2.1.	a description of the vehicle type and of engine-related vehicle parts comprising the particulars referred to in Annex 1, as applicable, and a copy of the approval communication form (Annex 2A) for the engine or engine family, if applicable, as a separate technical unit which is installed in the vehicle type.

3.3.3.	The manufacturer shall provide a description of the malfunction indicator (MI) used by the OBD system to signal the presence of a fault to a driver of the vehicle.

The manufacturer shall provide a description of the indicator and warning mode used to signal the lack of required reagent to a driver of the vehicle.

3.4.	On-board diagnostic systems

3.4.1.	The application for approval of an engine equipped with an on-board diagnostic (OBD) system shall be accompanied by the information required in paragraph 9. of Annex 1 (description of the parent engine) and/or paragraph 6. of Appendix 2 to Annex 1 (description of an engine type within the family) together with:

3.4.1.1.	Detailed written information fully describing the functional operation characteristics of the OBD system, including a listing of all relevant parts of the engine's emission control system, i.e. sensors, actuators and components, that are monitored by the OBD system;

3.4.1.2.	Where applicable, a declaration by the manufacturer of the parameters that are used as a basis for major functional failure monitoring and, in addition:

3.4.1.2.1.	The manufacturer shall provide the technical service with a description of potential failures within the emission control system that will have an effect on emissions. This information shall be subject to discussion and agreement between the technical service and the vehicle manufacturer.

3.4.1.3.	Where applicable, a description of the communication interface (hardware and messages) between the engine electronic control unit (EECU) and any other powertrain or vehicle control unit when the exchanged information has an influence on the correct functioning of the emission control system.

3.4.1.4.	Where appropriate, copies of other approvals with the relevant data to enable extensions of approvals.

3.4.1.5.	If applicable, the particulars of the engine family as referred to in paragraph 7. of this Regulation.

3.4.1.6.	The manufacturer shall describe provisions taken to prevent tampering with and modification of the EECU or any interface parameter considered in paragraph 3.4.1.3.

4.	APPROVAL

4.1.	Granting of a universal fuel approval

A universal fuel approval is granted subject to the following requirements.

4.1.1.	In the case of diesel or ethanol fuel the parent engine meets the requirements of this Regulation on the reference fuel specified in Annex 5.

4.1.2.	In the case of natural gas the parent engine should demonstrate its capability to adapt to any fuel composition that may occur across the market.  In the case of natural gas there are generally two types of fuel, high calorific fuel (H-gas) and low calorific fuel (L-gas), but with a significant spread within both ranges; they differ significantly in their energy content expressed by the Wobbe Index and in their λ-shift factor (Sλ).  The formulae for the calculation of the Wobbe index and Sλ are given in paragraph.  Natural gases with a λ-shift factor between 0.89 and 1.08 (0.89 ≤ Sλ ≤ 1.08) are considered to belong to H-range, while natural gases with a λ-shift factor between 1.08 and 1.19 (1.08 ≤ Sλ ≤ 1.19) are considered to belong to L-range.  The composition of the reference fuels reflects the extreme variations of Sλ.

The parent engine shall meet the requirements of this Regulation on the reference fuels GR (fuel 1) and G25 (fuel 2), as specified in Annex 5, without any readjustment to the fuelling between the two tests.  However, one adaptation run over one ETC cycle without measurement is permitted after the change of the fuel.  Before testing, the parent engine shall be run-in using the procedure given in paragraph 3. of Appendix 2 to Annex 4A.

4.1.2.1.	On the manufacturer's request the engine may be tested on a third fuel (fuel 3) if the λ-shift factor (Sλ) lies between 0.89 (i.e. the lower range of GR) and 1.19 (i.e. the upper range of G25), for example when fuel 3 is a market fuel.  The results of this test may be used as a basis for the evaluation of the conformity of the production.

4.1.3.	In the case of an engine fuelled with natural gas which is self-adaptive for the range of H-gases on the one hand and the range of L-gases on the other hand, and which switches between the H-range and the L-range by means of a switch, the parent engine shall be tested on the relevant reference fuel as specified in Annex 5 for each range, at each position of the switch.  The fuels are GR (fuel 1) and G23 (fuel 3) for the H-range of gases and G25 (fuel 2) and G23 (fuel 3) for the L-range of gases.  The parent engine shall meet the requirements of this Regulation at both positions of the switch without any readjustment to the fuelling between the two tests at each position of the switch.  However, one adaptation run over one ETC cycle without measurement is permitted after the change of the fuel.  Before testing, the parent engine shall be run-in using the procedure given in paragraph 3. of Appendix 2 to Annex 4A.

4.1.3.1.	At the manufacturer's request the engine may be tested on a third fuel instead of G23 (fuel 3) if the λ-shift factor (Sλ) lies between 0.89 (i.e. the lower range of GR) and 1.19 (i.e. the upper range of G25), for example when fuel 3 is a market fuel.  The results of this test may be used as a basis for the evaluation of the conformity of the production.

4.1.4.	In the case of natural gas engines, the ratio of the emission results "r" shall be determined for each pollutant as follows:

or,

and,

4.1.5.	In the case of LPG the parent engine should demonstrate its capability to adapt to any fuel composition that may occur across the market.  In the case of LPG there are variations in C3/C4 composition.  These variations are reflected in the reference fuels.  The parent engine should meet the emission requirements on the reference fuels A and B as specified in Annex 5 without any readjustment to the fuelling between the two tests.  However, one adaptation run over one ETC cycle without measurement is permitted after the change of the fuel.  Before testing, the parent engine shall be run-in using the procedure defined in paragraph 3. of Appendix 2 to Annex 4A.

4.1.5.1.	The ratio of emission results "r" shall be determined for each pollutant as follows:

4.2.	Granting of a fuel range restricted approval

Fuel range restricted approval is granted subject to the following requirements.

4.2.1.	Exhaust emissions approval of an engine running on natural gas and laid out for operation on either the range of H-gases or on the range of L-gases

The parent engine shall be tested on the relevant reference fuel, as specified in Annex 5, for the relevant range. The fuels are GR (fuel 1) and G23 (fuel 3) for the H-range of gases and G25 (fuel 2) and G23 (fuel 3) for the L-range of gases.  The parent engine shall meet the requirements of this Regulation without any readjustment to the fuelling between the two tests.  However, one adaptation run over one ETC cycle without measurement is permitted after the change of the fuel.  Before testing, the parent engine shall be run-in using the procedure defined in paragraph 3. of Appendix 2 to Annex 4A.

4.2.1.1.	At the manufacturer's request the engine may be tested on a third fuel instead of G23 (fuel 3) if the λ-shift factor (Sλ) lies between 0.89 (i.e. the lower range of GR) and 1.19 (i.e. the upper range of G25), for example when fuel 3 is a market fuel.  The results of this test may be used as a basis for the evaluation of the conformity of the production.

4.2.1.2.	The ratio of emission results "r" shall be determined for each pollutant as follows:

or,

and,

4.2.1.3.	On delivery to the customer the engine shall bear a label (see paragraph 4.11.) stating for which range of gases the engine is approved.

4.2.2.	Exhaust emissions approval of an engine running on natural gas or LPG and laid out for operation on one specific fuel composition:

4.2.2.1.	The parent engine shall meet the emission requirements on the reference fuels GR and G25 in the case of natural gas, or the reference fuels A and B in the case of LPG, as specified in Annex 5.  Between the tests fine-tuning of the fuelling system is allowed.  This fine-tuning will consist of a recalibration of the fuelling database, without any alteration to either the basic control strategy or the basic structure of the database.  If necessary the exchange of parts that are directly related to the amount of fuel flow (such as injector nozzles) is allowed.

4.2.2.2.	At the manufacturer's request the engine may be tested on the reference fuels GR and G23, or on the reference fuels G25 and G23, in which case the approval is only valid for the H-range or the L-range of gases respectively.

4.2.2.3.	On delivery to the customer the engine shall bear a label (see paragraph 4.11.) stating for which fuel composition the engine has been calibrated.

APPROVAL OF NG-FUELLED ENGINES

|  | Paragraph 4.1.: Granting of a universal fuel approval | Number of test runs | Calculation of "r" | Paragraph 4.2.: Granting of a fuel restricted approval | Number of test runs | Calculation of "r" |
| --- | --- | --- | --- | --- | --- | --- |
| Refer to para. 4.1.2. NG-engine adaptable to any fuel composition | GR (1) and G25 (2) at manufacturer’s request engine may be tested on an additional market fuel (3), if S = 0.89 – 1.19 | 2 (max. 3) | r=fuel 2 (G25)fuel 1 (GR) and, if tested with an additional fuel ra=fuel 2 (G25)fuel 3 (market fuel) and rb=fuel 1 (GR)fuel 3 (G23 or market fuel) |  |  |  |
| Refer to para. 4.1.3. NG-engine which is self adaptive by a switch | GR (1) and G23 (3) for H and G25 (2) and G23 (3) for L at manufacturer’s request engine may be tested on a market fuel (3) instead of G23, if S = 0.89 – 1.19 | 2 for the H-range, and 2 for the L-range; at respective position of switch 4 | rb=fuel 1 (GR)fuel 3 (G23 or market fuel) and ra=fuel 2 (G25)fuel 3 (G23 or market fuel) |  |  |  |
| Refer to para. 4.2.1. NG-engine laid out for operation on either H-range gas or L-range gas |  |  |  | GR (1) and G23 (3) for H or G25 (2) and G23 (3) for L at manufacturer’s request engine may be tested on a market fuel (3) instead of G23, if S = 0.89 – 1.19 | 2 for the H-range or 2 for the L-range 2 | rb=fuel 1 (GR)fuel 3 (G23 or market fuel) for the H-range or ra=fuel 2 (G25)fuel 3 (G23 or market fuel) for the L-range |
| Refer to para. 4.2.2. NG-engine laid out for operation on one specific fuel composition |  |  |  | GR (1) and G25 (2), fine-tuning between the tests allowed; at manufacturer’s request engine may be tested on: GR (1) and G23 (3) for H or G25 (2) and G23 (3) for L | 2 or 2 for the H-range or 2 for the L-range 2 |  |

APPROVAL OF LPG-FUELLED ENGINES

|  | Paragraph 4.1.: Granting of a universal fuel approval | Number of test runs | Calculation of "r" | Paragraph 4.2.: Granting of a fuel restricted approval | Number of test runs | Calculation of "r" |
| --- | --- | --- | --- | --- | --- | --- |
| refer to para. 4.1.5 LPG-engine adaptable to any fuel composition | # fuel A and fuel B | 2 | r=fuel Bfuel A |  |  |  |
| refer to para. 4.2.2 LPG-engine laid out for operation on one specific fuel composition |  |  |  | fuel A and fuel B, fine-tuning between the tests allowed | 2 |  |

4.3.	Exhaust emissions approval of a member of a family

4.3.1.	With the exception of the case mentioned in paragraph 4.3.2., the approval of a parent engine shall be extended to all family members without further testing, for any fuel composition within the range for which the parent engine has been approved (in the case of engines described in paragraph 4.2.2.) or the same range of fuels (in the case of engines described in either paragraphs 4.1. or 4.2.) for which the parent engine has been approved.

4.3.2.	Secondary test engine

In case of an application for approval of an engine, or a vehicle in respect of its engine, that engine belonging to an engine family, if the technical service determines that, with regard to the selected parent engine the submitted application does not fully represent the engine family defined in Annex I, Apppendix 1, an alternative and if necessary an additional reference test engine may be selected by the technical service and tested.

4.4.	An approval number shall be assigned to each type approved.  Its first two digits (at present 05, corresponding to 05 series of amendments) shall indicate the series of amendments incorporating the most recent major technical amendments made to the Regulation at the time of issue of the approval.  The same Contracting Party shall not assign the same number to another engine type or vehicle type.

4.5.	Notice of approval or of extension or of refusal of approval or production definitely discontinued of an engine type or vehicle type pursuant to this Regulation shall be communicated to the Parties to the 1958 Agreement which apply this Regulation, by means of a form conforming to the model in annexes 2A or 2B, as applicable, to this Regulation.  Values measured during the type test shall also be shown.

4.6.	There shall be affixed, conspicuously and in a readily accessible place to every engine conforming to an engine type approved under this Regulation, or to every vehicle conforming to a vehicle type approved under this Regulation, an international approval mark consisting of:

4.6.1.	a circle surrounding the letter "E" followed by the distinguishing number of the country which has granted approval /;

4.6.2.	the number of this Regulation, followed by the letter "R", a dash and the approval number to the right of the circle prescribed in paragraph 4.6.1.

4.6.3.	However, the approval mark shall contain an additional character after the letter "R", the purpose of which is to distinguish the emission stages (emission limits, OBD, etc.) for which the approval has been granted according to the following table:

| Character | Row (a) | OBD Stage I (b) | OBD Stage II | Durability and in-use | NOx control (c) |
| --- | --- | --- | --- | --- | --- |
| B | B1(2005) | YES | - | YES | - |
| C | B1(2005) | YES | - | YES | YES |
| D | B2(2008) | YES | - | YES | - |
| E | B2(2008) | YES | - | YES | YES |
| F | B2(2008) | - | YES | YES | - |
| G | B2(2008) | - | YES | YES | YES |
| H | C | YES | - | YES | - |
| I | C | YES | - | YES | YES |
| J | C | - | YES | YES | - |
| K | C | - | YES | YES | YES |

(a)	In accordance with tables of paragraph 5.2.1. of this Regulation.

(b)	In accordance with paragraph 5.4. of this Regulation, gas engines are excluded from OBD stage I.

(c)	In accordance with paragraph 5.5. of this Regulation.

4.6.3.1.	For NG fuelled engines the approval mark shall contain a suffix after the national symbol, the purpose of which is to distinguish which range of gases the approval has been granted.  This mark will be as follows;

4.6.3.1.1.	H in case of the engine being approved and calibrated for the H-range of gases;

4.6.3.1.2.	L in case of the engine being approved and calibrated for the L-range of gases;

4.6.3.1.3.	HL in case of the engine being approved and calibrated for both the H-range and L-range of gases;

4.6.3.1.4.	Ht in case of the engine being approved and calibrated for a specific gas composition in the H-range of gases and transformable to another specific gas in the H-range of gases by fine tuning of the engine fuelling;

4.6.3.1.5.	Lt in case of the engine being approved and calibrated for a specific gas composition in the L-range of gases and transformable to another specific gas in the L-range of gases after fine tuning of the engine fuelling;

4.6.3.1.6.	HLt in the case of the engine being approved and calibrated for a specific gas composition in either the H-range or the L-range of gases and transformable to another specific gas in either the H-range or the L-range of gases by fine tuning of the engine fuelling.

4.7.	If the vehicle or engine conforms to an approved type under one or more other Regulations annexed to the Agreement, in the country which has granted approval under this Regulation, the symbol prescribed in paragraph 4.6.1. need not be repeated.  In such a case, the Regulation and approval numbers and the additional symbols of all the Regulations under which approval has been granted under this Regulation shall be placed in vertical columns to the right of the symbol prescribed in paragraph 4.6.1.

4.8.	The approval mark shall be placed close to or on the data plate affixed by the manufacturer to the approved type.

4.9.	Annex 3 to this Regulation gives examples of arrangements of approval marks.

4.10.	The engine approved as a technical unit shall bear, in addition to the approved mark:

4.10.1.	the trademark or trade name of the manufacturer of the engine;

4.10.2.	the manufacturer's commercial description.

4.11.	Labels

In the case of NG and LPG fuelled engines with a fuel range restricted type approval, the following labels are applicable:

4.11.1.	Content

The following information shall be given:

In the case of paragraph 4.2.1.3., the label shall state "ONLY FOR USE WITH NATURAL GAS RANGE H".  If applicable, "H" is replaced by "L".

In the case of paragraph 4.2.2.3., the label shall state "ONLY FOR USE WITH NATURAL GAS SPECIFICATION ......." or "ONLY FOR USE WITH LIQUEFIED PETROLEUM GAS SPECIFICATION .........", as applicable.  All the information in the relevant table(s) in Annex 5 shall be given with the individual constituents and limits specified by the engine manufacturer.

The letters and figures shall be at least 4 mm in height.

Note: If lack of space prevents such labelling, a simplified code may be used.  In this event, explanatory notes containing all the above information shall be easily accessible to any person filling the fuel tank or performing maintenance or repair on the engine and its accessories, as well as to the authorities concerned.  The site and content of these explanatory notes will be determined by agreement between the manufacturer and the approval authority.

4.11.2.	Properties

Labels shall be durable for the useful life of the engine.  Labels shall be clearly legible and their letters and figures shall be indelible.  Additionally, labels shall be attached in such a manner that their fixing is durable for the useful life of the engine, and the labels cannot be removed without destroying or defacing them.

4.11.3.	Placing

Labels shall be secured to an engine part necessary for normal engine operation and not normally requiring replacement during engine life.  Additionally, these labels shall be located so as to be readily visible to the average person after the engine has been completed with all the auxiliaries necessary for engine operation.

4.12.	In case of an application for approval for a vehicle type in respect of its engine, the marking specified in paragraph 4.11. shall also be placed close to fuel filling aperture.

4.13.	In case of an application for approval for a vehicle type with an approved engine, the marking specified in paragraph 4.11. shall also be placed close to the fuel filling aperture.

5.	SPECIFICATIONS AND TESTS

5.1.	General

5.1.1.	Emission control equipment

5.1.1.1.	The components liable to affect, where appropriate, the emission of gaseous and particulate pollutants from diesel and gas engines shall be so designed, constructed, assembled and installed as to enable the engine, in normal use, to comply with the provisions of this Regulation.

5.1.2.	The use of a defeat strategy is forbidden.

5.1.2.1.	The use of a multi-setting engine is forbidden until appropriate and robust provisions for multi-setting engines are laid down in this Regulation.

5.1.3.	Emission control strategy

5.1.3.1.	Any element of design and emission control strategy (ECS) liable to affect the emission of gaseous and particulate pollutants from diesel engines and the emission of gaseous pollutants from gas engines shall be so designed, constructed, assembled and installed as to enable the engine, in normal use, to comply with the provisions of this Regulation.  ECS consists of the base emission control strategy (BECS) and usually one or more auxiliary emission control strategies (AECS).

5.1.4.	Requirements for base emission control strategy

5.1.4.1.	The base emission control strategy (BECS) shall be so designed as to enable the engine, in normal use, to comply with the provisions of this Regulation.  Normal use is not restricted to the conditions of use as specified in paragraph 5.1.5.4.

5.1.5.	Requirements for auxiliary emission control strategy

5.1.5.1.	An auxiliary emission control strategy (AECS) may be installed to an engine or on a vehicle provided that the AECS:

- (a)	operates only outside the conditions of use specified in paragraph 5.1.5.4. for the purposes defined in paragraph 5.1.5.5. or,

- (b)	is activated only exceptionally within the conditions of use specified in paragraph 5.1.5.4. for the purposes defined in paragraph 5.1.5.6. and not longer than is needed for these purposes.

5.1.5.2.	An auxiliary emission control strategy (AECS) that operates within the conditions of use specified in paragraph 5.1.5.4. and which results in the use of a different or modified emission control strategy (ECS) to that normally employed during the applicable emission test cycles will be permitted if, in complying with the requirements of paragraph 5.1.7., it is fully demonstrated that the measure does not permanently reduce the effectiveness of the emission control system. In all other cases, such strategy shall be considered to be a defeat strategy.

5.1.5.3.	An auxiliary emission control strategy (AECS) that operates outside the conditions of use specified in paragraph 5.1.5.4. will be permitted if, in complying with the requirements of paragraph 5.1.7., it is fully demonstrated that the measure is the minimum strategy necessary for the purposes of paragraph 5.1.5.6. with respect to environmental protection and other technical aspects. In all other cases, such a strategy shall be considered to be a defeat strategy.

5.1.5.4.	As provided for in paragraph 5.1.5.1., the following conditions of use apply under steady state and transient engine operations:

- (a)	an altitude not exceeding 1,000 metres (or equivalent atmospheric pressure of 90 kPa), and;

- (b)	an ambient temperature within the range 275 K to 303 K (2 °C to 30 °C) / / and;

- (c)	engine coolant temperature within the range 343 K to 373 K (70 °C to 100 °C).

5.1.5.5.	An auxiliary emission control strategy (AECS) may be installed to an engine, or on a vehicle, provided that the operation of the AECS is included in the applicable approval test and is activated according to paragraph 5.1.5.6.

5.1.5.6.	The AECS is activated:

- (a)	only by on-board signals for the purpose of protecting the engine system (including air-handling device protection) and/or vehicle from damage, or;

- (b)	for purposes such as operational safety, emission default modes and limp-home strategies, or;

- (c)	for such purposes as excessive emissions prevention, cold start or warming-up, or;

- (d)	if it is used to trade-off the control of one regulated pollutant under specific ambient or operating conditions in order to maintain control of all other regulated pollutants within the emission limit values that are appropriate for the engine in question.  The overall effects of such an AECS is to compensate for naturally occurring phenomena and do so in a manner that provides acceptable control of all emission constituents.

5.1.6.	Requirements for torque limiters

5.1.6.1.	A torque limiter will be permitted if it complies with the requirements of paragraph 5.1.6.2. or 5.5.5.  In all other cases, a torque limiter shall be considered to be a defeat strategy.

5.1.6.2.	A torque limiter may be installed to an engine, or on a vehicle, provided that:

- (a)	the torque limiter is activated only by on-board signals for the purpose of protecting the powertrain or vehicle construction from damage and/or for the purpose of vehicle safety, or for power take-off activation when the vehicle is stationary, or for measures to ensure the correct functioning of the deNOx system, and;

- (b)	the torque limiter is active only temporarily, and;

- (c)	the torque limiter does not modify the emission control strategy (ECS), and;

- (d)	in case of power take-off or powertrain protection the torque is limited to a constant value, independent from the engine speed, while never exceeding the full-load torque, and;

- (e)	is activated in the same manner to limit the performance of a vehicle in order to encourage the driver to take the necessary measures in order to ensure the correct functioning of NOx control measures within the engine system.

5.1.7.	Special requirements for electronic emission control systems

5.1.7.1.	Documentation requirements

The manufacturer shall provide a documentation package that gives access to any element of design and emission control strategy (ECS), and torque limiter of the engine system and the means by which it controls its output variables, whether that control is direct or indirect.  The documentation shall be made available in two parts:

(a)	the formal documentation package, which shall be supplied to the technical service at the time of submission of the approval application, shall include a full description of the ECS and, if applicable, the torque limiter.  This documentation may be brief, provided that it exhibits evidence that all outputs permitted by a matrix obtained from the range of control of the individual unit inputs have been identified.  This information shall be attached to the documentation required in paragraph 3. of this Regulation;

(b)	additional material that shows the parameters that are modified by any auxiliary emission control strategy (AECS) and the boundary conditions under which the AECS operates.  The additional material shall include a description of the fuel system control logic, timing strategies and switch points during all modes of operation.  It shall also include a description of the torque limiter described in paragraph 5.5.5. of this Regulation.

The additional material shall also contain a justification for the use of any AECS and include additional material and test data to demonstrate the effect on exhaust emissions of any AECS installed to the engine or on the vehicle.  The justification for the use of an AECS may be based on test data and/or sound engineering analysis.

This additional material shall remain strictly confidential, and be made available to the approval authority on request.  The approval authority will keep this material confidential.

5.1.8.	Specifically for the approval of engines according to row A of the tables in paragraph 5.2.1. (engines not normally tested on ETC)

5.1.8.1.	To verify whether any strategy or measure should be considered a defeat strategy according to the definitions given in paragraph 2., the approval authority and/or the technical service may additionally request a NOx screening test using the ETC which may be carried out in combination with either the approval test or the procedures for checking the conformity of production.

5.1.8.2.	In verifying whether any strategy or measure should be considered a defeat strategy according to the definitions given in paragraph 2., an additional margin of 10 per cent, related to the appropriate NOx limit value, shall be accepted.

5.1.9.	Provisions for electronic system security

5.1.9.1.	Any vehicle with an Emission Control Unit shall include features to deter modification, except as authorized by the manufacturer.  The manufacturer shall authorize modifications if these modifications are necessary for the diagnosis, servicing, inspection, retrofitting or repair of the vehicle.  Any reprogrammable computer codes or operating parameters shall be resistant to tampering and afford a level of protection at least as good as the provisions in ISO 15031-7 (SAE J2186) provided that the security exchange is conducted using the protocols and diagnostic connector as prescribed in paragraph 6. of Annex 9A to this Regulation.  Any removable calibration memory chips shall be plotted, encased in a sealed container or protected by electronic algorithms and shall not be changeable without the use of specialised tools and procedures.

5.1.9.2.	Computer-coded engine operating parameters shall not be changeable without the use of specialised tools and procedures (e.g. soldered or potted computer components or sealed (or soldered) computer enclosures).

5.1.9.3.	Manufacturers shall take adequate steps to protect the maximum fuel delivery setting from tampering while a vehicle is in-service.

5.1.9.4.	Manufacturers may apply to the approval authority for an exemption from one of these requirements for those vehicles that are unlikely to require protection.  The criteria that the approval authority will evaluate in considering an exemption will include, but are not limited to, the current availability of performance chips, the high-performance capability of the vehicle and the projected sales volume of the vehicle.

5.1.9.5.	Manufacturers using programmable computer code systems (e.g. electrical erasable programmable read-only memory, EEPROM) shall deter unauthorized reprogramming.  Manufacturers shall include enhanced tamper-protection strategies and write protect features requiring electronic access to an off-site computer maintained by the manufacturer.  Alternative methods giving an equivalent level of tamper protection may be approved by the authority.

5.2.	Specifications concerning the emission of gaseous and particulate pollutants and smoke

For type approval testing to either row B1 or B2 or row C of the tables in paragraph 5.2.1. the emissions shall be determined on the ESC, ELR and ETC tests.

For gas engines, the gaseous emissions shall be determined on the ETC test.

The ESC and ELR test procedures are described in Annex 4A, Appendix 1, the ETC test procedure in Annex 4A, Appendices 2 and 3.

The emissions of gaseous pollutants and particulate pollutants, if applicable, and smoke, if applicable, by the engine submitted for testing shall be measured by the methods described in Annex 4A, Appendix 4.  Annex 4A, Appendix 7 describes the recommended analytical systems for the gaseous pollutants, the recommended particulate sampling systems, and the recommended smoke measurement system.

Other systems or analysers may be approved by the Technical Service if it is found that they yield equivalent results on the respective test cycle.  The determination of system equivalency shall be based upon a 7-sample pair (or larger) correlation study between the system under consideration and one of the reference systems of this Regulation.  For particulate emissions, only the full flow dilution system or the partial flow dilution system meeting the requirements of ISO 16183 are recognized as equivalent reference systems.  "Results" refer to the specific cycle emissions value.  The correlation testing shall be performed at the same laboratory, test cell, and on the same engine, and is preferred to be run concurrently.  The equivalency of the sample pair averages shall be determined by F-test and t-test statistics as described in Appendix 4 to this Regulation obtained under these laboratory, test cell and engine conditions.  Outliers shall be determined in accordance with ISO 5725 and excluded from the database.  For introduction of a new system into this Regulation, determination of equivalency shall be based upon the calculation of repeatability and reproducibility, as described in ISO 5725.

5.2.1.	Limit Values

The specific mass of the carbon monoxide, of the total hydrocarbons, of the oxides of nitrogen and of the particulates, as determined on the ESC test, and of the smoke opacity, as determined on the ELR test, shall not exceed the amounts shown in table 1.

Table 1

Limit values — ESC and ELR tests

| Row | Mass of carbon monoxide (CO) g/kWh | Mass of hydrocarbons (HC) g/kWh | Mass of nitrogen oxides (NOx) g/kWh | Mass of particulates (PT) g/kWh | Smoke m−1 |
| --- | --- | --- | --- | --- | --- |
| A (2000) | 2.1 | 0.66 | 5.0 | 0.10 // 0.13 (a) | 0.8 |
| B1 (2005) | 1.5 | 0.46 | 3.5 | 0.02 | 0.5 |
| B2 (2008) | 1.5 | 0.46 | 2.0 | 0.02 | 0.5 |
| C (EEV) | 1.5 | 0.25 | 2.0 | 0.02 | 0.15 |
| (a) For engines having a swept volume of less than 0.75 dm³ per cylinder and a rated power speed of more than 3,000 min-1 |  |  |  |  |  |

Table 2

Limit values — ETC tests

| Row | Mass of carbon monoxide (CO) g/kWh | Mass of non-methane hydrocarbons (NMHC) g/kWh | Mass of (CH4)(a) g/kWh | Mass of nitrogen oxides (NOx) g/kWh | Mass of particulates (PT) (PT) (b) g/kWh |
| --- | --- | --- | --- | --- | --- |
| A (2000) | 5.45 | 0.78 | 1.6 | 5.0 | 0.16 // 0.21 (c) |
| B1 (2005) | 4.0 | 0.55 | 1.1 | 3.5 | 0.03 |
| B2 (2008) | 4.0 | 0.55 | 1.1 | 2.0 | 0.03 |
| C (EEV) | 3.0 | 0.40 | 0.65 | 2.0 | 0.02 |
| (a) For NG engines only. (b) Not applicable for gas fuelled engines at stages B1 and B2. (c) For engines having a swept volume of less than 0.75 dm³ per cylinder and a rated power speed of more than 3,000 min-1 |  |  |  |  |  |

5.2.2.	Hydrocarbon measurement for diesel and gas fuelled engines

5.2.2.1.	A manufacturer may choose to measure the mass of total hydrocarbons (THC) on the ETC test instead of measuring the mass of non-methane hydrocarbons.  In this case, the limit for the mass of total hydrocarbons is the same as shown in table 2 for the mass of non-methane hydrocarbons.

5.2.3.	Specific requirements for diesel engines

5.2.3.1.	The specific mass of the oxides of nitrogen measured at the random check points within the control area of the ESC test shall not exceed by more than 10 per cent the values interpolated from the adjacent test modes (reference Annex 4A, Appendix 1, paragraphs 5.6.2. and 5.6.3.).

5.2.3.2.	The smoke value on the random test speed of the ELR shall not exceed the highest smoke value of the two adjacent test speeds by more than 20 per cent, or by more than 5 per cent of the limit value, whichever is greater.

5.3.	Durability and deterioration factors

5.3.1.	The manufacturer shall demonstrate that a compression-ignition or gas engine approved by reference to the emission limits set out in row B1 or row B2 or row C of the tables in paragraph 5.2.1. will comply with those emission limits for a useful life of:

5.3.1.1.	100,000 km or five years, whichever is the sooner, in the case of engines to be fitted to vehicles of category N1 and M2;

5.3.1.2.	200,000 km or six years, whichever is the sooner, in the case of engines to be fitted to vehicles of category N2, N3 with a maximum technically permissible mass not exceeding 16 tonnes and M3 Class I, Class II and Class A, and Class B with a maximum technically permissible mass not exceeding 7.5 tonnes;

5.3.1.3.	500,000 km or seven years, whichever is the sooner, in the case of engines to be fitted to vehicles of category N3 with a maximum technically permissible mass exceeding 16 tonnes and M3, Class III and Class B with a maximum technically permissible mass exceeding 7.5 tonnes.

5.3.2.	For the purposes of this Regulation, the manufacturer shall determine deterioration factors that will be used to demonstrate that the gaseous and particulate emissions of an engine family or engine-aftertreatment system family remain in conformity with the appropriate emission limits specified in the tables in paragraph 5.2.1. over the appropriate durability period laid down in paragraph 5.3.1.

5.3.3.	The procedures for demonstrating the compliance of an engine or engine-aftertreatment system family with the relevant emission limits over the appropriate durability period are given in Annex 7 to this Regulation.

5.4.	On-board diagnostic (OBD) system

5.4.1.	A compression-ignition engine approved by reference to the emission limit values set out in row B1 or row C of the tables in paragraph 5.2.1. or a vehicle propelled by such an engine shall be fitted with an on-board diagnostic (OBD) system that signals the presence of a fault to the driver if the OBD threshold limits set out in row B1 or row C of the table in paragraph 5.4.4. are exceeded.  The OBD system for emission control shall be in accordance with the requirements of Annex 9A to this Regulation.

5.4.1.1.	In the case of exhaust after-treatment systems, the OBD system may monitor for major functional failure of any of the following:

(a)	a catalyst, where fitted as a separate unit, whether or not it is part of a deNOx system or a diesel particulate filter;

(b)	a deNOx system, where fitted;

(c)	a diesel particulate filter, where fitted;

(d)	a combined deNOx-diesel particulate filter system.

5.4.2.	From 1 October 2008 for new approvals and from 1 October 2009 for all approvals, a compression-ignition or a gas engine approved by reference to the emission limit values set out in row B2 or row C of the tables in paragraph 5.2.1., or a vehicle propelled by such an engine shall be fitted with an OBD system that signals the presence of a fault to the driver if the OBD threshold limits set out in row B2 or row C of the table in paragraph 5.4.4. are exceeded.  The on-board diagnostic (OBD) system for emission control shall be in accordance with the requirements of Annex 9A to this Regulation.

5.4.3.	The OBD system shall also include an interface between the engine electronic control unit (EECU) and any other engine or vehicle electrical or electronic systems that provide an input to or receive an output from the EECU and which affect the correct functioning of the emission control system, such as the interface between the EECU and a transmission electronic control unit.

5.4.4.	The OBD threshold limits shall be as follows:

| Row | Compression-ignition engines |  |
| --- | --- | --- |
|  | Mass of oxides of nitrogen | Mass of particulate |
|  | (NOx) g/kWh | (PT) g/kWh |
| B1 (2005) | 7.0 | 0.1 |
| B2 (2008) | 7.0 | 0.1 |
| C (EEV) | 7.0 | 0.1 |

5.4.5.	Full and uniform access to OBD information shall be provided for the purposes of testing, diagnosis, servicing and repair in keeping with the relevant provisions of ECE Regulation No. 83 and provisions regarding replacement components ensuring compatibility with OBD systems.

5.4.6.	Small batch engine production

As an alternative to the requirements of this paragraph, engine manufacturers whose world-wide annual production of a type of engine, belonging to an OBD engine family:

- (a)	is less than 500 units per year, may obtain approval on the basis of the requirements of the present Regulation where the engine is monitored only for circuit continuity and the after-treatment system is monitored for major functional failure;

- (b)	is less than 50 units per year, may obtain approval on the basis of the requirements of the present Regulation where the complete emission control system (i.e. the engine and after-treatment system) are monitored only for circuit continuity.

The approval authority shall inform the other Contracting Parties of the circumstances of each approval granted under this provision.

5.5.	Requirements to ensure correct operation of NOx control measures

5.5.1.	General

5.5.1.1.	This paragraph is applicable to compression-ignition engine systems irrespective of the technology used to comply with the emission limit values provided in the tables in paragraph 5.2.1.

5.5.1.2.	Application dates

The application dates shall be in accordance with paragraph 13. of this Regulation.

5.5.1.3.	Any engine system covered by this paragraph shall be designed, constructed and installed so as to be capable of meeting these requirements over the useful life of the engine.

5.5.1.4.	Information that fully describes the functional operational characteristics of an engine system covered by this paragraph shall be provided by the manufacturer in Annex 1.

5.5.1.5.	In its application for approval, if the engine system requires a reagent, the manufacturer shall specify the characteristics of all reagent(s) consumed by any exhaust aftertreatment system, e.g. type and concentrations, operational temperature conditions, reference to international standards etc.

5.5.1.6.	Subject to requirements set out in paragraph 5.1., any engine system covered by this paragraph shall retain its emission control function during all conditions regularly pertaining in the territory of the Community, especially at low ambient temperatures.

5.5.1.7.	For the purpose of approval, the manufacturer shall demonstrate to the Technical Service that for engine systems that require a reagent, any emission of ammonia does not exceed, over the applicable emissions test cycle, a mean value of 25 ppm.

5.5.1.8.	For engine systems requiring a reagent, each separate reagent tank installed on a vehicle shall include means for taking a sample of any fluid inside the tank.  The sampling point shall be easily accessible without the use of any specialised tool or device.

5.5.2.	Maintenance requirements

5.5.2.1.	The manufacturer shall furnish or cause to be furnished to all owners of new heavy-duty vehicles or new heavy-duty engines written instructions that shall state that if the vehicle emission control system is not functioning correctly, the driver shall be informed of a problem by the malfunction indicator (MI) and the engine shall consequentially operate with a reduced performance.

5.5.2.2.	The instructions will indicate requirements for the proper use and maintenance of vehicles, including where relevant the use of consumable reagents.

5.5.2.3.	The instructions shall be written in clear and non-technical language and in the language of the country in which a new heavy-duty vehicle or new heavy-duty engine is sold or registered.

5.5.2.4.	The instructions shall specify if consumable reagents have to be refilled by the vehicle operator between normal maintenance intervals and shall indicate a likely rate of reagent consumption according to the type of new heavy-duty vehicle.

5.5.2.5.	The instructions shall specify that use of and refilling of a required reagent of the correct specifications when indicated is mandatory for the vehicle to comply with the certificate of conformity issued for that vehicle or engine type.

5.5.2.6.	The instructions shall state that it may be a criminal offence to use a vehicle that does not consume any reagent if it is required for the reduction of pollutant emissions and that, in consequence, any favourable conditions for the purchase or operation of the vehicle obtained in the country of registration or other country in which the vehicle is used may become invalid.

5.5.3.	Engine system NOx control

5.5.3.1.	Incorrect operation of the engine system with respect to NOx emissions control (for example due to lack of any required reagent, incorrect EGR flow or deactivation of EGR) shall be determined through monitoring of the NOx level by sensors positioned in the exhaust stream.

5.5.3.2.	Any deviation in NOx level more than 1.5 g/kWh above the applicable limit value given in table 1 of paragraph 5.2.1., shall result in the driver being informed by activation of the MI as referred to in paragraph 3.6.5. of Annex 9A to this Regulation.

5.5.3.3.	In addition, a non-erasable fault code identifying the reason why NOx exceeds the levels specified in paragraph 5.5.3.2. shall be stored in accordance with paragraph 3.9.2. of Annex 9A to this Regulation for at least 400 days or 9,600 hours of engine operation.

The reasons for the NOx exceedance shall, at a minimum, and where applicable, be identified in the cases of empty reagent tank, interruption of reagent dosing activity, insufficient reagent quality, too low reagent consumption, incorrect EGR flow or deactivation of the EGR. In all other cases, the manufacturer is permitted to refer to a non-erasable fault code "high NOx - root cause unknown".

5.5.3.4.	If the NOx level exceeds the OBD threshold limit values given in the table in Paragraph 5.4.4., a torque limiter shall reduce the performance of the engine according to the requirements of paragraph 5.5.5. in a manner that is clearly perceived by the driver of the vehicle. When the torque limiter is activated the driver shall continue to be alerted according to the requirements of paragraph 5.5.3.2. and a non-erasable fault code shall be stored in accordance with paragraph 5.5.3.3.

5.5.3.5	In the case of engine systems that rely on the use of EGR and no other aftertreatment system for NOx emissions control, the manufacturer may utilise an alternative method to the requirements of paragraph 5.5.3.1. for the determination of the NOx level. At the time of type approval the manufacturer shall demonstrate that the alternative method is equally timely and accurate in determining the NOx level compared to the requirements of paragraph 5.5.3.1. and that it triggers the same consequences as those referred to in paragraphs 5.5.3.2., 5.5.3.3. and 5.5.3.4.

5.5.4.	Reagent control

5.5.4.1.	For vehicles that require the use of a reagent to fulfil the requirements of this paragraph, the driver shall be informed of the level of reagent in the on-vehicle reagent storage tank through a specific mechanical or electronic indication on the vehicle's dashboard.  This shall include a warning when the level of reagent goes:

- (a)	below 10 per cent of the tank or a higher percentage at the choice of the manufacturer, or;

- (b)	below the level corresponding to the driving distance possible with the fuel reserve level specified by the manufacturer.

The reagent indicator shall be placed in close proximity to the fuel level indicator.

5.5.4.2.	The driver shall be informed, according to the requirements of paragraph 3.6.5. of Annex 9A to this Regulation, if the reagent tank becomes empty.

5.5.4.3.	As soon as the reagent tank becomes empty, the requirements of paragraph 5.5.5. shall apply in addition to the requirements of paragraph 5.5.4.2.

5.5.4.4.	A manufacturer may choose to comply with the paragraphs 5.5.4.5. to 5.5.4.12. as an alternative to complying with the requirements of paragraph 5.5.3.

5.5.4.5.	Engine systems shall include a means of determining that a fluid corresponding to the reagent characteristics declared by the manufacturer and recorded in Annex 1 to this Regulation is present on the vehicle.

5.5.4.6.	If the fluid in the reagent tank does not correspond to the minimum requirements declared by the manufacturer as recorded in Annex 1 to this Regulation the additional requirements of paragraph 5.5.4.12. shall apply.

5.5.4.7.	Engine systems shall include a means for determining reagent consumption and providing off-board access to consumption information.

5.5.4.8.	Average reagent consumption and average demanded reagent consumption by the engine system either over the previous complete 48 hour period of engine operation or the period needed for a demanded reagent consumption of at least 15 litres, whichever is longer, shall be available via the serial port of the standard diagnostic connector as referred to in paragraph 6.8.3. of Annex 9A to this Regulation.

5.5.4.9.	In order to monitor reagent consumption, at least the following parameters within the engine shall be monitored:

- (a)	level of reagent in on-vehicle storage tank;

- (b)	flow of reagent or injection of reagent as close as technically possible to the point of injection into an exhaust aftertreatment system.

5.5.4.10.	Any deviation more than 50 per cent in average reagent consumption and average demanded reagent consumption by the engine system over the period defined in paragraph 5.5.4.8. shall result in application of the measures laid down in paragraph 5.5.4.12.

5.5.4.11.	In the case of interruption in reagent dosing activity the measures laid down in paragraph 5.5.4.12. shall apply.  This is not required where such interruption is demanded by the engine ECU because engine operating conditions are such that the engine's emission performance does not require reagent dosing, provided that the manufacturer has clearly informed the approval authority when such operating conditions apply.

5.5.4.12.	Any failure detected with respect to paragraphs 5.5.4.6., 5.5.4.10. or 5.5.4.11. shall trigger the same consequences in the same order as those referred to in paragraphs 5.5.3.2., 5.5.3.3. or 5.5.3.4.

5.5.5.	Measures to discourage tampering of exhaust aftertreatment systems

5.5.5.1.	Any engine system covered by this paragraph shall include a torque limiter that will alert the driver that the engine system is operating incorrectly or the vehicle is being operated in an incorrect manner and thereby encourage the prompt rectification of any fault(s).

5.5.5.2.	The torque limiter shall be activated when the vehicle becomes stationary for the first time after the conditions of either paragraphs 5.5.3.4., 5.5.4.3., 5.5.4.6., 5.5.4.10. or 5.5.4.11. have occurred.

5.5.5.3.	Where the torque limiter comes into effect, the engine torque shall not, in any case, exceed a constant value of:

- (a)	60 per cent of the engine maximum torque for vehicles of category N3 > 16 tons, M1 > 7.5 tons, M3/III and M3/B > 7.5 tons /;

- (b)	75 per cent of the engine maximum torque for vehicles of category N1, N2, N3 ≤ 16 tons, 3.5 < M1  7.5 tons, M2, M3/I, M3/II, M3/A and M3/B ≤ 7.5 tons7.

5.5.5.4.	Requirements for documentation and the torque limiter are set out in paragraphs 5.5.5.5. to 5.5.5.8.

5.5.5.5.	Detailed written information fully describing the functional operation characteristics of the emission control monitoring system and the torque limiter shall be specified according to the documentation requirements of paragraph 5.1.7.1.(b).  Specifically, the manufacturer shall provide information on the algorithms used by the ECU for relating the NOx concentration to the specific NOx emission (in g/kWh) on the ETC in accordance with paragraph 5.5.6.5.

5.5.5.6.	The torque limiter shall be deactivated when the engine speed is at idle if the conditions for its activation have ceased to exist.  The torque limiter shall not be automatically deactivated without the reason for its activation being remedied.

5.5.5.7.	Deactivation of the torque limiter shall not be feasible by means of a switch or a maintenance tool.

5.5.5.8.	The torque limiter shall not apply to engines or vehicles for use by the armed services, by rescue services and by fire-services and ambulances.  Permanent deactivation shall only be done by the engine or vehicle manufacturer, and a special engine type within the engine family shall be designated for proper identification.

5.5.6.	Operating conditions of the emission control monitoring system

5.5.6.1.	The emission control monitoring system shall be operational,

- (a)	at all ambient temperatures between 266 K and 308 K (-7 °C and 35 °C);

- (b)	at all altitudes below 1600 m;

- (c)	at engine coolant temperatures above 343 K (70 °C).

This paragraph does not apply in the case of monitoring for reagent level in the storage tank where monitoring shall be conducted under all conditions of use.

5.5.6.2.	The emission control monitoring system may be deactivated when a limp-home strategy is active and which results in a torque reduction greater than the levels indicated in paragraph 5.5.5.3. for the appropriate vehicle category.

5.5.6.3.	If an emission default mode is active, the emission control monitoring system shall remain operational and comply with the provisions of paragraph 5.5.

5.5.6.4.	The incorrect operation of NOx control measures shall be detected within four OBD test cycles as referred to in the definition given in paragraph 6.1. of Appendix 1 of Annex 9A to this Regulation.

5.5.6.5.	Algorithms used by the ECU for relating the actual NOx concentration to the specific NOx emission (in g/kWh) on the ETC shall not be considered to be a defeat strategy.

5.5.6.6.	If an AECS that has been approved by the approval authority in accordance with paragraph 5.1.5. becomes operational, any increase in NOx due to the operation of the AECS may be applied to the appropriate NOx level referred to in paragraph 5.5.3.2.  In all such cases, the influence of the AECS on the NOx threshold shall be described in accordance with paragraph 5.5.5.5.

5.5.7.	Failure of the emission control monitoring system

5.5.7.1.	The emission control monitoring system shall be monitored for electrical failures and for removal or deactivation of any sensor that prevents it from diagnosing an emission increase as required by paragraphs 5.5.3.2. and 5.5.3.4.

Examples of sensors that affect the diagnostic capability are those directly measuring NOx concentration, urea quality sensors, and sensors used for monitoring reagent dosing activity, reagent level, reagent consumption or EGR rate.

5.5.7.2.	If a failure of the emission control monitoring system is confirmed, the driver shall be immediately alerted by the activation of the warning signal according to paragraph 3.6.5. of Annex 9A to this Regulation.

5.5.7.3.	The torque limiter shall be activated in accordance with paragraph 5.5.5. if the failure is not remedied within 50 hours of engine operation.

The period laid down in the first subparagraph shall be reduced to 36 hours from the dates specified in paragraph 14.2.3.

5.5.7.4.	When the emission control monitoring system has determined that the failure has ceased to exist, the fault code(s) associated with that failure may be cleared from the system memory, except in the cases referred to in paragraph 5.5.7.5., and the torque limiter, if applicable, shall be deactivated according to paragraph 5.5.5.6.

Fault code(s) associated with a failure of the emission control monitoring system shall not be capable of being cleared from the system memory by any scan tool.

5.5.7.5.	In the case of the removal or deactivation of elements of the emission control monitoring system, in accordance with paragraph 5.5.7.1., a non-erasable fault code shall be stored in accordance with paragraph 3.9.2. of Annex 9A to this Regulation for a minimum of 400 days or 9,600 hours of engine operation.

5.5.8.	Demonstration of the emission control monitoring system

5.5.8.1.	As part of the application for approval provided for in paragraph 3., the manufacturer shall demonstrate the conformity of the provisions of this paragraph by tests on an engine dynamometer in accordance with paragraphs 5.5.8.2. to 5.5.8.7.

5.5.8.2.	The compliance of an engine family or an OBD engine family to the requirements of this paragraph may be demonstrated by testing the emission control monitoring system of one of the members of the family (the parent engine), provided the manufacturer demonstrates to the type approval authority that the emission control monitoring systems are similar within the family.

This demonstration may be performed by presenting to the approval authorities such elements as algorithms, functional analyses, etc.

The parent engine is selected by the manufacturer in agreement with the type approval authority.

5.5.8.3.	The testing of the emission control monitoring system consists of the following three phases:

(a)	Selection:

An incorrect operation of the NOx control measures or a failure of the emission control monitoring system is selected by the authority within a list of incorrect operations provided by the manufacturer.

(b)	Qualification:

The influence of the incorrect operation is validated by measuring the NOx level over the ETC on an engine test bed.

(c)	Demonstration:

The reaction of the system (torque reduction, warning signal, etc.) shall be demonstrated by running the engine on four OBD test cycles.

5.5.8.3.1.	For the selection phase, the manufacturer shall provide the type approval authority with a description of the monitoring strategies used to determine potential incorrect operation of any NOx control measure and potential failures in the emission control monitoring system that would lead either to activation of the torque limiter or to activation of the warning signal only.

Typical examples of incorrect operations for this list are an empty reagent tank, an incorrect operation leading to an interruption of reagent dosing activity, an insufficient reagent quality, an incorrect operation leading to low reagent consumption, an incorrect EGR flow or a deactivation of the EGR.

A minimum of two and a maximum of three incorrect operations of the NOx control system or failures of the emission control monitoring system shall be selected by the type approval authority from this list.

5.5.8.3.2.	For the qualification phase, the NOx emissions shall be measured over the ETC test cycle, according to the provisions of Appendix 2 to Annex 4A.  The result of the ETC test shall be used to determine in which way the NOx control monitoring system is expected to react during the demonstration process (torque reduction and/or warning signal).  The failure shall be simulated in a way that the NOx level does not exceed by more than 1 g/kWh any of the threshold levels given in paragraphs 5.5.3.2. or 5.5.3.4.

Emissions qualification is not required in case of an empty reagent tank or for demonstrating a failure of the emission control monitoring system.

The torque limiter shall be deactivated during the qualification phase.

5.5.8.3.3.	For the demonstration phase, the engine shall be run over a maximum of four OBD test cycles.

No failure other than the ones which are being considered for demonstration purposes shall be present.

5.5.8.3.4.	Prior to starting the test sequence of paragraph 5.5.8.3.3., the emission control monitoring system shall be set to a "no failure" status.

5.5.8.3.5.	Depending on the NOx level selected, the system shall activate a warning signal and in addition, if applicable, the torque limiter at any time before the end of the detection sequence.  The detection sequence may be stopped once the NOx control monitoring system has properly reacted.

5.5.8.4.	In the case of an emission control monitoring system principally based on monitoring the NOx level by sensors positioned in the exhaust stream, the manufacturer may choose to directly monitor certain system functionalities (e.g. interruption of dosing activity, closed EGR valve) for the determination of compliance. In that case, the selected system functionality shall be demonstrated.

5.5.8.5.	The level of torque reduction required in paragraph 5.5.5.3. by the torque limiter shall be approved together with the general engine performance approval in accordance with Regulation No. 85.  For the demonstration process, the manufacturer shall demonstrate to the approval authority the inclusion of the correct torque limiter into the engine ECU.  Separate torque measurement during the demonstration is not required.

5.5.8.6.	As an alternative to paragraphs 5.5.8.3.3. to 5.5.8.3.5., the demonstration of the emission control monitoring system and the torque limiter may be performed by testing a vehicle. The vehicle shall be driven on the road or on a test track with the selected incorrect operations or failures of the emission control monitoring system to demonstrate that the warning signal and activation of the torque limiter will operate in accordance with the requirements of paragraph 5.5., and, in particular, those in paragraphs 5.5.5.2. and 5.5.5.3.

5.5.8.7.	If the storage in the computer memory of a non-erasable fault code is required for complying with the requirements of paragraph 5.5., the following three conditions shall be met by the end of demonstration sequence:

- (a)	that it is possible to confirm via the OBD scan tool the presence in the OBD computer memory of the appropriate non-erasable fault code described in paragraph 5.5.3.3. and that it can be shown to the satisfaction of the type approval authority that the scan tool cannot erase it, and;

- (b)	that it is possible to confirm the time spent during the detection sequence with the warning signal activated by reading the non-erasable counter referred to in paragraph 3.9.2. of Annex 9A to this Regulation , and that it can be shown to the satisfaction of the type approval authority that the scan tool cannot erase it, and;

- (c)	that the approval authority has approved the elements of design showing that this non-erasable information is stored in accordance with paragraph 3.9.2. of Annex 9A to this Regulation for a minimum of 400 days or 9,600 hours of engine operation.

6.	INSTALLATION ON THE VEHICLE

6.1.	The engine installation on the vehicle shall comply with the following characteristics in respect to the approval of the engine:

6.1.1.	intake depression shall not exceed that specified for the approved engine in Annex 2A;

6.1.2.	exhaust back pressure shall not exceed that specified for the approved engine in Annex 2A;

6.1.3.	power absorbed by the auxiliaries needed for operating the engine shall not exceed that specified for the approved engine in Annex 2A.

7.	ENGINE FAMILY

7.1.	Parameters defining the engine family

The engine family, as determined by the engine manufacturer shall comply with the provisions of ISO 16185.

7.1.1.	Combustion cycle:

- (a)	2 cycle

- (b)	4 cycle

7.1.2.	Cooling medium:

- (a)	air

- (b)	water

- (c)	oil

7.1.3.	For gas engines and engines with aftertreatment

- (a)	Number of cylinders (other diesel engines with fewer cylinders than the parent engine may be considered to belong to the same engine family provided the fuelling system meters fuel for each individual cylinder).

7.1.4.	Individual cylinder displacement:

- (a)	engines to be within a total spread of 15 per cent

7.1.5.	Method of air aspiration:

- (a)	naturally aspirated

- (b)	pressure charged

- (c)	pressure charged with charge air cooler

7.1.6.	Combustion chamber type/design:

- (a)	pre-chamber

- (b)	swirl chamber

- (c)	open chamber

7.1.7.	Valve and porting — configuration, size and number:

- (a)	cylinder head

- (b)	cylinder wall

- (c)	crankcase

7.1.8.	Fuel injection system (diesel engines):

- (a)	pump-line-injector

- (b)	in-line pump

- (c)	distributor pump

- (d)	single element

- (e)	unit injector

7.1.9.	Fuelling system (gas engines):

- (a)	mixing unit

- (b)	gas induction/injection (single point, multi-point)

- (c)	liquid injection (single point, multi-point).

7.1.10.	Ignition system (gas engines)

7.1.11.	Miscellaneous features:

- (a)	exhaust gas recirculation

- (b)	water injection/emulsion

- (c)	secondary air injection

- (d)	charge cooling system

7.1.12.	Exhaust aftertreatment:

- (a)	3-way-catalyst

- (b)	oxidation catalyst

- (c)	reduction catalyst

- (d)	thermal reactor

- (e)	particulate trap

7.2.	Choice of the parent engine

7.2.1.	Diesel engines

The parent engine of the family shall be selected using the primary criteria of the highest fuel delivery per stroke at the declared maximum torque speed.  In the event that two or more engines share this primary criteria, the parent engine shall be selected using the secondary criteria of highest fuel delivery per stroke at rated speed.  Under certain circumstances, the approval authority may conclude that the worst case emission rate of the family can best be characterized by testing a second engine.  Thus, the approval authority may select an additional engine for test based upon features which indicate that it may have the highest emission level of the engines within that family.

If engines within the family incorporate other variable features which could be considered to affect exhaust emissions, these features shall also be identified and taken into account in the selection of the parent engine.

7.2.2.	Gas engines

The parent engine of the family shall be selected using the primary criteria of the largest displacement.  In the event that two or more engines share this primary criteria, the parent engine shall be selected using the secondary criteria in the following order:

- (a)	the highest fuel delivery per stroke at the speed of declared rated power;

- (b)	the most advanced spark timing;

- (c)	the lowest EGR rate;

- (d)	no air pump or lowest actual air flow pump.

Under certain circumstances, the approval authority may conclude that the worst case emission rate of the family can best be characterized by testing a second engine.  Thus, the approval authority may select an additional engine for test based upon features which indicate that it may have the highest emission level of the engines within that family.

7.3.	Parameters for defining an OBD-engine family

The OBD-engine family may be defined by basic design parameters that shall be common to engine systems within the family.

In order that engine systems may be considered to belong to the same OBD-engine family, the following list of basic parameters shall be common:

- (a)	the methods of OBD monitoring;

- (b)	the methods of malfunction detection;

unless these methods have been shown as equivalent by the manufacturer by means of relevant engineering demonstration or other appropriate procedures.

Note: engines that do not belong to the same engine family may still belong to the same OBD-engine family provided the above-mentioned criteria are satisfied.

8.	CONFORMITY OF PRODUCTION

The conformity of production procedures shall comply with those set out in the Agreement, Appendix 2 (E/ECE/324-E/ECE/TRANS/505/Rev.2), with the following requirements:

8.1.	Every engine or vehicle bearing an approval mark as prescribed under this Regulation shall be so manufactured as to conform, with regard to the description as given in the approval form and its annexes, to the approved type.

8.2.	As a general rule, conformity of production with regard to limitation of emissions is checked based on the description given in the communication form and its annexes.

8.3.	If emissions of pollutants are to be measured and an engine approval has had one or several extensions, the tests will be carried out on the engine(s) described in the information package relating to the relevant extension.

8.3.1.	Conformity of the engine subjected to a pollutant test:

After submission of the engine to the authorities, the manufacturer shall not carry out any adjustment to the engines selected.

8.3.1.1.	Three engines are randomly taken in the series.  Engines that are subject to testing only on the ESC and ELR tests or only on the ETC test for type approval to row A of the tables in paragraph 5.2.1. are subject to those applicable tests for the checking of production conformity.  With the agreement of the authority, all other engines type approved to row A, B1 or B2, or C of the tables in paragraph 5.2.1. are subjected to testing either on the ESC and ELR cycles or on the ETC cycle for the checking of the production conformity.  The limit values are given in paragraph 5.2.1. of this Regulation.

8.3.1.2.	The tests are carried out according to Appendix 1 to this Regulation where the competent authority is satisfied with the production standard deviation given by the manufacturer.

The tests are carried out according to Appendix 2 to this Regulation, where the competent authority is not satisfied with the production standard deviation given by the manufacturer.

At the manufacturer's request, the tests may be carried out in accordance with Appendix 3 to this Regulation.

8.3.1.3.	On the basis of a test of the engine by sampling, the production of a series is regarded as conforming where a pass decision is reached for all the pollutants and non conforming where a fail decision is reached for one pollutant, in accordance with the test criteria applied in the appropriate Appendix.

When a pass decision has been reached for one pollutant, this decision may not be changed by any additional tests made in order to reach a decision for the other pollutants.

If no pass decision is reached for all the pollutants and if no fail decision is reached for one pollutant, a test is carried out on another engine (see figure 2).

If no decision is reached, the manufacturer may at any time decide to stop testing. In that case a fail decision is recorded.

8.3.2.	The tests will be carried out on newly manufactured engines.  Gas fuelled engines shall be run-in using the procedure defined in paragraph 3. of Appendix 2 to Annex 4A.

8.3.2.1.	However, at the request of the manufacturer, the tests may be carried out on diesel or gas engines which have been run-in more than the period referred to in paragraph 8.3.2., up to a maximum of 100 hours.  In this case, the running-in procedure will be conducted by the manufacturer who shall undertake not to make any adjustments to those engines.

8.3.2.2.	When the manufacturer asks to conduct a running-in procedure in accordance with paragraph 8.3.2.1., it may be carried out on:

- (a)	all the engines that are tested, or;

- (b)	the first engine tested, with the determination of an evolution coefficient as follows:

- (i)	the pollutant emissions will be measured at zero and at "x" hours on the first engine tested,

- (ii)	the evolution coefficient of the emissions between zero and "x" hours will be calculated for each pollutant:

- a.	Emissions "x" hours/Emissions zero hours

- b.	It may be less than one.

The subsequent test engines will not be subjected to the running-in procedure, but their zero hour emissions will be modified by the evolution coefficient.

In this case, the values to be taken will be:

- (a)	the values at "x" hours for the first engine;

- (b)	the values at zero hour multiplied by the evolution coefficient for the other engines.

8.3.2.3.	For diesel and LPG fuelled engines, all these tests may be conducted with commercial fuel.  However, at the manufacturer's request, the reference fuels described in Annex 5 may be used.  This implies tests, as described in paragraph 4. of this Regulation, with at least two of the reference fuels for each gas engine.

8.3.2.4.	For NG fuelled engines, all these tests may be conducted with commercial fuel in the following way:

- (a)	for H marked engines with a commercial fuel within the H-range (0.89 ≤ Sλ ≤ 1.00),

- (b)	for L marked engines with a commercial fuel within the L-range (1.00 ≤ Sλ ≤ 1.19),

- (c)	for HL marked engines with a commercial fuel within the extreme range of the λ-shift factor (0.89 ≤ Sλ ≤ 1.19).

However, at the manufacturer's request, the reference fuels described in Annex 5 may be used.  This implies tests, as described in paragraph 4. of this Regulation.

8.3.2.5.	In the case of dispute caused by the non-compliance of gas fuelled engines when using a commercial fuel, the tests shall be performed with a reference fuel on which the parent engine has been tested, or with the possible additional fuel 3 as referred to in paragraphs 4.1.3.1. and 4.2.1.1. on which the parent engine may have been tested.  Then, the result has to be converted by a calculation applying the relevant factor(s) "r", "ra" or "rb" as described in paragraphs 4.1.4., 4.1.5.1. and 4.2.1.2.  If "r", "ra" or "rb" are less than 1 no correction shall take place.  The measured results and the calculated results shall demonstrate that the engine meets the limit values with all relevant fuels (fuels 1, 2 and, if applicable, fuel 3 in the case of natural gas engines and fuels A and B in the case of LPG engines).

8.3.2.6.	Tests for conformity of production of a gas fuelled engine laid out for operation on one specific fuel composition shall be performed on the fuel for which the engine has been calibrated.

Figure 2

Schematic of production conformity testing

8.4.	On-board diagnostics (OBD)

8.4.1.	If a verification of the conformity of production of the OBD system is to be carried out, it shall be conducted in accordance with the following:

8.4.2.	When the approval authority determines that the quality of production seems unsatisfactory an engine is randomly taken from the series and subjected to the tests described in Appendix 1 to Annex 9A to this Regulation.  The tests may be carried out on an engine that has been run-in up to a maximum of 100 hours.

8.4.3.	The production is deemed to conform if this engine meets the requirements of the tests described in Appendix 1 to Annex 9A to this Regulation.

8.4.4	If the engine taken from the series does not satisfy the requirements of paragraph 8.4.2., a further random sample of four engines shall be taken from the series and subjected to the tests described in Appendix 1 to Annex 9A to this Regulation.  The tests may be carried out on engines that have been run-in up to a maximum of 100 hours.

8.4.5.	The production is deemed to conform if at least three engines out of the further random sample of four engines meet the requirements of the tests described in Appendix 1 to Annex 9A to this Regulation.

9.	CONFORMITY OF IN-SERVICE VEHICLES/ENGINES

9.1.	For the purpose of this Regulation, the conformity of in-service vehicles/engines shall be checked periodically over the useful life period of an engine installed in a vehicle.

9.2.	With reference to approvals granted for emissions, additional measures are appropriate for confirming the functionality of the emission control devices during the useful life of an engine installed in a vehicle under normal conditions of use.

9.3.	The procedures to be followed regarding the conformity of in-service vehicles/engines are given in Annex 8 to this Regulation.

10.	PENALTIES FOR NON-CONFORMITY OF PRODUCTION

10.1.	The approval granted in respect of an engine or vehicle type pursuant to this Regulation may be withdrawn if the requirements laid down in paragraph 8.1. are not complied with, or if the engine(s) or vehicle(s) taken fail to pass the tests prescribed in paragraph 8.3.

10.2.	If a Contracting Party to the Agreement applying this Regulation withdraws an approval it has previously granted, it shall forthwith so notify the other Contracting Parties applying this Regulation by means of a communication form conforming to the model in Annexes 2A or 2B to this Regulation.

11.	MODIFICATION AND EXTENSION OF APPROVAL OF THE APPROVED TYPE

11.1.	Every modification of the approved type shall be notified to the administrative department which approved the type.  The department may then either:

11.1.1.	Consider that the modifications made are unlikely to have an appreciable adverse effect and that in any case the modified type still complies with the requirement; or

11.1.2.	Require a further test report from the technical service conducting the tests.

11.2.	Confirmation or refusal of approval, specifying the alterations, shall be communicated by the procedure specified in paragraph 4.5. to the Contracting Parties to the Agreement applying this Regulation.

11.3.	The competent authority issuing the extension of approval shall assign a series number for such an extension and inform thereof the other Parties to the 1958 Agreement applying this Regulation by means of a communication form conforming to the model in Annexes 2A or 2B to this Regulation.

12.	PRODUCTION DEFINITELY DISCONTINUED

If the holder of the approval completely ceases to manufacture the type approved in accordance with this Regulation, he shall so inform the authority which granted the approval.  Upon receiving the relevant communication that authority shall inform thereof the other Parties to the 1958 Agreement which apply this Regulation by means of a communication form conforming to the model in annexes 2A or 2B to this Regulation.

13.	TRANSITIONAL PROVISIONS

13.1.	General

13.1.1.	As from the official date of entry into force of the 05 series of amendments, no Contracting Party applying this Regulation may refuse to grant ECE approval under this Regulation as amended by the 05 series of amendments.

13.1.2.	As from the date of entry into force of the 05 series of amendments, Contracting Parties applying this Regulation shall grant ECE approvals only if the engine meets the requirements of this Regulation as amended by the 05 series of amendments.

The engine shall be subject to the relevant tests set out in paragraph 5. and shall comply with paragraphs 13.2.1., 13.2.2. and 13.2.3.

13.2.	New type approvals

13.2.1.	Notwithstanding the provisions of paragraph 13.4. and 13.5., Contracting Parties applying this Regulation shall, from the date of entry into force of the 05 series of amendments to this Regulation, grant an ECE approval to an engine only if that engine satisfies:

- (a)	the relevant emission limits of Rows B1, B2 or C in the tables to paragraph 5.2.1. of this Regulation;

- (b)	the durability requirements set out in paragraph 5.3.;

- (c)	the OBD requirements set out in paragraph 5.4.;

- (d)	the additional provisions set out in paragraph 5.5.

| Character | Date New types - all types | Row (a) | OBD Stage I (b) | OBD Stage II | Durability and in-use | NOx control (c) |
| --- | --- | --- | --- | --- | --- | --- |
| B | 01/10/05 01/10/06 | B1(2005) | YES | - | YES | - |
| C | 09/11/06 01/10/07 | B1(2005) | YES | - | YES | YES |
| D |  | B2(2008) | YES | - | YES | - |
| E |  | B2(2008) | YES | - | YES | YES |
| F |  | B2(2008) | - | YES | YES | - |
| G |  | B2(2008) | - | YES | YES | YES |
| H |  | C | YES | - | YES | - |
| I |  | C | YES | - | YES | YES |
| J |  | C | - | YES | YES | - |
| K |  | C | - | YES | YES | YES |

(a)	In accordance with tables of paragraph 5.2.1. of this Regulation.

(b)	In accordance with paragraph 5.4. of this Regulation, gas engines are excluded from OBD stage I.

(c)	In accordance with paragraph 5.5. of this Regulation.

13.2.2.	Notwithstanding the provisions of paragraph 13.4. and 13.5., Contracting Parties applying this Regulation shall, from 9 November 2006, grant an ECE approval to an engine only if that engine satisfies all conditions set out in paragraph 13.2.1. and the additional provisions set out in paragraph 5.5. of this Regulation.

13.2.3.	Notwithstanding the provisions of paragraph 13.4.1. and 13.5. Contracting Parties applying this Regulation shall, from 1 October 2008, grant an ECE approval to an engine only if that engine satisfies:

- (a)	the relevant emission limits of rows B2 or C in the tables to paragraph 5.2.1.

- (b)	the durability requirements set out in paragraph 5.3.

- (c)	the OBD requirements set out in paragraph 5.4. (OBD stage 2)

- (d)	the additional provisions set out in paragraph 5.5.

13.3.	Limit of validity of old type approvals

13.3.1.	As from the official date of entry into force of the 05 series of amendments, type approvals granted to this Regulation as amended by the 04 series of amendments shall cease to be valid.

13.3.2.	As from 01 October 2007, type approvals granted to this Regulation as amended by the 05 series of amendments, which do not comply with the requirement of paragraph 13.2.2., shall cease to be valid.

13.3.3.	As from 01 October 2009, type approvals granted to this Regulation as amended by the 05 series of amendments, which do not comply with the requirements of paragraph 13.2.3., shall cease to be valid.

13.4.	Gas engines

13.4.1.	Gas engines do not need to comply with provisions set out in paragraphs 5.5.

13.4.2	Gas engines do not need to comply with the provisions set out in paragraph 5.4.1. (OBD stage 1).

13.5.	Replacement engines for vehicles in use

13.5.1.	Contracting Parties applying this Regulation may continue to grant approvals to those engines which comply with the requirements of this Regulation as amended by any previous series of amendments, or to any level of the Regulation as amended by the 05 series of amendments, provided that the engine is intended as a replacement for a vehicle in-use and for which that earlier standard was applicable at the date of that vehicle's entry into service.

14.	NAMES AND ADDRESSES OF TECHNICAL SERVICES RESPONSIBLE FOR CONDUCTING APPROVAL TESTS AND OF ADMINISTRATIVE DEPARTMENTS

The Parties to the 1958 Agreement applying this Regulation shall communicate to the United Nations secretariat the names and addresses of the technical services responsible for conducting approval tests and the administrative departments which grant approval and to which forms certifying approval or extension or refusal or withdrawal of approval, issued in other countries, are to be sent.

Appendix 1

PROCEDURE FOR PRODUCTION CONFORMITY TESTING

WHEN STANDARD DEVIATION IS SATISFACTORY

1.	This appendix describes the procedure to be used to verify production conformity for the emissions of pollutants when the manufacturer's production standard deviation is satisfactory.

2.	With a minimum sample size of three engines the sampling procedure is set so that the probability of a lot passing a test with 40 per cent of the engines defective is 0.95 (producer's risk = 5 per cent) while the probability of a lot being accepted with 65 per cent of the engines defective is 0.10 (consumer's risk = 10 per cent).

3.	The following procedure is used for each of the pollutants given in paragraph 5.2.1. of this Regulation (see figure 2):

Let:

| L | = | the natural logarithm of the limit value for the pollutant; |
| --- | --- | --- |
| xi | = | the natural logarithm of the measurement (after having applied the relevant DF) for the i-th engine of the sample; |
| s | = | an estimate of the production standard deviation (after taking the natural logarithm of the measurements); |
| n | = | the current sample number. |

4.	For each sample the sum of the standardized deviations to the limit is calculated using the following formula:

5.	Then:

- (a)	if the test statistic result is greater than the pass decision number for the sample size given in table 3, a pass decision is reached for the pollutant;

- (b)	if the test statistic result is less than the fail decision number for the sample size given in table 3, a fail decision is reached for the pollutant;

- (c)	otherwise, an additional engine is tested according to paragraph 8.3.1. and the calculation procedure is applied to the sample increased by one more unit.

Table 3: Pass and Fail Decision Numbers of Appendix 1 Sampling Plan

Minimum sample size: 3

| Cumulative number of engines tested (sample size) | Pass decision number An | Fail decision number Bn |
| --- | --- | --- |
| 3 | 3.327 | − 4.724 |
| 4 | 3.261 | − 4.790 |
| 5 | 3.195 | − 4.856 |
| 6 | 3.129 | − 4.922 |
| 7 | 3.063 | − 4.988 |
| 8 | 2.997 | − 5.054 |
| 9 | 2.931 | − 5.120 |
| 10 | 2.865 | − 5.185 |
| 11 | 2.799 | − 5.251 |
| 12 | 2.733 | − 5.317 |
| 13 | 2.667 | − 5.383 |
| 14 | 2.601 | − 5.449 |
| 15 | 2.535 | − 5.515 |
| 16 | 2.469 | − 5.581 |
| 17 | 2.403 | − 5.647 |
| 18 | 2.337 | − 5.713 |
| 19 | 2.271 | − 5.779 |
| 20 | 2.205 | − 5.845 |
| 21 | 2.139 | − 5.911 |
| 22 | 2.073 | − 5.977 |
| 23 | 2.007 | − 6.043 |
| 24 | 1.941 | − 6.109 |
| 25 | 1.875 | − 6.175 |
| 26 | 1.809 | − 6.241 |
| 27 | 1.743 | − 6.307 |
| 28 | 1.677 | − 6.373 |
| 29 | 1.611 | − 6.439 |
| 30 | 1.545 | − 6.505 |
| 31 | 1.479 | − 6.571 |
| 32 | − 2.112 | − 2.112 |

Appendix 2

PROCEDURE FOR PRODUCTION CONFORMITY TESTING

WHEN STANDARD DEVIATION IS UNSATISFACTORY OR UNAVAILABLE

1.	This appendix describes the procedure to be used to verify production conformity for the emissions of pollutants when the manufacturer's production standard deviation is either unsatisfactory or unavailable.

2.	With a minimum sample size of three engines the sampling procedure is set so that the probability of a lot passing a test with 40 per cent of the engines defective is 0.95 (producer's risk = 5 per cent) while the probability of a lot being accepted with 65 per cent of the engines defective is 0.10 (consumer's risk = 10 per cent).

3.	The values of the pollutants given in paragraph 5.2.1. of this Regulation, after having applied the relevant DF, are considered to be log normally distributed and should be transformed by taking their natural logarithms.  Let m0 and m denote the minimum and maximum sample size respectively (m0 = 3 and m = 32) and let n denote the current sample number.

4.	If the natural logarithms of the measured values (after having applied the relevant DF) in the series are x1, x2, … xi and L is the natural logarithm of the limit value for the pollutant, then, define:

5.	Table 4 shows values of the pass (An) and fail (Bn) decision numbers against current sample number.  The test statistic result is the ratio  and shall be used to determine whether the series has passed or failed as follows:

For m0 ≤ n ≤ m:

- (a)	pass the series if ,

- (b)	fail the series if ,

- (c)	take another measurement if .

6.	Remarks

The following recursive formulae are useful for calculating successive values of the test statistic:

Table 4: Pass and fail decision numbers of Appendix 2 sampling plan

Minimum sample size: 3

| Cumulative number of engines tested (sample size) | Pass decision number An | Fail decision number Bn |
| --- | --- | --- |
| 3 | − 0.80381 | 16.64743 |
| 4 | − 0.76339 | 7.68627 |
| 5 | − 0.72982 | 4.67136 |
| 6 | − 0.69962 | 3.25573 |
| 7 | − 0.67129 | 2.45431 |
| 8 | − 0.64406 | 1.94369 |
| 9 | − 0.61750 | 1.59105 |
| 10 | − 0.59135 | 1.33295 |
| 11 | − 0.56542 | 1.13566 |
| 12 | − 0.53960 | 0.97970 |
| 13 | − 0.51379 | 0.85307 |
| 14 | − 0.48791 | 0.74801 |
| 15 | − 0.46191 | 0.65928 |
| 16 | − 0.43573 | 0.58321 |
| 17 | − 0.40933 | 0.51718 |
| 18 | − 0.38266 | 0.45922 |
| 19 | − 0.35570 | 0.40788 |
| 20 | − 0.32840 | 0.36203 |
| 21 | − 0.30072 | 0.32078 |
| 22 | − 0.27263 | 0.28343 |
| 23 | − 0.24410 | 0.24943 |
| 24 | − 0.21509 | 0.21831 |
| 25 | − 0.18557 | 0.18970 |
| 26 | − 0.15550 | 0.16328 |
| 27 | − 0.12483 | 0.13880 |
| 28 | − 0.09354 | 0.11603 |
| 29 | − 0.06159 | 0.09480 |
| 30 | − 0.02892 | 0.07493 |
| 31 | − 0.00449 | 0.05629 |
| 32 | 0.03876 | 0.03876 |

Appendix 3

PROCEDURE FOR PRODUCTION CONFORMITY TESTING

AT MANUFACTURER'S REQUEST

1.	This appendix describes the procedure to be used to verify, at the manufacturer's request, production conformity for the emissions of pollutants.

2.	With a minimum sample size of three engines the sampling procedure is set so that the probability of a lot passing a test with 30 per cent of the engines defective is 0.90 (producer's risk = 10 per cent) while the probability of a lot being accepted with 65 per cent of the engines defective is 0.10 (consumer's risk = 10 per cent).

3.	The following procedure is used for each of the pollutants given in paragraph 5.2.1. of this Regulation (see figure 2):

Let:

| L | = | the natural logarithm of the limit value for the pollutant; |
| --- | --- | --- |
| xi | = | the natural logarithm of the measurement (after having applied the relevant DF) for the i-th engine of the sample; |
| s | = | an estimate of the production standard deviation (after taking the natural logarithm of the measurements); |
| n | = | the current sample number. |

4.	Calculate for the sample the test statistic quantifying the number of non-conforming engines, i.e. xi ≥ L

5.	Then:

- (a)	if the test statistic is less than or equal to the pass decision number for the sample size given in table 5, a pass decision is reached for the pollutant;

- (b)	if the test statistic is greater than or equal to the fail decision number for the sample size given in table 5, a fail decision is reached for the pollutant;

- (c)	otherwise, an additional engine is tested according to paragraph 8.3.1. of this Regulation and the calculation procedure is applied to the sample increased by one more unit.

In table 5 the pass and fail decision numbers are calculated by means of the International Standard ISO 8422/1991.

Table 5: Pass and fail decision numbers of Appendix 3 sampling plan

Minimum sample size: 3

| Cumulative number of engines tested (sample size) | Pass decision number | Fail decision number |
| --- | --- | --- |
| 3 | — | 3 |
| 4 | 0 | 4 |
| 5 | 0 | 4 |
| 6 | 1 | 5 |
| 7 | 1 | 5 |
| 8 | 2 | 6 |
| 9 | 2 | 6 |
| 10 | 3 | 7 |
| 11 | 3 | 7 |
| 12 | 4 | 8 |
| 13 | 4 | 8 |
| 14 | 5 | 9 |
| 15 | 5 | 9 |
| 16 | 6 | 10 |
| 17 | 6 | 10 |
| 18 | 7 | 11 |
| 19 | 8 | 9 |

# Appendix 4

DETERMINATION OF SYSTEM EQUIVALENCE

The determination of system equivalency according to paragraph 5.2. of this Regulation shall be based on a 7-sample pair (or larger) correlation study between the candidate system and one of the accepted reference systems of this Regulation using the appropriate test cycle(s).  The equivalency criteria to be applied shall be the F-test and the two-sided Student t-test.

This statistical method examines the hypothesis that the population standard deviation and mean value for an emission measured with the candidate system do not differ from the standard deviation and population mean value for that emission measured with the reference system.  The hypothesis shall be tested on the basis of a 5 per cent significance level of the F and t values.  The critical F and t values for 7- to 10-sample pairs are given in the table below.  If the F and t values calculated according to the formulae below are greater than the critical F and t values, the candidate system is not equivalent.

The following procedure shall be followed.  The subscripts R and C refer to the reference and candidate system, respectively:

(a)	Conduct at least 7 tests with the candidate and reference systems preferably operated in parallel.  The number of tests is referred to as nR and nC.

(b)	Calculate the mean values xR and xC and the standard deviations sR and sC.

(c)	Calculate the F value, as follows:

(the greater of the two standard deviations SR or SC shall be in the numerator)

(d)	Calculate the t value, as follows:

(e)	Compare the calculated F and t values with the critical F and t values corresponding to the respective number of tests indicated in table below.  If larger sample sizes are selected, consult statistical tables for 5 per cent significance (95 per cent confidence) level.

(f)	Determine the degrees of freedom (df), as follows:

for the F-test:	df = nR –1 / nC –1

for the t-test:	df = nC + nR –2

F and t values for selected sample sizes:

| Sample Size | F-test |  | t-test |  |
| --- | --- | --- | --- | --- |
|  | df | Fcrit | df | tcrit |
| 7 | 6/6 | 4.284 | 12 | 2.179 |
| 8 | 7/7 | 3.787 | 14 | 2.145 |
| 9 | 8/8 | 3.438 | 16 | 2.120 |
| 10 | 9/9 | 3.179 | 18 | 2.101 |

(g)	Determine the equivalency, as follows:

- (i)	if F < Fcrit and t < tcrit, then the candidate system is equivalent to the reference system of this Regulation;

- (ii)	if F  Fcrit and t  tcrit , then the candidate system is different from the reference system of this Regulation.

Annex 1

ESSENTIAL CHARACTERISTICS OF THE (PARENT) ENGINE AND

INFORMATION CONCERNING THE CONDUCT OF TEST /

1.	Description of engine

1.1.	Manufacturer:.

1.2.	Manufacturer's engine code:

1.3.	Cycle: four stroke / two stroke /:

1.4.	Number and arrangement of cylinders:

1.4.1.	Bore:	mm

1.4.2.	Stroke:	mm

1.4.3.	Firing order:

1.5.	Engine capacity:	cm³

1.6.	Volumetric compression ratio /:

1.7.	Drawing(s) of combustion chamber and piston crown:

1.8.	Minimum cross-sectional area of inlet and outlet ports:	cm²

1.9.	Idling speed:	min-1

1.10.	Maximum net power:	kW at	min-1

1.11.	Maximum permitted engine speed:	min-1

1.12.	Maximum net torque:	Nm at	min-1

1.13.	Combustion system: compression ignition/positive ignition 2/

1.14.	Fuel: Diesel/LPG/NG-H/NG-L/NG-HL/ethanol 2/

1.15.	Cooling system

1.15.1.	Liquid

1.15.1.1.	Nature of liquid:

1.15.1.2.	Circulating pump(s): yes/no 2/

1.15.1.3.	Characteristics or make(s) and type(s) (if applicable):

1.15.1.4.	Drive ratio(s) (if applicable):

1.15.2.	Air

1.15.2.1.	Blower: yes/no 2/

1.15.2.2.	Characteristics or make(s) and type(s) (if applicable):

1.15.2.3.	Drive ratio(s) (if applicable):

1.16.	Temperature permitted by the manufacturer

1.16.1.	Liquid cooling: Maximum temperature at outlet:	K

1.16.2.	Air cooling: reference point:

Maximum temperature at reference point:	K

1.16.3.	Maximum temperature of the air at the outlet of the intake intercooler (if applicable):

1.16.4.	Maximum exhaust temperature at the point in the exhaust pipe(s) adjacent to the outer flange(s) of the exhaust manifold(s) or turbocharger(s): 
	K

1.16.5.	Fuel temperature: min.	K, max.	K

for diesel engines at injection pump inlet, for gas fuelled engines at pressure regulator final stage

1.16.6.	Fuel pressure: min.	kPa, max.	kPa

at pressure regulator final stage, NG fuelled gas engines only

1.16.7.	Lubricant temperature: min.	K, max.	K

1.17.	Pressure charger: yes/no 2/

1.17.1.	Make:

1.17.2.	Type:

1.17.3.	Description of the system (e.g. max. charge pressure, waste gate, if applicable):

1.17.4.	Intercooler: yes/no 2/

1.18.	Intake system

Maximum allowable intake depression at rated engine speed and at 100 per cent load as specified in and under the operating conditions of Regulation No. 24, 02 series of amendments:
	kPa

1.19.	Exhaust system

Maximum allowable exhaust back pressure at rated engine speed and at 100 per cent load as specified in and under the operating conditions of Regulation No. 24, 02 series of amendments.
	kPa

Exhaust system volume:	dm³

1.20.	Engine Electronic Control Unit (EECU) (all engine types):

1.20.1.	Make:

1.20.2.	Type:

1.20.3.	Software calibration number(s):

2.	Measures taken against air pollution

2.1.	Device for recycling crankcase gases (description and drawings):

2.2.	Additional anti-pollution devices
(if any, and if not covered by another heading)

2.2.1.	Catalytic converter: yes/no 2/

2.2.1.1.	Make(s):

2.2.1.2.	Type(s):

2.2.1.3.	Number of catalytic converters and elements:

2.2.1.4.	Dimensions, shape and volume of the catalytic converter(s):

2.2.1.5.	Type of catalytic action:

2.2.1.6.	Total charge of precious metals:

2.2.1.7.	Relative concentration:

2.2.1.8.	Substrate (structure and material):

2.2.1.9.	Cell density:

2.2.1.10.	Type of casing for the catalytic converter(s):

2.2.1.11.	Location of the catalytic converter(s) (place and reference distance in the exhaust line):

2.2.1.12.	Normal operating temperature range (K):

2.2.1.13.	Consumable reagents (where appropriate):

2.2.1.13.1.	Type and concentration of reagent needed for catalytic action:

2.2.1.13.2.	Normal operational temperature range of reagent:

2.2.1.13.3.	International standard (where appropriate):

2.2.1.13.4.	Frequency of reagent refill: continuous/maintenance /:

2.2.2.	Oxygen sensor: yes/no 2/

2.2.2.1.	Make(s):

2.2.2.2.	Type:

2.2.2.3.	Location:

2.2.3.	Air injection: yes/no 2/

2.2.3.1.	Type (pulse air, air pump, etc.):

2.2.4.	EGR: yes/no 2/

2.2.4.1.	Characteristics (make, type, flow etc):

2.2.5.	Particulate trap: yes/no 2/:

2.2.5.1.	Dimensions, shape and capacity of the particulate trap:

2.2.5.2.	Type and design of the particulate trap:

2.2.5.3.	Location (reference distance in the exhaust line):

2.2.5.4.	Method or system of regeneration, description and/or drawing:

2.2.5.5.	Normal operating temperature (K) and pressure (kPa) range:	"

2.2.5.6.	In case of periodic regeneration:

- (a) Number of ETC test cycles between 2 regenerations (n1):

- (b) Number of ETC test cycles during regeneration (n2):

2.2.6.	Other systems: yes/no 2/

2.2.6.1.	Description and operation:

3.	Fuel feed

3.1.	Diesel engines

3.1.1.	Feed pump

Pressure 3/:	kPa or characteristic diagram 2/:

3.1.2.	Injection system

3.1.2.1.	Pump

3.1.2.1.1.	Make(s):

3.1.2.1.2.	Type(s):

3.1.2.1.3.	Delivery:	mm³ 3/ per stroke at engine speed of	rpm at full injection, or characteristic diagram 2/ 3/

Mention the method used: On engine/on pump bench 2/

If boost control is supplied, state the characteristic fuel delivery and boost pressure versus engine speed.

3.1.2.1.4.	Injection advance

3.1.2.1.4.1.	Injection advance curve 3/:

3.1.2.1.4.2.	Static injection timing 3/:

3.1.2.2.	Injection piping

3.1.2.2.1.	Length:	mm

3.1.2.2.2.	Internal diameter:	mm

3.1.2.2.3.	Common rail, make and type:

3.1.2.3.	Injector(s)

3.1.2.3.1.	Make(s):

3.1.2.3.2.	Type(s):

3.1.2.3.3.	"Opening pressure":	kPa 3/

or characteristic diagram 2/ 3/:

3.1.2.4.	Governor

3.1.2.4.1.	Make(s):

3.1.2.4.2.	Type(s):

3.1.2.4.3.	Speed at which cut-off starts under full load:	rpm

3.1.2.4.4.	Maximum no-load speed:	rpm

3.1.2.4.5.	Idling speed:	rpm

3.1.3.	Cold start system

3.1.3.1.	Make(s):

3.1.3.2.	Type(s):

3.1.3.3.	Description:

3.1.3.4.	Auxiliary starting aid:

3.1.3.4.1.	Make:

3.1.3.4.2.	Type:

3.2.	Gas fuelled engines /

3.2.1.	Fuel: Natural gas/LPG 2/

3.2.2.	Pressure regulator(s) or vaporiser/pressure regulator(s) 3/

3.2.2.1.	Make(s):

3.2.2.2.	Type(s):

3.2.2.3.	Number of pressure reduction stages:

3.2.2.4.	Pressure in final stage: min.	kPa, max.	kPa

3.2.2.5.	Number of main adjustment points:

3.2.2.6.	Number of idle adjustment points:

3.2.2.7.	Certification number:

3.2.3.	Fuelling system: mixing unit / gas injection / liquid injection / direct injection 2/

3.2.3.1.	Mixture strength regulation:

3.2.3.2.	System description and/or diagram and drawings:

3.2.3.3.	Certification number:

3.2.4.	Mixing unit

3.2.4.1.	Number:

3.2.4.2.	Make(s):

3.2.4.3.	Type(s):

3.2.4.4.	Location:

3.2.4.5.	Adjustment possibilities:

3.2.4.6.	Certification number:

3.2.5.	Inlet manifold injection

3.2.5.1.	Injection: single point/multipoint 2/

3.2.5.2.	Injection: continuous/simultaneously timed/sequentially timed 2/

3.2.5.3.	Injection equipment

3.2.5.3.1.	Make(s):

3.2.5.3.2.	Type(s):

3.2.5.3.3.	Adjustment possibilities:

3.2.5.3.4.	Certification number:

3.2.5.4.	Supply pump (if applicable):

3.2.5.4.1.	Make(s):

3.2.5.4.2.	Type(s):

3.2.5.4.3	Certification number:

3.2.5.5.	Injector(s):

3.2.5.5.1.	Make(s):

3.2.5.5.2.	Type(s):

3.2.5.5.3.	Certification number:

3.2.6.	Direct injection

3.2.6.1.	Injection pump / pressure regulator 2/

3.2.6.1.1.	Make(s):

3.2.6.1.2.	Type(s):

3.2.6.1.3	.Injection timing:

3.2.6.1.4.	Certification number:

3.2.6.2.	Injector(s)

3.2.6.2.1.	Make(s):

3.2.6.2.2.	Type(s):

3.2.6.2.3.	Opening pressure or characteristic diagram 3/:

3.2.6.2.4.	Certification number:

3.2.7.	Electronic control unit (ECU)

3.2.7.1.	Make(s):

3.2.7.2.	Type(s):

3.2.7.3.	Adjustment possibilities:

3.2.8.	NG fuel-specific equipment

3.2.8.1.	Variant 1

(only in the case of approvals of engines for several specific fuel compositions)

3.2.8.1.1.	Fuel composition:

methane (CH4):	basis:	 %mole	min.	 %mole	max.	 %mole

ethane (C2H6):	basis:	 %mole	min.	 %mole	max.	 %mole

propane (C3H8):	basis:	 %mole	min.	 %mole	max.	 %mole

butane (C4H10):	basis:	 %mole	min.	 %mole	max.	 %mole

C5/C5+:	basis:	 %mole	min.	 %mole	max.	 %mole

oxygen (O2):	basis:	 %mole	min.	 %mole	max.	 %mole

inert (N2, He etc):	basis:	 %mole	min.	 %mole	max.	 %mole

3.2.8.1.2.	Injector(s)

3.2.8.1.2.1.	Make(s):

3.2.8.1.2.2.	Type(s):

3.2.8.1.3.	Others (if applicable)

3.2.8.2.	Variant 2

(only in the case of approvals for several specific fuel compositions)

4.	Valve timing

4.1.	Maximum lift of valves and angles of opening and closing in relation to dead centres of equivalent data:

4.2.	Reference and/or setting ranges 2/:

5.	Ignition system (spark ignition engines only)

5.1.	Ignition system type:common coil and plugs/individual coil and plugs/coil on plug/other (specify) 2/

5.2.	Ignition control unit

5.2.1.	Make(s):

5.2.2.	Type(s):

5.3.	Ignition advance curve / advance map) 2/ 3/:

5.4.	Ignition timing 3/:	degrees before TDC at a speed of	rpm
and a MAP of	Pa

5.5.	Spark plugs

5.5.1.	Make(s):

5.5.2.	Type(s):

5.5.3.	Gap setting:	mm

5.6.	Ignition coil(s)

5.6.1.	Make(s):

5.6.2.	Type(s):

6.	Engine-driven equipment

The engine shall be submitted for testing with the auxiliaries needed for operating the engine (e.g. fan, water pump etc.), as specified in and under the operating conditions of Regulation No. 24, 02 series of amendments, Annex 10, paragraph 5.1.1.

6.1.	Auxiliaries to be fitted for the test

If it is impossible or inappropriate to install the auxiliaries on the test bench, the power absorbed by them shall be determined and subtracted from the measured engine power over the whole operating area of the test cycle(s).

6.2.	Auxiliaries to be removed for the test

Auxiliaries needed only for the operation of the vehicle (e.g. air compressor, air-conditioning system etc.) shall be removed for the test.  Where the auxiliaries cannot be removed, the power absorbed by them may be determined and added to the measured engine power over the whole operating area of the test cycle(s).

7.	Additional information on test conditions

7.1.	Lubricant used

7.1.1.	Make:

7.1.2.	Type:

(State percentage of oil in mixture if lubricant and fuel are mixed):

7.2.	Engine-driven equipment (if applicable)

The power absorbed by the auxiliaries needs only be determined,

- (a) if auxiliaries needed for operating the engine, are not fitted to the engine and/or

- (b) if auxiliaries not needed for operating the engine, are fitted to the engine.

7.2.1.	Enumeration and identifying details:

7.2.2.	Power absorbed at various indicated engine speeds:

| - Equipment | - Power absorbed (kW) at various engine speeds |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Idle | - Low Speed | - High Speed | - Speed A (a) | - Speed B (a) | - Speed C (a) | - Ref. Speed (b) |
| - P(a) - Auxiliaries needed for operating the engine (to be subtracted from measured engine power) see paragraph 5.1.1. of Regulation No. 24/02, Annex 10 |  |  |  |  |  |  |  |
| - P(b) - Auxiliaries not needed for operating the engine (to be added to measured engine power) see paragraph 5.1.2. of Regulation No. 24/02, Annex 10 |  |  |  |  |  |  |  |
| (a) ESC test. (b) ETC test only. |  |  |  |  |  |  |  |

8.	Engine performance

8.1.	Engine speeds /

Low speed (nlo):	rpm

High speed (nhi):	rpm

for ESC and ELR Cycles

Idle:	rpm

Speed A:	rpm

Speed B:	rpm

Speed C:	rpm

for ETC cycle

Reference speed:	rpm

8.2.	Engine power (measured in accordance with the provisions of Regulation No. 24, 02 series of amendments) in kW

|  | - Engine Speed |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - Idle | - Speed A (a) | - Speed B (a) | - Speed C (a) | - Ref. Speed (b) |
| - P(m) - Power measured on test bed |  |  |  |  |  |
| - P(a) - Power absorbed by auxiliaries to be fitted for test (paragraph 5.1.1. of Regulation No. 24/02, Annex 10) |  |  |  |  |  |
| (a) if fitted |  |  |  |  |  |
| (b) if not fitted | - 0 | - 0 | - 0 | - 0 | - 0 |
| - P(b) - Power absorbed by auxiliaries to be removed for test (paragraph 5.1.2. of Regulation No. 24/02, Annex 10) |  |  |  |  |  |
| (a) if fitted |  |  |  |  |  |
| (b) if not fitted | - 0 | - 0 | - 0 | - 0 | - 0 |
| - P(n) - Net engine power = P(m)-P(a)+P(b) |  |  |  |  |  |
| (a) ESC test. (b) ETC test only. |  |  |  |  |  |

8.3.	Dynamometer settings (kW)

The dynamometer settings for the ESC and ELR tests and for the reference cycle of the ETC test shall be based upon the net engine power P(n) of paragraph 8.2.  It is recommended to install the engine on the test bed in the net condition.  In this case, P(m) and P(n) are identical.  If it is impossible or inappropriate to operate the engine under net conditions, the dynamometer settings shall be corrected to net conditions using the above formula.

8.3.1.	ESC and ELR Tests

The dynamometer settings shall be calculated according to the formula in Annex III, Appendix 1, paragraph 1.2.

| - Percent Load | - Engine Speed |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Idle | - Speed A | - Speed B | - Speed C |
| - 10 | - --- |  |  |  |
| - 25 | - --- |  |  |  |
| - 50 | - --- |  |  |  |
| - 75 | - --- |  |  |  |
| - 100 |  |  |  |  |

8.3.2.	ETC Test

If the engine is not tested under net conditions, the correction formula for converting the measured power or measured cycle work, as determined according to Annex 4A, Appendix 2, paragraph 2., to net power or net cycle work shall be submitted by the engine manufacturer for the whole operating area of the cycle, and approved by the Technical Service.

9.	On-board diagnostic (OBD) system

9.1.	Written description and/or drawing of the MI 4/:

9.2.	List and purpose of all components monitored by the OBD system:

9.3.	Written description (general OBD working principles) for:

9.3.1.	Diesel/gas engines

9.3.1.1.	Catalyst monitoring

9.3.1.2.	deNOx system monitoring

9.3.1.3.	Diesel particulate filter monitoring

9.3.1.4.	Electronic fuelling system monitoring

9.3.1.5.	Other components monitored by the OBD system

9.4.	Criteria for MI activation (fixed number of driving cycles or statistical method):

9.5.	List of all OBD output codes and formats used (with explanation of each):

10.	Torque limiter

10.1.	Description of the torque limiter activation

10.2.	Description of the full load curve limitation

Appendix 1

ESSENTIAL CHARACTERISTICS OF THE ENGINE FAMILY

1.	Common parameters

1.1.	Combustion cycle:

1.2.	Cooling medium:

1.3.	Number of cylinders /:

1.4.	Individual cylinder displacement:

1.5.	Method of air aspiration:

1.6.	Combustion chamber type/design:

1.7.	Valve and porting - configuration, size and number:

1.8.	Fuel system:

1.9.	Ignition system (gas engines):

1.10.	Miscellaneous features:

- (a)	charge cooling system 1/:

- (b)	exhaust gas recirculation 1/:

- (c)	water injection/emulsion 1/:

- (d)	air injection 1/:

1.11.	Exhaust aftertreatment 1/:

Proof of identical (or lowest for the parent engine) ratio: system capacity/fuel delivery per stroke, pursuant to diagram number(s):

2.	Engine family listing

2.1.	Name of diesel engine family:

2.1.1.	Specification of engines within this family:

|  |  |  |  |  | Parent Engine |
| --- | --- | --- | --- | --- | --- |
| Engine Type |  |  |  |  |  |
| No. of cylinders |  |  |  |  |  |
| Rated Speed (rpm) |  |  |  |  |  |
| Fuel flow per stroke (mm3) |  |  |  |  |  |
| Rated net power (kW) |  |  |  |  |  |
| Maximum torque speed (rpm) |  |  |  |  |  |
| Fuel delivery per stroke (mm3) |  |  |  |  |  |
| Maximum torque (Nm) |  |  |  |  |  |
| Low idle speed (rpm) |  |  |  |  |  |
| Cylinder displacement (in per cent of parent engine) |  |  |  |  | 100 |

2.2.	Name of gas engine family:

2.2.1.	Specification of engines within this family:

|  |  |  |  |  | Parent Engine |
| --- | --- | --- | --- | --- | --- |
| Engine Type |  |  |  |  |  |
| No. of cylinders |  |  |  |  |  |
| Rated Speed (rpm) |  |  |  |  |  |
| Fuel delivery per stroke (mm3) |  |  |  |  |  |
| Rated net power (kW) |  |  |  |  |  |
| Maximum torque speed (rpm) |  |  |  |  |  |
| Fuel delivery per stroke (mm3) |  |  |  |  |  |
| Maximum torque (Nm) |  |  |  |  |  |
| Low idle speed (rpm) |  |  |  |  |  |
| Cylinder displacement (in per cent of parent engine) |  |  |  |  | 100 |
| Spark timing |  |  |  |  |  |
| EGR flow |  |  |  |  |  |
| Air pump yes/no |  |  |  |  |  |
| Air pump actual flow |  |  |  |  |  |

Appendix 2

ESSENTIAL CHARACTERISTICS OF THE ENGINE TYPE WITHIN THE FAMILY /

1.	Description of engine

1.1.	Manufacturer:

1.2.	Manufacturer's engine code:

1.3.	Cycle: four stroke / two stroke /:

1.4.	Number and arrangement of cylinders:

1.4.1.	Bore:	mm

1.4.2.	Stroke:	mm

1.4.3.	Firing order:

1.5.	Engine capacity:	cm³

1.6.	Volumetric compression ratio /:

1.7.	Drawing(s) of combustion chamber and piston crown:

1.8.	Minimum cross-sectional area of inlet and outlet ports:	cm²

1.9.	Idling speed:	min-1

1.10.	Maximum net power:	kW at	min-1

1.11.	Maximum permitted engine speed:	min-1

1.12.	Maximum net torque:	Nm at	min-1

1.13.	Combustion system: compression ignition/positive ignition 2/

1.14.	Fuel: Diesel/LPG/NG-H/NG-L/NG-HL/ethanol 2/

1.15.	Cooling system

1.15.1.	Liquid

1.15.1.1.	Nature of liquid:

1.15.1.2.	Circulating pump(s): yes/no 2/

1.15.1.3.	Characteristics or make(s) and type(s) (if applicable):

1.15.1.4.	Drive ratio(s) (if applicable):

1.15.2.	Air

1.15.2.1.	Blower: yes/no 2/

1.15.2.2.	Characteristics or make(s) and type(s) (if applicable):

1.15.2.3.	Drive ratio(s) (if applicable):

1.16.	Temperature permitted by the manufacturer

1.16.1.	Liquid cooling: Maximum temperature at outlet:	K

1.16.2.	Air cooling: reference point:

Maximum temperature at reference point:	K

1.16.3.	Maximum temperature of the air at the outlet of the intake intercooler (if applicable):
	K

1.16.4.	Maximum exhaust temperature at the point in the exhaust pipe(s) adjacent to the outer flange(s) of the exhaust manifold(s) or turbocharger(s):	K

1.16.5.	Fuel temperature: min.	K, max.	K

for diesel engines at injection pump inlet, for gas fuelled engines at pressure regulator final stage

1.16.6.	Fuel pressure: min.	kPa, max.	kPa

at pressure regulator final stage, NG fuelled gas engines only

1.16.7.	Lubricant temperature: min.	K, max.	K

1.17.	Pressure charger: yes/no 2/

1.17.1.	Make:

1.17.2.	Type:

1.17.3.	Description of the system (e.g. max. charge pressure, waste gate, if applicable):

1.17.4.	Intercooler: yes/no 2/

1.18.	Intake system

Maximum allowable intake depression at rated engine speed and at 100 per cent load as specified in and under the operating conditions of Regulation No. 24, 02 series of amendments:	kPa

1.19.	Exhaust system

Maximum allowable exhaust back pressure at rated engine speed and at 100 per cent load as specified in and under the operating conditions of Regulation No. 24, 02 series of amendments:	kPa

Exhaust system volume:	dm³

1.20.	Engine Electronic Control Unit (EECU) (all engine types):

1.20.1.	Make:

1.20.2.	Type:

1.20.3.	Software calibration number(s):

2.	Measures taken against air pollution

2.1.	Device for recycling crankcase gases (description and drawings):

2.2.	Additional anti-pollution devices (if any, and if not covered by another heading)

2.2.1.	Catalytic converter: yes/no 2/

2.2.1.1.	Make(s):

2.2.1.2.	Type(s):

2.2.1.3.	Number of catalytic converters and elements:

2.2.1.4.	Dimensions, shape and volume of the catalytic converter(s):

2.2.1.5.	Type of catalytic action:

2.2.1.6.	Total charge of precious metals:

2.2.1.7.	Relative concentration:

2.2.1.8.	Substrate (structure and material):

2.2.1.9.	Cell density:

2.2.1.10.	Type of casing for the catalytic converter(s):

2.2.1.11.	Location of the catalytic converter(s) (place and reference distance in the exhaust line):

2.2.1.12.	Normal operating temperature range (K):

2.2.1.13.	Consumable reagents (where appropriate):

2.2.1.13.1.	Type and concentration of reagent needed for catalytic action:

2.2.1.13.2.	Normal operational temperature range of reagent:

2.2.1.13.3.	International standard (where appropriate):

2.2.1.13.4.	Frequency of reagent refill: continuous/maintenance /:

2.2.2.	Oxygen sensor: yes/no 2/

2.2.2.1.	Make(s):

2.2.2.2.	Type:

2.2.2.3.	Location:

2.2.3.	Air injection: yes/no 2/

2.2.3.1.	Type (pulse air, air pump, etc.):

2.2.4.	EGR: yes/no 2/

2.2.4.1.	Characteristics (make, type, flow etc):

2.2.5.	Particulate trap: yes/no 2/:

2.2.5.1.	Dimensions, shape and capacity of the particulate trap:

2.2.5.2.	Type and design of the particulate trap:

2.2.5.3.	Location (reference distance in the exhaust line):

2.2.5.4.	Method or system of regeneration, description and/or drawing:

2.2.5.5.	Normal operating temperature (K) and pressure (kPa) range:

2.2.5.6.	In case of periodic regeneration:

- (a)	Number of ETC test cycles between 2 regenerations (n1)

- (b)	Number of ETC test cycles during regeneration (n2)

2.2.6.	Other systems: yes/no 2/

2.2.6.1.	Description and operation:

3.	Fuel feed

3.1.	Diesel engines

3.1.1.	Feed pump

Pressure 3/:	kPa or characteristic diagram 2/:

3.1.2.	Injection system

3.1.2.1.	Pump

3.1.2.1.1.	Make(s):

3.1.2.1.2.	Type(s):

3.1.2.1.3.	Delivery:	mm³ 3/ per stroke at engine speed of	rpm at full injection, or characteristic diagram 2/ 3/:

Mention the method used: On engine/on pump bench 2/

If boost control is supplied, state the characteristic fuel delivery and boost pressure versus engine speed.

3.1.2.1.4.	Injection advance

3.1.2.1.4.1.	Injection advance curve 3/ :

3.1.2.1.4.2.	Static injection timing 3/:

3.1.2.2.	Injection piping

3.1.2.2.1.	Length:	mm

3.1.2.2.2.	Internal diameter:	mm

3.1.2.2.3.	Common rail, make and type:

3.1.2.3.	Injector(s)

3.1.2.3.1.	Make(s):

3.1.2.3.2.	Type(s):

3.1.2.3.3.	"Opening pressure":	kPa 3/ or characteristic diagram 2/ 3/:

3.1.2.4.	Governor

3.1.2.4.1.	Make(s):

3.1.2.4.2.	Type(s):

3.1.2.4.3.	Speed at which cut-off starts under full load:	rpm

3.1.2.4.4.	Maximum no-load speed:	rpm

3.1.2.4.5.	Idling speed:	rpm

3.1.3.	Cold start system

3.1.3.1.	Make(s):

3.1.3.2.	Type(s):

3.1.3.3.	Description:

3.1.3.4.	Auxiliary starting aid:

3.1.3.4.1.	Make:

3.1.3.4.2.	Type:

3.2.	Gas fuelled engines /

3.2.1.	Fuel: Natural gas/LPG  2/

3.2.2.	Pressure regulator(s) or vaporiser/pressure regulator(s) 3/

3.2.2.1.	Make(s):

3.2.2.2.	Type(s):

3.2.2.3.	Number of pressure reduction stages:

3.2.2.4.	Pressure in final stage: min.	kPa, max.	kPa

3.2.2.5.	Number of main adjustment points:

3.2.2.6.	Number of idle adjustment points:

3.2.2.7.	Certification number:

3.2.3.	Fuelling system: mixing unit / gas injection / liquid injection / direct injection 2/

3.2.3.1.	Mixture strength regulation:

3.2.3.2.	System description and/or diagram and drawings:

3.2.3.3.	Certification number:

3.2.4.	Mixing unit

3.2.4.1.	Number:

3.2.4.2.	Make(s):

3.2.4.3.	Type(s):

3.2.4.4.	Location:

3.2.4.5.	Adjustment possibilities:

3.2.4.6.	Certification number:

3.2.5.	Inlet manifold injection

3.2.5.1.	Injection: single point/multipoint 2/

3.2.5.2.	Injection: continuous/simultaneously timed/sequentially timed 2/

3.2.5.3.	Injection equipment

3.2.5.3.1.	Make(s):

3.2.5.3.2.	Type(s):

3.2.5.3.3.	Adjustment possibilities:

3.2.5.3.4.	Certification number:

3.2.5.4.	Supply pump (if applicable):

3.2.5.4.1.	Make(s):

3.2.5.4.2.	Type(s):

3.2.5.4.3.	Certification number:

3.2.5.5.	Injector(s):

3.2.5.5.1.	Make(s):

3.2.5.5.2.	Type(s):

3.2.5.5.3.	Certification number:

3.2.6.	Direct injection

3.2.6.1.	Injection pump / pressure regulator 2/

3.2.6.1.1.	Make(s):

3.2.6.1.2.	Type(s):

3.2.6.1.3.	Injection timing:

3.2.6.1.4.	Certification number:

3.2.6.2.	Injector(s)

3.2.6.2.1.	Make(s):

3.2.6.2.2.	Type(s):

3.2.6.2.3.	Opening pressure or characteristic diagram 3/:

3.2.6.2.4.	Certification number:

3.2.7.	Electronic control unit (ECU)

3.2.7.1.	Make(s):

3.2.7.2.	Type(s):

3.2.7.3.	Adjustment possibilities:

3.2.8.	NG fuel-specific equipment

3.2.8.1.	Variant 1

(only in the case of approvals of engines for several specific fuel compositions)

3.2.8.1.1.	Fuel composition:

methane (CH4):	basis:	 %mole	min.	 %mole	max.	 %mole

ethane (C2H6):	basis:	 %mole	min.	 %mole	max.	 %mole

propane (C3H8):	basis:	 %mole	min.	 %mole	max.	 %mole

butane (C4H10):	basis:	 %mole	min.	 %mole	max.	 %mole

C5/C5+:	basis:	 %mole	min.	 %mole	max.	 %mole

oxygen (O2):	basis:	 %mole	min.	 %mole	max.	 %mole

inert (N2, He etc):	basis:	 %mole	min.	 %mole	max.	 %mole

3.2.8.1.2.	Injector(s)

3.2.8.1.2.1.	Make(s):

3.2.8.1.2.2.	Type(s):

3.2.8.1.3.	Others (if applicable)

3.2.8.2.	Variant 2

(only in the case of approvals for several specific fuel compositions)

4.	Valve timing

4.1.	Maximum lift of valves and angles of opening and closing in relation to dead centres of equivalent data:

4.2.	Reference and/or setting ranges 2/:

5.	Ignition system (spark ignition engines only)

5.1.	Ignition system type: common coil and plugs/individual coil and plugs/coil on plug/other (specify) 2/

5.2.	Ignition control unit

5.2.1.	Make(s):

5.2.2.	Type(s):

5.3.	Ignition advance curve / advance map 2/ 3/:

5.4.	Ignition timing 3/:	degrees before TDC at a speed of	rpm

and a MAP of	kPa

5.5.	Spark plugs

5.5.1.	Make(s):

5.5.2.	Type(s):

5.5.3.	Gap setting:	mm

5.6.	Ignition coil(s)

5.6.1.	Make(s):

5.6.2.	Type(s):

6.	On-board diagnostic (OBD) system

6.1.	Written description and/or drawing of the MI 4/:

6.2.	List and purpose of all components monitored by the OBD system:

6.3.	Written description (general OBD working principles) for:

6.3.1.	Diesel/gas engines Error: Reference source not found:

6.3.1.1.	Catalyst monitoring Error: Reference source not found:

6.3.1.2.	deNOx system monitoring Error: Reference source not found:

6.3.1.3.	Diesel particulate filter monitoring Error: Reference source not found:

6.3.1.4.	Electronic fuelling system monitoring Error: Reference source not found:

6.3.1.5.	Other components monitored by the OBD system Error: Reference source not found:

6.4.	Criteria for MI activation (fixed number of driving cycles or statistical method):

6.5.	List of all OBD output codes and formats used (with explanation of each):

7.	Torque limiter

7.1.	Description of the torque limiter activation

7.2.	Description of the full load curve limitation

Appendix 3

CHARACTERISTICS OF THE ENGINE-RELATED VEHICLE PARTS

1.	Intake system depression at rated engine speed and at 100 per cent load:	kPa

2.	Exhaust system back pressure at rated engine speed and at 100 per cent load:	kPa

3.	Volume of exhaust system:	cm³

4.	Power absorbed by the auxiliaries needed for operating the engine as specified in and under the operation conditions of Regulation No. 24, 02 series of amendments, Annex10, paragraph 5.1.1.

| Equipment | Power absorbed (kW) at various engine speeds |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Idle | Low Speed | High Speed | Speed A (a) | Speed B (a) | Speed C (a) | Ref. Speed (b) |
| P(a) Auxiliaries needed for operating the engine (to be subtracted from measured engine power) See paragraph 5.1.1. of Regulation No. 24/02, Annex 10. |  |  |  |  |  |  |  |
| (a) ESC test. (b) ETC test only. |  |  |  |  |  |  |  |

Appendix 4

OBD-RELATED INFORMATION

1.	In accordance with the provisions of paragraph 5. of Annex 9A to this Regulation, the following additional information shall be provided by the vehicle manufacturer for the purposes of enabling the manufacture of OBD-compatible replacement or service parts and diagnostic tools and test equipment, unless such information is covered by intellectual property rights or constitutes specific know-how of the manufacturer or the OEM supplier(s).  The information given in this paragraph shall be repeated in Annex 2A to this Regulation:

1.1.	A description of the type and number of the pre-conditioning cycles used for the original type approval of the vehicle.

1.2.	A description of the type of the OBD demonstration cycle used for the original approval of the vehicle for the component monitored by the OBD system.

1.3.	A comprehensive document describing all sensed components with the strategy for fault detection and MI activation (fixed number of driving cycles or statistical method), including a list of relevant secondary sensed parameters for each component monitored by the OBD system.  A list of all OBD output codes and format used (with an explanation of each) associated with individual emission related powertrain components and individual non-emission related components, where monitoring of the component is used to determine MI activation.

1.3.1.	The information required by this paragraph may, for example, be defined by completing a table as follows, which shall be attached to this annex:

| Component | Fault code | Monitoring strategy | Fault detection criteria | MI activation criteria | Secondary parameters | Precondi-tioning | Demon-stration test |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SCR catalyst | Pxxxx | NOx sensor 1 and 2 signals | Difference between sensor 1 and sensor 2 signals | 3rd cycle | Engine speed, engine load, catalyst temperature, reagent activity | Three OBD test cycles (3 short ESC cycles) | OBD test cycle (short ESC cycle) |

1.3.2.	The information required by this appendix may be limited to the complete list of the fault codes recorded by the OBD system where paragraph 5.1.2.1. of Annex 9A to this Regulation is not applicable as in the case of replacement or service components.  This information may, for example, be defined by completing the two first columns of the table of paragraph 1.3.1. above.

The complete information package should be made available to the approval authority as part of the additional material requested in paragraph 5.1.7.1. "documentation requirements" of this Regulation.

1.3.3.	The information required by this paragraph shall be repeated in Annex 2A to this Regulation.

Where paragraph 5.1.2.1. of Annex 9A to this Regulation is not applicable in the case of replacement or service components, the information provided in Annex 2A can be limited to the one mentioned in paragraph 1.3.2.

Annex 2A

COMMUNICATION

(maximum format:  A4 (210 x 297 mm))

issued by:	Name of administration:

.......................

.......................

.......................

concerning: 2/		APPROVAL GRANTED

APPROVAL EXTENDED

APPROVAL REFUSED

APPROVAL WITHDRAWN

PRODUCTION DEFINITELY DISCONTINUED

of a compression-ignition (C.I.) engine type (Diesel or Ethanol), or a positive-ignition (P.I.) engine type (NG or LPG), 2/ as a separate technical unit with regard to the emission of pollutants pursuant to Regulation No. 49, 05 series of amendments

Approval No. .....					Extension No. .....

1.	Trade name or mark of the engine:

2.	Engine type / Engine family:

2.1	Manufacturer´s code as marked on the engine:

3.	Combustion type:  compression-ignition/positive-ignition 2/

3.1.	Type of fuel:

4.	Manufacturer's name and address:

5.	If applicable, name and address of manufacturer's representative:

6.	Maximum allowable intake depression:	kPa

7.	Maximum allowable back-pressure:	kPa

8.	Maximum permissible power absorbed by the engine-driven equipment:

Idle:	kW; Low Speed:	kW; High Speed:	kW

Speed A:	kW; Speed B:	kW; Speed C:	kW;

Reference Speed:	kW

9.	Restrictions of use (if any):

10.	Emission levels of the engine/parent engine

10.1.	Emission stage (according to table in paragraph 4.6.3.) ………

10.2.	ESC test (if applicable):

Deterioration factor (DF):	calculated/fixed 2/

Specify the DF values and the emissions on the ESC test in the table below:

| ESC test |  |  |  |  |
| --- | --- | --- | --- | --- |
| DF: | CO | THC | NOx | PT |
| Emissions | CO (g/kWh) | THC (g/kWh) | NOx (g/kWh) | PT (g/kWh) |
| Measured: |  |  |  |  |
| Calculated with DF: |  |  |  |  |

10.3.	ELR test (if applicable):

smoke value:	m-1

10.4.	ETC test:

Deterioration factor (DF):	calculated/fixed 2/

| ETC test |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DF: | CO | NMHC | CH4 | NOx | PT |
| Emissions | CO (g/kWh) | NMHC (g/kWh) 2/ | CH4 (g/kWh) 2/ | NOx (g/kWh) | PT (g/kWh) 2/ |
| Measured with regeneration: |  |  |  |  |  |
| Measured without regeneration: |  |  |  |  |  |
| Measured/weighted: |  |  |  |  |  |
| Calculated with DF: |  |  |  |  |  |

11.	Engine submitted for tests on:

12.	Technical service responsible for conducting the approval tests:

13.	Date of test report issued by that service:

14.	Number of the test report issued by that service:

15.	Site of approval mark on the engine:

16.	Place:

17.	Date:

18.	Signature:

19.	The following documents, bearing the approval number shown above, are annexed to this communication:

One copy of Annex 1 to this Regulation completed and with the drawings and diagrams referred to attached.

____________

1/	Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

2/	Strike out what does not apply.

Appendix 1

OBD-RELATED INFORMATION

As noted in Appendix 4 of Annex 1 to this Regulation, the information in this appendix is provided by the engine/vehicle manufacturer for the purposes of enabling the manufacture of OBD-compatible replacement or service parts and diagnostic tools and test equipment.  Such information need not be supplied by the engine/vehicle manufacturer if it is covered by intellectual property rights or constitutes specific know-how of the manufacturer or the OEM supplier(s).

Upon request, this appendix will be made available to any interested component, diagnostic tools or test equipment manufacturer, on a non-discriminatory basis.

In compliance with the provisions of paragraph 1.3.3. of Appendix 4 to Annex 1, the information required by this paragraph shall be identical to that provided in that Appendix.

1.	A description of the type and number of the pre-conditioning cycles used for the original type approval of the vehicle.

2.	A description of the type of the OBD demonstration cycle used for the original type approval of the vehicle for the component monitored by the OBD system.

3.	A comprehensive document describing all sensed components with the strategy for fault detection and MI activation (fixed number of driving cycles or statistical method), including a list of relevant secondary sensed parameters for each component monitored by the OBD system.  A list of all OBD output codes and format used (with an explanation of each) associated with individual emission related powertrain components and individual non-emission related components, where monitoring of the component is used to determine MI activation.

Annex 2B

COMMUNICATION

(maximum format:  A4 (210 x 297 mm))

issued by:	Name of administration:

.......................

.......................

.......................

concerning: 2/		APPROVAL GRANTED

APPROVAL EXTENDED

APPROVAL REFUSED

APPROVAL WITHDRAWN

PRODUCTION DEFINITELY DISCONTINUED

of a vehicle type with regard to the emission of pollutants by the engine pursuant to Regulation No. 49

Approval No. ...					Extension No. ...

1.	Trade name or mark of the engine:

1.1.	Manufacturer's code as marked on the engine:

2.	Vehicle type:

3.	Manufacturer's name and address:

4.	If applicable, name and address of manufacturer's representative:

5.	Maximum allowable intake depression:	kPa

6.	Maximum allowable back-pressure: 	kPa

7.	Maximum permissible power absorbed by the engine-driven equipment:

Idle:	kW; Low Speed:	kW; High Speed:	kW

Speed A:	kW; Speed B:	kW; Speed C:	kW;

Reference Speed:	kW

8.	Make and type of the engine:

9.	Emission levels of the engine/parent engine

9.1.	Emission stage (according to table in paragraph 4.6.3.)

9.2.	ESC test (if applicable):

Deterioration factor (DF):	calculated/fixed 2/

Specify the DF values and the emissions on the ESC test in the table below:

| ESC test |  |  |  |  |
| --- | --- | --- | --- | --- |
| DF: | CO | THC | NOx | PT |
| Emissions | CO (g/kWh) | THC (g/kWh) | NOx (g/kWh) | PT (g/kWh) |
| Measured: |  |  |  |  |
| Calculated with DF: |  |  |  |  |

9.3.	ELR test (if applicable):

smoke value:	m-1

9.4.	ETC test:

Deterioration factor (DF):	calculated/fixed 2/

| ETC test |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DF: | CO | NMHC | CH4 | NOx | PT |
| Emissions | CO (g/kWh) | NMHC (g/kWh) 2/ | CH4 (g/kWh) 2/ | NOx (g/kWh) | PT (g/kWh) 2/ |
| Measured with regeneration: |  |  |  |  |  |
| Measured without regeneration: |  |  |  |  |  |
| Measured/weighted: |  |  |  |  |  |
| Calculated with DF: |  |  |  |  |  |

10.	Engine submitted for tests on:

11.	Technical service responsible for conducting the approval tests:

12.	Date of test report issued by that service:

13.	Number of test report issued by that service:

14.	Site of approval mark on the vehicle/engine 2/:

15.	Place:

16.	Date:

17.	Signature:

____________

1/	Distinguishing number of the country which has granted/extended/refused/withdrawn approval (see approval provisions in the Regulation).

2/	Strike out what does not apply.

Annex 3

ARRANGEMENTS OF APPROVAL MARKS

(See table in paragraph 4.6.3. of this Regulation)

I.	APPROVAL "B" (Row B1, OBD stage 1, without NOx control).

##### Example 1

Diesel engines:

a

a

2

E

11

49 RB - 052439

a

3

a

= 8

mm min

.

##### Example 2

Natural gas (NG) engines:

The suffix after the national symbol indicates the fuel qualification determined in accordance with paragraph 4.6.3.1. of this Regulation.

HL

t

a

3

a

a

2

E

11

49 RB - 052439

a

3

a

= 8

mm min

.

The above approval marks affixed to an engine/vehicle show that the engine/vehicle type concerned has been approved in the United Kingdom (E11) pursuant to Regulation No 49 and under approval number 052439.  This approval indicates that the approval was given in accordance with the requirements of Regulation No. 49 with the 05 series of amendments incorporated and satisfying the relevant emission stages detailed in paragraph 4.6.3. of this Regulation.

II.	APPROVAL "C" (Row B1, OBD stage 1, with NOx control).

##### Example 3

Diesel engines:

a

a

2

E

11

49 RC - 052439

a

3

a

= 8

mm min

.

The above approval mark affixed to an engine/vehicle shows that the engine/vehicle type concerned has been approved in the United Kingdom (E11) pursuant to Regulation No. 49 and under approval number 052439.  This approval indicates that the approval was given in accordance with the requirements of Regulation No. 49 with the 05 series of amendments incorporated and satisfying the relevant emission stages detailed in paragraph 4.6.3. of this Regulation.

III.	APPROVAL "F" (Row B2, OBD stage 2, without NOx control).

##### Example 4

LPG engines:

a

a

2

E

11

49 RF - 052439

a

3

a

= 8

mm min

.

The above approval mark affixed to an engine/vehicle shows that the engine/vehicle type concerned has been approved in the United Kingdom (E11) pursuant to Regulation No. 49 and under approval number 052439.  This approval indicates that the approval was given in accordance with the requirements of Regulation No. 49 with the 05 series of amendments incorporated and satisfying the relevant emission stages detailed in paragraph 4.6.3. of this Regulation.

IV.	APPROVAL "G" (Row B2, OBD stage 2, with NOx control).

##### Example 5

Diesel engine:

a

a

2

E

11

49 RG - 052439

a

3

a

= 8

mm min

.

The above approval mark affixed to an engine/vehicle shows that the engine/vehicle type concerned has been approved in the United Kingdom (E11) pursuant to Regulation No. 49 and under approval number 052439.  This approval indicates that the approval was given in accordance with the requirements of Regulation No. 49 with the 05 series of amendments incorporated and satisfying the relevant emission stages detailed in paragraph 4.6.3. of this Regulation.

V.	APPROVAL "J" (Row C, OBD stage 2, without NOx control).

##### Example 6

LPG engine:

a

a

2

E

11

49 RJ - 052439

a

3

a

= 8

mm min

.

The above approval mark affixed to an engine/vehicle shows that the engine/vehicle type concerned has been approved in the United Kingdom (E11) pursuant to Regulation No. 49 and under approval number 052439.  This approval indicates that the approval was given in accordance with the requirements of Regulation No. 49 with the 05 series of amendments incorporated and satisfying the relevant emission stages detailed in paragraph 4.6.3. of this Regulation.

VI.	ENGINE/VEHICLE APPROVED TO ONE OR MORE REGULATIONS

(See paragraph 4.7. of this Regulation)

##### Example 7

49 G HL

24

03 1628

05 2439

a

2

a

E

11

a

3

a

2

a

2

a

3

a

3

The above approval mark affixed to a HL natural gas engine/vehicle shows that the engine/vehicle type concerned has been approved in the United Kingdom (E11) pursuant to Regulation No. 49 (emission stage G) and Regulation No. 24 1/.  The first two digits of the approval numbers indicate that, at the dates when the respective approvals were given, Regulation No. 49 included the 05 series of amendments, and Regulation No. 24 the 03 series of amendments.

__________

1/	The second Regulation number is given merely as an example.

Annex 4A

TEST PROCEDURE

1.	INTRODUCTION

1.1.	This annex describes the methods of determining emissions of gaseous components, particulates and smoke from the engines to be tested.  Three test cycles are described that shall be applied according to the provisions of paragraph 5.2.:

- (a)	the ESC which consists of a steady state 13-mode cycle,

- (b)	the ELR which consists of transient load steps at different speeds, which are integral parts of one test procedure, and are run concurrently,

- (c)	the ETC which consists of a second-by-second sequence of transient modes.

1.2.	The test shall be carried out with the engine mounted on a test bench and connected to a dynamometer.

1.3.	Measurement principle

The emissions to be measured from the exhaust of the engine include the gaseous components (carbon monoxide, total hydrocarbons for diesel engines on the ESC test only; non-methane hydrocarbons for diesel and gas engines on the ETC test only; methane for gas engines on the ETC test only and oxides of nitrogen), the particulates (diesel engines only) and smoke (diesel engines on the ELR test only).  Additionally, carbon dioxide is often used as a tracer gas for determining the dilution ratio of partial and full flow dilution systems.  Good engineering practice recommends the general measurement of carbon dioxide as an excellent tool for the detection of measurement problems during the test run.

1.3.1.	ESC Test

During a prescribed sequence of warmed-up engine operating conditions the amounts of the above exhaust emissions shall be examined continuously by taking a sample from the raw or diluted exhaust gas.  The test cycle consists of a number of speed and power modes which cover the typical operating range of diesel engines.  During each mode the concentration of each gaseous pollutant, exhaust flow and power output shall be determined, and the measured values weighted.  For particulate measurement, the exhaust gas shall be diluted with conditioned ambient air using either a partial flow or full flow dilution system.  The particulates shall be collected on a single suitable filter in proportion to the weighting factors of each mode.  The grams of each pollutant emitted per kilowatt hour shall be calculated as described in Appendix 1 to this annex.  Additionally, NOx shall be measured at three test points within the control area selected by the Technical Service and the measured values compared to the values calculated from those modes of the test cycle enveloping the selected test points.  The NOx control check ensures the effectiveness of the emission control of the engine within the typical engine operating range.

1.3.2.	ELR Test

During a prescribed load response test, the smoke of a warmed-up engine shall be determined by means of an opacimeter.  The test consists of loading the engine at constant speed from 10 per cent to 100 per cent load at three different engine speeds.  Additionally, a fourth load step selected by the Technical Service / shall be run, and the value compared to the values of the previous load steps.  The smoke peak shall be determined using an averaging algorithm, as described in Appendix 1 to this annex.

1.3.3.	ETC Test

During a prescribed transient cycle of warmed-up engine operating conditions, which is based closely on road-type-specific driving patterns of heavy-duty engines installed in trucks and buses, the above pollutants shall be examined either after diluting the total exhaust gas with conditioned ambient air (CVS system with double dilution for particulates) or by determining the gaseous components in the raw exhaust gas and the particulates with a partial flow dilution system.  Using the engine torque and speed feedback signals of the engine dynamometer, the power shall be integrated with respect to time of the cycle resulting in the work produced by the engine over the cycle.  For a CVS system, the concentration of NOx and HC shall be determined over the cycle by integration of the analyser signal, whereas the concentration of CO, CO2, and NMHC may be determined by integration of the analyser signal or by bag sampling.  If measured in the raw exhaust gas, all gaseous components shall be determined over the cycle by integration of the analyser signal.  For particulates, a proportional sample shall be collected on a suitable filter.  The raw or diluted exhaust gas flow rate shall be determined over the cycle to calculate the mass emission values of the pollutants.  The mass emission values shall be related to the engine work to get the grams of each pollutant emitted per kilowatt hour, as described in Appendix 2 to this annex.

2.	TEST CONDITIONS

2.1.	Engine test conditions

2.1.1.	The absolute temperature (Ta) of the engine air at the inlet to the engine expressed in Kelvin, and the dry atmospheric pressure (ps), expressed in kPa shall be measured and the parameter fa shall be determined according to the following provisions.  In multi-cylinder engines having distinct groups of intake manifolds, for example, in a "V" engine configuration, the average temperature of the distinct groups shall be taken.

(a)	for compression-ignition engines:

Naturally aspirated and mechanically supercharged engines:

Turbocharged engines with or without cooling of the intake air:

(b)	for spark-ignition engines:

2.1.2.	Test validity

For a test to be recognized as valid, the parameter fa shall be such that:

0.96  fa  1.06

2.2.	Engines with charge air cooling

The charge air temperature shall be recorded and shall be, at the speed of the declared maximum power and full load, within ± 5 K of the maximum charge air temperature specified in Annex 1, Appendix 1, paragraph 1.16.3.  The temperature of the cooling medium shall be at least 293 K (20 °C).

If a test shop system or external blower is used, the charge air temperature shall be within ± 5 K of the maximum charge air temperature specified in Annex 1, Appendix 1, paragraph 1.16.3. at the speed of the declared maximum power and full load.  The setting of the charge air cooler for meeting the above conditions shall be used for the whole test cycle.

2.3.	Engine air intake system

An engine air intake system shall be used presenting an air intake restriction within ± 100 Pa of the upper limit of the engine operating at the speed at the declared maximum power and full load.

2.4.	Engine exhaust system

An exhaust system shall be used presenting an exhaust back pressure within ± 1,000 Pa of the upper limit of the engine operating at the speed of declared maximum power and full load and a volume within ± 40 per cent of that specified by the manufacturer.  A test shop system may be used, provided it represents actual engine operating conditions.  The exhaust system shall conform to the requirements for exhaust gas sampling, as set out in paragraph 3.4. of Appendix 4 to this annex, and in Appendix 7, paragraph 2.2.1., EP and paragraph 2.3.1., EP.

If the engine is equipped with an exhaust aftertreatment device, the exhaust pipe shall have the same diameter as found in-use for at least 4 pipe diameters upstream to the inlet of the beginning of the expansion section containing the aftertreatment device.  The distance from the exhaust manifold flange or turbocharger outlet to the exhaust aftertreatment device shall be the same as in the vehicle configuration or within the distance specifications of the manufacturer.  The exhaust backpressure or restriction shall follow the same criteria as above, and may be set with a valve.  The aftertreatment container may be removed during dummy tests and during engine mapping, and replaced with an equivalent container having an inactive catalyst support.

2.5.	Cooling system

An engine cooling system with sufficient capacity to maintain the engine at normal operating temperatures prescribed by the manufacturer shall be used.

2.6.	Lubricating oil

Specifications of the lubricating oil used for the test shall be recorded and presented with the results of the test, as specified in Annex 1, paragraph 7.1.

2.7.	Fuel

The fuel shall be the reference fuel specified in Annex 5.

The fuel temperature and measuring point shall be specified by the manufacturer within the limits given in Annex 1, paragraph 1.16.5. The fuel temperature shall not be lower than 306 K (33 °C).  If not specified, it shall be 311 K ± 5 K (38 °C ± 5 °C) at the inlet to the fuel supply.

For NG and LPG fuelled engines, the fuel temperature and measuring point shall be within the limits given in Annex 1, paragraph 1.16.5. or in Annex 1, Appendix 3, paragraph 1.16.5. in cases where the engine is not a parent engine.

2.8	If the engine is equipped with an exhaust aftertreatment system, the emissions measured on the test cycle shall be representative of the emissions in the field.  In the case of an engine equipped with a exhaust aftertreatment system that requires the consumption of a reagent, the reagent used for all tests shall comply with Annex 1, paragraph 2.2.1.13.

2.8.1.	For an exhaust aftertreatment system based on a continuous regeneration process the emissions shall be measured on a stabilised aftertreatment system.

The regeneration process shall occur at least once during the ETC test and the manufacturer shall declare the normal conditions under which regeneration occurs (soot load, temperature, exhaust back-pressure, etc).

In order to verify the regeneration process at least 5 ETC tests shall be conducted.  During the tests the exhaust temperature and pressure shall be recorded (temperature before and after the aftertreatment system, exhaust back pressure, etc).

The aftertreatment system is considered to be satisfactory if the conditions declared by the manufacturer occur during the test during a sufficient time.

The final test result shall be the arithmetic mean of the different ETC test results.

If the exhaust aftertreatment has a security mode that shifts to a periodic regeneration mode it should be checked following paragraph 2.8.2. of this annex.  For that specific case the emission limits in table 2 of paragraph 5.2. could be exceeded and would not be weighted.

2.8.2.	For an exhaust aftertreatment based on a periodic regeneration process, the emissions shall be measured on at least two ETC tests, one during and one outside a regeneration event on a stabilised aftertreatment system, and the results be weighted.

The regeneration process shall occur at least once during the ETC test.  The engine may be equipped with a switch capable of preventing or permitting the regeneration process provided this operation has no effect on the original engine calibration.

The manufacturer shall declare the normal parameter conditions under which the regeneration process occurs (soot load, temperature, exhaust back-pressure etc) and its duration time (n2).  The manufacturer shall also provide all the data to determine the time between two regenerations (n1).  The exact procedure to determine this time shall be agreed by the Technical Service based upon good engineering judgement.

The manufacturer shall provide an aftertreatment system that has been loaded in order to achieve regeneration during an ETC test.  Regeneration shall not occur during this engine conditioning phase.

Average emissions between regeneration phases shall be determined from the arithmetic mean of several approximately equidistant ETC tests.  It is recommended to run at least one ETC as close as possible prior to a regeneration test and one ETC immediately after a regeneration test. As an alternative, the manufacturer may provide data to show that the emissions remain constant (15 per cent) between regeneration phases.  In this case, the emissions of only one ETC test may be used.

During the regeneration test, all the data needed to detect regeneration shall be recorded (CO or NOx emissions, temperature before and after the aftertreatment system, exhaust back pressure etc).

During the regeneration process, the emission limits in table 2 of paragraph 5.2. can be exceeded.

The measured emissions shall be weighted according to paragraphs 5.5. and 6.3. of Appendix 2 to this annex and the final result shall not exceed the limits in table 2 of paragraph 5.2.

Appendix 1

ESC AND ELR TEST CYCLES

1.	ENGINE AND DYNAMOMETER SETTINGS

1.1	Determination of engine speeds A, B and C

The engine speeds A, B and C shall be declared by the manufacturer in accordance with the following provisions:

The high speed nhi shall be determined by calculating 70 per cent of the declared maximum net power P(n), as determined in Annex 1, paragraph 8.2.  The highest engine speed where this power value occurs on the power curve is defined as nhi.

The low speed nlo shall be determined by calculating 50 per cent of the declared maximum net power P(n), as determined in Annex 1, paragraph 8.2.  The lowest engine speed where this power value occurs on the power curve is defined as nlo.

The engine speeds A, B and C shall be calculated as follows:

Speed A = nlo + 25 per cent (nhi − nlo)

Speed B = nlo + 50 per cent (nhi − nlo)

Speed C = nlo + 75 per cent (nhi − nlo)

The engine speeds A, B and C may be verified by either of the following methods:

(a)	Additional test points shall be measured during engine power approval according to Regulation 85 for an accurate determination of nhi and nlo.  The maximum power, nhi and nlo shall be determined from the power curve, and engine speeds A, B and C shall be calculated according to the above provisions.

(b)	The engine shall be mapped along the full load curve, from maximum no load speed to idle speed, using at least 5 measurement points per 1,000 rpm intervals and measurement points within ± 50 rpm of the speed at declared maximum power.  The maximum power, nhi and nlo shall be determined from this mapping curve, and engine speeds A, B and C shall be calculated according to the above provisions.

If the measured engine speeds A, B and C are within ± 3 per cent of the engine speeds as declared by the manufacturer, the declared engine speeds shall be used for the emissions test. If the tolerance is exceeded for any of the engine speeds, the measured engine speeds shall be used for the emissions test.

1.2.	Determination of dynamometer settings

The torque curve at full load shall be determined by experimentation to calculate the torque values for the specified test modes under net conditions, as specified in Annex 1, paragraph 8.2.  The power absorbed by engine-driven equipment, if applicable, shall be taken into account.  The dynamometer setting for each test mode shall be calculated using the formula:

s = P(n) * (L/100) if tested under net conditions

s = P(n) * (L/100) + (P(a) − P(b)) if not tested under net conditions

where:

| s | = | dynamometer setting, kW |
| --- | --- | --- |
| P(n) | = | net engine power as indicated in Annex 1, paragraph 8.2., kW |
| L | = | per cent load as indicated in paragraph 2.7.1., per cent |
| P(a) | = | power absorbed by auxiliaries to be fitted as indicated in Annex 1, paragraph 6.1. |
| P(b) | = | power absorbed by auxiliaries to be removed as indicated in Annex 1, paragraph 6.2. |

2.	ESC TEST RUN

At the manufacturers request, a dummy test may be run for conditioning of the engine and exhaust system before the measurement cycle.

2.1.	Preparation of the sampling filter

At least one hour before the test, each filter shall be placed in a partially covered petri dish which is protected against dust contamination, and placed in a weighing chamber for stabilisation.  At the end of the stabilisation period each filter shall be weighed and the tare weight shall be recorded.  The filter shall then be stored in a closed petri dish or sealed filter holder until needed for testing.  The filter shall be used within eight hours of its removal from the weighing chamber.  The tare weight shall be recorded.

2.2.	Installation of the measuring equipment

The instrumentation and sample probes shall be installed as required.  When using a full flow dilution system for exhaust gas dilution, the tailpipe shall be connected to the system.

2.3.	Starting the dilution system and the engine

The dilution system and the engine shall be started and warmed up until all temperatures and pressures have stabilised at maximum power according to the recommendation of the manufacturer and good engineering practice.

2.4.	Starting the particulate sampling system

The particulate sampling system shall be started and running on by-pass.  The particulate background level of the dilution air may be determined by passing dilution air through the particulate filters.  If filtered dilution air is used, one measurement may be done prior to or after the test.  If the dilution air is not filtered, measurements at the beginning and at the end of the cycle, may be done, and the values averaged.

2.5.	Adjustment of the dilution ratio

The dilution air shall be set such that the temperature of the diluted exhaust gas measured immediately prior to the filter shall not exceed 325 K (52 °C) at any mode.  The dilution ratio (q) shall not be less than 4.

For systems that use CO2 or NOx concentration measurement for dilution ratio control, the CO2 or NOx content of the dilution air shall be measured at the beginning and at the end of each test.  The pre- and post test background CO2 or NOx concentration measurements of the dilution air shall be within 100 ppm or 5 ppm of each other, respectively.

2.6.	Checking the analysers

The emission analysers shall be set at zero and spanned.  The sample bags, if used, shall be evacuated.

2.7.	Test cycle

2.7.1.	The following 13-mode cycle shall be followed in dynamometer operation on the test engine

| Mode Number | Engine Speed | Percent Load | Weighting Factor | Mode Length |
| --- | --- | --- | --- | --- |
| 1 | idle | — | 0.15 | 4 minutes |
| 2 | A | 100 | 0.08 | 2 minutes |
| 3 | B | 50 | 0.10 | 2 minutes |
| 4 | B | 75 | 0.10 | 2 minutes |
| 5 | A | 50 | 0.05 | 2 minutes |
| 6 | A | 75 | 0.05 | 2 minutes |
| 7 | A | 25 | 0.05 | 2 minutes |
| 8 | B | 100 | 0.09 | 2 minutes |
| 9 | B | 25 | 0.10 | 2 minutes |
| 10 | C | 100 | 0.08 | 2 minutes |
| 11 | C | 25 | 0.05 | 2 minutes |
| 12 | C | 75 | 0.05 | 2 minutes |
| 13 | C | 50 | 0.05 | 2 minutes |

2.7.2.	Test sequence

The test sequence shall be started.  The test shall be performed in the order of the mode numbers as set out in paragraph 2.7.1.

The engine shall be operated for the prescribed time in each mode, completing engine speed and load changes in the first 20 seconds.  The specified speed shall be held to within ± 50 rpm and the specified torque shall be held to within ± 2 per cent of the maximum torque at the test speed.

At the manufacturer's request, the test sequence may be repeated a sufficient number of times for sampling more particulate mass on the filter.  The manufacturer shall supply a detailed description of the data evaluation and calculation procedures.  The gaseous emissions shall only be determined on the first cycle.

2.7.3.	Analyser response

The output of the analysers shall be recorded on a strip chart recorder or measured with an equivalent data acquisition system with the exhaust gas flowing through the analysers throughout the test cycle.

2.7.4.	Particulate sampling

A single filter shall be used for the complete test procedure.  The modal weighting factors specified in the test cycle procedure shall be taken into account by taking a sample proportional to the exhaust mass flow during each individual mode of the cycle.  This can be achieved by adjusting sample flow rate, sampling time, and/or dilution ratio, accordingly, so that the criterion for the effective weighting factors in paragraph 6.6. is met.

The sampling time per mode shall be at least 4 seconds per 0.01 weighting factor.  Sampling shall be conducted as late as possible within each mode.  Particulate sampling shall be completed no earlier than 5 seconds before the end of each mode.

2.7.5.	Engine conditions

The engine speed and load, intake air temperature and depression, exhaust temperature and backpressure, fuel flow and air or exhaust flow, charge air temperature, fuel temperature and humidity shall be recorded during each mode, with the speed and load requirements (see paragraph 2.7.2.) being met during the time of particulate sampling, but in any case during the last minute of each mode.

Any additional data required for calculation shall be recorded (see paragraphs 4. and 5.).

2.7.6.	NOx check within the control area

The NOx check within the control area shall be performed immediately upon completion of mode 13.

The engine shall be conditioned at mode 13 for a period of three minutes before the start of the measurements.  Three measurements shall be made at different locations within the control area, selected by the Technical Service /.  The time for each measurement shall be 2 minutes.

The measurement procedure is identical to the NOx measurement on the 13-mode cycle, and shall be carried out in accordance with paragraphs 2.7.3., 2.7.5., and 4.1. of this appendix, and Appendix 4, paragraph 3.

The calculation shall be carried out in accordance with paragraph 4.

2.7.7.	Rechecking the analysers

After the emission test a zero gas and the same span gas shall be used for rechecking.  The test will be considered acceptable if the difference between the pre-test and post-test results is less than 2 per cent of the span gas value.

3.	ELR TEST RUN

3.1.	Installation of the measuring equipment

The opacimeter and sample probes, if applicable, shall be installed after the exhaust silencer or any aftertreatment device, if fitted, according to the general installation procedures specified by the instrument manufacturer.  Additionally, the requirements of paragraph 10. of ISO 11614 shall be observed.

Prior to any zero and full-scale checks, the opacimeter shall be warmed up and stabilised according to the instrument manufacturer's recommendations.  If the opacimeter is equipped with a purge air system to prevent sooting of the meter optics, this system shall also be activated and adjusted according to the manufacturer's recommendations.

3.2.	Checking of the opacimeter

The zero and full scale checks shall be made in the opacity readout mode, since the opacity scale offers two truly definable calibration points, namely 0 per cent opacity and 100 per cent opacity.  The light absorption coefficient is then correctly calculated based upon the measured opacity and the LA, as submitted by the opacimeter manufacturer, when the instrument is returned to the k readout mode for testing.

With no blockage of the opacimeter light beam, the readout shall be adjusted to 0.0 per cent ± 1.0 per cent opacity. With the light being prevented from reaching the receiver, the readout shall be adjusted to 100.0 per cent ± 1.0 per cent opacity.

3.3.	Test cycle

3.3.1.	Conditioning of the engine

Warming up of the engine and the system shall be at maximum power in order to stabilise the engine parameters according to the recommendation of the manufacturer.  The preconditioning phase should also protect the actual measurement against the influence of deposits in the exhaust system from a former test.

When the engine is stabilised, the cycle shall be started within 20 ± 2 s after the preconditioning phase.  At the manufacturers request, a dummy test may be run for additional conditioning before the measurement cycle.

3.3.2.	Test sequence

The test consists of a sequence of three load steps at each of the three engine speeds A (cycle 1), B (cycle 2) and C (cycle 3) determined in accordance with Annex III, paragraph 1.1., followed by cycle 4 at a speed within the control area and a load between 10 per cent and 100 per cent, selected by the Technical Service /.  The following sequence shall be followed in dynamometer operation on the test engine, as shown in figure 3.

Figure 3

Sequence of ELR Test

(a)	The engine shall be operated at engine speed A and 10 per cent load for 20 ± 2 s.  The specified speed shall be held to within ± 20 rpm and the specified torque shall be held to within ± 2 per cent of the maximum torque at the test speed.

(b)	At the end of the previous segment, the speed control lever shall be moved rapidly to, and held in, the wide open position for 10 ± 1 s.  The necessary dynamometer load shall be applied to keep the engine speed within ± 150 rpm during the first 3 s, and within ± 20 rpm during the rest of the segment.

(c)	The sequence described in (a) and (b) shall be repeated two times.

(d)	Upon completion of the third load step, the engine shall be adjusted to engine speed B and 10 per cent load within 20 ± 2 s.

(e)	The sequence (a) to (c) shall be run with the engine operating at engine speed B.

(f)	Upon completion of the third load step, the engine shall be adjusted to engine speed C and 10 per cent load within 20 ± 2 s.

(g)	The sequence (a) to (c) shall be run with the engine operating at engine speed C.

(h)	Upon completion of the third load step, the engine shall be adjusted to the selected engine speed and any load above 10 per cent within 20 ± 2 s.

(i)	The sequence (a) to (c) shall be run with the engine operating at the selected engine speed.

3.4.	Cycle validation

The relative standard deviations of the mean smoke values at each test speed (SVA, SVB, SVC, as calculated in accordance with paragraph 6.3.3. of this appendix from the three successive load steps at each test speed) shall be lower than 15 per cent of the mean value, or 10 per cent of the limit value shown in table 1 of paragraph 5.2., whichever is greater. If the difference is greater, the sequence shall be repeated until 3 successive load steps meet the validation criteria.

3.5.	Rechecking of the opacimeter

The post-test opacimeter zero drift value shall not exceed ± 5,0 per cent of the limit value shown in table 1 of paragraph 5.2.

4.	CALCULATION OF THE EXHAUST GAS FLOW

4.1.	Determination of raw exhaust gas mass flow

For calculation of the emissions in the raw exhaust, it is necessary to know the exhaust gas flow.  The exhaust gas mass flow rate shall be determined in accordance with paragraph 4.1.1. or 4.1.2.  The accuracy of exhaust flow determination shall be ± 2,5 per cent of reading or  1.5 per cent of the engine's maximum value whichever is the greater.  Equivalent methods (e.g. those described in paragraph 4.2. of Appendix 2 to this annex) may be used.

4.1.1.	Direct measurement method

Direct measurement of the exhaust flow may be done by systems such as:

- (a)	pressure differential devices, like flow nozzle;

- (b)	ultrasonic flowmeter;

- (c)	vortex flowmeter.

Precautions shall be taken to avoid measurement errors which will impact emission value errors.  Such precautions include the careful installation of the device in the engine exhaust system according to the instrument manufacturers" recommendations and to good engineering practice.  Especially, engine performance and emissions shall not be affected by the installation of the device.

4.1.2.	Air and fuel measurement method

This involves measurement of the air flow and the fuel flow.  Air flowmeters and fuel flowmeters shall be used that meet the total accuracy requirement of paragraph 4.1.  The calculation of the exhaust gas flow is as follows:

4.2.	Determination of diluted exhaust gas mass flow

For calculation of the emissions in the diluted exhaust using a full flow dilution system it is necessary to know the diluted exhaust gas flow.  The flow rate of the diluted exhaust (qmedw) shall be measured over each mode with a PDP-CVS, CFV-CVS or SSV-CVS in line with the general formulae given in paragraph 4.1. of Appendix 2 to this annex.  The accuracy shall be ± 2 per cent of reading or better, and shall be determined according to the provisions of paragraph 2.4. of Appendix 5 to this annex.

5.	CALCULATION OF THE GASEOUS EMISSIONS

5.1.	Data evaluation

For the evaluation of the gaseous emissions, the chart reading of the last 30 seconds of each mode shall be averaged and the average concentrations (conc) of HC, CO and NOx during each mode shall be determined from the average chart readings and the corresponding calibration data.  A different type of recording can be used if it ensures an equivalent data acquisition.

For the NOx check within the control area, the above requirements apply for NOx only.

The exhaust gas flow qmew or the diluted exhaust gas flow qmdew, if used optionally, shall be determined in accordance with paragraph 2.3. of Appendix 4 to this annex.

5.2.	Dry / wet correction

The measured concentration shall be converted to a wet basis according to the following formulae, if not already measured on a wet basis.  The conversion shall be done for each individual mode.

For the raw exhaust gas:

or

or

kw,a=11+α×0.005×cCO2+cCO−kw1×1.008

with

kf	=	0.055594 x wALF + 0.0080021 x wDEL + 0.0070046 x wEPS

and

kw1 = 1.608×Ha1,000+1.608×Ha

where:

Ha	= intake air humidity, g water per kg dry air

wALF	= hydrogen content of the fuel, per cent mass

qmf,i	= instantaneous fuel mass flow rate, kg/s

qmad,I	= instantaneous dry intake air mass flow rate, kg/s

pr	= water vapour pressure after cooling bath, kPa

pb	= total atmospheric pressure, kPa

wDEL	= nitrogen content of the fuel, per cent mass

wEPS	= oxygen content of the fuel, per cent mass

	= molar hydrogen ratio of the fuel

cCO2	= dry CO2 concentration, per cent

cCO	= dry CO concentration, per cent

For the diluted exhaust gas:

or,

For the dilution air:

For the intake air:

where:

Ha	=	intake air humidity, g water per kg dry air

Hd	=	dilution air humidity, g water per kg dry air

and may be derived from relative humidity measurement, dewpoint measurement, vapour pressure measurement or dry/wet bulb measurement using the generally accepted formulae.

5.3.	NOx correction for humidity and temperature

As the NOx emission depends on ambient air conditions, the NOx concentration shall be corrected for ambient air temperature and humidity with the factors given in the following formulae.  The factors are valid in the range between 0 and 25 g/kg dry air.

(a)	for compression ignition engines:

with:

Ta =	temperature of the intake air, K

Ha =	humidity of the intake air, g water per kg dry air

where:

Ha may be derived from relative humidity measurement, dewpoint measurement, vapour pressure measurement or dry/wet bulb measurement using the generally accepted formulae.

(b)	for spark ignition engines

kh.G	=	0.6272 + 44.030 x 10-3 x Ha – 0.862 x 10-3 x Ha²

where:

Ha may be derived from relative humidity measurement, dew point measurement, vapour pressure measurement or dry/wet bulb measurement using the generally accepted formulae.

5.4.	Calculation of the emission mass flow rates

The emission mass flow rate (g/h) for each mode shall be calculated as follows.  For the calculation of NOx, the humidity correction factor kh,D, or kh,G, as applicable, as determined according to paragraph 5.3., shall be used.

The measured concentration shall be converted to a wet basis according to paragraph 5.2. if not already measured on a wet basis. Values for ugas are given in table 6 for selected components based on ideal gas properties and the fuels relevant for this Regulation.

(a)	for the raw exhaust gas

mgas =	ugas x cgas x qmew

where:

ugas	=	ratio between density of exhaust component and density of exhaust gas

cgas	=	concentration of the respective component in the raw exhaust gas, ppm

qmew	=	exhaust mass flow rate, kg/h

(b)	for the diluted gas

where:

ugas	=	ratio between density of exhaust component and density of air

cgas,c	=	background corrected concentration of the respective component in the diluted exhaust gas, ppm

qmdew	=	diluted exhaust mass flow rate, kg/h

where:

The dilution factor D shall be calculated according to paragraph 5.4.1. of Appendix 2 to this annex.

5.5.	Calculation of the specific emissions

The emissions (g/kWh) shall be calculated for all individual components in the following way:

where:

mgas is the mass of individual gas

Pn is the net power determined according to paragraph 8.2. in Annex 1.

The weighting factors used in the above calculation are according to paragraph 2.7.1.

Table 6

Values of ugas in the raw and dilute exhaust gas for various exhaust components

| Fuel |  | NOx | CO | THC/NMHC | CO2 | CH4 | Density |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel | Exhaust raw | 0.001587 | 0.000966 | 0.000479 | 0.001518 | 0.000553 | 1.2943 |
|  | Exhaust dilute | 0.001588 | 0.000967 | 0.000480 | 0.001519 | 0.000553 | 1.293 |
| Ethanol | Exhaust raw | 0.001609 | 0.000980 | 0.000805 | 0.001539 | 0.000561 | 1.2757 |
|  | Exhaust dilute | 0.001588 | 0.000967 | 0.000795 | 0.001519 | 0.000553 | 1.293 |
| CNG | Exhaust raw | 0.001622 | 0.000987 | 0.000523 | 0.001552 | 0.000565 | 1.2661 |
|  | Exhaust dilute | 0.001588 | 0.000967 | 0.000584 | 0.001519 | 0.000553 | 1.293 |
| Propane | Exhaust raw | 0.001603 | 0.000976 | 0.000511 | 0.001533 | 0.000559 | 1.2805 |
|  | Exhaust dilute | 0.001588 | 0.000967 | 0.000507 | 0.001519 | 0.000553 | 1.293 |
| Butane | Exhaust raw | 0.001600 | 0.000974 | 0.000505 | 0.001530 | 0.000558 | 1.2832 |
|  | Exhaust dilute | 0.001588 | 0.000967 | 0.000501 | 0.001519 | 0.000553 | 1.293 |
| Notes: - u values of raw exhaust based on ideal gas properties at  = 2. dry air, 273 K, 101.3 kPa - u values of dilute exhaust based on ideal gas properties and density of air - u values of CNG accurate within 0.2 per cent for mass composition of: C = 66 - 76 per cent; H = 22 - 25 per cent; N = 0 - 12 per cent - u value of CNG for HC corresponds to CH2.93 (for total HC use u value of CH4) |  |  |  |  |  |  |  |

5.6.	Calculation of the area control values

For the three control points selected according to paragraph 2.7.6., the NOX emission shall be measured and calculated according to paragraph 5.6.1. and also determined by interpolation from the modes of the test cycle closest to the respective control point according to paragraph 5.6.2.  The measured values are then compared to the interpolated values according to paragraph 5.6.3.

5.6.1.	Calculation of the specific emission

The NOx emission for each of the control points (Z) shall be calculated as follows:

mNOx,Z =	0.001587 x cNOx,Z x kh,D x qmew

5.6.2.	Determination of the emission value from the test cycle

The NOx emission for each of the control points shall be interpolated from the four closest modes of the test cycle that envelop the selected control point Z as shown in figure 4.  For these modes (R, S, T, U), the following definitions apply:

Speed(R) = Speed(T) = nRT

Speed(S) = Speed(U) = nSU

Per cent load(R) = Per cent load(S)

Per cent load(T) = Per cent load(U).

The NOx emission of the selected control point Z shall be calculated as follows:

and:

where:

ER, ES, ET, EU =	specific NOx emission of the enveloping modes calculated in accordance with paragraph 5.6.1.

MR, MS, MT, MU =	engine torque of the enveloping modes

Figure 4

Interpolation of NOx Control Point

5.6.3.	Comparison of NOx emission values

The measured specific NOx emission of the control point Z (NOx,Z) is compared to the interpolated value (EZ) as follows:

6.	CALCULATION OF THE PARTICULATE EMISSIONS

6.1.	Data evaluation

For the evaluation of the particulates, the total sample masses (msep) through the filter shall be recorded for each mode.

The filter shall be returned to the weighing chamber and conditioned for at least one hour, but not more than 80 hours, and then weighed.  The gross weight of the filters shall be recorded and the tare weight (see paragraph 2.1.) subtracted, which results in the particulate sample mass mf.

If background correction is to be applied, the dilution air mass (md) through the filter and the particulate mass (mf, d) shall be recorded.  If more than one measurement was made, the quotient mf,d/md shall be calculated for each single measurement and the values averaged.

6.2.	Partial flow dilution system

The final reported test results of the particulate emission shall be determined through the following steps.  Since various types of dilution rate control may be used, different calculation methods for qmedf apply.  All calculations shall be based upon the average values of the individual modes during the sampling period.

6.2.1.	Isokinetic systems

where ra corresponds to the ratio of the cross sectional areas of the isokinetic probe and the exhaust pipe:

6.2.2.	Systems with measurement of CO2 or NOx concentration

where:

| cwE | = wet concentration of the tracer gas in the raw exhaust |
| --- | --- |
| cwD | = wet concentration of the tracer gas in the diluted exhaust |
| cwA | = wet concentration of the tracer gas in the dilution air |

Concentrations measured on a dry basis shall be converted to a wet basis according to paragraph 5.2. of this appendix.

6.2.3.	Systems with CO2 measurement and carbon balance method /

where:

| c(CO2)D | = CO2 concentration of the diluted exhaust |
| --- | --- |
| c(CO2)A | = CO2 concentration of the dilution air |

(concentrations in vol per cent on wet basis)

This equation is based upon the carbon balance assumption (carbon atoms supplied to the engine are emitted as CO2) and determined through the following steps:

and

6.2.4.	Systems with flow measurement

6.3.	Full flow dilution system

All calculations shall be based upon the average values of the individual modes during the sampling period.  The diluted exhaust gas flow qmdew shall be determined in accordance with paragraph 4.1. of Appendix 2 to this annex.  The total sample mass msep shall be calculated in accordance with paragraph 6.2.1. of Appendix 2 to this annex.

6.4.	Calculation of the particulate mass flow rate

The particulate mass flow rate shall be calculated as follows. If a full flow dilution system is used, qmedf as determined according to paragraph 6.2. shall be replaced with qmdew as determined according to paragraph 6.3.

i = 1, ... n

The particulate mass flow rate may be background corrected as follows:

where D shall be calculated in accordance with paragraph 5.4.1. of Appendix 2 to this annex.

6.5.	Calculation of the specific emission

The particulate emission shall be calculated in the following way:

PT = PTmassi = 1i = nPi × Wfi

6.6.	Effective weighting factor

The effective weighting factor Wfei for each mode shall be calculated in the following way:

Wfei = msepi × qmedfmsep × qmedfi

The value of the effective weighting factors shall be within ± 0.003 ( 0.005 for the idle mode) of the weighting factors listed in paragraph 2.7.1. of this appendix.

7.	CALCULATION OF THE SMOKE VALUES

7.1.	Bessel algorithm

The Bessel algorithm shall be used to compute the 1 s average values from the instantaneous smoke readings, converted in accordance with paragraph 6.3.1.  The algorithm emulates a low pass second order filter, and its use requires iterative calculations to determine the coefficients. These coefficients are a function of the response time of the opacimeter system and the sampling rate.  Therefore, paragraph 6.1.1. shall be repeated whenever the system response time and/or sampling rate changes.

7.1.1.	Calculation of filter response time and Bessel constants

The required Bessel response time (tF) is a function of the physical and electrical response times of the opacimeter system, as specified in Appendix 4 to this annex and shall be calculated by the following equation:

where:

| tp | = physical response time, s |
| --- | --- |
| te | = electrical response time, s |

The calculations for estimating the filter cut-off frequency (fc) are based on a step input 0 to 1 in ≤ 0.01 s (see Annex 6).  The response time is defined as the time between when the Bessel output reaches 10 per cent (t10) and when it reaches 90 per cent (t90) of this step function.  This shall be obtained by iterating on fc until t90-t10≈tF.  The first iteration for fc is given by the following formula:

The Bessel constants E and K shall be calculated by the following equations:

where:

| D | = 0,618034 |
| --- | --- |
| Δt |  |
| Ω |  |

7.1.2.	Calculation of the Bessel algorithm

Using the values of E and K, the 1 s Bessel averaged response to a step input Si shall be calculated as follows:

Yi = Yi−1 + E  (Si + 2  Si−1 + Si−2 − 4  Yi−2) + K  (Yi−1 − Yi−2)

where:

| Si-2 | = Si-1 = 0 |
| --- | --- |
| Si | = 1 |
| Yi-2 | = Yi-1 = 0 |

The times t10 and t90 shall be interpolated.  The difference in time between t90 and t10 defines the response time tF for that value of fc.  If this response time is not close enough to the required response time, iteration shall be continued until the actual response time is within 1 per cent of the required response as follows:

((t90 − t10) − tF) ≤ 0.01  tF

7.2.	Data evaluation

The smoke measurement values shall be sampled with a minimum rate of 20 Hz.

7.3.	Determination of smoke

7.3.1.	Data Conversion

Since the basic measurement unit of all opacimeters is transmittance, the smoke values shall be converted from transmittance (τ) to the light absorption coefficient (k) as follows:

and

N = 100 − τ

where:

| k | = light absorption coefficient, m-1 |
| --- | --- |
| LA | = effective optical path length, as submitted by instrument manufacturer, m |
| N | = opacity, per cent |
| τ | = transmittance, per cent |

The conversion shall be applied, before any further data processing is made.

7.3.2.	Calculation of Bessel averaged smoke

The proper cut-off frequency fc is the one that produces the required filter response time tF. Once this frequency has been determined through the iterative process of paragraph 6.1.1., the proper Bessel algorithm constants E and K shall be calculated.  The Bessel algorithm shall then be applied to the instantaneous smoke trace (k-value), as described in paragraph 6.1.2.:

Yi = Yi−1 + E  (Si + 2  Si−1 + Si−2 − 4  Yi−2) + K  (Yi−1 − Yi−2)

The Bessel algorithm is recursive in nature.  Thus, it needs some initial input values of Si-1 and Si-2 and initial output values Yi-1 and Yi-2 to get the algorithm started.  These may be assumed to be 0.

For each load step of the three speeds A, B and C, the maximum 1s value Ymax shall be selected from the individual Yi values of each smoke trace.

7.3.3.	Final result

The mean smoke values (SV) from each cycle (test speed) shall be calculated as follows:

| For test speed A: | SVA = (Ymax1,A + Ymax2,A + Ymax3,A) / 3 |
| --- | --- |
| For test speed B: | SVB = (Ymax1,B + Ymax2,B + Ymax3,B) / 3 |
| For test speed C: | SVC = (Ymax1,C + Ymax2,C + Ymax3,C) / 3 |

where:

| Ymax1, Ymax2, Ymax3 | = highest 1 s Bessel averaged smoke value at each of the three load steps |
| --- | --- |

The final value shall be calculated as follows:

SV = (0.43  SVA) + (0.56  SVB) + (0.01  SVC)

Appendix 2

ETC TEST CYCLE

1.	ENGINE MAPPING PROCEDURE

1.1.	Determination of the Mapping Speed Range

For generating the ETC on the test cell, the engine needs to be mapped prior to the test cycle for determining the speed vs. torque curve.  The minimum and maximum mapping speeds are defined as follows:

| Minimum mapping speed | = idle speed |
| --- | --- |
| Maximum mapping speed | = nhi  1.02 or speed where full load torque drops off to zero, whichever is lower |

1.2.	Performing the Engine Power Map

The engine shall be warmed up at maximum power in order to stabilise the engine parameters according to the recommendation of the manufacturer and good engineering practice. When the engine is stabilised, the engine map shall be performed as follows:

(a)	the engine shall be unloaded and operated at idle speed;

(b)	the engine shall be operated at full load setting of the injection pump at minimum mapping speed;

(c)	the engine speed shall be increased at an average rate of 8 ± 1 min−1 /s from minimum to maximum mapping speed.  Engine speed and torque points shall be recorded at a sample rate of a least one point per second.

1.3.	Mapping Curve Generation

All data points recorded under paragraph 1.2. of this appendix shall be connected using linear interpolation between points.  The resulting torque curve is the mapping curve and shall be used to convert the normalized torque values of the engine cycle into actual torque values for the test cycle, as described in paragraph 2. of this appendix.

1.4.	Alternate Mapping

If a manufacturer believes that the above mapping techniques are unsafe or unrepresentative for any given engine, alternate mapping techniques may be used.  These alternate techniques shall satisfy the intent of the specified mapping procedures to determine the maximum available torque at all engine speeds achieved during the test cycles.  Deviations from the mapping techniques specified in this paragraph for reasons of safety or representativeness shall be approved by the Technical Service along with the justification for their use.  In no case, however, shall descending continual sweeps of engine speed be used for governed or turbocharged engines.

1.5.	Replicate Tests

An engine need not be mapped before each and every test cycle. An engine shall be remapped prior to a test cycle if:

- (a)	an unreasonable amount of time has transpired since the last map, as determined by engineering judgement,

or,

- (b)	physical changes or recalibrations have been made to the engine which may potentially affect engine performance.

2.	GENERATION OF THE REFERENCE TEST CYCLE

The transient test cycle is described in Appendix 3 to this annex.  The normalized values for torque and speed shall be changed to the actual values, as follows, resulting in the reference cycle.

2.1.	Actual Speed

The speed shall be unnormalized using the following equation:

The reference speed (nref) corresponds to the 100 per cent speed values specified in the engine dynamometer schedule of Appendix 3.  It is defined as follows (see figure 1 of paragraph 2.):

nref = nlo + 95 per cent  (nhi − nlo)

where nhi and nlo are either specified according to paragraph 2. or determined according to paragraph 1.1. of Appendix 1 to this annex.

2.2.	Actual torque

The torque is normalized to the maximum torque at the respective speed.  The torque values of the reference cycle shall be unnormalized, using the mapping curve determined according to paragraph 1.3. of this appendix, as follows:

Actual torque = ( per cent torque  max. torque/100)

for the respective actual speed as determined in paragraph 2.1.of this appendix.

The negative torque values of the motoring points ("m") shall take on, for purposes of reference cycle generation, unnormalized values determined in either of the following ways:

- (a)	negative 40 per cent of the positive torque available at the associated speed point,

- (b)	mapping of the negative torque required to motor the engine from minimum to maximum mapping speed,

- (c)	determination of the negative torque required to motor the engine at idle and reference speeds and linear interpolation between these two points.

2.3.	Example of the unnormalization procedure

As an example, the following test point shall be unnormalized:

| per cent speed | = 43 |
| --- | --- |
| per cent torque | = 82 |

Given the following values:

| reference speed | = 2,200 min−1 |
| --- | --- |
| idle speed | = 600 min−1 |

results in,

actual speed = (43  (2,200 − 600)/100) + 600 = 1,288 min−1

actual torque = (82  700/100) = 574 Nm

where the maximum torque observed from the mapping curve at 1,288 min−1 is 700 Nm.

3.	EMISSIONS TEST RUN

At the manufacturers request, a dummy test may be run for conditioning of the engine and exhaust system before the measurement cycle.

NG and LPG fuelled engines shall be run-in using the ETC test.  The engine shall be run over a minimum of two ETC cycles and until the CO emission measured over one ETC cycle does not exceed by more than 10 per cent the CO emission measured over the previous ETC cycle.

3.1.	Preparation of the sampling filters (if applicable)

At least one hour before the test, each filter shall be placed in a partially covered petri dish, which is protected against dust contamination, and placed in a weighing chamber for stabilisation.  At the end of the stabilisation period, each filter shall be weighed and the tare weight shall be recorded.  The filter shall then be stored in a closed petri dish or sealed filter holder until needed for testing.  The filter shall be used within eight hours of its removal from the weighing chamber.  The tare weight shall be recorded.

3.2.	Installation of the measuring equipment

The instrumentation and sample probes shall be installed as required.  The tailpipe shall be connected to the full flow dilution system, if used.

3.3.	Starting the dilution system and the engine

The dilution system and the engine shall be started and warmed up until all temperatures and pressures have stabilised at maximum power according to the recommendation of the manufacturer and good engineering practice.

3.4.	Starting the particulate sampling system (diesel engines only)

The particulate sampling system shall be started and running on by-pass.  The particulate background level of the dilution air may be determined by passing dilution air through the particulate filters.  If filtered dilution air is used, one measurement may be done prior to or after the test.  If the dilution air is not filtered, measurements at the beginning and at the end of the cycle may be done and the values averaged.

The dilution system and the engine shall be started and warmed up until all temperatures and pressures have stabilised according to the recommendation of the manufacturer and good engineering practice.

In case of periodic regeneration aftertreatment, the regeneration shall not occur during the warm-up of the engine.

3.5.	Adjustment of the dilution system

The flow rates of the dilution system (full flow or partial flow) shall be set to eliminate water condensation in the system, and to obtain a filter face temperature of maximum 325 K (52°C) or less (see paragraph 2.3.1. of Appendix 7, DT).

3.6.	Checking the analysers

The emission analysers shall be set at zero and spanned.  If sample bags are used, they shall be evacuated.

3.7.	Engine starting procedure

The stabilized engine shall be started according to the manufacturer's recommended starting procedure in the owner's manual, using either a production starter motor or the dynamometer.  Optionally, the test may start directly from the engine-preconditioning phase without shutting the engine off, when the engine has reached the idle speed.

3.8.	Test cycle

3.8.1.	Test sequence

The test sequence shall be started, if the engine has reached idle speed.  The test shall be performed according to the reference cycle as set out in paragraph 2. of this appendix.  Engine speed and torque command set points shall be issued at 5 Hz (10 Hz recommended) or greater.  Feedback engine speed and torque shall be recorded at least once every second during the test cycle, and the signals may be electronically filtered.

3.8.2.	Gaseous emissions measurement

3.8.2.1.	Full flow dilution system

At the start of the engine or test sequence, if the cycle is started directly from the preconditioning, the measuring equipment shall be started, simultaneously:

- (a)	start collecting or analysing dilution air;

- (b)	start collecting or analysing diluted exhaust gas;

- (c)	start measuring the amount of diluted exhaust gas (CVS) and the required temperatures and pressures;

- (d)	start recording the feedback data of speed and torque of the dynamometer.

HC and NOx shall be measured continuously in the dilution tunnel with a frequency of 2 Hz.  The average concentrations shall be determined by integrating the analyzer signals over the test cycle.  The system response time shall be no greater than 20 s, and shall be coordinated with CVS flow fluctuations and sampling time/test cycle offsets, if necessary. CO, CO2, NMHC and CH4 shall be determined by integration or by analysing the concentrations in the sample bag, collected over the cycle.  The concentrations of the gaseous pollutants in the dilution air shall be determined by integration or by collecting into the background bag.  All other values shall be recorded with a minimum of one measurement per second (1 Hz).

3.8.2.2	Raw exhaust measurement

At the start of the engine or test sequence, if the cycle is started directly from the preconditioning,, the measuring equipment shall be started, simultaneously:

- (a)	start analysing the raw exhaust gas concentrations;

- (b)	start measuring the exhaust gas or intake air and fuel flow rate;

- (c)	start recording the feedback data of speed and torque of the dynamometer.

For the evaluation of the gaseous emissions, the emission concentrations (HC, CO and NOx) and the exhaust gas mass flow rate shall be recorded and stored with at least 2 Hz on a computer system.  The system response time shall be no greater than 10 s.  All other data may be recorded with a sample rate of at least 1 Hz.  For analogue analysers the response shall be recorded, and the calibration data may be applied online or offline during the data evaluation.

For calculation of the mass emission of the gaseous components the traces of the recorded concentrations and the trace of the exhaust gas mass flow rate shall be time aligned by the transformation time as defined in paragraph 2. of this Regulation.  Therefore, the response time of each gaseous emissions analyser and of the exhaust gas mass flow system shall be determined according to the provisions of paragraph 4.2.1. and paragraph 1.5. of Appendix 5 to this annex and recorded.

3.8.3.	Particulate sampling (if applicable)

3.8.3.1.	Full flow dilution system

At the start of the engine or test sequence, if the cycle is started directly from the preconditioning, the particulate sampling system shall be switched from by-pass to collecting particulates.

If no flow compensation is used, the sample pump(s) shall be adjusted so that the flow rate through the particulate sample probe or transfer tube is maintained at a value within ± 5 per cent of the set flow rate.  If flow compensation (i.e., proportional control of sample flow) is used, it shall be demonstrated that the ratio of main tunnel flow to particulate sample flow does not change by more than ± 5 per cent of its set value (except for the first 10 seconds of sampling).

For double dilution operation, sample flow is the net difference between the flow rate through the sample filters and the secondary dilution air flow rate.

The average temperature and pressure at the gas meter(s) or flow instrumentation inlet shall be recorded.  If the set flow rate cannot be maintained over the complete cycle (within ± 5 per cent) because of high particulate loading on the filter, the test shall be voided.  The test shall be rerun using a lower flow rate and/or a larger diameter filter.

3.8.3.2.	Partial flow dilution system

At the start of the engine or test sequence, if the cycle is started directly from the preconditioning, the particulate sampling system shall be switched from by-pass to collecting particulates.

For the control of a partial flow dilution system, a fast system response is required.  The transformation time for the system shall be determined by the procedure in paragraph 3.3. of Appendix 5 to this annex.  If the combined transformation time of the exhaust flow measurement (see paragraph 4.2.1. of this appendix) and the partial flow system is ≤ 0.3 s, online control may be used.  If the transformation time exceeds 0.3 s, look ahead control based on a pre-recorded test run shall be used.  In this case, the rise time shall be  1 s and the delay time of the combination  10 s.

The total system response shall be designed as to ensure a representative sample of the particulates, qmp,i, proportional to the exhaust mass flow.  To determine the proportionality, a regression analysis of qmp,i versus qmew,i shall be conducted on a minimum 1 Hz data acquisition rate, and the following criteria shall be met:

- (a)	The correlation coefficient R2 of the linear regression between qmp,i and qmew,i shall not be less than 0.95;

- (b)	The standard error of estimate of qmp,i on qmew,i shall not exceed 5 per cent of qmp maximum;

- (c)	qmp intercept of the regression line shall not exceed  2 per cent of qmp maximum.

Optionally, a pretest may be run, and the exhaust mass flow signal of the pretest be used for controlling the sample flow into the particulate system (look-ahead control).  Such a procedure is required if the transformation time of the particulate system, t50,P or the transformation time of the exhaust mass flow signal, t50,F, or both, are > 0.3 s.  A correct control of the partial dilution system is obtained, if the time trace of qmew,pre of the pretest, which controls qmp, is shifted by a look-ahead time of t50,P + t50,F .

For establishing the correlation between qmp,i and qmew,i the data taken during the actual test shall be used, with qmew,i time aligned by t50,F relative to qmp,i (no contribution from t50,P to the time alignment).  That is, the time shift between qmew and qmp is the difference in their transformation times that were determined in paragraph 3.3. of Appendix 5 to this annex.

3.8.4.	Engine stalling

If the engine stalls anywhere during the test cycle, the engine shall be preconditioned and restarted, and the test repeated.  If a malfunction occurs in any of the required test equipment during the test cycle, the test shall be voided.

3.8.5.	Operations after test

At the completion of the test, the measurement of the diluted exhaust gas volume or raw exhaust gas flow rate, the gas flow into the collecting bags and the particulate sample pump shall be stopped.  For an integrating analyser system, sampling shall continue until system response times have elapsed.

The concentrations of the collecting bags, if used, shall be analysed as soon as possible and in any case not later than 20 minutes after the end of the test cycle.

After the emission test, a zero gas and the same span gas shall be used for re-checking the analysers.  The test will be considered acceptable if the difference between the pre-test and post-test results is less than 2 per cent of the span gas value.

3.9.	Verification of the test run

3.9.1.	Data shift

To minimize the biasing effect of the time lag between the feedback and reference cycle values, the entire engine speed and torque feedback signal sequence may be advanced or delayed in time with respect to the reference speed and torque sequence.  If the feedback signals are shifted, both speed and torque shall be shifted the same amount in the same direction.

3.9.2.	Calculation of the cycle work

The actual cycle work Wact (kWh) shall be calculated using each pair of engine feedback speed and torque values recorded.  This shall be done after any feedback data shift has occurred, if this option is selected.  The actual cycle work Wact is used for comparison to the reference cycle work Wref and for calculating the brake specific emissions (see paragraphs 5.5. and 6.3. of this appendix).  The same methodology shall be used for integrating both reference and actual engine power.  If values are to be determined between adjacent reference or adjacent measured values, linear interpolation shall be used.

In integrating the reference and actual cycle work, all negative torque values shall be set equal to zero and included.  If integration is performed at a frequency of less than 5 Hz, and if, during a given time segment, the torque value changes from positive to negative or negative to positive, the negative portion shall be computed and set equal to zero.  The positive portion shall be included in the integrated value.

Wact shall be between − 15 per cent and + 5 per cent of Wref

3.9.3.	Validation statistics of the test cycle

Linear regressions of the feedback values on the reference values shall be performed for speed, torque and power.  This shall be done after any feedback data shift has occurred, if this option is selected.  The method of least squares shall be used, with the best-fit equation having the form:

y = mx + b

where:

| y | = Feedback (actual) value of speed (min−1), torque (Nm), or power (kW) |
| --- | --- |
| m | = slope of the regression line |
| x | = reference value of speed (min−1), torque (Nm), or power (kW) |
| b | = y intercept of the regression line |

The standard error of estimate (SE) of y on x and the coefficient of determination (r2) shall be calculated for each regression line.

It is recommended that this analysis be performed at 1 Hz.  All negative reference torque values and the associated feedback values shall be deleted from the calculation of cycle torque and power validation statistics.  For a test to be considered valid, the criteria of table 7 shall be met.

Table 7

Regression line tolerances

|  | Speed | Torque | Power |
| --- | --- | --- | --- |
| Standard error of estimate (SE) of Y on X | Max 100 min–1 | Max 13 per cent of power map maximum engine torque | Max 8 per cent of power map maximum engine power |
| Slope of the regression line, m | 0.95 to 1.03 | 0.83-1.03 | 0.89-1.03 |
| Coefficient of determination, r2 | min 0.9700 | min 0.8800 | min 0.9100 |
| Y intercept of the regression line, b | ± 50 min–1 | ± 20 Nm or ± 2 per cent of max torque whichever is greater | ± 4 kW or ± 2 per cent of max power whichever is greater |

Point deletions from the regression analyses are permitted where noted in table 8.

Table 8

Permitted point deletions from regression analysis

| Conditions | Points to be deleted |
| --- | --- |
| Full load demand and torque feedback < 95 per cent torque reference | Torque and/or power |
| Full load demand and speed feedback < 95 per cent speed reference | Speed and/or power |
| No load, not an idle point, and torque feedback > torque reference | Torque and/or power |
| No load, speed feedback  idle speed + 50 min-1 and torque feedback = manufacturer defined/measured idle torque  2 per cent of max. torque | Speed and/or power |
| No load, speed feedback > idle speed + 50 min-1 and torque feedback > 105 per cent torque reference | Torque and/or power |
| No load and speed feedback > 105 per cent speed reference | Speed and/or power |

4.	CALCULATION OF THE EXHAUST GAS FLOW

4.1.	Determination of the diluted exhaust gas flow

The total diluted exhaust gas flow over the cycle (kg/test) shall be calculated from the measurement values over the cycle and the corresponding calibration data of the flow measurement device (V0 for PDP, KV for CFV, Cd for SSV), as determined in paragraph 2. of Appendix 5 to this annex).  The following formulae shall be applied, if the temperature of the diluted exhaust is kept constant over the cycle by using a heat exchanger (±6 K for a PDP-CVS, ±11 K for a CFV-CVS or ±11 K for a SSV-CVS), see paragraph 2.3. of Annex V).

For the PDP-CVS system:

med = 1.293 · V0 · NP · (pb - p1) · 273 / (101.3 · T)

where:

V0	=	volume of gas pumped per revolution under test conditions, m³/rev

NP	=	total revolutions of pump per test

pb	=	atmospheric pressure in the test cell, kPa

p1	=	pressure depression below atmospheric at pump inlet, kPa

T	=	average temperature of the diluted exhaust gas at pump inlet over the cycle, K

For the CFV-CVS system:

med	= 1.293 · t · Kv · pp / T 0,5

where:

t	=	cycle time, s

KV	=	calibration coefficient of the critical flow venturi for standard conditions,

pp	=	absolute pressure at venturi inlet, kPa

T	=	absolute temperature at venturi inlet, K

For the SSV-CVS system

where:

with:

A0	=	collection of constants and units conversions

=	0.006111 in SI units of m3minK12kPa1mm2

d	=	diameter of the SSV throat, m

Cd	=	discharge coefficient of the SSV

pp	=	absolute pressure at venturi inlet, kPa

T	=	temperature at the venturi inlet, K

rp	=	ratio of the SSV throat to inlet absolute, static pressure = 1−Δppa

rD	=	ratio of the SSV throat diameter, d, to the inlet pipe inner diameter D

If a system with flow compensation is used (i.e. without heat exchanger), the instantaneous mass emissions shall be calculated and integrated over the cycle.  In this case, the instantaneous mass of the diluted exhaust gas shall be calculated as follows.

For the PDP-CVS system:

med,i = 1.293 · V0 · NP,i · (pb - p1) · 273 / (101.3 · T)

where:

NP,i	=	total revolutions of pump per time interval

For the CFV-CVS system:

med,i = 1.293 · ti · KV · pp / T 0,5

where:

ti	=	time interval, s

For the SSV-CVS system:

med,i	=	1.293 · QSSV · ti

where:

ti	=	time interval, s

The real time calculation shall be initialised with either a reasonable value for Cd, such as 0.98, or a reasonable value of Qssv.  If the calculation is initialised with Qssv, the initial value of Qssv shall be used to evaluate Re.

During all emissions tests, the Reynolds number at the SSV throat shall be in the range of Reynolds numbers used to derive the calibration curve developed in paragraph 2.4. of Appendix 5 to this annex.

4.2.	Determination of raw exhaust gas mass flow

For calculation of the emissions in the raw exhaust gas and for controlling of a partial flow dilution system, it is necessary to know the exhaust gas mass flow rate.  For the determination of the exhaust mass flow rate, either of the methods described in paragraphs 4.2.2. to 4.2.5. of this appendix may be used.

4.2.1.	Response time

For the purpose of emissions calculation, the response time of either method described below shall be equal to or less than the requirement for the analyzer response time, as defined in paragraph 1.5. of Appendix 5 to this annex.

For the purpose of controlling of a partial flow dilution system, a faster response is required.  For partial flow dilution systems with online control, a response time of  0.3 seconds is required.  For partial flow dilution systems with look ahead control based on a pre-recorded test run, a response time of the exhaust flow measurement system of  5 seconds with a rise time of  1 second is required.  The system response time shall be specified by the instrument manufacturer.  The combined response time requirements for exhaust gas flow and partial flow dilution system are indicated in paragraph 3.8.3.2.

4.2.2.	Direct measurement method

Direct measurement of the instantaneous exhaust flow may be done by systems such as:

- (a)	pressure differential devices, like flow nozzle;

- (b)	ultrasonic flowmeter;

- (c)	vortex flowmeter.

Precautions shall be taken to avoid measurement errors which will impact emission value errors.  Such precautions include the careful installation of the device in the engine exhaust system according to the instrument manufacturers" recommendations and to good engineering practice.  Engine performance and emissions shall especially not be affected by the installation of the device.

The accuracy of exhaust flow determination shall be at least ± 2.5 per cent of reading or  1.5 per cent of engine's maximum value, whichever is the greater.

4.2.3.	Air and fuel measurement method

This involves measurement of the air flow and the fuel flow.  Air flowmeters and fuel flowmeters shall be used that meet the total exhaust flow accuracy requirement of paragraph 4.2.2. of this appendix.  The calculation of the exhaust gas flow is as follows:

4.2.4.	Tracer measurement method

This involves measurement of the concentration of a tracer gas in the exhaust.  A known amount of an inert gas (e.g. pure helium) shall be injected into the exhaust gas flow as a tracer.  The gas is mixed and diluted by the exhaust gas, but shall not react in the exhaust pipe.  The concentration of the gas shall then be measured in the exhaust gas sample.

In order to ensure complete mixing of the tracer gas, the exhaust gas sampling probe shall be located at least 1 m or 30 times the diameter of the exhaust pipe, whichever is larger, downstream of the tracer gas injection point.  The sampling probe may be located closer to the injection point if complete mixing is verified by comparing the tracer gas concentration with the reference concentration when the tracer gas is injected upstream of the engine.

The tracer gas flow rate shall be set so that the tracer gas concentration at engine idle speed after mixing becomes lower than the full scale of the trace gas analyser.

The calculation of the exhaust gas flow is as follows:

qmew,i=qvt×ρe60×cmix,i−cb

where:

qmew,i	=	instantaneous exhaust mass flow, kg/s

qvt	=	tracer gas flow, cm³/min

cmix.i	=	instantaneous concentration of the tracer gas after mixing, ppm

e	=	density of the exhaust gas, kg/m³ (cf. table 6)

cb	=	background concentration of the tracer gas in the intake air, ppm

When the background concentration is less than 1 per cent of the concentration of the tracer gas after mixing (cmix.i) at maximum exhaust flow, the background concentration may be neglected.

The total system shall meet the accuracy specifications for the exhaust gas flow, and shall be calibrated according to paragraph 1.7. of Appendix 5 to this annex.

4.2.5.	Air flow and air-to-fuel ratio measurement method

This involves exhaust mass calculation from the air flow and the air to fuel ratio.  The calculation of the instantaneous exhaust gas mass flow is as follows:

with:

AFst=138.0×1+α4−ε2+γ12.011+1.00794×α+15.9994×ε+14.0067×δ+32.065×γ

λi=100-cCOd×10−42−cHCw×10−4+α4×1−2×cCOd×10−43.5×cCO2d1+cCO×10−43.5×cCO2d−ε2−δ2×cCO2d+cCOd×10−44.764×1+α4−ε2+γ×cCO2d+cCOd×10−4+cHCw×10−4

where:

A/Fst	=	stoichiometric air to fuel ratio, kg/kg

	=	excess air ratio

cCO2	=	dry CO2 concentration, per cent

cCO	=	dry CO concentration, ppm

cHC	=	HC concentration, ppm

The air flowmeter shall meet the accuracy specifications of paragraph 2.2. of Appendix 4 to this annex, the CO2 analyser used shall meet the specifications of paragraph 3.3.2. of Appendix 4 to this annex and the total system shall meet the accuracy specifications for the exhaust gas flow.

Optionally, air to fuel ratio measurement equipment such as a zirconia type sensor may be used for the measurement of the excess air ratio which meets the specifications of paragraph 3.3.6. of Appendix 4 to this annex.

5.	CALCULATION OF THE GASEOUS EMISSIONS

5.1.	Data evaluation

For the evaluation of the gaseous emissions in the diluted exhaust gas, the emission concentrations (HC, CO and NOx) and the diluted exhaust gas mass flow rate shall be recorded according to paragraph 3.8.2.1. of this appendix and stored on a computer system.  For analogue analysers the response shall be recorded, and the calibration data may be applied online or offline during the data evaluation.

For the evaluation of the gaseous emissions in the raw exhaust gas, the emission concentrations (HC, CO and NOx) and the exhaust gas mass flow rate shall be recorded according to paragraph 3.8.2.2. of this appendix and stored on a computer system.  For analogue analysers the response shall be recorded, and the calibration data may be applied online or offline during the data evaluation.

5.2.	Dry / wet correction

If the concentration is measured on a dry basis, it shall be converted to a wet basis according to the following formula.  For continuous measurement, the conversion shall be applied to each instantaneous measurement before any further calculation.

cw = kw × cd

The conversion equations of paragraph 5.2. of Appendix 1 to this annex shall apply.

5.3.	NOx correction for humidity and temperature

As the NOx emission depends on ambient air conditions, the NOx concentration shall be corrected for ambient air temperature and humidity with the factors given in paragraph 5.3. of Appendix 1 to this annex.  The factors are valid in the range between 0 and 25 g/kg dry air.

5.4.	Calculation of the emission mass flow rates

The emission mass over the cycle (g/test) shall be calculated as follows depending on the measurement method applied.  The measured concentration shall be converted to a wet basis according to paragraph 5.2. of Appendix 1 to this annex, if not already measured on a wet basis.  The respective values for ugas shall be applied that are given in table 6 of Appendix 1 to this annex for selected components based on ideal gas properties and the fuels relevant for this Regulation.

(a)	for the raw exhaust gas:

mgas =	ugas×i=1i=ncgas,i×qmew,i×1f

where:

ugas	=	ratio between density of exhaust component and density of exhaust gas from table 6

cgas,i	=	instantaneous concentration of the respective component in the raw exhaust gas, ppm

qmew,i	=	instantaneous exhaust mass flow rate, kg/s

f	=	data sampling rate, Hz

n	=	number of measurements

(b)	for the diluted exhaust gas without flow compensation:

mgas	=	ugas x cgas x med

where:

ugas	=	ratio between density of exhaust component and density of air from table 6

cgas	=	average background corrected concentration of the respective component, ppm

med	=	total diluted exhaust mass over the cycle, kg

(c)	for the diluted exhaust gas with flow compensation:

mgas	=

where:

ce,i	=	instantaneous concentration of the respective component measured in the diluted exhaust gas, ppm

cd	=	concentration of the respective component measured in the dilution air, ppm

qmdew,i	=	instantaneous diluted exhaust gas mass flow rate, kg/s

med	=	total mass of diluted exhaust gas over the cycle, kg

ugas	=	ratio between density of exhaust component and density of air from table 6

D	=	dilution factor (see paragraph 5.4.1.)

If applicable, the concentration of NMHC and CH4 shall be calculated by either of the methods shown in paragraph 3.3.4. of Appendix 4 to this annex, as follows:

(a)	GC method (full flow dilution system, only):

cNMHC = cHC – cCH4

(b)	NMC method:

cNMHC=cHC(w/oCutter)×1−EM−cHC(w/Cutter)EE−EM

cCH4=cHC(w/Cutter)−cHC(w/oCutter)1−EEEE−EM

where:

cHC(w/Cutter)	=	HC concentration with the sample gas flowing through the NMC

cHC(w/oCutter)	=	HC concentration with the sample gas bypassing the NMC

5.4.1.	Determination of the background corrected concentrations (full flow dilution system, only)

The average background concentration of the gaseous pollutants in the dilution air shall be subtracted from measured concentrations to get the net concentrations of the pollutants.  The average values of the background concentrations can be determined by the sample bag method or by continuous measurement with integration.  The following formula shall be used.

c	=	ce - cd x (1 - (1/D))

where:

ce	=	concentration of the respective pollutant measured in the diluted exhaust gas, ppm

cd	=	concentration of the respective pollutant measured in the dilution air, ppm

D	=	dilution factor

The dilution factor shall be calculated as follows:

(a)	for diesel and LPG fueled gas engines

(b)	for NG fueled gas engines

D	=	FScCO2,e+cNMHC,e+cCO,e×10−4

where:

cCO2	=	concentration of CO2 in the diluted exhaust gas, per cent vol

cHC	=	concentration of HC in the diluted exhaust gas, ppm C1

cNMHC	=	concentration of NMHC in the diluted exhaust gas, ppm C1

cCO	=	concentration of CO in the diluted exhaust gas, ppm

FS	=	stoichiometric factor

Concentrations measured on dry basis shall be converted to a wet basis in accordance with paragraph 5.2. of Appendix 1 to this annex.

The stoichiometric factor shall be calculated as follows:

FS	=

where:

,  are the molar ratios referring to a fuel C H O

Alternatively, if the fuel composition is not known, the following stoichiometric factors may be used:

FS (diesel)	=	13.4

FS (LPG)	=	11.6

FS (NG)	=	9.5

5.5.	Calculation of the specific emissions

The emissions (g/kWh) shall be calculated in the following way:

(a)	all components, except NOx:

Mgas=mgasWact

(b)	NOx:

Mgas=mgas×khWact

where:

Wact	=	actual cycle work as determined according to paragraph 3.9.2.

5.5.1.	In case of a periodic exhaust aftertreatment system, the emissions shall be weighted as follows:

where:

n1	=	number of ETC tests between two regenerations;

n2	=	number of ETC during a regeneration (minimum of one ETC test);

Mgas,n2	=	emissions during a regeneration;

Mgas,n1	=	emissions after a regeneration.

6.	CALCULATION OF THE PARTICULATE EMISSION (IF APPLICABLE)

6.1.	Data evaluation

The particulate filter shall be returned to the weighing chamber no later than one hour after completion of the test.  It shall be conditioned in a partially covered petri dish, which is protected against dust contamination, for at least one hour, but not more than 80 hours, and then weighed.  The gross weight of the filters shall be recorded and the tare weight subtracted, which results in the particulate sample mass mf.  For the evaluation of the particulate concentration, the total sample mass (msep) through the filters over the test cycle shall be recorded.

If background correction is to be applied, the dilution air mass (md) through the filter and the particulate mass (mf, d) shall be recorded.

6.2.	Calculation of the mass flow

6.2.1.	Full flow dilution system

The particulate mass (g/test) shall be calculated as follows:

where:

mf	=	particulate mass sampled over the cycle, mg

msep	=	mass of diluted exhaust gas passing the particulate collection filters, kg

med	=	mass of diluted exhaust gas over the cycle, kg

If a double dilution system is used, the mass of the secondary dilution air shall be subtracted from the total mass of the double diluted exhaust gas sampled through the particulate filters.

msep	=	mset - mssd

where:

mset	=	mass of double diluted exhaust gas through particulate filter, kg

mssd	=	mass of secondary dilution air, kg

If the particulate background level of the dilution air is determined in accordance with paragraph 3.4., the particulate mass may be background corrected.  In this case, the particulate mass (g/test) shall be calculated as follows:

mPT = mfmsep−mdmf,d1-1Dmed1,000

where:

mPT, msep, med	=	see above

md	=	mass of primary dilution air sampled by background particulate sampler, kg

mf,d	=	mass of the collected background particulates of the primary dilution air, mg

D	=	dilution factor as determined in paragraph 5.4.1.

6.2.2.	Partial flow dilution system

The mass of particulates (g/test) shall be calculated by either of the following methods:

(a)

mPT =  mmmedf1,000

where:

mf	=	particulate mass sampled over the cycle, mg

msep	=	mass of diluted exhaust gas passing the particulate collection filters, kg

medf	=	mass of equivalent diluted exhaust gas over the cycle, kg

The total mass of equivalent diluted exhaust gas mass over the cycle shall be determined as follows:

medf	= i=1i=nqmedf,i×1f

qmedf,i	= qmew,i x rd,i

rd,i	= qmdew,,iqmdew,,i−qmdw,,i

where:

qmedf,i	=	instantaneous equivalent diluted exhaust mass flow rate, kg/s

qmew,i	=	instantaneous exhaust mass flow rate, kg/s

rd,i	=	instantaneous dilution ratio

qmdew,i	=	instantaneous diluted exhaust mass flow rate through dilution tunnel, kg/s

qmdw,i	=	instantaneous dilution air mass flow rate, kg/s

f	=	data sampling rate, Hz

n	=	number of measurements

(b)

mPT = mf / (rs x 1,000)

where:

mf	=	particulate mass sampled over the cycle, mg

rs	=	average sample ratio over the test cycle

with:

rs	=	msemew×msepmsed

where:

mse	=	sample mass over the cycle, kg

mew	=	total exhaust mass flow over the cycle, kg

msep	=	mass of diluted exhaust gas passing the particulate collection filters, kg

msed	=	mass of diluted exhaust gas passing the dilution tunnel, kg

NOTE:	In case of the total sampling type system, msep and Msed are identical

6.3.	Calculation of the Specific Emission

The particulate emission (g/kWh) shall be calculated in the following way:

MPT=mPTWact

where:

Wact	=	actual cycle work as determined according to paragraph 3.9.2., kWh.

6.3.1	In case of a periodic regeneration aftertreatment system, the emissions shall be weighted as follows:

where:

n1	=	number of ETC tests between two regeneration events;

n2	=	number of ETC tests during a regeneration (minimum of one ETC);

=	emissions during a regeneration;

=	emissions outside a regeneration.

Appendix 3

ETC ENGINE DYNAMOMETER SCHEDULE

| Time (s) | Norm. speed (per cent) | Norm. torque (per cent) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0 | 0 |
| 3 | 0 | 0 |
| 4 | 0 | 0 |
| 5 | 0 | 0 |
| 6 | 0 | 0 |
| 7 | 0 | 0 |
| 8 | 0 | 0 |
| 9 | 0 | 0 |
| 10 | 0 | 0 |
| 11 | 0 | 0 |
| 12 | 0 | 0 |
| 13 | 0 | 0 |
| 14 | 0 | 0 |
| 15 | 0 | 0 |
| 16 | 0.1 | 1.5 |
| 17 | 23.1 | 21.5 |
| 18 | 12.6 | 28.5 |
| 19 | 21.8 | 71 |
| 20 | 19.7 | 76.8 |
| 21 | 54.6 | 80.9 |
| 22 | 71.3 | 4.9 |
| 23 | 55.9 | 18.1 |
| 24 | 72 | 85.4 |
| 25 | 86.7 | 61.8 |
| 26 | 51.7 | 0 |
| 27 | 53.4 | 48.9 |
| 28 | 34.2 | 87.6 |
| 29 | 45.5 | 92.7 |
| 30 | 54.6 | 99.5 |
| 31 | 64.5 | 96.8 |
| 32 | 71.7 | 85.4 |
| 33 | 79.4 | 54.8 |
| 34 | 89.7 | 99.4 |
| 35 | 57.4 | 0 |
| 36 | 59.7 | 30.6 |
| 37 | 90.1 | "m" |
| 38 | 82.9 | "m" |
| 39 | 51.3 | "m" |
| 40 | 28.5 | "m" |
| 41 | 29.3 | "m" |
| 42 | 26.7 | "m" |
| 43 | 20.4 | "m" |
| 44 | 14.1 | 0 |
| 45 | 6.5 | 0 |
| 46 | 0 | 0 |
| 47 | 0 | 0 |
| 48 | 0 | 0 |
| 49 | 0 | 0 |
| 50 | 0 | 0 |
| 51 | 0 | 0 |
| 52 | 0 | 0 |
| 53 | 0 | 0 |
| 54 | 0 | 0 |
| 55 | 0 | 0 |
| 56 | 0 | 0 |
| 57 | 0 | 0 |
| 58 | 0 | 0 |
| 59 | 0 | 0 |
| 60 | 0 | 0 |
| 61 | 0 | 0 |
| 62 | 25.5 | 11.1 |
| 63 | 28.5 | 20.9 |
| 64 | 32 | 73.9 |
| 65 | 4 | 82.3 |
| 66 | 34.5 | 80.4 |
| 67 | 64.1 | 86 |
| 68 | 58 | 0 |
| 69 | 50.3 | 83.4 |
| 70 | 66.4 | 99.1 |
| 71 | 81.4 | 99.6 |
| 72 | 88.7 | 73.4 |
| 73 | 52.5 | 0 |
| 74 | 46.4 | 58.5 |
| 75 | 48.6 | 90.9 |
| 76 | 55.2 | 99.4 |
| 77 | 62.3 | 99 |
| 78 | 68.4 | 91.5 |
| 79 | 74.5 | 73.7 |
| 80 | 38 | 0 |
| 81 | 41.8 | 89.6 |
| 82 | 47.1 | 99.2 |
| 83 | 52.5 | 99.8 |
| 84 | 56.9 | 80.8 |
| 85 | 58.3 | 11.8 |
| 86 | 56.2 | "m" |
| 87 | 52 | "m" |
| 88 | 43.3 | "m" |
| 89 | 36.1 | "m" |
| 90 | 27.6 | "m" |
| 91 | 21.1 | "m" |
| 92 | 8 | 0 |
| 93 | 0 | 0 |
| 94 | 0 | 0 |
| 95 | 0 | 0 |
| 96 | 0 | 0 |
| 97 | 0 | 0 |
| 98 | 0 | 0 |
| 99 | 0 | 0 |
| 100 | 0 | 0 |
| 101 | 0 | 0 |
| 102 | 0 | 0 |
| 103 | 0 | 0 |
| 104 | 0 | 0 |
| 105 | 0 | 0 |
| 106 | 0 | 0 |
| 107 | 0 | 0 |
| 108 | 11.6 | 14.8 |
| 109 | 0 | 0 |
| 110 | 27.2 | 74.8 |
| 111 | 17 | 76.9 |
| 112 | 36 | 78 |
| 113 | 59.7 | 86 |
| 114 | 80.8 | 17.9 |
| 115 | 49.7 | 0 |
| 116 | 65.6 | 86 |
| 117 | 78.6 | 72.2 |
| 118 | 64.9 | "m" |
| 119 | 44.3 | "m" |
| 120 | 51.4 | 83.4 |
| 121 | 58.1 | 97 |
| 122 | 69.3 | 99.3 |
| 123 | 72 | 20.8 |
| 124 | 72.1 | "m" |
| 125 | 65.3 | "m" |
| 126 | 64 | "m" |
| 127 | 59.7 | "m" |
| 128 | 52.8 | "m" |
| 129 | 45.9 | "m" |
| 130 | 38.7 | "m" |
| 131 | 32.4 | "m" |
| 132 | 27 | "m" |
| 133 | 21.7 | "m" |
| 134 | 19.1 | 0.4 |
| 135 | 34.7 | 14 |
| 136 | 16.4 | 48.6 |
| 137 | 0 | 11.2 |
| 138 | 1.2 | 2.1 |
| 139 | 30.1 | 19.3 |
| 140 | 30 | 73.9 |
| 141 | 54.4 | 74.4 |
| 142 | 77.2 | 55.6 |
| 143 | 58.1 | 0 |
| 144 | 45 | 82.1 |
| 145 | 68.7 | 98.1 |
| 146 | 85.7 | 67.2 |
| 147 | 60.2 | 0 |
| 148 | 59.4 | 98 |
| 149 | 72.7 | 99.6 |
| 150 | 79.9 | 45 |
| 151 | 44.3 | 0 |
| 152 | 41.5 | 84.4 |
| 153 | 56.2 | 98.2 |
| 154 | 65.7 | 99.1 |
| 155 | 74.4 | 84.7 |
| 156 | 54.4 | 0 |
| 157 | 47.9 | 89.7 |
| 158 | 54.5 | 99.5 |
| 159 | 62.7 | 96.8 |
| 160 | 62.3 | 0 |
| 161 | 46.2 | 54.2 |
| 162 | 44.3 | 83.2 |
| 163 | 48.2 | 13.3 |
| 164 | 51 | "m" |
| 165 | 50 | "m" |
| 166 | 49.2 | "m" |
| 167 | 49.3 | "m" |
| 168 | 49.9 | "m" |
| 169 | 51.6 | "m" |
| 170 | 49.7 | "m" |
| 171 | 48.5 | "m" |
| 172 | 50.3 | 72.5 |
| 173 | 51.1 | 84.5 |
| 174 | 54.6 | 64.8 |
| 175 | 56.6 | 76.5 |
| 176 | 58 | "m" |
| 177 | 53.6 | "m" |
| 178 | 40.8 | "m" |
| 179 | 32.9 | "m" |
| 180 | 26.3 | "m" |
| 181 | 20.9 | "m" |
| 182 | 10 | 0 |
| 183 | 0 | 0 |
| 184 | 0 | 0 |
| 185 | 0 | 0 |
| 186 | 0 | 0 |
| 187 | 0 | 0 |
| 188 | 0 | 0 |
| 189 | 0 | 0 |
| 190 | 0 | 0 |
| 191 | 0 | 0 |
| 192 | 0 | 0 |
| 193 | 0 | 0 |
| 194 | 0 | 0 |
| 195 | 0 | 0 |
| 196 | 0 | 0 |
| 197 | 0 | 0 |
| 198 | 0 | 0 |
| 199 | 0 | 0 |
| 200 | 0 | 0 |
| 201 | 0 | 0 |
| 202 | 0 | 0 |
| 203 | 0 | 0 |
| 204 | 0 | 0 |
| 205 | 0 | 0 |
| 206 | 0 | 0 |
| 207 | 0 | 0 |
| 208 | 0 | 0 |
| 209 | 0 | 0 |
| 210 | 0 | 0 |
| 211 | 0 | 0 |
| 212 | 0 | 0 |
| 213 | 0 | 0 |
| 214 | 0 | 0 |
| 215 | 0 | 0 |
| 216 | 0 | 0 |
| 217 | 0 | 0 |
| 218 | 0 | 0 |
| 219 | 0 | 0 |
| 220 | 0 | 0 |
| 221 | 0 | 0 |
| 222 | 0 | 0 |
| 223 | 0 | 0 |
| 224 | 0 | 0 |
| 225 | 21.2 | 62.7 |
| 226 | 30.8 | 75.1 |
| 227 | 5.9 | 82.7 |
| 228 | 34.6 | 80.3 |
| 229 | 59.9 | 87 |
| 230 | 84.3 | 86.2 |
| 231 | 68.7 | "m" |
| 232 | 43.6 | "m" |
| 233 | 41.5 | 85.4 |
| 234 | 49.9 | 94.3 |
| 235 | 60.8 | 99 |
| 236 | 70.2 | 99.4 |
| 237 | 81.1 | 92.4 |
| 238 | 49.2 | 0 |
| 239 | 56 | 86.2 |
| 240 | 56.2 | 99.3 |
| 241 | 61.7 | 99 |
| 242 | 69.2 | 99.3 |
| 243 | 74.1 | 99.8 |
| 244 | 72.4 | 8.4 |
| 245 | 71.3 | 0 |
| 246 | 71.2 | 9.1 |
| 247 | 67.1 | "m" |
| 248 | 65.5 | "m" |
| 249 | 64.4 | "m" |
| 250 | 62.9 | 25.6 |
| 251 | 62.2 | 35.6 |
| 252 | 62.9 | 24.4 |
| 253 | 58.8 | "m" |
| 254 | 56.9 | "m" |
| 255 | 54.5 | "m" |
| 256 | 51.7 | 17 |
| 257 | 56.2 | 78.7 |
| 258 | 59.5 | 94.7 |
| 259 | 65.5 | 99.1 |
| 260 | 71.2 | 99.5 |
| 261 | 76.6 | 99.9 |
| 262 | 79 | 0 |
| 263 | 52.9 | 97.5 |
| 264 | 53.1 | 99.7 |
| 265 | 59 | 99.1 |
| 266 | 62.2 | 99 |
| 267 | 65 | 99.1 |
| 268 | 69 | 83.1 |
| 269 | 69.9 | 28.4 |
| 270 | 70.6 | 12.5 |
| 271 | 68.9 | 8.4 |
| 272 | 69.8 | 9.1 |
| 273 | 69.6 | 7 |
| 274 | 65.7 | "m" |
| 275 | 67.1 | "m" |
| 276 | 66.7 | "m" |
| 277 | 65.6 | "m" |
| 278 | 64.5 | "m" |
| 279 | 62.9 | "m" |
| 280 | 59.3 | "m" |
| 281 | 54.1 | "m" |
| 282 | 51.3 | "m" |
| 283 | 47.9 | "m" |
| 284 | 43.6 | "m" |
| 285 | 39.4 | "m" |
| 286 | 34.7 | "m" |
| 287 | 29.8 | "m" |
| 288 | 20.9 | 73.4 |
| 289 | 36.9 | "m" |
| 290 | 35.5 | "m" |
| 291 | 20.9 | "m" |
| 292 | 49.7 | 11.9 |
| 293 | 42.5 | "m" |
| 294 | 32 | "m" |
| 295 | 23.6 | "m" |
| 296 | 19.1 | 0 |
| 297 | 15.7 | 73.5 |
| 298 | 25.1 | 76.8 |
| 299 | 34.5 | 81.4 |
| 300 | 44.1 | 87.4 |
| 301 | 52.8 | 98.6 |
| 302 | 63.6 | 99 |
| 303 | 73.6 | 99.7 |
| 304 | 62.2 | "m" |
| 305 | 29.2 | "m" |
| 306 | 46.4 | 22 |
| 307 | 47.3 | 13.8 |
| 308 | 47.2 | 12.5 |
| 309 | 47.9 | 11.5 |
| 310 | 47.8 | 35.5 |
| 311 | 49.2 | 83.3 |
| 312 | 52.7 | 96.4 |
| 313 | 57.4 | 99.2 |
| 314 | 61.8 | 99 |
| 315 | 66.4 | 60.9 |
| 316 | 65.8 | "m" |
| 317 | 59 | "m" |
| 318 | 50.7 | "m" |
| 319 | 41.8 | "m" |
| 320 | 34.7 | "m" |
| 321 | 28.7 | "m" |
| 322 | 25.2 | "m" |
| 323 | 43 | 24.8 |
| 324 | 38.7 | 0 |
| 325 | 48.1 | 31.9 |
| 326 | 40.3 | 61 |
| 327 | 42.4 | 52.1 |
| 328 | 46.4 | 47.7 |
| 329 | 46.9 | 30.7 |
| 330 | 46.1 | 23.1 |
| 331 | 45.7 | 23.2 |
| 332 | 45.5 | 31.9 |
| 333 | 46.4 | 73.6 |
| 334 | 51.3 | 60.7 |
| 335 | 51.3 | 51.1 |
| 336 | 53.2 | 46.8 |
| 337 | 53.9 | 50 |
| 338 | 53.4 | 52.1 |
| 339 | 53.8 | 45.7 |
| 340 | 50.6 | 22.1 |
| 341 | 47.8 | 26 |
| 342 | 41.6 | 17.8 |
| 343 | 38.7 | 29.8 |
| 344 | 35.9 | 71.6 |
| 345 | 34.6 | 47.3 |
| 346 | 34.8 | 80.3 |
| 347 | 35.9 | 87.2 |
| 348 | 38.8 | 90.8 |
| 349 | 41.5 | 94.7 |
| 350 | 47.1 | 99.2 |
| 351 | 53.1 | 99.7 |
| 352 | 46.4 | 0 |
| 353 | 42.5 | 0.7 |
| 354 | 43.6 | 58.6 |
| 355 | 47.1 | 87.5 |
| 356 | 54.1 | 99.5 |
| 357 | 62.9 | 99 |
| 358 | 72.6 | 99.6 |
| 359 | 82.4 | 99.5 |
| 360 | 88 | 99.4 |
| 361 | 46.4 | 0 |
| 362 | 53.4 | 95.2 |
| 363 | 58.4 | 99.2 |
| 364 | 61.5 | 99 |
| 365 | 64.8 | 99 |
| 366 | 68.1 | 99.2 |
| 367 | 73.4 | 99.7 |
| 368 | 73.3 | 29.8 |
| 369 | 73.5 | 14.6 |
| 370 | 68.3 | 0 |
| 371 | 45.4 | 49.9 |
| 372 | 47.2 | 75.7 |
| 373 | 44.5 | 9 |
| 374 | 47.8 | 10.3 |
| 375 | 46.8 | 15.9 |
| 376 | 46.9 | 12.7 |
| 377 | 46.8 | 8.9 |
| 378 | 46.1 | 6.2 |
| 379 | 46.1 | "m" |
| 380 | 45.5 | "m" |
| 381 | 44.7 | "m" |
| 382 | 43.8 | "m" |
| 383 | 41 | "m" |
| 384 | 41.1 | 6.4 |
| 385 | 38 | 6.3 |
| 386 | 35.9 | 0.3 |
| 387 | 33.5 | 0 |
| 388 | 53.1 | 48.9 |
| 389 | 48.3 | "m" |
| 390 | 49.9 | "m" |
| 391 | 48 | "m" |
| 392 | 45.3 | "m" |
| 393 | 41.6 | 3.1 |
| 394 | 44.3 | 79 |
| 395 | 44.3 | 89.5 |
| 396 | 43.4 | 98.8 |
| 397 | 44.3 | 98.9 |
| 398 | 43 | 98.8 |
| 399 | 42.2 | 98.8 |
| 400 | 42.7 | 98.8 |
| 401 | 45 | 99 |
| 402 | 43.6 | 98.9 |
| 403 | 42.2 | 98.8 |
| 404 | 44.8 | 99 |
| 405 | 43.4 | 98.8 |
| 406 | 45 | 99 |
| 407 | 42.2 | 54.3 |
| 408 | 61.2 | 31.9 |
| 409 | 56.3 | 72.3 |
| 410 | 59.7 | 99.1 |
| 411 | 62.3 | 99 |
| 412 | 67.9 | 99.2 |
| 413 | 69.5 | 99.3 |
| 414 | 73.1 | 99.7 |
| 415 | 77.7 | 99.8 |
| 416 | 79.7 | 99.7 |
| 417 | 82.5 | 99.5 |
| 418 | 85.3 | 99.4 |
| 419 | 86.6 | 99.4 |
| 420 | 89.4 | 99.4 |
| 421 | 62.2 | 0 |
| 422 | 52.7 | 96.4 |
| 423 | 50.2 | 99.8 |
| 424 | 49.3 | 99.6 |
| 425 | 52.2 | 99.8 |
| 426 | 51.3 | 100 |
| 427 | 51.3 | 100 |
| 428 | 51.1 | 100 |
| 429 | 51.1 | 100 |
| 430 | 51.8 | 99.9 |
| 431 | 51.3 | 100 |
| 432 | 51.1 | 100 |
| 433 | 51.3 | 100 |
| 434 | 52.3 | 99.8 |
| 435 | 52.9 | 99.7 |
| 436 | 53.8 | 99.6 |
| 437 | 51.7 | 99.9 |
| 438 | 53.5 | 99.6 |
| 439 | 52 | 99.8 |
| 440 | 51.7 | 99.9 |
| 441 | 53.2 | 99.7 |
| 442 | 54.2 | 99.5 |
| 443 | 55.2 | 99.4 |
| 444 | 53.8 | 99.6 |
| 445 | 53.1 | 99.7 |
| 446 | 55 | 99.4 |
| 447 | 57 | 99.2 |
| 448 | 61.5 | 99 |
| 449 | 59.4 | 5.7 |
| 450 | 59 | 0 |
| 451 | 57.3 | 59.8 |
| 452 | 64.1 | 99 |
| 453 | 70.9 | 90.5 |
| 454 | 58 | 0 |
| 455 | 41.5 | 59.8 |
| 456 | 44.1 | 92.6 |
| 457 | 46.8 | 99.2 |
| 458 | 47.2 | 99.3 |
| 459 | 51 | 100 |
| 460 | 53.2 | 99.7 |
| 461 | 53.1 | 99.7 |
| 462 | 55.9 | 53.1 |
| 463 | 53.9 | 13.9 |
| 464 | 52.5 | "m" |
| 465 | 51.7 | "m" |
| 466 | 51.5 | 52.2 |
| 467 | 52.8 | 80 |
| 468 | 54.9 | 95 |
| 469 | 57.3 | 99.2 |
| 470 | 60.7 | 99.1 |
| 471 | 62.4 | "m" |
| 472 | 60.1 | "m" |
| 473 | 53.2 | "m" |
| 474 | 44 | "m" |
| 475 | 35.2 | "m" |
| 476 | 30.5 | "m" |
| 477 | 26.5 | "m" |
| 478 | 22.5 | "m" |
| 479 | 20.4 | "m" |
| 480 | 19.1 | "m" |
| 481 | 19.1 | "m" |
| 482 | 13.4 | "m" |
| 483 | 6.7 | "m" |
| 484 | 3.2 | "m" |
| 485 | 14.3 | 63.8 |
| 486 | 34.1 | 0 |
| 487 | 23.9 | 75.7 |
| 488 | 31.7 | 79.2 |
| 489 | 32.1 | 19.4 |
| 490 | 35.9 | 5.8 |
| 491 | 36.6 | 0.8 |
| 492 | 38.7 | "m" |
| 493 | 38.4 | "m" |
| 494 | 39.4 | "m" |
| 495 | 39.7 | "m" |
| 496 | 40.5 | "m" |
| 497 | 40.8 | "m" |
| 498 | 39.7 | "m" |
| 499 | 39.2 | "m" |
| 500 | 38.7 | "m" |
| 501 | 32.7 | "m" |
| 502 | 30.1 | "m" |
| 503 | 21.9 | "m" |
| 504 | 12.8 | 0 |
| 505 | 0 | 0 |
| 506 | 0 | 0 |
| 507 | 0 | 0 |
| 508 | 0 | 0 |
| 509 | 0 | 0 |
| 510 | 0 | 0 |
| 511 | 0 | 0 |
| 512 | 0 | 0 |
| 513 | 0 | 0 |
| 514 | 30.5 | 25.6 |
| 515 | 19.7 | 56.9 |
| 516 | 16.3 | 45.1 |
| 517 | 27.2 | 4.6 |
| 518 | 21.7 | 1.3 |
| 519 | 29.7 | 28.6 |
| 520 | 36.6 | 73.7 |
| 521 | 61.3 | 59.5 |
| 522 | 40.8 | 0 |
| 523 | 36.6 | 27.8 |
| 524 | 39.4 | 80.4 |
| 525 | 51.3 | 88.9 |
| 526 | 58.5 | 11.1 |
| 527 | 60.7 | "m" |
| 528 | 54.5 | "m" |
| 529 | 51.3 | "m" |
| 530 | 45.5 | "m" |
| 531 | 40.8 | "m" |
| 532 | 38.9 | "m" |
| 533 | 36.6 | "m" |
| 534 | 36.1 | 72.7 |
| 535 | 44.8 | 78.9 |
| 536 | 51.6 | 91.1 |
| 537 | 59.1 | 99.1 |
| 538 | 66 | 99.1 |
| 539 | 75.1 | 99.9 |
| 540 | 81 | 8 |
| 541 | 39.1 | 0 |
| 542 | 53.8 | 89.7 |
| 543 | 59.7 | 99.1 |
| 544 | 64.8 | 99 |
| 545 | 70.6 | 96.1 |
| 546 | 72.6 | 19.6 |
| 547 | 72 | 6.3 |
| 548 | 68.9 | 0.1 |
| 549 | 67.7 | "m" |
| 550 | 66.8 | "m" |
| 551 | 64.3 | 16.9 |
| 552 | 64.9 | 7 |
| 553 | 63.6 | 12.5 |
| 554 | 63 | 7.7 |
| 555 | 64.4 | 38.2 |
| 556 | 63 | 11.8 |
| 557 | 63.6 | 0 |
| 558 | 63.3 | 5 |
| 559 | 60.1 | 9.1 |
| 560 | 61 | 8.4 |
| 561 | 59.7 | 0.9 |
| 562 | 58.7 | "m" |
| 563 | 56 | "m" |
| 564 | 53.9 | "m" |
| 565 | 52.1 | "m" |
| 566 | 49.9 | "m" |
| 567 | 46.4 | "m" |
| 568 | 43.6 | "m" |
| 569 | 40.8 | "m" |
| 570 | 37.5 | "m" |
| 571 | 27.8 | "m" |
| 572 | 17.1 | 0.6 |
| 573 | 12.2 | 0.9 |
| 574 | 11.5 | 1.1 |
| 575 | 8.7 | 0.5 |
| 576 | 8 | 0.9 |
| 577 | 5.3 | 0.2 |
| 578 | 4 | 0 |
| 579 | 3.9 | 0 |
| 580 | 0 | 0 |
| 581 | 0 | 0 |
| 582 | 0 | 0 |
| 583 | 0 | 0 |
| 584 | 0 | 0 |
| 585 | 0 | 0 |
| 586 | 0 | 0 |
| 587 | 8.7 | 22.8 |
| 588 | 16.2 | 49.4 |
| 589 | 23.6 | 56 |
| 590 | 21.1 | 56.1 |
| 591 | 23.6 | 56 |
| 592 | 46.2 | 68.8 |
| 593 | 68.4 | 61.2 |
| 594 | 58.7 | "m" |
| 595 | 31.6 | "m" |
| 596 | 19.9 | 8.8 |
| 597 | 32.9 | 70.2 |
| 598 | 43 | 79 |
| 599 | 57.4 | 98.9 |
| 600 | 72.1 | 73.8 |
| 601 | 53 | 0 |
| 602 | 48.1 | 86 |
| 603 | 56.2 | 99 |
| 604 | 65.4 | 98.9 |
| 605 | 72.9 | 99.7 |
| 606 | 67.5 | "m" |
| 607 | 39 | "m" |
| 608 | 41.9 | 38.1 |
| 609 | 44.1 | 80.4 |
| 610 | 46.8 | 99.4 |
| 611 | 48.7 | 99.9 |
| 612 | 50.5 | 99.7 |
| 613 | 52.5 | 90.3 |
| 614 | 51 | 1.8 |
| 615 | 50 | "m" |
| 616 | 49.1 | "m" |
| 617 | 47 | "m" |
| 618 | 43.1 | "m" |
| 619 | 39.2 | "m" |
| 620 | 40.6 | 0.5 |
| 621 | 41.8 | 53.4 |
| 622 | 44.4 | 65.1 |
| 623 | 48.1 | 67.8 |
| 624 | 53.8 | 99.2 |
| 625 | 58.6 | 98.9 |
| 626 | 63.6 | 98.8 |
| 627 | 68.5 | 99.2 |
| 628 | 72.2 | 89.4 |
| 629 | 77.1 | 0 |
| 630 | 57.8 | 79.1 |
| 631 | 60.3 | 98.8 |
| 632 | 61.9 | 98.8 |
| 633 | 63.8 | 98.8 |
| 634 | 64.7 | 98.9 |
| 635 | 65.4 | 46.5 |
| 636 | 65.7 | 44.5 |
| 637 | 65.6 | 3.5 |
| 638 | 49.1 | 0 |
| 639 | 50.4 | 73.1 |
| 640 | 50.5 | "m" |
| 641 | 51 | "m" |
| 642 | 49.4 | "m" |
| 643 | 49.2 | "m" |
| 644 | 48.6 | "m" |
| 645 | 47.5 | "m" |
| 646 | 46.5 | "m" |
| 647 | 46 | 11.3 |
| 648 | 45.6 | 42.8 |
| 649 | 47.1 | 83 |
| 650 | 46.2 | 99.3 |
| 651 | 47.9 | 99.7 |
| 652 | 49.5 | 99.9 |
| 653 | 50.6 | 99.7 |
| 654 | 51 | 99.6 |
| 655 | 53 | 99.3 |
| 656 | 54.9 | 99.1 |
| 657 | 55.7 | 99 |
| 658 | 56 | 99 |
| 659 | 56.1 | 9.3 |
| 660 | 55.6 | "m" |
| 661 | 55.4 | "m" |
| 662 | 54.9 | 51.3 |
| 663 | 54.9 | 59.8 |
| 664 | 54 | 39.3 |
| 665 | 53.8 | "m" |
| 666 | 52 | "m" |
| 667 | 50.4 | "m" |
| 668 | 50.6 | 0 |
| 669 | 49.3 | 41.7 |
| 670 | 50 | 73.2 |
| 671 | 50.4 | 99.7 |
| 672 | 51.9 | 99.5 |
| 673 | 53.6 | 99.3 |
| 674 | 54.6 | 99.1 |
| 675 | 56 | 99 |
| 676 | 55.8 | 99 |
| 677 | 58.4 | 98.9 |
| 678 | 59.9 | 98.8 |
| 679 | 60.9 | 98.8 |
| 680 | 63 | 98.8 |
| 681 | 64.3 | 98.9 |
| 682 | 64.8 | 64 |
| 683 | 65.9 | 46.5 |
| 684 | 66.2 | 28.7 |
| 685 | 65.2 | 1.8 |
| 686 | 65 | 6.8 |
| 687 | 63.6 | 53.6 |
| 688 | 62.4 | 82.5 |
| 689 | 61.8 | 98.8 |
| 690 | 59.8 | 98.8 |
| 691 | 59.2 | 98.8 |
| 692 | 59.7 | 98.8 |
| 693 | 61.2 | 98.8 |
| 694 | 62.2 | 49.4 |
| 695 | 62.8 | 37.2 |
| 696 | 63.5 | 46.3 |
| 697 | 64.7 | 72.3 |
| 698 | 64.7 | 72.3 |
| 699 | 65.4 | 77.4 |
| 700 | 66.1 | 69.3 |
| 701 | 64.3 | "m" |
| 702 | 64.3 | "m" |
| 703 | 63 | "m" |
| 704 | 62.2 | "m" |
| 705 | 61.6 | "m" |
| 706 | 62.4 | "m" |
| 707 | 62.2 | "m" |
| 708 | 61 | "m" |
| 709 | 58.7 | "m" |
| 710 | 55.5 | "m" |
| 711 | 51.7 | "m" |
| 712 | 49.2 | "m" |
| 713 | 48.8 | 40.4 |
| 714 | 47.9 | "m" |
| 715 | 46.2 | "m" |
| 716 | 45.6 | 9.8 |
| 717 | 45.6 | 34.5 |
| 718 | 45.5 | 37.1 |
| 719 | 43.8 | "m" |
| 720 | 41.9 | "m" |
| 721 | 41.3 | "m" |
| 722 | 41.4 | "m" |
| 723 | 41.2 | "m" |
| 724 | 41.8 | "m" |
| 725 | 41.8 | "m" |
| 726 | 43.2 | 17.4 |
| 727 | 45 | 29 |
| 728 | 44.2 | "m" |
| 729 | 43.9 | "m" |
| 730 | 38 | 10.7 |
| 731 | 56.8 | "m" |
| 732 | 57.1 | "m" |
| 733 | 52 | "m" |
| 734 | 44.4 | "m" |
| 735 | 40.2 | "m" |
| 736 | 39.2 | 16.5 |
| 737 | 38.9 | 73.2 |
| 738 | 39.9 | 89.8 |
| 739 | 42.3 | 98.6 |
| 740 | 43.7 | 98.8 |
| 741 | 45.5 | 99.1 |
| 742 | 45.6 | 99.2 |
| 743 | 48.1 | 99.7 |
| 744 | 49 | 100 |
| 745 | 49.8 | 99.9 |
| 746 | 49.8 | 99.9 |
| 747 | 51.9 | 99.5 |
| 748 | 52.3 | 99.4 |
| 749 | 53.3 | 99.3 |
| 750 | 52.9 | 99.3 |
| 751 | 54.3 | 99.2 |
| 752 | 55.5 | 99.1 |
| 753 | 56.7 | 99 |
| 754 | 61.7 | 98.8 |
| 755 | 64.3 | 47.4 |
| 756 | 64.7 | 1.8 |
| 757 | 66.2 | "m" |
| 758 | 49.1 | "m" |
| 759 | 52.1 | 46 |
| 760 | 52.6 | 61 |
| 761 | 52.9 | 0 |
| 762 | 52.3 | 20.4 |
| 763 | 54.2 | 56.7 |
| 764 | 55.4 | 59.8 |
| 765 | 56.1 | 49.2 |
| 766 | 56.8 | 33.7 |
| 767 | 57.2 | 96 |
| 768 | 58.6 | 98.9 |
| 769 | 59.5 | 98.8 |
| 770 | 61.2 | 98.8 |
| 771 | 62.1 | 98.8 |
| 772 | 62.7 | 98.8 |
| 773 | 62.8 | 98.8 |
| 774 | 64 | 98.9 |
| 775 | 63.2 | 46.3 |
| 776 | 62.4 | "m" |
| 777 | 60.3 | "m" |
| 778 | 58.7 | "m" |
| 779 | 57.2 | "m" |
| 780 | 56.1 | "m" |
| 781 | 56 | 9.3 |
| 782 | 55.2 | 26.3 |
| 783 | 54.8 | 42.8 |
| 784 | 55.7 | 47.1 |
| 785 | 56.6 | 52.4 |
| 786 | 58 | 50.3 |
| 787 | 58.6 | 20.6 |
| 788 | 58.7 | "m" |
| 789 | 59.3 | "m" |
| 790 | 58.6 | "m" |
| 791 | 60.5 | 9.7 |
| 792 | 59.2 | 9.6 |
| 793 | 59.9 | 9.6 |
| 794 | 59.6 | 9.6 |
| 795 | 59.9 | 6.2 |
| 796 | 59.9 | 9.6 |
| 797 | 60.5 | 13.1 |
| 798 | 60.3 | 20.7 |
| 799 | 59.9 | 31 |
| 800 | 60.5 | 42 |
| 801 | 61.5 | 52.5 |
| 802 | 60.9 | 51.4 |
| 803 | 61.2 | 57.7 |
| 804 | 62.8 | 98.8 |
| 805 | 63.4 | 96.1 |
| 806 | 64.6 | 45.4 |
| 807 | 64.1 | 5 |
| 808 | 63 | 3.2 |
| 809 | 62.7 | 14.9 |
| 810 | 63.5 | 35.8 |
| 811 | 64.1 | 73.3 |
| 812 | 64.3 | 37.4 |
| 813 | 64.1 | 21 |
| 814 | 63.7 | 21 |
| 815 | 62.9 | 18 |
| 816 | 62.4 | 32.7 |
| 817 | 61.7 | 46.2 |
| 818 | 59.8 | 45.1 |
| 819 | 57.4 | 43.9 |
| 820 | 54.8 | 42.8 |
| 821 | 54.3 | 65.2 |
| 822 | 52.9 | 62.1 |
| 823 | 52.4 | 30.6 |
| 824 | 50.4 | "m" |
| 825 | 48.6 | "m" |
| 826 | 47.9 | "m" |
| 827 | 46.8 | "m" |
| 828 | 46.9 | 9.4 |
| 829 | 49.5 | 41.7 |
| 830 | 50.5 | 37.8 |
| 831 | 52.3 | 20.4 |
| 832 | 54.1 | 30.7 |
| 833 | 56.3 | 41.8 |
| 834 | 58.7 | 26.5 |
| 835 | 57.3 | "m" |
| 836 | 59 | "m" |
| 837 | 59.8 | "m" |
| 838 | 60.3 | "m" |
| 839 | 61.2 | "m" |
| 840 | 61.8 | "m" |
| 841 | 62.5 | "m" |
| 842 | 62.4 | "m" |
| 843 | 61.5 | "m" |
| 844 | 63.7 | "m" |
| 845 | 61.9 | "m" |
| 846 | 61.6 | 29.7 |
| 847 | 60.3 | "m" |
| 848 | 59.2 | "m" |
| 849 | 57.3 | "m" |
| 850 | 52.3 | "m" |
| 851 | 49.3 | "m" |
| 852 | 47.3 | "m" |
| 853 | 46.3 | 38.8 |
| 854 | 46.8 | 35.1 |
| 855 | 46.6 | "m" |
| 856 | 44.3 | "m" |
| 857 | 43.1 | "m" |
| 858 | 42.4 | 2.1 |
| 859 | 41.8 | 2.4 |
| 860 | 43.8 | 68.8 |
| 861 | 44.6 | 89.2 |
| 862 | 46 | 99.2 |
| 863 | 46.9 | 99.4 |
| 864 | 47.9 | 99.7 |
| 865 | 50.2 | 99.8 |
| 866 | 51.2 | 99.6 |
| 867 | 52.3 | 99.4 |
| 868 | 53 | 99.3 |
| 869 | 54.2 | 99.2 |
| 870 | 55.5 | 99.1 |
| 871 | 56.7 | 99 |
| 872 | 57.3 | 98.9 |
| 873 | 58 | 98.9 |
| 874 | 60.5 | 31.1 |
| 875 | 60.2 | "m" |
| 876 | 60.3 | "m" |
| 877 | 60.5 | 6.3 |
| 878 | 61.4 | 19.3 |
| 879 | 60.3 | 1.2 |
| 880 | 60.5 | 2.9 |
| 881 | 61.2 | 34.1 |
| 882 | 61.6 | 13.2 |
| 883 | 61.5 | 16.4 |
| 884 | 61.2 | 16.4 |
| 885 | 61.3 | "m" |
| 886 | 63.1 | "m" |
| 887 | 63.2 | 4.8 |
| 888 | 62.3 | 22.3 |
| 889 | 62 | 38.5 |
| 890 | 61.6 | 29.6 |
| 891 | 61.6 | 26.6 |
| 892 | 61.8 | 28.1 |
| 893 | 62 | 29.6 |
| 894 | 62 | 16.3 |
| 895 | 61.1 | "m" |
| 896 | 61.2 | "m" |
| 897 | 60.7 | 19.2 |
| 898 | 60.7 | 32.5 |
| 899 | 60.9 | 17.8 |
| 900 | 60.1 | 19.2 |
| 901 | 59.3 | 38.2 |
| 902 | 59.9 | 45 |
| 903 | 59.4 | 32.4 |
| 904 | 59.2 | 23.5 |
| 905 | 59.5 | 40.8 |
| 906 | 58.3 | "m" |
| 907 | 58.2 | "m" |
| 908 | 57.6 | "m" |
| 909 | 57.1 | "m" |
| 910 | 57 | 0.6 |
| 911 | 57 | 26.3 |
| 912 | 56.5 | 29.2 |
| 913 | 56.3 | 20.5 |
| 914 | 56.1 | "m" |
| 915 | 55.2 | "m" |
| 916 | 54.7 | 17.5 |
| 917 | 55.2 | 29.2 |
| 918 | 55.2 | 29.2 |
| 919 | 55.9 | 16 |
| 920 | 55.9 | 26.3 |
| 921 | 56.1 | 36.5 |
| 922 | 55.8 | 19 |
| 923 | 55.9 | 9.2 |
| 924 | 55.8 | 21.9 |
| 925 | 56.4 | 42.8 |
| 926 | 56.4 | 38 |
| 927 | 56.4 | 11 |
| 928 | 56.4 | 35.1 |
| 929 | 54 | 7.3 |
| 930 | 53.4 | 5.4 |
| 931 | 52.3 | 27.6 |
| 932 | 52.1 | 32 |
| 933 | 52.3 | 33.4 |
| 934 | 52.2 | 34.9 |
| 935 | 52.8 | 60.1 |
| 936 | 53.7 | 69.7 |
| 937 | 54 | 70.7 |
| 938 | 55.1 | 71.7 |
| 939 | 55.2 | 46 |
| 940 | 54.7 | 12.6 |
| 941 | 52.5 | 0 |
| 942 | 51.8 | 24.7 |
| 943 | 51.4 | 43.9 |
| 944 | 50.9 | 71.1 |
| 945 | 51.2 | 76.8 |
| 946 | 50.3 | 87.5 |
| 947 | 50.2 | 99.8 |
| 948 | 50.9 | 100 |
| 949 | 49.9 | 99.7 |
| 950 | 50.9 | 100 |
| 951 | 49.8 | 99.7 |
| 952 | 50.4 | 99.8 |
| 953 | 50.4 | 99.8 |
| 954 | 49.7 | 99.7 |
| 955 | 51 | 100 |
| 956 | 50.3 | 99.8 |
| 957 | 50.2 | 99.8 |
| 958 | 49.9 | 99.7 |
| 959 | 50.9 | 100 |
| 960 | 50 | 99.7 |
| 961 | 50.2 | 99.8 |
| 962 | 50.2 | 99.8 |
| 963 | 49.9 | 99.7 |
| 964 | 50.4 | 99.8 |
| 965 | 50.2 | 99.8 |
| 966 | 50.3 | 99.8 |
| 967 | 49.9 | 99.7 |
| 968 | 51.1 | 100 |
| 969 | 50.6 | 99.9 |
| 970 | 49.9 | 99.7 |
| 971 | 49.6 | 99.6 |
| 972 | 49.4 | 99.6 |
| 973 | 49 | 99.5 |
| 974 | 49.8 | 99.7 |
| 975 | 50.9 | 100 |
| 976 | 50.4 | 99.8 |
| 977 | 49.8 | 99.7 |
| 978 | 49.1 | 99.5 |
| 979 | 50.4 | 99.8 |
| 980 | 49.8 | 99.7 |
| 981 | 49.3 | 99.5 |
| 982 | 49.1 | 99.5 |
| 983 | 49.9 | 99.7 |
| 984 | 49.1 | 99.5 |
| 985 | 50.4 | 99.8 |
| 986 | 50.9 | 100 |
| 987 | 51.4 | 99.9 |
| 988 | 51.5 | 99.9 |
| 989 | 52.2 | 99.7 |
| 990 | 52.8 | 74.1 |
| 991 | 53.3 | 46 |
| 992 | 53.6 | 36.4 |
| 993 | 53.4 | 33.5 |
| 994 | 53.9 | 58.9 |
| 995 | 55.2 | 73.8 |
| 996 | 55.8 | 52.4 |
| 997 | 55.7 | 9.2 |
| 998 | 55.8 | 2.2 |
| 999 | 56.4 | 33.6 |
| 1000 | 55.4 | "m" |
| 1001 | 55.2 | "m" |
| 1002 | 55.8 | 26.3 |
| 1003 | 55.8 | 23.3 |
| 1004 | 56.4 | 50.2 |
| 1005 | 57.6 | 68.3 |
| 1006 | 58.8 | 90.2 |
| 1007 | 59.9 | 98.9 |
| 1008 | 62.3 | 98.8 |
| 1009 | 63.1 | 74.4 |
| 1010 | 63.7 | 49.4 |
| 1011 | 63.3 | 9.8 |
| 1012 | 48 | 0 |
| 1013 | 47.9 | 73.5 |
| 1014 | 49.9 | 99.7 |
| 1015 | 49.9 | 48.8 |
| 1016 | 49.6 | 2.3 |
| 1017 | 49.9 | "m" |
| 1018 | 49.3 | "m" |
| 1019 | 49.7 | 47.5 |
| 1020 | 49.1 | "m" |
| 1021 | 49.4 | "m" |
| 1022 | 48.3 | "m" |
| 1023 | 49.4 | "m" |
| 1024 | 48.5 | "m" |
| 1025 | 48.7 | "m" |
| 1026 | 48.7 | "m" |
| 1027 | 49.1 | "m" |
| 1028 | 49 | "m" |
| 1029 | 49.8 | "m" |
| 1030 | 48.7 | "m" |
| 1031 | 48.5 | "m" |
| 1032 | 49.3 | 31.3 |
| 1033 | 49.7 | 45.3 |
| 1034 | 48.3 | 44.5 |
| 1035 | 49.8 | 61 |
| 1036 | 49.4 | 64.3 |
| 1037 | 49.8 | 64.4 |
| 1038 | 50.5 | 65.6 |
| 1039 | 50.3 | 64.5 |
| 1040 | 51.2 | 82.9 |
| 1041 | 50.5 | 86 |
| 1042 | 50.6 | 89 |
| 1043 | 50.4 | 81.4 |
| 1044 | 49.9 | 49.9 |
| 1045 | 49.1 | 20.1 |
| 1046 | 47.9 | 24 |
| 1047 | 48.1 | 36.2 |
| 1048 | 47.5 | 34.5 |
| 1049 | 46.9 | 30.3 |
| 1050 | 47.7 | 53.5 |
| 1051 | 46.9 | 61.6 |
| 1052 | 46.5 | 73.6 |
| 1053 | 48 | 84.6 |
| 1054 | 47.2 | 87.7 |
| 1055 | 48.7 | 80 |
| 1056 | 48.7 | 50.4 |
| 1057 | 47.8 | 38.6 |
| 1058 | 48.8 | 63.1 |
| 1059 | 47.4 | 5 |
| 1060 | 47.3 | 47.4 |
| 1061 | 47.3 | 49.8 |
| 1062 | 46.9 | 23.9 |
| 1063 | 46.7 | 44.6 |
| 1064 | 46.8 | 65.2 |
| 1065 | 46.9 | 60.4 |
| 1066 | 46.7 | 61.5 |
| 1067 | 45.5 | "m" |
| 1068 | 45.5 | "m" |
| 1069 | 44.2 | "m" |
| 1070 | 43 | "m" |
| 1071 | 42.5 | "m" |
| 1072 | 41 | "m" |
| 1073 | 39.9 | "m" |
| 1074 | 39.9 | 38.2 |
| 1075 | 40.1 | 48.1 |
| 1076 | 39.9 | 48 |
| 1077 | 39.4 | 59.3 |
| 1078 | 43.8 | 19.8 |
| 1079 | 52.9 | 0 |
| 1080 | 52.8 | 88.9 |
| 1081 | 53.4 | 99.5 |
| 1082 | 54.7 | 99.3 |
| 1083 | 56.3 | 99.1 |
| 1084 | 57.5 | 99 |
| 1085 | 59 | 98.9 |
| 1086 | 59.8 | 98.9 |
| 1087 | 60.1 | 98.9 |
| 1088 | 61.8 | 48.3 |
| 1089 | 61.8 | 55.6 |
| 1090 | 61.7 | 59.8 |
| 1091 | 62 | 55.6 |
| 1092 | 62.3 | 29.6 |
| 1093 | 62 | 19.3 |
| 1094 | 61.3 | 7.9 |
| 1095 | 61.1 | 19.2 |
| 1096 | 61.2 | 43 |
| 1097 | 61.1 | 59.7 |
| 1098 | 61.1 | 98.8 |
| 1099 | 61.3 | 98.8 |
| 1100 | 61.3 | 26.6 |
| 1101 | 60.4 | "m" |
| 1102 | 58.8 | "m" |
| 1103 | 57.7 | "m" |
| 1104 | 56 | "m" |
| 1105 | 54.7 | "m" |
| 1106 | 53.3 | "m" |
| 1107 | 52.6 | 23.2 |
| 1108 | 53.4 | 84.2 |
| 1109 | 53.9 | 99.4 |
| 1110 | 54.9 | 99.3 |
| 1111 | 55.8 | 99.2 |
| 1112 | 57.1 | 99 |
| 1113 | 56.5 | 99.1 |
| 1114 | 58.9 | 98.9 |
| 1115 | 58.7 | 98.9 |
| 1116 | 59.8 | 98.9 |
| 1117 | 61 | 98.8 |
| 1118 | 60.7 | 19.2 |
| 1119 | 59.4 | "m" |
| 1120 | 57.9 | "m" |
| 1121 | 57.6 | "m" |
| 1122 | 56.3 | "m" |
| 1123 | 55 | "m" |
| 1124 | 53.7 | "m" |
| 1125 | 52.1 | "m" |
| 1126 | 51.1 | "m" |
| 1127 | 49.7 | 25.8 |
| 1128 | 49.1 | 46.1 |
| 1129 | 48.7 | 46.9 |
| 1130 | 48.2 | 46.7 |
| 1131 | 48 | 70 |
| 1132 | 48 | 70 |
| 1133 | 47.2 | 67.6 |
| 1134 | 47.3 | 67.6 |
| 1135 | 46.6 | 74.7 |
| 1136 | 47.4 | 13 |
| 1137 | 46.3 | "m" |
| 1138 | 45.4 | "m" |
| 1139 | 45.5 | 24.8 |
| 1140 | 44.8 | 73.8 |
| 1141 | 46.6 | 99 |
| 1142 | 46.3 | 98.9 |
| 1143 | 48.5 | 99.4 |
| 1144 | 49.9 | 99.7 |
| 1145 | 49.1 | 99.5 |
| 1146 | 49.1 | 99.5 |
| 1147 | 51 | 100 |
| 1148 | 51.5 | 99.9 |
| 1149 | 50.9 | 100 |
| 1150 | 51.6 | 99.9 |
| 1151 | 52.1 | 99.7 |
| 1152 | 50.9 | 100 |
| 1153 | 52.2 | 99.7 |
| 1154 | 51.5 | 98.3 |
| 1155 | 51.5 | 47.2 |
| 1156 | 50.8 | 78.4 |
| 1157 | 50.3 | 83 |
| 1158 | 50.3 | 31.7 |
| 1159 | 49.3 | 31.3 |
| 1160 | 48.8 | 21.5 |
| 1161 | 47.8 | 59.4 |
| 1162 | 48.1 | 77.1 |
| 1163 | 48.4 | 87.6 |
| 1164 | 49.6 | 87.5 |
| 1165 | 51 | 81.4 |
| 1166 | 51.6 | 66.7 |
| 1167 | 53.3 | 63.2 |
| 1168 | 55.2 | 62 |
| 1169 | 55.7 | 43.9 |
| 1170 | 56.4 | 30.7 |
| 1171 | 56.8 | 23.4 |
| 1172 | 57 | "m" |
| 1173 | 57.6 | "m" |
| 1174 | 56.9 | "m" |
| 1175 | 56.4 | 4 |
| 1176 | 57 | 23.4 |
| 1177 | 56.4 | 41.7 |
| 1178 | 57 | 49.2 |
| 1179 | 57.7 | 56.6 |
| 1180 | 58.6 | 56.6 |
| 1181 | 58.9 | 64 |
| 1182 | 59.4 | 68.2 |
| 1183 | 58.8 | 71.4 |
| 1184 | 60.1 | 71.3 |
| 1185 | 60.6 | 79.1 |
| 1186 | 60.7 | 83.3 |
| 1187 | 60.7 | 77.1 |
| 1188 | 60 | 73.5 |
| 1189 | 60.2 | 55.5 |
| 1190 | 59.7 | 54.4 |
| 1191 | 59.8 | 73.3 |
| 1192 | 59.8 | 77.9 |
| 1193 | 59.8 | 73.9 |
| 1194 | 60 | 76.5 |
| 1195 | 59.5 | 82.3 |
| 1196 | 59.9 | 82.8 |
| 1197 | 59.8 | 65.8 |
| 1198 | 59 | 48.6 |
| 1199 | 58.9 | 62.2 |
| 1200 | 59.1 | 70.4 |
| 1201 | 58.9 | 62.1 |
| 1202 | 58.4 | 67.4 |
| 1203 | 58.7 | 58.9 |
| 1204 | 58.3 | 57.7 |
| 1205 | 57.5 | 57.8 |
| 1206 | 57.2 | 57.6 |
| 1207 | 57.1 | 42.6 |
| 1208 | 57 | 70.1 |
| 1209 | 56.4 | 59.6 |
| 1210 | 56.7 | 39 |
| 1211 | 55.9 | 68.1 |
| 1212 | 56.3 | 79.1 |
| 1213 | 56.7 | 89.7 |
| 1214 | 56 | 89.4 |
| 1215 | 56 | 93.1 |
| 1216 | 56.4 | 93.1 |
| 1217 | 56.7 | 94.4 |
| 1218 | 56.9 | 94.8 |
| 1219 | 57 | 94.1 |
| 1220 | 57.7 | 94.3 |
| 1221 | 57.5 | 93.7 |
| 1222 | 58.4 | 93.2 |
| 1223 | 58.7 | 93.2 |
| 1224 | 58.2 | 93.7 |
| 1225 | 58.5 | 93.1 |
| 1226 | 58.8 | 86.2 |
| 1227 | 59 | 72.9 |
| 1228 | 58.2 | 59.9 |
| 1229 | 57.6 | 8.5 |
| 1230 | 57.1 | 47.6 |
| 1231 | 57.2 | 74.4 |
| 1232 | 57 | 79.1 |
| 1233 | 56.7 | 67.2 |
| 1234 | 56.8 | 69.1 |
| 1235 | 56.9 | 71.3 |
| 1236 | 57 | 77.3 |
| 1237 | 57.4 | 78.2 |
| 1238 | 57.3 | 70.6 |
| 1239 | 57.7 | 64 |
| 1240 | 57.5 | 55.6 |
| 1241 | 58.6 | 49.6 |
| 1242 | 58.2 | 41.1 |
| 1243 | 58.8 | 40.6 |
| 1244 | 58.3 | 21.1 |
| 1245 | 58.7 | 24.9 |
| 1246 | 59.1 | 24.8 |
| 1247 | 58.6 | "m" |
| 1248 | 58.8 | "m" |
| 1249 | 58.8 | "m" |
| 1250 | 58.7 | "m" |
| 1251 | 59.1 | "m" |
| 1252 | 59.1 | "m" |
| 1253 | 59.4 | "m" |
| 1254 | 60.6 | 2.6 |
| 1255 | 59.6 | "m" |
| 1256 | 60.1 | "m" |
| 1257 | 60.6 | "m" |
| 1258 | 59.6 | 4.1 |
| 1259 | 60.7 | 7.1 |
| 1260 | 60.5 | "m" |
| 1261 | 59.7 | "m" |
| 1262 | 59.6 | "m" |
| 1263 | 59.8 | "m" |
| 1264 | 59.6 | 4.9 |
| 1265 | 60.1 | 5.9 |
| 1266 | 59.9 | 6.1 |
| 1267 | 59.7 | "m" |
| 1268 | 59.6 | "m" |
| 1269 | 59.7 | 22 |
| 1270 | 59.8 | 10.3 |
| 1271 | 59.9 | 10 |
| 1272 | 60.6 | 6.2 |
| 1273 | 60.5 | 7.3 |
| 1274 | 60.2 | 14.8 |
| 1275 | 60.6 | 8.2 |
| 1276 | 60.6 | 5.5 |
| 1277 | 61 | 14.3 |
| 1278 | 61 | 12 |
| 1279 | 61.3 | 34.2 |
| 1280 | 61.2 | 17.1 |
| 1281 | 61.5 | 15.7 |
| 1282 | 61 | 9.5 |
| 1283 | 61.1 | 9.2 |
| 1284 | 60.5 | 4.3 |
| 1285 | 60.2 | 7.8 |
| 1286 | 60.2 | 5.9 |
| 1287 | 60.2 | 5.3 |
| 1288 | 59.9 | 4.6 |
| 1289 | 59.4 | 21.5 |
| 1290 | 59.6 | 15.8 |
| 1291 | 59.3 | 10.1 |
| 1292 | 58.9 | 9.4 |
| 1293 | 58.8 | 9 |
| 1294 | 58.9 | 35.4 |
| 1295 | 58.9 | 30.7 |
| 1296 | 58.9 | 25.9 |
| 1297 | 58.7 | 22.9 |
| 1298 | 58.7 | 24.4 |
| 1299 | 59.3 | 61 |
| 1300 | 60.1 | 56 |
| 1301 | 60.5 | 50.6 |
| 1302 | 59.5 | 16.2 |
| 1303 | 59.7 | 50 |
| 1304 | 59.7 | 31.4 |
| 1305 | 60.1 | 43.1 |
| 1306 | 60.8 | 38.4 |
| 1307 | 60.9 | 40.2 |
| 1308 | 61.3 | 49.7 |
| 1309 | 61.8 | 45.9 |
| 1310 | 62 | 45.9 |
| 1311 | 62.2 | 45.8 |
| 1312 | 62.6 | 46.8 |
| 1313 | 62.7 | 44.3 |
| 1314 | 62.9 | 44.4 |
| 1315 | 63.1 | 43.7 |
| 1316 | 63.5 | 46.1 |
| 1317 | 63.6 | 40.7 |
| 1318 | 64.3 | 49.5 |
| 1319 | 63.7 | 27 |
| 1320 | 63.8 | 15 |
| 1321 | 63.6 | 18.7 |
| 1322 | 63.4 | 8.4 |
| 1323 | 63.2 | 8.7 |
| 1324 | 63.3 | 21.6 |
| 1325 | 62.9 | 19.7 |
| 1326 | 63 | 22.1 |
| 1327 | 63.1 | 20.3 |
| 1328 | 61.8 | 19.1 |
| 1329 | 61.6 | 17.1 |
| 1330 | 61 | 0 |
| 1331 | 61.2 | 22 |
| 1332 | 60.8 | 40.3 |
| 1333 | 61.1 | 34.3 |
| 1334 | 60.7 | 16.1 |
| 1335 | 60.6 | 16.6 |
| 1336 | 60.5 | 18.5 |
| 1337 | 60.6 | 29.8 |
| 1338 | 60.9 | 19.5 |
| 1339 | 60.9 | 22.3 |
| 1340 | 61.4 | 35.8 |
| 1341 | 61.3 | 42.9 |
| 1342 | 61.5 | 31 |
| 1343 | 61.3 | 19.2 |
| 1344 | 61 | 9.3 |
| 1345 | 60.8 | 44.2 |
| 1346 | 60.9 | 55.3 |
| 1347 | 61.2 | 56 |
| 1348 | 60.9 | 60.1 |
| 1349 | 60.7 | 59.1 |
| 1350 | 60.9 | 56.8 |
| 1351 | 60.7 | 58.1 |
| 1352 | 59.6 | 78.4 |
| 1353 | 59.6 | 84.6 |
| 1354 | 59.4 | 66.6 |
| 1355 | 59.3 | 75.5 |
| 1356 | 58.9 | 49.6 |
| 1357 | 59.1 | 75.8 |
| 1358 | 59 | 77.6 |
| 1359 | 59 | 67.8 |
| 1360 | 59 | 56.7 |
| 1361 | 58.8 | 54.2 |
| 1362 | 58.9 | 59.6 |
| 1363 | 58.9 | 60.8 |
| 1364 | 59.3 | 56.1 |
| 1365 | 58.9 | 48.5 |
| 1366 | 59.3 | 42.9 |
| 1367 | 59.4 | 41.4 |
| 1368 | 59.6 | 38.9 |
| 1369 | 59.4 | 32.9 |
| 1370 | 59.3 | 30.6 |
| 1371 | 59.4 | 30 |
| 1372 | 59.4 | 25.3 |
| 1373 | 58.8 | 18.6 |
| 1374 | 59.1 | 18 |
| 1375 | 58.5 | 10.6 |
| 1376 | 58.8 | 10.5 |
| 1377 | 58.5 | 8.2 |
| 1378 | 58.7 | 13.7 |
| 1379 | 59.1 | 7.8 |
| 1380 | 59.1 | 6 |
| 1381 | 59.1 | 6 |
| 1382 | 59.4 | 13.1 |
| 1383 | 59.7 | 22.3 |
| 1384 | 60.7 | 10.5 |
| 1385 | 59.8 | 9.8 |
| 1386 | 60.2 | 8.8 |
| 1387 | 59.9 | 8.7 |
| 1388 | 61 | 9.1 |
| 1389 | 60.6 | 28.2 |
| 1390 | 60.6 | 22 |
| 1391 | 59.6 | 23.2 |
| 1392 | 59.6 | 19 |
| 1393 | 60.6 | 38.4 |
| 1394 | 59.8 | 41.6 |
| 1395 | 60 | 47.3 |
| 1396 | 60.5 | 55.4 |
| 1397 | 60.9 | 58.7 |
| 1398 | 61.3 | 37.9 |
| 1399 | 61.2 | 38.3 |
| 1400 | 61.4 | 58.7 |
| 1401 | 61.3 | 51.3 |
| 1402 | 61.4 | 71.1 |
| 1403 | 61.1 | 51 |
| 1404 | 61.5 | 56.6 |
| 1405 | 61 | 60.6 |
| 1406 | 61.1 | 75.4 |
| 1407 | 61.4 | 69.4 |
| 1408 | 61.6 | 69.9 |
| 1409 | 61.7 | 59.6 |
| 1410 | 61.8 | 54.8 |
| 1411 | 61.6 | 53.6 |
| 1412 | 61.3 | 53.5 |
| 1413 | 61.3 | 52.9 |
| 1414 | 61.2 | 54.1 |
| 1415 | 61.3 | 53.2 |
| 1416 | 61.2 | 52.2 |
| 1417 | 61.2 | 52.3 |
| 1418 | 61 | 48 |
| 1419 | 60.9 | 41.5 |
| 1420 | 61 | 32.2 |
| 1421 | 60.7 | 22 |
| 1422 | 60.7 | 23.3 |
| 1423 | 60.8 | 38.8 |
| 1424 | 61 | 40.7 |
| 1425 | 61 | 30.6 |
| 1426 | 61.3 | 62.6 |
| 1427 | 61.7 | 55.9 |
| 1428 | 62.3 | 43.4 |
| 1429 | 62.3 | 37.4 |
| 1430 | 62.3 | 35.7 |
| 1431 | 62.8 | 34.4 |
| 1432 | 62.8 | 31.5 |
| 1433 | 62.9 | 31.7 |
| 1434 | 62.9 | 29.9 |
| 1435 | 62.8 | 29.4 |
| 1436 | 62.7 | 28.7 |
| 1437 | 61.5 | 14.7 |
| 1438 | 61.9 | 17.2 |
| 1439 | 61.5 | 6.1 |
| 1440 | 61 | 9.9 |
| 1441 | 60.9 | 4.8 |
| 1442 | 60.6 | 11.1 |
| 1443 | 60.3 | 6.9 |
| 1444 | 60.8 | 7 |
| 1445 | 60.2 | 9.2 |
| 1446 | 60.5 | 21.7 |
| 1447 | 60.2 | 22.4 |
| 1448 | 60.7 | 31.6 |
| 1449 | 60.9 | 28.9 |
| 1450 | 59.6 | 21.7 |
| 1451 | 60.2 | 18 |
| 1452 | 59.5 | 16.7 |
| 1453 | 59.8 | 15.7 |
| 1454 | 59.6 | 15.7 |
| 1455 | 59.3 | 15.7 |
| 1456 | 59 | 7.5 |
| 1457 | 58.8 | 7.1 |
| 1458 | 58.7 | 16.5 |
| 1459 | 59.2 | 50.7 |
| 1460 | 59.7 | 60.2 |
| 1461 | 60.4 | 44 |
| 1462 | 60.2 | 35.3 |
| 1463 | 60.4 | 17.1 |
| 1464 | 59.9 | 13.5 |
| 1465 | 59.9 | 12.8 |
| 1466 | 59.6 | 14.8 |
| 1467 | 59.4 | 15.9 |
| 1468 | 59.4 | 22 |
| 1469 | 60.4 | 38.4 |
| 1470 | 59.5 | 38.8 |
| 1471 | 59.3 | 31.9 |
| 1472 | 60.9 | 40.8 |
| 1473 | 60.7 | 39 |
| 1474 | 60.9 | 30.1 |
| 1475 | 61 | 29.3 |
| 1476 | 60.6 | 28.4 |
| 1477 | 60.9 | 36.3 |
| 1478 | 60.8 | 30.5 |
| 1479 | 60.7 | 26.7 |
| 1480 | 60.1 | 4.7 |
| 1481 | 59.9 | 0 |
| 1482 | 60.4 | 36.2 |
| 1483 | 60.7 | 32.5 |
| 1484 | 59.9 | 3.1 |
| 1485 | 59.7 | "m" |
| 1486 | 59.5 | "m" |
| 1487 | 59.2 | "m" |
| 1488 | 58.8 | 0.6 |
| 1489 | 58.7 | "m" |
| 1490 | 58.7 | "m" |
| 1491 | 57.9 | "m" |
| 1492 | 58.2 | "m" |
| 1493 | 57.6 | "m" |
| 1494 | 58.3 | 9.5 |
| 1495 | 57.2 | 6 |
| 1496 | 57.4 | 27.3 |
| 1497 | 58.3 | 59.9 |
| 1498 | 58.3 | 7.3 |
| 1499 | 58.8 | 21.7 |
| 1500 | 58.8 | 38.9 |
| 1501 | 59.4 | 26.2 |
| 1502 | 59.1 | 25.5 |
| 1503 | 59.1 | 26 |
| 1504 | 59 | 39.1 |
| 1505 | 59.5 | 52.3 |
| 1506 | 59.4 | 31 |
| 1507 | 59.4 | 27 |
| 1508 | 59.4 | 29.8 |
| 1509 | 59.4 | 23.1 |
| 1510 | 58.9 | 16 |
| 1511 | 59 | 31.5 |
| 1512 | 58.8 | 25.9 |
| 1513 | 58.9 | 40.2 |
| 1514 | 58.8 | 28.4 |
| 1515 | 58.9 | 38.9 |
| 1516 | 59.1 | 35.3 |
| 1517 | 58.8 | 30.3 |
| 1518 | 59 | 19 |
| 1519 | 58.7 | 3 |
| 1520 | 57.9 | 0 |
| 1521 | 58 | 2.4 |
| 1522 | 57.1 | "m" |
| 1523 | 56.7 | "m" |
| 1524 | 56.7 | 5.3 |
| 1525 | 56.6 | 2.1 |
| 1526 | 56.8 | "m" |
| 1527 | 56.3 | "m" |
| 1528 | 56.3 | "m" |
| 1529 | 56 | "m" |
| 1530 | 56.7 | "m" |
| 1531 | 56.6 | 3.8 |
| 1532 | 56.9 | "m" |
| 1533 | 56.9 | "m" |
| 1534 | 57.4 | "m" |
| 1535 | 57.4 | "m" |
| 1536 | 58.3 | 13.9 |
| 1537 | 58.5 | "m" |
| 1538 | 59.1 | "m" |
| 1539 | 59.4 | "m" |
| 1540 | 59.6 | "m" |
| 1541 | 59.5 | "m" |
| 1542 | 59.6 | 0.5 |
| 1543 | 59.3 | 9.2 |
| 1544 | 59.4 | 11.2 |
| 1545 | 59.1 | 26.8 |
| 1546 | 59 | 11.7 |
| 1547 | 58.8 | 6.4 |
| 1548 | 58.7 | 5 |
| 1549 | 57.5 | "m" |
| 1550 | 57.4 | "m" |
| 1551 | 57.1 | 1.1 |
| 1552 | 57.1 | 0 |
| 1553 | 57 | 4.5 |
| 1554 | 57.1 | 3.7 |
| 1555 | 57.3 | 3.3 |
| 1556 | 57.3 | 16.8 |
| 1557 | 58.2 | 29.3 |
| 1558 | 58.7 | 12.5 |
| 1559 | 58.3 | 12.2 |
| 1560 | 58.6 | 12.7 |
| 1561 | 59 | 13.6 |
| 1562 | 59.8 | 21.9 |
| 1563 | 59.3 | 20.9 |
| 1564 | 59.7 | 19.2 |
| 1565 | 60.1 | 15.9 |
| 1566 | 60.7 | 16.7 |
| 1567 | 60.7 | 18.1 |
| 1568 | 60.7 | 40.6 |
| 1569 | 60.7 | 59.7 |
| 1570 | 61.1 | 66.8 |
| 1571 | 61.1 | 58.8 |
| 1572 | 60.8 | 64.7 |
| 1573 | 60.1 | 63.6 |
| 1574 | 60.7 | 83.2 |
| 1575 | 60.4 | 82.2 |
| 1576 | 60 | 80.5 |
| 1577 | 59.9 | 78.7 |
| 1578 | 60.8 | 67.9 |
| 1579 | 60.4 | 57.7 |
| 1580 | 60.2 | 60.6 |
| 1581 | 59.6 | 72.7 |
| 1582 | 59.9 | 73.6 |
| 1583 | 59.8 | 74.1 |
| 1584 | 59.6 | 84.6 |
| 1585 | 59.4 | 76.1 |
| 1586 | 60.1 | 76.9 |
| 1587 | 59.5 | 84.6 |
| 1588 | 59.8 | 77.5 |
| 1589 | 60.6 | 67.9 |
| 1590 | 59.3 | 47.3 |
| 1591 | 59.3 | 43.1 |
| 1592 | 59.4 | 38.3 |
| 1593 | 58.7 | 38.2 |
| 1594 | 58.8 | 39.2 |
| 1595 | 59.1 | 67.9 |
| 1596 | 59.7 | 60.5 |
| 1597 | 59.5 | 32.9 |
| 1598 | 59.6 | 20 |
| 1599 | 59.6 | 34.4 |
| 1600 | 59.4 | 23.9 |
| 1601 | 59.6 | 15.7 |
| 1602 | 59.9 | 41 |
| 1603 | 60.5 | 26.3 |
| 1604 | 59.6 | 14 |
| 1605 | 59.7 | 21.2 |
| 1606 | 60.9 | 19.6 |
| 1607 | 60.1 | 34.3 |
| 1608 | 59.9 | 27 |
| 1609 | 60.8 | 25.6 |
| 1610 | 60.6 | 26.3 |
| 1611 | 60.9 | 26.1 |
| 1612 | 61.1 | 38 |
| 1613 | 61.2 | 31.6 |
| 1614 | 61.4 | 30.6 |
| 1615 | 61.7 | 29.6 |
| 1616 | 61.5 | 28.8 |
| 1617 | 61.7 | 27.8 |
| 1618 | 62.2 | 20.3 |
| 1619 | 61.4 | 19.6 |
| 1620 | 61.8 | 19.7 |
| 1621 | 61.8 | 18.7 |
| 1622 | 61.6 | 17.7 |
| 1623 | 61.7 | 8.7 |
| 1624 | 61.7 | 1.4 |
| 1625 | 61.7 | 5.9 |
| 1626 | 61.2 | 8.1 |
| 1627 | 61.9 | 45.8 |
| 1628 | 61.4 | 31.5 |
| 1629 | 61.7 | 22.3 |
| 1630 | 62.4 | 21.7 |
| 1631 | 62.8 | 21.9 |
| 1632 | 62.2 | 22.2 |
| 1633 | 62.5 | 31 |
| 1634 | 62.3 | 31.3 |
| 1635 | 62.6 | 31.7 |
| 1636 | 62.3 | 22.8 |
| 1637 | 62.7 | 12.6 |
| 1638 | 62.2 | 15.2 |
| 1639 | 61.9 | 32.6 |
| 1640 | 62.5 | 23.1 |
| 1641 | 61.7 | 19.4 |
| 1642 | 61.7 | 10.8 |
| 1643 | 61.6 | 10.2 |
| 1644 | 61.4 | "m" |
| 1645 | 60.8 | "m" |
| 1646 | 60.7 | "m" |
| 1647 | 61 | 12.4 |
| 1648 | 60.4 | 5.3 |
| 1649 | 61 | 13.1 |
| 1650 | 60.7 | 29.6 |
| 1651 | 60.5 | 28.9 |
| 1652 | 60.8 | 27.1 |
| 1653 | 61.2 | 27.3 |
| 1654 | 60.9 | 20.6 |
| 1655 | 61.1 | 13.9 |
| 1656 | 60.7 | 13.4 |
| 1657 | 61.3 | 26.1 |
| 1658 | 60.9 | 23.7 |
| 1659 | 61.4 | 32.1 |
| 1660 | 61.7 | 33.5 |
| 1661 | 61.8 | 34.1 |
| 1662 | 61.7 | 17 |
| 1663 | 61.7 | 2.5 |
| 1664 | 61.5 | 5.9 |
| 1665 | 61.3 | 14.9 |
| 1666 | 61.5 | 17.2 |
| 1667 | 61.1 | "m" |
| 1668 | 61.4 | "m" |
| 1669 | 61.4 | 8.8 |
| 1670 | 61.3 | 8.8 |
| 1671 | 61 | 18 |
| 1672 | 61.5 | 13 |
| 1673 | 61 | 3.7 |
| 1674 | 60.9 | 3.1 |
| 1675 | 60.9 | 4.7 |
| 1676 | 60.6 | 4.1 |
| 1677 | 60.6 | 6.7 |
| 1678 | 60.6 | 12.8 |
| 1679 | 60.7 | 11.9 |
| 1680 | 60.6 | 12.4 |
| 1681 | 60.1 | 12.4 |
| 1682 | 60.5 | 12 |
| 1683 | 60.4 | 11.8 |
| 1684 | 59.9 | 12.4 |
| 1685 | 59.6 | 12.4 |
| 1686 | 59.6 | 9.1 |
| 1687 | 59.9 | 0 |
| 1688 | 59.9 | 20.4 |
| 1689 | 59.8 | 4.4 |
| 1690 | 59.4 | 3.1 |
| 1691 | 59.5 | 26.3 |
| 1692 | 59.6 | 20.1 |
| 1693 | 59.4 | 35 |
| 1694 | 60.9 | 22.1 |
| 1695 | 60.5 | 12.2 |
| 1696 | 60.1 | 11 |
| 1697 | 60.1 | 8.2 |
| 1698 | 60.5 | 6.7 |
| 1699 | 60 | 5.1 |
| 1700 | 60 | 5.1 |
| 1701 | 60 | 9 |
| 1702 | 60.1 | 5.7 |
| 1703 | 59.9 | 8.5 |
| 1704 | 59.4 | 6 |
| 1705 | 59.5 | 5.5 |
| 1706 | 59.5 | 14.2 |
| 1707 | 59.5 | 6.2 |
| 1708 | 59.4 | 10.3 |
| 1709 | 59.6 | 13.8 |
| 1710 | 59.5 | 13.9 |
| 1711 | 60.1 | 18.9 |
| 1712 | 59.4 | 13.1 |
| 1713 | 59.8 | 5.4 |
| 1714 | 59.9 | 2.9 |
| 1715 | 60.1 | 7.1 |
| 1716 | 59.6 | 12 |
| 1717 | 59.6 | 4.9 |
| 1718 | 59.4 | 22.7 |
| 1719 | 59.6 | 22 |
| 1720 | 60.1 | 17.4 |
| 1721 | 60.2 | 16.6 |
| 1722 | 59.4 | 28.6 |
| 1723 | 60.3 | 22.4 |
| 1724 | 59.9 | 20 |
| 1725 | 60.2 | 18.6 |
| 1726 | 60.3 | 11.9 |
| 1727 | 60.4 | 11.6 |
| 1728 | 60.6 | 10.6 |
| 1729 | 60.8 | 16 |
| 1730 | 60.9 | 17 |
| 1731 | 60.9 | 16.1 |
| 1732 | 60.7 | 11.4 |
| 1733 | 60.9 | 11.3 |
| 1734 | 61.1 | 11.2 |
| 1735 | 61.1 | 25.6 |
| 1736 | 61 | 14.6 |
| 1737 | 61 | 10.4 |
| 1738 | 60.6 | "m" |
| 1739 | 60.9 | "m" |
| 1740 | 60.8 | 4.8 |
| 1741 | 59.9 | "m" |
| 1742 | 59.8 | "m" |
| 1743 | 59.1 | "m" |
| 1744 | 58.8 | "m" |
| 1745 | 58.8 | "m" |
| 1746 | 58.2 | "m" |
| 1747 | 58.5 | 14.3 |
| 1748 | 57.5 | 4.4 |
| 1749 | 57.9 | 0 |
| 1750 | 57.8 | 20.9 |
| 1751 | 58.3 | 9.2 |
| 1752 | 57.8 | 8.2 |
| 1753 | 57.5 | 15.3 |
| 1754 | 58.4 | 38 |
| 1755 | 58.1 | 15.4 |
| 1756 | 58.8 | 11.8 |
| 1757 | 58.3 | 8.1 |
| 1758 | 58.3 | 5.5 |
| 1759 | 59 | 4.1 |
| 1760 | 58.2 | 4.9 |
| 1761 | 57.9 | 10.1 |
| 1762 | 58.5 | 7.5 |
| 1763 | 57.4 | 7 |
| 1764 | 58.2 | 6.7 |
| 1765 | 58.2 | 6.6 |
| 1766 | 57.3 | 17.3 |
| 1767 | 58 | 11.4 |
| 1768 | 57.5 | 47.4 |
| 1769 | 57.4 | 28.8 |
| 1770 | 58.8 | 24.3 |
| 1771 | 57.7 | 25.5 |
| 1772 | 58.4 | 35.5 |
| 1773 | 58.4 | 29.3 |
| 1774 | 59 | 33.8 |
| 1775 | 59 | 18.7 |
| 1776 | 58.8 | 9.8 |
| 1777 | 58.8 | 23.9 |
| 1778 | 59.1 | 48.2 |
| 1779 | 59.4 | 37.2 |
| 1780 | 59.6 | 29.1 |
| 1781 | 50 | 25 |
| 1782 | 40 | 20 |
| 1783 | 30 | 15 |
| 1784 | 20 | 10 |
| 1785 | 10 | 5 |
| 1786 | 0 | 0 |
| 1787 | 0 | 0 |
| 1788 | 0 | 0 |
| 1789 | 0 | 0 |
| 1790 | 0 | 0 |
| 1791 | 0 | 0 |
| 1792 | 0 | 0 |
| 1793 | 0 | 0 |
| 1794 | 0 | 0 |
| 1795 | 0 | 0 |
| 1796 | 0 | 0 |
| 1797 | 0 | 0 |
| 1798 | 0 | 0 |
| 1799 | 0 | 0 |
| 1800 | 0 | 0 |
| "m" = motoring |  |  |

A graphical display of the ETC dynamometer schedule is shown in figure 5.

Figure 5

ETC dynamometer schedule

Appendix 4

MEASUREMENT AND SAMPLING PROCEDURES

1.	INTRODUCTION

Gaseous components, particulates, and smoke emitted by the engine submitted for testing shall be measured by the methods described in Appendix 7.  The respective paragraphs of Appendix 7 describe the recommended analytical systems for the gaseous emissions (paragraph 1.), the recommended particulate dilution and sampling systems (paragraph 2.), and the recommended opacimeters for smoke measurement (paragraph 3.).

For the ESC, the gaseous components shall be determined in the raw exhaust gas.  Optionally, they may be determined in the diluted exhaust gas, if a full flow dilution system is used for particulate determination.  Particulates shall be determined with either a partial flow or a full flow dilution system.

For the ETC, the following systems may be used:

- (a)	a CVS full flow dilution system for determining gaseous and particulate emissions (double dilution systems are permissible), or,

- (b)	a combination of raw exhaust measurement for the gaseous emissions and a partial flow dilution system for particulate emissions, or,

- (c)	any combination of the two principles (e.g. raw gaseous measurement and full flow particulate measurement).

2.	DYNAMOMETER AND TEST CELL EQUIPMENT

The following equipment shall be used for emission tests of engines on engine dynamometers.

2.1.	Engine dynamometer

An engine dynamometer shall be used with adequate characteristics to perform the test cycles described in Appendices 1 and 2 to this annex.  The speed measuring system shall have an accuracy of ± 2 per cent of reading.  The torque measuring system shall have an accuracy of ± 3 per cent of reading in the range > 20 per cent of full scale, and an accuracy of ± 0,6 per cent of full scale in the range ≤ 20 per cent of full scale.

2.2.	Other instruments

Measuring instruments for fuel consumption, air consumption, temperature of coolant and lubricant, exhaust gas pressure and intake manifold depression, exhaust gas temperature, air intake temperature, atmospheric pressure, humidity and fuel temperature shall be used, as required.  These instruments shall satisfy the requirements given in table 9:

Table 9

Accuracy of measuring instruments

| Measuring instrument | Accuracy |
| --- | --- |
| Fuel consumption | ± 2 per cent of engine's maximum value |
| Air consumption |  2 per cent of reading or  1 per cent of engine's maximum value whichever is greater |
| Exhaust gas flow |  2.5 per cent of reading or  1.5 per cent of engine's maximum value whichever is greater |
| Temperatures ≤ 600 K (327 °C) | ± 2 K absolute |
| Temperatures ≥ 600 K (327 °C) | ± 1 per cent of reading |
| Atmospheric pressure | ± 0.1 kPa absolute |
| Exhaust gas pressure | ± 0.2 kPa absolute |
| Intake depression | ± 0.05 kPa absolute |
| Other pressures | ± 0.1 kPa absolute |
| Relative humidity | ± 3 per cent absolute |
| Absolute humidity | ± 5 per cent of reading |
| Dilution air flow | ± 2 per cent of reading |
| Diluted exhaust gas flow | ± 2 per cent of reading |

3.	DETERMINATION OF THE GASEOUS COMPONENTS

3.1.	General analyser specifications

The analysers shall have a measuring range appropriate for the accuracy required to measure the concentrations of the exhaust gas components (paragraph 3.1.1.).  It is recommended that the analysers be operated such that the measured concentration falls between 15 per cent and 100 per cent of full scale.

If read-out systems (computers, data loggers) can provide sufficient accuracy and resolution below 15 per cent of full scale, measurements below 15 per cent of full scale are also acceptable.  In this case, additional calibrations of at least 4 non-zero nominally equally spaced points are to be made to ensure the accuracy of the calibration curves according to paragraph 1.6.4. of Appendix 5 to this annex.

The electromagnetic compatibility (EMC) of the equipment shall be on a level as to minimize additional errors.

3.1.1.	Accuracy

The analyser shall not deviate from the nominal calibration point by more than ± 2 per cent of the reading over the whole measurement range except zero, or ± 0,3 per cent of full scale whichever is larger.  The accuracy shall be determined according to the calibration requirements laid down in paragraph 1.6. of Appendix 5 to this annex.

NOTE:	For the purpose of this Regulation, accuracy is defined as the deviation of the analyser reading from the nominal calibration values using a calibration gas (= true value).

3.1.2.	Precision

The precision, defined as 2.5 times the standard deviation of 10 repetitive responses to a given calibration or span gas, has to be not greater than ± 1 per cent of full scale concentration for each range used above 155 ppm (or ppmC) or ± 2 per cent of each range used below 155 ppm (or ppmC).

3.1.3.	Noise

The analyser peak-to-peak response to zero and calibration or span gases over any 10 second period shall not exceed 2 per cent of full scale on all ranges used.

3.1.4.	Zero drift

Zero response is defined as the mean response, including noise, to a zero gas during a 30 seconds time interval.  The drift of the zero response during a one hour period shall be less than 2 per cent of full scale on the lowest range used.

3.1.5.	Span drift

Span response is defined as the mean response, including noise, to a span gas during a 30 seconds time interval.  The drift of the span response during a one hour period shall be less than 2 per cent of full scale on the lowest range used.

3.1.6.	Rise time

The rise time of the analyser installed in the measurement system shall not exceed 3.5 s.

NOTE:	Only evaluating the response time of the analyser alone will not clearly define the suitability of the total system for transient testing.  Volumes and especially dead volumes through out the system will not only effect the transportation time from the probe to the analyser, but also effect the rise time.  Also transport times inside of an analyser would be defined as analyser response time, like the converter or water traps inside NOx analysers.  The determination of the total system response time is described in paragraph 1.5. of Appendix 5 to this annex.

3.2.	Gas drying

The optional gas drying device shall have a minimal effect on the concentration of the measured gases.  Chemical dryers are not an acceptable method of removing water from the sample.

3.3.	Analysers

Paragraphs 3.3.1 to 3.3.4 describe the measurement principles to be used. A detailed description of the measurement systems is given in Appendix 7. The gases to be measured shall be analysed with the following instruments. For non-linear analysers, the use of linearizing circuits is permitted.

3.3.1.	Carbon monoxide (CO) analysis

The carbon monoxide analyser shall be of the Non-Dispersive InfraRed (NDIR) absorption type.

3.3.2.	Carbon dioxide (CO2) analysis

The carbon dioxide analyser shall be of the Non-Dispersive InfraRed (NDIR) absorption type.

3.3.3.	Hydrocarbon (HC) analysis

For diesel and LPG fuelled gas engines, the hydrocarbon analyser shall be of the Heated Flame Ionisation Detector (HFID) type with detector, valves, pipework, etc. heated so as to maintain a gas temperature of 463 K ± 10 K (190 ± 10 °C).  For NG fuelled gas engines, the hydrocarbon analyser may be of the non heated Flame Ionisation Detector (FID) type depending upon the method used (see paragraph 1.3. of Appendix 7).

3.3.4.	Non-Methane Hydrocarbon (NMHC) analysis (NG fuelled gas engines only)

Non-methane hydrocarbons shall be determined by either of the following methods:

3.3.4.1.	Gas chromatographic (GC) method

Non-methane hydrocarbons shall be determined by subtraction of the methane analysed with a Gas Chromatograph (GC) conditioned at 423 K (150 °C) from the hydrocarbons measured according to paragraph 3.3.3.

3.3.4.2.	Non-Methane Cutter (NMC) method

The determination of the non-methane fraction shall be performed with a heated NMC operated in line with an FID as per paragraph 3.3.3. by subtraction of the methane from the hydrocarbons.

3.3.5.	Oxides of nitrogen (NOx) analysis

The oxides of nitrogen analyser shall be of the ChemiLuminescent Detector (CLD) or Heated ChemiLuminescent Detector (HCLD) type with a NO2/NO converter, if measured on a dry basis.  If measured on a wet basis, a HCLD with converter maintained above 328 K (55 °C) shall be used, provided the water quench check (see paragraph 1.9.2.2. of Appendix 5 to this annex) is satisfied.

3.3.6.	Air-to-fuel measurement

The air to fuel measurement equipment used to determine the exhaust gas flow as specified in paragraph 4.2.5. of Appendix 2 to this annex shall be a wide range air to fuel ratio sensor or lambda sensor of Zirconia type.  The sensor shall be mounted directly on the exhaust pipe where the exhaust gas temperature is high enough to eliminate water condensation.

The accuracy of the sensor with incorporated electronics shall be within:

 3 per cent of reading	 < 2

 5 per cent of reading	2   < 5

 10 per cent of reading	5  

To fulfil the accuracy specified above, the sensor shall be calibrated as specified by the instrument manufacturer.

3.4.	Sampling of gaseous emissions

3.4.1.	Raw exhaust gas

The gaseous emissions sampling probes shall be fitted at least 0.5 m or 3 times the diameter of the exhaust pipe - whichever is the larger - upstream of the exit of the exhaust gas system but sufficiently close to the engine as to ensure an exhaust gas temperature of at least 343 K (70°C) at the probe.

In the case of a multi-cylinder engine with a branched exhaust manifold, the inlet of the probe shall be located sufficiently far downstream so as to ensure that the sample is representative of the average exhaust emissions from all cylinders.  In multi-cylinder engines having distinct groups of manifolds, such as in a "Vee" engine configuration, it is recommended to combine the manifolds upstream of the sampling probe.  If this is not practical, it is permissible to acquire a sample from the group with the highest CO2 emission.  Other methods which have been shown to correlate with the above methods may be used.  For exhaust emission calculation the total exhaust mass flow shall be used.

If the engine is equipped with an exhaust aftertreatment system, the exhaust sample shall be taken downstream of the exhaust aftertreatment system.

3.4.2.	Diluted exhaust gas

The exhaust pipe between the engine and the full flow dilution system shall conform to the requirements of paragraph 2.3.1. of Appendix 7 (EP).

The gaseous emissions sample probe(s) shall be installed in the dilution tunnel at a point where the dilution air and exhaust gas are well mixed, and in close proximity to the particulates sampling probe.

Sampling can generally be done in two ways:

- (a)	the pollutants are sampled into a sampling bag over the cycle and measured after completion of the test;

- (b)	the pollutants are sampled continuously and integrated over the cycle; this method is mandatory for HC and NOx.

4.	DETERMINATION OF THE PARTICULATES

The determination of the particulates requires a dilution system.  Dilution may be accomplished by a partial flow dilution system or a full flow double dilution system.  The flow capacity of the dilution system shall be large enough to completely eliminate water condensation in the dilution and sampling systems.  The temperature of the diluted exhaust gas shall be below 325 K (52 °C) immediately upstream of the filter holders.  Humidity control of the dilution air before entering the dilution system is permitted, and especially dehumidifying is useful if dilution air humidity is high.  The temperature of the dilution air shall be higher than 288 K (15 °C) in close proximity to the entrance into the dilution tunnel.

The partial flow dilution system has to be designed to extract a proportional raw exhaust sample from the engine exhaust stream, thus responding to excursions in the exhaust stream flow rate, and introduce dilution air to this sample to achieve a temperature of below 325 K (52 °C) at the test filter.  For this it is essential that the dilution ratio or the sampling ratio rdil or rs be determined such that the accuracy limits of paragraph 3.2.1. of Appendix 5 to this annex are fulfilled.  Different extraction methods can be applied, whereby the type of extraction used dictates to a significant degree the sampling hardware and procedures to be used (paragraph 2.2. of Appendix 7).

In general, the particulate sampling probe shall be installed in close proximity to the gaseous emissions sampling probe, but sufficiently distant as to not cause interference.  Therefore, the installation provisions of paragraph 3.4.1. also apply to particulate sampling.  The sampling line shall conform to the requirements of paragraph 2. of Appendix 7.

In the case of a multi-cylinder engine with a branched exhaust manifold, the inlet of the probe shall be located sufficiently far downstream so as to ensure that the sample is representative of the average exhaust emissions from all cylinders.  In multi-cylinder engines having distinct groups of manifolds, such as in a "Vee" engine configuration, it is recommended to combine the manifolds upstream of the sampling probe.  If this is not practical, it is permissible to acquire a sample from the group with the highest particulate emission.  Other methods which have been shown to correlate with the above methods may be used.  For exhaust emission calculation the total exhaust mass flow shall be used.

To determine the mass of the particulates, a particulate sampling system, particulate sampling filters, a microgram balance, and a temperature and humidity controlled weighing chamber, are required.

For particulate sampling, the single filter method shall be applied which uses one filter (see paragraph 4.1.3. of this appendix) for the whole test cycle.  For the ESC, considerable attention shall be paid to sampling times and flows during the sampling phase of the test.

4.1	Particulate sampling filters

The diluted exhaust shall be sampled by a filter that meets the requirements of paragraphs 4.1.1. and 4.1.2. during the test sequence.

4.1.1.	Filter specification

Fluorocarbon coated glass fiber filters are required.  All filter types shall have a 0.3 µm DOP (di-octylphthalate) collection efficiency of at least 99 per cent at a gas face velocity between 35 and 100 cm/s.

4.1.2.	Filter size

Particulate filters with a diameter of 47 mm or 70 mm are recommended.  Larger diameter filters are acceptable (paragraph 4.1.4.), but smaller diameter filters are not permitted.

4.1.3.	Filter face velocity

A gas face velocity through the filter of 35 to 100 cm/s shall be achieved.  The pressure drop increase between the beginning and the end of the test shall be no more than 25 kPa.

4.1.4.	Filter loading

The required minimum filter loadings for the most common filter sizes are shown in table 10.  For larger filter sizes, the minimum filter loading shall be 0.065 mg/1,000 mm² filter area.

Table 10

Minimum Filter Loadings

| Filter Diameter (mm) | Minimum loading (mg) |
| --- | --- |
| 47 | 0.11 |
| 70 | 0.25 |
| 90 | 0.41 |
| 110 | 0.62 |

If, based on previous testing, the required minimum filter loading is unlikely to be reached on a test cycle after optimisation of flow rates and dilution ratio, a lower filter loading may be acceptable, with the agreement of the parties involved (manufacturer and approval authority), if it can be shown to meet the accuracy requirements of paragraph 4.2., e.g. with a 0.1 µg balance.

4.1.5.	Filter holder

For the emissions test, the filters shall be placed in a filter holder assembly meeting the requirements of paragraph 2.2. of Appendix 7.  The filter holder assembly shall be of a design that provides an even flow distribution across the filter stain area.  Quick acting valves shall be located either upstream or downstream of the filter holder.  An inertial pre-classifier with a 50 per cent cut point between 2.5 µm and 10 µm may be installed immediately upstream of the filter holder.  The use of the pre-classifier is strongly recommended if an open tube sampling probe facing upstream into the exhaust flow is used.

4.2	Weighing chamber and analytical balance specifications

4.2.1.	Weighing chamber conditions

The temperature of the chamber (or room) in which the particulate filters are conditioned and weighed shall be maintained to within 295 K ± 3 K (22 °C ± 3 °C) during all filter conditioning and weighing.  The humidity shall be maintained to a dewpoint of 282.5 K ± 3 K (9.5 °C ± 3 °C) and a relative humidity of 45 per cent ± 8 per cent.

4.2.2.	Reference filter weighing

The chamber (or room) environment shall be free of any ambient contaminants (such as dust) that would settle on the particulate filters during their stabilisation.  Disturbances to weighing room specifications as outlined in paragraph 4.2.1. will be allowed if the duration of the disturbances does not exceed 30 minutes.  The weighing room should meet the required specifications prior to personal entrance into the weighing room.  At least two unused reference filters shall be weighed within 4 hours of, but preferably at the same time as the sample filter weightings.  They shall be the same size and material as the sample filters.

If the average weight of the reference filters changes between sample filter weightings by more than 10 µg, then all sample filters shall be discarded and the emissions test repeated.

If the weighing room stability criteria outlined in paragraph 4.2.1. is not met, but the reference filter weightings meet the above criteria, the engine manufacturer has the option of accepting the sample filter weights or voiding the tests, fixing the weighing room control system and re-running the test.

4.2.3.	Analytical balance

The analytical balance used to determine the filter weight shall have a precision (standard deviation) of at least 2 µg and a resolution of at least 1 µg (1 digit = 1 µg) specified by the balance manufacturer.

4.2.4.	Elimination of static electricity effects

To eliminate the effects of static electricity, the filters shall be neutralized prior to weighing, e.g. by a Polonium neutralizer, a Faraday cage or a device of similar effect.

4.2.5.	Specifications for flow measurement

4.2.5.1.	General requirements

Absolute accuracies of flow meter or flow measurement instrumentation shall be as specified in paragraph 2.2.

4.2.5.2.	Special provisions for partial flow dilution systems

For partial flow dilution systems, the accuracy of the sample flow qmp is of special concern, if not measured directly, but determined by differential flow measurement:

In this case an accuracy of  2 per cent for qmdew and qmdw is not sufficient to guarantee acceptable accuracies of qmp.  If the gas flow is determined by differential flow measurement, the maximum error of the difference shall be such that the accuracy of qmp is within  5 per cent when the dilution ratio is less than 15.  It can be calculated by taking root-mean-square of the errors of each instrument.

Acceptable accuracies of qmp can be obtained by either of the following methods:

The absolute accuracies of qmdew and qmdw are  0.2 per cent which guarantees an accuracy of qmp of  5 per cent at a dilution ratio of 15.  However, greater errors will occur at higher dilution ratios;

Calibration of qmdw relative to qmdew is carried out such that the same accuracies for qmp as in a) are obtained.  For the details of such a calibration see paragraph 3.2.1. of Appendix 5 to this annex;

The accuracy of qmp is determined indirectly from the accuracy of the dilution ratio as determined by a tracer gas, e.g. CO2.  Again, accuracies equivalent to method a) for qmp are required;

The absolute accuracy of qmdew and qmdw is within  2 per cent of full scale, the maximum error of the difference between qmdew and qmdw is within 0.2 per cent, and the linearity error is within  0.2 per cent of the highest qmdew observed during the test.

5.	DETERMINATION OF SMOKE

This paragraph provides specifications for the required and optional test equipment to be used for the ELR test.  The smoke shall be measured with an opacimeter having an opacity and a light absorption coefficient readout mode.  The opacity readout mode shall only be used for calibration and checking of the opacimeter.  The smoke values of the test cycle shall be measured in the light absorption coefficient readout mode.

5.1.	General requirements

The ELR requires the use of a smoke measurement and data processing system which includes three functional units.  These units may be integrated into a single component or provided as a system of interconnected components.  The three functional units are:

- (a)	An opacimeter meeting the specifications of Appendix 7, paragraph 3.

- (b)	A data processing unit capable of performing the functions described in paragraph 6. of Appendix 1 to this Annex

- (c)	A printer and/or electronic storage medium to record and output the required smoke values specified in paragraph 6.3 of Appendix 1 to this Annex.

5.2.	Specific requirements

5.2.1	Linearity

The linearity shall be within ± 2 per cent opacity.

5.2.2.	Zero Drift

The zero drift during a one hour period shall not exceed ± 1 per cent opacity.

5.2.3.	Opacimeter Display and Range

For display in opacity, the range shall be 0-100 per cent opacity, and the readability 0.1 per cent opacity. For display in light absorption coefficient, the range shall be 0-30 m−1 light absorption coefficient, and the readability 0.01 m−1 light absorption coefficient.

5.2.4.	Instrument response time

The physical response time of the opacimeter shall not exceed 0.2 s.  The physical response time is the difference between the times when the output of a rapid response receiver reaches 10 and 90 per cent of the full deviation when the opacity of the gas being measured is changed in less than 0.1 s.

The electrical response time of the opacimeter shall not exceed 0.05 s.  The electrical response time is the difference between the times when the opacimeter output reaches 10 and 90 per cent of the full scale when the light source is interrupted or completely extinguished in less than 0.01 s.

5.2.5.	Neutral density filters

Any neutral density filter used in conjunction with opacimeter calibration, linearity measurements, or setting span shall have its value known to within 1.0 per cent opacity. The filter's nominal value shall be checked for accuracy at least yearly using a reference traceable to a national or international standard.

Neutral density filters are precision devices and can easily be damaged during use. Handling should be minimized and, when required, should be done with care to avoid scratching or soiling of the filter.

Appendix 5

CALIBRATION PROCEDURE

1.	CALIBRATION OF THE ANALYTICAL INSTRUMENTS

1.1.	Introduction

Each analyser shall be calibrated as often as necessary to fulfil the accuracy requirements of this Regulation.  The calibration method that shall be used is described in this paragraph for the analysers indicated in Appendix 4, paragraph 3. and Appendix 7, paragraph 1.

1.2.	Calibration Gases

The shelf life of all calibration gases shall be respected.

The expiration date of the calibration gases stated by the manufacturer shall be recorded.

1.2.1.	Pure Gases

The required purity of the gases is defined by the contamination limits given below. The following gases shall be available for operation:

Purified nitrogen

(Contamination ≤ 1 ppm C1, ≤ 1 ppm CO, ≤ 400 ppm CO2, ≤ 0.1 ppm NO)

Purified oxygen

(Purity > 99.5 per cent vol O2)

Hydrogen-helium mixture

(40 ± 2 per cent hydrogen, balance helium)

(Contamination ≤ 1 ppm C1, ≤ 400 ppm CO2)

Purified synthetic air

(Contamination ≤ 1 ppm C1, ≤ 1 ppm CO, ≤ 400 ppm CO2, ≤ 0,1 ppm NO)

(Oxygen content between 18-21 per cent vol.)

Purified propane or CO for the CVS verification

1.2.2.	Calibration and Span Gases

Mixtures of gases having the following chemical compositions shall be available:

C3H8 and purified synthetic air (see paragraph 1.2.1.);

CO and purified nitrogen;

NOx and purified nitrogen (the amount of NO2 contained in this calibration gas shall not exceed 5 per cent of the NO content);

CO2 and purified nitrogen

CH4 and purified synthetic air

C2H6 and purified synthetic air

Note:	Other gas combinations are allowed provided the gases do not react with one another.

The true concentration of a calibration and span gas shall be within ± 2 per cent of the nominal value.  All concentrations of calibration gas shall be given on a volume basis (volume percent or volume ppm).

The gases used for calibration and span may also be obtained by means of a gas divider, diluting with purified N2 or with purified synthetic air.  The accuracy of the mixing device shall be such that the concentration of the diluted calibration gases may be determined to within ± 2 per cent.

1.2.3.	Use of precision blending devices

The gases used for calibration and span may also be obtained by means of precision blending devices (gas dividers), diluting with purified N2 or with purified synthetic air.  The accuracy of the mixing device shall be such that the concentration of the blended calibration gases is accurate to within  2 per cent.  This accuracy implies that primary gases used for blending shall be known to an accuracy of at least  1 per cent, traceable to national or international gas standards.  The verification shall be performed at between 15 and 50 per cent of full scale for each calibration incorporating a blending device.

Optionally, the blending device may be checked with an instrument which by nature is linear, e.g. using NO gas with a CLD.  The span value of the instrument shall be adjusted with the span gas directly connected to the instrument.  The blending device shall be checked at the used settings and the nominal value shall be compared to the measured concentration of the instrument.  This difference shall in each point be within  1 per cent of the nominal value.

1.3.	Operating Procedure for Analysers and Sampling System

The operating procedure for analysers shall follow the start-up and operating instructions of the instrument manufacturer.  The minimum requirements given in paragraphs 1.4 to 1.9 shall be included.

1.4.	Leakage test

A system leakage test shall be performed.  The probe shall be disconnected from the exhaust system and the end plugged.  The analyser pump shall be switched on.  After an initial stabilisation period all flow meters should read zero.  If not, the sampling lines shall be checked and the fault corrected.

The maximum allowable leakage rate on the vacuum side shall be 0.5 per cent of the in-use flow rate for the portion of the system being checked.  The analyser flows and bypass flows may be used to estimate the in-use flow rates.

Alternatively, the system may be evacuated to a pressure of at least 20 kPa vacuum (80 kPa absolute).  After an initial stabilisation period the pressure increase p (kPa/min) in the system should not exceed:

p = p / Vs × 0.005 × qvs

where:

Vs	=	system volume, l

qvs	=	system flow rate, l/min

Another method is the introduction of a concentration step change at the beginning of the sampling line by switching from zero to span gas.  If after an adequate period of time the reading is about 1 per cent low compared to the introduced concentration, these points to calibration or leakage problems.

1.5.	Response time check of analytical system

The system settings for the response time evaluation shall be exactly the same as during measurement of the test run (i.e. pressure, flow rates, filter settings on the analyzers and all other response time influences).  The response time determination shall be done with gas switching directly at the inlet of the sample probe.  The gas switching shall be done in less than 0.1 second.  The gases used for the test shall cause a concentration change of at least 60 per cent FS.

The concentration trace of each single gas component shall be recorded.  The response time is defined to be the difference in time between the gas switching and the appropriate change of the recorded concentration.  The system response time (t90) consists of the delay time to the measuring detector and the rise time of the detector. The delay time is defined as the time from the change (t0) until the response is 10 per cent of the final reading (t10).  The rise time is defined as the time between 10 per cent and 90 per cent response of the final reading (t90 – t10).

For time alignment of the analyzer and exhaust flow signals in the case of raw measurement, the transformation time is defined as the time from the change (t0) until the response is 50 per cent of the final reading (t50).

The system response time shall be  10 seconds with a rise time  3.5 seconds for all limited components (CO, NOx, HC or NMHC) and all ranges used.

1.6.	Calibration

1.6.1.	Instrument assembly

The instrument assembly shall be calibrated and calibration curves checked against standard gases.  The same gas flow rates shall be used as when sampling exhaust.

1.6.2.	Warming-up time

The warming-up time should be according to the recommendations of the manufacturer. If not specified, a minimum of two hours is recommended for warming up the analysers.

1.6.3.	NDIR and HFID analyser

The NDIR analyser shall be tuned, as necessary, and the combustion flame of the HFID analyser shall be optimised (paragraph 1.8.1.).

1.6.4.	Establishment of the calibration curve

- (a)	Each normally used operating range shall be calibrated;

- (b)	Using purified synthetic air (or nitrogen), the CO, CO2, NOx and HC analysers shall be set at zero;

- (c)	The appropriate calibration gases shall be introduced to the analysers, the values recorded, and the calibration curve established;

- (d)	The calibration curve shall be established by at least 6 calibration points (excluding zero) approximately equally spaced over the operating range. The highest nominal concentration shall be equal to or higher than 90 per cent of full scale;

- (e)	The calibration curve shall be calculated by the method of least-squares. A best-fit linear or non-linear equation may be used;

- (f)	The calibration points shall not differ from the least-squares best-fit line by more than ± 2 per cent of reading or ± 0.3 per cent of full scale whichever is larger;

- (g)	The zero setting shall be rechecked and the calibration procedure repeated, if necessary.

1.6.5.	Alternative methods

If it can be shown that alternative technology (e.g. computer, electronically controlled range switch, etc.) can give equivalent accuracy, then these alternatives may be used.

1.6.6.	Calibration of tracer gas analyser for exhaust flow measurement

The calibration curve shall be established by at least 6 calibration points (excluding zero) approximately equally spaced over the operating range.  The highest nominal concentration shall be equal to or higher than 90 per cent of full scale.  The calibration curve is calculated by the method of least squares.

The calibration points shall not differ from the least-squares best-fit line by more than ± 2 per cent of reading or ± 0.3 per cent of full scale whichever is larger.

The analyser shall be set at zero and spanned prior to the test run using a zero gas and a span gas whose nominal value is more than 80 per cent of the analyser full scale.

1.6.7.	Verification of the calibration

Each normally used operating range shall be checked prior to each analysis in accordance with the following procedure.

The calibration shall be checked by using a zero gas and a span gas whose nominal value is more than 80 per cent of full scale of the measuring range.

If, for the two points considered, the value found does not differ by more than ± 4 per cent of full scale from the declared reference value, the adjustment parameters may be modified.  Should this not be the case, a new calibration curve shall be established in accordance with paragraph 1.5.5.

1.7.	Efficiency test of the NOx converter

The efficiency of the converter used for the conversion of NO2 into NO shall be tested as given in paragraphs 1.7.1. to 1.7.8. (figure 6).

1.7.1.	Test set-up

Using the test set-up as shown in figure 6 (see also Appendix 4, paragraph 3.3.5. to this annex) and the procedure below, the efficiency of converters can be tested by means of an ozonator.

1.7.2.	Calibration

The CLD and the HCLD shall be calibrated in the most common operating range following the manufacturer's specifications using zero and span gas (the NO content of which shall amount to about 80 per cent of the operating range and the NO2 concentration of the gas mixture to less than 5 per cent of the NO concentration).  The NOx analyser shall be in the NO mode so that the span gas does not pass through the converter.  The indicated concentration has to be recorded.

1.7.3.	Calculation

The efficiency of the NOx converter is calculated as follows:

where,

| a | is the NOx concentration according to paragraph 1.7.6. |
| --- | --- |
| b | is the NOx concentration according to paragraph 1.7.7. |
| c | is the NO concentration according to paragraph 1.7.4. |
| d | is the NO concentration according to paragraph 1.7.5. |

1.7.4.	Adding of oxygen

Via a T-fitting, oxygen or zero air is added continuously to the gas flow until the concentration indicated is about 20 per cent less than the indicated calibration concentration given in paragraph 1.7.2. (the analyser is in the NO mode).  The indicated concentration c shall be recorded.  The ozonator is kept deactivated throughout the process.

1.7.5.	Activation of the ozonator

The ozonator is now activated to generate enough ozone to bring the NO concentration down to about 20 per cent (minimum 10 per cent) of the calibration concentration given in paragraph 1.7.2.  The indicated concentration d shall be recorded (the analyser is in the NO mode).

1.7.6.	NOx mode

The NO analyser is then switched to the NOx mode so that the gas mixture (consisting of NO, NO2, O2 and N2) now passes through the converter.  The indicated concentration "a" shall be recorded (the analyser is in the NOx mode).

1.7.7.	Deactivation of the ozonator

The ozonator is now deactivated.  The mixture of gases described in paragraph 1.7.6. passes through the converter into the detector.  The indicated concentration b shall be recorded (the analyser is in the NOx mode).

1.7.8.	NO mode

Switched to NO mode with the ozonator deactivated, the flow of oxygen or synthetic air is also shut off.  The NOx reading of the analyser shall not deviate by more than ± 5 per cent from the value measured according to paragraph 1.7.2. (the analyser is in the NO mode).

1.7.9.	Test interval

The efficiency of the converter shall be tested prior to each calibration of the NOx analyser.

1.7.10.	Efficiency requirement

The efficiency of the converter shall not be less than 90 per cent, but a higher efficiency of 95 per cent is strongly recommended.

Note:	If, with the analyser in the most common range, the ozonator cannot give a reduction from 80 per cent to 20 per cent according to paragraph 1.7.5., then the highest range which will give the reduction shall be used.

Figure 6

Schematic of NOx converter efficiency device

1.8.	Adjustment of the FID

1.8.1.	Optimisation of the detector response

The FID shall be adjusted as specified by the instrument manufacturer.  A propane in air span gas should be used to optimise the response on the most common operating range.

With the fuel and air flow rates set at the manufacturer's recommendations, a 350 ± 75 ppm C span gas shall be introduced to the analyser.  The response at a given fuel flow shall be determined from the difference between the span gas response and the zero gas response.  The fuel flow shall be incrementally adjusted above and below the manufacturer's specification.  The span and zero response at these fuel flows shall be recorded.  The difference between the span and zero response shall be plotted and the fuel flow adjusted to the rich side of the curve.

1.8.2.	Hydrocarbon response factors

The analyser shall be calibrated using propane in air and purified synthetic air, according to paragraph 1.5.

Response factors shall be determined when introducing an analyser into service and after major service intervals.  The response factor (Rf) for a particular hydrocarbon species is the ratio of the FID C1 reading to the gas concentration in the cylinder expressed by ppm C1.

The concentration of the test gas shall be at a level to give a response of approximately 80 per cent of full scale.  The concentration shall be known to an accuracy of ± 2 per cent in reference to a gravimetric standard expressed in volume.  In addition, the gas cylinder shall be preconditioned for 24 hours at a temperature of 298 K ± 5 K (25 °C ± 5 °C).

The test gases to be used and the recommended relative response factor ranges are as follows:

Methane and purified synthetic air 1.00 ≤ Rf ≤ 1.15

Propylene and purified synthetic air 0.90 ≤ Rf ≤ 1.10

Toluene and purified synthetic air 0.90 ≤ Rf ≤ 1.10

These values are relative to the response factor (Rf) of 1.00 for propane and purified synthetic air.

1.8.3.	Oxygen interference check

The oxygen interference check shall be determined when introducing an analyser into service and after major service intervals.

The response factor is defined and shall be determined as described in paragraph 1.8.2.  The test gas to be used and the recommended relative response factor range are as follows:

Propane and nitrogen 0.95 ≤ Rf ≤ 1.05

This value is relative to the response factor (Rf) of 1.00 for propane and purified synthetic air.

The FID burner air oxygen concentration shall be within ± 1 mole per cent of the oxygen concentration of the burner air used in the latest oxygen interference check.  If the difference is greater, the oxygen interference shall be checked and the analyser adjusted, if necessary.

1.8.4.	Efficiency of the non-methane cutter (NMC, for NG fuelled gas engines only)

The NMC is used for the removal of the non-methane hydrocarbons from the sample gas by oxidizing all hydrocarbons except methane.  Ideally, the conversion for methane is 0 per cent, and for the other hydrocarbons represented by ethane is 100 per cent.  For the accurate measurement of NMHC, the two efficiencies shall be determined and used for the calculation of the NMHC emission mass flow rate (see Annex III, Appendix 2, paragraph 4.3.).

1.8.4.1.	Methane efficiency

Methane calibration gas shall be flown through the FID with and without bypassing the NMC and the two concentrations recorded.  The efficiency shall be determined as follows:

EM=1−cHC(w/cutter)cHC(w/o cutter)

where,

| cw | = HC concentration with CH4 flowing through the NMC |
| --- | --- |
| cw/o | = HC concentration with CH4 bypassing the NMC |

1.8.4.2.	Ethane efficiency

Ethane calibration gas shall be flown through the FID with and without bypassing the NMC and the two concentrations recorded.  The efficiency shall be determined as follows

EE=1−cHC(w/cutter)cHC(w/o cutter)

where,

| cw | = HC concentration with C2H6 flowing through the NMC |
| --- | --- |
| cw/o | = HC concentration with C2H6 bypassing the NMC |

1.9.	Interference effects with CO, CO2, and NOx analysers

Gases present in the exhaust other than the one being analysed can interfere with the reading in several ways.  Positive interference occurs in NDIR instruments where the interfering gas gives the same effect as the gas being measured, but to a lesser degree.  Negative interference occurs in NDIR instruments by the interfering gas broadening the absorption band of the measured gas, and in CLD instruments by the interfering gas quenching the radiation.  The interference checks in paragraphs 1.9.1. and 1.9.2. shall be performed prior to an analyser's initial use and after major service intervals.

1.9.1.	CO analyser interference check

Water and CO2 can interfere with the CO analyser performance.  Therefore, a CO2 span gas having a concentration of 80 to 100 per cent of full scale of the maximum operating range used during testing shall be bubbled through water at room temperature and the analyser response recorded.  The analyser response shall not be more than 1 per cent of full scale for ranges equal to or above 300 ppm or more than 3 ppm for ranges below 300 ppm.

1.9.2.	NOx analyser quench checks

The two gases of concern for CLD (and HCLD) analysers are CO2 and water vapour.  Quench responses to these gases are proportional to their concentrations, and therefore require test techniques to determine the quench at the highest expected concentrations experienced during testing.

1.9.2.1.	CO2 quench check

A CO2 span gas having a concentration of 80 to 100 per cent of full scale of the maximum operating range shall be passed through the NDIR analyser and the CO2 value recorded as A.  It shall then be diluted approximately 50 per cent with NO span gas and passed through the NDIR and (H)CLD, with the CO2 and NO values recorded as B and C, respectively.  The CO2 shall then be shut off and only the NO span gas be passed through the (H)CLD and the NO value recorded as D.

The quench, which shall not be greater than 3 per cent of full scale, shall be calculated as follows:

with:

| A | is the undiluted CO2 concentration measured with NDIR in per cent |
| --- | --- |
| B | is the diluted CO2 concentration measured with NDIR in per cent |
| C | is the diluted NO concentration measured with (H)CLD in ppm |
| D | is the undiluted NO concentration measured with (H)CLD in ppm |

Alternative methods of diluting and quantifying of CO2 and NO span gas values such as dynamic mixing/blending can be used.

1.9.2.2.	Water quench check

This check applies to wet gas concentration measurements only.  Calculation of water quench shall consider dilution of the NO span gas with water vapour and scaling of water vapour concentration of the mixture to that expected during testing.

A NO span gas having a concentration of 80 to 100 per cent of full scale of the normal operating range shall be passed through the (H)CLD and the NO value recorded as D.  The NO span gas shall then be bubbled through water at room temperature and passed through the (H)CLD and the NO value recorded as C.  The analyser's absolute operating pressure and the water temperature shall be determined and recorded as E and F, respectively.  The mixture's saturation vapour pressure that corresponds to the bubbler water temperature F shall be determined and recorded as G.  The water vapour concentration (H, in per cent) of the mixture shall be calculated as follows:

H = 100  (G/E)

The expected diluted NO span gas (in water vapour) concentration (De) shall be calculated as follows:

De = D  (1 − H/100)

For diesel exhaust, the maximum exhaust water vapour concentration (Hm, in per cent) expected during testing shall be estimated, under the assumption of a fuel atom H/C ratio of 1.8:1, from the undiluted CO2 span gas concentration (A, as measured in paragraph 1.9.2.1.) as follows:

Hm = 0.9  A

The water quench, which shall not be greater than 3 per cent, shall be calculated as follows:

per cent quench = 100  ((De − C)/De)  (Hm/H)

where

| De | = is the expected diluted NO concentration in ppm |
| --- | --- |
| C | = is the diluted NO concentration in ppm |
| Hm | = is the maximum water vapour concentration in per cent |
| H | = is the actual water vapour concentration in per cent |

Note:	It is important that the NO span gas contains minimal NO2 concentration for this check, since absorption of NO2 in water has not been accounted for in the quench calculations.

1.10.	Calibration intervals

The analysers shall be calibrated according to paragraph 1.5. at least every 3 months or whenever a system repair or change is made that could influence calibration.

2.	CALIBRATION OF THE CVS-SYSTEM

2.1.	General

The CVS system shall be calibrated by using an accurate flowmeter traceable to national or international standards and a restricting device.  The flow through the system shall be measured at different restriction settings, and the control parameters of the system shall be measured and related to the flow.

Various types of flowmeters may be used, e.g. calibrated venturi, calibrated laminar flowmeter, calibrated turbinemeter.

2.2.	Calibration of the positive displacement pump (PDP)

All parameters related to the pump shall be simultaneously measured with the parameters related to the flowmeter which is connected in series with the pump.  The calculated flow rate (in m3/min at pump inlet, absolute pressure and temperature) shall be plotted versus a correlation function which is the value of a specific combination of pump parameters.  The linear equation which relates the pump flow and the correlation function shall then be determined.  If a CVS has a multiple speed drive, the calibration shall be performed for each range used.  Temperature stability shall be maintained during calibration.

2.2.1.	Data analysis

The air flowrate (Qs) at each restriction setting (minimum 6 settings) shall be calculated in standard m3/min from the flowmeter data using the manufacturer's prescribed method. The air flow rate shall then be converted to pump flow (V0) in m3/rev at absolute pump inlet temperature and pressure as follows:

V0=qvCVSn×T273×101.3pp

where,

| qvCVS | = air flow rate at standard conditions (101.3 kPa, 273 K), m3/s |
| --- | --- |
| T | = temperature at pump inlet, K |
| pp | = absolute pressure at pump inlet (pB-p1), kPa |
| n | = pump speed, rev/s |

To account for the interaction of pressure variations at the pump and the pump slip rate, the correlation function (X0) between pump speed, pressure differential from pump inlet to pump outlet and absolute pump outlet pressure shall be calculated as follows:

X0=1n×Δpppp

where,

| Δpp | pressure differential from pump inlet to pump outlet, kPa |
| --- | --- |
| pp | absolute outlet pressure at pump outlet, kPa |

A linear least-square fit shall be performed to generate the calibration equation as follows:

V0=D0−m×X0

D0 and m are the intercept and slope constants, respectively, describing the regression lines.

For a CVS system with multiple speeds, the calibration curves generated for the different pump flow ranges shall be approximately parallel, and the intercept values (D0) shall increase as the pump flow range decreases.

The calculated values from the equation shall be within ± 0,5 per cent of the measured value of V0.  Values of m will vary from one pump to another.  Particulate influx over time will cause the pump slip to decrease, as reflected by lower values for m.  Therefore, calibration shall be performed at pump start-up, after major maintenance, and if the total system verification (paragraph 2.4.) indicates a change of the slip rate.

2.3.	Calibration of the critical flow venturi (CFV)

Calibration of the CFV is based upon the flow equation for a critical venturi.  Gas flow is a function of inlet pressure and temperature.

2.3.1.	Data analysis

The air flowrate (Qs) at each restriction setting (minimum 8 settings) shall be calculated in standard m3/min from the flowmeter data using the manufacturer's prescribed method. The calibration coefficient shall be calculated from the calibration data for each setting as follows:

Kv=qvCVS×Tpp

where,

| qvCVS | = air flow rate at standard conditions (101.3 kPa, 273 K), m3/s |
| --- | --- |
| T | = temperature at the venturi inlet, K |
| pp | = absolute pressure at venturi inlet, kPa |

To determine the range of critical flow, Kv shall be plotted as a function of venturi inlet pressure.  For critical (choked) flow, Kv will have a relatively constant value.  As pressure decreases (vacuum increases), the venturi becomes unchoked and Kv decreases, which indicates that the CFV is operated outside the permissible range.

For a minimum of eight points in the region of critical flow, the average Kv and the standard deviation shall be calculated.  The standard deviation shall not exceed ± 0,3 per cent of the average KV.

2.4.	Calibration of the subsonic venturi (SSV)

Calibration of the SSV is based upon the flow equation for a subsonic venturi.  Gas flow is a function of inlet pressure and temperature, pressure drop between the SSV inlet and throat.

2.4.1.	Data analysis

The air flowrate (QSSV) at each restriction setting (minimum 16 settings) shall be calculated in standard m3/min from the flowmeter data using the manufacturer's prescribed method.  The discharge coefficient shall be calculated from the calibration data for each setting as follows:

where:

QSSV	=	air flow rate at standard conditions (101.3 kPa, 273 K), m3/s

T	=	temperature at the venturi inlet, K

d	=	diameter of the SSV throat, m

rp	=	ratio of the SSV throat to inlet absolute, static pressure = 1−Δppp

rD	=	ratio of the SSV throat diameter, d, to the inlet pipe inner diameter D

To determine the range of subsonic flow, Cd shall be plotted as a function of Reynolds number at the SSV throat.  The Re at the SSV throat is calculated with the following formula:

where:

A1	=	a collection of constants and units conversions

= 25.55152  1m3 mins mmm

QSSV	=	air flow rate at standard conditions (101.3 kPa, 273 K), m3/s

d	=	diameter of the SSV throat, m

μ	=	absolute or dynamic viscosity of the gas, calculated with the following formula:

μ=b×T1.5S+T kg/m-s

b	=	empirical constant	= 1.458 x 106, kg/ms K0.5

S	=	empirical constant	= 110.4 K

Because QSSV is an input to the Re formula, the calculations shall be started with an initial guess for QSSV or Cd of the calibration venturi, and repeated until QSSV converges. The convergence method shall be accurate to 0.1 per cent of point or better.

For a minimum of sixteen points in the region of subsonic flow, the calculated values of Cd from the resulting calibration curve fit equation shall be within ± 0.5 per cent of the measured Cd for each calibration point.

2.5.	Total system verification

The total accuracy of the CVS sampling system and analytical system shall be determined by introducing a known mass of a pollutant gas into the system while it is being operated in the normal manner.  The pollutant is analysed, and the mass calculated according to Annex 4A, Appendix 2, paragraph 4.3. except in the case of propane where a factor of 0.000472 is used in place of 0.000479 for HC.  Either of the following two techniques shall be used.

2.5.1.	Metering with a critical flow orifice

A known quantity of pure gas (carbon monoxide or propane) shall be fed into the CVS system through a calibrated critical orifice.  If the inlet pressure is high enough, the flow rate, which is adjusted by means of the critical flow orifice, is independent of the orifice outlet pressure (≡ critical flow).  The CVS system shall be operated as in a normal exhaust emission test for about 5 to 10 minutes.  A gas sample shall be analysed with the usual equipment (sampling bag or integrating method), and the mass of the gas calculated.  The mass so determined shall be within ± 3 per cent of the known mass of the gas injected.

2.5.2.	Metering by means of a gravimetric technique

The weight of a small cylinder filled with carbon monoxide or propane shall be determined with a precision of ± 0.01 gram.  For about 5 to 10 minutes, the CVS system shall be operated as in a normal exhaust emission test, while carbon monoxide or propane is injected into the system.  The quantity of pure gas discharged shall be determined by means of differential weighing.  A gas sample shall be analysed with the usual equipment (sampling bag or integrating method), and the mass of the gas calculated.  The mass so determined shall be within ± 3 per cent of the known mass of the gas injected.

3.	CALIBRATION OF THE PARTICULATE MEASURING SYSTEM

3.1.	Introduction

The calibration of the particulate measurement is limited to the flow meters used to determine sample flow and dilution ratio.  Each flow meter shall be calibrated as often as necessary to fulfil the accuracy requirements of this Regulation.  The calibration method that shall be used is described in paragraph 3.2.

3.2.	Flow measurement

3.2.1.	Periodical calibration

- (a)	To fulfil the absolute accuracy of the flow measurements as specified in paragraph 2.2. of Appendix 4 to this annex, the flow meter or the flow measurement instrumentation shall be calibrated with an accurate flow meter traceable to international and/or national standards.

- (b)	If the sample gas flow is determined by differential flow measurement the flow meter or the flow measurement instrumentation shall be calibrated in one of the following procedures, such that the probe flow qmp into the tunnel shall fulfil the accuracy requirements of paragraph 4.2.5.2. of Appendix 4 to this annex:

(i)	The flow meter for qmdw shall be connected in series to the flow meter for qmdew, the difference between the two flow meters shall be calibrated for at least 5 set points with flow values equally spaced between the lowest qmdw value used during the test and the value of qmdew used during the test.  The dilution tunnel may be bypassed.

(ii)	A calibrated mass flow device shall be connected in series to the flowmeter for qmdew and the accuracy shall be checked for the value used for the test.  Then the calibrated mass flow device shall be connected in series to the flow meter for qmdw, and the accuracy shall be checked for at least 5 settings corresponding to dilution ratio between 3 and 50, relative to qmdew used during the test.

(iii)	The transfer tube TT shall be disconnected from the exhaust, and a calibrated flow measuring device with a suitable range to measure qmp shall be connected to the transfer tube.  Then qmdew shall be set to the value used during the test, and qmdw shall be sequentially set to at least 5 values corresponding to dilution ratios q between 3 and 50.  Alternatively, a special calibration flow path, may be provided, in which the tunnel is bypassed, but the total and dilution air flow through the corresponding meters as in the actual test.

(iv)	A tracer gas, shall be fed into the exhaust transfer tube TT.  This tracer gas may be a component of the exhaust gas, like CO2 or NOx.  After dilution in the tunnel the tracer gas component shall be measured.  This shall be carried out for 5 dilution ratios between 3 and 50.  The accuracy of the sample flow shall be determined from the dilution ratio rd:

- (c)	The accuracies of the gas analysers shall be taken into account to guarantee the accuracy of qmp.

3.2.2.	Carbon flow check

- (a)	A carbon flow check using actual exhaust is recommended for detecting measurement and control problems and verifying the proper operation of the partial flow system.  The carbon flow check should be run at least each time a new engine is installed, or something significant is changed in the test cell configuration.

- (b)	The engine shall be operated at peak torque load and speed or any other steady state mode that produces 5 per cent or more of CO2.  The partial flow sampling system shall be operated with a dilution factor of about 15 to 1.

- (c)	If a carbon flow check is conducted, the procedure given in Appendix 6 to this annex shall be applied.  The carbon flow rates shall be calculated according to paragraphs 2.1. to 2.3. of Appendix 6 to this annex.  All carbon flow rates should agree to within 6 per cent of each other.

3.2.3.	Pre-test check

- (a)	A pre-test check shall be performed within 2 hours before the test run in the following way:

- (b)	The accuracy of the flow meters shall be checked by the same method as used for calibration (see paragraph 3.2.1. of this appendix) for at least two points, including flow values of qmdw that correspond to dilution ratios between 5 and 15 for the qmdew value used during the test.

- (c)	If it can be demonstrated by records of the calibration procedure under paragraph 3.2.1. that the flow meter calibration is stable over a longer period of time, the pre-test check may be omitted.

3.3.	Determination of transformation time (for partial flow dilution systems on ETC only)

- (a)	The system settings for the transformation time evaluation shall be exactly the same as during measurement of the test run.  The transformation time shall be determined by the following method:

- (b)	An independent reference flowmeter with a measurement range appropriate for the probe flow shall be put in series with and closely coupled to the probe.  This flowmeter shall have a transformation time of less than 100 ms for the flow step size used in the response time measurement, with flow restriction sufficiently low as to not affect the dynamic performance of the partial flow dilution system, and consistent with good engineering practice.

- (c)	A step change shall be introduced to the exhaust flow (or air flow if exhaust flow is calculated) input of the partial flow dilution system, from a low flow to at least 90 per cent of full scale.  The trigger for the step change should be the same one used to start the look-ahead control in actual testing.  The exhaust flow step stimulus and the flowmeter response shall be recorded at a sample rate of at least 10 Hz.

- (d)	From this data, the transformation time shall be determined for the partial flow dilution system, which is the time from the initiation of the step stimulus to the 50 per cent point of the flowmeter response.  In a similar manner, the transformation times of the qmp signal of the partial flow dilution system and of the qmew,i signal of the exhaust flow meter shall be determined.  These signals are used in the regression checks performed after each test (see paragraph 3.8.3.2. of Appendix 2 to this annex).

- (e)	The calculation shall be repeated for at least 5 rise and fall stimuli, and the results shall be averaged.  The internal transformation time (<100 msec) of the reference flowmeter shall be subtracted from this value.  This is the "look-ahead" value of the partial flow dilution system, which shall be applied in accordance with paragraph 3.8.3.2. of Appendix 2 to this annex.

- 3.4.	Checking the partial flow conditions

The range of the exhaust gas velocity and the pressure oscillations shall be checked and adjusted according to the requirements of paragraph 2.2.1. of Appendix 7 (EP), if applicable.

3.5.	Calibration intervals

The flow measurement instrumentation shall be calibrated at least every 3 months or whenever a system repair or change is made that could influence calibration.

4.	CALIBRATION OF THE SMOKE MEASUREMENT EQUIPMENT

4.1.	Introduction

The opacimeter shall be calibrated as often as necessary to fulfil the accuracy requirements of this Regulation.  The calibration method to be used is described in this paragraph for the components indicated in Appendix 4, paragraph 5 and Appendix 7, paragraph 3. to this annex.

4.2.	Calibration procedure

4.2.1.	Warming-up time

The opacimeter shall be warmed up and stabilised according to the manufacturer's recommendations.  If the opacimeter is equipped with a purge air system to prevent sooting of the instrument optics, this system should also be activated and adjusted according to the manufacturer's recommendations.

4.2.2.	Establishment of the linearity response

The linearity of the opacimeter shall be checked in the opacity readout mode as per the manufacturer's recommendations.  Three neutral density filters of known transmittance, which shall meet the requirements of Appendix 4, paragraph 5.2.5. to this annex, shall be introduced to the opacimeter and the value recorded.  The neutral density filters shall have nominal opacities of approximately 10 per cent, 20 per cent and 40 per cent.

The linearity shall not differ by more than ± 2 per cent opacity from the nominal value of the neutral density filter.  Any non-linearity exceeding the above value shall be corrected prior to the test.

4.3.	Calibration intervals

The opacimeter shall be calibrated according to paragraph 4.2.2. at least every 3 months or whenever a system repair or change is made that could influence calibration.

Appendix 6

CARBON FLOW CHECK

1.	INTRODUCTION

All but a tiny part of the carbon in the exhaust comes from the fuel, and all but a minimal part of this is manifest in the exhaust gas as CO2.  This is the basis for a system verification check based on CO2 measurements.

The flow of carbon into the exhaust measurement systems is determined from the fuel flow rate.  The flow of carbon at various sampling points in the emissions and particulate sampling systems is determined from the CO2 concentrations and gas flow rates at those points.

In this sense, the engine provides a known source of carbon flow, and observing the same carbon flow in the exhaust pipe and at the outlet of the partial flow PM sampling system verifies leak integrity and flow measurement accuracy.  This check has the advantage that the components are operating under actual engine test conditions of temperature and flow.

The following diagram shows the sampling points at which the carbon flows shall be checked. The specific equations for the carbon flows at each of the sample points are given below.

Figure 7

Measuring points for carbon flow check

Air     Fuel

ENGINE

CO

2

RAW

CO

2

PFS

Partial Flow System

1

2

3

2.	CALCULATIONS

2.1.	Carbon flow rate into the engine (location 1)

The carbon mass flow rate into the engine for a fuel CHαOε is given by:

where:

qmf	=	fuel mass flow rate, kg/s

2.2.	Carbon flow rate in the raw exhaust (location 2)

The carbon mass flow rate in the exhaust pipe of the engine shall be determined from the raw CO2 concentration and the exhaust gas mass flow rate:

where:

cCO2,r	=	wet CO2 concentration in the raw exhaust gas, per cent

cCO2,a	=	wet CO2 concentration in the ambient air, per cent (around 0.04 per cent)

qmew	=	exhaust gas mass flow rate on wet basis, kg/s

Mre	=	molecular mass of exhaust gas

If CO2 is measured on a dry basis it shall be converted to a wet basis according to paragraph 5.2. of Appendix 1 to this annex.

2.3.	Carbon flow rate in the dilution system (location 3)

The carbon flow rate shall be determined from the dilute CO2 concentration, the exhaust gas mass flow rate and the sample flow rate:

where:

cCO2,d	=	wet CO2 concentration in the dilute exhaust gas at the outlet of the dilution tunnel, per cent

cCO2,a	=	wet CO2 concentration in the ambient air, per cent (around 0.04 per cent)

qmdew	=	diluted exhaust gas mass flow rate on wet basis, kg/s

qmew	=	exhaust gas mass flow rate on wet basis, kg/s (partial flow system only)

qmp	=	sample flow of exhaust gas into partial flow dilution system, kg/s (partial flow system only)

Mre	=	molecular mass of exhaust gas

If CO2 is measured on a dry basis, it shall be converted to wet basis according to paragraph 5.2. of Appendix 1 to this annex.

2.4.	The molecular mass (Mre) of the exhaust gas shall be calculated as follows:

where:

qmf	=	fuel mass flow rate, kg/s

qmaw	=	intake air mass flow rate on wet basis, kg/s

Ha	=	humidity of intake air, g water per kg dry air

Mra	=	molecular mass of dry intake air (= 28.9 g/mol)

, , , 	= molar ratios referring to a fuel C H O N S

Alternatively, the following molecular masses may be used:

Mre (diesel)	=	28.9 g/mol

Mre (LPG)	=	28.6 g/mol

Mre (NG)	=	28.3 g/mol

Appendix 7

ANALYTICAL AND SAMPLING SYSTEMS

1.	DETERMINATION OF THE GASEOUS EMISSIONS

1.1.	Introduction

Paragraph 1.2. and figures 7 and 8 contain detailed descriptions of the recommended sampling and analysing systems.  Since various configurations can produce equivalent results, exact conformance with figures 7 and 8 is not required.  Additional components such as instruments, valves, solenoids, pumps, and switches may be used to provide additional information and co-ordinate the functions of the component systems.  Other components which are not needed to maintain the accuracy on some systems, may be excluded if their exclusion is based upon good engineering judgement.

Figure 7

Flow diagram of raw exhaust gas analysis system for CO, CO2, NOx, HC ESC only

1.2.	Description of the analytical system

An analytical system for the determination of the gaseous emissions in the raw (figure 7, ESC only) or diluted (figure 8, ETC and ESC) exhaust gas is described based on the use of:

- (a)	HFID analyser for the measurement of hydrocarbons;

- (b)	NDIR analysers for the measurement of carbon monoxide and carbon dioxide;

- (c)	HCLD or equivalent analyser for the measurement of the oxides of nitrogen;

The sample for all components may be taken with one sampling probe or with two sampling probes located in close proximity and internally split to the different analysers.  Care shall be taken that no condensation of exhaust components (including water and sulphuric acid) occurs at any point of the analytical system.

Figure 8

Flow diagram of diluted exhaust gas analysis system for

CO, CO2, NOx, HC ETC, optional for ESC

1.2.1.	Components of figures 7 and 8

EP exhaust pipe

Exhaust gas sampling probe (figure 7 only)

A stainless steel straight closed end multi-hole probe is recommended.  The inside diameter shall not be greater than the inside diameter of the sampling line.  The wall thickness of the probe shall not be greater than 1 mm.  There shall be a minimum of 3 holes in 3 different radial planes sized to sample approximately the same flow.  The probe shall extend across at least 80 per cent of the diameter of the exhaust pipe.  One or two sampling probes may be used.

SP2 diluted exhaust gas HC sampling probe (figure 8 only)

The probe shall:

- (a)	be defined as the first 254 mm to 762 mm of the heated sampling line HSL1;

- (b)	have a 5 mm minimum inside diameter;

- (c)	be installed in the dilution tunnel DT (see paragraph 2.3., figure 20) at a point where the dilution air and exhaust gas are well mixed (i.e. approximately 10 tunnel diameters downstream of the point where the exhaust enters the dilution tunnel);

- (d)	be sufficiently distant (radially) from other probes and the tunnel wall so as to be free from the influence of any wakes or eddies;

- (e)	be heated so as to increase the gas stream temperature to 463 K ± 10 K (190 °C ± 10 °C) at the exit of the probe.

SP3 diluted exhaust gas CO, CO2, NOx sampling probe (figure 8 only)

The probe shall:

- (a)	be in the same plane as SP 2;

- (b)	be sufficiently distant (radially) from other probes and the tunnel wall so as to be free from the influence of any wakes or eddies;

- (c)	be heated and insulated over its entire length to a minimum temperature of 328 K (55 °C) to prevent water condensation.

HSL1 heated sampling line

The sampling line provides a gas sample from a single probe to the split point(s) and the HC analyser.

The sampling line shall:

- (a)	have a 5 mm minimum and a 13,5 mm maximum inside diameter;

- (b)	be made of stainless steel or PTFE.

- (c)	maintain a wall temperature of 463 K ± 10 K (190 °C ± 10 °C) as measured at every separately controlled heated section, if the temperature of the exhaust gas at the sampling probe is equal to or below 463 K (190 °C);

- (d)	maintain a wall temperature greater than 453 K (180 °C), if the temperature of the exhaust gas at the sampling probe is above 463 K (190 °C);

- (e)	maintain a gas temperature of 463 K ± 10 K (190 °C  ± 10 °C) immediately before the heated filter F2 and the HFID;

HSL2 heated NOx sampling line

The sampling line shall:

- (a)	maintain a wall temperature of 328 K to 473 K (55 °C to 200 °C), up to the converter C when using a cooling bath B, and up to the analyser when a cooling bath B is not used.

- (b)	be made of stainless steel or PTFE.

SL sampling line for CO and CO2

The line shall be made of PTFE or stainless steel.  It may be heated or unheated.

BK background bag (optional; figure 8 only)

For the sampling of the background concentrations.

BG sample bag (optional; figure 8 CO and CO2 only)

For the sampling of the sample concentrations.

F1 heated pre-filter (optional)

The temperature shall be the same as HSL1.

F2 Heated filter

The filter shall extract any solid particles from the gas sample prior to the analyser.  The temperature shall be the same as HSL1.  The filter shall be changed as needed.

P heated sampling pump

The pump shall be heated to the temperature of HSL1.

HC

Heated flame ionisation detector (HFID) for the determination of the hydrocarbons. The temperature shall be kept at 453 K to 473 K (180 °C to 200 °C).

CO, CO2

NDIR analysers for the determination of carbon monoxide and carbon dioxide (optional for the determination of the dilution ratio for PT measurement).

NO

CLD or HCLD analyser for the determination of the oxides of nitrogen.  If a HCLD is used it shall be kept at a temperature of 328 K to 473 K (55 °C to 200 °C).

C converter

A converter shall be used for the catalytic reduction of NO2 to NO prior to analysis in the CLD or HCLD.

B cooling bath (optional)

To cool and condense water from the exhaust sample.  The bath shall be maintained at a temperature of 273 K to 277 K (0 °C to 4 °C) by ice or refrigeration.  It is optional if the analyser is free from water vapour interference as determined in paragraphs 1.9.1. and 1.9.2. of Appendix 5 to this annex.  If water is removed by condensation, the sample gas temperature or dew point shall be monitored either within the water trap or downstream. The sample gas temperature or dew point shall not exceed 280 K (7 °C).  Chemical dryers are not allowed for removing water from the sample.

T1, T2, T3 temperature sensor

To monitor the temperature of the gas stream.

T4 temperature sensor

To monitor the temperature of the NO2-NO converter.

T5 temperature sensor

To monitor the temperature of the cooling bath.

G1, G2, G3 pressure gauge

To measure the pressure in the sampling lines.

R1, R2 pressure regulator

To control the pressure of the air and the fuel, respectively, for the HFID.

R3, R4, R5 pressure regulator

To control the pressure in the sampling lines and the flow to the analysers.

FL1, FL2, FL3 flowmeter

To monitor the sample by-pass flow rate.

FL4 to FL6 flowmeter (optional)

To monitor the flow rate through the analysers.

V1 to V5 selector valve

Suitable valving for selecting sample, span gas or zero gas flow to the analysers.

V6, V7 solenoid valve

To by-pass the NO2-NO converter.

V8 needle valve

To balance the flow through the NO2-NO converter C and the by-pass.

V9, V10 needle valve

To regulate the flows to the analysers.

V11, V12 Toggle valve (optional)

To drain the condensate from the bath B.

1.3.	NMHC analysis (NG fuelled gas engines only)

1.3.1.	Gas chromatographic method (GC, figure 9)

When using the GC method, a small measured volume of a sample is injected onto an analytical column through which it is swept by an inert carrier gas.  The column separates various components according to their boiling points so that they elute from the column at different times.  They then pass through a detector which gives an electrical signal that depends on their concentration.  Since it is not a continuous analysis technique, it can only be used in conjunction with the bag sampling method as described in paragraph 3.4.2. of Appendix 4 to this annex.

For NMHC an automated GC with a FID shall be used.  The exhaust gas shall be sampled into a sampling bag from which a part shall be taken and injected into the GC.  The sample is separated into two parts (CH4/Air/CO and NMHC/CO2/H2O) on the Porapak column.  The molecular sieve column separates CH4 from the air and CO before passing it to the FID where its concentration is measured.  A complete cycle from injection of one sample to injection of a second can be made in 30 s.  To determine NMHC, the CH4 concentration shall be subtracted from the total HC concentration (see paragraph 4.3.1. of Appendix 2 to this annex).

Figure 9 shows a typical GC assembled to routinely determine CH4.  Other GC methods can also be used based on good engineering judgement.

Figure 9

Flow diagram for methane analysis (GC method)

Components of figure 9

PC Porapak column

Porapak N, 180/300 μm (50/80 mesh), 610 mm length × 2.16 mm ID shall be used and conditioned at least 12 h at 423 K (150 °C) with carrier gas prior to initial use.

MSC Molecular sieve column

Type 13X, 250/350 μm (45/60 mesh), 1220 mm length × 2.16 mm ID shall be used and conditioned at least 12 h at 423 K (150 °C) with carrier gas prior to initial use.

OV Oven

To maintain columns and valves at stable temperature for analyser operation, and to condition the columns at 423 K (150 °C).

SLP Sample loop

A sufficient length of stainless steel tubing to obtain approximately 1 cm3 volume.

P Pump

To bring the sample to the gas chromatograph.

D Dryer

A dryer containing a molecular sieve shall be used to remove water and other contaminants which might be present in the carrier gas.

HC

Flame ionisation detector (FID) to measure the concentration of methane.

V1 Sample injection valve

To inject the sample taken from the sampling bag via SL of figure 8.  It shall be low dead volume, gas tight, and heatable to 423 K (150 °C).

V3 Selector valve

To select span gas, sample, or no flow.

V2, V4, V5, V6, V7, V8 Needle valve

To set the flows in the system.

R1, R2, R3 Pressure regulator

To control the flows of the fuel (= carrier gas), the sample, and the air, respectively.

FC Flow capillary

To control the rate of air flow to the FID

G1, G2, G3 Pressure gauge

To control the flows of the fuel (= carrier gas), the sample, and the air, respectively.

F1, F2, F3, F4, F5 Filter

Sintered metal filters to prevent grit from entering the pump or the instrument.

FL1

To measure the sample by-pass flow rate.

1.3.2.	Non-Methane Cutter Method (NMC, figure 10)

The cutter oxidizes all hydrocarbons except CH4 to CO2 and H2O, so that by passing the sample through the NMC only CH4 is detected by the FID.  If bag sampling is used, a flow diverter system shall be installed at SL (see paragraph 1.2., figure 8) with which the flow can be alternatively passed through or around the cutter according to the upper part of figure 10.  For NMHC measurement, both values (HC and CH4) shall be observed on the FID and recorded.  If the integration method is used, an NMC in line with a second FID shall be installed parallel to the regular FID into HSL1 (see paragraph 1.2., figure 8) according to the lower part of figure 10.  For NMHC measurement, the values of the two FID's (HC and CH4) shall be observed and recorded.

The cutter shall be characterized at or above 600 K (327 °C) prior to test work with respect to its catalytic effect on CH4 and C2H6 at H2O values representative of exhaust stream conditions.  The dewpoint and O2 level of the sampled exhaust stream shall be known.  The relative response of the FID to CH4 shall be recorded (see paragraph 1.8.2. of Appendix 5 to this annex).

Figure 10

Flow diagram for methane analysis with the non-methane cutter (NMC)

Components of figure 10

NMC Non-methane cutter

To oxidize all hydrocarbons except methane.

HC

Heated flame ionisation detector (HFID) to measure the HC and CH4 concentrations.  The temperature shall be kept at 453 K to 473 K (180 °C to 200 °C).

V1 Selector valve

To select sample, zero and span gas.  V1 is identical with V2 of figure 8.

V2, V3 Solenoid valve

To by-pass the NMC.

V4 Needle valve

To balance the flow through the NMC and the by-pass.

R1 Pressure regulator

To control the pressure in the sampling line and the flow to the HFID.  R1 is identical with R3 of figure 8.

FL1 Flowmeter

To measure the sample by-pass flow rate.  FL1 is identical with FL1 of figure 8.

2.	EXHAUST GAS DILUTION AND DETERMINATION OF THE PARTICULATES

2.1.	Introduction

Paragraphs 2.2., 2.3. and 2.4. and figures 11 to 22 contain detailed descriptions of the recommended dilution and sampling systems.  Since various configurations can produce equivalent results, exact conformance with these figures is not required.  Additional components such as instruments, valves, solenoids, pumps, and switches may be used to provide additional information and coordinate the functions of the component systems.  Other components which are not needed to maintain the accuracy on some systems, may be excluded if their exclusion is based upon good engineering judgement.

2.2.	Partial Flow Dilution System

A dilution system is described in figures 11 to 19 based upon the dilution of a part of the exhaust stream.  Splitting of the exhaust stream and the following dilution process may be done by different dilution system types.  For subsequent collection of the particulates, the entire dilute exhaust gas or only a portion of the dilute exhaust gas is passed to the particulate sampling system (paragraph 2.4., figure 21).  The first method is referred to as total sampling type, the second method as fractional sampling type.

The calculation of the dilution ratio depends upon the type of system used. The following types are recommended:

Isokinetic systems (figures 11, 12)

With these systems, the flow into the transfer tube is matched to the bulk exhaust flow in terms of gas velocity and/or pressure, thus requiring an undisturbed and uniform exhaust flow at the sampling probe.  This is usually achieved by using a resonator and a straight approach tube upstream of the sampling point.  The split ratio is then calculated from easily measurable values like tube diameters.  It should be noted that isokinesis is only used for matching the flow conditions and not for matching the size distribution.  The latter is typically not necessary, as the particles are sufficiently small as to follow the fluid streamlines.

Flow controlled systems with concentration measurement (figures 13 to 17)

With these systems, a sample is taken from the bulk exhaust stream by adjusting the dilution air flow and the total dilute exhaust flow.  The dilution ratio is determined from the concentrations of tracer gases, such as CO2 or NOx naturally occurring in the engine exhaust.  The concentrations in the dilute exhaust gas and in the dilution air are measured, whereas the concentration in the raw exhaust gas can be either measured directly or determined from fuel flow and the carbon balance equation, if the fuel composition is known.  The systems may be controlled by the calculated dilution ratio (figures 13 and 14) or by the flow into the transfer tube (figures 12, 13 and 14).

Flow controlled systems with flow measurement (figures 18 and 19)

With these systems, a sample is taken from the bulk exhaust stream by setting the dilution air flow and the total dilute exhaust flow.  The dilution ratio is determined from the difference of the two flows rates.  Accurate calibration of the flow meters relative to one another is required, since the relative magnitude of the two flow rates can lead to significant errors at higher dilution ratios (of 15 and above).  Flow control is very straight forward by keeping the dilute exhaust flow rate constant and varying the dilution air flow rate, if needed.

When using partial flow dilution systems, attention shall be paid to avoiding the potential problems of loss of particulates in the transfer tube, ensuring that a representative sample is taken from the engine exhaust, and determination of the split ratio.  The systems described pay attention to these critical areas.

Figure 11

Partial flow dilution system with isokinetic probe and fractional sampling (SB control)

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the transfer tube TT by the isokinetic sampling probe ISP.  The differential pressure of the exhaust gas between exhaust pipe and inlet to the probe is measured with the pressure transducer DPT.  This signal is transmitted to the flow controller FC1 that controls the suction blower SB to maintain a differential pressure of zero at the tip of the probe.  Under these conditions, exhaust gas velocities in EP and ISP are identical, and the flow through ISP and TT is a constant fraction (split) of the exhaust gas flow.  The split ratio is determined from the cross sectional areas of EP and ISP.  The dilution air flow rate is measured with the flow measurement device FM1.  The dilution ratio is calculated from the dilution air flow rate and the split ratio.

Figure 12

Partial flow dilution system with isokinetic probe and fractional sampling (PB control)

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the transfer tube TT by the isokinetic sampling probe ISP.  The differential pressure of the exhaust gas between exhaust pipe and inlet to the probe is measured with the pressure transducer DPT.  This signal is transmitted to the flow controller FC1 that controls the pressure blower PB to maintain a differential pressure of zero at the tip of the probe.  This is done by taking a small fraction of the dilution air whose flow rate has already been measured with the flow measurement device FM1, and feeding it to TT by means of a pneumatic orifice.  Under these conditions, exhaust gas velocities in EP and ISP are identical, and the flow through ISP and TT is a constant fraction (split) of the exhaust gas flow.  The split ratio is determined from the cross sectional areas of EP and ISP.  The dilution air is sucked through DT by the suction blower SB, and the flow rate is measured with FM1 at the inlet to DT.  The dilution ratio is calculated from the dilution air flow rate and the split ratio.

Figure 13

Partial flow dilution system with CO2 or NOx concentration measurement

and fractional sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT.  The concentrations of a tracer gas (CO2 or NOx) are measured in the raw and diluted exhaust gas as well as in the dilution air with the exhaust gas analyser(s) EGA.  These signals are transmitted to the flow controller FC2 that controls either the pressure blower PB or the suction blower SB to maintain the desired exhaust split and dilution ratio in DT.  The dilution ratio is calculated from the tracer gas concentrations in the raw exhaust gas, the diluted exhaust gas, and the dilution air.

Figure 14

Partial flow dilution system with CO2 concentration measurement,

carbon balance and total sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT.  The CO2 concentrations are measured in the diluted exhaust gas and in the dilution air with the exhaust gas analyser(s) EGA.  The CO2 and fuel flow GFUEL signals are transmitted either to the flow controller FC2, or to the flow controller FC3 of the particulate sampling system (see figure 21).  FC2 controls the pressure blower PB, FC3 the sampling pump P (see figure 21), thereby adjusting the flows into and out of the system so as to maintain the desired exhaust split and dilution ratio in DT.  The dilution ratio is calculated from the CO2 concentrations and GFUEL using the carbon balance assumption.

Figure 15

Partial flow dilution system with single venturi,

concentration measurement and fractional sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT due to the negative pressure created by the venturi VN in DT.  The gas flow rate through TT depends on the momentum exchange at the venturi zone, and is therefore affected by the absolute temperature of the gas at the exit of TT.  Consequently, the exhaust split for a given tunnel flow rate is not constant, and the dilution ratio at low load is slightly lower than at high load.  The tracer gas concentrations (CO2 or NOx) are measured in the raw exhaust gas, the diluted exhaust gas, and the dilution air with the exhaust gas analyser(s) EGA, and the dilution ratio is calculated from the values so measured.

Figure 16

Partial flow dilution system with twin venturi or twin orifice,

concentration measurement and fractional sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT by a flow divider that contains a set of orifices or venturis.  The first one (FD1) is located in EP, the second one (FD2) in TT. Additionally, two pressure control valves (PCV1 and PCV2) are necessary to maintain a constant exhaust split by controlling the backpressure in EP and the pressure in DT.  PCV1 is located downstream of SP in EP, PCV2 between the pressure blower PB and DT.  The tracer gas concentrations (CO2 or NOx) are measured in the raw exhaust gas, the diluted exhaust gas, and the dilution air with the exhaust gas analyser(s) EGA.  They are necessary for checking the exhaust split, and may be used to adjust PCV1 and PCV2 for precise split control.  The dilution ratio is calculated from the tracer gas concentrations.

Figure 17

Partial flow dilution system with multiple tube splitting,

concentration measurement and fractional sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the transfer tube TT by the flow divider FD3 that consists of a number of tubes of the same dimensions (same diameter, length and bend radius) installed in EP.  The exhaust gas through one of these tubes is lead to DT, and the exhaust gas through the rest of the tubes is passed through the damping chamber DC.  Thus, the exhaust split is determined by the total number of tubes.  A constant split control requires a differential pressure of zero between DC and the outlet of TT, which is measured with the differential pressure transducer DPT. A differential pressure of zero is achieved by injecting fresh air into DT at the outlet of TT. The tracer gas concentrations (CO2 or NOx) are measured in the raw exhaust gas, the diluted exhaust gas, and the dilution air with the exhaust gas analyser(s) EGA.  They are necessary for checking the exhaust split and may be used to control the injection air flow rate for precise split control.  The dilution ratio is calculated from the tracer gas concentrations.

Figure 18

Partial flow dilution system with flow control and total sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT.  The total flow through the tunnel is adjusted with the flow controller FC3 and the sampling pump P of the particulate sampling system (see figure 18).  The dilution air flow is controlled by the flow controller FC2, which may use GEXHW, GAIRW, or GFUEL as command signals, for the desired exhaust split.  The sample flow into DT is the difference of the total flow and the dilution air flow.  T he dilution air flow rate is measured with the flow measurement device FM1, the total flow rate with the flow measurement device FM3 of the particulate sampling system (see figure 21).  The dilution ratio is calculated from these two flow rates.

Figure 19

Partial flow dilution system with flow control and fractional sampling

Raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT.  The exhaust split and the flow into DT is controlled by the flow controller FC2 that adjusts the flows (or speeds) of the pressure blower PB and the suction blower SB, accordingly.  This is possible since the sample taken with the particulate sampling system is returned into DT.  GEXHW, GAIRW, or GFUEL may be used as command signals for FC2.  The dilution air flow rate is measured with the flow measurement device FM1, the total flow with the flow measurement device FM2.  The dilution ratio is calculated from these two flow rates.

2.2.1.	Components of figures 11 to 19

EP exhaust pipe

The exhaust pipe may be insulated. To reduce the thermal inertia of the exhaust pipe a thickness to diameter ratio of 0.015 or less is recommended.  The use of flexible sections shall be limited to a length to diameter ratio of 12 or less.  Bends shall be minimized to reduce inertial deposition.  If the system includes a test bed silencer the silencer may also be insulated.

For an isokinetic system, the exhaust pipe shall be free of elbows, bends and sudden diameter changes for at least 6-pipe diameters upstream and 33-pipe diameters downstream of the tip of the probe.  The gas velocity at the sampling zone shall be higher than 10 m/s except at idle mode.  Pressure oscillations of the exhaust gas shall not exceed ± 500 Pa on the average.  Any steps to reduce pressure oscillations beyond using a chassis-type exhaust system (including silencer and aftertreatment devices) shall not alter engine performance nor cause the deposition of particulates.

For systems without isokinetic probe, it is recommended to have a straight pipe of 6-pipe diameters upstream and 3-pipe diameters downstream of the tip of the probe.

SP sampling probe (figures 10, 14, 15, 16, 18 and 19)

The minimum inside diameter shall be 4 mm. The minimum diameter ratio between exhaust pipe and probe shall be 4.  The probe shall be an open tube facing upstream on the exhaust pipe centreline, or a multiple hole probe as described under SP1 in paragraph 1.2.1., figure 5.

ISP isokinetic sampling probe (figures 11 and 12)

The isokinetic sampling probe shall be installed facing upstream on the exhaust pipe centreline where the flow conditions in paragraph EP are met, and designed to provide a proportional sample of the raw exhaust gas.  The minimum inside diameter shall be 12 mm.

A control system is necessary for isokinetic exhaust splitting by maintaining a differential pressure of zero between EP and ISP.  Under these conditions exhaust gas velocities in EP and ISP are identical and the mass flow through ISP is a constant fraction of the exhaust gas flow.  ISP has to be connected to a differential pressure transducer DPT.  The control to provide a differential pressure of zero between EP and ISP is done with the flow controller FC1.

FD1, FD2 flow divider (figure 16)

A set of venturis or orifices is installed in the exhaust pipe EP and in the transfer tube TT, respectively, to provide a proportional sample of the raw exhaust gas.  A control system consisting of two pressure control valves PCV1 and PCV2 is necessary for proportional splitting by controlling the pressures in EP and DT.

FD3 flow divider (figure 17)

A set of tubes (multiple tube unit) is installed in the exhaust pipe EP to provide a proportional sample of the raw exhaust gas.  One of the tubes feeds exhaust gas to the dilution tunnel DT, whereas the other tubes exit exhaust gas to a damping chamber DC.  The tubes shall have the same dimensions (same diameter, length, bend radius), so that the exhaust split depends on the total number of tubes.  A control system is necessary for proportional splitting by maintaining a differential pressure of zero between the exit of the multiple tube unit into DC and the exit of TT.  Under these conditions, exhaust gas velocities in EP and FD3 are proportional, and the flow TT is a constant fraction of the exhaust gas flow.  The two points have to be connected to a differential pressure transducer DPT.  The control to provide a differential pressure of zero is done with the flow controller FC1.

EGA exhaust gas analyser (figures 13, 14, 15, 16 and 17)

CO2 or NOx analysers may be used (with carbon balance method CO2 only).  The analysers shall be calibrated like the analysers for the measurement of the gaseous emissions.  One or several analysers may be used to determine the concentration differences.  The accuracy of the measuring systems has to be such that the accuracy of GEDFW,i is within ± 4 per cent.

TT transfer tube (figures 11 to 19)

The transfer tube shall be:

- (a)	As short as possible, but not more than 5 m in length.

- (b)	Equal to or greater than the probe diameter, but not more than 25 mm in diameter.

- (c)	Exiting on the centreline of the dilution tunnel and pointing downstream.

If the tube is 1 meter or less in length, it shall be insulated with material with a maximum thermal conductivity of 0.05 W/m·K with a radial insulation thickness corresponding to the diameter of the probe.  If the tube is longer than 1 meter, it shall be insulated and heated to a minimum wall temperature of 523 K (250 °C).

DPT differential pressure transducer (figures 11, 12 and 17)

The differential pressure transducer shall have a range of ± 500 Pa or less.

FC1 flow controller (figures 11, 12 and 17)

For isokinetic systems (figures 11 and12), a flow controller is necessary to maintain a differential pressure of zero between EP and ISP. The adjustment can be done by:

(a)	controlling the speed or flow of the suction blower SB and keeping the speed or flow of the pressure blower PB constant during each mode (figure 11) or;

(b)	adjusting the suction blower SB to a constant mass flow of the diluted exhaust gas and controlling the flow of the pressure blower PB, and therefore the exhaust sample flow in a region at the end of the transfer tube TT (figure 12).

In the case of a pressure controlled system the remaining error in the control loop shall not exceed ± 3 Pa. The pressure oscillations in the dilution tunnel shall not exceed ± 250 Pa on the average.

For a multi tube system (figure 17), a flow controller is necessary for proportional exhaust splitting to maintain a differential pressure of zero between the exit of the multi tube unit and the exit of TT.  The adjustment is done by controlling the injection air flow rate into DT at the exit of TT.

PCV1, PCV2 pressure control valve (figure 16)

Two pressure control valves are necessary for the twin venturi/twin orifice system for proportional flow splitting by controlling the backpressure of EP and the pressure in DT.  The valves shall be located downstream of SP in EP and between PB and DT.

DC damping chamber (figure 17)

A damping chamber shall be installed at the exit of the multiple tube unit to minimize the pressure oscillations in the exhaust pipe EP.

VN venturi (figure 15)

A venturi is installed in the dilution tunnel DT to create a negative pressure in the region of the exit of the transfer tube TT.  The gas flow rate through TT is determined by the momentum exchange at the venturi zone, and is basically proportional to the flow rate of the pressure blower PB leading to a constant dilution ratio.  Since the momentum exchange is affected by the temperature at the exit of TT and the pressure difference between EP and DT, the actual dilution ratio is slightly lower at low load than at high load.

FC2 flow controller (figures 13, 14, 18 and 19, optional)

A flow controller may be used to control the flow of the pressure blower PB and/or the suction blower SB.  It may be connected to the exhaust, intake air, or fuel flow signals and/or to the CO2 or NOx differential signals.  When using a pressurised air supply (figure 18), FC2 directly controls the air flow.

FM1 flow measurement device (figures 11, 12, 18 and 19)

Gas meter or other flow instrumentation to measure the dilution air flow.  FM1 is optional if the pressure blower PB is calibrated to measure the flow.

FM2 flow measurement device (figure 19)

Gas meter or other flow instrumentation to measure the diluted exhaust gas flow.  FM2 is optional if the suction blower SB is calibrated to measure the flow.

PB dressures blower (figures 11, 12, 13, 14, 15, 16 and 19)

To control the dilution air flow rate, PB may be connected to the flow controllers FC1 or FC2.  PB is not required when using a butterfly valve.  PB may be used to to measure the dilution air flow, if calibrated.

SB suction blower (figures 11, 12, 13, 16, 17, 19)

For fractional sampling systems only.  SB may be used to measure the diluted exhaust gas flow, if calibrated.

DAF dilution air filter (figures 11 to 19)

It is recommended that the dilution air be filtered and charcoal scrubbed to eliminate background hydrocarbons.  At the engine manufacturers request the dilution air shall be sampled according to good engineering practice to determine the background particulate levels, which can then be subtracted from the values measured in the diluted exhaust.

DT Dilution tunnel (figures 11 to 19)

The dilution tunnel:

- (a)	shall be of a sufficient length to cause complete mixing of the exhaust and dilution air under turbulent flow conditions;

- (b)	shall be constructed of stainless steel with:

- (i)	thickness/diameter ratio of 0.025 or less for dilution tunnels with inside diameters greater than 75 mm;

- (ii)	a nominal thickness of no less then 1.5 mm for dilution tunnels with inside diameters of equal to or less than 75 mm;

- (c)	shall be at least 75 mm in diameter for the fractional sampling type;

- (d)	is recommended to be at least 25 mm in diameter for the total sampling type;

- (e)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust in the dilution tunnel;

- (f)	may be insulated.

The engine exhaust shall be thoroughly mixed with the dilution air.  For fractional sampling systems, the mixing quality shall be checked after introduction into service by means of a CO2 -profile of the tunnel with the engine running (at least four equally spaced measuring points).  If necessary, a mixing orifice may be used.

Note:	If the ambient temperature in the vicinity of the dilution tunnel (DT) is below 293 K (20 °C), precautions should be taken to avoid particle losses onto the cool walls of the dilution tunnel.  Therefore, heating and/or insulating the tunnel within the limits given above is recommended.

At high engine loads, the tunnel may be cooled by a non-aggressive means such as a circulating fan, as long as the temperature of the cooling medium is not below 293 K (20 °C).

HE heat exchanger (figures 16 and17)

The heat exchanger shall be of sufficient capacity to maintain the temperature at the inlet to the suction blower SB within ± 11 K of the average operating temperature observed during the test.

2.3.	Full flow dilution system

A dilution system is described in figure 20 based upon the dilution of the total exhaust using the CVS (constant volume dampling) concept.  The total volume of the mixture of exhaust and dilution air shall be measured.  Either a PDP or a CFV system may be used.

For subsequent collection of the particulates, a sample of the dilute exhaust gas is passed to the particulate sampling system (paragraph 2.4., figures 21 and 22).  If this is done directly, it is referred to as single dilution. If the sample is diluted once more in the secondary dilution tunnel, it is referred to as double dilution.  This is useful, if the filter face temperature requirement cannot be met with single dilution.  Although partly a dilution system, the double dilution system is described as a modification of a particulate sampling system in paragraph 2.4., figure 22, since it shares most of the parts with a typical particulate sampling system.

Figure 20

Full flow dilution system to background filter

The total amount of raw exhaust gas is mixed in the dilution tunnel DT with the dilution air.  The diluted exhaust gas flow rate is measured either with a positive displacement pump PDP or with a critical flow venturi CFV.  A heat exchanger HE or electronic flow compensation EFC may be used for proportional particulate sampling and for flow determination.  Since particulate mass determination is based on the total diluted exhaust gas flow, the dilution ratio is not required to be calculated.

2.3.1.	Components of figure 20

EP exhaust pipe

The exhaust pipe length from the exit of the engine exhaust manifold, turbocharger outlet or aftertreatment device to the dilution tunnel shall not exceed 10 m.  If the exhaust pipe downstream of the engine exhaust manifold, turbocharger outlet or aftertreatment device exceeds 4 m in length, then all tubing in excess of 4 m shall be insulated, except for an in-line smokemeter, if used.  The radial thickness of the insulation shall be at least 25 mm.  The thermal conductivity of the insulating material shall have a value no greater than 0.1 W/mK measured at 673 K.  To reduce the thermal inertia of the exhaust pipe a thickness to diameter ratio of 0.015 or less is recommended.  The use of flexible sections shall be limited to a length to diameter ratio of 12 or less.

PDP positive displacement pump

The PDP meters total diluted exhaust flow from the number of the pump revolutions and the pump displacement.  The exhaust system backpressure shall not be artificially lowered by the PDP or dilution air inlet system.  Static exhaust backpressure measured with the PDP system operating shall remain within ± 1.5 kPa of the static pressure measured without connection to the PDP at identical engine speed and load.  The gas mixture temperature immediately ahead of the PDP shall be within ± 6 K of the average operating temperature observed during the test, when no flow compensation is used.  Flow compensation may only be used if the temperature at the inlet to the PDP does not exceed 323 K (50 °C).

CFV critical flow venturi

CFV measures total diluted exhaust flow by maintaining the flow at choked conditions (critical flow).  Static exhaust backpressure measured with the CFV system operating shall remain within ± 1.5 kPa of the static pressure measured without connection to the CFV at identical engine speed and load.  The gas mixture temperature immediately ahead of the CFV shall be within ± 11 K of the average operating temperature observed during the test, when no flow compensation is used.

HE heat exchanger (optional, if EFC is used)

The heat exchanger shall be of sufficient capacity to maintain the temperature within the limits required above.

EFC electronic flow compensation (optional, if HE is used)

If the temperature at the inlet to either the PDP or CFV is not kept within the limits stated above, a flow compensation system is required for continuous measurement of the flow rate and control of the proportional sampling in the particulate system.  To that purpose, the continuously measured flow rate signals are used to correct the sample flow rate through the particulate filters of the particulate sampling system (see paragraph 2.4., figures 21and 22), accordingly.

DT dilution tunnel

The dilution tunnel:

- (a)	shall be small enough in diameter to cause turbulent flow (Reynolds number greater than 4,000) and of sufficient length to cause complete mixing of the exhaust and dilution air; a mixing orifice may be used;

- (b)	shall be at least 460 mm in diameter with a single dilution system;

- (c)	shall be at least 210 mm in diameter with a double dilution system;

- (d)	may be insulated.

The engine exhaust shall be directed downstream at the point where it is introduced into the dilution tunnel, and thoroughly mixed.

When using single dilution, a sample from the dilution tunnel is transferred to the particulate sampling system (paragraph 2.4., figure 21).  The flow capacity of the PDP or CFV shall be sufficient to maintain the diluted exhaust at a temperature of less than or equal to 325 K (52°C) immediately before the primary particulate filter.

When using double dilution, a sample from the dilution tunnel is transferred to the secondary dilution tunnel where it is further diluted, and then passed through the sampling filters (paragraph 2.4., figure 22).  The flow capacity of the PDP or CFV shall be sufficient to maintain the diluted exhaust stream in the DT at a temperature of less than or equal to 464 K (191 °C) at the sampling zone.  The secondary dilution system shall provide sufficient secondary dilution air to maintain the doubly-diluted exhaust stream at a temperature of less than or equal to 325 K (52 °C) immediately before the primary particulate filter.

DAF dilution air filter

It is recommended that the dilution air be filtered and charcoal scrubbed to eliminate background hydrocarbons.  At the engine manufacturers request the dilution air shall be sampled according to good engineering practice to determine the background particulate levels, which can then be subtracted from the values measured in the diluted exhaust.

PSP particulate sampling probe

The probe is the leading section of PTT and:

- (a)	shall be installed facing upstream at a point where the dilution air and exhaust gas are well mixed, i.e. on the dilution tunnel (DT) centreline approximately 10 tunnel diameters downstream of the point where the exhaust enters the dilution tunnel;

- (b)	shall be of 12 mm minimum inside diameter;

- (c)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust in the dilution tunnel;

- (d)	may be insulated.

2.4.	Particulate sampling system

The particulate sampling system is required for collecting the particulates on the particulate filter.  In the case of total sampling partial flow dilution, which consists of passing the entire diluted exhaust sample through the filters, dilution (paragraph 2.2., figures 14 and 18) and sampling system usually form an integral unit.  In the case of fractional sampling partial flow dilution or full flow dilution, which consists of passing through the filters only a portion of the diluted exhaust, the dilution (paragraph 2.2., figures 11, 12, 13, 15, 16, 17, 19; paragraph 2.3., figure 20) and sampling systems usually form different units.

In this Regulation, the double dilution system (figure 22) of a full flow dilution system is considered as a specific modification of a typical particulate sampling system as shown in figure 21.  The double dilution system includes all important parts of the particulate sampling system, like filter holders and sampling pump, and additionally.

In order to avoid any impact on the control loops, it is recommended that the sample pump be running throughout the complete test procedure.  For the single filter method, a bypass system shall be used for passing the sample through the sampling filters at the desired times.  Interference of the switching procedure on the control loops shall be minimized.

Figure 21

Particulate sampling system

A sample of the diluted exhaust gas is taken from the dilution tunnel DT of a partial flow or full flow dilution system through the particulate sampling probe PSP and the particulate transfer tube PTT by means of the sampling pump P.  The sample is passed through the filter holder(s) FH that contain the particulate sampling filters.  The sample flow rate is controlled by the flow controller FC3.  If electronic flow compensation EFC (see figure 20) is used, the diluted exhaust gas flow is used as command signal for FC3.

Figure 22

Double dilution system (full flow system only)

A sample of the diluted exhaust gas is transferred from the dilution tunnel DT of a full flow dilution system through the particulate sampling probe PSP and the particulate transfer tube PTT to the secondary dilution tunnel SDT, where it is diluted once more.  The sample is then passed through the filter holder(s) FH that contain the particulate sampling filters.  The dilution air flow rate is usually constant whereas the sample flow rate is controlled by the flow controller FC3.  If electronic flow compensation EFC (see figure 20) is used, the total diluted exhaust gas flow is used as command signal for FC3.

2.4.1.	Components of figures 21 and 22

PTT Particulate transfer tube (figures 21 and 22)

The particulate transfer tube shall not exceed 1,020 mm in length, and shall be minimized in length whenever possible.  As indicated below (i.e. for partial flow dilution fractional sampling systems and for full flow dilution systems), the length of the sampling probes (SP, ISP, PSP, respectively, see paragraphs 2.2. and 2.3.) shall be included.

The dimensions are valid for:

- (a)	the partial flow dilution fractional sampling type and the full flow single dilution system from the tip of the probe (SP, ISP, PSP, respectively) to the filter holder;

- (b)	the partial flow dilution total sampling type from the end of the dilution tunnel to the filter holder;

- (c)	the full flow double dilution system from the tip of the probe (PSP) to the secondary dilution tunnel.

The transfer tube:

- (a)	may be heated to no greater than 325K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust in the dilution tunnel;

- (b)	may be insulated.

SDT secondary dilution tunnel (figure 22)

The secondary dilution tunnel should have a minimum diameter of 75 mm, and should be of sufficient length so as to provide a residence time of at least 0,25 seconds for the doubly-diluted sample.  The primary filter holder FH shall be located within 300 mm of the exit of the SDT.

The secondary dilution tunnel:

- (a)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust in the dilution tunnel;

- (b)	may be insulated.

FH Filter holder(s) (figures 21 and 22)

The filter holder shall meet the requirements of paragraph 4.1.3. of Appendix 4 to this annex.

The filter holder:

- (a)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust in the dilution tunnel;

- (b)	may be insulated.

P sampling pump (figures 21 and 22)

The particulate sampling pump shall be located sufficiently distant from the tunnel so that the inlet gas temperature is maintained constant (± 3 K), if flow correction by FC3 is not used.

DP dilution air pump (figure 22)

The dilution air pump shall be located so that the secondary dilution air is supplied at a temperature of 298 K ± 5 K (25°C ± 5°C), if the dilution air is not preheated.

FC3 Flow controller (figures 21 and 22)

A flow controller shall be used to compensate the particulate sample flow rate for temperature and backpressure variations in the sample path, if no other means are available. The flow controller is required if electronic flow compensation EFC (see figure 20) is used.

FM3 Flow measurement device (figures 21 and 22)

The gas meter or flow instrumentation for the particulate sample flow shall be located sufficiently distant from the sampling pump P so that the inlet gas temperature remains constant (± 3 K), if flow correction by FC3 is not used.

FM4 flow measurement device (figure 22)

The gas meter or flow instrumentation for the dilution air flow shall be located so that the inlet gas temperature remains at 298 K ± 5 K (25 °C ± 5 °C).

BV ball valve (optional)

The ball valve shall have an inside diameter not less than the inside diameter of the particulate transfer tube PTT, and a switching time of less than 0.5 seconds.

Note:	If the ambient temperature in the vicinity of PSP, PTT, SDT, and FH is below 293 K (20 °C), precautions should be taken to avoid particle losses onto the cool wall of these parts.  Therefore, heating and/or insulating these parts within the limits given in the respective descriptions is recommended.  It is also recommended that the filter face temperature during sampling be not below 293 K (20 °C).

At high engine loads, the above parts may be cooled by a non-aggressive means such as a circulating fan, as long as the temperature of the cooling medium is not below 293 K (20 °C).

3.	DETERMINATION OF SMOKE

3.1.	Introduction

Paragraphs 3.2. and 3.3. and figures 23 and 24 contain detailed descriptions of the recommended opacimeter systems.  Since various configurations can produce equivalent results, exact conformance with figures 23 and 24 is not required.  Additional components such as instruments, valves, solenoids, pumps, and switches may be used to provide additional information and coordinate the functions of the component systems.  Other components which are not needed to maintain the accuracy on some systems, may be excluded if their exclusion is based upon good engineering judgement.

The principle of measurement is that light is transmitted through a specific length of the smoke to be measured and that proportion of the incident light which reaches a receiver is used to assess the light obscuration properties of the medium.  The smoke measurement depends upon the design of the apparatus, and may be done in the exhaust pipe (full flow in-line opacimeter), at the end of the exhaust pipe (full flow end-of-line opacimeter) or by taking a sample from the exhaust pipe (partial flow opacimeter).  For the determination of the light absorption coefficient from the opacity signal, the optical path length of the instrument shall be supplied by the instrument manufacturer.

3.2.	Full flow opacimeter

Two general types of full flow opacimeters may be used (figure 23).  With the in-line opacimeter, the opacity of the full exhaust plume within the exhaust pipe is measured.  With this type of opacimeter, the effective optical path length is a function of the opacimeter design.

With the end-of-line opacimeter, the opacity of the full exhaust plume is measured as it exits the exhaust pipe.  With this type of opacimeter, the effective optical path length is a function of the exhaust pipe design and the distance between the end of the exhaust pipe and the opacimeter.

Figure 23

Full flow opacimeter

3.2.1.	Components of figure 23

EP exhaust pipe

With an in-line opacimeter, there shall be no change in the exhaust pipe diameter within 3 exhaust pipe diameters before or after the measuring zone.  If the diameter of the measuring zone is greater than the diameter of the exhaust pipe, a pipe gradually convergent before the measuring zone is recommended.

With an end-of-line opacimeter, the terminal 0.6 m of the exhaust pipe shall be of circular cross section and be free from elbows and bends.  The end of the exhaust pipe shall be cut off squarely.  The opacimeter shall be mounted centrally to the plume within 25 ± 5 mm of the end of the exhaust pipe.

OPL optical path length

The length of the smoke obscured optical path between the opacimeter light source and the receiver, corrected as necessary for non-uniformity due to density gradients and fringe effect.  The optical path length shall be submitted by the instrument manufacturer taking into account any measures against sooting (e.g. purge air).  If the optical path length is not available, it shall be determined in accordance with ISO 11614, paragraph 11.6.5.  For the correct determination of the optical path length, a minimum exhaust gas velocity of 20 m/s is required.

LS light source

The light source shall be an incandescent lamp with a colour temperature in the range of 2,800 to 3,250 K or a green light emitting diode (LED) with a spectral peak between 550 and 570 nm.  The light source shall be protected against sooting by means that do not influence the optical path length beyond the manufacturers specifications.

LD light detector

The detector shall be a photocell or a photodiode (with a filter, if necessary).  In the case of an incandescent light source, the receiver shall have a peak spectral response similar to the phototopic curve of the human eye (maximum response) in the range of 550 to 570 nm, to less than 4 per cent of that maximum response below 430 nm and above 680 nm.  The light detector shall be protected against sooting by means that do not influence the optical path length beyond the manufacturers specifications.

CL collimating lens

The light output shall be collimated to a beam with a maximum diameter of 30 mm.  The rays of the light beam shall be parallel within a tolerance of 3° of the optical axis.

T1 temperature sensor (optional)

The exhaust gas temperature may be monitored over the test.

3.3.	Partial flow opacimeter

With the partial flow opacimeter (figure 24), a representative exhaust sample is taken from the exhaust pipe and passed through a transfer line to the measuring chamber.  With this type of opacimeter, the effective optical path length is a function of the opacimeter design.  The response times referred to in the following paragraph apply to the minimum flow rate of the opacimeter, as specified by the instrument manufacturer.

Figure 24

Partial flow opacimeter

3.3.1.	Components of figure 24

EP exhaust pipe

The exhaust pipe shall be a straight pipe of at least 6-pipe diameters upstream and 3-pipe diameters downstream of the tip of the probe.

SP sampling probe

The sampling probe shall be an open tube facing upstream on or about the exhaust pipe centreline. The clearance with the wall of the tailpipe shall be at least 5 mm. The probe diameter shall ensure a representative sampling and a sufficient flow through the opacimeter.

TT transfer tube

The transfer tube shall:

- (a)	Be as short as possible and ensure an exhaust gas temperature of 373 ± 30 K (100 °C ± 30 °C) at the entrance to the measuring chamber.

- (b)	Have a wall temperature sufficiently above the dew point of the exhaust gas to prevent condensation.

- (c)	Be equal to the diameter of the sampling probe over the entire length.

- (d)	Have a response time of less than 0.05 s at minimum instrument flow, as determined according to paragraph 5.2.4. of Appendix 4 to this annex.

- (e)	Have no significant effect on the smoke peak.

FM flow measurement device

Flow instrumentation to detect the correct flow into the measuring chamber.  The minimum and maximum flow rates shall be specified by the instrument manufacturer, and shall be such that the response time requirement of TT and the optical path length specifications are met.  The flow measurement device may be close to the sampling pump, P, if used.

MC measuring chamber

The measuring chamber shall have a non-reflective internal surface, or equivalent optical environment.  The impingement of stray light on the detector due to internal reflections of diffusion effects shall be reduced to a minimum.

The pressure of the gas in the measuring chamber shall not differ from the atmospheric pressure by more than 0.75 kPa.  Where this is not possible by design, the opacimeter reading shall be converted to atmospheric pressure.

The wall temperature of the measuring chamber shall be set to within ± 5 K between 343 K (70 °C) and 373 K (100 °C), but in any case sufficiently above the dew point of the exhaust gas to prevent condensation.  The measuring chamber shall be equipped with appropriate devices for measuring the temperature.

OPL optical path length

The length of the smoke obscured optical path between the opacimeter light source and the receiver, corrected as necessary for non-uniformity due to density gradients and fringe effect.  The optical path length shall be submitted by the instrument manufacturer taking into account any measures against sooting (e.g. purge air).  If the optical path length is not available, it shall be determined in accordance with ISO 11614, paragraph 11.6.5.

LS light source

The light source shall be an incandescent lamp with a colour temperature in the range of 2,800 to 3,250 K or a green light emitting diode (LED) with a spectral peak between 550 and 570 nm. The light source shall be protected against sooting by means that do not influence the optical path length beyond the manufacturers specifications.

LD light detector

The detector shall be a photocell or a photodiode (with a filter, if necessary).  In the case of an incandescent light source, the receiver shall have a peak spectral response similar to the phototopic curve of the human eye (maximum response) in the range of 550 to 570 nm, to less than 4 per cent of that maximum response below 430 nm and above 680 nm.  The light detector shall be protected against sooting by means that do not influence the optical path length beyond the manufacturers specifications.

CL collimating lens

The light output shall be collimated to a beam with a maximum diameter of 30 mm.  The rays of the light beam shall be parallel within a tolerance of 3° of the optical axis.

T1 temperature sensor

To monitor the exhaust gas temperature at the entrance to the measuring chamber.

P sampling pump (optional)

A sampling pump downstream of the measuring chamber may be used to transfer the sample gas through the measuring chamber.

Annex 5

TECHNICAL CHARACTERISTICS OF REFERENCE FUEL PRESCRIBED FOR APPROVAL TESTS AND TO VERIFY THE CONFORMITY OF PRODUCTION

1.1.	DIESEL REFERENCE FUEL FOR TESTING ENGINES TO THE EMISSION LIMITS GIVEN IN ROW A OF THE TABLES IN PARAGRAPH 5.2.1. OF THIS REGULATION (a)

| Parameter | Unit | Limits (b) |  | Test Method | Publication |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |  |
| Cetane number (c) |  | 52 | 54 | EN-ISO 5165 | 1998 (d) |
| Density at 15 °C | kg/m3 | 833 | 837 | EN-ISO 3675 | 1995 |
| Distillation: |  |  |  |  |  |
| — 50 per cent point | °C | 245 | — | EN-ISO 3405 | 1998 |
| — 95 per cent point | °C | 345 | 350 | EN-ISO 3405 | 1998 |
| — final boiling point | °C | — | 370 | EN-ISO 3405 | 1998 |
| Flash point | °C | 55 | — | EN 27719 | 1993 |
| CFPP | °C | — | − 5 | EN 116 | 1981 |
| Viscosity at 40 °C | mm2/s | 2.5 | 3.5 | EN-ISO 3104 | 1996 |
| Polycyclic aromatic hydrocarbons | per cent m/m | 3.0 | 6.0 | IP 391 (*) | 1995 |
| Sulphur content (e) | mg/kg | — | 300 | pr. EN-ISO/DIS 14596 | 1998 (d) |
| Copper corrosion |  | — | 1 | EN-ISO 2160 | 1995 |
| Conradson carbon residue (10 per cent DR) | per cent m/m | — | 0.2 | EN-ISO 10370 |  |
| Ash content | per cent m/m | — | 0.01 | EN-ISO 6245 | 1995 |
| Water content | per cent m/m | — | 0.05 | EN-ISO 12937 | 1995 |
| Neutralisation (strong acid) number | mg KOH/g | — | 0.02 | ASTM D 974-95 | 1998 (d) |
| Oxidation stability (f) | mg/ml | — | 0.025 | EN-ISO 12205 | 1996 |
| (*) New and better method for polycyclic aromatics under development | per cent m/m | — | — | EN 12916 | [1997] (d) |
| (a) If it is required to calculate the thermal efficiency of an engine or vehicle, the calorific value of the fuel can be calculated from: Specific energy (calorific value)(net) in MJ/kg = (46.423 - 8.792d2 + 3.170d)(1 - (x + y + s)) + 9.420s - 2.499x where. d = the density at 15 °C x = the proportion by mass of water ( per cent divided by 100) y = the proportion by mass of ash ( per cent divided by 100) s = the proportion by mass of sulphur ( per cent divided by 100). (b) The values quoted in the specification are "true values". In establishment of their limit values the terms of ISO 4259, Petroleum products - Determination and application of precision data in relation to methods of test, have been applied and in fixing a minimum value, a minimum difference of 2R above zero has been taken into account; in fixing a maximum and minimum value, the minimum difference is 4R (R = reproducibility). Notwithstanding this measure, which is necessary for statistical reasons, the manufacturer of a fuel should nevertheless aim at a zero value where the stipulated maximum value is 2R and at the mean value in the case of quotations of maximum and minimum limits. Should it be necessary to clarify the question as to whether a fuel meets the requirements of the specification, the terms of ISO 4259 should be applied. (c) The range for cetane number is not in accordance with the requirement of a minimum range of 4R. However, in the case of dispute between fuel supplier and fuel user. the terms in ISO 4259 can be used to resolve such disputes provided replicate measurements, of sufficient number to achieve the necessary precision, are made in preference to single determinations. (d) The month of publication will be completed in due course. (e) The actual sulphur content of the fuel used for the test shall be reported. (f) Even though oxidation stability is controlled, it is likely that shelf life will be limited. Advice should be sought from the supplier as to storage conditions and life. |  |  |  |  |  |

1.2.	DIESEL REFERENCE FUEL FOR TESTING ENGINES TO THE EMISSION LIMITS GIVEN IN ROWS B1, B2 OR C OF THE TABLES IN PARAGRAPH 5.2.1. OF THIS REGULATION

| Parameter | Unit | Limits (a) |  | Test Method |
| --- | --- | --- | --- | --- |
|  |  | minimum | maximum |  |
| Cetane number (b) |  | 52.0 | 54.0 | EN-ISO 5165 |
| Density at 15°C | kg/m3 | 833 | 837 | EN-ISO 3675 |
| Distillation: |  |  |  |  |
| 50 per cent point | °C | 245 | - | EN-ISO 3405 |
| 95 per cent point | °C | 345 | 350 | EN-ISO 3405 |
| - Final boiling point | °C | - | 370 | EN-ISO 3405 |
| Flash point | °C | 55 | - | EN 22719 |
| CFPP | °C | - | -5 | EN 116 |
| Viscosity at 40°C | mm2/s | 2.3 | 3.3 | EN-ISO 3104 |
| Polycyclic aromatic hydrocarbons | per cent m/m | 2.0 | 6.0 | IP 391 |
| Sulphur content (c) | mg/kg | - | 10 | ASTM D 5453 |
| Copper corrosion |  | - | class 1 | EN-ISO 2160 |
| Conradson carbon residue (10 per cent DR) | per cent m/m | - | 0.2 | EN-ISO 10370 |
| Ash content | per cent m/m | - | 0.01 | EN-ISO 6245 |
| Water content | per cent m/m | - | 0.02 | EN-ISO 12937 |
| Neutralisation (strong acid) number | Mg KOH/g | - | 0.02 | ASTM D 974 |
| Oxidation stability (d) | mg/ml | - | 0.025 | EN-ISO 12205 |
| Lubricity (HFRR wear scan diameter at 60°C) | Μm | - | 400 | CEC F-06-A-96 |
| FAME | prohibited |  |  |  |
| (a) The values quoted in the specifications are "true values". In establishment of their limit values the terms of ISO 4259 "Petroleum products – Determination and application of precision data in relation to methods of test" have been applied and in fixing a minimum value, a minimum difference of 2R above zero has been taken into account; in fixing a maximum and minimum value, the minimum difference is 4R (R = reproducibility). Notwithstanding this measure, which is necessary for technical reasons, the manufacturer of fuels should nevertheless aim at a zero value where the stipulated maximum value is 2R and at the mean value in the case of quotations of maximum and minimum limits. Should it be necessary to clarify the questions as to whether a fuel meets the requirements of the specifications, the terms of ISO 4259 should be applied. (b) The range for cetane number is not in accordance with the requirements of a minimum range of 4R. However, in the case of a dispute between fuel supplier and fuel user, the terms of ISO 4259 may be used to resolve such disputes provided replicate measurements, of sufficient number to archive the necessary precision, are made in preference to single determinations. (c) The actual sulphur content of the fuel used for the Type I test shall be reported. (d) Even though oxidation stability is controlled, it is likely that shelf life will be limited. Advice should be sought from the supplier as to storage conditions and life. |  |  |  |  |

1.3.	ETHANOL FOR DIESEL ENGINES (a)

| Parameter | Unit | Limits (b) |  | Test method (c) |
| --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |
| Alcohol, mass | per cent m/m | 92.4 | — | ASTM D 5501 |
| Other alcohol than ethanol contained in total alcohol, mass | per cent m/m | — | 2 | ADTM D 5501 |
| Density at 15 °C | kg/m3 | 795 | 815 | ASTM D 4052 |
| Ash content | per cent m/m |  | 0.001 | ISO 6245 |
| Flash point | °C | 10 |  | ISO 2719 |
| Acidity, calculated as acetic acid | per cent m/m | — | 0.0025 | ISO 1388-2 |
| Neutralisation (strong acid) number | KOH mg/1 | — | 1 |  |
| Colour | According to scale | — | 10 | ASTM D 1209 |
| Dry residue at 100 °C | mg/kg |  | 15 | ISO 759 |
| Water content | per cent m/m |  | 6.5 | ISO 760 |
| Aldehydes calculated as acetic acid | per cent m/m |  | 0.0025 | ISO 1388-4 |
| Sulphur content | mg/kg | — | 10 | ASTM D 5453 |
| Esters, calculated as ethylacetate | per cent m/m | — | 0.1 | ASSTM D 1617 |
| (a) Cetane improver, as specified by the engine manufacturer, may be added to the ethanol fuel. The maximum allowed amount is 10 per cent m/m. (b) The values quoted in the specification are "true values". In establishment of their limit values the terms of ISO 4259, Petroleum products - Determination and application of precision data in relation to methods of test, have been applied and in fixing a minimum value, a minimum difference of 2R above zero has been taken into account; in fixing a maximum and minimum value, the minimum difference is 4R (R - reproducibility). Notwithstanding this measure, which is necessary for statistical reasons, the manufacturer of a fuel should nevertheless aim at a zero value where the stipulated maximum value is 2R and at the mean value in the case of quotations of maximum and minimum limits. Should it be necessary to clarify the question as to whether a fuel meets the requirements of the specification, the terms of ISO 4259 should be applied. (c) Equivalent ISO methods will be adopted when issued for all properties listed above |  |  |  |  |

2.	NATURAL GAS (NG)

European market fuels are available in two ranges:

- (a)	the H range, whose extreme reference fuels are GR and G23;

- (b)	the L range, whose extreme reference fuels are G23 and G25.

The characteristics of GR, G23 and G25 reference fuels are summarized below:

| Reference fuel GR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Units | Basis | Limits |  | Test Method |
|  |  |  | Minimum | Maximum |  |
| Composition: |  |  |  |  |  |
| Methane |  | 87 | 84 | 89 |  |
| Ethane |  | 13 | 11 | 15 |  |
| Balance (a) | per cent-mole | - | - | 1 | ISO 6974 |
| Sulphur content | mg/m3 (b) | - | - | 10 | ISO 6326-5 |
| (a) Inerts +C2+ (b) Value to be determined at standard conditions (293.2 K (20 °C) and 101.3 kPa). |  |  |  |  |  |

| Reference fuel G23 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Units | Basis | Limits |  | Test Method |
|  |  |  | Minimum | Maximum |  |
| Composition: |  |  |  |  |  |
| Methane |  | 92.5 | 91.5 | 93.5 |  |
| Balance (a) | per cent-mole | - | - | 1 | ISO 6974 |
| N2 |  | 7.5 | 6.5 | 8.5 |  |
| Sulphur content | mg/m3 (b) | - | - | 10 | ISO 6326-5 |
| (a) Inerts (different from N2) +C2+ +C2+ (b) Value to be determined at standard conditions (293.2 K (20 °C) and 101.3 kPa). |  |  |  |  |  |

| Reference fuel G25 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristics | Units | Basis | Limits |  | Test Method |
|  |  |  | Minimum | Maximum |  |
| Composition: |  |  |  |  |  |
| Methane |  | 86 | 84 | 88 |  |
| Balance (a) | per cent-mole | - | - | 1 | ISO 6974 |
| N2 |  | 14 | 12 | 16 |  |
| Sulphur content | mg/m3 (b) | - | - | 10 | ISO 6326-5 |
| (a) Inerts (different from N2) +C2+ +C2+ (b) Value to be determined at standard conditions (293.2 K (20 °C) and 101.3 kPa). |  |  |  |  |  |

3.	TECHNICAL DATA OF THE LPG REFERENCE FUELS

A.	Technical data of the LPG reference fuels used for testing vehicles to the emission limits given in row A of the tableS in paragraph 5.2.1. of THIS REGULATION

| Parameter | Unit | Fuel A | Fuel B | Test method |
| --- | --- | --- | --- | --- |
| Composition: |  |  |  | ISO 7941 |
| C3-content | per cent vol | 50 ± 2 | 85 ± 2 |  |
| C4-content | per cent vol | balance | balance |  |
| < C3 , >C4 | per cent vol | Max. 2 | max. 2 |  |
| Olefins | per cent vol | max. 12 | max. 14 |  |
| Evaporation residue | mg/kg | max. 50 | max. 50 | ISO 13757 |
| Water at 0C |  | free | free | visual inspection |
| Total sulphur content | mg/kg | max. 50 | max. 50 | EN 24260 |
| Hydrogen sulphide |  | none | none | ISO 8819 |
| Copper strip corrosion | rating | Class 1 | class 1 | ISO 6251 (a) |
| Odour |  | characteristic | characteristic |  |
| Motor octane number |  | min. 92.5 | min. 92.5 | EN 589 Annex B |
| (a) This method may not accurately determine the presence of corrosive materials if the sample contains corrosion inhibitors or other chemicals which diminish the corrosivity of the sample to the copper strip. Therefore, the addition of such compounds for the sole purpose of biasing the test method is prohibited. |  |  |  |  |

B.	Technical data of the LPG reference fuels used for testing vehicles to the emission limits given in row B1, B2 or C of the tables in paragraph 5.2.1. of this regulation

| Parameter | Unit | Fuel A | Fuel B | Test method |
| --- | --- | --- | --- | --- |
| Composition: |  |  |  | ISO 7941 |
| C3-content | per cent vol | 50 ± 2 | 85 ± 2 |  |
| C4-content | per cent vol | balance | balance |  |
| < C3 , >C4 | per cent vol | max. 2 | max. 2 |  |
| Olefins | per cent vol | max. 12 | max. 14 |  |
| Evaporation residue | mg/kg | max. 50 | max. 50 | ISO 13757 |
| Water at 0C |  | free | free | Visual inspection |
| Total sulphur content | mg/kg | max. 10 | max. 10 | EN 24260 |
| Hydrogen sulphide |  | none | none | ISO 8819 |
| Copper strip corrosion | Rating | class 1 | class 1 | ISO 6251 (a) |
| Odour |  | characteristic | characteristic |  |
| Motor octane number |  | min. 92.5 | min. 92.5 | EN 589 Annex B |
| (a) This method may not accurately determine the presence of corrosive materials if the sample contains corrosion inhibitors or other chemicals which diminish the corrosivity of the sample to the copper strip. Therefore, the addition of such compounds for the sole purpose of biasing the test method is prohibited. |  |  |  |  |

Annex 6

EXAMPLE OF CALCULATION PROCEDURE

1.	ESC TEST

1.1.	Gaseous emissions

The measurement data for the calculation of the individual mode results are shown below.  In this example, CO and NOx are measured on a dry basis, HC on a wet basis.  The HC concentration is given in propane equivalent (C3) and has to be multiplied by 3 to result in the C1 equivalent.  The calculation procedure is identical for the other modes.

| P | Ta | Ha | GEXH | GAIRW | GFUEL | HC | CO | NOx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (kW) | (K) | (g/kg) | (kg) | (kg) | (kg) | (ppm) | (ppm) | (ppm) |
| 82.9 | 294.8 | 7.81 | 563.38 | 545.29 | 18.09 | 6.3 | 41.2 | 495 |

Calculation of the dry to wet correction factor KW,r (Annex 4A, Appendix 1, paragraph 45.2):

and

Calculation of the wet concentrations:

CO = 41.2  0.9239 = 38.1 ppm

NOx = 495  0.9239 = 457 ppm

Calculation of the NOx humidity correction factor KH,D (Annex 4A, Appendix 1, paragraph 54.3):

A = 0.309  18.09/541.06 − 0.0266 = −0.0163

B = − 0.209  18.09/541.06 + 0.00954 = 0.0026

Calculation of the emission mass flow rates (Annex 4A, Appendix 1, paragraph 54.4.):

NOx = 0.001587  457  0.9625  563.38 = 393.27 g/h

CO = 0.000966  38.1  563.38 = 20.735 g/h

HC = 0.000479  6.3  3  563.38 = 5.100 g/h

Calculation of the specific emissions (Annex 4A, Appendix 1, paragraph 54.5.):

The following example calculation is given for CO; the calculation procedure is identical for the other components.

The emission mass flow rates of the individual modes are multiplied by the respective weighting factors, as indicated in Annex 4A, Appendix 1, paragraph 2.7.1., and summed up to result in the mean emission mass flow rate over the cycle:

CO	=	(6.7  0.15) + (24.6  0.08) + (20.5  0.10) + (20.7  0.10) + (20.6  0.05) + (15.0  0.05) + (19.7  0.05) + (74.5  0.09) + (31.5  0.10) + (81.9  0.08) + (34.8  0.05) + (30.8  0.05) + (27.3  0.05)

=	30.91 g/h

The engine power of the individual modes is multiplied by the respective weighting factors, as indicated in Annex 4A, Appendix 1, paragraph 2.7.1., and summed up to result in the mean cycle power:

P(n)	=	(0.1  0.15) + (96.8  0.08) + (55.2  0.10) + (82.9  0.10) + (46.8  0.05) + (70.1  0.05) + (23.0  0.05) + (114.3  0.09) + (27.0  0.10) + (122.0  0.08) + (28.6  0.05) + (87.4  0.05) + (57.9  0.05)

=	60.006 kW

Calculation of the specific NOx emission of the random point (Annex 4A, Appendix 1, paragraph 54.6.1):

Assume the following values have been determined on the random point:

| nZ | 1 600 min−1 |
| --- | --- |
| MZ | 495 Nm |
| NOx mass,Z | 487.9 g/h (calculated according to the previous formulae) |
| P(n)Z | 83 kW |
| NOx,Z | 487.9/83 = 5.878 g/kWh |

Determination of the emission value from the test cycle (Annex 4A, Appendix 1, paragraph 45.6.2.):

Assume the values of the four enveloping modes on the ESC to be as follows:

| nRT | nSU | ER | ES | ET | EU | MR | MS | MT | MU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1368 | 1785 | 5.943 | 5.565 | 5.889 | 4.973 | 515 | 460 | 681 | 610 |

ETU = 5.889 + (4.973 − 5.889)  (1 600 − 1 368) / (1 785 − 1 368) = 5.377 g/kWh

ERS = 5.943 + (5.565 − 5.943)  (1 600 − 1 368) / (1 785 − 1 368) = 5.732 g/kWh

MTU = 681 + (601 − 681)  (1 600 − 1 368) / (1 785 − 1 368) = 641.3 Nm

MRS = 515 + (460 − 515)  (1 600 − 1 368) / (1 785 − 1 368) = 484.3 Nm

EZ = 5.732 + (5.377 − 5.732)  (495 − 484.3) / (641.3 − 484.3) = 5.708 g/kWh

Comparison of the NOx emission values (Annex 4A, Appendix 1, paragraph 45.6.3.):

NOx diff = 100  (5.878 − 5.708) / 5.708 = 2.98 per cent

1.2.	Particulate emissions

Particulate measurement is based on the principle of sampling the particulates over the complete cycle, but determining the sample and flow rates (MSAM and GEDF) during the individual modes.  The calculation of GEDF depends on the system used.  In the following examples, a system with CO2 measurement and carbon balance method and a system with flow measurement are used.  When using a full flow dilution system, GEDF is directly measured by the CVS equipment.

Calculation of GEDF (Annex 4A, Appendix 1, paragraphs 6.2.3. and 6.2.4.):

Assume the following measurement data of mode 4.  The calculation procedure is identical for the other modes.

| GEXH | GFUEL | GDILW | GTOTW | CO2D | CO2A |
| --- | --- | --- | --- | --- | --- |
| (kg/h) | (kg/h) | (kg/h) | (kg/h) | (per cent) | (per cent) |
| 334.02 | 10.76 | 5.4435 | 6.0 | 0.657 | 0.040 |

(a)	carbon balance method

(b)	flow measurement method

GEDF W = 334.02  10.78 = 3 600.7 kg/h

Calculation of the mass flow rate (Annex 4A, Appendix 1, paragraph 6.4.):

The GEDFW flow rates of the individual modes are multiplied by the respective weighting factors, as indicated in Annex 4A, Appendix 1, paragraph 2.7.1., and summed up to result in the mean GEDF over the cycle.  The total sample rate MSAM is summed up from the sample rates of the individual modes.

=	(3 567  0.15) + (3 592  0.08) + (3 611  0.10) + (3 600  0.10) + (3 618  0.05) + (3 600  0.05) + (3 640  0.05) + (3 614  0.09) + (3 620  0.10) + (3 601  0.08) + (3 639  0.05) + (3 582  0.05) + (3 635  0.05)

=	3 604.6 kg/h

MSAM	=	0.226 + 0.122 + 0.151 + 0.152 + 0.076 + 0.076 + 0.076 + 0.136 + 0.151 + 0.121 + 0.076 + 0.076 + 0.075

=	1.515 kg

Assume the particulate mass on the filters to be 2.5 mg, then

Background correction (optional)

Assume one background measurement with the following values.  The calculation of the dilution factor DF is identical to paragraph 3.1. of this annex and not shown here.

Md = 0.1 mg; MDIL = 1.5 kg

Sum of DF =	[(1−1/119.15)  0.15] + [(1−1/8.89)  0.08] + [(1−1/14.75)  0.10] + [(1−1/10.10)  0.10] + [(1−1/18.02)  0.05] + [(1−1/12.33)  0.05] + [(1−1/32.18)  0.05] + [(1−1/6.94)  0.09] + [(1−1/25.19)  0.10] + [(1−1/6.12)  0.08] + [(1−1/20.87)  0.05] + [(1−1/8.77)  0.05] + [(1−1/12.59)  0.05]

=	0.923

Calculation of the specific emission (Annex 4A, Appendix 1, paragraph 6.5.):

P(n)	=	(0.1  0.15) + (96.8  0.08) + (55.2  0.10) + (82.9  0.10) + (46.8  0.05 + (70.1  0.05) + (23.0  0.05) + (114.3  0.09) + (27.0  0.10) + (122.0  0.08) + (28.6  0.05) + (87.4  0.05) + (57.9  0.05)

=	60.006 kW

if background corrected  = (5.726/60.006) = 0.095 g/kWh,

Calculation of the specific weighting factor (Annex 4A, Appendix 1, paragraph 6.6.):

Assume the values calculated for mode 4 above, then Wfei = (0.152  360 4.6/1.515  360 0.7) = 0.1004

This value is within the required value of 0.10 ± 0.003.

2.	ELR TEST

Since Bessel filtering is a completely new averaging procedure in European exhaust legislation, an explanation of the Bessel filter, an example of the design of a Bessel algorithm, and an example of the calculation of the final smoke value is given below.  The constants of the Bessel algorithm only depend on the design of the opacimeter and the sampling rate of the data acquisition system.  It is recommended that the opacimeter manufacturer provide the final Bessel filter constants for different sampling rates and that the customer use these constants for designing the Bessel algorithm and for calculating the smoke values.

2.1.	General remarks on the Bessel filter

Due to high frequency distortions, the raw opacity signal usually shows a highly scattered trace. To remove these high frequency distortions a Bessel filter is required for the ELR-Test.  The Bessel filter itself is a recursive, second-order low-pass filter which guarantees the fastest signal rise without overshoot.

Assuming a real time raw exhaust plume in the exhaust tube, each opacimeter shows a delayed and differently measured opacity trace.  The delay and the magnitude of the measured opacity trace is primarily dependent on the geometry of the measuring chamber of the opacimeter, including the exhaust sample lines, and on the time needed for processing the signal in the electronics of the opacimeter.  The values that characterize these two effects are called the physical and the electrical response time which represent an individual filter for each type of opacimeter.

The goal of applying a Bessel filter is to guarantee a uniform overall filter characteristic of the whole opacimeter system, consisting of:

- (a)	physical response time of the opacimeter (tp)

- (b)	electrical response time of the opacimeter (te)

- (c)	filter response time of the applied Bessel filter (tF)

The resulting overall response time of the system tAver is given by:

and shall be equal for all kinds of opacimeters in order to give the same smoke value.  Therefore, a Bessel filter has to be created in such a way, that the filter response time (tF) together with the physical (tp) and electrical response time (te) of the individual opacimeter shall result in the required overall response time (tAver).  Since tp and te are given values for each individual opacimeter, and tAver is defined to be 1.0 s in this Regulation, tF can be calculated as follows:

By definition, the filter response time tF is the rise time of a filtered output signal between 10 per cent and 90 per cent on a step input signal.  Therefore the cut-off frequency of the Bessel filter has to be iterated in such a way, that the response time of the Bessel filter fits into the required rise time.

Figure a

Traces of a step input signal and the filtered output signal

In figure a, the traces of a step input signal and Bessel filtered output signal as well as the response time of the Bessel filter (tF) are shown.

Designing the final Bessel filter algorithm is a multi step process which requires several iteration cycles.  The scheme of the iteration procedure is presented below.

2.2.	Calculation of the Bessel algorithm

In this example a Bessel algorithm is designed in several steps according to the above iteration procedure which is based upon Annex 4A, Appendix 1, paragraph 7.1.

For the opacimeter and the data acquisition system, the following characteristics are assumed:

- (a)	physical response time tp 0.15 s

- (b)	electrical response time te 0.05 s

- (c)	overall response time tAver 1.00 s (by definition in this Regulation)

- (d)	sampling rate 150 Hz

Step 1: Required Bessel filter response time tF:

Step 2: Estimation of cut-off frequency and calculation of Bessel constants E, K for first iteration:

| fc |  |
| --- | --- |
| Δt | = 1/150 = 0.006667 s |
| Ω |  |
| E |  |
| K | = 2  7.0794810-5  (0.618034  150.0766442 − 1) − 1 = 0.970783 |

This gives the Bessel algorithm:

Yi = Yi−1 + 7.07948 E − 5  (Si + 2  Si−1 + Si−2 − 4  Yi−2) + 0.970783  (Yi−1 − Yi−2)

where Si represents the values of the step input signal (either "0" or "1") and Yi represents the filtered values of the output signal.

Step 3: Application of Bessel filter on step input:

The Bessel filter response time tF is defined as the rise time of the filtered output signal between 10 per cent and 90 per cent on a step input signal.  For determining the times of 10 per cent (t10) and 90 per cent (t90) of the output signal, a Bessel filter has to be applied to a step input using the above values of fc, E and K.

The index numbers, the time and the values of a step input signal and the resulting values of the filtered output signal for the first and the second iteration are shown in table B.  The points adjacent to t10 and t90 are marked in bold numbers.

In table B, first iteration, the 10 per cent value occurs between index number 30 and 31 and the 90 per cent value occurs between index number 191 and 192.  For the calculation of tF,iter the exact t10 and t90 values are determined by linear interpolation between the adjacent measuring points, as follows:

t10 = tlower + Δt  (0.1 − outlower)/(outupper − outlower)

t90 = tlower + Δt  (0.9 − outlower)/(outupper − outlower)

where outupper and outlower, respectively, are the adjacent points of the Bessel filtered output signal, and tlower is the time of the adjacent time point, as indicated in table B.

t10 = 0.200000 + 0.006667  (0.1 − 0.099208)/(0.104794 − 0.099208) = 0.200945 s

t90 = 0.273333 + 0.006667  (0.9 − 0.899147)/(0.901168 − 0.899147) = 1.276147 s

Step 4: Filter response time of first iteration cycle:

tF,iter = 1.276147 − 0.200945 = 1.075202 s

Step 5: Deviation between required and obtained filter response time of first iteration cycle:

Δ = (1.075202 − 0.987421)/0.987421 = 0.081641

Step 6: Checking the iteration criteria:

|Δ| ≤ 0.01 is required.  Since 0.081641 > 0.01, the iteration criteria is not met and a further iteration cycle has to be started.  For this iteration cycle, a new cut-off frequency is calculated from fc and Δ as follows: fc,new = 0.318152  (1 + 0.081641) = 0.344126 Hz

This new cut-off frequency is used in the second iteration cycle, starting at step 2 again.  The iteration has to be repeated until the iteration criterion is met.  The resulting values of the first and second iteration are summarized in table A.

Table A

Values of the first and second iteration

| Parameter |  | 1. Iteration | 2. Iteration |
| --- | --- | --- | --- |
| fc | (Hz) | 0.318152 | 0.344126 |
| E | (−) | 7.07948 E-5 | 8.272777 E-5 |
| K | (−) | 0.970783 | 0.968410 |
| t10 | (s) | 0.200945 | 0.185523 |
| t90 | (s) | 1.276147 | 1.179562 |
| tF,iter | (s) | 1.075202 | 0.994039 |
| Δ | (−) | 0.081641 | 0.006657 |
| fc,new | (Hz) | 0.344126 | 0.346417 |

Step 7: Final Bessel algorithm:

As soon as the iteration criterion has been met, the final Bessel filter constants and the final Bessel algorithm are calculated according to step 2.  In this example, the iteration criterion has been met after the second iteration (Δ= 0.006657 ≤ 0.01).  The final algorithm is then used for determining the averaged smoke values (see next paragraph 2.3.).

Yi = Yi−1 + 8.27277710-5  (Si + 2  Si−1 + Si−2 − 4  Yi−2) + 0.968410  (Yi−1 − Yi−2)

Table B

Values of step input signal and Bessel filtered output signal for the first and second iteration cycle

| Index i [−] | Time [s] | Step Input Signal Si [−] | Filtered Output Signal Yi [−] |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1. Iteration | 2. Iteration |
| − 2 | − 0.013333 | 0 | 0.000000 | 0.000000 |
| − 1 | − 0.006667 | 0 | 0.000000 | 0.000000 |
| 0 | 0.000000 | 1 | 0.000071 | 0.000083 |
| 1 | 0.006667 | 1 | 0.000352 | 0.000411 |
| 2 | 0.013333 | 1 | 0.000908 | 0.001060 |
| 3 | 0.020000 | 1 | 0.001731 | 0.002019 |
| 4 | 0.026667 | 1 | 0.002813 | 0.003278 |
| 5 | 0.033333 | 1 | 0.004145 | 0.004828 |
| ∼ | ∼ | ∼ | ∼ | ∼ |
| 24 | 0.160000 | 1 | 0.067877 | 0.077876 |
| 25 | 0.166667 | 1 | 0.072816 | 0.083476 |
| 26 | 0.173333 | 1 | 0.077874 | 0.089205 |
| 27 | 0.180000 | 1 | 0.083047 | 0.095056 |
| 28 | 0.186667 | 1 | 0.088331 | 0.101024 |
| 29 | 0.193333 | 1 | 0.093719 | 0.107102 |
| 30 | 0.200000 | 1 | 0.099208 | 0.113286 |
| 31 | 0.206667 | 1 | 0.104794 | 0.119570 |
| 32 | 0.213333 | 1 | 0.110471 | 0.125949 |
| 33 | 0.220000 | 1 | 0.116236 | 0.132418 |
| 34 | 0.226667 | 1 | 0.122085 | 0.138972 |
| 35 | 0.233333 | 1 | 0.128013 | 0.145605 |
| 36 | 0.240000 | 1 | 0.134016 | 0.152314 |
| 37 | 0.246667 | 1 | 0.140091 | 0.159094 |
| ∼ | ∼ | ∼ | ∼ | ∼ |
| 175 | 1.166667 | 1 | 0.862416 | 0.895701 |
| 176 | 1.173333 | 1 | 0.864968 | 0.897941 |
| 177 | 1.180000 | 1 | 0.867484 | 0.900145 |
| 178 | 1.186667 | 1 | 0.869964 | 0.902312 |
| 179 | 1.193333 | 1 | 0.872410 | 0.904445 |
| 180 | 1.200000 | 1 | 0.874821 | 0.906542 |
| 181 | 1.206667 | 1 | 0.877197 | 0.908605 |
| 182 | 1.213333 | 1 | 0.879540 | 0.910633 |
| 183 | 1.220000 | 1 | 0.881849 | 0.912628 |
| 184 | 1.226667 | 1 | 0.884125 | 0.914589 |
| 185 | 1.233333 | 1 | 0.886367 | 0.916517 |
| 186 | 1.240000 | 1 | 0.888577 | 0.918412 |
| 187 | 1.246667 | 1 | 0.890755 | 0.920276 |
| 188 | 1.253333 | 1 | 0.892900 | 0.922107 |
| 189 | 1.260000 | 1 | 0.895014 | 0.923907 |
| 190 | 1.266667 | 1 | 0.897096 | 0.925676 |
| 191 | 1.273333 | 1 | 0.899147 | 0.927414 |
| 192 | 1.280000 | 1 | 0.901168 | 0.929121 |
| 193 | 1.286667 | 1 | 0.903158 | 0.930799 |
| 194 | 1.293333 | 1 | 0.905117 | 0.932448 |
| 195 | 1.300000 | 1 | 0.907047 | 0.934067 |
| ∼ | ∼ | ∼ | ∼ | ∼ |

2.3.	Calculation of the smoke values

In the scheme below the general procedure of determining the final smoke value is presented.

In figure b, the traces of the measured raw opacity signal, and of the unfiltered and filtered light absorption coefficients (k-value) of the first load step of an ELR-Test are shown, and the maximum value Ymax1,A (peak) of the filtered k trace is indicated.  Correspondingly, table C contains the numerical values of index i, time (sampling rate of 150 Hz), raw opacity, unfiltered k and filtered k.  Filtering was conducted using the constants of the Bessel algorithm designed in paragraph 2.2. of this annex.  Due to the large amount of data, only those sections of the smoke trace around the beginning and the peak are tabled.

Figure b

Traces of measured opacity N, of unfiltered smoke k and of filtered smoke k

The peak value (i = 272) is calculated assuming the following data of table C.  All other individual smoke values are calculated in the same way.  For starting the algorithm, S−1, S−2, Y−1 and Y−2 are set to zero.

| LA (m) | 0.430 |
| --- | --- |
| Index i | 272 |
| N ( per cent) | 16.783 |
| S271 (m−1) | 0.427392 |
| S270 (m−1) | 0.427532 |
| Y271 (m−1) | 0.542383 |
| Y270 (m−1) | 0.542337 |

Calculation of the k-value (Annex 4A, Appendix 1, paragraph 7.3.1.):

k = − (1/0,430)  ln (1 − (16,783/100)) = 0,427252 m− 1

This value corresponds to S272 in the following equation.

Calculation of Bessel averaged smoke (Annex 4A, Appendix 1, paragraph 7.3.2.):

In the following equation, the Bessel constants of the previous paragraph 2.2. are used.  The actual unfiltered k-value, as calculated above, corresponds to S272 (Si). S271 (Si-1) and S270 (Si-2) are the two preceding unfiltered k-values, Y271 (Yi-1) and Y270 (Yi-2) are the two preceding filtered k-values.

Y272	=	0.542383 + 8.27277710-5  (0.427252 + 2  0.427392 + 0.427532 − 4  0.542337) + 0.968410  (0.542383 − 0.542337)

=	0.542389 m−1

This value corresponds to Ymax1,A in the following equation.

Calculation of the final smoke value (Annex 4A, Appendix 1, paragraph 7.3.3.):

From each smoke trace, the maximum filtered k-value is taken for the further calculation.  Assume the following values

| Speed | Ymax (m−1) |  |  |
| --- | --- | --- | --- |
|  | Cycle 1 | Cycle 2 | Cycle 3 |
| A | 0.5424 | 0.5435 | 0.5587 |
| B | 0.5596 | 0.5400 | 0.5389 |
| C | 0.4912 | 0.5207 | 0.5177 |

SVA = (0.5424 + 0.5435 + 0.5587) / 3 = 0.5482 m− 1

SVB = (0.5596 + 0.5400 + 0.5389) / 3 = 0.5462 m− 1

SVC = (0.4912 + 0.5207 + 0.5177) / 3 = 0.5099 m− 1

SV = (0.43  0.5482) + (0.56  0.5462) + (0.01  0.5099) = 0.5467 m− 1

Cycle validation (Annex 4A, Appendix 1, paragraph 3.4.)

Before calculating SV, the cycle shall be validated by calculating the relative standard deviations of the smoke of the three cycles for each speed.

| Speed | Mean SV (m−1) | absolute standard deviation (m−1) | relative standard deviation (per cent) |
| --- | --- | --- | --- |
| A | 0.5482 | 0.0091 | 1.7 |
| B | 0.5462 | 0.0116 | 2.1 |
| C | 0.5099 | 0.0162 | 3.2 |

In this example, the validation criteria of 15 per cent are met for each speed.

Table C

Values of opacity N, unfiltered and filtered k-value at beginning of load step

| Index i | Time | Opacity N | unfiltered k-value | filtered k-value |
| --- | --- | --- | --- | --- |
| [−] | [s] | [per cent] | [m−1] | [m−1] |
| − 2 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
| − 1 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
| 0 | 0.000000 | 0.000000 | 0.000000 | 0.000000 |
| 1 | 0.006667 | 0.020000 | 0.000465 | 0.000000 |
| 2 | 0.013333 | 0.020000 | 0.000465 | 0.000000 |
| 3 | 0.020000 | 0.020000 | 0.000465 | 0.000000 |
| 4 | 0.026667 | 0.020000 | 0.000465 | 0.000001 |
| 5 | 0.033333 | 0.020000 | 0.000465 | 0.000002 |
| 6 | 0.040000 | 0.020000 | 0.000465 | 0.000002 |
| 7 | 0.046667 | 0.020000 | 0.000465 | 0.000003 |
| 8 | 0.053333 | 0.020000 | 0.000465 | 0.000004 |
| 9 | 0.060000 | 0.020000 | 0.000465 | 0.000005 |
| 10 | 0.066667 | 0.020000 | 0.000465 | 0.000006 |
| 11 | 0.073333 | 0.020000 | 0.000465 | 0.000008 |
| 12 | 0.080000 | 0.020000 | 0.000465 | 0.000009 |
| 13 | 0.086667 | 0.020000 | 0.000465 | 0.000011 |
| 14 | 0.093333 | 0.020000 | 0.000465 | 0.000012 |
| 15 | 0.100000 | 0.192000 | 0.004469 | 0.000014 |
| 16 | 0.106667 | 0.212000 | 0.004935 | 0.000018 |
| 17 | 0.113333 | 0.212000 | 0.004935 | 0.000022 |
| 18 | 0.120000 | 0.212000 | 0.004935 | 0.000028 |
| 19 | 0.126667 | 0.343000 | 0.007990 | 0.000036 |
| 20 | 0.133333 | 0.566000 | 0.013200 | 0.000047 |
| 21 | 0.140000 | 0.889000 | 0.020767 | 0.000061 |
| 22 | 0.146667 | 0.929000 | 0.021706 | 0.000082 |
| 23 | 0.153333 | 0.929000 | 0.021706 | 0.000109 |
| 24 | 0.160000 | 1.263000 | 0.029559 | 0.000143 |
| 25 | 0.166667 | 1.455000 | 0.034086 | 0.000185 |
| 26 | 0.173333 | 1.697000 | 0.039804 | 0.000237 |
| 27 | 0.180000 | 2.030000 | 0.047695 | 0.000301 |
| 28 | 0.186667 | 2.081000 | 0.048906 | 0.000378 |
| 29 | 0.193333 | 2.081000 | 0.048906 | 0.000469 |
| 30 | 0.200000 | 2.424000 | 0.057067 | 0.000573 |
| 31 | 0.206667 | 2.475000 | 0.058282 | 0.000693 |
| 32 | 0.213333 | 2.475000 | 0.058282 | 0.000827 |
| 33 | 0.220000 | 2.808000 | 0.066237 | 0.000977 |
| 34 | 0.226667 | 3.010000 | 0.071075 | 0.001144 |
| 35 | 0.233333 | 3.253000 | 0.076909 | 0.001328 |
| 36 | 0.240000 | 3.606000 | 0.085410 | 0.001533 |
| 37 | 0.246667 | 3.960000 | 0.093966 | 0.001758 |
| 38 | 0.253333 | 4.455000 | 0.105983 | 0.002007 |
| 39 | 0.260000 | 4.818000 | 0.114836 | 0.002283 |
| 40 | 0.266667 | 5.020000 | 0.119776 | 0.002587 |
| ∼ | ∼ | ∼ | ∼ | ∼ |

Values of opacity N, unfiltered and filtered k-value around Ymax1,A (peak value, indicated in bold number)

| Index i | Time | Opacity N | unfiltered k-value | filtered k-value |
| --- | --- | --- | --- | --- |
| [−] | [s] | [ per cent] | [m−1] | [m−1] |
| ∼ | ∼ | ∼ | ∼ | ∼ |
| 259 | 1.726667 | 17.182000 | 0.438429 | 0.538856 |
| 260 | 1.733333 | 16.949000 | 0.431896 | 0.539423 |
| 261 | 1.740000 | 16.788000 | 0.427392 | 0.539936 |
| 262 | 1.746667 | 16.798000 | 0.427671 | 0.540396 |
| 263 | 1.753333 | 16.788000 | 0.427392 | 0.540805 |
| 264 | 1.760000 | 16.798000 | 0.427671 | 0.541163 |
| 265 | 1.766667 | 16.798000 | 0.427671 | 0.541473 |
| 266 | 1.773333 | 16.788000 | 0.427392 | 0.541735 |
| 267 | 1.780000 | 16.788000 | 0.427392 | 0.541951 |
| 268 | 1.786667 | 16.798000 | 0.427671 | 0.542123 |
| 269 | 1.793333 | 16.798000 | 0.427671 | 0.542251 |
| 270 | 1.800000 | 16.793000 | 0.427532 | 0.542337 |
| 271 | 1.806667 | 16.788000 | 0.427392 | 0.542383 |
| 272 | 1.813333 | 16.783000 | 0.427252 | 0.542389 |
| 273 | 1.820000 | 16.780000 | 0.427168 | 0.542357 |
| 274 | 1.826667 | 16.798000 | 0.427671 | 0.542288 |
| 275 | 1.833333 | 16.778000 | 0.427112 | 0.542183 |
| 276 | 1.840000 | 16.808000 | 0.427951 | 0.542043 |
| 277 | 1.846667 | 16.768000 | 0.426833 | 0.541870 |
| 278 | 1.853333 | 16.010000 | 0.405750 | 0.541662 |
| 279 | 1.860000 | 16.010000 | 0.405750 | 0.541418 |
| 280 | 1.866667 | 16.000000 | 0.405473 | 0.541136 |
| 281 | 1.873333 | 16.010000 | 0.405750 | 0.540819 |
| 282 | 1.880000 | 16.000000 | 0.405473 | 0.540466 |
| 283 | 1.886667 | 16.010000 | 0.405750 | 0.540080 |
| 284 | 1.893333 | 16.394000 | 0.416406 | 0.539663 |
| 285 | 1.900000 | 16.394000 | 0.416406 | 0.539216 |
| 286 | 1.906667 | 16.404000 | 0.416685 | 0.538744 |
| 287 | 1.913333 | 16.394000 | 0.416406 | 0.538245 |
| 288 | 1.920000 | 16.394000 | 0.416406 | 0.537722 |
| 289 | 1.926667 | 16.384000 | 0.416128 | 0.537175 |
| 290 | 1.933333 | 16.010000 | 0.405750 | 0.536604 |
| 291 | 1.940000 | 16.010000 | 0.405750 | 0.536009 |
| 292 | 1.946667 | 16.000000 | 0.405473 | 0.535389 |
| 293 | 1.953333 | 16.010000 | 0.405750 | 0.534745 |
| 294 | 1.960000 | 16.212000 | 0.411349 | 0.534079 |
| 295 | 1.966667 | 16.394000 | 0.416406 | 0.533394 |
| 296 | 1.973333 | 16.394000 | 0.416406 | 0.532691 |
| 297 | 1.980000 | 16.192000 | 0.410794 | 0.531971 |
| 298 | 1.986667 | 16.000000 | 0.405473 | 0.531233 |
| 299 | 1.993333 | 16.000000 | 0.405473 | 0.530477 |
| 300 | 2.000000 | 16.000000 | 0.405473 | 0.529704 |
| ∼ | ∼ | ∼ | ∼ | ∼ |

3.	ETC TEST

3.1.	Gaseous emissions (diesel engine)

Assume the following test results for a PDP-CVS system

| V0 (m3/rev) | 0.1776 |
| --- | --- |
| Np (rev) | 23073 |
| pB (kPa) | 98.0 |
| p1 (kPa) | 2.3 |
| T (K) | 322.5 |
| Ha (g/kg) | 12.8 |
| NOx conce (ppm) | 53.7 |
| NOx concd (ppm) | 0.4 |
| COconce (ppm) | 38.9 |
| COconcd (ppm) | 1.0 |
| HCconce (ppm) | 9.00 |
| HCconcd (ppm) | 3.02 |
| CO2,conce (per cent) | 0.723 |
| Wact (kWh) | 62.72 |

Calculation of the diluted exhaust gas flow (Annex 4A, Appendix 2, paragraph 4.1.):

MTOTW	= 1.293  0.1776  23 073  (98.0 − 2.3)  273 / (101.3  322.5)

= 423 7.2 kg

Calculation of the NOx correction factor (Annex 4A, Appendix 2, paragraph 4.2.):

Calculation of the background corrected concentrations (Annex 4A, Appendix 2, paragraph 4.3.1.1.):

Assuming a diesel fuel of the composition C1H1,8

NOx conc = 53.7 − 0.4  (1 − (1/18.69)) = 53.3 ppm

COconc = 38.9 − 1.0  (1 − (1/18.69)) = 37.9 ppm

HCconc = 9.00 − 3.02  (1 − (1/18.69)) = 6.14 ppm

Calculation of the emissions mass flow (Annex 4A, Appendix 2, paragraph 4.3.1.):

NOx mass = 0.001587  53.3  1.039  423 7.2 = 372.391 g

COmass = 0.000966  37.9  423 7.2 = 155.129 g

HCmass = 0.000479  6.14  423 7.2 = 12.462 g

Calculation of the specific emissions (Annex 4A, Appendix 2, paragraph 4.4.):

= 372.391/62.72 = 5.94 g/kWh

= 155.129/62.72 = 2.47 g/kWh

= 12.462/62.72 = 0.199 g/kWh

3.2.	Particulate emissions (diesel engine)

Assume the following test results for a PDP-CVS system with double dilution

| MTOTW (kg) | 4237.2 |
| --- | --- |
| Mf,p (mg) | 3.030 |
| Mf,b (mg) | 0.044 |
| MTOT (kg) | 2.159 |
| MSEC (kg) | 0.909 |
| Md (mg) | 0.341 |
| MDIL (kg) | 1.245 |
| DF | 18.69 |
| Wact (kWh) | 62.72 |

Calculation of the mass emission (Annex 4A, Appendix 2, paragraph 5.1.):

Mf = 3.030 + 0.044 = 3.074 mg

MSAM = 2.159 − 0.909 = 1.250 kg

Calculation of the background corrected mass emission (Annex 4A, Appendix 2, paragraph 5.1.):

Calculation of the specific emission (Annex 4A, Appendix 2, paragraph 5.2.):

= 10.42/62.72 = 0.166 g/kWh

= 9.32/62.72 = 0.149 g/kWh, if background corrected

3.3.	Gaseous emissions (CNG engine)

Assume the following test results for a PDP-CVS system with double dilution

| MTOTW (kg) | 4237.2 |
| --- | --- |
| Ha (g/kg) | 12.8 |
| NOx conce (ppm) | 17.2 |
| NOx concd (ppm) | 0.4 |
| COconce (ppm) | 44.3 |
| COconcd (ppm) | 1.0 |
| HCconce (ppm) | 27.0 |
| HCconcd (ppm) | 3.02 |
| CH4 conce (ppm) | 18.0 |
| CH4 concd (ppm) | 1.7 |
| CO2,conce ( per cent) | 0.723 |
| Wact (kWh) | 62.72 |

Calculation of the NOx, correction factor (Annex 4A, Appendix 2, paragraph 4.2.):

Calculation of the NMHC concentration (Annex 4A, Appendix 2, paragraph 4.3.1.):

(a)	GC method

NMHCconce = 27.0 − 18.0 = 9.0 ppm

(b)	NMC method

Assuming a methane efficiency of 0.04 and an ethane efficiency of 0.98 (see Annex III, Appendix 5, paragraph 1.8.4.)

Calculation of the background corrected concentrations (Annex 4A, Appendix 2, paragraph 4.3.1.1.):

Assuming a G20 reference fuel (100 per cent methane) of the composition C1H4:

For NMHC, the background concentration is the difference between HCconcd and CH4concd

NOx conc = 17.2 − 0.4  (1 − (1/13.01)) = 16.8 ppm

COconc = 44.3 − 1.0  (1 − (1/13.01)) = 43.4 ppm

NMHCconc = 8.4 − 1.32  (1 − (1/13.01)) = 7.2 ppm

CH4 conc = 18.0 − 1.7  (1 − (1/13.01)) = 16.4 ppm

Calculation of the emissions mass flow (Annex 4A, Appendix 2, paragraph 4.3.1.):

NOx mass = 0.001587  16.8  1.074  423 7.2 = 121.330 g

COmass = 0.000966  43.4  423 7.2 = 177.642 g

NMHCmass = 0.000502  7.2  423 7.2 = 15.315 g

CH4 mass = 0.000554  16.4  423 7.2 = 38.498 g

Calculation of the specific emissions (Annex 4A, Appendix 2, paragraph 4.4.):

= 121.330/62.72 = 1.93 g/kWh

= 177.642/62.72 = 2.83 g/kWh

= 15.315/62.72 = 0.244 g/kWh

= 38.498/62.72 = 0.614 g/kWh

4.	λ-SHIFT FACTOR (Sλ)

4.1.	Calculation of the λ-shift factor (Sλ) /

where:

| Sλ | = λ-shift factor; |
| --- | --- |
| inert per cent | = per cent by volume of inert gases in the fuel (i.e. N2, CO2, He, etc.); |
|  | = per cent by volume of original oxygen in the fuel; |
| N and m | = refer to average CnHm representing the fuel hydrocarbons, i.e: |

where:

| CH4 | = per cent by volume of methane in the fuel; |
| --- | --- |
| C2 | = per cent by volume of all C2 hydrocarbons (e.g.: C2H6, C2H4, etc.) in the fuel; |
| C3 | = per cent by volume of all C3 hydrocarbons (e.g.: C3H8, C3H6, etc.) in the fuel; |
| C4 | = per cent by volume of all C4 hydrocarbons (e.g.: C4H10, C4H8, etc.) in the fuel |
| C5 | = per cent by volume of all C5 hydrocarbons (e.g.: C5H12, C5H10, etc.) in the fuel; |
| diluent | = per cent by volume of dilution gases in the fuel (i.e.: O2*, N2, CO2, He, etc.). |

4.2.	Examples for the calculation of the λ-shift factor Sλ:

Example 1: G25: CH4 = 86 per cent, N2 = 14 per cent (by volume)

Example 2: GR: CH4 = 87 per cent, C2H6 = 13 per cent (by vol)

Example 3: USA: CH4 = 89 per cent, C2H6 = 4.5 per cent, C3H8 = 2.3 per cent, C6H14 = 0.2 per cent, O2 = 0.6 per cent, N2 = 4 per cent

Annex 7

PROCEDURES FOR CONDUCTING THE TEST FOR DURABILITY

OF EMISSION CONTROL SYSTEMS

1.	INTRODUCTION

This annex details the procedures for selecting a family of engines to be tested over a service accumulation schedule for the purpose of determining deterioration factors. Such deterioration factors will be applied to the measured emissions from engines undergoing a periodical audit to ensure that in-service engine emissions remain in conformity with the applicable emission limits, as given in the tables in paragraph 5.2.1. to this Regulation, over the durability period applicable to the vehicle in which the engine is installed.

This annex also details the emission and non-emission-related maintenance that will be carried out on engines undergoing a service accumulation schedule. Such maintenance will be performed on in-service engines and communicated to owners of new heavy-duty engines.

2.	SELECTION OF ENGINES FOR ESTABLISHING USEFUL LIFE DETERIORATION FACTORS

2.1.	Engines will be selected from the engine family defined in paragraph 7.1. to this Regulation for emission testing to establish useful life deterioration factors.

2.2.	Engines from different engine families may be further combined into families based on the type of exhaust aftertreatment system utilised.  In order to place engines with different numbers of cylinders and different cylinder configuration but having the same technical specifications and installation for the exhaust aftertreatment systems into the same engine-aftertreatment system family, the manufacturer shall provide data to the approval authority that demonstrates that the emissions of such engines are equivalent.

2.3.	One engine representing the engine-aftertreatment system family shall be selected by the engine manufacturer for testing over the service accumulation schedule defined in paragraph 3.2. of this annex, according to the criteria for selecting engines given in paragraph 7.2. to this Regulation and shall be reported to the approval authority before any testing commences.

2.3.1.	If the approval authority decides that the worst case emission rate of the engine-aftertreatment system family can be characterized better by another engine then the test engine shall be selected by the approval authority after consultation with the engine manufacturer.

3.	ESTABLISHING USEFUL LIFE DETERIORATION FACTORS

3.1.	General

Deterioration factors applicable to an engine-aftertreatment system family are developed from the selected engines based on a distance and service accumulation procedure that includes periodic testing for gaseous and particulate emissions over the ESC and ETC tests.

3.2.	Service accumulation schedule

Service accumulation schedules may be carried out at the choice of the manufacturer by running a vehicle equipped with the selected parent engine over an "in-service accumulation" schedule or by running the selected parent engine over a "dynamometer service accumulation" schedule.

3.2.1.	In-service and dynamometer service accumulation

3.2.1.1.	The manufacturer shall determine the form and extent of the distance and service accumulation for engines, consistent with good engineering practice.

3.2.1.2.	The manufacturer will determine when the engine will be tested for gaseous and particulate emissions over the ESC and ETC tests.

3.2.1.3.	A single engine-operating schedule shall be used for all engines in an engine-aftertreatment system family.

3.2.1.4.	At the request of the manufacturer and with the agreement of the approval authority, only one test cycle (either the ESC or ETC test) need be run at each test point with the other test cycle run only at the beginning and at the end of the service accumulation schedule.

3.2.1.5.	Operating schedules may be different for different engine-aftertreatment system families.

3.2.1.6.	Operating schedules may be shorter than the useful life period provided that the number of test points allows for a proper extrapolation of the test results, according to paragraph 3.5.2. In any case, the service accumulation shall not be shorter than shown in the table in paragraph 3.2.1.8.

3.2.1.7.	The manufacturer has to provide the applicable correlation between minimum service accumulation period (driving distance) and engine dynamometer hours, for example, fuel consumption correlation, vehicle speed versus engine revolutions correlation etc.

3.2.1.8.	Minimum service accumulation

| Category of vehicle in which engine will be installed | Minimum service accumulation period | Useful life (paragraph of this Regulation) |
| --- | --- | --- |
| Category N1 vehicles | 100,000 km | Paragraph 5.3.1.1. |
| Category N2 vehicles | 125,000 km | Paragraph 5.3.1.2. |
| Category N3 vehicles with a maximum technically permissible mass not exceeding 16 tonnes | 125,000 km | Paragraph 5.3.1.2. |
| Category N3 vehicles with a maximum technically permissible mass exceeding 16 tonnes | 167,000 km | Paragraph 5.3.1.3. |
| Category M2 vehicles | 100,000 km | Paragraph 5.3.1.1. |
| Category M3 vehicles of classes I, II, A and B, with a maximum technically permissible mass not exceeding 7.5 tonnes | 125,000 km | Paragraph 5.3.1.2. |
| Category M3 vehicles of classes III and B, with a maximum technically permissible mass exceeding 7.5 tonnes | 167,000 km | Paragraph 5.3.1.3. |

3.2.1.9.	The in-service accumulation schedule shall be fully described in the application for approval and reported to the approval authority before the start of any testing.

3.2.2.	If the approval authority decides that additional measurements need to be carried out on the ESC and ETC tests between the points selected by the manufacturer it shall notify the manufacturer.  The revised in-service accumulation schedule or dynamometer service accumulation schedule shall be prepared by the manufacturer and agreed by the approval authority.

3.3.	Engine testing

3.3.1.	Start of the service accumulation schedule

3.3.1.1.	For each engine-aftertreatment system family, the manufacturer shall determine the number of hours of engine running after which the operation of the engine-after-treatment system has stabilised.  If requested by the approval authority the manufacturer shall make available the data and analysis used to make this determination.  As an alternative, the manufacturer may elect to run the engine for 125 hours to stabilise the engine-aftertreatment system.

3.3.1.2.	The stabilisation period determined in paragraph 3.3.1.1. will be deemed to be the start of the service accumulation schedule.

3.3.2.	Service accumulation testing

3.3.2.1.	After stabilisation, the engine will be run over the service accumulation schedule selected by the manufacturer, as described in paragraph 3.2. above.  At the periodic intervals in the service accumulation schedule determined by the manufacturer, and, where appropriate, also stipulated by the approval authority according to paragraph 3.2.2., the engine shall be tested for gaseous and particulate emissions over the ESC and ETC tests.  In accordance with paragraph 3.2., if it has been agreed that only one test cycle (ESC or ETC) be run at each test point, the other test cycle (ESC or ETC) shall be run at the beginning and end of the service accumulation schedule.

3.3.2.2.	During the service accumulation schedule, maintenance will be carried out on the engine according to paragraph 4.

3.3.2.3.	During the service accumulation schedule, unscheduled maintenance on the engine or vehicle may be performed, for example if the OBD system has specifically detected a problem that has resulted in the malfunction indicator (MI) being activated.

3.4.	Reporting

3.4.1.	The results of all emission tests (ESC and ETC) conducted during the service accumulation schedule shall be made available to the approval authority.  If any emission test is declared to be void, the manufacturer shall provide an explanation of why the test has been declared void.  In such a case, another series of emission tests over the ESC and ETC tests shall be carried out within a further 100 hours of service accumulation.

3.4.2.	Whenever a manufacturer tests an engine over a service accumulation schedule for the establishment of deterioration factors, the manufacturer shall retain in its records all information concerning all the emission tests and maintenance carried out on the engine during the service accumulation schedule.  This information shall be submitted to the approval authority along with the results of the emission tests conducted over the service accumulation schedule.

3.5.	Determination of deterioration factors

3.5.1.	For each pollutant measured on the ESC and ETC tests and at each test point during the service accumulation schedule, a "best fit" regression analysis shall be made on the basis of all test results.  The results of each test for each pollutant shall be expressed to the same number of decimal places as the limit value for that pollutant, as shown in the tables in paragraph 5.2.1. to this Regulation, plus one additional decimal place.  In accordance with paragraph 3.2., if it has been agreed that only one test cycle (ESC or ETC) be run at each test point and the other test cycle (ESC or ETC) run only at the beginning and end of the service accumulation schedule, the regression analysis shall be made only on the basis of the test results from the test cycle run at each test point.

3.5.2.	On the basis of the regression analysis, the manufacturer shall calculate the projected emission values for each pollutant at the start of the service accumulation schedule and at the useful life that is applicable for the engine under test by extrapolation of the regression equation as determined in paragraph 3.5.1.

3.5.3.	For engines not equipped with an exhaust aftertreatment system, the deterioration factor for each pollutant is the difference between the projected emission values at the useful life period and at the start of the service accumulation schedule.

For engines equipped with an exhaust aftertreatment system, the deterioration factor for each pollutant is the ratio of the projected emission values at the useful life period and at the start of the service accumulation schedule.

In accordance with paragraph 3.2., if it has been agreed that only one test cycle (ESC or ETC) be run at each test point and the other test cycle (ESC or ETC) run only at the beginning and end of the service accumulation schedule, the deterioration factor calculated for the test cycle that has been run at each test point shall be applicable also for the other test cycle, provided that for both test cycles, the relationship between the measured values run at the beginning and at the end of the service accumulation schedule are similar.

3.5.4.	The deterioration factors for each pollutant on the appropriate test cycles shall be recorded in paragraph 1.4. of Appendix 1 to Annex 6 to this Regulation.

3.6.	As an alternative to using a service accumulation schedule to determine deterioration factors, engine manufacturers may choose to use the following deterioration factors:

| Engine type | Test cycle | CO | HC | NMHC | CH4 | NOx | PM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel engine | ESC | 1.1 | 1.05 | - | - | 1.05 | 1.1 |
|  | ETC | 1.1 | 1.05 | - | - | 1.05 | 1.1 |
| Gas engine | ETC | 1.1 | 1.05 | 1.05 | 1.2 | 1.05 | - |

3.6.1.	The manufacturer may select to carry across the DF's determined for an engine or engine/aftertreatment combination to engines or engine/aftertreatment combinations that do not fall into the same engine family category as determined according to paragraph 2.1.  In such cases, the manufacturer shall demonstrate to the approval authority that the base engine or engine/aftertreatment combination and the engine or engine/aftertreatment combination for which the DF's are being carried over have the same technical specifications and installation requirements on the vehicle and that the emissions of such engine or engine/aftertreatment combinations are similar.

3.7.	Checking of conformity of production

3.7.1.	Conformity of production for emissions compliance is checked on the basis of paragraph 8. to this Regulation.

3.7.2.	At the time of approval, the manufacturer may choose to measure at the same time the pollutant emissions before any exhaust aftertreatment system.  In so doing, the manufacturer may develop an informal deterioration factor separately for the engine and the aftertreatment system that may be used by the manufacturer as an aid to end of production line auditing.

3.7.3.	For the purposes of approval, only the deterioration factors adopted by the manufacturer from paragraph 3.6.1. or the deterioration factors developed according to paragraph 3.5. shall be recorded in paragraph 1.4. of Appendix 1 to Annex 6 to this Regulation.

4.	MAINTENANCE

During the service accumulation schedule, maintenance performed on engines and proper consumption of any required reagent used to determine deterioration factors are classified as either emission-related or non-emission-related and each of these can be classified as scheduled and unscheduled.  Some emission-related maintenance is also classified as critical emission-related maintenance.

4.1.	Emission-related scheduled maintenance

4.1.1.	This paragraph specifies emission-related scheduled maintenance for the purpose of conducting a service accumulation schedule and for inclusion in the maintenance instructions furnished to owners of new heavy-duty vehicles and heavy-duty engines.

4.1.2.	All emission-related scheduled maintenance for purposes of conducting a service accumulation schedule shall occur at the same or equivalent distance intervals that will be specified in the manufacturer's maintenance instructions to the owner of the heavy-duty vehicle or heavy-duty engine.  This maintenance schedule may be updated as necessary throughout the service accumulation schedule provided that no maintenance operation is deleted from the maintenance schedule after the operation has been performed on the test engine.

4.1.3.	Any emission-related maintenance performed on engines shall be necessary to assure in-use conformity with the relevant emission standards.  The manufacturer shall submit data to the approval authority to demonstrate that all of the emission-related scheduled maintenance is technically necessary.

4.1.4.	The engine manufacturer shall specify the adjustment, cleaning and maintenance (where necessary) of the following items:

- (a)	Filters and coolers in the exhaust gas re-circulation system ;

- (b)	Positive crankcase ventilation valve;

- (c)	Fuel injector tips (cleaning only);

- (d)	Fuel injectors;

- (e)	Turbocharger;

- (f)	Electronic engine control unit and its associated sensors and actuators;

- (g)	Particulate filter system (including related components);

- (h)	Exhaust gas re-circulation system, including all related control valves and tubing;

- (i)	Any exhaust aftertreatment system.

4.1.5.	For the purposes of maintenance, the following components are defined as critical emission-related items:

- (a)	Any exhaust aftertreatment system;

- (b)	Electronic engine control unit and its associated sensors and actuators;

- (c)	Exhaust gas re-circulation system including all related filters, coolers, control valves and tubing;

- (d)	Positive crankcase ventilation valve.

4.1.6.	All critical emission-related scheduled maintenance shall have a reasonable likelihood of being performed in-use.  The manufacturer shall demonstrate to the approval authority the reasonable likelihood of such maintenance being performed in-use and such demonstration shall be made prior to the performance of the maintenance during the service accumulation schedule.

4.1.7.	Critical emission-related scheduled maintenance items that satisfy any of the conditions defined in paragraphs 4.1.7.1. to 4.1.7.4. will be accepted as having a reasonable likelihood of the maintenance item being performed in-use.

4.1.7.1.	Data is submitted which establishes a connection between emissions and vehicle performance such that as emissions increase due to lack of maintenance, vehicle performance will simultaneously deteriorate to a point unacceptable for typical driving.

4.1.7.2.	Survey data is submitted which demonstrates that, at an 80 per cent confidence level, 80 per cent of such engines already have this critical maintenance item performed in-use at the recommended interval(s).

4.1.7.3.	In association with the requirements of paragraph [3.6….] of Annex 9A to this Regulation, a clearly visible indicator shall be installed on the dashboard of the vehicle to alert the driver that maintenance is due.  The indicator shall be actuated at the appropriate distance or by component failure.  The indicator shall remain activated while the engine is in operation and shall not be erased without the required maintenance being carried out.  Re-setting of the signal shall be a required step in the maintenance schedule.  The system shall not be designed to deactivate upon the end of the appropriate useful life period of the engine or thereafter.

4.1.7.4.	Any other method which the approval authority determines as establishing a reasonable likelihood that the critical maintenance will be performed in-use.

4.2.	Changes to scheduled maintenance

4.2.1.	The manufacturer shall submit a request to the approval authority for approval of any new scheduled maintenance that it wishes to perform during the service accumulation schedule and thereby recommend to owners of heavy-duty vehicles and engines.  The manufacturer shall also include its recommendation as to the category (i.e. emission-related, non-emission-related, critical or non-critical) of the new scheduled maintenance being proposed and, for emission-related maintenance, the maximum feasible maintenance interval.  The request shall be accompanied by data supporting the need for the new scheduled maintenance and the maintenance interval.

4.3.	Non-emission-related scheduled maintenance

4.3.1.	Non-emission-related scheduled maintenance which is reasonable and technically necessary (e.g. oil change, oil filter change, fuel filter change, air filter change, cooling system maintenance, idle speed adjustment, governor, engine bolt torque, valve lash, injector lash, timing, adjustment of the tension of any drive-belt, etc) may be performed on engines or vehicles selected for the service accumulation schedule at the least frequent intervals recommended by the manufacturer to the owner (e.g. not at the intervals recommended for severe service).

4.4.	Maintenance on engines selected for testing over a service accumulation schedule

4.4.1.	Repairs to the components of an engine selected for testing over a service accumulation schedule other than the engine, emission control system or fuel system shall be performed only as a result of part failure or engine system malfunction.

4.4.2.	Equipment, instruments or tools may not be used to identify malfunctioning, maladjusted or defective engine components unless the same or equivalent equipment, instruments or tools will be available to dealerships and other service outlets and,

- (a)	Are used in conjunction with scheduled maintenance on such components, and;

- (b)	Are used subsequent to the identification of an engine malfunction.

4.5.	Critical emission-related unscheduled maintenance

4.5.1.	The consumption of a required reagent is defined as critical emission-related unscheduled maintenance for the purpose of conducting a service accumulation schedule and for inclusion in the maintenance instructions furnished by manufacturers to owners of new heavy-duty vehicles or heavy-duty engines.

Annex 8

CONFORMITY OF IN-SERVICE VEHICLES/ENGINES

1.	GENERAL

1.1.	With reference to approvals granted for emissions, measures are appropriate for confirming the functionality of the emission control devices during the useful life of an engine installed in a vehicle under normal conditions of use (conformity of in-service vehicles/engines properly maintained and used).

1.2.	For the purpose of this Regulation these measures shall be checked over a period corresponding to the appropriate useful life period defined in paragraph 5.3. of this Regulation for vehicles or engines which are approved to either row B1, row B2 or row C of the tables in paragraph 5.2.1. of this Regulation.

1.3.	The checking of conformity of in-service vehicles/engines is done on the basis of information provided by the manufacturer to the approval authority conducting an audit of the emissions-performance of a range of representative vehicles or engines of which the manufacturer holds the approval.

Figures 1 in this annex illustrates the procedure for in-service conformity checking.

2.	PROCEDURES FOR AUDIT

2.1.	Audit of in-service conformity by the administrative department is conducted on the basis of any relevant information that the manufacturer has, under procedures similar to those defined in Appendix 2 of the 1958 Agreement (E/ECE/324-E/ECE/TRANS/505/Rev.2).  Alternatives are in-service monitoring reports supplied by the manufacturer, approval authority surveillance testing and/or information on surveillance testing performed by a Contracting Party.  The procedures to be used are given in paragraph 3.

3.	AUDIT PROCEDURES

3.1.	An audit of in-service conformity will be conducted by the approval authority on the basis of information supplied by the manufacturer.  The manufacturer's in-service monitoring (ISM) report should be based on in-use testing of engines or vehicles using proven and relevant testing protocols.  Such information (the ISM report) shall include, but is not limited to, the following (see paragraphs 3.1.1. to 3.1.13.):

3.1.1.	The name and address of the manufacturer.

3.1.2.	The name, address, telephone and fax numbers and e-mail address of his authorized representative within the areas covered by the manufacturer's information.

3.1.3.	The model name(s) of the engines included in the manufacturer's information.

3.1.4.	The list of engine types covered within the manufacturer's information, i.e. the engine-after-treatment system family.

3.1.5.	The vehicle identification number (VIN) codes applicable to the vehicles equipped with an engine that is part of the audit.

Figure 1. In-service conformity checking – audit procedure

Does the

AA(a) accept that

manufacturer's in-service

conformity report confirms acceptability of a vehicle or engine type within the family? (paragraph 3.4. of

this annex)

Does

the AA(a)

decide to audit the manufacturer's compliance data for this vehicle or engine type or family?

In-house in-service conformity report for approved vehicle or engine type or family

START

NO

YES

NO

Manufacturer submits in-service conformity report to AA(a) for audit

YES

AA(a) reviews manufacturer's in-service conformity report

Process Completed.

No further action required.

YES

Manufacturer provides or obtains additional information or test data.

Manufacturer compiles new in-service conformity report.

Does AA(a)

decide that information is insufficient to reach a decision?

NO

AA(a) begins formal testing of suspect engine type or family (as described in paragraph 5. of this annex )

Manufacturer files report for future reference

Vehicle or engine manufacturer and Approval Authority complete vehicle or engine approval for the new vehicle or engine type.

Approval Authority (AA) grants approval.

Manufacture and sales of approved vehicle or engine type.

Vehicle or engine manufacturer develops own in-service conformity procedure

Vehicle or engine manufacturer carries out own in-service conformity procedure (vehicle or engine type or family)

Vehicle manufacturer compiles report of the in-house procedure (including all data required by paragraph 3. of this annex)

(a)	In this case, AA means the Approval Authority that granted the approval.

3.1.6.	The numbers of the type approvals applicable to the engine types within the in-service family, including, where applicable, the numbers of all extensions and field fixes/recalls (re-works):

3.1.7.	Details of extensions, field fixes/recalls to those type approvals for the engines covered within the manufacturer's information (if requested by the approval authority).

3.1.8.	The period of time over which the manufacturer's information was collected.

3.1.9.	The engine build period covered within the manufacturer's information (e.g. "vehicles or engines manufactured during the 2005 calendar year").

3.1.10.	The manufacturer's in-service conformity checking procedure, including:

3.1.10.1.	Vehicle or engine location method;

3.1.10.2.	Selection and rejection criteria for vehicle or engine;

3.1.10.3.	Test types and procedures used for the programme;

3.1.10.4.	The manufacturer's acceptance/rejection criteria for the in-service family group;

3.1.10.5.	Geographical area(s) within which the manufacturer has collected information;

3.1.10.6.	Sample size and sampling plan used.

3.1.11.	The results from the manufacturer's in-service conformity procedure, including:

3.1.11.1.	Identification of the engines included in the programme (whether tested or not).  The identification will include:

- (a)	model name;

- (b)	vehicle identification number (VIN);

- (c)	engine identification number;

- (d)	vehicle registration number equipped with an engine that is part of the audit;

- (e)	date of manufacture;

- (f)	region of use (where known);

- (g)	type of use of the vehicle (where known), i.e. urban delivery, long haul etc.

3.1.11.2.	The reason(s) for rejecting a vehicle or engine from a sample (e.g., vehicle being in-use for less than one year, improper emission-related maintenance, evidence of using a fuel having a higher sulphur content than required for normal vehicle use, emission control equipment not in conformity with approval).  The reason for rejection shall be substantiated (e.g., the nature of non-fulfilment of maintenance instructions, etc.).  A vehicle should not be excluded solely on the ground that the AECS may have been excessively in operation.

3.1.11.3.	Emission-related servicing and maintenance history for each engine in the sample (including any re-works).

3.1.11.4.	Repair history for each engine in the sample (where known).

3.1.11.5.	Test data, including:

(a)	date of test;

(b)	location of test;

(c)	distance indicated odometer of vehicle equipped with an engine that is covered by the audit;

(d)	test fuel specifications (e.g. test reference fuel or market fuel);

(e)	test conditions (temperature, humidity, dynamometer inertia weight);

(f)	dynamometer settings (e.g. power setting);

(g)	emission test results conducted on the ESC, ETC and ELR tests according to paragraph 4. of this annex.  A minimum of five engines shall be tested;

(h)	alternative to item (g) above, tests may be conducted using another protocol.  The relevance for monitoring in-service functionality with such a test shall be stated and substantiated by manufacturer in conjunction with the approval process (paragraphs 3. and 4. in to this Regulation).

3.1.12.	Records of indication from the OBD system.

3.1.13.	Record of experiences of the use of consumable reagent.  Reports should detail, but not be limited to, operator experiences with the handling of filling, refilling and consumption of the reagent, and the conduct of the filling installations, and, specifically, the frequency of activation in-use of the temporary performance limiter and events of other defect instances, activation of the MI and the registering of a fault code relating to a lack of the consumable reagent.

3.1.13.1.	The manufacturer shall supply in-use and defect reports.  The manufacturer shall report on warranty claims and their nature, and in-field indications of activation/ deactivation of the MI and the registering of a fault code relating to a lack of the consumable reagent and the activation/deactivation of the engine performance limiter (see paragraph 5.5.5. of this Regulation).

3.2	The information gathered by the manufacturer shall be sufficiently comprehensive to ensure that in-service performance can be assessed for normal conditions over the appropriate durability/useful life period defined in paragraph 6.3. of this Regulation and in a way representative of the manufacturer's geographic penetration.

3.3	The manufacturer may whish to run in-service monitoring comprising fewer engines/vehicles than the number given in paragraph 3.1.11.5., item (g), and using a procedure defined under paragraph 3.1.11.5., item (h).  The reason could be that the engines in the engine family(-ies) covered by the report are in a small number.  The conditions should have been agreed on beforehand by the approval authority.

3.4	On the basis of the monitoring report referred to in this paragraph, the approval authority shall either:

- (a)	decide that the in-service conformity of an engine type or an engine family is satisfactory and not to take any further action;

- (b)	decide that the data provided by the manufacturer is insufficient to reach a decision and request additional information and/or test data from the manufacturer.  Where requested, and depending on the approval of the engine, such additional test data shall include ESC, ELR, and ETC test results, or from other proven procedures according to paragraph 3.1.11.5., item (h);

- (c)	decide that the in-service conformity of an engine family is unsatisfactory and proceed to have confirmatory testing carried out on a sample of engines from the engine family, according to paragraph 5. of this annex.

3.5	A Contracting Party may conduct and report its surveillance testing, based on the audit procedure spelled out in this paragraph. Information on the procurement, maintenance, and manufacturer's participation in the activities may be recorded.  Likewise, the Contracting Party may use alternative emission test protocols, according to paragraph 3.1.11.5., item (h).

3.6	The approval authority may take up surveillance testing conducted and reported by a Contracting Party as a basis for the decisions according to paragraph 3.4.

3.7	The manufacturer should report to the approval authority and the Contracting Party(s) where the subject engines/vehicles are kept in service when planning to conduct a voluntary remedial action.  The reporting shall be supplied by the manufacturer in conjunction with taking the decision to take action, specifying the particulars of the action, describe the groups of engines/vehicles to be included in the action, and regularly thereafter on the commencement of the campaign.  The applicable particulars of paragraph 7. to this annex may be used.

4.	EMISSION TESTS

4.1	An engine selected from the engine family shall be tested over the ESC and ETC test cycles for gaseous and particulate emissions and over the ELR test cycle for smoke emission.  The engine shall be representative of the type of use expected for this type of engine, and come from a vehicle in normal use.  The procurement, inspection, and restorative maintenance of the engine/vehicle shall be conducted using a protocol such as is specified in paragraph 3., and shall be documented.

The appropriate maintenance schedule, referred to in paragraph 4. of Annex II, shall have been carried out on the engine.

4.2	The emission values determined from the ESC, ETC and ELR tests shall be expressed to the same number of decimal places as the limit value for that pollutant, as shown in the tables in paragraph 5.2.1. of this Regulation, plus one additional decimal place.

5.	CONFIRMATORY TESTING

5.1	Confirmatory testing is done for the purpose of confirmation of the in-service emission functionality of an engine family.

5.1.1.	If the approval authority is not satisfied with the manufacturers" ISM according to paragraph 3.4. or on a reported evidence of unsatisfactory in-service conformity, e.g., according to paragraph 3.5., may order the manufacturer to run test for confirmatory purposes.  The approval authority will examine the confirmatory test report supplied by the manufacturer.

5.1.2.	The approval authority may conduct confirmatory testing.

5.2	The confirmatory test should be applicable engine ESC, ETC and ELR tests, as specified in paragraph 4.  Representative engines to be tested should be dismounted from vehicles used under normal conditions and be tested.  Alternatively, after prior agreement with the approval authority, the manufacturer may test emission control components from vehicles in use, after being dismounted, transferred and mounted on properly used and representative engine(s).  For each series of tests, the same package of emission control components shall be selected.  The reason for the selection shall be stated.

5.3	A test result may be regarded as non-satisfactory when, from tests of two or more engines representing the same engine family, for any regulated pollutant component, the limit value as shown in paragraph 5.2.1. of this Regulation is exceeded significantly.

6.	ACTIONS TO BE TAKEN

6.1	Where the approval authority is not satisfied with the information or test data supplied by the manufacturer, and, having carried out confirmatory engine testing according to paragraph 5., or based on confirmatory testing conducted by a Contracting Party (paragraph 5.3.), and it is certain that an engine type is not in conformity with the requirements of these provisions, the approval authority shall request the manufacturer to submit a plan of remedial measure to remedy the non-conformity.

6.2	In this case, the remedial measures referred to in Appendix 2 of the 1958 Agreement (E/ECE/324-E/ECE/TRANS/505/Rev.2) are extended to engines in service belonging to the same vehicle type which are likely to be affected with the same defects, in accordance with paragraph 8.

To be valid the plan of remedial measures presented by the manufacturer shall be approved by the approval authority.  The manufacturer is responsible for the execution of the remedial plan as approved.

The approval authority shall notify its decision to all Contracting Parties within 30 days.  The Contracting Parties may require that the same plan of remedial measures be applied to all engines of the same type registered in their territory.

6.3	If a Party to the Agreement has established that a vehicle type does not conform to the applicable requirements of this annex, it shall notify without delay the Party to the Agreement which granted the original type approval in accordance with the requirements of the Agreement.

Then, subject to the provision of the Agreement, the competent authority of the Party to the Agreement which granted the original type approval shall inform the manufacturer that a vehicle type fails to satisfy the requirements of these provisions and that certain measures are expected from the manufacturer.  The manufacturer shall submit to the authority, within two months after this notification, a plan of measures to overcome the defects, the substance of which should correspond with the requirements of paragraph 7.  The competent authority which granted the original approval shall, within two months, consult the manufacturer in order to secure agreement on a plan of measures and on carrying out the plan.  If the competent authority which granted the original type approval establishes that no agreement can be reached, the relevant procedures to the Agreement shall be initiated.

7.	PLAN OF REMEDIAL MEASURES

7.1.	The plan of remedial measures, requested according to paragraph 6.1, shall be filed with the approval authority not later than 60 working days from the date of the notification referred to in paragraph 6.1.  The approval authority shall within 30 working days declare its approval or disapproval of the plan of remedial measures.  However, where the manufacturer can demonstrate to the satisfaction of the competent approval authority, that further time is required to investigate the non-compliance in order to submit a plan of remedial measures, an extension is granted.

7.2.	The remedial measures shall apply to all engines likely to be affected by the same defect.  The need to amend the approval documents shall be assessed.

7.3.	The manufacturer shall provide a copy of all communications related to the plan of remedial measures, shall also maintain a record of the recall campaign, and supply regular status reports to the approval authority.

7.4.	The plan of remedial measures shall include the requirements specified in paragraphs 7.4.1. to 7.4.11.  The manufacturer shall assign a unique identifying name or number to the plan of remedial measures.

7.4.1.	A description of each engine type included in the plan of remedial measures.

7.4.2.	A description of the specific modifications, alterations, repairs, corrections, adjustments, or other changes to be made to bring the engines into conformity including a brief summary of the data and technical studies which support the manufacturer's decision as to the particular measures to be taken to correct the non-conformity.

7.4.3.	A description of the method by which the manufacturer informs the engine or vehicle owners about the remedial measures.

7.4.4.	A description of the proper maintenance or use, if any, which the manufacturer stipulates as a conditions of eligibility for repair under the plan of remedial measures, and an explanation of the manufacturer's reasons for imposing any such condition.  No maintenance or use conditions may be imposed unless it is demonstrably related to the non-conformity and the remedial measures.

7.4.5.	A description of the procedure to be followed by engine owners to obtain correction of the non-conformity.  This shall include a date after which the remedial measures may be taken, the estimated time for the workshop to perform the repairs and where they can be done.  The repair shall be done expediently, within a reasonable time after delivery of the vehicle.

7.4.6.	A copy of the information transmitted to the vehicle owner.

7.4.7.	A brief description of the system which the manufacturer uses to assure an adequate supply of component or systems for fulfilling the remedial action.  It shall be indicated when there will be an adequate supply of components or systems to initiate the campaign.

7.4.8.	A copy of all instructions to be sent to those persons who are to perform the repair.

7.4.9.	A description of the impact of the proposed remedial measures on the emissions, fuel consumption, driveability, and safety of each engine type, covered by the plan of remedial measures with data, technical studies, etc. which support these conclusions.

7.4.10.	Any other information, reports or data the approval authority may reasonably determine is necessary to evaluate the plan of remedial measures.

7.4.11.	Where the plan of remedial measures includes a recall, a description of the method for recording the repair shall be submitted to the approval authority. If a label is used, an example of it shall be submitted.

7.5.	The manufacturer may be required to conduct reasonably designed and necessary tests on components and engines incorporating a proposed change, repair, or modification to demonstrate the effectiveness of the change, repair, or modification.

7.6.	The manufacturer is responsible for keeping a record of every engine or vehicle recalled and repaired and the workshop which performed the repair.  The approval authority shall have access to the record on request for a period of 5 years from the implementation of the plan of remedial measures.

7.7.	The repair and/or modification or addition of new equipment shall be recorded in a certificate supplied by the manufacturer to the owner of the engine.

Annex 9A

ON-BOARD DIAGNOSTIC SYSTEMS (OBD)

1.	INTRODUCTION

This annex describes the provisions specific to the on-board diagnostic (OBD) system for the emission control systems of motor vehicles.

2.	DEFINITIONS

For the purposes of this annex, the following definitions, in addition to the definitions contained in paragraph 2. of this Regulation, apply:

"warm-up cycle" means sufficient engine operation such that the coolant temperature has risen by at least 22 K from engine starting and reaches a minimum temperature of 343 K (70 °C);

"access" means the availability of all emission-related OBD data including all fault codes required for the inspection, diagnosis, servicing or repair of emissions related parts of the vehicle, via the serial interface of the standard diagnostic connector;

"deficiency" means, in respect of engine OBD systems, that up to two separate components or systems that are monitored contain temporary or permanent operating characteristics that impair the otherwise efficient OBD monitoring of those components or systems or do not meet all the other detailed requirements for OBD. Engines or vehicles in respect of their engine may be approved, registered and sold with such deficiencies according to the requirements of paragraph 4.3. of this annex;

"deteriorated component/system" means an engine or exhaust aftertreatment component/system that has been intentionally deteriorated in a controlled manner by the manufacturer for the purpose of conducting a approval test on the OBD system;

"OBD test cycle" means a driving cycle which is a version of the ESC test cycle having the same running-order of the 13 individual modes as described in paragraph 2.7.1. of Appendix 1 to Annex 4A to this Regulation  but where the length of each mode is reduced to 60 seconds;

"operating sequence" means the sequence used for determining the conditions for extinguishing the MI. It consists of an engine start-up, an operating period, an engine shut-off, and the time until the next start-up, where the OBD monitoring is running and a malfunction would be detected if present;

"preconditioning cycle" means the running of at least three consecutive OBD test cycles or emission test cycles for the purpose of achieving stability of the engine operation, the emission control system and OBD monitoring readiness;

"repair information" means all information required for diagnosis, servicing, inspection, periodic monitoring or repair of the engine and which the manufacturers provide for their authorized dealers/repair shops.  Where necessary, such information shall include service handbooks, technical manuals, diagnosis information (e.g. minimum and maximum theoretical values for measurements), wiring diagrams, the software calibration identification number applicable to an engine type, information enabling the update of the software of the electronic systems in accordance with the specifications of the vehicle manufacturer, instructions for individual and special cases, information provided concerning tools and equipment, data record information and two-directional monitoring and test data.  The manufacturer shall not be obliged to make available that information which is covered by intellectual property rights or constitutes specific know-how of manufacturers and/or OEM suppliers; in this case the necessary technical information shall not be improperly withheld;

"standardized" means that all emission related OBD data (i.e. stream information in the case a scanning tool is used), including all fault codes used, shall be produced only in accordance with industry standards which, by virtue of the fact that their format and the permitted options are clearly defined, provide for a maximum level of harmonisation in the motor vehicle industry, and whose use is expressly permitted in this Regulation;

"unrestricted" means:

- (a)	access not dependent on an access code obtainable only from the manufacturer, or a similar device, or,

- (b)	access allowing evaluation of the data produced without the need for any unique decoding information, unless that information itself is standardized.

3.	REQUIREMENTS AND TESTS

3.1.	General requirements

3.1.1.	OBD systems shall be designed, constructed and installed in a vehicle so as to enable it to identify types of malfunction over the entire life of the engine.  In achieving this objective the approval authority shall accept that engines which have been used in excess of the appropriate durability period defined in paragraph 5.3. of this Regulation may show some deterioration in OBD system performance such that the OBD thresholds given in the table in paragraph 5.4.4. of this Regulation may be exceeded before the OBD system signals a failure to the driver of the vehicle.

3.1.2.	A sequence of diagnostic checks shall be initiated at each engine start and completed at least once provided that the correct test conditions are met.  The test conditions shall be selected in such a way that they all occur under the driving conditions as represented by the test defined in paragraph 2. of Appendix 1 to this annex.

3.1.2.1.	Manufacturers are not required to activate a component/system exclusively for the purpose of OBD functional monitoring under vehicle operating conditions when it would not normally be active (e.g. activation of a reagent tank heater of a deNOx system or combined deNOx-particulate filter when such a system would not normally be active).

3.1.3.	OBD may involve devices, which measure, senses or responds to operating variables (e.g. vehicle speed, engine speed, gear used, temperature, intake pressure or any other parameter) for the purpose of detecting malfunctions and of minimizing the risk of indicating false malfunction. These devices are not defeat devices.

3.1.4.	Access to the OBD system required for the inspection, diagnosis, servicing or repair of the engine shall be unrestricted and standardized.  All emission related fault codes shall be consistent with those described in paragraph 6.8.5. of this annex.

3.2.	OBD Stage 1 requirements

3.2.1.	From the dates given in paragraph 5.4.1. of this Regulation, the OBD system of all diesel engines and of vehicles equipped with a diesel engine shall indicate the failure of an emission-related component or system when that failure results in an increase in emissions above the appropriate OBD thresholds given in the table in paragraph 5.4.4. of this Regulation.

3.2.2.	In satisfying the Stage 1 requirements, the OBD system shall monitor for:

3.2.2.1.	complete removal of a catalyst, where fitted in a separate housing, that may or may not be part of a deNOx system or particulate filter.

3.2.2.2.	reduction in the efficiency of the deNOx system, where fitted, with respect to the emissions of NOx only.

3.2.2.3.	reduction in the efficiency of the particulate filter, where fitted, with respect to the emissions of particulate only.

3.2.2.4.	reduction in the efficiency of a combined deNOx-particulate filter system, where fitted, with respect to both the emissions of NOx and particulate.

3.2.3.	Major functional failure

3.2.3.1.	As an alternative to monitoring against the appropriate OBD threshold limits with respect to paragraphs 3.2.2.1. to 3.2.2.4., OBD systems of diesel engines may in accordance with paragraph 5.4.1.1. of this Regulation monitor for major functional failure of the following components:

- (a)	a catalyst, where fitted as a separate unit, that may or may not be part of a deNOx system or particulate filter;

- (b)	a deNOx system, where fitted;

- (c)	a particulate filter, where fitted;

- (d)	a combined deNOx-particulate filter system.

3.2.3.2.	In the case of an engine equipped with a deNOx system, examples of monitoring for major functional failure are for complete removal of the system or replacement of the system by a bogus system (both intentional major functional failure), lack of required reagent for a deNOx system, failure of any SCR electrical component, any electrical failure of a component (e.g. sensors and actuators, dosing control unit) of a deNOx system including, when applicable, the reagent heating system, failure of the reagent dosing system (e.g. missing air supply, clogged nozzle, dosing pump failure).

3.2.3.3.	In the case of an engine equipped with a particulate filter, examples of monitoring for major functional failure are for major melting of the trap substrate or a clogged trap resulting in a differential pressure out of the range declared by the manufacturer, any electrical failure of a component (e.g. sensors and actuators, dosing control unit) of a particulate filter, any failure, when applicable, of a reagent dosing system (e.g. clogged nozzle, dosing pump failure).

3.2.4.	Manufacturers may demonstrate to the approval authority that certain components or systems need not be monitored if, in the event of their total failure or removal, emissions do not exceed the applicable thresholds limits for OBD Stage 1 given in the table in paragraph 5.4.4. of this Regulation when measured over the cycles shown in paragraph 1.1. of Appendix 1 to this annex.  This provision shall not apply to an exhaust gas recirculation (EGR) device, a deNOx system, a particulate filter or a combined deNOx-particulate filter system nor shall it apply to a component or system that is monitored for major functional failure.

3.3.	OBD Stage 2 requirements

3.3.1.	From the dates given in paragraph 5.4.2. of this Regulation the OBD system of all diesel or gas engines and of vehicles equipped with a diesel or a gas engine shall indicate the failure of an emission-related component or system of the engine system when that failure results in an increase in emissions above the appropriate OBD thresholds given in the table in paragraph 5.4.4. of this Regulation.

The OBD system shall consider the communication interface (hardware and messages) between the engine system electronic control unit(s) (EECU) and any other power train or vehicle control unit when the exchanged information has an influence on the correct functioning of the emission control.  The OBD system shall diagnose the integrity of the connection between the EECU and the medium that provides the link with these other vehicle components (e.g. the communication bus).

3.3.2.	In satisfying the Stage 2 requirements, the OBD system shall monitor for:

3.3.2.1.	reduction in the efficiency of the catalyst, where fitted in a separate housing, that may or may not be part of a deNOx system or particulate filter.

3.3.2.2.	reduction in the efficiency of the deNOx system, where fitted, with respect to the emissions of NOx only.

3.3.2.3.	reduction in the efficiency of the particulate filter, where fitted, with respect to the emissions of particulate only.

3.3.2.4.	reduction in the efficiency of a combined deNOx-particulate filter system, where fitted, with respect to both the emissions of NOx and particulate.

3.3.2.5.	the interface between the engine electronic control unit (EECU) and any other powertrain or vehicle electrical or electronic system (e.g. the transmission control unit (TECU)) for electrical disconnection.

3.3.3.	Manufacturers may demonstrate to the approval authority that certain components or systems need not be monitored if, in the event of their total failure or removal, emissions do not exceed the applicable thresholds limits for OBD Stage 2 given in the table in paragraph 5.4.4. of this Regulation when measured over the cycles shown in paragraph 1.1. of Appendix 1 to this annex.  This provision shall not apply to an exhaust gas recirculation (EGR) device, a deNOx system, a particulate filter or a combined deNOx-particulate filter system.

3.4.	Stage 1 and Stage 2 requirements

3.4.1.	In satisfying both the Stage 1 or Stage 2 requirements the OBD system shall monitor:

3.4.1.1.	The fuel-injection system electronic, fuel quantity and timing actuator(s) for circuit continuity (i.e. open circuit or short circuit) and total functional failure.

3.4.1.2.	All other engine or exhaust aftertreatment emission-related components or systems, which are connected to a computer, the failure of which would result in tailpipe emissions exceeding the OBD threshold limits given in the table in paragraph 5.4.4. of this Regulation.  At a minimum, examples include the exhaust gas recirculation (EGR) system, systems or components for monitoring and control of air mass-flow, air volumetric flow (and temperature), boost pressure and inlet manifold pressure (and relevant sensors to enable these functions to be carried out), sensors and actuators of a deNOx system, sensors and actuators of an electronically activated active particulate filter.

3.4.1.3.	Any other emission-related engine or exhaust aftertreatment component or system connected to an electronic control unit shall be monitored for electrical disconnection unless otherwise monitored.

3.4.1.4.	In the case of engines equipped with an aftertreatment system using a consumable reagent, the OBD system shall monitor for:

- (a)	lack of any required reagent;

- (b)	the quality of the required reagent being within the specifications declared by the manufacturer in Annex 1 to this Regulation;

- (c)	reagent consumption and dosing activity;

according to paragraph 5.5.4. of this Regulation.

3.5.	OBD operation and temporary disablement of certain OBD monitoring capabilities

3.5.1.	The OBD system shall be so designed, constructed and installed in a vehicle as to enable it to comply with the requirements of this annex during the conditions of use defined in paragraph 5.1.5.4. of this Regulation.

Outside these normal operating conditions the emission control system may show some degradation in OBD system performance such that the thresholds given in the table in paragraph 5.4.4. of this Regulation may be exceeded before the OBD system signals a failure to the driver of the vehicle.

The OBD system shall not be disabled unless one or more of the following conditions for disablement are met:

3.5.1.1.	The affected OBD monitoring systems may be disabled if its ability to monitor is affected by low fuel levels.  For this reason, disablement is permitted when the fuel tank level falls below 20 per cent of the nominal capacity of the fuel tank.

3.5.1.2.	The affected OBD monitoring systems may be temporarily disabled during the operation of an auxiliary emission control strategy as described in paragraph 5.1.5.1. of this Regulation.

3.5.1.3.	The affected OBD monitoring systems may be temporarily disabled when operational safety or limp-home strategies are activated.

3.5.1.4.	For vehicles designed to accommodate the installation of power take-off units, disablement of affected OBD monitoring systems is permitted provided disablement takes place only when the power take-off unit is active and the vehicle is not being driven.

3.5.1.5.	The affected OBD monitoring systems may be disabled temporarily during the periodic regeneration of an emission control system downstream of the engine (i.e. a particulate filter, deNOx system or combined deNOx-particulate filter).

3.5.1.6.	The affected OBD monitoring systems may be disabled temporarily outside the conditions of use defined in paragraph 5.1.5.4. of this Regulation when this disablement can be justified by a limitation of the OBD monitoring (including modelling) capability.

3.5.2.	The OBD monitoring system is not required to evaluate components during malfunction if such evaluation would result in a risk to safety or component failure.

3.6.	Activation of malfunction indicator (MI)

3.6.1.	The OBD system shall incorporate a malfunction indicator readily visible to the vehicle operator.  Except in the case of paragraph 3.6.2. of this annex, the MI (e.g. symbol or lamp) shall not be used for any purpose other than emission related malfunction except to indicate emergency start-up or limp-home routines to the driver.  Safety related messages can be given the highest priority.  The MI shall be visible in all reasonable lighting conditions. When activated, it shall display a symbol in conformity with ISO 2575 / (as a dashboard telltale lamp or a symbol on a dashboard display).  A vehicle shall not be equipped with more than one general purpose MI for emission-related problems.  Displaying separate specific information is permitted (e.g. such as information dealing with brake system, fasten seat belt, oil pressure, servicing requirements, or indicating the lack of necessary reagent for the deNOx system).  The use of red for the MI is prohibited.

3.6.2.	The MI may be used to indicate to the driver that an urgent service task needs to be carried out.  Such an indication may also be accompanied by an appropriate message on a dashboard display that an urgent servicing requirement needs to be carried out.

3.6.3.	For strategies requiring more than a preconditioning cycle for MI activation, the manufacturer shall provide data and/or an engineering evaluation which adequately demonstrates that the monitoring system is equally effective and timely in detecting component deterioration.  Strategies requiring on average more than ten OBD or emission test cycles for MI activation are not accepted.

3.6.4.	The MI shall also activate whenever the engine control enters a emission default mode of operation.  The MI shall also activate if the OBD system is unable to fulfil the basic monitoring requirements specified in this Regulation.

3.6.5.	Where reference is made to this paragraph, the MI shall be activated and, in addition, a distinct warning mode should also be activated, e.g. flashing MI or activation of a symbol in conformity with ISO 2575 / in addition to MI activation.

3.6.6.	The MI shall activate when the vehicle's ignition is in the "key-on" position before engine starting or cranking and de-activate within 10 seconds after engine starting if no malfunction has previously been detected.

3.7.	Fault code storage

The OBD system shall record fault code(s) indicating the status of the emission-control system.  A fault code shall be stored for any detected and verified malfunction causing MI activation and shall identify the malfunctioning system or component as uniquely as possible.  A separate code should be stored indicating the expected MI activation status (e.g. MI commanded "ON", MI commanded "OFF").

Separate status codes shall be used to identify correctly functioning emission control systems and those emission control systems that need further engine operation to be fully evaluated.  If the MI is activated due to malfunction or emission default modes of operation, a fault code shall be stored that identifies the likely area of malfunction.  A fault code shall also be stored in the cases referred to in paragraphs 3.4.1.1. and 3.4.1.3. of this annex.

3.7.1.	If monitoring has been disabled for 10 driving cycles due to the continued operation of the vehicle under conditions conforming to those specified in paragraph 3.5.1.2. of this annex, readiness for the subject monitoring system may be set to "ready" status without monitoring having been completed.

3.7.2.	The hours run by the engine while the MI is activated shall be available upon request at any instant through the serial port on the standard link connector, according to the specifications given in paragraph 6.8. of this annex.

3.8.	Extinguishing the MI

3.8.1.	The MI may be de-activated after three subsequent sequential operating sequences or 24 engine running hours during which the monitoring system responsible for activating the MI ceases to detect the malfunction and if no other malfunction has been identified that would independently activate the MI.

3.8.2.	In the case of MI activation due to lack of reagent for the deNOx system, or combined deNOx-particulate after-treatment device or use of a reagent outside the specifications declared by the manufacturer, the MI may be switched back to the previous state of activation after filling or replacement of the storage medium with a reagent having the correct specifications.

3.8.3.	In the case of MI activation due to incorrect operation of the engine system with respect to NOx control measures, or incorrect reagent consumption and dosing activity, the MI may be switched back to the previous state of activation if the conditions given in paragraphs 5.5.3., 5.5.4 and 5.5.7. of this Regulation no longer apply.

3.9.	Erasing a fault code

3.9.1	The OBD system may erase a fault code and the hours run by the engine and freeze-frame information if the same fault is not re-registered in at least 40 engine warm-up cycles or 100 engine running hours, whichever occurs first, with the exception of the cases referred to in paragraph 3.9.2.

3.9.2	From 9 November 2006 for new type approvals and from 1 October 2007 for all registrations, in the case of a non-erasable fault code being generated according to paragraph 5.5.3. or 5.5.4. of this Regulation, the OBD system shall retain a record of the fault code and the hours run by the engine during the MI activation for at least 400 days or 9,600 hours of engine operation.

Any such fault code and the corresponding hours run by the engine during MI activation shall not be erased through use of any external diagnostic or other tool as referred to in paragraph 6.8.3. of this annex.

4.	REQUIREMENTS RELATING TO THE APPROVAL OF OBD SYSTEMS

4.1.	For the purpose of approval, the OBD system shall be tested according to the procedures given in Appendix 1 to this annex.

An engine representative of its engine family (see paragraph 7. of this Regulation) shall be used for the OBD demonstration tests or the test report of the parent OBD system of the OBD engine family will be provided to the approval authority as an alternative to carrying out the OBD demonstration test.

4.1.1.	In the case of OBD stage 1 referred to in paragraph 3.2., the OBD system shall:

4.1.1.1.	indicate the failure of an emission-related component or system when that failure results in an increase in emissions above the OBD thresholds given in the table in paragraph 5.4.4. of this Regulation, or;

4.1.1.2.	where appropriate, indicate any major functional failure of an exhaust aftertreatment system.

4.1.2.	In the case of OBD stage 2 referred to in paragraph 3.3., the OBD system shall indicate the failure of an emission-related component or system when that failure results in an increase in emissions above the OBD thresholds given in the table in paragraph 5.4.4. of this Regulation.

4.1.3.	In the case of both OBD 1 and OBD 2, the OBD system shall indicate the lack of any required reagent necessary for the operation of an exhaust aftertreatment system.

4.2.	Installation requirements

4.2.1.	The installation on the vehicle of an engine equipped with an OBD system shall comply with the following provisions of this annex with respect to the vehicle equipment:

- (a)	the provisions of paragraphs 3.6.1., 3.6.2. and 3.6.5. concerning the MI and, where appropriate, additional warning modes;

- (b)	when applicable, the provisions of paragraph 6.8.3.1. concerning the use of an on-board diagnostic facility;

- (c)	the provisions of paragraph 6.8.6. concerning the connection interface.

4.3.	Approval of an OBD system containing deficiencies

4.3.1.	A manufacturer may request to the authority that an OBD system be accepted for approval even though the system contains one or more deficiencies such that the specific requirements of this annex are not fully met.

4.3.2.	In considering the request, the authority shall determine whether compliance with the requirements of this annex is feasible or unreasonable.

The authority shall take into consideration data from the manufacturer that details such factors as, but not limited to, technical feasibility, lead time and production cycles including phase-in or phase-out of engines designs and programmed upgrades of computers, the extend to which the resultant OBD system will be effective in complying with the requirements of this Regulation and that the manufacturer has demonstrated an acceptable level of effort toward the requirements of this Regulation.

4.3.3.	The authority will not accept any deficiency request that includes the complete lack of a required diagnostic monitor.

4.3.4.	The authority shall not accept any deficiency request that does not respect the OBD threshold limits given in the table in paragraph 5.4.4. of this Regulation.

4.3.5.	In determining the identified order of deficiencies, deficiencies relating to OBD Stage 1 in respect of paragraphs 3.2.2.1., 3.2.2.2., 3.2.2.3., 3.2.2.4. and 3.4.1.1. and OBD Stage 2 in respect of paragraphs 3.3.2.1., 3.3.2.2., 3.3.2.3., 3.3.2.4. and 3.4.1.1. of this annex shall be identified first.

4.3.6.	Prior to or at the time of approval, no deficiency shall be granted in respect of the requirements of paragraph 3.2.3. and paragraph 6., except sub-paragraph 6.8.5. of this annex.

4.3.7.	Deficiency period

4.3.7.1.	A deficiency may be carried-over for a period of two years after the date of approval of the engine type or vehicle in respect of its engine type, unless it can be adequately demonstrated that substantial engine modifications and additional lead-time beyond two years would be necessary to correct the deficiency. In such a case, the deficiency may be carried-out for a period not exceeding three years.

4.3.7.2.	A manufacturer may request that the original approval authority grant a deficiency retrospectively when such a deficiency is discovered after the original approval. In this case, the deficiency may be carried-over for a period of two years after the date of notification to the approval authority unless it can be adequately demonstrated that substantial engine modifications and additional lead-time beyond two years would be necessary to correct the deficiency. In such a case, the deficiency may be carried-out for a period not exceeding three years.

4.3.7.3.	The authority shall notify its decision in granting a deficiency request to all Contracting Parties.

5.	ACCESS TO OBD INFORMATION

5.1.	Replacement parts, diagnostic tools and test equipment

5.1.1.	Applications for approval or amendment of an approval shall be accompanied by the relevant information concerning the OBD system.  This relevant information shall enable manufacturers of replacement or retrofit components to make the parts they manufacture compatible with the OBD system with a view to fault-free operation assuring the vehicle user against malfunctions. Similarly, such relevant information shall enable the manufacturers of diagnostic tools and test equipment to make tools and equipment that provide for effective and accurate diagnosis of emission control systems.

5.1.2.	Upon request, the approval authorities shall make Appendix 1 to Annex 2A,  containing the relevant information on the OBD system as specified in Appendix 2 to Annex 6 to this Regulation, available to any interested components, diagnostic tools or test equipment manufacturer on a non-discriminatory basis.

5.1.2.1.	In the case of replacement or service components, information can only be requested for such components that are subject to approval, or for components that form part of a system that is subject to approval.

5.1.2.2.	The request for information shall identify the exact specification of the engine model type/engine model type within an engine family for which the information is required.  It shall confirm that the information is required for the development of replacement or retrofit parts or components or diagnostic tools or test equipment.

5.2	Repair information

5.2.1.	No later than three months after the manufacturer has provided any authorized dealer or repair shop within the Community with repair information, the manufacturer shall make that information (including all subsequent amendments and supplements) available upon reasonable and non-discriminatory payment.

5.2.2.	The manufacturer shall also make accessible, where appropriate upon payment the technical information required for the repair or maintenance of motor vehicles unless that information is covered by an intellectual property right or constitutes essential, secret know-how which is identified in an appropriate form; in such case, the necessary technical information shall not be withheld improperly.

Entitled to such information is any person engaged in commercially servicing or repairing, road-side rescuing, inspecting or testing of vehicles or in manufacturing or selling replacement or retro-fit components, diagnostic tools and test equipment.

5.2.3.	In the event of failure to comply with these provisions the approval authority shall take appropriate measures to ensure that repair information is available, in accordance with the procedures laid down for approval and in-service surveys.

6.	DIAGNOSTIC SIGNALS

6.1.	Upon determination of the first malfunction of any component or system, "freeze-frame" engine conditions present at the time shall be stored in computer memory.  Stored engine conditions shall include, but are not limited to calculated load value, engine speed, coolant temperature, intake manifold pressure (if available), and the fault code which caused the data to be stored.  For freeze-frame storage, the manufacturer shall choose the most appropriate set of conditions facilitating effective repairs.

6.2.	Only one frame of data is required.  Manufacturers may choose to store additional frames provided that at least the required frame can be read by a generic scan tool meeting the specifications of paragraphs 6.8.3. and 6.8.4.  If the fault code causing the conditions to be stored is erased in accordance with paragraph 3.9. of this annex, the stored engine conditions may also be erased.

6.3.	If available, the following signals in addition to the required freeze-frame information shall be made available on demand through the serial port on the standardized data link connector, if the information is available to the on-board computer or can be determined using information available to the on-board computer: diagnostic trouble codes, engine coolant temperature, injection timing, intake air temperature, manifold air pressure, air flow rate, engine speed, pedal position sensor output value, calculated load value, vehicle speed and fuel pressure.

The signals shall be provided in standard units based on the specifications given in paragraph 6.8.  Actual signals shall be clearly identified separately from default value or limp-home signals.

6.4.	For all emission control systems for which specific on-board evaluation tests are conducted, separate status codes, or readiness codes, shall be stored in computer memory to identify correctly functioning emission control systems and those emission control systems which require further vehicle operation to complete a proper diagnostic evaluation.  A readiness code need not be stored for those monitors that can be considered continuously operating monitors.  Readiness codes should never be set to "not ready" status upon "key-on" or "key-off".  The intentional setting of readiness codes to "not ready" status via service procedures shall apply to all such codes, rather than applying to individual codes.

6.5.	The OBD requirements to which the vehicle is certified (i.e. stage 1 OBD or stage 2 OBD) and the major emission control systems monitored by the OBD system consistent with paragraph 6.8.4. shall be available through the serial data port on the standardized data link connector according to the specifications given in paragraph 6.8.

6.6.	The software calibration identification number as declared in Annexes 1 and 2A to this Regulation shall be made available through the serial port of the standardized diagnostic connector.  The software calibration identification number shall be provided in a standardized format.

6.7.	The vehicle identification number (VIN) number shall be made available through the serial port of the standardized diagnostic connector.  The VIN number shall be provided in a standardized format.

6.8.	The emission control diagnostic system shall provide for standardized or unrestricted access and conform to either ISO 15765 or SAE J1939, as specified in the following paragraphs /.

6.8.1.	The use of either ISO 15765 or SAE J1939 shall be consistent throughout paragraphs 6.8.2. to 6.8.5.

6.8.2.	The on-board to off-board communications link shall conform to ISO 15765-4 or to the similar clauses within the SAE J1939 series of standards.

6.8.3.	Test equipment and diagnostic tools needed to communicate with OBD systems shall meet or exceed the functional specification given in ISO 15031-4 or SAE J1939-73 paragraph 5.2.2.1.

6.8.3.1.	The use of an on-board diagnostic facility such as a dashboard mounted video display device for enabling access to OBD information is permitted but this is in addition to enabling access to OBD information by means of the standard diagnostic connector.

6.8.4.	Diagnostic data, (as specified in this paragraph) and bi-directional control information shall be provided using the format and units described in ISO 15031-5 or SAE J1939-73 paragraph 5.2.2.1. and shall be available using a diagnostic tool meeting the requirements of ISO 15031-4 or SAE J1939-73 paragraph 5.2.2.1.

The manufacturer shall provide a national standardization body with emission-related diagnostic data, e.g. PID's, OBD monitor Id's, Test Id's not specified in ISO 15031-5 but related to this Regulation.

6.8.5.	When a fault is registered, the manufacturer shall identify the fault using the most appropriate fault code consistent with those given in paragraph 6.3. of ISO 15031-6 relating to emission-related system diagnostic trouble codes.  If such identification is not possible, the manufacturer may use diagnostic trouble codes according to paragraphs 5.3. and 5.6. of ISO 15031-6.  The fault codes shall be fully accessible by standardized diagnostic equipment complying with the provisions of paragraph 6.8.3. of this annex.

The manufacturer shall provide a national standardization body with emission-related diagnostic data, e.g. PID's, OBD monitor Id's, Test Id's not specified in ISO 15031-5 but related to this Regulation.

As an alternative, the manufacturer may identify the fault using the most appropriate fault code consistent with those given in SAE J2012 or in SAE J1939-73.

6.8.6.	The connection interface between the vehicle and the diagnostic tester shall be standardized and shall meet all the requirements of ISO 15031-3 or SAE J1939-13.

In the case of category N2, N3, M2, and M3 vehicles, as an alternative to the connector location described in the above standards and provided all other requirements of ISO 15031-3 are met, the connector may be located in a suitable position by the side of the driver's seat, including on the floor of the cabin. In this case the connector should be accessible by a person standing outside the vehicle and not restrict access to the driver's seat.

The installation position shall be subject to agreement of the approval authority such that it is readily accessible by service personnel but protected from accidental damage during normal conditions of use.

#### Appendix 1

ON-BOARD DIAGNOSTIC (OBD) SYSTEM APPROVAL TESTS

1.	INTRODUCTION

This appendix describes the procedure for checking the function of the on-board diagnostic (OBD) system installed on the engine by failure simulation of relevant emission-related systems in the engine management or emission control system.  It also sets procedures for determining the durability of OBD systems.

1.1.	Deteriorated components/systems

In order to demonstrate the efficient monitoring of an emission control system or component, the failure of which may result in tailpipe emissions exceeding the appropriate OBD threshold limits, the manufacturer shall make available the deteriorated components and/or electrical devices which would be used to simulate failures.

Such deteriorated components or devices shall not cause emissions to exceed the OBD threshold limits referred to in the table in paragraph 5.4.4. of this Regulation by more than 20 per cent.

In the case of approval of an OBD system according to paragraph 5.4.1. of this Regulation, the emissions shall be measured over the ESC test cycle (see Appendix 1 to Annex 4A to this Regulation).  In the case of approval of an OBD system according to paragraph 5.4.2. of this Regulation, the emissions shall be measured over the ETC test cycle (see Appendix 2 to Annex 4A to this Regulation).

1.1.1.	If it is determined that the installation of a deteriorated component or device on an engine means that a comparison with the OBD threshold limits is not possible (e.g. because the statistical conditions for validating the ETC test cycle are not met), the failure of that component or device may be considered as qualified upon the agreement of the approval authority based on technical argumentation provided by the manufacturer.

1.1.2.	In the case that the installation of a deteriorated component or device on an engine means that the full load curve (as determined with a correctly operating engine) cannot (even partially) be attained during the test, the deteriorated component or device is considered as qualified upon the agreement of the approval authority based on technical argumentation provided by the manufacturer.

1.1.3.	The use of deteriorated components or devices that cause engine emissions to exceed the OBD threshold limits referred to in the table in paragraph 5.4.4. of this Regulation by no more than 20 per cent may not be required in some very specific cases (for example, if a limp home strategy is activated, if the engine cannot run any test, or in case of EGR sticking valves, etc).  This exception shall be documented by the manufacturer.  It is subject to the agreement of the technical service.

1.2.	Test principle

When the engine is tested with the deteriorated component or device fitted, the OBD system is approved if the MI is activated.  The OBD system is also approved if the MI is activated below the OBD threshold limits.

The use of deteriorated components or devices that cause the engine emissions to exceed the OBD threshold limits referred to in the table in paragraph 5.4.4. of this Regulation by no more than 20 per cent are not required in the specific case of the failure modes described in paragraphs 6.3.1.6. and 6.3.1.7. of this appendix and also with respect to monitoring for major functional failure.

1.2.1.	The use of deteriorated components or devices that cause engine emissions to exceed the OBD threshold limits referred to in the table in paragraph 5.4.4. of this Regulation by no more than 20 per cent may not be required in some very specific cases (for example, if a limp home strategy is activated, if the engine cannot run any test, or in case of EGR sticking valves, etc).  This exception shall be documented by the manufacturer. It is subject to the agreement of the technical service.

2.	DESCRIPTION OF TEST

2.1.	The testing of OBD systems consists of the following phases:

- (a)	simulating the malfunction of a component of the engine management or emission control system as described in paragraph 1.1. of this appendix;

- (b)	preconditioning of the OBD system with a simulated malfunction over the preconditioning cycle specified in paragraph 6.2.;

- (c)	operating the engine with a simulated malfunction over the OBD test cycle referred to in paragraph 6.1.;

- (d)	determining whether the OBD system reacts to the simulated malfunction and indicates malfunction in an appropriate manner.

2.1.1.	Should the performance (e.g. power curve) of the engine be affected by the malfunction, the OBD test-cycle remains the shortened version of the ESC test-cycle used for the assessing the exhaust emissions of the engine without that malfunction.

2.2.	Alternatively, at the request of the manufacturer, malfunction of one or more components may be electronically simulated according to the requirements of paragraph 6.

2.3.	Manufacturers may request that monitoring take place outside the OBD test cycle referred to in paragraph 6.1. if it can be demonstrated to the authority that monitoring during conditions encountered during this OBD test cycle would impose restrictive monitoring conditions when the vehicle is used in service.

3.	TEST ENGINE AND FUEL

3.1.	Engine

The test engine shall comply with the specifications laid down in Annex 1 to this Regulation.

3.2.	Fuel

The appropriate reference fuel as described in Annex 5 to this Regulation shall be used for testing.

4.	TEST CONDITIONS

The test conditions shall satisfy the requirements of the emission test described in the present Regulation.

5.	TEST EQUIPMENT

The engine dynamometer shall meet the requirements of Annex 4A to this Regulation.

6.	OBD TEST CYCLE

6.1.	The OBD test cycle is a single shortened ESC test cycle.  The individual modes shall be performed in the same order as the ESC test cycle, as defined in paragraph 2.7.1. of Appendix 1 to Annex 4A to this Regulation .

The engine shall be operated for a maximum of 60 seconds in each mode, completing engine speed and load changes in the first 20 seconds.  The specified speed shall be held to within  50 rpm and the specified torque shall be held to within  2 per cent of the maximum torque at each speed.

Exhaust emissions are not required to be measured during the OBD test cycle.

6.2.	Preconditioning cycle

6.2.1.	After introduction of one of the failure modes given in paragraph 6.3., the engine and its OBD system shall be preconditioned by performing a preconditioning cycle.

6.2.2.	At the request of the manufacturer and with the agreement of the approval authority, an alternative number of a maximum of nine consecutive OBD test cycles may be used.

6.3.	OBD system test

6.3.1.	Diesel engines and vehicles equipped with a diesel engine

6.3.1.1.	After preconditioning according to paragraph 6.2., the test engine is operated over the OBD test cycle described in paragraph 6.1. of this appendix.  The MI shall activate before the end of this test under any of the conditions given in 6.3.1.2. to 6.3.1.7.  The technical service may substitute those conditions by others in accordance with paragraph 6.3.1.7.  For the purposes of approval, the total number of failures subject to testing, in the case of different systems or components, shall not exceed four.

If the test is being carried out to type-approve an OBD-engine family consisting of engines that do not belong to the same engine family, the type approval authority will increase the number of failures subject to testing up to a maximum of four times the number of engine families present in the OBD-engine family.  The approval authority may decide to curtail the test at any time before this maximum number of failure tests has been reached.

6.3.1.2.	Where fitted in a separate housing that may or may not be part of a deNOx system or diesel particulate filter, replacement of any catalyst with a deteriorated or defective catalyst or electronic simulation of such a failure.

6.3.1.3.	Where fitted, replacement of a deNOx system (including any sensors that are an integral part of the system) with a deteriorated or defective deNOx system or electronic simulation of a deteriorated or defective deNOx system that results in emissions exceeding the OBD NOx threshold limit referred to in the table given in paragraph 5.4.4. of this Regulation.

In the case that the engine is being approved according to paragraph 5.4.1. of this Regulation in relation to monitoring for major functional failure, the test of the deNOx system shall determine that the MI illuminates under any of the following conditions:

- (a)	complete removal of the system or replacement of the system by a bogus system;

- (b)	lack of any required reagent for a deNOx system;

- (c)	any electrical failure of a component (e.g. sensors and actuators, dosing control unit) of a deNOx system, including, when applicable, the reagent heating system;

- (d)	failure of a reagent dosing system (e.g. missing air supply, clogged nozzle, dosing pump failure) of a deNOx system;

- (e)	major breakdown of the system.

6.3.1.4.	Where fitted, total removal of the particulate filter or replacement of the particulate filter with a defective particulate filter that results in emissions exceeding the OBD particulate threshold limit given in the table in paragraph 5.4.4. of this Regulation.

In the case that the engine is being approved according to paragraph 5.4.1. of this Regulation in relation to monitoring for major functional failure, the test of the particulate filter shall determine that the MI illuminates under any of the following conditions:

- (a)	complete removal of the particulate filter or replacement of the system by a bogus system;

- (b)	major melting of the particulate filter substrate;

- (c)	major cracking of the particulate filter substrate;

- (d)	any electrical failure of a component (e.g. sensors and actuators, dosing control unit) of a particulate filter;

- (e)	failure, when applicable, of the reagent dosing system (e.g. clogged nozzle, dosing pump failure) of a particulate filter;

- (f)	a clogged particulate filter resulting in a differential pressure out of the range declared by the manufacturer.

6.3.1.5.	Where fitted, replacement of a combined deNOx-particulate filter system (including any sensors that are an integral part of the device) with a deteriorated or defective system or electronic simulation of a deteriorated or defective system that results in emissions exceeding the OBD NOx and particulate threshold limits given in the table in paragraph 5.4.4. of this Regulation.

In the case that the engine is being approved according to paragraph 5.4.1. of this Regulation in relation to monitoring for major functional failure, the test of the combined deNOx-particulate filter system shall determine that the MI illuminates under any of the following conditions:

- (a)	complete removal of the system or replacement of the system by a bogus system;

- (b)	lack of any required reagent for a combined deNOx-particulate filter system;

- (c)	any electrical failure of a component (e.g. sensors and actuators, dosing control unit) of a combined deNOx-particulate filter system, including, when applicable, the reagent heating system;

- (d)	failure of a reagent dosing system (e.g. missing air supply, clogged nozzle, dosing pump failure) of a combined deNOx-particulate filter system;

- (e)	major breakdown of a NOx trap system;

- (f)	major melting of the particulate filter substrate;

- (g)	major cracking of the particulate filter substrate;

- (h)	a clogged particulate filter resulting in a differential pressure out of the range declared by the manufacturer.

6.3.1.6.	Disconnection of any fuelling system electronic fuel quantity and timing actuator that results in emissions exceeding any of the OBD thresholds referred to in the table given in paragraph 5.4.4. of this Regulation.

6.3.1.7.	Disconnection of any other emission-related engine component connected to a computer that results in emissions exceeding any of the thresholds referred to in the table given in paragraph 5.4.4. of this Regulation.

6.3.1.8.	In demonstrating compliance with the requirements of paragraphs 6.3.1.6. and 6.3.1.7. and with the agreement of the approval authority, the manufacturer may take appropriate steps to demonstrate that the OBD system will indicate a fault when disconnection occurs.

- - - - -

/	Delete where inapplicable

/	In the case of systems laid-out in a different manner, supply equivalent information (for paragraph 3.2.).

/	Specify the tolerance; to be within  3 per cent of the values declared by the manufacturer.

/	If not applicable, mark n.a.

/	To be submitted for each engine of the family.

/	Strike out what does not apply.

/	Specify the tolerance.

/	Delete where inapplicable.

/	In the case of systems laid-out in a different manner, supply equivalent information (for paragraph 3.2.).

/	As defined in Annex 7 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), (document TRANS/WP.29/78/Rev.1/Amend.2 as last amended by Amend.4).

/	Paragraph 5.4.1. of this Regulation provides for the monitoring for major functional failure instead of monitoring for the degradation or the loss of catalytic/filtering efficiency of an exhaust aftertreatment system.  Examples of major functional failure are given in paragraphs 3.2.3.2. and 3.2.3.3. of Annex 9A to this Regulation.

/	1 for Germany, 2 for France, 3 for Italy, 4 for the Netherlands, 5 for Sweden, 6 for Belgium, 7 for Hungary, 8 for the Czech Republic, 9 for Spain, 10 for Serbia [and Montenegro], 11 for the United Kingdom, 12 for Austria, 13 for Luxembourg, 14 for Switzerland, 15 (vacant), 16 for Norway, 17 for Finland, 18 for Denmark, 19 for Romania, 20 for Poland, 21 for Portugal, 22 for the Russian Federation, 23 for Greece, 24 for Ireland, 25 for Croatia, 26 for Slovenia, 27 for Slovakia, 28 for Belarus, 29 for Estonia, 30 (vacant), 31 for Bosnia and Herzegovina, 32 for Latvia, 33 (vacant), 34 for Bulgaria, 35 (vacant), 36 for Lithuania, 37 for Turkey, 38 (vacant), 39 for Azerbaijan, 40 for The former Yugoslav Republic of Macedonia, 41 (vacant), 42 for the European Community (Approvals are granted by its Member States using their respective ECE symbol), 43 for Japan, 44 (vacant), 45 for Australia, 46 for Ukraine, 47 for South Africa, 48 for New Zealand, 49 for Cyprus, 50 for Malta, 51 for the Republic of Korea, 52 for Malaysia and 53 for Thailand.  Subsequent numbers shall be assigned to other countries in the chronological order in which they ratify or accede to the Agreement Concerning the Adoption of Uniform Technical Prescriptions for Wheeled Vehicles, Equipment and Parts which can be Fitted and/or be Used on Wheeled Vehicles and the Conditions for Reciprocal Recognition of Approvals Granted on the Basis of these Prescriptions, and the numbers thus assigned shall be communicated by the Secretary-General of the United Nations to the Contracting Parties to the Agreement.

/	Up to 1 October 2008, the following applies: "an ambient temperature within the range 279 K to 303 K (6 °C to 30 °C)".

/	This temperature range will be reconsidered as part of the review of this Regulation with special emphasis on the appropriateness of the lower temperature boundary.

/	As defined in the Consolidated Resolution on the Construction of Vehicles (R.E.3).

/	In the case of non-conventional engines and systems, particulars equivalent to those referred to here shall be supplied by the manufacturer.

/	Strike out what does not apply

/	Specify the tolerance

/	The test points shall be selected using approved statistical methods of randomisation.

/	The test points shall be selected using approved statistical methods of randomisation.

/	The test points shall be selected using approved statistical methods of randomisation.

/	The value is only valid for the reference fuel specified in Annex IV.

/	Stoichiometric Air/Fuel ratios of automotive fuels - SAE J1829, June 1987. John B. Heywood, Internal combustion engine fundamentals, McGraw-Hill, 1988, Chapter 3.4 "Combustion stoichiometry" (pp. 68 to 72)

/	Symbol numbers F01 or F22.

/	Symbol number F24.

/	The use of the ISO single protocol standard (ISO/PAS 27145) developed for a world-wide global technical regulation on heavy-duty OBD will be considered to satisfy the appropriate requirements of paragraph 6.