| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRPE/2007/9 26 March 2007 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Fifty-fourth session

Geneva, 5-8 June 2007

Item 4. of the provisional agenda

GLOBAL TECHNICAL REGULATION No. 2 ON THE

WORLD-WIDE HARMONIZED MOTORCYCLE EMISSION TEST CYCLE

#### Proposal for draft amendments to gtr No. 2

Submitted by the expert from Germany

The text reproduced below was prepared by the expert from Germany on behalf of the GRPE informal group on world-wide harmonized motorcycle emission test cycle (WMTC) to align gtr No. 2 according to the concerns by India and China.  It is mainly based on a document without a symbol (informal document No. GRPE-53-11), distributed during the fifty-third session of GRPE (see report ECE/TRANS/WP.29/GRPE/53, para. 12).

The modifications to the existing text of the gtr are marked in bold characters or marked as strikethrough.

A.	PROPOSAL

Section A

Paragraph 2., add three new subparagraphs to read:

"… in Part B of this document.

At its April 2006 meeting held in Pune (India), WMTC/FEG agreed to prepare new test cycle proposals and a new vehicle classification for draft amendments to the gtr in order to suit low-powered vehicles, such as commonly used in India and China.

A small WMTC Task Force, coordinated by IMMA, was set up to prepare a proposal on the test cycle(s) and any new classification that might be necessary to achieve this objective.  The Task Force was attended by Germany (TUV), India, Italy, Japan (JASIC) and IMMA.  Task Force meetings were held in August and October 2006.

At its November 2006 meeting held in Ann Arbor (United States of America), WMTC/FEG agreed to a modified version of one of the WMTC Task Force proposals and forwarded it to WMTC Informal Group in January 2007 where it was approved for submission to GRPE."

Paragraph 3., amend to read:

"…

EU:

Directive 2002/51/EC amending directive 97/24/EC:  The reduction of the level of pollutant emissions from two- and three-wheeled motor vehicles

Indian Regulation:

MoSRT&H/ CMVR/ TAP-115/116 and Central Motor Vehicle Rule No. 115

Japanese Regulation Trias:

Road vehicle Act, Article 41 "Systems and Devices of Motor Vehicles"

Safety Regulations for Road Vehicles, Article 31 "Emission Control Devices"

United States of America regulation:

US-FTP Subpart F, Emission Regulations for 1978 and Later New Motorcycles

ISO standards:

ISO 11486 (Motorcycles - Chassis dynamometer setting method)

ISO 6460 (gas sampling)

ISO 7860 (fuel consumption)

ISO 4106 (Motorcycles -- Engine test code -- Net power)

Most of these …"

Paragraph.4., add a subparagraph to read:

"…

Finally, a first draft of the World-wide Motorcycle Test Cycle (WMTC) was produced.  It was foreseen that this first draft needed to be modified on the basis of an evaluation concerning driveability and practical points concerning the measurement procedure.  Since this process is iterative by nature, several adaptation rounds including the driveability tests were carried out.

In order to take into account the performances and the use of low-powered motorcycles, additional data were supplied by India.  After further analysis, it was agreed that the previously agreed reduced speed test cycles should be modified and that the class 1 vehicle subcategories could be merged.

In each of these steps, specific technical issues were …"

Paragraph 4.(d), amend to read:

"… as required by the 1998 Agreement.

The Informal group will continue its work on the comparative database of results from the different test procedures, which will act as a major input on the discussion of limit values that are compatible to the existing limit values in different regions/countries."

### Section B

Insert a new paragraph 3.9., to read:

"3.9.	"Maximum net engine power" is the maximum net engine power of the vehicle as declared by the manufacturer, measured in accordance with ISO 4106:2004 (Motorcycles -- Engine test code -- Net power)."

Paragraph 6.3.1., amend to read:

"6.3.1.	Class 1

Vehicles that fulfil the following specifications belong to class 1:

| 50 cm³ < engine capacity < 150 cm³ and vmax < 50 km/h or engine capacity < 150 cm³ and 50 km/h ≤ vmax < 100 km/h | class 1 |
| --- | --- |

"

Paragraph 6.3.3., figure 6-1, replace by the following:

Figu

re 6

-

1

: vehicle classification

0

10

20

30

40

50

60

70

80

90

100

110

120

130

140

150

0

150

300

450

600

750

900

1050

1200

1350

1500

engine capacity in cm

3

1

3

-

1

2

-

1

2

-

2

3

-

2

1

max. vehicle speed in km/h

"

"

Paragraph 6.5.4.1., amend to read:

"6.5.4.1.	Test cycles

Test cycles (vehicle speed patterns), for the Type I test consists of up to three parts that are shown in Annex 5.  Depending on the vehicle class (see paragraph 6.3.) the following test cycle parts have to be run:

| Class 1: | part 1, reduced speed in cold condition, followed by part 1 reduced speed in hot condition. |
| --- | --- |
| Class 2: |  |
| Subclass 2-1: | part 1, reduced speed in cold condition, followed by part 2, reduced speed in hot condition. |
| Subclass 2-2: | part 1 in cold condition, followed by part 2 in hot condition. |
| Class 3: |  |
| Subclass 3-1: | part 1 in cold condition, followed by part 2 in hot condition, followed by part 3 reduced speed in hot condition. |
| Subclass 3-2: | part 1 in cold condition, followed by part 2 in hot condition, followed by part 3 in hot condition. |

"

Paragraph 6.5.5.2.1.2., amend to read:

"6.5.5.2.1.2.	Downshift speeds (vi→i-1) in km/h during cruise or deceleration phases in gears 3 (3rd gear) to ng shall be calculated using the following formula:

Equation 6-3:

v i → i−1= (0.5753×e (−1.9×Pnmk+75) )×(s−n idle)+n idle1ndvi-2, i = 4 to ng

Equation 6-4:

v3 → 2= (0.5753×e (−1.9×Pnmk+75) −0.1)×(s−n idle)+n idle1ndv1

"

Equations 6-4 to 6-7 (former), renumber as equations 6-5 to 6-8.

Paragraphs 6.5.5.2.2.1. and 6.5.5.2.2.2., amend to read:

"6.5.5.2.2.1.	Gear lever in neutral and clutch disengaged;

The gear lever shall be set to first gear and the clutch shall be disengaged for the last 5 s of stop phases.

6.5.5.2.2.2.	The clutch shall be disengaged, if the following conditions are met:

(a)	During cruise or deceleration phases, if:

(i)	the vehicle speed drops below 10 km/h or

(ii)	the vehicle speed in 2nd gear where the engine speed drops below

nidle + 0.03  (s – nidle); if this speed is higher than 10 km/h.  This speed is called v2->cl.

(b)	Gear choice for acceleration phases:

Gear = 6, if v > v5→6,

Gear = 5, if v > v4→5,

Gear = 4, if v > v3→4,

Gear = 3, if v > v2→3,

Gear = 2, if v > v1→2,

Gear = 1, if v ≤ v1→2.

(c)	Gear choice for deceleration or cruise phases:

Gear = 6, if v > v6→5, and the engine speed n >= 0.15  (s – nidle)+ nidle,

Gear = 5, if v > v5→4, and the engine speed n >= 0.12  (s – nidle)+ nidle,

Gear = 4, if v > v4→3, and the engine speed n >= 0.09  (s – nidle)+ nidle,

Gear = 3, if v > v3→2, and the engine speed n >= 0.06  (s – nidle)+ nidle,

Gear = 2, if v ≤ v3→2, and the engine speed n >= 0.03  (s – nidle)+ nidle,."

Paragraph 6.5.5.2.3.1., amend to read:

"6.5.5.2.3.1.	The gear choice has then to be modified according to the following requirements:

a)	No gearshift at a transition from an acceleration phase to a deceleration phase: keep the gear that was used for the last second of the acceleration phase also for the following deceleration phase unless the speed drops below a downshift speed.

b)	No upshifts during deceleration phases and transitions from cruise phases to deceleration phases.

c)	No gearshift in cycle phases, where "no gearshift" is indicated.

d)	If a gear is used for only one second, this gear shall also be assigned to the following second during acceleration phases and to the previous second during deceleration phases.  Since it could happen that the modifications according to this criterion create new phases where a gear is used for only one second, this modification step has to be applied several times.

e)	Upshifts from gear i to gear i+1 lasting for up to three seconds followed by gear i again shall be replaced by gear i.

f)	Downshifts from gear i to gear i-1 lasting for up to three seconds followed by gear i again shall be replaced by gear i, if the trace can be followed in gear i and the engine speed does not drop below:

0.03 x (s – nidle) in 2. gear,

0.06 x (s – nidle) in 3. gear,

0.09 x (s – nidle) in 4. gear,

0.12 x (s – nidle) in 5. gear,

0.15 x (s – nidle) in 6. gear.

g)	No downshifts or upshifts by more than 1 gear.

h)	The difference between the vehicle speeds

v3→2 and v2->cl,

v4→3 and v3→2,

v5→4 and v4→3,

v6→5 and v5→4,

shall be at least 5 km/h.  Corrections have to be related to v2->cl."

Paragraph 6.5.6.1., amend to read:

"6.5.6.1.	Chassis dynamometer setting derived from on-road coast down measurements

To use this alternative on road coast down measurements have to be carried out as specified in Annex 7."

Paragraph 8.1.1.4.4., amend to read:

"	……

DF	is the coefficient defined in paragraph 8.1.1.4.6. below.

……"

Annex 5, figure A5-1, replace by the following:

"

"

Annex 5, figure A5-2, replace by the following:

"

"

Annex 5, Tables A5-1 to A5-15, replace by the following (see following pages):

"Table A5-1: Cycle part 1, 1 to 120 s

Table A5-2: Cycle part 1, 121 to 240 s

Table A5-3: Cycle part 1, 241 to 360 s

Table A5-4: Cycle part 1, 361 to 480 s

Table A5-5: Cycle part 1, 481 to 600 s

Table A5-6: Cycle part 1, reduced speed, 1 to 120 s

Table A5-7: Cycle part 1, reduced speed, 121 to 240 s

Table A5-8: Cycle part 1, reduced speed, 241 to 360 s

Table A5-9: Cycle part 1, reduced speed, 361 to 480 s

Table A5-10: Cycle part 1, reduced speed, 481 to 600 s

Table A5-11: Cycle part 2, 1 to 120 s

Table A5-12: Cycle part 2, 121 to 240 s

Table A5-13: Cycle part 2, 241 to 360 s

Table A5-14: Cycle part 2, 361 to 480 s

Table A5-15: Cycle part 2, 481 to 600 s

Table A5-16: Cycle part 2, reduced speed, 1 to 120 s

Table A5-17: Cycle part 2, reduced speed, 121 to 240 s

Table A5-18: Cycle part 2, reduced speed, 241 to 360 s

Table A5-19: Cycle part 2, reduced speed, 361 to 480 s

Table A5-20: Cycle part 2, reduced speed, 481 to 600 s

Table A5-21: Cycle part 3, 1 to 120 s

Table A5-22: Cycle part 3, 121 to 240 s

Table A5-23: Cycle part 3, 241 to 360 s

Table A5-24: Cycle part 3, 361 to 480 s

Table A5-25: Cycle part 3, 481 to 600 s

Table A5-26: Cycle part 3, reduced speed, 1 to 120 s

Table A5-27: Cycle part 3, reduced speed, 121 to 240 s

Table A5-28: Cycle part 3, reduced speed, 241 to 360 s

Table A5-29: Cycle part 3, reduced speed, 361 to 480 s

Table A5-30: Cycle part 3, reduced speed, 481 to 600 s

"

Annex 13, amend to read:

"Annex 13

EXPLANATORY NOTE ON GEARSHIFT PROCEDURE

This explanatory note is not a part of the standard, but explains matters specified or described in the standard or appendix, and matters related thereto.

1.	Approach

1.1.	The development of the gearshift procedure was based on an analysis of the gearshift points in the in-use data.  In order to get generalised relations between technical specifications of the vehicles and gearshift speeds the engine speeds were normalised to the utilisable band between rated speed and idling speed.

1.2.	In a second step the end speeds (vehicle speed as well as normalised engine speed) for upshifts and downshifts were determined and collected in a separate table.  The averages of these speeds for each gear and vehicle were calculated and correlated with technical specifications of the vehicles.

1.3.	The results of these analyses and calculations can be summarised as follows:

(a)	The gearshift behaviour is engine speed related rather than vehicle speed related.

(b)	The best correlation between gearshift speeds and technical data was found for normalised engine speeds and the power to mass ratio (rated power/(unladen mass + 75 kg)).

(c)	The residual variations cannot be explained by other technical data or by different transmission ratios.  They are most probably assigned to differences in traffic conditions and individual driver behaviour.

(d)	The best approximation between gearshift speeds and power to mass ratio was found for exponential functions.

(e)	The gearshift function for the first gear is significantly lower than for all other gears.

(f)	The gearshift speeds for all other gears can be approximated by one common function.

(g)	No differences were found between five-speed and six-speed gearboxes.

(h)	The gearshift behaviour in Japan is significantly different from the equal-type gearshift behaviour in the European Union (EU) and in the United States of America (USA).

1.4.	In order to find a balanced compromise between the three regions a new approximation function for normalised upshift speeds versus power to mass ratio was calculated as weighted average of the EU/USA curve (with 2/3 weighting) and the Japanese curve (with 1/3 weighting).

1.5.	In use driving behaviour data from India was added to the WMTC database at a later stage.  This resulted in modifications of the part 1 cycles and the part 2, reduced speed cycle.  Within this modification work also the gearshift behaviour was checked.  Fortunately it could be proven that the WMTC gearshift prescriptions are also suitable for the Indian gearshift behaviour.

2.	Example

Figure A13-1 shows an example of gearshift use for a small vehicle.  The lines in bold show the gear use for acceleration phases; the dotted lines show the downshift points for deceleration phases.  During cruising phases the whole speed range between downshift speed and upshift speed may be used.

In order to allow to the test service more flexibility and to assure driveability the gearshift regression functions should be considered as lower limits.  Higher engine speeds are permitted in any cycle phase.

Figure A13-1: Example of a gearshift sketch for a small vehicle

3.	Calculation example

3.1.	An example of input data necessary for the calculation of shift speeds is shown in table A13-1.  The upshift speeds for acceleration phases for the first gear and higher gears are calculated using equations 6-1 and 6-2.  The denormalisation of engine speeds can be performed by using the equation n = n_norm x (s – nidle) + nidle.

3.2.	The downshift speeds for deceleration phases can be calculated with equations 6-3 and 6-4.  The ndv values in table A13-1 can be used as gear ratios.  These values can also be used to calculate the affiliated vehicle speeds (vehicle shift speed in gear i = engine shift speed in gear i / ndvi).  The corresponding results are shown in tables A13-2 and A13-3.

3.3.	In a further step the possibility of a simplification of the above-described gearshift algorithms was examined by additional analyses and calculations.  It should especially be checked whether engine shift speeds could be replaced by vehicle shift speeds.  The analysis showed that vehicle speeds could not be brought in line with the gearshift behaviour of the in-use data.

Table A13-1: Input data for the calculation of engine and vehicle shift speeds

| Item | Input Data |
| --- | --- |
| Engine capacity in cm3 | 600 |
| Pn in kW | 72 |
| mk in kg | 199 |
| s in min-1 | 11,800 |
| nidle in min-1 | 1,150 |
| ndv1 */ | 133.66 |
| ndv2 | 94.91 |
| ndv3 | 76.16 |
| ndv4 | 65.69 |
| ndv5 | 58.85 |
| ndv6 | 54.04 |
| pmr **/ in kW/t | 262.8 |
| */ ndv means the ratio between engine speed in min-1 and vehicle speed in km/h **/ pmr means the power to mass ratio calculated by Pn / (mk+75)  1,000; Pn in kW, mk in kg |  |

Table A13-2: Shift speeds for acceleration phases for the first gear and for higher gears (according to table A13-1)

|  | EU/USA/Japan driving behaviour |  |
| --- | --- | --- |
|  | n_acc_max (1) | n_acc_max (i) |
| n_norm */ in per cent | 24.8 per cent | 34.8 per cent |
| n in min-1 | 3,804 | 4,869 |
| */ n_norm means the calculated value by equation A13-1 and equation A13-2. |  |  |

Table A13-3: Engine and vehicle shift speeds according to table A13-2

| Gearshift |  | EU/USA/Japan driving behaviour |  |  |
| --- | --- | --- | --- | --- |
|  |  | v in km/h | n_norm (i) in per cent | n in min-1 |
| Upshift | 12 | 28.5 | 2.49 | 3,804 |
|  | 23 | 51.3 | 34.9 | 4,869 |
|  | 34 | 63.9 | 34.9 | 4,869 |
|  | 45 | 74.1 | 34.9 | 4,869 |
|  | 56 | 82.7 | 34.9 | 4,869 |
| Downshift | 2cl */ | 15.5 | 3.0 | 1,470 |
|  | 32 | 28.5 | 9.6 | 2,167 |
|  | 43 | 51.3 | 20.8 | 3,370 |
|  | 54 | 63.9 | 24.5 | 3,762 |
|  | 65 | 74.1 | 26.8 | 4,005 |
| */ "cl" means "Clutch-Off" timing. " |  |  |  |  |

### "

### B.		JUSTIFICATION

### Section A

Paragraph 2.

Additional information is needed to complete the procedural sequence.

Paragraph 3.

A reference to the relevant Indian Regulation has to be included while TRIAS is not a proper reference. Thus, it has to be deleted; addition of ISO 4106.

Paragraph 4.

An additional reference to the result of the inclusion of the Indian in-use data is necessary.

Paragraph 4(d).

The future work programme for the WMTC Informal Group has to be reformulated in view of completion of the modification of the cycle modules as a result of the inclusion of the Indian in-use data.

### Section B

Paragraph 3.9.

Additional definition of net engine power on the basis of applicable ISO standard has to be included to enable gtr application.

Paragraph 6.3.1.

This is the new classification, resulting from the incorporation and assessment of the Indian in-use data.

Paragraph 6.3.3.

The amended figure graphically illustrates the new classification.

Paragraph 6.5.4.1.

The amended table links the (sub)classes with the applicable test cycle parts to be run.

Paragraph 6.5.5.2.1.2.

An additional equation (6-4) is needed to ensure that downshift and upshift speeds are aligned.

Paragraphs 6.5.5.2.2.1. to 6.5.5.2.3.1.

Additional rules are required to prevent unrealistic gearshift sequences.  They improve driveability significantly.  For very high-powered vehicles a downshift to the first gear is required due to the fact that the engine speed in second gear would be lower than idling speed.  With the updated requirements the whole bullet point can be deleted.

Paragraph 6.5.6.1.

Correction of a typing error.

Annex 5, figure A5-1.

Amendment of test cycle part traces in line with cycle module modifications.

Annex 5, figure A5-2.

Amendment of test cycle part traces in line with cycle module modifications.

Annex 5, Tables A5-1 to A5-15.

Amendment of test cycle second by second data tables in line with module modifications.

Annex 13

Needs to be brought in line with the modifications of paragraph 6.5.5.

- - - - -