| UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRPE/2007/10 26 March 2007 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Pollution and Energy

Fifty-fourth session

Geneva, 5-8 June 2007

Item 6(a) of the provisional agenda

AMENDMENTS TO UNECE REGULATIONS

REGULATION No. 83

#### Proposal for draft Corrigendum to Revision 3 of Regulation No. 83

(Emissions of M1 and N1 categories of vehicles)

#### Submitted by the expert from the United Kingdom

The text reproduced below was prepared by the expert from the United Kingdom to correct typographical errors, amongst others, in the determination of power absorbed in the coast-down in Appendix 3 of Annex 4.

The modifications to the current text of the Regulation are marked in bold characters.

A.	PROPOSAL

Annex 4 - Appendix 3

Paragraph 5.1.1.2.5., correct to read:

"5.1.1.2.5.	Take the average T of the two times t1 and t2"

Paragraph 5.1.1.2.7., correct to read:

"5.1.1.2.7.	Calculate the power by the formula

P=M.V.ΔV500 T

where:

P is expressed in kW

V =	speed of the test in m/s

ΔV =	speed deviation from speed V, in m/s as specified in paragraph 5.1.1.2.3. of this appendix

M =	reference mass in kg

T =	time in seconds (s) "

Annex 7, paragraph 5.2.1., correct "± 5 KPa" to read "± 0.5 kPa".

B.	JUSTIFICATION

Annex 4 - Appendix 3, paragraph 5.1.1.2.5.

This is the correction of a typographical error.

Annex 4 - Appendix 3, paragraph 5.1.1.2.7

The formula as currently published miscalculates the power requirement, and this amendment corrects that miscalculation.

From first principles, kinetic energy is given by:

KE=M.V22

The change in kinetic energy for a variation in speed from ΔV above speed V to ΔV below speed V is given by:

ΔKE=M.[(V + ΔV)2−(V−ΔV)2]2

ΔKE=M.4.V.ΔV2

ΔKE=2.M.V.ΔV

Where:

KE = kinetic energy expressed in joules

M = mass of the vehicle in kg

V = speed of the test in m/s

ΔV = deviation from speed V

T = time in seconds (s)

Power is energy delivered or absorbed per unit time, therefore divide KE in Joules by T to convert to power absorbed in Watts, and by 1000 to convert to kW:

P=2.M.V.ΔV1000.T

thus:

P=M.V.ΔV500.T

Annex 7, paragraph 5.2.1.

This is a correction of a typographical error.

- - - - -