| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRSP/2007/7 21 February 2007 Original: ENGLISH ENGLISH AND FRENCH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Passive Safety

Forty-first session

Geneva, 7-11 May 2007

Item 8.2. of the provisional agenda

REGULATION No. 14

(Safety-belt anchorages)

Mandatory fitting of safety-belt anchorages for Class II buses

Proposal for draft amendments to Regulation No. 14

Submitted by the expert from Germany

The text reproduced below was prepared by the expert from Germany in order to introduce exemptions to low-floor buses to the proposed mandatory fittings of safety-belt anchorages for Class II buses.  It is based on a document without a symbol (informal document No. GRSP-40-08), distributed during the fortieth session of GRSP (see report ECE/TRANS/WP.29/GRSP/40, para. 23).

The modifications to the existing text of the Regulation are marked in bold characters or marked as strikethrough.

A.	PROPOSAL

Insert a new paragraph 2.32., to read:

"2.32.	"Low-floor bus" means a vehicle of Class I, II or A in which at least 35 per cent of the area available for standing passengers (or in its forward section in the case of articulated vehicles, or in its lower deck in the case of double-deck vehicles) forms an area without steps and includes access to at least one service door."

Paragraph 5.3.1., amend to read:

"5.3.1.	Any vehicle in categories M and N (except those vehicles of categories M2 or M3 which belong to Classes I or II (low floor buses only) or A 1/) shall be equipped with safety-belt anchorages which satisfy the requirements of this Regulation.

Vehicles of category M1 shall be equipped with ISOFIX anchorage systems, which satisfy the requirements of this Regulation in accordance with paragraph 5.3.8.

N1 vehicles fitted with ISOFIX anchorages have also to comply with the provisions of this Regulation.

1/	As defined in Annex 7 to the Consolidated Resolution on the Construction of vehicles (R.E.3), document TRANS/WP.29/78/Rev.1, as last amended by Amendment 4."

### B.	JUSTIFICATION

According to the proposal non-low-floor buses of Class II have to be fitted with safety-belts, while low-floor vehicles are not mandated to receive this equipment.

Non-low-floor vehicles of Class II are constructed principally for the carriage of seated passengers and provide interurban services.  The design of those vehicles derives to a large extent from vehicles of Class III, generally known as coaches.

Low-floor buses of Class II are principally based on low-floor buses of Class I.  They provide areas for standing passengers in considerable sizes and the accommodation of wheelchairs and are often used in modes of operation quite similar to vehicles of Class I.

Therefore the same prescriptions should apply.

- - - - -