## PROPOSAL FOR DRAFT CORRIGENDUM TO REGULATION No. 14

SAFETY-BELT ANCHORAGES

# ISOFIX Static Test Requirements

### A.	PROPOSAL

Paragraph 6.6.4.2, amend to read:

“... Full application of the force shall achieved within a period of 2 s or less. The force shall be fully applied as quickly as possible and shall be maintained within a period of [30 s] at maximum; provided, however, that the manufacturer may request that the specified force be applied within a period of 2 s or less. ...”

### B.	JUSTIFICATION

For ISOFIX, the 8 kN force is required to be applied within a period of 2 s or less. On the other hand, for safety-belt anchorages, the 1,350 daN is required to be applied within a period of 60 s or less. This means that the force application standards are not harmonized between ISOFIX and safety-belt anchorages. The force application time for ISOFIX, if harmonized with the force application conditions for safety-belt anchorages, would be approximately 35 s.

Since it is sometimes difficult to achieve the force application time of 2 s or less for those vehicles with a large displacement, we propose to change the force application time to 30 s or less, which is harmonized with the safety-belt anchorage standard.

------