#### DRAFT CORRIGENDUM 1 TO SUPPLEMENT 4 TO THE 04 SERIES

#### OF AMENDMENTS TO REGULATION No.44

(Child restraints)

Correction of Errors in Paragraph 6.1.3. of WP29 2007/21

POSSIBLE CONFIGURATIONS FOR APPROVAL GROUPS / CATEGORIES TABLE

- PROPOSAL

In document WP29 2007/21 accepted at the last WP29

Paragraph 6.1.3., the table, replace in the fifth column, second row: A by N/A

| "Forward facing (non-integral) | NA | NA | NA | NA | NA | NA | NA | NA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Forward facing (non-integral – see point 6.1.12.) | A | NA | A | A N/A | A | NA | A | A" |

the proposal is in opposition with the text of the note (2) of the table and not in accordance with the table of Addendum 43:  Regulation No. 44  Revision 1 - Amendment 4 – Corrigendum 1

here after :

| GROUP CATEGORY |  | Universal (1) |  | Semi-universal (2) |  | Restricted |  | Specific Vehicle |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CRS | ISOFIXCRS | CRS | ISOFIXCRS | CRS | ISOFIXCRS | CRS | ISOFIXCRS |
| 0 | Carry-cot | A | NA | A | A | A | NA | A | A |
|  | Rearward facing | A | NA | A | A | A | NA | A | A |
| 0+ | Rearward facing | A | NA | A | A | A | NA | A | A |
| I | Rearward facing | A | NA | A | A | A | NA | A | A |
|  | Forward facing (integral) | A | A | A | A | A | NA | A | A |
|  | Forward facing (non integral) | A | NA | A | NA | A | NA | A | A |
| II | Rearward facing | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (integral) | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (non integral) | A | NA | A | NA | A | NA | A | A |
| III | Rearward facing | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (integral) | A | NA | A | NA | A | NA | A | A |
|  | Forward facing (non integral) | A | NA | A | NA | A | NA | A | A |

B. JUSTIFICATION

Concerning forward facing non integral child restraints equipped with a shield fitted with the adult safety belt, it is not possible to ensure every time a good fitting on the ISOFIX anchorages together with the adult safety belt. ISOFIX CRS equipped with a shield and only approved as “specific vehicle” could be compatible.

- - - - -