WP.11/2008/64/INF.5

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Perishable Foodstuffs

Sixty-fourth session

Geneva, 14-17 October 2008

- Item 8 of the provisional agenda

ANNEX 4 OF ATP

Regulations on fresh fruit and vegetables transportation

temperature conditions to be assured during transportation of fresh fruit and vegetables

During transportation, the temperature of indicated fruits and vegetables should be the in following range:

| Apricots, oranges, thick-skinned water melons, cherries, grapes, pears of late ripeness, grapefruits, pomegranates early-ripe food potatoes, white cabbage (early-rape, mid-season), cauliflower, vegetable marrows, gooseberries, onions, table carrots, mandarins, peaches, red beet without leaves, plum, currants beans, garlic, late-ripe apples | From +2 to +5°С |
| --- | --- |
| Pineapples, aubergines, melons, lemons, open soil cucumbers, tomatoes of rosy ripeness | From +6 tо 9°С |
| Tomatoes of milky ripeness | From +11 to 13°С |

In case the temperature of fresh fruit and vegetables needs to be checked, the regulation provided in annex 2, appendix 2 of ATP х) applies.

х) In the case of the adoption of Annex 4 (Regulation on fresh fruit and vegetables transportation), the modifications to annex 2, appendix 2 of ATP shall be made.

_____________