| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2008/14 13 December 2007 ENGLISH Original: ENGLISH AND FRENCH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-fourth session

Geneva, 11-14 March 2008

Item 4.2.9. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

Proposal for Supplement 1 to 03 series of amendments to the Regulation No. 19

(Front fog lamps)

Submitted by the Working Party on Lighting and Light-Signalling (GRE) */

The text reproduced below was adopted by GRE at its fifty-eighth session.  It is based on ECE/TRANS/WP.29/GRE/2007/43, not amended, and on ECE/TRANS/WP.29/GRE/2007/62, not amended.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRE/58, paras. 25 and 35).

Add a new paragraph 1.4.7., to read:

"1.4.7.	However, a device intended for the installation on the left side of the vehicle and the corresponding device intended for the installation of the right side of the vehicle shall be considered to be of the same type."

Insert a new paragraph 1.5., to read:

"1.5.	"Colour of the light emitted from the device."  The definitions of the colour of the light emitted, given in Regulation No. 48 and its series of amendments in force at the time of application for type approval, shall apply to this Regulation."

Paragraph 1.5. (former), renumber as paragraph 1.6.

Paragraph 2.3.2. (applicable to Class B front fog lamps), amend to read:

"2.3.2. 	Two samples of each type of front fog lamp, one sample intended for the installation on the left side of the vehicle and one sample intended for the installation of the right side of the vehicle."

Paragraph 2.4.9. (applicable to Class F3 front fog lamps), amend to read:

"2.4.9.	Two samples of each type of front fog lamp, one sample intended for the installation on the left side of the vehicle and one sample intended for the installation on the right side of the vehicle; or a matched pair of front fog lamps."

Paragraph 7., amend to read (all colour coordinates and pertaining paragraphs 7.1.1. and 7.1.2. should be deleted):

"7.	COLOUR

The colour of the light emitted by the front fog lamp shall be either white or selective yellow by choice of the applicant.  The selective yellow colour, if any, of the beam may be obtained either by the colour of the light source or by the lens of the front foglamp or by any other suitable means.

7.1.	The colorimetric characteristics of the front fog lamp shall be measured with voltages as defined in paragraphs 6.3. and 6.4."

- - - - -

*/ In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles.  The present document is submitted in conformity with that mandate.