| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2008/32 13 December 2007 ENGLISH Original: ENGLISH AND FRENCH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-fourth session

Geneva, 11-14 March 2008

Item 4.2.33. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

Proposal for Supplement 4 to the Regulation No. 99

(Gas-discharge light sources for use in approved gas-discharge lamp units of                       power-driven vehicles)

Submitted by the Working Party on Lighting and Light-Signalling (GRE) */

The text reproduced below was adopted by GRE at its fifty-eighth session.  It is based on ECE/TRANS/WP.29/GRE/2007/62, not amended.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRE/58, para. 25).

Paragraph 3.9.1., amend to read:

"3.9.1.	The colour of the light emitted shall be white.  Moreover the colorimetric characteristics, expressed in CIE chromaticity coordinates, shall lie within the boundaries given on the relevant data sheet."

Paragraph 3.9.2., amend to read:

"3.9.2.	The definitions of the colour of the light emitted, given in Regulation No. 48 and its series of amendments in force at the time of application for type approval shall apply to this Regulation."

Annex 1, sheets DxR/4 and DxS/4, amend the bottom part of the both tables to read:

"…

| Luminous flux | …. | …. | …. | …. |
| --- | --- | --- | --- | --- |
| Chromaticity coordinates | Objective |  | x = 0.375 | y = 0.375 |
|  | Tolerance area 3/ | ### Boundaries | x = 0.345 x = 0.405 | y = 0.150 + 0.640 x y = 0.050 + 0.750 x |
|  |  | Intersection points | x = 0.345 | y = 0.371 |
|  |  |  | x = 0.405 | y = 0.409 |
|  |  |  | x = 0.405 | y = 0.354 |
|  |  |  | x = 0.345 | y = 0.309 |
| Hot-restrike switch-off time |  | .... | .... | .... |

…"

- - - - -

*/ In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles.  The present document is submitted in conformity with that mandate.