Proposal for corrigendum to document ECE/TRANS/WP.29/2008/94

(Proposal for Supplement 11 to Regulation No 43)
(Safety glazing)

The following text is based on document ECE/TRANS/WP.29/2008/94 submitted for adoption to WP.29 and AC.1 by the Working Party on General Safety.

An editorial correction is inserted in ‘bold character’ and the deleted text is shown in ‘strike-through’.

Page 2,

Annex 3,

Paragraph 6.4.1.5., 2nd sub-paragraph, correct to read:

“The luminous transmittance regular light transmittance measured must not differ from the original test on unexposed samples by more than 5 per cent and shall not fall below:”

Justification

The light transmission factor through a glazing must be measured in accordance with the provisions of Section 9.1. of Annex 3. In this respect it is important to clarify that only the regular light transmittance factor is affected by paragraph 6.4.1.5. as the absorption in the visible spectre is influenced by the angle of incidence of the light with respect to the glazing.

The term ‘luminous transmittance’ is therefore inappropriate in this context.

- - - - -