| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2008/15 18 January 2008 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-ninth session

Geneva, 31 March - 4 April 2008

Item 5(g) of the provisional agenda

COLLECTIVE AMENDMENTS

Regulations Nos. 6, 7 and 48

Proposal for Supplement 2 to the 04 series of amendments to Regulation No. 48

(Installation of lighting and light-signalling devices)

Submitted by the expert from France /

The text reproduced below was prepared by the expert from France proposing to align with the state-of-the-art realizations for the visibility angles requirements of the position lamps, end-outline marker lamps and direction indicator lamps.  It supersedes ECE/TRANS/WP.29/GRE/2007/64, distributed during the fifty-eighth session of the Working Party on Lighting and Light-Signalling (GRE) (see report ECE/TRANS/WP.29/GRE/58, para. 32).  The modifications to the current text of the Regulation are marked in bold characters.

A.	PROPOSAL

Paragraph 6.5.5.1., amend to read:

"6.5.5.1.	Horizontal angles, (see figure below)

Vertical angles: 15° above and below the horizontal for direction-indicator lamps of categories 1, 1a, 1b, 2a, 2b and 5.  The vertical angle below the horizontal may be reduced to 5° if the lamps are less than 750 mm above the ground; 30° above and 5° below the horizontal for direction-indicator lamps of category 6.  The vertical angle above the horizontal may be reduced to 5° if the optional lamps are not less than 2,100 mm above the ground.

FIGURE (see paragraph 6. 5.)

Categories 1, 1a or 1b

Categories 5 or 6

Categories 2a or 2b

d

Arrangement A

*

80°

80°

45°

45°**

5°*

30°

________________________

(*)	The value of 5° given for dead angle of visibility to the rear of the side-direction indicator is an upper limit.  d < 1.80 m( for M1 and N1 category vehicles d < 2.50 m).

(**)	The value of 45° for the front direction indicator maybe reduced to 20° under the HH line of this lamp."

Paragraph 6.5.5.2., amend to read:

"6.5.5.2. 	or, at the discretion of the manufacturer, for M1 and N1 category vehicles (***):

Front and rear direction-indicator lamps, as well as side-marker lamps

Horizontal angles see figure below:

45°

Categories 1, 1a or 1b

Categories 2a or 2b

d

45°

45°

45°**

30°

45°

45°

30°

30°

Amber rearmost side

-

marker

Side Direction indicator

Front side

-

marker

5°

-

45°

Categories 1, 1a or 1b

Categories 2a or 2b

d

45°

45°

30°

45°

45°

30°

30°

Amber rearmost side

-

Side Direction indicator

Front side

-

5°

-

Vertical angles: 15°…….

____________________

(***)	The value of 5° given for dead angle of visibility to the rear of the side-direction indicator is an upper limit. d < 2.50 m." (for M1 and N1 category vehicles d < 2. 50 m)."

B. 	JUSTIFICATION

The current visibility requirements of the front and rear direction indicators and position lamps were written in the early 1970s.  They were easily fulfilled in those times because the shapes of the vehicles were simple and rather boxy.  In recent years, fuel-efficient aerodynamics and pedestrian protection have led to more contoured shapes making the fulfilment of the 15° down and 45° inboard (15D-45inboard) visibility requirement impossible to achieve.  This proposal aims at eliminating this requirement.

- - - - -

/  In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles.  The present document is submitted in conformity with that mandate.