| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2008/27 11 July 2008 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Sixtieth session

Geneva, 1 - 3 October 2008

Item 3(a) of the provisional agenda

REGULATION No. 37

(Filament lamps)

Proposal for Supplement 33 to the 03 series of amendments to Regulation No. 37

Submitted by the expert from India /

The text reproduced below was prepared by the expert from India in order to introduce into Regulation No. 37 new (WY10W; WY16W) light source categories for signalling lamps.  The proposal is based on a document without symbol (informal document No. GRE-59-38), distributed during the fifty-ninth GRE session (see report ECE/TRANS/WP.29/GRE/59, para. 6). The proposal is based on the current text of the Regulation including draft Supplement 32 to the 03 series of amendments (ECE/TRANS/WP.29/2008/84).  The modifications to the existing text of the Regulation are marked in bold characters.

A.	PROPOSAL

The list of contents, the annexes, amend to read:

Annex 1,

The list of categories of filament lamps, grouped, and their sheet numbers, amend to read:

| "Group 2: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Only for use in signalling lamps, cornering lamps, reversing lamps and rear registration plate lamps: |  |  |  |  |
|  | Category |  | Sheet number(s) |  |
|  | C5W |  | C5W/1 |  |

….					….

|  | W5W |  | W5W/1 |  |
| --- | --- | --- | --- | --- |
|  | WY10W |  | WY10W/1 |  |
|  | W15/5W |  | W15/5W/1 to 3 |  |
|  | W16W |  | W16W/1 |  |
|  | WY16W |  | WY16W/1 |  |
|  | W21W |  | W21W/1 to 2 |  |

…					…."

The list of sheets for filament lamps and their sequence, amend to read:

"

|  | Sheet number(s) |  |
| --- | --- | --- |

…

|  | C5W/1 |  |
| --- | --- | --- |

…

|  | W5W/1 |  |
| --- | --- | --- |
|  | WY10W/1 |  |
|  | W15/5W/1 to 3 |  |
|  | W16W/1 |  |
|  | WY16W/1 |  |
|  | W21W/1 to 2 |  |

…"

Insert a new sheet WY10W/1, between sheet W5W/1 and sheet W15/5W/1 to 3, to read

(see next pages).

"	CATEGORY WY10W 	Sheet WY10W/1

The drawings are intended only to illustrate the essential dimensions (in mm) of the filament lamp

| Dimensions in mm |  | Filament lamps of normal production |  |  | Standard filament lamp |
| --- | --- | --- | --- | --- | --- |
|  |  | min. | nom. | max. |  |
| e |  | 18.3 | 20.6 | 22.9 | 20.6  0.3 |
| Lateral deviation 1/ |  |  |  | 1.0 | 0.5 max. |
|  |  | -15° | 0° | + 15° | 0°  5° |
| Cap W3x9.5d – As per the attached drawing. (To be added in IEC Publication 60061) |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARACTERISTICS |  |  |  |  |  |
| Rated values | Volts | 12 |  |  | 12 |
|  | Watts | 10 |  |  | 10 |
| Test voltage | Volts | 13.5 |  |  | 13.5 |
| Objective values | Watts | 11 max. |  |  | 11 max. |
|  | Luminous flux | 75  20 % |  |  |  |
| Reference luminous flux at approximately 13.5 V: |  | White: 125 lm |  |  |  |
|  |  | Amber: 75 lm 280 lm |  |  |  |

1/	Maximum lateral deviation of filament centre from two mutually perpendicular planes both containing the reference axis and one containing axis X-X."

Insert a new sheet WY16W /1, between sheet W16W/1 and sheet W21W/1 to 2, to read:

"	CATEGORY WY16W 	Sheet WY16W/1

The drawings are intended only to illustrate the essential dimensions (in mm) of the filament lamp

| Dimensions in mm |  | Filament lamps of normal production |  |  | Standard filament lamp |
| --- | --- | --- | --- | --- | --- |
|  |  | min. | nom. | max. |  |
| E |  | 18.3 | 20.6 | 22.9 | 20.6  0.3 |
| Lateral deviation 1/ |  |  |  | 1.0 | 0.5 max. |
|  |  | -15° | 0° | + 15° | 0°  5° |
| Cap W3x9.5d – As per the attached drawing. (To be added in IEC Publication 60061) |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARACTERISTICS |  |  |  |  |  |
| Rated values | Volts | 12 |  |  | 12 |
|  | Watts | 16 |  |  | 10 |
| Test voltage | Volts | 13.5 |  |  | 13.5 |
| Objective values | Watts | 21.35 max. |  |  | 21.35 max. |
|  | Luminous flux | 189  20 % |  |  |  |
| Reference luminous flux at approximately 13.5 V: |  | White: 310 lm |  |  |  |
|  |  | Amber: 189 lm 280 lm |  |  |  |

1/	Maximum lateral deviation of filament centre from two mutually perpendicular planes both containing the reference axis and one containing axis X-X."

B.	JUSTIFICATION

This proposal is intended to introduce into Regulation No. 37 new WY10W and WY16W light source categories for signalling lamps.  W16W category already exists in Regulation No. 37 and the expert from India has proposed, separately, the inclusion of W10W category.  Amber versions of these bulbs will be useful for direction indicator for L category vehicles.

/  In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles.  The present document is submitted in conformity with that mandate.