| Transmitted by the Expert from CLCCR | Informal document No. GRE-59-07 (59th GRE, 31 March – 4 April, 2007, Agenda item 4(j)) |
| --- | --- |

REGULATION No. 48

(Installation of lighting and light-signalling devices)

Requirements for Conspicuity Markings

A.	PROPOSAL

Paragraphs 6.21.1.2.1. and 6.21.1.2.2.1., amend to read:

"6.21.1.2.1.	to the rear:

full contour marking on vehicles exceeding 2,100 mm in width of the following categories:

-	N2 with a maximum mass exceeding 7.5 tonnes and N3 (with the exception of chassis-cabs, incomplete vehicles and tractors for semi-trailers)

-	O3 and O4 (with the exception of incomplete vehicles)”

“6.21.1.2.2.1.	partial contour marking on vehicles exceeding 6,000 mm in length (including the drawbar for trailers) of the following categories:

-	N2 with a maximum mass exceeding 7.5 tonnes and N3 (with the exception of chassis-cabs, incomplete vehicles and tractors for semi-trailers)

-	O3 and O4 (with the exception of incomplete vehicles)”

# B.	JUSTIFICATION

The proposal is intended to apply the same exception to incomplete vehicles of categories O3 and O4 as that already existing for incomplete vehicles of categories N2 with a maximum mass exceeding 7.5 tonnes and N3.

- - - - -