| Transmitted by the expert from Germany | Informal document No. GRE-59-31 (59th GRE, 31 March - 4 April 2008 agenda item 6) |
| --- | --- |

| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2007/.. 12 July 2007 Original: ENGLISH ENGLISH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-ninth session

Geneva, 31 March to 5 April 2008

Item 6. of the provisional agenda

REGULATION No. 19

(Front fog lamps)

Submitted by the expert from Germany

The text reproduced below was prepared by the expert from Germany in order to correct the draft 03 series of amendments to Regulation No. 19, as approved and amended at the fifty-seventh GRE session (ECE/TRANS/WP.29/GRE/2006/19

and Corr.1, TRANS/WP.29/GRE/57, par.49 and Annex VI).

The modifications to this text are marked in bold characters.

A	PROPOSAL:

Paragraph 6.4,  add the footnote *** to Line 6, 7 and  Zone D, to read:

6.4.3.When so adjusted, the front fog lamp shall meet the photometric requirements of column I or column II in the table below.

|  | Designated Lines or Zones | Vertical position * above h+ below h- | Horizontal position * Left of v: - Right of v: + | Luminous Intensity in cd |  | To Comply |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | I | II |  |
|  | Point 1, 2(**) Point 3, 4(**) Point 5, 6(**) Point 7, 10(**) Point 8, 9(**) | +60° +40° +30° +20° +20° | ± 45° ± 30 ± 60 ± 40 ± 15 | 60* cd max | 90* cd max | all points |
|  | Line 1** | + 8° | - 26° to + 26° | 80* cd max | 120* cd max | all line |
|  | Line 2** | + 4° | - 26° to + 26° | 100* cd max | 150* cd max | all line |
|  | Line 3 | + 2° | - 26° to + 26° | 160*cd max | 160*cd max | all line |
|  | Line 4 | + 1° | - 26° to + 26° | 240*cd max | 240*cd max | all line |
|  | Line 5 | 0° | - 10° to + 10° | 320* cd max | 320* cd max | all line |
|  | Line 6*** | - 2.5° | - 10° to + 10° | 2 400 cd min | 2 400 cd min | all line |
|  | Line 7*** | - 6.0° | - 10° to + 10° | <50% of max. on line 6 | <50% of max. on line 6 | all line |
|  | Line 8L and R *** | -1.5° to - 3.5° | - 22° and + 22° | 1000 cd min | 1000 cd min | One or more points |
|  | Line 9Land R *** | -1.5° to - 4.5° | - 35° and + 35° | 400 cd min | 400 cd min | One or more points |
|  | Zone D *** | - 1° to - 3 ° | - 10° to + 10° | 8000cd max | 8000cd < Imax ≤ 12500cd | whole zone |
|  | * The co-ordinates are specified in degrees for an angular web with a vertical polar axis. ** see paragraph 6.6.3.1. *** see paragraph 6.6.1.. |  |  |  |  |  |

6.4.3.1. 	The illumination shall be measured either with white light or coloured light as

prescribed by the applicant for use of the fog lamp in normal service. Variations

in homogeneity detrimental to satisfactory visibility in the zone above the line 5

from 10 degrees left to 10 degrees right are not permitted.

Paragraph 6.43.2.,  amend to read:

6.4.3.2. 	At the request of the applicant, two front fog lamps constituting a matched pair

corresponding to paragraph 4.2.2.5. may be tested separately. In this case the

specified requirements for lines 6, 7, 8 ,9 and the Zone D  in the table in paragraph 6.4.3. apply to half the sum of readings of the right-hand and left-hand side front fog lamp.

B	JUSTIFICATION:

The expert of Germany are the opinion, that it is not logic to exclude the inner part between the Line 8 and Line 9 form the superposition principle of the matched pair procedure. This possibility could be also found in older GTB documents. So the experts of Germany see this proposal as a corrigendum to the existing draft 03 series of amendments to Regulation No. 19, as approved and amended at the fifty-seventh GRE session (ECE/TRANS/WP.29/GRE/2006/19 and Corr.1, TRANS/WP.29/GRE/57, par.49 and Annex VI).