| Transmitted by the expert from France | Informal document No. GRE-59-37 (59th GRE, 31 March – 4 April, 2008, agenda item 5(g)) |
| --- | --- |

A. Complement to ECE/TRANS/WP.29/GRE/2008/15

The modifications are marked in bold or strikethrough characters.

Paragraphs 6.9.5., amend to read:

"6.9.5.	Geometric visibility

6.9.5.1.	Horizontal angle for the two position lamps:

45 ° inwards and 80 ° outwards.

The value of 45° maybe reduced to 20° under the HH line of this lamp.

In the case of trailers, ...

6.9.5.2.	For M1 and N1 category vehicles, as an alternative to paragraph 6.9.5.1., at the discretion of the manufacturer or his duly accredited representative, and only if a front side-marker lamp is installed on the vehicle.

Horizontal angle: 45° outwards to 45° inwards.

The value of 45° maybe reduced to 20° under the HH line of this lamp.

Vertical angle: 15°...

B. Justification

With the modification coming from the text ECE/TRANS/WP.29/GRE/2008/17, it is necessary to modify the Regulation 48. This modification was omitted in the text ECE/TRANS/WP.29/GRE/2008/15.