| Transmitted by the expert from India | Informal document No. GRE-59-38 (59th GRE, 31 March – 4 April, 2008, agenda item 3(a)) |
| --- | --- |

# INDIA’S COMMENTS ON ECE REGULATION NO. 37

India suggests modification in the current text of the Regulation 37 as follows: (additions are marked in bold characters).

A. 	PROPOSAL

Annex 1

The list of categories of filament lamps, grouped, and their sheet numbers, amend to read:

"

| Group 2: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Only for use in signalling lamps, cornering lamps, reversing lamps and rear registration plate lamps: |  |  |  |  |
|  | Category |  | Sheet number(s) |  |
|  | C5W |  | C5W/1 |  |

….

|  | W5W |  | W5W/1 |  |
| --- | --- | --- | --- | --- |
|  | WY10W |  | WY10W/1 |  |
|  | W16W |  | W16W/1 |  |
|  | WY16W |  | WY16W/1 |  |
|  | W21W |  | W21W/1 to 2 |  |

…"

The list of sheets for filament lamps and their sequence, amend to read:

"

|  | Sheet number(s) |  |
| --- | --- | --- |

…

|  | C5W/1 |  |
| --- | --- | --- |

…

|  | W5W/1 |  |
| --- | --- | --- |
|  | WY10W/1 |  |
|  | W16W/1 |  |
|  | WY16W/1 |  |
|  | W21W/1 to 2 |  |

…"

Insert new sheets W10W /1 to 3, between sheet W5W/1 and sheet W16W/1, to read:

"

CATEGORY WY10W 	Sheet WY10W/1

The drawings are intended only to illustrate the essential dimensions (in mm) of the filament lamp

| Dimensions in mm |  | Filament lamps of normal production |  |  | Standard filament lamp |
| --- | --- | --- | --- | --- | --- |
|  |  | min. | nom. | max. |  |
| e |  | 18.3 | 20.6 | 22.9 | 20.6  0.3 |
| Lateral deviation 1/ |  |  |  | 1.0 | 0.5 max. |
|  |  | -15° | 0° | + 15° | 0°  5° |
| Cap W3x9.5d – As per the attached drawing. (To be added in IEC Publication 60061) |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARACTERISTICS |  |  |  |  |  |
| Rated values | Volts | 12 |  |  | 12 |
|  | Watts | 10 |  |  | 10 |
| Test voltage | Volts | 13.5 |  |  | 13.5 |
| Objective values | Watts | 11 max. |  |  | 11 max. |
|  | Luminous flux | 75  20 % |  |  |  |
| Reference luminous flux at approximately 13.5 V: |  | White: 125 lm |  |  |  |
|  |  | Amber: 75 lm 280 lm |  |  |  |

1/	Maximum lateral deviation of filament centre from two mutually perpendicular planes both containing the reference axis and one containing axis X-X.

Insert new sheets W10W /1 to 3, between sheet W16W/1 and sheet W21W/1 to 2, to read:

"

CATEGORY WY16W 	Sheet WY16W/1

The drawings are intended only to illustrate the essential dimensions (in mm) of the filament lamp

| Dimensions in mm |  | Filament lamps of normal production |  |  | Standard filament lamp |
| --- | --- | --- | --- | --- | --- |
|  |  | min. | nom. | max. |  |
| e |  | 18.3 | 20.6 | 22.9 | 20.6  0.3 |
| Lateral deviation 1/ |  |  |  | 1.0 | 0.5 max. |
|  |  | -15° | 0° | + 15° | 0°  5° |
| Cap W3x9.5d – As per the attached drawing. (To be added in IEC Publication 60061) |  |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARACTERISTICS |  |  |  |  |  |
| Rated values | Volts | 12 |  |  | 12 |
|  | Watts | 16 |  |  | 10 |
| Test voltage | Volts | 13.5 |  |  | 13.5 |
| Objective values | Watts | 21.35 max. |  |  | 21.35 max. |
|  | Luminous flux | 189  20 % |  |  |  |
| Reference luminous flux at approximately 13.5 V: |  | White: 310 lm |  |  |  |
|  |  | Amber: 189 lm 280 lm |  |  |  |

1/	Maximum lateral deviation of filament centre from two mutually perpendicular planes both containing the reference axis and one containing axis X-X.

- Lamp cap to be added in  IEC Publication 60061

|  |  | LAMP BASE W3x9.5d |  |  |  | Page 1/1 |
| --- | --- | --- | --- | --- | --- | --- |
| Dimensions in millimeters The drawing is intended only to illustrate the dimensions essential for interchangeability. For details of holder W3x9.5d, see sheet 7005-91. * This dimension is solely for base design and is not to be gauged. (1) Maximum contour of free space for exhaust tip allowing for eccentricity. |  |  |  |  |  |  |
|  | Standard dimensions |  |  |  |  |  |
|  | Dimensions |  | Min. | Max. |  |  |
|  | A |  | -- | 10,29* |  |  |
|  | B |  | 6,86* | -- |  |  |
|  | C (1) |  | -- | 3,95 |  |  |
|  | D |  | 8,90 | 9,50 |  |  |
|  | E |  | 1,65 | -- |  |  |
|  | F* |  | Nom. 1,52 |  |  |  |
|  | G |  | 3,4 | 4,6 |  |  |
|  | H |  | -- | 6,10 |  |  |
|  | J* |  | Nom. 0.76 |  |  |  |
|  | K* |  | Nom. 0.76 |  |  |  |
|  | M* |  | Nom. 1,52 |  |  |  |
|  | N |  | 2,80 | 3,30 |  |  |
|  | P |  | -- | 4,96 |  |  |
|  | Q |  | Approx 5,6 |  |  |  |
|  | S |  | 4,83 | -- |  |  |
|  | W |  | --- | 0,36 |  |  |
| 7004-91---- |  |  |  |  |  |  |

JUSTIFICATION

This proposal is intended to introduce into Regulation No. 37 new WY10W, WY16W and light source categories for signalling lamps. W16W is already existing in R 37 and India has proposed, separately, inclusion of W10W. Amber versions of these bulbs will be useful for direction indicator for L category vehicles.

A suitable cap, not interchangeable with white lamp does not exist in IEC. It is proposed that, the cap as per the details may be incorporated in IEC 60061.

-----