Proposal for draft Corrigendum 1 to Supplement 1 to the series 04 of amendments to the Regulation No. 48

(Installation of lights)

Paragraph 2.28.3., amend to read:

"2.28.3.	"Amber" means the chromaticity coordinates (x,y) 3/ of the light emitted that lie inside the chromaticity areas defined by the boundaries:

| A12 | green boundary: | y = x - 0.120 |
| --- | --- | --- |
| A23 | the spectral locus |  |
| A34 | red boundary: | y = 0.390 |
| A41 | white boundary: | y = 0.790 - 0.670 x |

with intersection points:

|  | x | y |
| --- | --- | --- |
| A1: | 0.545 | 0.425 |
| A2: | 0.560 | 0.440 |
| A3: | 0.609 | 0.390 |
| A4: | 0.597 | 0.390 |

"

- - - - - -