|  | Working paper No. GFV-01-09 (1st GFV, 15 January 2008 agenda item 4.) |
| --- | --- |

NEW WORK ITEM

Reference Fuel Specification: Deleting Wobbe Index Reference

INFORMAL GROUP ON GASEOUS FUEL VEHICLES

Within the UN GRPE (WP29)

Reference Fuel Specification: Deleting Wobbe Index Reference

CLEPA

Name of Organisation submitting Amendment/Work Item

CLEPA

Person submitting Item

Matthias Tappe

Address/phone/email coordinates

Robert Bosch GmbH

Systems Engineering (GS/ENS2)

Postfach 30 02 40

70442 Stuttgart

GERMANY

Tel. + 49 711 811 24860

Matthias.Tappe@de.bosch.com

Regulation name and reference number

Addendum to the 1958 Agreement (Reg. 83).

Annex 10a  1. SPECIFICATIONS OF GASEOUS REFERENCE FUELS

Annex 12 GRANTING OF AN ECE TYPE APPROVAL FOR A VEHICLE

FUELLED BY LPG OR NATURAL GAS (NG)

Any other related UN ECE regulation in which is referred to CNG fuel measurement and display units.

Name of Amendment/Work Item

Update reference fuel specification to harmonize with market fuel standardization.

Delete Wobbe Index from specification for CNG reference fuels.

Specific language for Amendment/Work Item

Annex 10a

1.2. TECHNICAL DATA OF THE NG REFERENCE FUELS

Delete in table the lines

“Wobbe Index (net) MJ/m3 3/ 48.2 47.2 49.2”

“Wobbe Index (net) MJ/m3 3/ 39.4 38.2 40.6”

Annex 12

3.1. Exhaust emissions approval of a parent vehicle

The parent vehicle should demonstrate its capability to adapt to any fuel

composition that may occur across the market. In the case of LPG there are

variations in C3/C4 composition. In the case of natural gas there are generally two

types of fuel, high calorific fuel (H-gas) and low calorific fuel (L-gas), but with a

significant spread within both ranges; they differ significantly in Wobbe index.

These This variations are is reflected in the reference fuels.

4.1. Tests for conformity of production may be performed with a commercial fuel of

which the C3/C4 ratio lies between those of the reference fuels in the case of LPG,

or of which the Wobbe index lies between those of the extreme reference fuels in

the case of NG. In that case a fuel analysis needs to be present.

Rationale: (Why is it important/required?)

The Wobbe Index is no significant fuel characteristic for engine operation. Methane content is the most important characteristic for e.g. controlling the engine, power output, consumption, drivability. The Wobbe Index is used by the gas suppliers to fulfill the requirement to indicate the energy content of the market fuel in the public distribution system. Possibly use heating value as new specification parameter

The Wobbe Index has also been deleted from the (German) draft DIN 51624 (February 2007) for CNG fuel.

Analysis/testing or data requirements to support the Amendment/Work item

(could be anticipated or existing supporting documentation)

JMS.C/Standards/Informal Working Group template.