INFORMAL GROUP ON GASEOUS FUEL VEHICLES

Within the UN GRPE (WP29)

Name of Organisation submitting Amendment/Work Item

Expert from Poland to GRPE

Person submitting Item: Stanislaw Radzimirski

Address/phone/email coordinates

Stanislaw Radzimirski, DSc

Head of Environment Protection Centre

Motor Transport Institute

03-301 Warszawa

Poland

Ph.: + 48 22 675 39 37

Fax: + 48 22 811 09 06

e-mail: stanislaw.radzimirski@its.waw.pl

Date Submitted: 18.02.08

Regulation name and reference number: Regulation 49 and 83

Name of Amendment/Work Item: Regulations 49 and 83 – LPG reference fuel

Specific language for Amendment/Work Item: English

Rationale: (Why is it important/required?)

The provisions in Regulations 49 and 83 related to LPG reference fuels have the following defects that may need correcting:

- composition of LPG reference fuels in Regulation 83 is not precisely defined,

- composition of LPG reference fuels B in Regulations 49 and 83 is different what is not justified,

- H/C ratio for LPG is not correct in Regulations 49 and 83.

It is proposed that GFV gives attention to the matter.

Analysis/testing or data requirements to support the Amendment/Work item

(could be anticipated or existing supporting documentation)

1. Composition of LPG reference fuels in Regulation 83

The composition of LPG reference fuels specified in Annex 10 A to Regulation 83 is as follows (Table 1).

Table 1

|  |  | Fuel A | Fuel B |
| --- | --- | --- | --- |
| Composition |  |  |  |
| C3-content | per cent vol. | 30±2 | 85±2 |
| C4-content | per cent vol. | balance | balance |
| <C3, >C4 | per cent vol. | max. 2 | max. 2 |
| Olefins | per cent vol. | max. 12 | max. 15 |

It is not clear what “balance” means:

- balance = 100 - C3 (if so, the total may be higher than 100, e.g. C3 - 31, C4 – 69, <C3 – 2, total = 102) or

- balance = 100 - C3 -  <C3 - >C4 ( the most probable) or even

- balance = 100 - C3 - <C3 - >C4 – olefins.

It is proposed to clarify this problem and correct Regulation 83.

2. Different composition of LPG reference fuels B in Regulations 49 and 83

The composition of LPG reference fuels specified in Annex 7 to Regulation 49 is as follows (Table 2).

Table 2

|  |  | Fuel A |  | Fuel B |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum | Minimum | Maximum |
| Composition |  |  |  |  |  |
| C3-content | % vol. | 48 | 52 | 83 | 87 |
| C4-content | % vol. | 48 | 52 | 13 | 17 |
| Olefins | % vol. | 0 | 12 | 0 | 14 |

There are the following differences between fuels B in Regulation 49 and Regulation 83:

- if “balance” in Regulation 83 means “balance = 100 – C3 - <C3 - >C4”, C4 content may be different, e.g.

  - Regulation 83: C3 = 87, <C3 + >C4 = 2, C4 = 11% vol.,

  - Regulation 49: C4 can not be lower than 13 % vol.,

    - <C3, >C4 are not permitted in Regulation 49,

    - maximum content of olefins:

  - Regulation 83 – 15 % vol.,

  - Regulation  49 – 14 % vol.

These differences do not seem to be justified. It is proposed to harmonize the composition of  LPG reference fuel B in both the Regulations.

3. H/C ratio for LPG

The H/C ratio for LPG is assumed to be equal to:

- 2,525 in Regulation 49 (paragraph 2.7),

- 2,525 in Regulation 83 (paragraph 2.4).

To verify the above ratios, the calculation has been made based on the compositions specified in Tables 1 and 2 and on the following assumptions:

- H/C ratio for LPG should be the mean value for the average composition of fuels A and B,

- H/C ratio is equal to:

  - 2,667  for C3 (without olefins),

  - 2,5 for C4 (without olefins),

  - 2 for all olefins,

  - 3 for <C3,

  - 2,4 for >C4,

- the average composition of fuels A and B is similar to that specified in Table 3 for Regulation 49 and Table 4 for Regulation 83.

Table 3

| H/C | Remark | Fuel A [% vol.] | Fuel B [% vol.] | Density* [kg/dcm3] |
| --- | --- | --- | --- | --- |
| 2,667 | C3 without olefins | 47 | 79,05 | 0,5074 |
| 2,5 | C4 without olefins | 47 | 13,95 | 0,5727 |
| 2 | C3 olefins | 3 | 5,95 | 0,5218 |
| 2 | C4 olefins | 3 | 1,05 | 0,6081 |

* Source: Keith Owen, Trevor Coley: Automotive Fuels Handbook.

Table 4

| H/C |  | Remark | Fuel A [% vol.] | Fuel B [% vol.] | Density* [kg/dcm3] |
| --- | --- | --- | --- | --- | --- |
| 2,667 | C3 without olefins |  | 28,2 | 78,625 | 0,5074 |
| 2,5 | C4 without olefins |  | 64,8 | 12,875 | 0,5727 |
| 2 | C3 olefins |  | 1,8 | 6,375 | 0,5218 |
| 2 | C4 olefins |  | 4,2 | 1,125 | 0,6081 |
| 3 | <C3 |  | 0,5 | 0,5 | 0,3730 |
| 2,4 | >C4 |  | 0,5 | 0,5 | 0,6297 |

* Source: Keith Owen, Trevor Coley: Automotive Fuels Handbook.

The results of calculation are shown in Table 5 where the following properties are specified:

- the average density and H/C ratio  for fuel A,

- the average density and H/C ratio  for fuel B,

- the average density and H/C ratio  for fuels A and B.

Table 5

|  | Regulation 49 |  |  | Regulation 83 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Fuel A | Fuel B | Mean | Fuel A | Fuel B | Mean |
| Density [kg/dcm3] | 0,5416 | 0,5184 | 0,5300 | 0,5542 | 0,5178 | 0,5360 |
| H/C ratio | 2,542 | 2,593 | 2,568 | 2,512 | 2,589 | 2,550 |

To confirm the above results, the calculation for different compositions of the LPG reference fuels  has been made. The average values of H/C ratio has been similar to those shown in Table 5. It proves that the average H/C ratio for LPG reference fuels differs from that specified in Regulations 49 and 83. It is important to note that the average H/C ratio for LPG specified in Regulation 49 should be different from that specified in Regulation 83 as the composition of fuels A is different (Table 1 and Table 2).

In light of the above, it is proposed to assume the following H/C ratio:

- 2.55 in Regulation 83,

- 2.57 in Regulation 49.

The HC density calculated for the above ratio is equal:

- 0.651 in Regulation 83,

- 0.652 in Regulation 49.

The following corrections are required, among other things:

- Regulation 83 – paragraph 2.4 in the body, paragraph 8.2 in Annex 4, paragraph 1.5.2.3 in Appendix 8 to Annex 4,

- Regulation 49 – paragraph 2.7 in the body, paragraph 4.3.1.1 in Appendix 2 to Annex 4.

Please submit comments  to e-mail: stanislaw.radzimirski@its.waw.pl

with copies to:

Andre Rijnders, Chairman (RDW, Netherlands) arijnders@rdw.nl

Acting secretariat(s)

Jeffrey Seisler (IANGV/Clean Fuels Consulting) jseisler@cleanfuelsconsulting.org

Arnaud Duvielbuerbigny (AEGPL) arnaud.duvielguerbigny@aegpl.be