STATUS OF THE WORK OF THE MOTOR INDUSTRY WITH RESPECT TO A GLOBAL TECHNICAL REGULATION ON WORLDWIDE HARMONIZED LIGHT-DUTY TEST PROCEDURES (WLTP)

A. Objective of the document

In previous discussions around the subject of WLTP, and specifically in Informal Document GRPE-55-12, OICA has clearly declared its support for the worldwide harmonisation of the emissions regulation of light duty vehicles. In order to ensure the smooth running of future discussions of WLTP and to secure successful development of a harmonised regulation, OICA proposes the early agreement of a common terminology and a clear statement of the existing diversity of worldwide emissions regulation.

The following papers describe the work performed to date within OICA and are proposed to form a prerequisite basis for future discussions which will be added to and developed throughout the WLTP project.

Comments and contributions from stakeholders and other interested parties to the accurate completion of these documents are requested.

B. OICA Papers

- The aspects of light duty vehicle emission testing (section C of this paper)

- Two separate informal documents addressing definitions:

    - Definitions of Vehicle Categories

    - General Technical Definitions

C. The aspects of light duty vehicle emission testing

# Scope

In order to describe the scope of this document, without overly restricting it and thereby missing important issues, listed below are the aspects of emissions testing that are deliberately excluded from this paper as they will be handled separately:

Vehicle categories, technical definitions (including treatment of defeat devices), measurement equipment, reference fuels, diesel smoke test, OBD, durability.

Some of these subjects are excluded because specialist groups will be expected to address the issues due to their complexity, others are excluded as they are seen to be aspects of the approval procedure rather than technical requirements and such procedures are not covered by global technical regulations.

# Drive Cycles

## EU / ECE / China / Australia etc.

| Time | 1180 s |
| --- | --- |
| Distance | 11007 m |
| Max. Speed | 120 km/h |
| Ave. Speed | 33.6 km/h |
| Soak | N/A |
| Gear shift (man) | Fixed speeds |

## USA “Standard Cycles”

| Time (excl. soak) | 1877 s |
| --- | --- |
| Distance | 17860 m |
| Max. Speed | 91.2 km/h |
| Ave. Speed | 34.2 km/h |
| Soak | 600 s |
| Gear shift (man) | Specific (with evidence) |

| Time | 765 s |
| --- | --- |
| Distance | 16500 m |
| Max. Speed | 96.4 km/h |
| Ave. Speed | 77.4 km/h |
| Soak | N/A |
| Gear shift (man) | Specific (with evidence) |

## USA “Off Cycle”

| Time (excl. soak) | 600 s |
| --- | --- |
| Distance | 12870 m |
| Max. Speed | 129 km/h |
| Ave. Speed | 77.2 km/h |
| Soak | N/A |
| Gear shift (man) | Specific (with evidence) |

| Time (excl. soak) | 594 s |
| --- | --- |
| Distance | 5760 m |
| Max. Speed | 88 km/h |
| Ave. Speed | 34.9 km/h |
| Soak | N/A |
| Gear shift (man) | Specific (with evidence) |

## Japan

| Time (excl. soak) | 1204 s |
| --- | --- |
| Distance | 8172 m |
| Max. Speed | 81.6 km/h |
| Ave. Speed | 24.4 km/h |
| Soak | Repeated as hot test |
| Gear shift (man) | Fixed speeds |

## India

| Time (excl. soak) | 1180 s |
| --- | --- |
| Distance | m |
| Max. Speed | 90 km/h |
| Ave. Speed | km/h |
| Soak | N/A |
| Gear shift (man) | Fixed speeds |

# Inertia Classes

|  | USA | EU | Japan |  |
| --- | --- | --- | --- | --- |
|  |  |  | old | new |
| Minimum | 1000 lbs | 455 kg (1000 lbs) | 500 kg | 455 kg |
| initial steps | 125 lbs (to 4000 lbs) | 55/60 kg (125 lbs) (to 800 kg) | 125 kg (to 1000 kg) | 55/60 kg (to 800 kg) |
| middle steps | 250 lbs (to 5500 lbs) | 110/120 kg (250 lbs) (> 800 kg) | 250 kg (to 3000 kg) | 110/120 kg (to 2270kg) |
| final steps | 500 lbs (> 5500 lbs) |  |  | 230/250 kg (to 3000 kg) |
| Maximum | 7500 lbs | 2270 kg (5000 lbs) | no max. （step by 500kg, >3000kg） |  |

A further slight offset of the inertia classes is evident due to the positioning of a vehicle within the inertia classes being based on its “reference weight” which is differently defined in each region:

|  | “Empty & dry vehicle” | Fluids | Fuel | Tools | Spare wheel | Optional equipments | Occupants & test equipment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EU |  |  | min. 90 % |  |  |  | 100 kg |
| Japan |  |  |  |  |  | Full | 110 kg |
| USA |  |  |  |  |  | over 33% | 300 lbs (136 kg) |

Additionally to the above items, the rules for consideration of accessories or optional fitments according to their (anticipated) take up rates vary between the regions.

# Determination of Road Load

The differences between the regions in terms of road load determination, whether by Coastdown or torque measurement procedure consist of the speeds at which the vehicle is operated (related to cycle speeds), the weight of the vehicle that is tested (related to definitions and rules for inclusion of optional features), the specification of the vehicle that is tested (related again to rules for inclusion of optional features e.g. tyres, spoilers), the process for translation of the measured data to the dynamometer and the process for repeated usage of the dynamometer settings.

# Off Cycle Tests

The issue of off-cycle emissions testing is very complex one as it needs to achieve a balance between preventing what may be termed as cycle beating, and a need to avoid a) a massive increase in certification workload and b) a manufacturer’s uncertainty of having to certify the unknown.  The subject of off-cycle testing will need to be considered in connection with that of defeat devices, the definitions of which are handled in another paper.

The current test cycles that are utilised for light duty off-cycle checks are tabulated below:

|  | Low Temperature | High Altitude | High temperature / Air Cond. | High Speed |
| --- | --- | --- | --- | --- |
| EU | Type 6, -7°C, Petrol & E85 only, CO & THC |  |  |  |
| USA | 20°F (-6.7°C), Petrol only, CO ,NMHC(10MY Phase-in) | over 4000 ft | SC 03, 35°C, CO, NMHC + NOx & PM | Highway NOx |
|  | 50°F (10°C) Petrol only, CO NOx, NMOG & HCHO |  |  | US 06, CO, NMHC + NOx & PM |

As off-cycle emissions are a matter for all motor vehicles, there are already existing solutions in other fields e.g. heavy duty vehicles, which will need to be discussed during the WLTP project, including for example the concept of “not to Exceed”.

The differences between certification procedures and concepts and more importantly the difference between absolute and specific emissions measurements introduce difficulties when making comparisons between concepts.

It will therefore be important during WLTP discussions to remember that the following alternatives need to be assessed:

Tests:

- defined test procedures or

- random “real life” testing.

Assessment criteria:

- Unique pass/fail limits independent of certification limits  or

- Pass/fail limits based on certification limits (offset or factor) or

- A statistical or dimensionless pass/fail assessment system

# Ambient Conditions

|  | Temperature | Humidity | Cooling Fan |
| --- | --- | --- | --- |
| EU | 293 to 303 °K (20 to 30 °C) | Absolute 5,5 g ≤ H ≤ 12,2 g H2O/kg dry air | - Speed proportional from 10 km/h to at least 50 km/h - Area: at least 0,2 m2 - Height of the lower edge above ground: approximately 20 cm - Distance from the front of the vehicle: approximately 30 cm As an alternative the blower speed shall be at least 6m/s (21,6 km/h). |
| Japan | 298 ± 5 K (25 ± 5°C) | Relative 30% to 75%. | During the test running, the test vehicle shall be cooled by a cooling fan or the like so that actual running conditions may be simulated. (bonnet closed) |
| USA (FTP75) | 68 to 86 °F (20 to 30 °C) | NA | fixed speed fan (not exceed 5300cfm) |

# Measured/Regulated Components

|  |  | CO | THC | NMHC | NMOG | HCHO | NOx | PM | P# |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EU | SI |  |  |  |  |  |  | DI only | EU 6 |
|  | CI |  | THC+NOx |  |  |  |  |  | EU 5b |
| Japan |  |  |  |  |  |  |  |  CI only, SIDI* |  |
| EPA |  |  |  |  |  |  |  |  |  |
| CARB |  |  |  |  |  |  |  |  |  |

* only DI with NOx catalyst

# Durability

As the differences between durability procedures cause difficulties for manufacturers, a brief textual overview is included below and a comparison table is contained in a separate document.  However, there is an argument that durability processes are part of the approval process and should therefore not be included in a GTR.

In Europe the “useful life” of a light duty vehicle is fixed at 5 years or 160,000 km.  The alternatives are to perform a demonstration of this, either with a whole vehicle or a bench test, or to utilise assigned deterioration factors (DFs).

The USA have a “full useful life” of 10 years or 120,000 miles (193,000 km) but also apply standards at the “half useful life” of 5 years or 50,000 miles (80,000 km).  There is the option to extend the “full useful life” to 15 years or 150,000 miles (240,000 km) and thereby either receive a fleet NOx/NMOG (EPA/CARB) bonus or an exemption from the half life standards.  Separate arrangements exist for the durability of evaporative emissions.  The alternatives are to perform a demonstration of this, either with a whole vehicle or a bench test (SI only).  Usage of assigned deterioration factors is restricted to small volume manufacturers.

In Japan the “useful life” of a light duty vehicle is fixed at 80,000 km.  The alternatives are to perform a demonstration of this (recognition of EU/US tests is possible), or to utilise assigned deterioration factors (DFs).

Whole Vehicle Ageing

Component Ageing

Otto

Diesel

All Fuels

Standard Road Cycle, 160.000 km

(SRC)

Standard Bench Cycle

(SBC)

Standard Diesel Bench Cycle (SDBC)

Alternative 2b

Alternative 1

Alternative 2a

Standard Road Cycle, 160.000 km

AMA Cycle

160.000 km

Alternatives for a Durability Test in Europe