| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.15/AC.2/2009/27 10 June 2009 ENGLISH Original: FRENCH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Dangerous Goods

Joint Meeting of Experts on the Regulations annexed to the 
European Agreement concerning the International Carriage 
of Dangerous Goods by Inland Waterways (ADN)
(ADN Safety Committee)

Fifteenth session
Geneva, 24-28 August 2009
Item 4 (a) of the provisional agenda

# Proposals for amendments to the Regulations annexed to ADN

## Report of the “Substances” informal working group 
transmitted by the Government of Germany1, 2

###### I.	INTRODUCTION

1.	The “Substances” informal working group met on 15 and 16 April 2009 in Strasbourg. Representatives of Germany, the Netherlands and the Central Commission for the Navigation of the Rhine (CCNR) took part in the meeting, under the chairmanship of Mr. Krischok (Germany).

###### II.	RESULTS

2.	In accordance with the mandate decided by the Safety Committee, the group considered the following issues.

### A.	Updating of ADN, Table A, column (8), for the new entries referred to in informal document INF.14, submitted to the fourteenth session

3.	Since UN No. 1267 PETROLEUM CRUDE OIL and UN No. 1992 FLAMMABLE LIQUID, TOXIC, N.O.S., were already being transported in tank vessels, the group concluded that for the new entries UN No. 3494 PETROLEUM CRUDE OIL, ACID, INFLAMMABLE, TOXIC, carriage in tank vessels should also be permitted. The amendments proposed to Tables A and C in this regard are contained in annex 1.

4.	With regard to the other new entries, the group was not aware of any existing need for transport in tank vessels or in bulk. Should such need arise, the profession would have to submit applications for special authorizations and request amendments to Tables A and C.

B.	Proposed decision concerning the issue with UN No. 2672 AMMONIA SOLUTION raised by the German delegation in document ECE/TRANS/WP.15/AC.2/2009/2

5.	Under the existing regulations, it was not possible to diverge from the parameters contained in the relevant provision, which related to a worst case scenario. Additional research had demonstrated that ammonia solution in a concentration of 25% or more by mass corresponded to group N1. Ammonia solutions in concentrations of less than 25% by mass corresponded to group N3 (Hazard evaluation of substances transported by ships - Report of the forty-third session of the Working Group on the Evaluation of the Hazards of Harmful Substances Carried by Ships).

6.	The working group therefore concluded that in Table C, entry UN No. 2672 AMMONIA SOLUTION, relative density between 0.880 and 0.957 at 15 °C in water, with more than 10% but not more than 35% ammonia, should be divided into two separate entries. Accordingly, carriage of ammonia solutions in concentrations of 25% or more would be permitted in vessels of type C and ammonia solutions in concentrations of less than 25% in vessels of type N. The German delegation would draft a proposal to that effect for consideration at the forthcoming meeting of the Safety Committee.

### C.	Examination of columns (9) (Equipment required), (10) (Ventilation) and (11) (Provisions concerning loading, unloading and carriage) of Table A to ensure that they are based on consistent criteria

7.	The informal working group examined the particulars in columns (9), (10) and (11) of Table A. The particulars in column (12) (Number of blue cones/lights) were also checked. The checks involved undertaking a comparison with the criteria adopted some time ago by CCNR. During the meeting, the criteria too came under discussion. The results of the checks are presented in five groups.

### 1.	Corrections resulting from the application of existing criteria

8.	The amendments and additions proposed on the basis of the application of existing criteria are set out in tabular form in annex 2.

### 2.	Proposed amendments to criteria

9.	Following discussion, the group reached the opinion that the criteria relating to the requirement for an escape device (EP) were not consistent. For all substances of Classes 6.1 and 8, an escape device was required, irrespective of the physical state of the substance (solid or liquid) and of the packing group. Conversely, for solid substances of Classes 4.1 and 5.1 with subsidiary hazards 6.1 and 8 (i.e. with a classification code containing T or C), no such device was prescribed even though, in the working group’s opinion, those substances posed a comparable danger. It was thus proposed to amend the criteria for requiring an escape device so that such a device would also be required on board for all substances of Classes 4.1 and 5.1 with a T or C classification code (column (3b)). It was also proposed that the criteria for requiring a toximeter (TOX) should be extended to cover solid substances of Class 4.3 which, in contact with water, emitted flammable gases. That requirement had already been applied to UN Nos. 1360, 1395, 1397, 1408, 1419, 1432, 1433, 1714, 2011, 2012 and 3013.

### 3.	Corrections resulting from the proposals to amend criteria

10.	In the event that the Safety Committee approves the proposal to amend the criteria for requiring an appropriate escape device, consequential amendments will be required to Table A. The amendments proposed are set out in tabular form in annex 3.

### 4.	Additional corrections

11.	During the discussion, the working group noted that for UN No. 1589 the hazard “unst.” should be added in column (5) and that for UN No. 3471 GE I+II special provision 802 should be added in column (6). The additions proposed are set out in tabular form in annex 4.

### 5.	Editorial changes to criteria which do not affect substantive content in Table A

12.	VE04 was inserted in column (6) for AEROSOLS (UN No. 1950) and special provision 327 in column (10) of Table A. That criterion is to be supplemented.

13.	In the criteria for ST01 and HA09* for Class 5.1, the entries which no longer exist, i.e. UN Nos. 2068, 2069 and 2070, should be deleted.

14.	For LO04, the phrase “for all goods with ‘B’ in column (8), ‘LO04*’ should be inserted in column (11)” should be transferred from Class 5.2 to Class 5.1, both in the table and in the text relating to Table A.†

15.	In addition, the group proposed that code RA02 should be grouped with RA03 and code HA03 with HA06, since they were only ever applied together.

### D.	Proposal to add entry UN No. 3495 IODINE

16.	It was proposed to add the entry UN No. 3495 IODINE to Table A as a corrosive solid substance of Class 8 with a toxic subsidiary hazard (informal document INF.14 submitted to the fourteenth session). In accordance with the criteria, personal protective equipment (PP) and an escape device (EP) were required. A particular property of iodine, however, was that it sublimed. The members of the informal working group therefore proposed that a toximeter (TOX) should be prescribed for iodine. That would involve amending the criteria as follows: “TOX for all liquid substances of Class 8 with T in column (3) and for UN No. 3495”.††

17.	Pursuant to the criteria, a breathing apparatus (A) was also required and the corresponding provisions with respect to ventilation (VE02) should be applied.

## Annex 1

# Petroleum crude oil

In Table A, for the three entries UN No. 3494 PETROLEUM CRUDE OIL, ACID, INFLAMMABLE, TOXIC, insert “T” in column (8).

Add the following three entries to Table C:

| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7) | (8) | (9) | (10) | (11) | (12) | (13) | (14) | (15) | (16) | (17) | (18) | (19) | (20) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UN No. or substance identification No. | Name and description | Class | Classification code | Packing group | Dangers | Type of tank vessel | Cargo tank design | Cargo tank type | Cargo tank equipment | Opening pressure of the high-velocity vent valve in kPa | Maximum degree of filling in % | Relative density at 20 C | Type of sampling device | Pump room below deck permitted | Temperature class | Explosion group | Anti-explosion protection required | Equipment required | Number of blue cones/lights | Additional requirements/Remarks |
| 3494 | PETROLEUM CRUDE OIL, ACID, INFLAMMABLE, TOXIC | 3 | TF1 | I | 3+6.1+(N1, N2, N3, CMR, F) | C | * | * | * | * | 95 |  | 1 | No | T43) | II B4) | Yes | PP, EP, EX, TOX, A | 2 | 14; 27; *See flowchart |
| 3494 | PETROLEUM CRUDE OIL, ACID, INFLAMMABLE, TOXIC | 3 | TF1 | II | 3+6.1+(N1, N2, N3, CMR, F) | C | * | * | * | * | 95 |  | 2 | No | T43) | II B4) | Yes | PP, EP, EX, TOX, A | 2 | 14; 27; *See flowchart |
| 3494 | PETROLEUM CRUDE OIL, ACID, INFLAMMABLE, TOXIC | 3 | TF1 | III | 3+6.1+(N1, N2, N3, CMR, F) | C | * | * | * | * | 95 |  | 2 | No | T43) | II B4) | Yes | PP, EP, EX, TOX, A | 0 | 14; 27; *See flowchart |

## Annex 2

# Amendments to Table c

Amendments and additions in respect of columns (9), (10), (11), (12) and (13) of Table A resulting from the application of existing criteria. Deletions are struck through and additions are underlined. The amendments in columns (9) and (12) should be reproduced in columns (18) and (19) of the corresponding entries in Table C.

| UN No. or ID No. | Name and description | Class | Classification code | Packing group | Labels | Special provisions | Limited and excepted quantities |  | Carriage permitted | Equipment required | Ventilation | Provisions concerning loading, unloading and carriage |  |  | Number of blue cones/lights | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.1.2 | 2.2 | 2.2 | 2.1.1.3 | 5.2.2 | 3.3 | 3.4.6 | 3.5.1.2 | 3.2.1 | 8.1.5 | 7.1.6 | 7.1.6 |  |  | 7.1.5 | 3.2.1 |
| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7a) | (7b) | (8) | (9) | (10) | (11) |  |  | (12) | (13) |
| 0154 | TRINITROPHENOL (PICRIC ACID), dry or wetted with less than 30% water, by mass | 1 | 1.1D |  | 1 |  | LQ0 | E0 |  | PP |  | LO01 | HA01, HA02, HA03, HA04, HA05, HA06 |  | 13 |  |
| 1391 | ALKALI METAL DISPERSION or ALKALINE EARTH METAL DISPERSION having a flashpoint of not more than 60 C | 4.3 | WF1 | I | 4.3 3 | 182 183 274 506 | LQ0 | E0 |  | PP, EX, A | VE01 |  | HA08 |  | 01 |  |
| 1431 | SODIUM METHYLATE | 4.2 | SC4 | II | 4.2+8 |  | LQ0 | E2 |  | PP, EP |  |  |  |  | 0 |  |
| 1463 | CHROMIUM TRIOXIDE, ANHYDROUS | 5.1 | OTC | II | 5.1+6.1+8 | 510 | LQ11 | E2 |  | PP |  |  |  |  | 02 |  |
| 1779 | FORMIC ACID with more than 85% acid by mass | 8 | CF1 | II | 8+3 |  | LQ22 | E2 | T | PP, EP, EX, A | VE01 |  |  |  | 01 |  |

| UN No. or ID No. | Name and description | Class | Classification code | Packing group | Labels | Special provisions | Limited and excepted quantities |  | Carriage permitted | Equipment required | Ventilation | Provisions concerning loading, unloading and carriage |  |  | Number of blue cones/lights | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1942 | AMMONIUM NITRATE with not more than 0.2% total combustible material, including any organic substance calculated as carbon, to the exclusion of any other added substance | 5.1 | O2 | III | 5.1 | 306 611 | LQ12 | E1 | B | PP |  | ST01, CO02, LO04 | HA09 |  | 0 | CO02, LO04 and HA09 apply only when this substance is carried in bulk or without packaging |
| 1950 | AEROSOLS, flammable, toxic | 2 | 5FC |  | 2.1+8 | 190 327 625 | LQ2 | E0 |  | PP, EP, EX, A | VE01, VE04 |  |  |  | 1 |  |
| 2235 | CHLOROBENZYL CHLORIDES, LIQUID | 6.1 | T1 | III | 6.1 | 802 | LQ7 | E1 |  | PP, EP, TOX, A | VE02 |  |  |  | 0 |  |
| 2236 | 3-CHLORO-4-METHYLPHENYL ISOCYANATE, LIQUID | 6.1 | T1 | II | 6.1 | 802 | LQ17 | E4 |  | PP, EP, TOX, A | VE02 |  |  |  | 2 |  |
| 2441 | TITANIUM TRICHLORIDE, PYROPHORIC or TITANIUM TRICHLORIDE MIXTURE, PYROPHORIC | 4.2 | SC4 | I | 4.2+8 | 537 | LQ0 | E0 |  | PP, EP |  |  |  |  | 0 |  |
| 2841 | DI-n-AMYLAMINE | 3 | FT1 | III | 3+6.1 | 802 | LQ7 | E1 |  | PP, EP, EX, TOX, A | VE01, VE02 |  |  |  | 20 |  |
| 3126 | SELF-HEATING SOLID, CORROSIVE, ORGANIC, N.O.S. | 4.2 | SC2 | II | 4.2+8 | 274 | LQ0 | E2 |  | PP, EP |  |  |  |  | 0 |  |
| 3126 | SELF-HEATING SOLID, CORROSIVE, ORGANIC, N.O.S. | 4.2 | SC2 | III | 4.2+8 | 274 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3128 | SELF-HEATING SOLID, TOXIC, ORGANIC, N.O.S. | 4.2 | ST2 | II | 4.2+6.1 | 274 802 | LQ0 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 3128 | SELF-HEATING SOLID, TOXIC, ORGANIC, N.O.S. | 4.2 | ST2 | III | 4.2+6.1 | 274 802 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3132 | WATER-REACTIVE SOLID, FLAMMABLE, N.O.S. | 4.3 | WF2 | I | 4.3+4.1 | 274 | LQ0 | E0 |  | PP, EX, A | VE01 |  | HA08 |  | 1 |  |
| 3132 | WATER-REACTIVE SOLID, FLAMMABLE, N.O.S. | 4.3 | WF2 | II | 4.3+4.1 | 274 | LQ11 | E2 |  | PP, EX, A | VE01 |  | HA08 |  | 1 |  |
| 3132 | WATER-REACTIVE SOLID, FLAMMABLE, N.O.S. | 4.3 | WF2 | III | 4.3+4.1 | 274 | LQ12 | E1 |  | PP, EX, A | VE01 |  | HA08 |  | 0 |  |
| 3135 | WATER-REACTIVE SOLID, SELF-HEATING, N.O.S. | 4.3 | WS | I | 4.3+4.2 | 274 | LQ0 | E0 |  | PP, EX, A | VE01 |  | HA08 |  | 0 |  |
| 3135 | WATER-REACTIVE SOLID, SELF-HEATING, N.O.S. | 4.3 | WS | II | 4.3+4.2 | 274 | LQ11 | E2 |  | PP, EX, A | VE01 |  | HA08 |  | 0 |  |
| 3135 | WATER-REACTIVE SOLID, SELF-HEATING, N.O.S. | 4.3 | WS | III | 4.3+4.2 | 274 | LQ12 | E1 |  | PP, EX, A | VE01 |  | HA08 |  | 0 |  |
| 3176 | FLAMMABLE SOLID, ORGANIC, MOLTEN, N.O.S. | 4.1 | F2 | II | 4.1 | 274 | LQ0 | E0 |  | PP |  |  |  |  | 01 |  |
| 3191 | SELF-HEATING SOLID, TOXIC, INORGANIC, N.O.S. | 4.2 | ST4 | II | 4.2+6.1 | 274 802 | LQ0 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 3191 | SELF-HEATING SOLID, TOXIC, INORGANIC, N.O.S. | 4.2 | ST4 | III | 4.2+6.1 | 274 802 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3192 | SELF-HEATING SOLID, CORROSIVE, INORGANIC, N.O.S. | 4.2 | SC4 | II | 4.2+8 | 274 | LQ0 | E2 |  | PP, EP |  |  |  |  | 0 |  |
| 3192 | SELF-HEATING SOLID, CORROSIVE, INORGANIC, N.O.S. | 4.2 | SC4 | III | 4.2+8 | 274 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3206 | ALKALI METAL ALCOHOLATES, SELF-HEATING, CORROSIVE N.O.S. | 4.2 | SC4 | II | 4.2+8 | 182 274 | LQ0 | E2 |  | PP, EP |  |  |  |  | 0 |  |
| 3206 | ALKALI METAL ALCOHOLATES, SELF-HEATING, CORROSIVE N.O.S. | 4.2 | SC4 | III | 4.2+8 | 183 274 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3396 | ORGANOMETALLIC SUBSTANCE, SOLID, WATER REACTIVE, FLAMMABLE | 4.3 | WF2 | I | 4.3+4.1 | 274 | LQ0 | E0 |  | PP, EX, A | VE01 |  | HA08 |  | 1 |  |
| 3396 | ORGANOMETALLIC SUBSTANCE, SOLID, WATER REACTIVE, FLAMMABLE | 4.3 | WF2 | II | 4.3+4.1 | 274 | LQ11 | E2 |  | PP, EX, A | VE01 |  | HA08 |  | 1 |  |
| 3396 | ORGANOMETALLIC SUBSTANCE, SOLID, WATER REACTIVE, FLAMMABLE | 4.3 | WF2 | III | 4.3+4.1 | 274 | LQ12 | E1 |  | PP, EX, A | VE01 |  | HA08 |  | 0 |  |
| 3408 | LEAD PERCHLORATE SOLUTION | 5.1 | OT1 | II | 5.1+6.1 |  | LQ10 | E2 |  | PP, EP | VE02 |  |  |  | 02 |  |
| 3409 | CHLORONITROBENZENES, LIQUID | 6.1 | T1 | II | 6.1 | 279 802 | LQ17 | E4 |  | PP, EP, TOX, A | VE02 |  |  |  | 2 |  |
| 3463 | PROPIONIC ACID with not less than 90% acid by mass | 8 | CF1 | II | 8+3 |  | LQ22 | E2 | T | PP, EP, EX, A | VE01 |  |  |  | 01 |  |
| 3470 | PAINT, CORROSIVE, FLAMMABLE (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer base) or PAINT RELATED MATERIAL CORROSIVE, FLAMMABLE (including paint thinning or reducing compound) | 8 | CF1 | II | 8+3 | 163 | LQ22 | E2 |  | PP, EP, EX, A | VE01 |  |  |  | 01 |  |
| 3471 | HYDROGENDIFLUORIDES SOLUTION, N.O.S. | 8 | CT1 | II | 8+6.1 |  | LQ22 | E2 |  | PP, EP |  |  |  |  | 02 |  |
| 3473 | FUEL CELL CARTRIDGES or FUEL CELL CARTRIDGES CONTAINED IN EQUIPMENT or FUEL CELL CARTRIDGES PACKED WITH EQUIPMENT containing flammable liquids | 3 | F1 |  | 3 | 328 | LQ13 | E0 |  | PP, EX, A | VE01 |  |  |  |  |  |
| 3477 | FUEL CELL CARTRIDGES or FUEL CELL CARTRIDGES CONTAINED IN EQUIPMENT or FUEL CELL CARTRIDGES PACKED WITH EQUIPMENT, containing corrosive substances | 8 | C11 |  | 8 | 328 334 | LQ12 LQ13 | E0 |  | PP, EX, EP, A |  |  |  |  | 0 |  |
| 3478 | FUEL CELL CARTRIDGES or FUEL CELL CARTRIDGES CONTAINED IN EQUIPMENT or FUEL CELL CARTRIDGES PACKED WITH EQUIPMENT, containing liquefied flammable gas | 2 | 6F |  | 2.1 | 328 338 | LQ1 | E0 |  | PP, EX, A | VE01 |  |  |  | 01 |  |
| 9000 | AMMONIA, DEEPLY REFRIGERATED | 2 | 3TC |  | 2.3+8 |  |  | E0 | T | PP, EP, TOX, A | VE02 |  |  |  | 2 | Admitted only for carriage in tank vessels |

## Annex 3

# Additions TO column (9)

Additions to column (9) resulting from the application of expanded criteria. The insertions are underlined. The additions to column (9) should be reproduced in column (18) of the corresponding entries in Table C.

| UN No. or ID No. | Name and description | Class | Classification code | Packing group | Labels | Special provisions | Limited and excepted quantities |  | Carriage permitted | Equipment required | Ventilation | Provisions concerning loading, unloading and carriage |  |  | Number of blue cones/lights | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.1.2 | 2.2 | 2.2 | 2.1.1.3 | 5.2.2 | 3.3 | 3.4.6 | 3.5.1.2 | 3.2.1 | 8.1.5 | 7.1.6 | 7.1.6 |  |  | 7.1.5 | 3.2.1 |
| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7a) | (7b) | (8) | (9) | (10) | (11) |  |  | (12) | (13) |
| 1320 | DINITROPHENOL, WETTED with not less than 15% water, by mass | 4.1 | DT | I | 4.1+6.1 | 802 | LQ0 | E0 |  | PP, EP |  |  |  |  | 2 |  |
| 1321 | DINITROPHENOLATES WETTED with not less than 15% water, by mass | 4.1 | DT | I | 4.1+6.1 | 802 | LQ0 | E0 |  | PP, EP |  |  |  |  | 2 |  |
| 1348 | SODIUM DINITRO-o-CRESOLATE, WETTED with not less than 15% water, by mass | 4.1 | DT | I | 4.1+6.1 | 802 | LQ0 | E0 |  | PP, EP |  |  |  |  | 2 |  |
| 1445 | BARIUM CHLORATE, SOLID | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1446 | BARIUM NITRATE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1447 | BARIUM PERCHLORATE, SOLID | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1448 | BARIUM PERMANGANATE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1449 | BARIUM PEROXIDE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1469 | LEAD NITRATE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1470 | LEAD PERCHLORATE, SOLID | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1500 | SODIUM NITRITE | 5.1 | OT2 | III | 5.1+6.1 | 802 | LQ12 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 1511 | UREA HYDROGEN PEROXIDE | 5.1 | OC2 | III | 5.1+8 |  | LQ12 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 1571 | BARIUM AZIDE, WETTED with not less than 50% water, by mass | 4.1 | DT | I | 4.1+6.1 | 568 802 | LQ0 | E0 |  | PP, EP |  |  |  |  | 2 |  |
| 1868 | DECABORANE | 4.1 | FT2 | II | 4.1+6.1 | 802 | LQ0 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 1872 | LEAD DIOXIDE | 5.1 | OT2 | III | 5.1+6.1 | 802 | LQ12 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 2464 | BERYLLIUM NITRATE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 2573 | THALLIUM CHLORATE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 2719 | BARIUM BROMATE | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 2741 | BARIUM HYPOCHLORITE with more than 22% available chlorine | 5.1 | OT2 | II | 5.1+6.1 | 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 2925 | FLAMMABLE SOLID, CORROSIVE, ORGANIC, N.O.S. | 4.1 | FC1 | II | 4.1+8 | 274 | LQ0 | E2 |  | PP, EP |  |  |  |  | 1 |  |
| 2925 | FLAMMABLE SOLID, CORROSIVE, ORGANIC, N.O.S. | 4.1 | FC1 | III | 4.1+8 | 274 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 2926 | FLAMMABLE SOLID, TOXIC, ORGANIC, N.O.S. | 4.1 | FT1 | II | 4.1+6.1 | 274 802 | LQ0 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 2926 | FLAMMABLE SOLID, TOXIC, ORGANIC, N.O.S. | 4.1 | FT1 | III | 4.1+6.1 | 274 802 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3085 | OXIDIZING SOLID, CORROSIVE, N.O.S. | 5.1 | OC2 | I | 5.1+8 | 274 | LQ0 | E0 |  | PP, EP |  |  |  |  | 0 |  |
| 3085 | OXIDIZING SOLID, CORROSIVE, N.O.S. | 5.1 | OC2 | II | 5.1+8 | 274 | LQ11 | E2 |  | PP, EP |  |  |  |  | 0 |  |
| 3085 | OXIDIZING SOLID, CORROSIVE, N.O.S. | 5.1 | OC2 | III | 5.1+8 | 274 | LQ12 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3087 | OXIDIZING SOLID, TOXIC, N.O.S. | 5.1 | OT2 | I | 5.1+6.1 | 274 802 | LQ0 | E0 |  | PP, EP |  |  |  |  | 2 |  |
| 3087 | OXIDIZING SOLID, TOXIC, N.O.S. | 5.1 | OT2 | II | 5.1+6.1 | 274 802 | LQ11 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 3087 | OXIDIZING SOLID, TOXIC, N.O.S. | 5.1 | OT2 | III | 5.1+6.1 | 274 802 | LQ12 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3134 | WATER-REACTIVE SOLID, TOXIC, N.O.S. | 4.3 | WT2 | I | 4.3+6.1 | 274 802 | LQ0 | E0 |  | PP, EP, EX, TOX, A | VE01 |  | HA08 |  | 2 |  |
| 3134 | WATER-REACTIVE SOLID, TOXIC, N.O.S. | 4.3 | WT2 | II | 4.3+6.1 | 274 802 | LQ11 | E2 |  | PP, EP, EX, TOX, A | VE01 |  | HA08 |  | 2 |  |
| 3134 | WATER-REACTIVE SOLID, TOXIC, N.O.S. | 4.3 | WT2 | III | 4.3+6.1 | 274 802 | LQ12 | E1 |  | PP, EP, EX, TOX, A | VE01 |  | HA08 |  | 0 |  |
| 3179 | FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. | 4.1 | FT2 | II | 4.1+6.1 | 274 802 | LQ0 | E2 |  | PP, EP |  |  |  |  | 2 |  |
| 3179 | FLAMMABLE SOLID, TOXIC, INORGANIC, N.O.S. | 4.1 | FT2 | III | 4.1+6.1 | 274 802 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3180 | FLAMMABLE SOLID, CORROSIVE, INORGANIC, N.O.S. | 4.1 | FC2 | II | 4.1+8 | 274 | LQ0 | E2 |  | PP, EP |  |  |  |  | 1 |  |
| 3180 | FLAMMABLE SOLID, CORROSIVE, INORGANIC, N.O.S. | 4.1 | FC2 | III | 4.1+8 | 274 | LQ0 | E1 |  | PP, EP |  |  |  |  | 0 |  |
| 3369 | SODIUM DINITRO-o-CRESOLATE, WETTED with not less than 10% water, by mass | 4.1 | DT | I | 4.1+6.1 | 802 | LQ0 | E0 |  | PP, EP |  |  |  |  | 2 |  |
| 3408 | LEAD PERCHLORATE SOLUTION | 5.1 | OT1 | II | 5.1+6.1 |  | LQ10 | E2 |  | PP, EP | VE02 |  |  |  | 2 |  |
| 3408 | LEAD PERCHLORATE SOLUTION | 5.1 | OT1 | III | 5.1+6.1 |  | LQ13 | E1 |  | PP, EP | VE02 |  |  |  | 0 |  |

## Annex 4

-----

Additional corrections

| UN No. or ID No. | Name and description | Class | Classification code | Packing group | Labels | Special provisions | Limited and excepted quantities |  | Carriage permitted | Equipment required | Ventilation | Provisions concerning loading, unloading and carriage |  |  | Number of blue cones/lights | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3.1.2 | 2.2 | 2.2 | 2.1.1.3 | 5.2.2 | 3.3 | 3.4.6 | 3.5.1.2 | 3.2.1 | 8.1.5 | 7.1.6 | 7.1.6 |  |  | 7.1.5 | 3.2.1 |
| (1) | (2) | (3a) | (3b) | (4) | (5) | (6) | (7a) | (7b) | (8) | (9) | (10) | (11) |  |  | (12) | (13) |
| 1589 | CYANOGEN CHLORIDE, STABILIZED | 2 | 2TC |  | 2.3+8 +unst. |  | LQ0 | E0 |  | PP, EP, TOX, A | VE02 |  |  |  | 2 |  |
| 3471 | HYDROGENDIFLUORIDES SOLUTION, N.O.S. | 8 | CT1 | II | 8+6.1 | 802 | LQ22 | E2 |  | PP, EP |  |  |  |  | 0 |  |
| 3471 | HYDROGENDIFLUORIDES SOLUTION, N.O.S. | 8 | CT1 | III | 8+6.1 | 802 | LQ7 | E1 |  | PP, EP |  |  |  |  | 0 |  |

1  Distributed in German by the Central Commission for the Navigation of the Rhine under the symbol CCNR/ZKR/ADN/WP.15/AC.2/2009/27.

2  In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.7 (b)).

†  Translator’s note:  Despite extensive consultations with the Transport Division, the translator was unable to trace the phrase within quotation marks; the words used are the translator’s own.

††  Translator’s note:  See note to para. 14.