Draft proposal for changes to the text of  TRANS/WP.29/GRB/2005/2/Rev.2 (R51.03)

In order to incorporate the Additional Sound Emission Provisions

Issued by the Chairman of the ASEP group

Reflecting the discussions up to January 27, 2009

And including proposals by the chairman

Text in black is not changed compared to TRANS/WP.29/GRB/2005/2/Rev.2

Text crossed out has been removed

Text marked in green bold has been changed or added compared to TRANS/WP.29/GRB/2005/2/Rev.2 and has been agreed by the group

[Text in square brackets still has to be discussed]

Text in blue is proposed to be changed compared to the status of the last meeting and may need to be confirmed

Text marked in orange bold has been added for clarification to remind the group and or GRB of earlier discussions or to highlight action points

#### Main body

6.2.3.	Additional sound emission provisions

The additional sound emission provisions apply only to vehicles of categories M1 and N1 equipped with an internal combustion engine

The purpose of Annex 10 the additional sound emission provisions is to ensure the sound emission of the vehicle under normal driving conditions different from the conditions of the type approval test in Annex 3 shall not differ deviate considerably from what can be expected from the Annex 3 test result for this specific vehicle.

They are preventive requirements intended to also cover driving conditions of the vehicle in real traffic, which can be environmentally relevant concerning their sound emission and which differs from those during type approval, described in Annex 3.

6.2.3.1.	The vehicle manufacturer shall not intentionally alter, adjust, or introduce any mechanical, electrical, thermal, or other device or procedure solely for the purpose of fulfilling the noise emission requirements as specified in this regulation and as determined by the test procedure of Annex 3 but which will not be operational during typical on-road operation. These measures are commonly referred to as "cycle detection".

This shall not prevent the installation on a vehicle of any control device, function, system or measure if:

-	it is activated only for such purposes as engine protection, cold starting or warming up, or

-	it is activated only for such purposes as operational security or safety and limp-home strategies, or

- it is required to fulfil this and/or other regulations

6.2.3.2.	The vehicle shall meet the requirements of annex 10

[6.2.3.3.	The sound emission of the vehicle under normal driving conditions different from the conditions of the type approval test in Annex 3 shall not differ considerably from what can be expected from the type approval test result for this specific vehicle with regard to technical practicability.  This is fulfilled if the requirements of Annex 10 are met.]

6.2.3.3. 	In the application for type approval the manufacturer shall provide a statement (conform par 20 of Annex 1) that the vehicle type to be approved complies with the requirements of paragraph 6.2.3 of this Regulation. The type approval authority may require any relevant test to verify the compliance of the vehicle type to the additional sound emission provisions.

(Note: In addition the 58 Agreement says:

”The approval authority may carry out any check or test prescribed... [] ... in the applicable Regulation (ECE R 51) annexed to this Agreement.”

For more clarity this sentence is incorporated into the regulation 51)

8. 	CONFORMITY OF PRODUCTION

The conformity of production procedures shall comply with those set out in the Agreement, Appendix 2 (E/ECE/324-E/ECE/TRANS/505/Rev.2) with the following requirements:

8.1. 	Vehicles approved to this Regulation shall be so manufactured as to conform to the type approved by meeting the requirements set forth in paragraph 6. above.

8.2. 	The minimum requirements for conformity of production control procedures set forth in Annex 7 to this Regulation shall be complied with.

8.3. 	The authority which has granted type approval may at any time verify the conformity control methods applied in each production facility. The normal frequency of these verifications shall be one every two years. The type approval authority may carry out any check or test prescribed in this regulation to verify the compliance with the requirements of this regulation.

(Note: This paragraph formulates that any produced vehicle must comply to all requirements of paragraph 6 including ASEP. Annex 7 regulates, which measurements are part of a regular production control process. All produced vehicles have to comply with ASEP but need not always to be tested, unless a manufacturer decide to do so on a voluntary base. In addition the 58 Agreement says: ”The approval authority may carry out any check or test prescribed... [] ... in the applicable Regulation (ECE R 51) annexed to this Agreement.” For more clarity this sentence is incorporated into the regulation 51)

Annex 1

(Maximum format:  A4 (210 x 297 mm))

COMMUNICATION

20.	Statement of compliance with the Additional Sound Emission Provisions

………………(Name of manufacturer) attests that vehicles of this vehicle type comply with the requirements of paragraph 6.2.3 of Regulation number 51

……………… (Name of manufacturer) makes this statement in good faith, after having performed an appropriate engineering evaluation of the sound emission performance of the vehicles.

Date:

Name of authorized representative

Signature of authorized representative:

Annex 7

CHECKS ON CONFORMITY OF PRODUCTION

1.	General

These requirements are consistent with the test to be held to check conformity of production according to paragraph 8. of this Regulation.

2.	Testing procedure

The test site and measuring instruments shall be those as described in Annex 3.

2.1.	The vehicle(s) under test shall be subjected to the test for measurement of sound of vehicle in motion as described in paragraph 3.1. of Annex 3.

2.2.	Compressed air sound

Vehicles having maximum mass exceeding 2,800 kg and equipped with compressed air systems must be subjected to an additional test for measurement of the compressed air sound as described in paragraph 1. of Annex 6.

2.3.	At the request of the type approval authority, the vehicle(s) under test may be subjected to the test for Additional Sound Emission Provisions as described in Annex 10.

3.	Sampling

One vehicle has to be chosen.  If after the test of paragraph 4.1. the vehicle is not considered to conform to the requirements of this Regulation, two more vehicles have to be tested.

4.	Evaluation of the results

4.1.	If the sound level of the vehicle tested pursuant to paragraphs 1. and 2. does not exceed by more than 1 dB(A) the limit value prescribed in

-	paragraph 6.2.2. of this Regulation, for measurement according to paragraph 2.1. above, and in

-	paragraph 3. of Annex 6 to this Regulation, for measurement according to paragraph 2.2. above, and in

-	paragraph 5 of Annex 10 to this Regulation, for measurements according to paragraph 2.3. above,

the vehicle type shall be considered to conform to the requirements of this Regulation.

4.2.	If the vehicle tested according to paragraph 4.1. does not satisfy the requirements laid down in that paragraph, two more vehicles of the same type shall be tested pursuant to paragraphs 1. and 2.

4.3.	If the sound level of the second and/or third vehicle of paragraph 4.2. exceeds by more than 1 dB(A) the limit values prescribed in paragraph 6.2.2. of this Regulation 4.1 above, the vehicle type shall be considered not to conform to the requirements of this Regulation and the manufacturer shall take the necessary measures to re-establish the conformity.

#### Annex 10

MEASURING METHOD TO EVALUATE THE COMPLIANCE WITH THE

ADDITIONAL SOUND EMISSION PROVISIONS

only applicable for vehicles of categories M1 and N1

which are equipped with an internal combustion engine

During development of the provisions for off cycle emissions the technical provisions of UNECE Regulation No. 51.02 series of amendments shall be fulfilled.]

Note:	this annex has been completely reworked compared to earlier versions. Blocks of text have been used from various earlier documents, but not in all cases 1 to 1 copied. In some cases the compromise proposal of the chairman has been given, following the discussions in the Working Group.

1.	GENERAL

This annex describes a measuring method to evaluate the compliance of the vehicle with the additional sound emission provisions conform paragraph 6.2.3. of this regulation. The expected noise emission is a linear function of engine speed or vehicle speed. The anchor point of this linear function is based on the test results of annex 3 and on the gear ratio under test compared to the gear ratio in annex 3. The slope of this linear function is based on actual test results and limited to a maximum value. The individual noise measurements shall not exceed the expected noise emission by more than the allowed limit.

Although the vehicle shall meet the requirements in this annex, it is not obligatory to perform actual tests when applying for type approval. It is obligatory for the manufacturer to sign a declaration of compliance conform annex 1. The type approval authority shall have the possibility to ask for additional technical information in order to check the compliance of the vehicle with the requirements in paragraph 8 and/or carry out the tests as described below.

2.	REQUIREMENTS

The measured noise emission of the vehicle (conform par 3 of this annex) shall not exceed the expected noise emission (conform par 4 of this annex) by more than the limit as given in paragraph 5.

3.	MEASURING METHOD

3.1	measuring instruments and condition of measurements

Unless specified differently here after, the measuring instruments, the conditions of the measurements and the condition of the vehicle are equal to those specified in Annex 3 paragraph 1 and 2.

Note:	In the following sub-paragraphs readings are assumed to be taken at line BB’. Currently  it is still under discussion whether readings should be taken at PP’, BB’ or at Lmax. In the latter case more sophisticated (continuous) measuring equipment is necessary, both for annex 3 and annex 10 measurements as compared to the current annex 3 procedure. See list of decision to be made at the end of this document.

Some vehicles may have different software programs or modes which affect the acceleration behavior of the vehicle. Such programs may include, but are not limited to, the transmission (e.g. sporty, winter, adaptive, locked, unlocked, etc.), the electronic stability program (on/off) and the intelligent cruise control (on/off). If the vehicle has different modes leading to valid accelerations, all these modes shall be in compliance with the requirements in this annex.

3.2.	Method of testing

Unless specified differently here after, the conditions and procedures of Annex 3 paragraph 3.1 until 3.1.2.1.2.2. have to be used.

3.3	Boundary conditions

There is a range of valid operation conditions which have to fall within the following boundary conditions:

Vehicle speed VBB_ASEP:			[20 ≤ vBB ≤ 80 km/h]

Vehicle acceleration aWOT ASEP:		[aWOT ≤ 5,0 m/s2]

Engine speed nBB_ASEP:			[nBB ≤ 0.9*(s- nidle) + nidle]

Note:	(OICA) Proposals for boundary conditions have been made, but there was no detailed discussion on this topic up to now. As a general goal OICA aims for making the requirements as design neutral as possible.)

See list of decision to be made at the end of this document.

3.4	Gear ratios

The tests shall be run in every gear ratio k that leads to test results within the boundary conditions as defined in paragraph 3.3 in this Annex.

In case of vehicles with automatic transmissions, adaptive transmissions and CVT’s tested with non-locked gear ratios, the test may include a gear ratio change to a lower range and a higher acceleration. A gear change to a higher range and a lower acceleration is not allowed. A gear shifting which leads to a condition that is not in compliance with the boundary conditions shall be avoided. In that case, it is permitted to establish and use electronic or mechanical devices, including alternate gear selector positions, to prevent a downshift to a gear ratio which leads to an acceleration higher than the boundary condition.

3.5	Target conditions

Four valid measurements shall be carried out in each valid gear ratio at the test points as specified below.

The first test point P1 is defined by using an entry speed vAA of 20 km/h. If a stable acceleration condition cannot be achieved the speed shall be increased in steps of 5 km/h until a stable acceleration is reached.

The fourth test point P4 is defined by the maximum vehicle speed at BB’ in that gear ratio within the boundary conditions according to paragraph 3.3

The other two test points are defined by the following formula:

Test Point Pj : 	vBB_j = vBB_1 + (j-1)/3 * (vBB_max - vBB_1) 	for j = 2 and 3

where	vBB_1 	=	vehicle speed at BB’ of test point P1

vBB_max 	=	maximum vehicle speed in this gear ratio taking into account the boundary conditions in paragraph 3.3

Tolerance for vBB_j: +/- [3] km/h

For all test points the boundary conditions as specified in 3.3 shall be met.

3.6.	Test of the vehicle

The path of the centerline of the vehicle shall follow line CC' as closely as possible throughout the entire test, from the approach to line AA' until the rear of the vehicle passes line BB'.

At the latest at line AA’ the accelerator shall be fully depressed. To achieve a more stable acceleration or to avoid a down shift between line AA’ and BB’ pre-acceleration before line AA’ may be used. The accelerator shall be kept in depressed condition until  the rear of the vehicle reaches line BB'.

For every separate test run the following parameters shall be determined and noted:

    - The maximum A-weighted sound pressure level indicated during each passage of the vehicle between the two lines AA' and BB', mathematically rounded to the first decimal place. (Lwot,kj). If a sound peak obviously out of character with the general sound pressure level is observed, the measurement shall be discarded. Left and right side may be measured simultaneously or separately. The results of each side shall be evaluated separately.

    - The vehicle speed readings at AA' and BB' shall be reported with the first significant digit after the decimal place. (vAA,kj; vBB,kj)

    - If applicable, the engine speed readings at AA' and BB' shall be reported as a full integer value. (nAA,kj; nBB,kj)

    - The calculated acceleration shall be determined in accordance to the formulas in annex 3 par 3.1.2.1.2 and reported to the second digit after the decimal place (awot,test,kj).

4.	Analysis of results

In this paragraph an analysis is made of the noise emission as function of engine- or vehicle speed. The expected noise emission is a linear function of speed. This linear function is constructed as a slope out of an anchor point. The anchor point is based on values from annex 3, the slope is based on values from annex 10.

4.1.	Determination of the anchor point for each gear ratio

For measurements in gear ratio i the anchor point consists of the maximum sound level Lwoti, the reported engine speed nwoti and vehicle speed vwoti at BB’ of gear ratio i of the acceleration test in Annex 3.

Lanchor,i = Lwoti,annex 3

nanchor,i = nBB,woti, annex 3

vanchor,i = vBB,woti, annex 3

For any gear ratio k different from gear ratio i a tyre rolling sound correction is needed for the sound level Lanchor,k. For this correction the cruise by level Lcruise from annex 3 in gear i or i+1 shall be used, which ever is the lowest.

The sound level of the anchor point in gear k is calculated by:

Lanchor,k = 10 *LOG(10(Lwoti/10) - 10(Lcruise/10) + 10((32*LOG(ik/ii)+Lcruise)/10))

where 	ik 		= gear ratio of the tested gear k in kmh/1000rpm

ii 		= gear ratio of gear i in kmh/1000rpm

If the vehicle is evaluated as function of engine speed, the engine speed of the anchor point is identical in all gear ratios.

nanchor,k = nanchor,i = nBB,woti, annex 3

If the vehicle is evaluated as function of vehicle speed, the vehicle speed of the anchor point is identical in all gear ratios.

vanchor,k = vanchor,i = vBB,woti, annex 3

4.2	Calculation of the slope of the regression line for each gear

In principle the noise measurements have to be evaluated as function of engine speed according to 4.2.1, however if the four measurements according to 3.5 result in engine speeds that are within a range of ± 10% of nanchor,i , the noise measurements shall not be evaluated as function of engine speed, but as function of vehicle speed according to 4.2.2

4.2.1	Calculation of the regression line as function of engine speed

The linear regression line is calculated using the anchor point and the four correlated additional measurements.

Slopek=j=15(nj−n)(Li−L)j=15(nj−n)2 (in dB/1000 rpm)

With	L=15j=15Lj ;

n=15j=15nj ;

where nj = engine speed measured at line BB’

4.2.2	Calculation of the regression line as function of vehicle speed

The linear regression line is calculated using the anchor point(s) and the four correlated additional measurements.

Slopek=j=15(nj−n)(Li−L)j=15(nj−n)2 (in dB/10 kmh)

With	L=15j=15Lj ;

v=15j=15vj ;

where vj = vehicle speed measured at line BB’

4.2.3	Slope for further calculation

The slope for the further calculation is the derived result of the calculation formulas under 4.2.1 or 4.2.2 rounded to the first decimal place, unless the below given borders are exceeded.

For calculation as function of engine speed: [5] dB/1000 rpm

For calculation as function of vehicle speed: [3] dB/ 10 kmh

4.3	Calculation of the linear noise level increase expected for each measurement

The expected sound level Lexpected,kj for measurement point j in gear k shall be calculated using the engine speeds - respective vehicle speeds - measured for each measurement point, using the Slope determined in 4.2 relative to the specific anchor point for each gear ratio.

Lexpected_k,j = Lanchor_k + Slopek * (nBB_k,j - nanchor,k)/1000, or

Lexpected_k.j = Lanchor_k + Slopek * (vBB_k,j - vanchor,k)/10

5.	Interpretation of results

Every individual noise measurement shall be evaluated.

The Requirements of Annex 10 are fulfilled if the difference between the measured sound level and the affiliated expected sound level for every measured point is lower than the limit.

Lkj – Lexpeted_k.j ≤ LimitAnnex 10

In which

LimitAnnex 10= 1 + allowance

The allowance is the difference between the type approval limit for this vehicle according to par 6.2.2 of this regulation and the urban noise level according to annex 3

Allowance = LimitAnnex 3 - Lurban,Annex 3

If the measured noise level in a point exceeds the allowable margin, two additional measurements at the same point shall be carried out to verify the measurement uncertainty. The vehicle is still in compliance with ASEP, if the average of the three valid measurements at this specific point fulfills the above specification.

Decisions to be made:

- Is it sufficiently accurate to take (engine)speed measurements at BB’ or is it necessary to take measurements at Lmax.

- Consequently: are continuous measuring devices necessary or not

- Boundary conditions