Proposal for draft Corrigendum 1 to Supplement 33 to the 03 series of amendments to Regulation No. 37 (Filament lamps)

(ECE/TRANS/WP.29/2009/18)

Annex 1,

The list of categories of filament lamps and their sheets, amend to read:

"

| Group 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Only for use in signalling lamps, cornering lamps, reversing lamps and rear registration plate lamps: |  |  |  |  |
|  | Category |  | Sheet number(s) |  |
|  | C5W |  | C5W/1 |  |
|  | H6W |  | H6W/1 |  |
|  | H10W/1 |  | H10W/1 to 2 |  |
|  | HY6W |  | H6W/1 |  |
|  | HY10W |  | H10W/1 to 2 |  |
|  | HY21W |  | H21W/1 to 2 |  |
|  | ... |  | … |  |

"

The new sheets H10W/1 to 2, between sheet H6W/1 and sheet H21W/1, amend to read (see next pages):

"	CATEGORIES H10W/1 AND HY10W	Sheet H10W/1

The drawings are intended only to illustrate the essential dimensions (in mm) of the filament lamp

β

γ1

γ2

Reference axis

Reference plane

e

28.0 max.

Ø 9.25 max.

4/, 5/

X

X

HY10W

3/

H10W/1

| Dimensions in mm |  |  |  |  | Filament lamps of normal production |  |  |  | Standard filament lamp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | min. |  | nom. | max. |  |
| e |  |  |  |  | 14.25 |  | 15.0 | 15.75 | 15.0  0.25 |
| Lateral deviation 1/ |  |  |  |  |  |  |  | 0.75 | 0.4 max |
| ß |  |  |  |  | 82.5° |  | 90° | 97.5° | 90°  5° |
| 1, 2 2/ |  |  |  |  | 30 ° |  |  |  | 30° min. |
| Cap: | H10W/1: |  | BAU9s |  | in accordance with IEC Publication 60061 (sheet 7004-[...]) |  |  |  |  |
|  | HY10W |  | BAUZ9s |  | in accordance with IEC Publication 60061 (sheet 7004-[...]) |  |  |  |  |
| ELECTRICAL AND PHOTOMETRIC CHARACTERISTICS |  |  |  |  |  |  |  |  |  |
| Rated values |  | Volts |  |  |  | 12 |  |  | 12 |
|  |  | Watts |  |  |  | 10 |  |  | 10 |
| Test voltage |  | Volts |  |  |  | 13.5 |  |  | 13.5 |
| Objective values |  | Watts |  |  |  | 12 max. |  |  | 12 max. |
|  |  | Luminous flux |  | H10W/1 |  | 200  12 % |  |  |  |
|  |  |  |  | HY10W |  | 120  17 % |  |  |  |
| Reference luminous flux at approximately 13.5 V |  |  |  |  |  |  |  |  | White: 200 lm |
|  |  |  |  |  |  |  |  |  | Amber: 120 lm |

CATEGORIES H10W/1 AND HY10W	Sheet H10W/2

1/	Maximum lateral deviation of filament centre from two mutually perpendicular planes both containing the reference axis and one containing axis X-X.

2/	In the area between the outer legs of the angles 1 and 2, the bulb shall have no optically distorting areas and the curvature of the bulb shall have a radius not less than 50 % of the actual bulb diameter.

3/	Over the entire length of the cap there shall be no projections or soldering exceeding the permissible maximum diameter of the cap.

4/	The light emitted from filament lamps of normal production shall be white for category H10W/1 and amber for category HY10W.

5/	The light emitted from standard filament lamps shall be white for category H10W/1 and amber or white for category HY10W. "

B.	JUSTIFICATION

The proposal for Supplement 33 to the 03 series of amendments to Regulation No. 37, was adopted by GRE at its last October 2008 session.  However, it has been investigated that the category H10W already exists in IEC60983 standard but with different technical specifications.  Accordingly, H10W category should be replaced by H10W/1 throughout the text.

- - - - -