- ANNEX A 1/2 - Fuel Quality limits – OICA proposal on GRPE two-step approach:

| Gasoline parameters | R83.03 (Euro 2) | R83.05 (row A) (Euro 3) | R83.05 (row B) (Euro 4) | Test method |
| --- | --- | --- | --- | --- |
| Step 1 parameters |  |  |  |  |
| Lead [g/l] | ≤ 0,013 | ≤ 0,005 | ≤ 0,005 | EN 237 |
| Sulphur [mg/kg] | ≤ 500 | ≤ 150 | ≤10 | EN ISO 20846 EN ISO 20884 |
| Metal Additives [mg/l] | ----------------- Not permitted ----------------- |  |  | ICP |
| Oxygen % [m/m] | ≤ 2,7 | ≤ 2,7 | ≤ 2,7 | EN 1601 EN 13132 |
| Oxygenates % [v/v] - methanol - ethanol - iso-propyl alcohol - iso-butyl alcohol - tert-butyl alcohol - ethers - other oxygenates | ≤ 3,0 ≤ 5,0 ≤ 10,0 ≤ 10,0 ≤ 7,0 ≤ 15,0 ≤ 10,0 | ≤ 3,0 ≤ 5,0 ≤ 10,0 ≤ 10,0 ≤ 7,0 ≤ 15,0 ≤ 10,0 | ≤ 3,0 ≤ 5,0 ≤ 10,0 ≤ 10,0 ≤ 7,0 ≤ 15,0 ≤ 10,0 | EN 1601 EN 13132 |
| RVP [kPa] | 35 - 100 | 45 – 100 | 45 – 100 | EN 13016/l DVPE |
| Density [kg/m3] | 725 - 780 | 720 – 7753 | 720 – 7753 | EN ISO 3675 EN ISO 12185 |
| RON [-] | ≥ 953 | ≥ 953 | ≥ 953 | EN ISO 5164 |
| MON [-] | ≥ 853 | ≥ 853 | ≥ 853 | EN ISO 5163 |
| Step 2 parameters |  |  |  |  |
| Benzene % [v/v] | ≤ 5 | ≤ 1 | ≤ 1 | EN 238 EN 14517 |
| Aromatics % [v/v] | - | ≤ 42 | ≤ 35 | EN 14517 & EN15553 |
| Olefins % [v/v] | - | ≤ 21 & 18 | ≤ 18 | EN 14517 & EN15553 |
| VLI (10VP + E70) | - | 1050 – 1250 | 1050 - 1250 |  |
| FBP [°C] | <215 | <210 | <210 | EN ISO 3405 |
| E70 % [v/v] | 15 - 47 | 20 – 50 | 20 – 50 | EN ISO 3405 |
| E100 % [v/v] | 40 - 70 | 46 – 71 | 46 – 71 | EN ISO 3405 |
| E150 % [v/v] | ≥ 75 | ≥ 75 | ≥ 75 | EN ISO 3405 |
| E180 % [v/v] | ≥ 85 | ≥ 85 | ≥ 85 | EN ISO 3405 |
| Residue % [v/v ] | < 2 | <2 | <2 | EN ISO 3405 |

- ANNEX A 2/2 - Fuel Quality limits – OICA proposal on GRPE two-step approach:

| Diesel parameters | R83.03 (Euro 2) | R83.05 (row A) (Euro 3) | R83.05 (row B) (Euro 4) |  | Test method |
| --- | --- | --- | --- | --- | --- |
| Step 1 parameters |  |  |  |  |  |
| Sulphur [mg/kg] | ≤ 500 | ≤ 350 |  | ≤ 10 | EN ISO 20846 EN ISO 20884 |
| Ash % [m/m] | ≤ 0,01 | ≤ 0,01 |  | ≤ 0,01 | EN/ISO 6245 |
| Total Contamination [mg/kg] | ≤ 24 | ≤ 24 |  | ≤ 24 | EN 12662 |
| Cetane Number | ≥ 49 | ≥ 515 |  | ≥ 515 | EN ISO 5165 |
| Cetane Index | ≥ 465 | ≥ 465 |  | ≥465 | EN ISO 4264 |
| Density [kg/m3] | 820 – 8605 | 820 – 8455 |  | 820 – 8455 | EN ISO 3675 EN ISO 12185 |
| Viscosity [mm2/s] | 2,0 - 4,55 | 2,0 - 4,55 |  | 2,0 – 4,55 | EN ISO 3104 |
| Flash Point [°C] | > 555 | > 555 |  | > 555 | EN ISO 2719 |
| Step 2 parameters |  |  |  |  |  |
| T50 [°C] | Report | T65 = 250 min |  | T65 = 250 min | EN ISO 3405 |
| T85 [°C] | ≤ 350 | ≤ 350 |  | ≤ 350 | EN ISO 3405 |
| T95 [°C] | ≤ 370 | ≤ 360 |  | ≤ 360 | EN ISO 3405 |
| PAH % [m/m] | ≤ 11 | ≤ 11 |  | ≤ 11 | EN 12916 |
| Carbon residue % [m/m] | ≤ 0,3 | ≤ 0,3 |  | ≤ 0,3 | EN ISO 10370 |
| CFPP [°C] | < -44 to < +5 | < -44 to < +5 |  | < -44 to < +5 | EN 116 |
| Cloud Point [°C] (severe winter conditions) | < -34 to < -10 | <-34 to <-10 |  | <-34 to <-10 | EN 23015 |
| Water [mg/kg] | ≤ 0,02 | ≤ 0,02 |  | ≤ 0,02 | EN ISO 12937 |
| Lubricity [micron] | --- | ≤ 460 |  | ≤ 460 | EN ISO 12156-1 |

- - - - -

Depending on the local market, a maximum of 50 is acceptable.

The limits have to be set according to climatic and seasonal conditions. Upper limit of 65 for summer season is required.

If these values are not currently met, local standards for density, MON, RON, subject to a RON value of not less than 90, have to be taken into account. At least, max. values for olefins and aromatics must be adhered to.

Depending on local market, a maximum of 50 is acceptable.

If these values are not currently met, local standards for cetane, density, viscosity, flash point, subject to a cetane index of not less than 45, have to be taken into account. At least, max. values for PAH must be adhered to.