INFORMAL GROUP ON GASEOUS FUEL VEHICLES

Within the UN GRPE (WP29)

PROPOSED AMENDMENT GFV-04-09

Name of Organisation submitting Amendment/Work Item:

Polish delegation together with AEGPL

Person submitting Item:

Stanislaw Radzimirski+ Arnaud Duvielguerbigny AEGPL

Address/phone/email coordinates

Arnaud Duvielguerbigny

AEGPL

Boulevard du souverain 165

Bruxelles 1160

Belgique

Work:	00.32.(0)2.566.91.25

Fax:	00.32.(0)2.566.91.29

Mobile:	00.32.(0)473.41.25.33

E-Mail:	arnaud.duvielguerbigny@aegpl.be

Regulation name and reference number: Regulation 83

Name of Amendment/Work Item:

Document aiming at clarifying the interpretation of the existing specification on the composition of LPG reference fuel in R83.

- Annex 10a:

1.	SPECIFICATIONS OF GASEOUS REFERENCE FUELS

1.1.	TECHNICAL DATA OF THE LPG REFERENCE FUELS

1.1.1.	Technical data of the LPG reference fuels used for testing vehicles to the emission limits given in row A of the table in paragraph 5.3.1.4. - Type I test

| Parameter | Unit | Fuel A | Fuel B | Test method |
| --- | --- | --- | --- | --- |
| Composition: |  |  |  | ISO 7941 |
| C3-content | per cent vol | 30 ± 2 | 85 ± 2 |  |
| C4-content | per cent vol | balance | Balance1 |  |
| < C3 , >C4 | per cent vol | max. 2 | max. 2 |  |
| Olefins | per cent vol | max. 12 | max. 15 |  |
| Evaporation residue | mg/kg | max. 50 | max. 50 | ISO 13757 |
| Water at 0C |  | free | free | visual inspection |
| Total sulphur content | mg/kg | max. 50 | max. 50 | EN 24260 |
| Hydrogen sulphide |  | none | none | ISO 8819 |
| Copper strip corrosion | rating | Class 1 | class 1 | ISO 6251 1/ |
| Odour |  | characteristic | characteristic |  |
| Motor octane number |  | min. 89 | min. 89 | EN 589 Annex B |

__________

1/	This method may not accurately determine the presence of corrosive materials if the sample contains corrosion inhibitors or other chemicals which diminish the corrosivity of the sample to the copper strip.  Therefore, the addition of such compounds for the sole purpose of biasing the test method is prohibited

1.1.2.	Technical data of the LPG reference fuels used for testing vehicles to the emission limits given in row B of the table in PARAGRAPH 5.3.1.4. of Annex I - Type I test

| Parameter | Unit | Fuel A | Fuel B | Test method |
| --- | --- | --- | --- | --- |
| Composition: |  |  |  | ISO 7941 |
| C3-content | per cent vol | 30 ± 2 | 85 ± 2 |  |
| C4-content | per cent vol | balance | Balance1 |  |
| < C3 , >C4 | per cent vol | max. 2 | max. 2 |  |
| Olefins | per cent vol | max. 12 | max. 15 |  |
| Evaporation residue | mg/kg | max. 50 | max. 50 | ISO 13757 |
| Water at 0 C |  | free | free | Visual inspection |
| Total sulphur content | mg/kg | max. 10 | max. 10 | EN 24260 |
| Hydrogen sulphide |  | none | none | ISO 8819 |
| Copper strip corrosion | Rating | class 1 | class 1 | ISO 6251 1/ |
| Odour |  | characteristic | characteristic |  |
| Motor octane number |  | min. 89 | min. 89 | EN 589 Annex B |

___________

1/	This method may not accurately determine the presence of corrosive materials if the sample contains corrosion inhibitors or other chemicals which diminish the corrosivity of the sample to the copper strip.  Therefore, the addition of such compounds for the sole purpose of biasing the test method is prohibited

Balance has to be read as follows: balance =100 - C3 - <C3 - >C4

Balance has to be read as follows: balance =100 - C3 - <C3 - >C4