###### GRPE/WHDC/FE31

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Pollution and Energy (GRPE)

Fifty-eighth session

Geneva, 9-12 June 2009

Item 1.1. of the provisional agenda

PROPOSAL FOR A DRAFT AMENDMENT TO GLOBAL TECHNICAL REGULATION (GTR) N° 4:

TEST PROCEDURE FOR COMPRESSION-IGNITION (C.I.) ENGINES AND POSITIVE-IGNITION (P.I.) ENGINES FUELLED WITH NATURAL GAS (NG) OR LIQUEFIED PETROLEUM GAS (LPG) WITH REGARD TO THE EMISSION OF POLLUTANTS

(World-wide harmonized heavy-duty certification (WHDC) procedure)

Submitted by the WHDC Secretary

Note:  The text reproduced below was prepared by the GRPE informal working group on the World-wide harmonized Heavy-Duty Certification procedure (WHDC) in order to solve the options contained in gtr n°4 and to align the gtr with the future NRMM gtr.  It is based on a document without an official symbol (informal document No. GRPE-57-07), distributed during the fifty-seventh GRPE session.  Following the agreements reached at the fifty-seventh GRPE session and the twenty seventh meeting of the WHDC informal working group from 10 to 12 March 2009, this document presents the final text of the amended WHDC gtr.

TABLE OF CONTENTS

##### Page

A.	STATEMENT OF TECHNICAL RATIONALE AND JUSTIFICATION	5

B.	TEXT OF THE REGULATION	7

1.	Purpose	7

2.	Scope	7

3.	Definitions, symbols and abbreviations	7

4.	General requirements	14

5.	Performance requirements	14

6.	Test conditions	19

7.	Test procedures	24

8.	Emission calculation	38

9.	Equipment specification and verification	61

ANNEXES

Annex 1	WHTC engine dynamometer schedule	87

Annex 2	Diesel reference fuel	100

Annex 3	Measurement equipment	102

Annex 4	Determination of system equivalence	119

Annex 5	Carbon flow check	121

Annex 6	Example of calculation procedure	124

Annex 7	Installation of auxiliaries for emissions test	129

A.	STATEMENT OF TECHNICAL RATIONALE AND JUSTIFICATION

1.	TECHNICAL AND ECONOMIC FEASIBILITY

The objective of this proposal is to establish a harmonized global technical regulation (gtr) covering the type-approval procedure for heavy-duty engine exhaust emissions.  The basis will be the test procedure developed by the WHDC informal group of GRPE (see the informal document No. 4 distributed during the forty-sixth GRPE session).

Regulations governing the exhaust emissions from heavy-duty engines have been in existence for many years but the test cycles and methods of emissions measurement vary significantly.  To be able to correctly determine the impact of a heavy-duty vehicle on the environment in terms of its exhaust pollutant emissions, a laboratory test procedure, and consequently the gtr, needs to be adequately representative of real-world vehicle operation.

The proposed regulation is based on new research into the world-wide pattern of real heavy commercial vehicle use.  From the collected data, two representative test cycles, a transient test cycle (WHTC) with both cold and hot start requirements and a hot start steady state test cycle (WHSC), have been created covering typical driving conditions in the European Union (EU), the United States of America, Japan and Australia.  Alternative emission measurement procedures have been developed by an expert committee in ISO and have been published in ISO 16183.  This standard reflects exhaust emissions measurement technology with the potential for accurately measuring the pollutant emissions from future low emission engines.  This work has been the basis for future Japanese and the EU emission legislation.  In parallel, substantial work has been undertaken on a different basis in the last several years in the United States of America to make major improvements to the emissions measurement procedures, testing protocols, and regulatory structure for both highway heavy-duty and non-road heavy-duty engines.  This work is documented in the rulemaking of the United States of America and was published on 13 July 2005.  Some of those new testing protocols are already reflected in this gtr.

It is recognized by the Contracting Parties to the 1998 Agreement that a long-term goal for highway heavy-duty diesel engine testing and non-road diesel engine testing would be gtrs which are similar in structure and substance with respect to measurement equipment, procedures and requirements.  Therefore, the Contracting Parties recognize there will be a need in the future to amend this gtr in order to have as much commonality as is possible between the highway heavy-duty diesel gtr and the non-road diesel gtr currently under development.  As discussed below, this gtr does not contain emission limit values.  When this gtr is amended in the future to include limit values, that may be the appropriate time to reconcile any substantive differences between the world-wide heavy-duty certification procedure (WHDC) gtr and the future gtr on non-road mobile machinery (NRMM).  At this stage, the limit values shall be developed by the Contracting Parties according to their own rules of procedure.

The WHTC and WHSC test procedures reflect world-wide on-road heavy-duty engine operation, as closely as possible, and provide a marked improvement in the realism of the test procedure for measuring the emission performance of existing and future heavy-duty engines.  In summary, the test procedure was developed so that it would be:

(a)	representative of world-wide on-road vehicle operations,

(b)	able to provide the highest possible level of efficiency in controlling on-road emissions,

(c)	corresponding to state-of-the-art testing, sampling and measurement technology,

(d)	applicable in practice to existing and foreseeable future exhaust emissions abatement technologies, and

(e)	capable of providing a reliable ranking of exhaust emission levels from different engine types.

At this stage, the gtr is being presented without limit values.  In this way, the test procedure can be given a legal status, based on which the Contracting Parties are required to start the process of implementing it into their national law.  The gtr contains several options, whose adoption is left to the discretion of the Contracting Parties.  Those options are related to the hot soak procedure between the cold and hot WHTC, the weighting factor of cold and hot WHTC, the particulate filter material and size, and the reference fuel.  However, these aspects have to be fully harmonized when common limit values are established.

When implementing the test procedure contained in this gtr as part of their national legislation or regulation, Contracting Parties are invited to use limit values which represent at least the same level of severity as their existing regulations, pending the development of harmonized limit values by the Executive Committee (AC.3) under the 1998 Agreement administered by the World Forum for Harmonization of Vehicle Regulations (WP.29).  The performance levels (emissions test results) to be achieved in the gtr will, therefore, be discussed on the basis of the most recently agreed legislation in the Contracting Parties, as required by the 1998 Agreement.

2.	ANTICIPATED BENEFITS

- Heavy commercial vehicles and their engines are increasingly produced for the world market.  It is economically inefficient for manufacturers to have to prepare substantially different models in order to meet different emission regulations and methods of measuring emissions, which, in principle, aim at achieving the same objective.  To enable manufacturers to develop new models more effectively and within a shorter time, it is desirable that a gtr should be developed.  These savings will accrue not only to the manufacturer, but more importantly, to the consumer as well.

- However, developing a test procedure just to address the economic question does not completely address the mandate given when work on this gtr was first started.  The test procedure must also improve the state of testing heavy-duty engines, and better reflect how heavy-duty engines are used today.  Compared to the measurement methods defined in existing legislation of the Contracting Parties to the 1998 Agreement, the testing methods defined in this gtr are much more representative of in-use driving behaviour of commercial vehicles world-wide.  It should be noted that the requirements of this gtr should be complemented by the requirements relating to the control of the Off-Cycle Emissions (OCE) and OBD systems.

- As a consequence, it can be expected that the application of this gtr for emissions legislation within the Contracting Parties to the 1998 Agreement will result in a higher control of in-use emissions due to the improved correlation of the test methods with in-use driving behaviour.

3.	POTENTIAL COST EFFECTIVENESS

Specific cost effectiveness values for this gtr have not been calculated.  The decision by the Executive Committee (AC.3) to the 1998 Agreement to move forward with this gtr without limit values is the key reason why this analysis has not been completed.  This common agreement has been made knowing that specific cost effectiveness values are not immediately available.  However, it is fully expected that this information will be developed, generally, in response to the adoption of this regulation in national requirements and also in support of developing harmonized limit values for the next step in this gtr's development.  For example, each Contracting Party adopting this gtr into its national law will be expected to determine the appropriate level of stringency associated with using these new test procedures, with these new values being at least as stringent as comparable existing requirements.  Also, experience will be gained by the heavy-duty engine industry as to any costs and cost savings associated with using this test procedure.  The cost and emissions performance data can then be analyzed as part of the next step in this gtr development to determine the cost effectiveness values of the test procedures being adopted today along with the application of harmonized limit values in the future.  While there are no values on calculated costs per ton, the belief of the GRPE experts is that there are clear benefits associated with this regulation.

B.	TEXT OF REGULATION

1.	PURPOSE

This regulation aims at providing a world-wide harmonized method for the determination of the levels of pollutant emissions from engines used in heavy vehicles in a manner which is representative of real world vehicle operation.  The results can be the basis for the regulation of pollutant emissions within regional type-approval and certification procedures.

2.	SCOPE

This regulation applies to the measurement of the emission of gaseous and particulate pollutants from compression-ignition engines and positive-ignition engines fuelled with natural gas (NG) or liquefied petroleum gas (LPG), used for propelling motor vehicles of categories 1-2 and 2, having a design speed exceeding 25 km/h and having a maximum mass exceeding 3.5 tonnes.

3.	DEFINITIONS, SYMBOLS AND ABBREVIATIONS

3.1.	Definitions

For the purpose of this regulation,

3.1.1.	"continuous regeneration" means the regeneration process of an exhaust after-treatment system that occurs either permanently or at least once per WHTC hot start test.  Such a regeneration process will not require a special test procedure.

3.1.2.	"delay time" means the difference in time between the change of the component to be measured at the reference point and a system response of 10 per cent of the final reading (t10) with the sampling probe being defined as the reference point.  For the gaseous components, this is the transport time of the measured component from the sampling probe to the detector.

3.1.3.	"deNOx system" means an exhaust after-treatment system designed to reduce emissions of oxides of nitrogen (NOx) (e.g. passive and active lean NOx catalysts, NOx adsorbers and selective catalytic reduction (SCR) systems).

3.1.4.	"diesel engine" means an engine which works on the compression-ignition principle.

3.1.5.	"engine family" means a manufacturers grouping of engines which, through their design as defined in paragraph 5.2. of this gtr, have similar exhaust emission characteristics; all members of the family must comply with the applicable emission limit values.

3.1.6.	"engine system" means the engine, the emission control system and the communication interface (hardware and messages) between the engine system electronic control unit(s) (ECU) and any other powertrain or vehicle control unit.

3.1.7.	"engine type" means a category of engines which do not differ in essential engine characteristics.

3.1.8.	"exhaust after-treatment system" means a catalyst (oxidation or 3-way), particulate filter, deNOx system, combined deNOx particulate filter or any other emission-reducing device that is installed downstream of the engine.  This definition excludes exhaust gas recirculation (EGR), which is considered an integral part of the engine.

3.1.9.	"full flow dilution method" means the process of mixing the total exhaust flow with dilution air prior to separating a fraction of the diluted exhaust stream for analysis.

3.1.11.	"gaseous pollutants" means carbon monoxide, hydrocarbons and/or non-methane hydrocarbons (assuming a ratio of CH1.85 for diesel, CH2.525 for LPG and CH2.93 for NG, and an assumed molecule CH3O0.5 for ethanol fuelled diesel engines), methane (assuming a ratio of CH4 for NG) and oxides of nitrogen (expressed in nitrogen dioxide (NO2) equivalent).

3.1.12.	"high speed (nhi)" means the highest engine speed where 70 per cent of the declared maximum power occurs.

3.1.13.	"low speed (nlo)" means the lowest engine speed where 55 per cent of the declared maximum power occurs.

3.1.14.	"maximum power (Pmax)" means the maximum power in kW as specified by the manufacturer.

3.1.15.	"maximum torque speed" means the engine speed at which the maximum torque is obtained from the engine, as specified by the manufacturer.

3.1.16.	"parent engine" means an engine selected from an engine family in such a way that its emissions characteristics are representative for that engine family.

3.1.17.	"particulate after-treatment device" means an exhaust after-treatment system designed to reduce emissions of particulate pollutants (PM) through a mechanical, aerodynamic, diffusional or inertial separation.

3.1.18.	"partial flow dilution method" means the process of separating a part from the total exhaust flow, then mixing it with an appropriate amount of dilution air prior to the particulate sampling filter.

3.1.19.	"particulate matter (PM)" means any material collected on a specified filter medium after diluting exhaust with clean filtered air to a temperature between 315 K (42 °C) and 325 K (52 °C), as measured at a point immediately upstream of the filter; this is primarily carbon, condensed hydrocarbons, and sulphates with associated water.

3.1.20.	"per cent load" means the fraction of the maximum available torque at an engine speed.

3.1.21.	"periodic regeneration" means the regeneration process of an exhaust after-treatment system that occurs periodically in typically less than 100 hours of normal engine operation.  During cycles where regeneration occurs, emission standards may be exceeded.

3.1.22.	"ramped steady state test cycle" means a test cycle with a sequence of steady state engine test modes with defined speed and torque criteria at each mode and defined ramps between these modes (WHSC).

3.1.23.	"rated speed" means the maximum full load speed allowed by the governor as specified by the manufacturer in his sales and service literature, or, if such a governor is not present, the speed at which the maximum power is obtained from the engine, as specified  by the manufacturer in his sales and service literature.

3.1.24.	"response time" means the difference in time between the change of the component to be measured at the reference point and a system response of 90 per cent of the final reading (t90) with the sampling probe being defined as the reference point, whereby the change of the measured component is at least 60 per cent full scale (FS) and takes place in less than 0.1 second.  The system response time consists of the delay time to the system and of the rise time of the system.

3.1.25.	"rise time" means the difference in time the 10 per cent and 90 per cent response of the final reading (t90 – t10).

3.1.26.	"specific emissions" means the mass emissions expressed in g/kWh.

3.1.27.	"test cycle" means a sequence of test points each with a defined speed and torque to be followed by the engine under steady state (WHSC) or transient operating conditions (WHTC).

3.1.28.	"transformation time" means the difference in time between the change of the component to be measured at the reference point and a system response of 50 per cent of the final reading (t50) with the sampling probe being defined as the reference point.  The transformation time is used for the signal alignment of different measurement instruments.

3.1.29.	"transient test cycle" means a test cycle with a sequence of normalized speed and torque values that vary relatively quickly with time (WHTC).

3.1.30.	"useful life" means the relevant period of distance and/or time over which compliance with the relevant gaseous and particulate emission limits has to be assured.

Figure 1:

Definitions of system response

3.2.	General symbols

| Symbol | Unit | Term |
| --- | --- | --- |
| A/Fst | - | Stoichiometric air to fuel ratio |
| c | ppm/Vol per cent | Concentration |
| cd | ppm/Vol per cent | Concentration on dry basis |
| cw | ppm/Vol per cent | Concentration on wet basis |
| cb | ppm/Vol per cent | Background concentration |
| Cd | - | Discharge coefficient of SSV |
| d | m | Diameter |
| dV | m | Throat diameter of venturi |
| D0 | m3/s | PDP calibration intercept |
| D | - | Dilution factor |
| t | s | Time interval |
| egas | g/kWh | Specific emission of gaseous components |
| ePM | g/kWh | Specific emission of particulates |
| ep | g/kWh | Specific emission during regeneration |
| ew | g/kWh | Weighted specific emission |
| ECO2 | per cent | CO2 quench of NOx analyzer |
| EE | per cent | Ethane efficiency |
| EH2O | per cent | Water quench of NOx analyzer |
| EM | per cent | Methane efficiency |
| ENOx | per cent | Efficiency of NOx converter |
| f | Hz | Data sampling rate |
| fa | - | Laboratory atmospheric factor |
| Fs | - | Stoichiometric factor |
| Ha | g/kg | Absolute humidity of the intake air |
| Hd | g/kg | Absolute humidity of the dilution air |
| i | - | Subscript denoting an instantaneous measurement (e.g. 1 Hz) |
| kf | - | Fuel specific factor |
| kh,D | - | Humidity correction factor for NOx for CI engines |
| kh,G | - | Humidity correction factor for NOx for PI engines |
| kr | - | Regeneration factor |
| kw,a | - | Dry to wet correction factor for the intake air |
| kw,d | - | Dry to wet correction factor for the dilution air |
| kw,e | - | Dry to wet correction factor for the diluted exhaust gas |
| kw,r | - | Dry to wet correction factor for the raw exhaust gas |
| KV | - | CFV calibration function |
|  | - | Excess air ratio |
| md | kg | Mass of the dilution air sample passed through the particulate sampling filters |
| med | kg | Total diluted exhaust mass over the cycle |
| medf | kg | Mass of equivalent diluted exhaust gas over the test cycle |
| mew | kg | Total exhaust mass over the cycle |
| mf | mg | Particulate sample mass collected |
| mf,d | mg | Particulate sample mass of the dilution air collected |
| mgas | g | Mass of gaseous emissions over the test cycle |
| mPM | g | Mass of particulate emissions over the test cycle |
| mse | kg | Exhaust sample mass over the test cycle |
| msed | kg | Mass of diluted exhaust gas passing the dilution tunnel |
| msep | kg | Mass of diluted exhaust gas passing the particulate collection filters |
| mssd | kg | Mass of secondary dilution air |
| Ma | g/mol | Molar mass of the intake air |
| Me | g/mol | Molar mass of the exhaust |
| Mgas | g/mol | Molar mass of gaseous components |
| n | - | Number of measurements |
| nr | - | Number of measurements during regeneration |
| n | min-1 | Engine rotational speed |
| nhi | min-1 | High engine speed |
| nlo | min-1 | Low engine speed |
| npref | min-1 | Preferred engine speed |
| np | r/s | PDP pump speed |
| pa | kPa | Saturation vapour pressure of engine intake air |
| pb | kPa | Total atmospheric pressure |
| pd | kPa | Saturation vapour pressure of the dilution air |
| pp | kPa | Absolute pressure |
| pr | kPa | Water vapour pressure after cooling bath |
| ps | kPa | Dry atmospheric pressure |
| qmad | kg/s | Intake air mass flow rate on dry basis |
| qmaw | kg/s | Intake air mass flow rate on wet basis |
| qmCe | kg/s | Carbon mass flow rate in the raw exhaust gas |
| qmCf | kg/s | Carbon mass flow rate into the engine |
| qmCp | kg/s | Carbon mass flow rate in the partial flow dilution system |
| qmdew | kg/s | Diluted exhaust gas mass flow rate on wet basis |
| qmdw | kg/s | Dilution air mass flow rate on wet basis |
| qmedf | kg/s | Equivalent diluted exhaust gas mass flow rate on wet basis |
| qmew | kg/s | Exhaust gas mass flow rate on wet basis |
| qmex | kg/s | Sample mass flow rate extracted from dilution tunnel |
| qmf | kg/s | Fuel mass flow rate |
| qmp | kg/s | Sample flow of exhaust gas into partial flow dilution system |
| qvCVS | m³/s | CVS volume rate |
| qvs | dm³/min | System flow rate of exhaust analyzer system |
| qvt | cm³/min | Tracer gas flow rate |
| rd | - | Dilution ratio |
| rD | - | Diameter ratio of SSV |
| rh | - | Hydrocarbon response factor of the FID |
| rm | - | Methanol response factor of the FID |
| rp | - | Pressure ratio of SSV |
| rs | - | Average sample ratio |
|  | kg/m³ | Density |
| e | kg/m³ | Exhaust gas density |
|  | - | Standard deviation |
| T | K | Absolute temperature |
| Ta | K | Absolute temperature of the intake air |
| t | s | Time |
| t10 | s | Time between step input and 10 per cent of final reading |
| t50 | s | Time between step input and 50 per cent of final reading |
| t90 | s | Time between step input and 90 per cent of final reading |
| u | - | Ratio between densities of gas component and exhaust gas |
| V0 | m3/r | PDP gas volume pumped per revolution |
| Vs | dm³ | System volume of exhaust analyzer bench |
| Wact | kWh | Actual cycle work of the test cycle |
| Wref | kWh | Reference cycle work of the test cycle |
| X0 | m3/r | PDP calibration function |

3.3.	Symbols and abbreviations for the fuel composition

| wALF | hydrogen content of fuel, per cent mass |
| --- | --- |
| wBET | carbon content of fuel, per cent mass |
| wGAM | sulphur content of fuel, per cent mass |
| wDEL | nitrogen content of fuel, per cent mass |
| wEPS | oxygen content of fuel, per cent mass |
|  | molar hydrogen ratio (H/C) |
|  | molar sulphur ratio (S/C) |
|  | molar nitrogen ratio (N/C) |
|  | molar oxygen ratio (O/C) |
| referring to a fuel CHONS |  |

3.4.	Symbols and abbreviations for the chemical components

| C1 | Carbon 1 equivalent hydrocarbon |
| --- | --- |
| CH4 | Methane |
| C2H6 | Ethane |
| C3H8 | Propane |
| CO | Carbon monoxide |
| CO2 | Carbon dioxide |
| DOP | Di-octylphtalate |
| HC | Hydrocarbons |
| H2O | Water |
| NMHC | Non-methane hydrocarbons |
| NOx | Oxides of nitrogen |
| NO | Nitric oxide |
| NO2 | Nitrogen dioxide |
| PM | Particulate matter |

3.5.	Abbreviations

| CFV | Critical Flow Venturi |
| --- | --- |
| CLD | Chemiluminescent Detector |
| CVS | Constant Volume Sampling |
| deNOx | NOx after-treatment system |
| EGR | Exhaust gas recirculation |
| FID | Flame Ionization Detector |
| GC | Gas Chromatograph |
| HCLD | Heated Chemiluminescent Detector |
| HFID | Heated Flame Ionization Detector |
| LPG | Liquefied Petroleum Gas |
| NDIR | Non-Dispersive Infrared (Analyzer) |
| NG | Natural Gas |
| NMC | Non-Methane Cutter |
| PDP | Positive Displacement Pump |
| Per cent FS | Per cent of full scale |
| PFS | Partial Flow System |
| SSV | Subsonic Venturi |
| VGT | Variable Geometry Turbine |

4.	GENERAL REQUIREMENTS

The engine system shall be so designed, constructed and assembled as to enable the engine in normal use to comply with the provisions of this gtr during its useful life, as defined by the Contracting Party, including when installed in the vehicle.

5.	PERFORMANCE REQUIREMENTS

When implementing the test procedure contained in this gtr as part of their national legislation, Contracting Parties to the 1998 Agreement are encouraged to use limit values which represent at least the same level of severity as their existing regulations; pending the development of harmonized limit values, by the Executive Committee (AC.3) of the 1998 Agreement, for inclusion in the gtr at a later date.

5.1.	Emission of gaseous and particulate pollutants

The emissions of gaseous and particulate pollutants by the engine shall be determined on the WHTC and WHSC test cycles, as described in paragraph 7.  The measurement systems shall meet the linearity requirements in paragraph 9.2. and the specifications in paragraph 9.3. (gaseous emissions measurement), paragraph 9.4. (particulate measurement) and in Annex 3.

Other systems or analyzers may be approved by the type approval or certification authority, if it is found that they yield equivalent results in accordance with paragraph 5.1.1.

5.1.1.	Equivalency

The determination of system equivalency shall be based on a seven-sample pair (or larger) correlation study between the system under consideration and one of the systems of this gtr.

"Results" refer to the specific cycle weighted emissions value.  The correlation testing is to be performed at the same laboratory, test cell, and on the same engine, and is preferred to be run concurrently.  The equivalency of the sample pair averages shall be determined by F-test and t-test statistics as described in Annex 4 obtained under the laboratory test cell and the engine conditions described above.  Outliers shall be determined in accordance with ISO 5725 and excluded from the database.  The systems to be used for correlation testing shall be subject to the approval by the type approval or certification authority.

5.2.	Engine family

5.2.1.	General

An engine family is characterized by design parameters.  These shall be common to all engines within the family.  The engine manufacturer may decide, which engines belong to an engine family, as long as the membership criteria listed in paragraph 5.2.3. are respected.  The engine family shall be approved by the type approval or certification authority.  The manufacturer shall provide to the type approval or certification authority the appropriate information relating to the emission levels of the members of the engine family.

5.2.2.	Special cases

In some cases there may be interaction between parameters.  This shall be taken into consideration to ensure that only engines with similar exhaust emission characteristics are included within the same engine family.  These cases shall be identified by the manufacturer and notified to the type approval or certification authority.  It shall then be taken into account as a criterion for creating a new engine family.

In case of devices or features, which are not listed in paragraph 5.2.3. and which have a strong influence on the level of emissions, this equipment shall be identified by the manufacturer on the basis of good engineering practice, and shall be notified to the type approval or certification authority.  It shall then be taken into account as a criterion for creating a new engine family.

In addition to the parameters listed in paragraph 5.2.3., the manufacturer may introduce additional criteria allowing the definition of families of more restricted size.  These parameters are not necessarily parameters that have an influence on the level of emissions.

5.2.3.	Parameters defining the engine family

5.2.3.1.						Combustion cycle

(a)	2-stroke cycle

(b)	4-stroke cycle

(c)	Rotary engine

(d)	Others

5.2.3.2.	Configuration of the cylinders

5.2.3.2.1.	Position of the cylinders in the block

(a)	V

(b)	In line

(c)	Radial

(d)	Others (F, W, etc.)

5.2.3.2.2.	Relative position of the cylinders

Engines with the same block may belong to the same family as long as their bore center-to-center dimensions are the same.

5.2.3.3.	Main cooling medium

(a)	air

(b)	water

(c)	oil

5.2.3.4.	Individual cylinder displacement

5.2.3.4.1.	Engine with a unit cylinder displacement  ≥  0.75 dm³

In order for engines with a unit cylinder displacement of  ≥  0.75 dm³ to be considered to belong to the same engine family, the spread of their individual cylinder displacements shall not exceed 15 per cent of the largest individual cylinder displacement within the family.

5.2.3.4.2.	Engine with a unit cylinder displacement < 0.75 dm³

In order for engines with a unit cylinder displacement of  < 0.75 dm³ to be considered to belong to the same engine family, the spread of their individual cylinder displacements shall not exceed 30 per cent of the largest individual cylinder displacement within the family.

5.2.3.4.3.	Engine with other unit cylinder displacement limits

Engines with an individual cylinder displacement that exceeds the limits defined in paragraphs 5.2.3.4.1. and 5.2.3.4.2. may be considered to belong to the same family with the approval of the type approval or certification authority.  The approval shall be based on technical elements (calculations, simulations, experimental results etc.) showing that exceeding the limits does not have a significant influence on the exhaust emissions.

5.2.3.5.	Method of air aspiration

(a)	naturally aspirated

(b)	pressure charged

(c)	pressure charged with charge cooler

5.2.3.6.	Fuel type

(a)	Diesel

(b)	Natural gas (NG)

(c)	Liquefied petroleum gas (LPG)

(d)	Ethanol

5.2.3.7.	Combustion chamber type

(a)	Open chamber

(b)	Divided chamber

(c)	Other types

5.2.3.8.	Ignition Type

(a)	Positive ignition

(b)	Compression ignition

5.2.3.9.	Valves and porting

(a)	Configuration

(b)	Number of valves per cylinder

5.2.3.10.	Fuel supply type

(a)	Liquid fuel supply type

(i)	Pump and (high pressure) line and injector

(ii)	In-line or distributor pump

(iii)	Unit pump or unit injector

(iv)	Common rail

(v)	Carburettor(s)

(vi)	Others

(b)	Gas fuel supply type

(i)	Gaseous

(ii)	Liquid

(iii)	Mixing units

(iv)	Others

(c)	Other types

5.2.3.11.	Miscellaneous devices

(a)	Exhaust gas recirculation (EGR)

(b)	Water injection

(c)	Air injection

(d)	Others

5.2.3.12.	Electronic control strategy

The presence or absence of an electronic control unit (ECU) on the engine is regarded as a basic parameter of the family.

In the case of electronically controlled engines, the manufacturer shall present the technical elements explaining the grouping of these engines in the same family, i.e. the reasons why these engines can be expected to satisfy the same emission requirements.

These elements can be calculations, simulations, estimations, description of injection parameters, experimental results, etc.

Examples of controlled features are:

(a)	Timing

(b)	Injection pressure

(c)	Multiple injections

(d)	Boost pressure

(e)	VGT

(f)	EGR

5.2.3.13.	Exhaust after-treatment systems

The function and combination of the following devices are regarded as membership criteria for an engine family:

(a)	Oxidation catalyst

(b)	Three-way catalyst

(c)	DeNOx system with selective reduction of NOx (addition of reducing agent)

(d)	Other DeNOx systems

(e)	Particulate trap with passive regeneration

(f)	Particulate trap with active regeneration

(g)	Other particulate traps

(h)	Other devices

When an engine has been certified without after-treatment system, whether as parent engine or as member of the family, then this engine, when equipped with an oxidation catalyst, may be included in the same engine family, if it does not require different fuel characteristics.

If it requires specific fuel characteristics (e.g. particulate traps requiring special additives in the fuel to ensure the regeneration process), the decision to include it in the same family shall be based on technical elements provided by the manufacturer.  These elements shall indicate that the expected emission level of the equipped engine complies with the same limit value as the non-equipped engine.

When an engine has been certified with after-treatment system, whether as parent engine or as member of a family, whose parent engine is equipped with the same after-treatment system, then this engine, when equipped without after-treatment system, must not be added to the same engine family.

5.2.4.	Choice of the parent engine

5.2.4.1.	Compression ignition engines

Once the engine family has been agreed by the type approval or certification authority, the parent engine of the family shall be selected using the primary criterion of the highest fuel delivery per stroke at the declared maximum torque speed.  In the event that two or more engines share this primary criterion, the parent engine shall be selected using the secondary criterion of highest fuel delivery per stroke at rated speed.

5.2.4.2.	Positive ignition engines

Once the engine family has been agreed by the type approval or certification authority, the parent engine of the family shall be selected using the primary criterion of the largest displacement.  In the event that two or more engines share this primary criterion, the parent engine shall be selected using the secondary criterion in the following order of priority:

(a)	the highest fuel delivery per stroke at the speed of declared rated power;

(b)	the most advanced spark timing;

(c)	the lowest EGR rate.

5.2.4.3.	Remarks on the choice of the parent engine

The type approval or certification authority may conclude that the worst-case emission of the family can best be characterized by testing additional engines.  In this case, the engine manufacturer shall submit the appropriate information to determine the engines within the family likely to have the highest emissions level.

If engines within the family incorporate other features which may be considered to affect exhaust emissions, these features shall also be identified and taken into account in the selection of the parent engine.

If engines within the family meet the same emission values over different useful life periods, this shall be taken into account in the selection of the parent engine.

6.	TEST CONDITIONS

6.1.	Laboratory test conditions

The absolute temperature (Ta) of the engine intake air expressed in Kelvin, and the dry atmospheric pressure (ps), expressed in kPa shall be measured and the parameter fa shall be determined according to the following provisions.  In multi-cylinder engines having distinct groups of intake manifolds, such as in a "Vee" engine configuration, the average temperature of the distinct groups shall be taken.  The parameter fa shall be reported with the test results.  For better repeatability and reproducibility of the test results, it is recommended that the parameter fa be such that: 0.93    fa    1.07.  Contracting Parties can make the parameter fa compulsory.

(a)	Compression-ignition engines:

Naturally aspirated and mechanically supercharged engines:

(1)

Turbocharged engines with or without cooling of the intake air:

(2)

(b)	Positive ignition engines:

(3)

6.2.	Engines with charge air-cooling

The charge air temperature shall be recorded and shall be, at the rated speed and full load, within  5 K of the maximum charge air temperature specified by the manufacturer.  The temperature of the cooling medium shall be at least 293 K (20 °C).

If a test laboratory system or external blower is used, the coolant flow rate shall be set to achieve a charge air temperature within  5 K of the maximum charge air temperature specified by the manufacturer at the rated speed and full load.  Coolant temperature and coolant flow rate of the charge air cooler at the above set point shall not be changed for the whole test cycle, unless this results in unrepresentative overcooling of the charge air.  The charge air cooler volume shall be based upon good engineering practice and shall be representative of the production engine's in-use installation.

If the engine manufacturer specifies pressure-drop limits across the charge-air cooling system, it shall be ensured that the pressure drop across the charge-air cooling system at engine conditions specified by the manufacturer is within the manufacturer’s specified limit(s).  The pressure drop shall be measured at the manufacturer’s specified locations.

6.3.	Engine power

The basis of specific emissions measurement is engine power and cycle work as determined in accordance with paragraphs 6.3.1. to 6.3.5.

6.3.1.	General engine installation

The engine shall be tested with the auxiliaries listed in Annex 7.

If auxiliaries are not installed as required, their power shall be taken into account in accordance with paragraphs 6.3.2. to 6.3.5.

6.3.2.	Auxiliaries to be fitted for the emissions test

If it is inappropriate to install the auxiliaries required according to Annex 7 on the test bench, the power absorbed by them shall be determined and subtracted from the measured engine power (reference and actual) over the whole engine speed range of the WHTC and over the test speeds of the WHSC.

6.3.3.	Auxiliaries to be removed for the test

Where the auxiliaries not required according to Annex 7 cannot be removed, the power absorbed by them may be determined and added to the measured engine power (reference and actual) over the whole engine speed range of the WHTC and over the test speeds of the WHSC.  If this value is greater than 3 per cent of the maximum power at the test speed it shall be verified by the type approval or certification authority.

6.3.4.	Determination of auxiliary power

The power absorbed by the auxiliaries needs only be determined, if

(a) auxiliaries required according to Annex 7, are not fitted to the engine

and/or

(b) auxiliaries not required according to Annex 7, are fitted to the engine.

The values of auxiliary power and the measurement/calculation method for determining auxiliary power shall be submitted by the engine manufacturer for the whole operating area of the test cycles, and approved by the certification or  type appproval authority.

6.3.5.	Engine cycle work

The calculation of reference and actual cycle work (see paragraphs 7.4.7. and 7.8.6.) shall be based upon engine power according to paragraph 6.3.1.  In this case, Pa and Pb of equation 4 are zero, and P equals Pm.

If auxiliaries are installed according to paragraphs 6.3.2. and/or 6.3.3., the power absorbed by them shall be used to correct each instantaneous cycle power value Pm, as follows:

(4)

where,

P	is the engine power, kW

Pm	is the measured engine power, kW

Pa	is the power absorbed by auxiliaries to be fitted, kW

Pb	is the power absorbed by auxiliaries to be removed, kW

6.4.	Engine air intake system

An engine air intake system or a test laboratory system shall be used presenting an air intake restriction within  300 Pa of the maximum value specified by the manufacturer for a clean air cleaner at the rated speed and full load.  The static differential pressure of the restriction shall be measured at the location specified by the manufacturer.

6.5.	Engine exhaust system

An engine exhaust system or a test laboratory system shall be used presenting an exhaust backpressure within 80 to 100% of the maximum value specified by the manufacturer at the rated speed and full load.  The exhaust system shall conform to the requirements for exhaust gas sampling, as set out in paragraphs 9.3.10. and 9.3.11.

6.6.	Engine with exhaust after-treatment system

If the engine is equipped with an exhaust after-treatment system, the exhaust pipe shall have the same diameter as found in-use, or as specified by the manufacturer, for at least four pipe diameters upstream of the expansion section containing the after-treatment device.  The distance from the exhaust manifold flange or turbocharger outlet to the exhaust after-treatment system shall be the same as in the vehicle configuration or within the distance specifications of the manufacturer.  The exhaust backpressure or restriction shall follow the same criteria as above, and may be set with a valve.  For variable-restriction aftertreatment devices, the maximum exhaust restriction is defined at the aftertreatment condition (degreening/aging and regeneration/loading level) specified by the manufacturer. If the maximum restriction is 5 kPa or less, the set point shall be no less than 1.0 kPa from the maximum.  The after-treatment container may be removed during dummy tests and during engine mapping, and replaced with an equivalent container having an inactive catalyst support.

The emissions measured on the test cycle shall be representative of the emissions in the field.  In the case of an engine equipped with a exhaust after-treatment system that requires the consumption of a reagent, the reagent used for all tests shall be declared by the manufacturer.

For engines equipped with exhaust after-treatment systems that are regenerated on a periodic basis, as described in paragraph 6.6.2., emission results shall be adjusted to account for regeneration events  In this case, the average emission depends on the frequency of the regeneration event in terms of fraction of tests during which the regeneration occurs.

After-treatment systems with continuous regeneration according to paragraph 6.6.1. do not require a special test procedure.

6.6.1.	Continuous regeneration

For an exhaust after-treatment system based on a continuous regeneration process the emissions shall be measured on an after-treatment system that has been stabilized so as to result in repeatable emissions behaviour.

The regeneration process shall occur at least once during the WHTC test and the manufacturer shall declare the normal conditions under which regeneration occurs (soot load, temperature, exhaust back-pressure, etc.).

In order to demonstrate that the regeneration process is continuous, at least three WHTC hot start tests shall be conducted.  For the purpose of this demonstration, the engine shall be warmed up in accordance with paragraph 7.4.2., the engine be soaked according to paragraph 7.6.3. and the WHTC hot start test be run.  During the tests, exhaust temperatures and pressures shall be recorded (temperature before and after the after-treatment system, exhaust back pressure, etc.).

The after-treatment system is considered to be of the continuous regeneration type if the conditions declared by the manufacturer occur during the test during a sufficient time and the emission results do not scatter by more than ± 25 per cent or 0.005 g/kWh, whichever is greater.

If the exhaust after-treatment system has a security mode that shifts to a periodic regeneration mode, it shall be checked according to paragraph 6.6.2.  For that specific case, the applicable emission limits may be exceeded and would not be weighted.

6.6.2.	Periodic regeneration

For an exhaust after-treatment based on a periodic regeneration process, the emissions shall be measured on at least three WHTC hot start tests, one during and two outside a regeneration event on a stabilized after-treatment system, and the results be weighted in accordance with equation 5.

For the purpose of this testing, the engine shall be warmed up in accordance with paragraph 7.4.2., the engine be soaked according to paragraph 7.6.3. and the WHTC hot start test be run.  If a regeneration event takes longer than one hot start test, the hot start test shall be repeated without soaking and shutting the engine off until regeneration is completed.

The regeneration process shall occur at least once during the WHTC test.  The engine may be equipped with a switch capable of preventing or permitting the regeneration process provided this operation has no effect on the original engine calibration.

The manufacturer shall declare the normal parameter conditions under which the regeneration process occurs (soot load, temperature, exhaust back-pressure, etc.) and its duration.  The manufacturer shall also provide the frequency of the regeneration event in terms of fraction of tests during which the regeneration occurs compared to fraction of tests without regeneration (F). The exact procedure to determine this fraction shall be based upon in use data using good engineering judgement, and shall be agreed by the type approval or certification authority.

The manufacturer shall provide an after-treatment system that has been loaded in order to achieve regeneration during a WHTC test.  Regeneration shall not occur during this engine-conditioning phase.

Average brake specific emissions between regeneration phases shall be determined from the arithmetic mean of several approximately equidistant WHTC hot start test results (g/kWh).  As a minimum, at least one WHTC as close as possible prior to a regeneration test and one WHTC immediately after a regeneration test shall be conducted.  As an alternative, the manufacturer may provide data to show that the emissions remain constant ( 25 per cent) between regeneration phases.  In this case, the emissions of only one WHTC test may be used.

During the regeneration test, all the data needed to detect regeneration shall be recorded (CO or NOx emissions, temperature before and after the after-treatment system, exhaust back pressure, etc.).

During the regeneration process, the applicable emission limits may be exceeded.

The test procedure is schematically shown in figure 2.

Figure 2:

Scheme of periodic regeneration

The WHTC hot start emissions shall be weighted as follows: (old para 8.5.2.2)

ew = er * F + (1-F) * e					(5)

where:

F 	is the time fraction of regeneration based on number of test cycles [-]

e	is the average specific emission outside regeneration, g/kWh

er	is the average specific emission during test cycle with regeneration, g/kWh

The regeneration adjustment factor kr may be applied either multiplicative (a) or additive (b) based upon good engineering analysis. The selection shall be approved by the type approval or certification authority.

(a) The multiplicative adjustment factor shall be calculated as follows:

(6)

(b) The additive adjustment factors shall be calculated as follows:

kr,u = ew - e (upward)													(7)

kr,d = ew - er (downward) 												(8)

With reference to paragraph 8.5. on brake specific emission calculations, the regeneration factor kr :

(a)	shall be applied to the weighted WHTC test result of paragraph 8.5.2.2.,

(b)	may be applied to the WHSC and cold WHTC, if a regeneration occurs during the cycle,

(c)	may be extended to other members of the same engine family,

(d)	may be extended to other engine families using the same aftertreatment system with the prior approval of the type approval or certification authority based on technical evidence to be supplied by the manufacturer that the emissions are similar.

6.7.	Cooling system

An engine cooling system with sufficient capacity to maintain the engine at normal operating temperatures prescribed by the manufacturer shall be used.

6.8.	Lubricating oil

The lubricating oil shall be specified by the manufacturer and be representative of lubricating oil available on the market; the specifications of the lubricating oil used for the test shall be recorded and presented with the results of the test.

6.9.	Specification of the reference fuel

The diesel reference fuel listed in Annex 2 shall be used for testing.  Since fuel characteristics influence the engine exhaust gas emission, the characteristics of the fuel used for the test shall be determined, recorded and declared with the results of the test.

No CNG and LPG reference fuels are listed due to the significant differences in local fuel qualities.

The fuel temperature shall be in accordance with the manufacturers recommendations.

6.10.	Crankcase emissions

No crankcase emissions shall be discharged directly into the ambient atmosphere, with the following exception: engines equipped with turbochargers, pumps, blowers, or superchargers for air induction may discharge crankcase emissions to the ambient atmosphere if the emissions are added to the exhaust emissions (either physically or mathematically) during all emission testing.  Manufacturers taking advantage of this exception shall install the engines so that all crankcase emission can be routed into the emissions sampling system.  For the purpose of this paragraph, crankcase emissions that are routed into the exhaust upstream of exhaust aftertreatment during all operation are not considered to be discharged directly into the ambient atmosphere.

Open crankcase emissions shall be routed into the exhaust system for emission measurement, as follows:

(1) 	The tubing materials shall be smooth-walled, electrically conductive, and not reactive with crankcase emissions.  Tube lengths shall be minimized as far as possible.

(2) 	The number of bends in the laboratory crankcase tubing shall be minimized,  and the radius of any unavoidable bend shall be maximized.

(3) 	The laboratory crankcase exhaust tubing shall meet the requirements of paragraph A.3.1.3. (HSL1) and the engine manufacturer’s specifications for crankcase back pressure.

(4) 	The crankcase exhaust tubing shall connect into the raw exhaust downstream of any aftertreatment system, downstream of any installed exhaust restriction, and sufficiently upstream of any sample probes to ensure complete mixing with the engine’s exhaust before sampling.  The crankcase exhaust tube shall extend into the free stream of exhaust to avoid boundary-layer effects and to promote mixing. The crankcase exhaust tube’s outlet may orient in any direction relative to the raw exhaust flow.

7.	TEST PROCEDURES

7.1.	Principles of emissions measurement

To measure the brake-specific emissions, the engine shall be operated over the test cycles defined in paragraphs 7.2.1. and 7.2.2.  The measurement of brake-specific emissions requires the determination of the mass of components in the exhaust and the corresponding engine cycle work.  The components are determined by the sampling methods described in paragraphs 7.1.1. and 7.1.2.

7.1.1. 	Continuous sampling

In continuous sampling, the component’s concentration is measured continuously from raw or dilute exhaust.  This concentration is multiplied by the continuous (raw or dilute) exhaust flow rate at the emission sampling location to determine the component’s mass flow rate.  The component’s emission is continuously summed over the test cycle.  This sum is the total mass of the emitted component.

7.1.2. 	Batch sampling

In batch sampling, a sample of raw or dilute exhaust is continuously extracted and stored for later measurement.  The extracted sample shall be proportional to the raw or dilute exhaust flow rate.  Examples of batch sampling are collecting diluted gaseous components in a bag and collecting PM on a filter.  The batch sampled concentrations are multiplied by the total exhaust mass or mass flow (raw or dilute) from which it was extracted during the test cycle.  This product is the total mass or mass flow of the emitted component.  To calculate the PM concentration, the PM deposited onto a filter from proportionally extracted exhaust shall be divided by the amount of filtered exhaust.

7.1.3. 	Measurement procedures (old para 7.1)

This gtr applies two measurement procedures that are functionally equivalent. Both procedures may be used for both the WHTC and the WHSC test cycle:

(a)	the gaseous components are sampled continuously in the raw exhaust gas, and the particulates are determined using a partial flow dilution system;

(b)	the gaseous components and the particulates are determined using a full flow dilution system (CVS system).

Any combination of the two principles (e.g. raw gaseous measurement and full flow particulate measurement) is permitted.

7.2.	Test cycles

7.2.1.	Transient test cycle WHTC (old para 7.2)

The transient test cycle WHTC is listed in Annex 1 as a second-by-second sequence of normalized speed and torque values.  In order to perform the test on an engine test cell, the normalized values shall be converted to the actual values for the individual engine under test based on the engine-mapping curve.  The conversion is referred to as denormalization, and the test cycle so developed as the reference cycle of the engine to be tested.  With those reference speed and torque values, the cycle shall be run on the test cell, and the actual speed, torque and power values shall be recorded.  In order to validate the test run, a regression analysis between reference and actual speed, torque and power values shall be conducted upon completion of the test.

For calculation of the brake specific emissions, the actual cycle work shall be calculated by integrating actual engine power over the cycle.  For cycle validation, the actual cycle work must be within prescribed limits of the reference cycle work.

For the gaseous pollutants, continuous sampling (raw or dilute exhaust gas) or batch sampling (dilute exhaust gas)  may be used.  The particulate sample shall be diluted with a conditioned diluent (such as ambient air), and collected on a single suitable filter.  The WHTC is shown schematically in figure 3.

Figure 3:

WHTC test cycle

7.2.2.	Ramped steady state test cycle WHSC (old para 7.3)

The ramped steady state test cycle WHSC consists of a number of normalized speed and load modes which shall be converted to the reference values for the individual engine under test based on the engine-mapping curve.  The engine shall be operated for the prescribed time in each mode, whereby engine speed and load shall be changed linearly within 20  1 seconds. In order to validate the test run, a regression analysis between reference and actual speed, torque and power values shall be conducted upon completion of the test.

The concentration of each gaseous pollutant, exhaust flow and power output shall be  determined over the test cycle.  The gaseous pollutants may be recorded continuously or sampled into a sampling bag.  The particulate sample shall be diluted with a conditioned diluent (such as ambient air).  One sample over the complete test procedure shall be taken, and collected on a single suitable filter.

For calculation of the brake specific emissions, the actual cycle work shall be calculated by integrating actual engine power over the cycle.

The WHSC is shown in table 1. Except for mode 1, the start of each mode is defined as the beginning of the ramp from the previous mode.

| Mode | Normalized Speed (per cent) | Normalized Torque (per cent) | Mode length (s) incl. 20 s ramp |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 210 |
| 2 | 55 | 100 | 50 |
| 3 | 55 | 25 | 250 |
| 4 | 55 | 70 | 75 |
| 5 | 35 | 100 | 50 |
| 6 | 25 | 25 | 200 |
| 7 | 45 | 70 | 75 |
| 8 | 45 | 25 | 150 |
| 9 | 55 | 50 | 125 |
| 10 | 75 | 100 | 50 |
| 11 | 35 | 50 | 200 |
| 12 | 35 | 25 | 250 |
| 13 | 0 | 0 | 210 |
| Sum |  |  | 1895 |

Table 1:

WHSC test cycle

7.3.	General test sequence

The following flow chart outlines the general guidance that should be followed during testing.  The details of each step are described in the relevant paragraphs.  Deviations from the guidance are permitted where appropriate, but the specific requirements of the relevant paragraphs are mandatory.

For the WHTC, the test procedure consists of a cold start test following either natural or forced cool-down of the engine, a hot soak period and a hot start test.  Selection of the hot soak period and the weighting factor between cold start test and hot start test shall be decided by the Contracting Parties.

For the WHSC, the test procedure consists of a hot start test following engine preconditioning at WHSC mode 9.

7.4.	Engine mapping and reference cycle

Pre-test engine measurements, pre-test engine performance checks and pre-test system calibrations shall be made prior to the engine mapping procedure in line with the general test sequence shown in paragraph 7.3.

As basis for WHTC and WHSC reference cycle generation, the engine shall be mapped under full load operation for determining the speed vs. max. torque and speed vs. max. power curves. The mapping curve shall be used for denormalizing engine speed (paragraph 7.4.5.) and engine torque (paragraph 7.4.6.).

7.4.1.	Determination of the mapping speed range

The minimum and maximum mapping speeds are defined as follows:

Minimum mapping speed	=	idle speed

Maximum mapping speed	=	nhi x 1.02 or speed where full load torque drops off to zero, whichever is smaller.

7.4.2.	Engine mapping curve

The engine shall be warmed up and towards the end of the warm up it shall be operated for at least 10 minutes at maximum power or according to the recommendation of the manufacturer and good engineering judgment in order to stabilize the engine coolant and lube oil temperatures.  When the engine is stabilized, the engine mapping shall be performed according to the following procedure.

(a)	The engine shall be unloaded and operated at idle speed.

(b)	The engine shall be operated at full load setting of the injection pump at minimum mapping speed.

(c)	The engine speed shall be increased at an average rate of 8 ± 1 min-1/s from minimum to maximum mapping speed.  Engine speed and torque points shall be recorded at a sample rate of at least one point per second.

7.4.3.	Alternate mapping

If a manufacturer believes that the above mapping techniques are unsafe or unrepresentative for any given engine, alternate mapping techniques may be used.  These alternate techniques must satisfy the intent of the specified mapping procedures to determine the maximum available torque at all engine speeds achieved during the test cycles.  Deviations from the mapping techniques specified in this paragraph for reasons of safety or representativeness shall be approved by the type approval or certification authority along with the justification for their use.  In no case, however, the torque curve shall be run by descending engine speeds for governed or turbocharged engines.

7.4.4.	Replicate tests

An engine need not be mapped before each and every test cycle.  An engine shall be remapped prior to a test cycle if:

(a)	an unreasonable amount of time has transpired since the last map, as determined by engineering judgement, or

(b)	physical changes or recalibrations have been made to the engine which potentially affect engine performance.

7.4.5.	Denormalization of engine speed

For generating the reference cycles, the normalized speeds of Annex 1 (WHTC) and table 1 (WHSC) shall be denormalized using the following equation:

nref	 = nnorm x (0.45 x nlo + 0.45 x npref + 0.1 x nhi – nidle) x 2.0327 + nidle	(9)

For determination of npref, the integral of the maximum torque shall be calculated from nidle to n95h from the engine mapping curve, as determined in accordance with paragraph 7.4.2.

The engine speeds in figures 4 and 5 are defined, as follows:

nlo	is the lowest speed where the power is 55 per cent of maximum power

npref	is the engine speed where the integral of max. mapped torque is 51 per cent of the whole integral between nidle and n95h

nhi	is the highest speed where the power is 70 per cent of maximum power

nidle	is the idle speed

n95h	is the highest speed where the power is 95 per cent of maximum power

For engines (mainly gas engines) with a steep governor droop curve, where fuel cut off does not permit to operate the engine up to nhi or n95h , the following provisions apply:

nhi	in equation 9 is replaced with nPmax * 1.02

n95h	is replaced with nPmax * 1.02

Figure 4:

Definition of test speeds

Figure 5:

Definition of npref

7.4.6.	Denormalization of engine torque

The torque values in the engine dynamometer schedule of Annex 1 (WHTC) and in table 1 (WHSC) are normalized to the maximum torque at the respective speed.  For generating the reference cycles, the torque values for each individual reference speed value as determined in paragraph 7.4.5. shall be denormalized, using the mapping curve determined according to paragraph 7.4.2., as follows:

Tref,i	=		(10)

where:

Tnorm			is the normalized torque, per cent

Tmax	is the maximum torque from the mapping curve, Nm

Ta	is the torque absorbed by auxiliaries to be fitted, Nm

Tb	is the torque absorbed by auxiliaries to be removed, Nm

If auxiliaries are fitted in accordance with paragraph 6.3.1., Ta and Tb are zero.

7.4.7.	Calculation of reference cycle work

Reference cycle work shall be determined over the test cycle by synchronously calculating instantaneous values for engine power from reference speed and  reference torque, as determined in paragraphs 7.4.5. and 7.4.6.  Instantaneous engine power values shall be integrated over the test cycle to calculate the reference cycle work Wref (kWh).  	If auxiliaries are not fitted in accordance with paragraph 6.3.1., the instantaneous power values shall be corrected using equation (4) in paragraph 6.3.5.

The same methodology shall be used for integrating both reference and actual engine power.  If values are to be determined between adjacent reference or adjacent measured values, linear interpolation shall be used.  In integrating the actual cycle work, any negative torque values shall be set equal to zero and included.  If integration is performed at a frequency of less than 5 Hz, and if, during a given time segment, the torque value changes from positive to negative or negative to positive, the negative portion shall be computed and set equal to zero.  The positive portion shall be included in the integrated value.

7.5.	Pre-test procedures

7.5.1.	Installation of the measurement equipment (old para 7.8.2.3.)

The instrumentation and sample probes shall be installed as required.  The tailpipe shall be connected to the full flow dilution system, if used.

7.5.2.	Preparation of measurement equipment for sampling

The following steps shall be taken before emission sampling begins:

(a) 	Leak checks shall be performed within 8 hours prior to emission sampling according to paragraph 9.3.4.

(b) 	For batch sampling, clean storage media shall be connected, such as evacuated bags.

(c) 	All measurement instruments shall be started according to the instrument manufacturer’s instructions and good engineering judgment.

(d) 	Dilution systems, sample pumps, cooling fans, and the data-collection system shall be started.

(e) 	The sample flow rates shall be adjusted to desired levels, using bypass flow, if desired.

(f) 	Heat exchangers in the sampling system shall be pre-heated or pre-cooled to within their operating temperature ranges for a test.

(g) 	Heated or cooled components such as sample lines, filters, coolers, and pumps shall be allowed to stabilize at their operating temperatures.

(h) 	Exhaust dilution system flow shall be switched on at least 10 minutes before a test sequence.

(i) 	Any electronic integrating devices shall be zeroed or re-zeroed, before the start of any test interval.

7.5.3	Checking the gas analyzers

Gas analyzer ranges shall be selected.  Emission analyzers with automatic or manual range switching are permitted.  During the test cycle, the range of the emission analyzers shall not be switched.  At the same time the gains of an analyzer’s analogue operational amplifier(s) may not be switched during the test cycle.

All analyzers shall be zeroed and spanned using internationally-traceable gases that meet the specifications of paragraph 9.3.3.  FID analyzers shall be spanned on a carbon number basis of one (C1).

7.5.4.	Preparation of the particulate sampling filter (old para 7.8.2.2.)

At least one hour before the test, the filter shall be placed in a petri dish, which is protected against dust contamination and allows air exchange, and placed in a weighing chamber for stabilization.  At the end of the stabilization period, the filter shall be weighed and the tare weight shall be recorded.  The filter shall then be stored in a closed petri dish or sealed filter holder until needed for testing.  The filter shall be used within eight hours of its removal from the weighing chamber.

7.5.5.	Adjustment of the dilution system (old para 7.8.2.6.)

The total diluted exhaust gas flow of a full flow dilution system or the diluted exhaust gas flow through a partial flow dilution system shall be set to eliminate water condensation in the system, and to obtain a filter face temperature between 315 K (42 °C) and 325 K (52 °C).

7.5.6.	Starting the particulate sampling system (old para 7.8.2.5.)

The particulate sampling system shall be started and operated on by-pass.  The particulate background level of the dilution air may be determined by sampling the dilution air prior to the entrance of the exhaust gas into the dilution tunnel.  The measurement may be done prior to or after the test.  If the measurement is done both at the beginning and at the end of the cycle, the values may be averaged.  If a different sampling system is used for background measurement, the measurement shall be done in parallel to the test run.

7.6.	WHTC cycle run

7.6.1.		Engine cool-down (old para 7.8.2.1.)

A natural or forced cool-down procedure may be applied.  For forced cool-down, good engineering judgment shall be used to set up systems to send cooling air across the engine, to send cool oil through the engine lubrication system, to remove heat from the coolant through the engine cooling system, and to remove heat from an exhaust after-treatment system.  In the case of a forced after-treatment system cool down, cooling air shall not be applied until the after-treatment system has cooled below its catalytic activation temperature.  Any cooling procedure that results in unrepresentative emissions is not permitted.

7.6.2.	Cold start test (old para 7.8.3.1.)

The cold-start test shall be started when the temperatures of the engine's lubricant, coolant, and after-treatment systems are all between 293 and 303 K (20 and 30 °C).  The engine shall be started using one of the following methods:

(a)	the engine shall be started as recommended in the owners manual using a production starter motor and adequately charged battery or a suitable power supply; or

(b)	the engine shall be started by using the dynamometer.  The engine shall be motored within   25 per cent of its typical in-use cranking speed.  Cranking shall be stopped within 1 second after the engine is running.  If the engine does not start after 15 seconds of cranking, cranking shall be stopped and the reason for the failure to start determined, unless the owners manual or the service-repair manual describes the longer cranking time as normal.

7.6.3.	Hot soak period (old para 7.8.3.2.)

Immediately upon completion of the cold start test, the engine shall be conditioned for the hot start test by using one of the following options:

(a)			5  1 minutes hot soak period

(b)	20  1 minutes hot soak period

The option shall be selected by the Contracting Parties.

7.6.4.	Hot start test (old para 7.8.3.3.1.)

The engine shall be started at the end of the hot soak period as defined in paragraph 7.6.3. using the starting methods given in paragraph 7.6.2.

7.6.5.	Test sequence (old para 7.8.4.1.)

The test sequence of both cold start and hot start test shall commence at the start of the engine. After the engine is running, cycle control shall be initiated so that engine operation matches the first set point of the cycle.

The WHTC shall be performed according to the reference cycle as set out in paragraph 7.4.  Engine speed and torque command set points shall be issued at 5 Hz (10 Hz recommended) or greater.  The set points shall be calculated by linear interpolation between the 1 Hz set points of the reference cycle.  Actual engine speed and torque shall be recorded at least once every second during the test cycle (1 Hz), and the signals may be electronically filtered.

7.6.6.	Collection of emission relevant data (old para 7.8.4.2.)

At the start of the test sequence, the measuring equipment shall be started, simultaneously:

(a)	start collecting or analyzing dilution air, if a full flow dilution system is used;

(b)	start collecting or analyzing raw or diluted exhaust gas, depending on the method used;

(c)	start measuring the amount of diluted exhaust gas and the required temperatures and pressures;

(d)	start recording the exhaust gas mass flow rate, if raw exhaust gas analysis is used;

(e)	start recording the feedback data of speed and torque of the dynamometer.

If raw exhaust measurement is used, the emission concentrations ((NM)HC, CO and NOx) and the exhaust gas mass flow rate shall be measured continuously and stored with at least 2 Hz on a computer system.  All other data may be recorded with a sample rate of at least 1 Hz.  For analogue analyzers the response shall be recorded, and the calibration data may be applied online or offline during the data evaluation.

If a full flow dilution system is used, HC and NOx shall be measured continuously in the dilution tunnel with a frequency of at least 2 Hz.  The average concentrations shall be determined by integrating the analyzer signals over the test cycle.  The system response time shall be no greater than 20 s, and shall be coordinated with CVS flow fluctuations and sampling time/test cycle offsets, if necessary.  CO, CO2, and NMHC may be determined by integration of continuous measurement signals or by analyzing the concentrations in the sample bag, collected over the cycle.  The concentrations of the gaseous pollutants in the dilution air shall be determined by integration or by collecting into the background bag.  All other parameters that need to be measured shall be recorded with a minimum of one measurement per second (1 Hz).

7.6.7.	Particulate sampling (old para 7.8.4.3.)

At the start of the test sequence, the particulate sampling system shall be switched from by-pass to collecting particulates.

If a partial flow dilution system is used, the sample pump(s) shall be controlled, so that the flow rate through the particulate sample probe or transfer tube is maintained proportional to the exhaust mass flow rate as determined in accordance with paragraph 9.4.6.1.

If a full flow dilution system is used, the sample pump(s) shall be adjusted so that the flow rate through the particulate sample probe or transfer tube is maintained at a value within ± 2.5 per cent of the set flow rate.  If flow compensation (i.e., proportional control of sample flow) is used, it must be demonstrated that the ratio of main tunnel flow to particulate sample flow does not change by more than ± 2.5 per cent of its set value (except for the first 10 seconds of sampling).  The average temperature and pressure at the gas meter(s) or flow instrumentation inlet shall be recorded.  If the set flow rate cannot be maintained over the complete cycle within ± 2.5 per cent because of high particulate loading on the filter, the test shall be voided.  The test shall be rerun using a lower sample flow rate.

7.6.8.	Engine stalling and equipment malfunction (old para 7.8.4.4.)

If the engine stalls anywhere during the cold start test, the test shall be voided.  The engine shall be preconditioned and restarted according to the requirements of paragraph 7.6.2., and the test repeated.

If the engine stalls anywhere during the hot start test, the hot start test shall be voided.  The engine shall be soaked according to paragraph 7.6.3., and the hot start test repeated.  In this case, the cold start test need not be repeated.

If a malfunction occurs in any of the required test equipment during the test cycle, the test shall be voided and repeated in line with the above provisions.

7.7.	WHSC cycle run

7.7.1.	Preconditioning the dilution system and the engine (old para 7.8.2.4.)

The dilution system and the engine shall be started and warmed up.  After warm-up, the engine and sampling system shall be preconditioned by operating the engine at mode 9 for a minimum of 10 minutes while simultaneously operating the dilution system.  Dummy particulate emissions samples may be collected.  Those sample filters need not be stabilized or weighed, and may be discarded.  Flow rates shall be set at the approximate flow rates selected for testing.  The engine shall be shut off after preconditioning.

7.7.2.	Engine starting

5  1 minutes after completion of preconditioning at mode 9 as described in paragraph 7.7.1., the engine shall be started according to the manufacturer's recommended starting procedure in the owner's manual, using either a production starter motor or the dynamometer in accordance with paragraph 7.6.2.

7.7.3.	Test sequence

The test sequence shall commence after the engine is running and engine operation is controlled to match the first mode of the cycle.

The WHSC shall be performed according to the order of test modes listed in table 1 of paragraph 7.2.2.

7.7.4.	Collection of emission relevant data

At the start of the test sequence, the measuring equipment shall be started, simultaneously:

(a)	start collecting or analyzing dilution air, if a full flow dilution system is used;

(b)	start collecting or analyzing raw or diluted exhaust gas, depending on the method used;

(c)	start measuring the amount of diluted exhaust gas and the required temperatures and pressures;

(d)	start recording the exhaust gas mass flow rate, if raw exhaust gas analysis is used;

(e)	start recording the feedback data of speed and torque of the dynamometer.

If raw exhaust measurement is used, the emission concentrations ((NM)HC, CO and NOx) and the exhaust gas mass flow rate shall be measured continuously and stored with at least 2 Hz on a computer system.  All other data may be recorded with a sample rate of at least 1 Hz.  For analogue analyzers the response shall be recorded, and the calibration data may be applied online or offline during the data evaluation.

If a full flow dilution system is used, HC and NOx shall be measured continuously in the dilution tunnel with a frequency of at least 2 Hz.  The average concentrations shall be determined by integrating the analyzer signals over the test cycle.  The system response time shall be no greater than 20 s, and shall be coordinated with CVS flow fluctuations and sampling time/test cycle offsets, if necessary.  CO, CO2, and NMHC may be determined by integration of continuous measurement signals or by analyzing the concentrations in the sample bag, collected over the cycle.  The concentrations of the gaseous pollutants in the dilution air shall be determined by integration or by collecting into the background bag.  All other parameters that need to be measured shall be recorded with a minimum of one measurement per second (1 Hz).

7.7.5.	Particulate sampling

At the start of the test sequence, the particulate sampling system shall be switched from by-pass to collecting particulates.

If a partial flow dilution system is used, the sample pump(s) shall be controlled, so that the flow rate through the particulate sample probe or transfer tube is maintained proportional to the exhaust mass flow rate as determined in accordance with paragraph 9.4.6.1.

If a full flow dilution system is used, the sample pump(s) shall be adjusted so that the flow rate through the particulate sample probe or transfer tube is maintained at a value within ± 2.5 per cent of the set flow rate.  If flow compensation (i.e., proportional control of sample flow) is used, it must be demonstrated that the ratio of main tunnel flow to particulate sample flow does not change by more than ± 2.5 per cent of its set value (except for the first 10 seconds of sampling).  The average temperature and pressure at the gas meter(s) or flow instrumentation inlet shall be recorded.  If the set flow rate cannot be maintained over the complete cycle within ± 2.5 per cent because of high particulate loading on the filter, the test shall be voided.  The test shall be rerun using a lower sample flow rate.

7.7.6.	Engine stalling and equipment malfunction

If the engine stalls anywhere during the cycle, the test shall be voided.  The engine shall be preconditioned according to paragraph 7.7.1. and restarted according to  paragraph 7.7.2., and the test repeated.

If a malfunction occurs in any of the required test equipment during the test cycle, the test shall be voided and repeated in line with the above provisions.

7.8.	Post-test procedures

7.8.1.	Operations after test (old para 7.8.4.5.)

At the completion of the test, the measurement of the exhaust gas mass flow rate, the diluted exhaust gas volume, the gas flow into the collecting bags and the particulate sample pump shall be stopped.  For an integrating analyzer system, sampling shall continue until system response times have elapsed.

7.8.2. 	Verification of proportional sampling

For any proportional batch sample, such as a bag sample or PM sample, it shall be verified that proportional sampling was maintained according to paragraphs 7.6.7. and 7.7.5.  Any sample that does not fulfil the requirements shall be voided.

7.8.3.. 	PM conditioning and weighing

The particulate filter shall be placed into covered or sealed containers or the filter holders shall be closed, in order to protect the sample filters against ambient contamination.  Thus protected, the filter shall be returned to the weighing chamber. The filter shall be conditioned for at least one hour, and then weighed according to paragraph 8.2.4.  The gross weight of the filter shall be recorded.

7.8.4. 	Drift verification

As soon as practical but no later than 30 minutes after the test cycle is complete or during the soak period, the gaseous analyzer ranges used shall be zeroed and spanned to check their drift compared to the pre-test zero and span procedures in paragraph 7.5.3.  For the purpose of this paragraph, test cycle is defined as follows:

(a)	for the WHTC: the complete sequence cold - soak - hot;

(b)	for the hot start WHTC in paragraph 6.6: the sequence soak - hot;

(c)	for the multiple regeneration hot start WHTC in paragraph 6.6: the combined number of hot start tests;

(d)	for the WHSC: the test cycle.

The following provisions apply for analyzer drift:

(a)	If the difference between the pre-test and post-test results is less than 2 per cent as specified in paragaphs 9.3.1.5. and 9.3.1.6., the measured concentrations may be used uncorrected or may be corrected for drift according to paragraph 8.5.1.

(b)	If the difference between the pre-test and post-test results is equal to or greater than 2 per cent as specified in paragaphs 9.3.1.5. and 9.3.1.6., the test shall be voided or the measured concentrations shall be corrected for drift according to paragraph 8.5.1.

7.8.5. 	Analysis of gaseous bag sampling

As soon as practical, the following shall be performed:

(a) 	Gaseous bag samples shall be analyzed no later than 30 minutes after the hot start test is complete or during the soak period for the cold start test.

(b) 	Background samples shall be analyzed no later than 60 minutes after the hot start test is complete.

7.8.6.	Validation of cycle work (old para 7.7.1.)

Before calculating actual cycle work, any points recorded during engine starting shall be omitted.  Actual cycle work shall be determined over the test cycle by synchronously using actual speed and actual torque values to calculate instantaneous values for engine power.  Instantaneous engine power values shall be integrated over the test cycle to calculate the actual cycle work Wact (kWh).  	If auxiliaries are not fitted in accordance with paragraph 6.3.1., the instantaneous power values shall be corrected using equation (4) in paragraph 6.3.5.

The same methodology as described in paragraph 7.4.7. shall be used for integrating actual engine power.

The actual cycle work Wact is used for comparison to the reference cycle work Wref and for calculating the brake specific emissions (see paragraph 8.5.4.).

Wact shall be between 85 per cent and 105 per cent of Wref.

7.8.7.	Validation statistics of the test cycle (old para 7.7.2.)

Linear regressions of the actual values on the reference values shall be performed for speed, torque and power.

To minimize the biasing effect of the time lag between the actual and reference cycle values, the entire engine speed and torque actual signal sequence may be advanced or delayed in time with respect to the reference speed and torque sequence.  If the actual signals are shifted, both speed and torque must be shifted the same amount in the same direction.

The method of least squares shall be used, with the best-fit equation having the form:

y  =  a1x + a0	(11)

where:

y		 			=	 actual value of speed (min-1), torque (Nm), or power (kW)

a1 	=	 slope of the regression line

x 	=	 reference value of speed (min-1), torque (Nm), or power (kW)

a0 	=	 y intercept of the regression line

The standard error of estimate (SEE) of y on x and the coefficient of determination (r²) shall be calculated for each regression line.

It is recommended that this analysis be performed at 1 Hz.  For a test to be considered valid, the criteria of table 2 shall be met.

|  | Speed | Torque | Power |
| --- | --- | --- | --- |
| Standard error of estimate (SEE) of y on x | max. 100 min-1 | max. 13 per cent of maximum engine torque | max. 8 per cent of maximum engine power |
| Slope of the regression line, a1 | 0.95 to 1.03 | 0.83 - 1.03 | 0.89 - 1.03 |
| Coefficient of determination, r² | min. 0.970 | min. 0.850 | Min. 0.910 |
| y intercept of the regression line, a0 | ± 50 min-1 | ± 20 Nm or  2 per cent of maximum torque whichever is greater | ± 4 kW or  2 per cent of maximum power whichever is greater |

Table 2:

Regression line tolerances

For regression purposes only, point deletions are permitted where noted in table 3 before doing the regression calculation.  However, those points shall not be deleted for the calculation of cycle work and emissions.  Point deletion may be applied to the whole or to any part of the cycle.

| Condition | Points to be deleted |
| --- | --- |
| No load demand (idle point) Actual torque >  2 per cent of max. mapped torque | Speed and power |
| No load demand (motoring point) Reference torque < 0 per cent | Power and torque |
| No load demand Actual speed > 102 per cent reference speed and actual torque < reference torque + 2 per cent of max. mapped torque | Speed and power |
| No load demand Actual torque > reference torque + 2 per cent of max. mapped torque and actual speed < 102 per cent reference speed | Torque and power |
| Full load demand Actual speed < 98 per cent reference speed and actual torque > reference torque - 2 per cent of max. mapped torque | Speed and power |
| Full load demand Actual torque  reference torque - 2 per cent of max. mapped torque and actual speed > 98 per cent reference speed | Torque and power |

Table 3:

Permitted point deletions from regression analysis

8.	EMISSION CALCULATION

The final test result shall be rounded in one step to the number of places to the right of the decimal point indicated by the applicable emission standard plus one additional significant figure, in accordance with ASTM E 29-04.  No rounding of intermediate values leading to the final break-specific emission result is permitted.

8.1.	Dry/wet correction

If the emissions are measured on a dry basis, the measured concentration shall be converted to a wet basis according to the following equation:

(12)

where:

cw is the wet concentration in ppm or per cent volume

cd is the dry concentration in ppm or per cent volume

kw is the dry/wet correction factor

8.1.1.	Raw exhaust gas

kw,a	=		(13)

or

kw,a	=		(14)

or

(15)

with

kf	=	0.055594 x wALF + 0.0080021 x wDEL + 0.0070046 x wEPS	(16)

and

kw1 = 	(17)

where:

Ha	is the intake air humidity, g water per kg dry air

wALF	is the hydrogen content of the fuel, per cent mass

qmf,i	is the instantaneous fuel mass flow rate, kg/s

qmad,I	is the instantaneous dry intake air mass flow rate, kg/s

pr	is the water vapour pressure after cooling bath, kPa

pb	is the total atmospheric pressure, kPa

wDEL	is the nitrogen content of the fuel, per cent mass

wEPS	is the oxygen content of the fuel, per cent mass

	is the molar hydrogen ratio of the fuel

cCO2	is the dry CO2 concentration, per cent

cCO	is the dry CO concentration, per cent

Equations (13) and (14) are principally identical with the factor 1.008 in equations (13) and (15) being an approximation for the more accurate denominator in equation (14).

8.1.2.	Diluted exhaust gas

(18)

or

(19)

with

(20)

where:

	is the	molar hydrogen ratio of the fuel

cCO2w	is the wet CO2 concentration, per cent

cCO2d	is the dry CO2 concentration, per cent

Hd	is the dilution air humidity, g water per kg dry air

Ha	is the intake air humidity, g water per kg dry air

D	is the dilution factor (see paragraph 8.4.2.4.2.)

8.1.3.	Dilution air

(21)

with

(22)

where:

Hd				is the dilution air humidity, g water per kg dry air

8.2.	NOx correction for humidity

As the NOx emission depends on ambient air conditions, the NOx concentration shall be corrected for humidity with the factors given in paragraph 8.2.1. or 8.2.2.  For engines with NOx aftertreatment devices using consumable reagents (e.g. SCR systems), NOx correction is not permitted.  In this case, it is recommended to run the tests with an intake air humidity close to the reference condition of 10.71 g water per kg dry air.

The intake air humidity Ha may be derived from relative humidity measurement, dew point measurement, vapour pressure measurement or dry/wet bulb measurement using generally accepted equations.

8.2.1.	Compression-ignition engines

(23)

where:

Ha	is the intake air humidity, g water per kg dry air

8.2.2.	Positive ignition engines

kh.G	= 0.6272 + 44.030 x 10-3 x Ha – 0.862 x 10-3 x Ha²	(24)

where:

Ha	is the intake air humidity, g water per kg dry air

8.3.	Partial flow dilution (PFS) and raw gaseous measurement

The instantaneous concentration signals of the gaseous components are used for the calculation of the mass emissions by multiplication with the instantaneous exhaust mass flow rate.  The exhaust mass flow rate may be measured directly, or calculated using the methods of intake air and fuel flow measurement, tracer method or intake air and air/fuel ratio measurement.  Special attention shall be paid to the response times of the different instruments.  These differences shall be accounted for by time aligning the signals.  For particulates, the exhaust mass flow rate signals are used for controlling the partial flow dilution system to take a sample proportional to the exhaust mass flow rate.  The quality of proportionality shall be checked by applying a regression analysis between sample and exhaust flow in accordance with paragraph 9.4.6.1.  The complete test set up is schematically shown in figure 6.

Figure 6:

Scheme of raw/partial flow measurement system

8.3.1.	Determination of exhaust gas mass flow

8.3.1.1.	Introduction

For calculation of the emissions in the raw exhaust gas and for controlling of a partial flow dilution system, it is necessary to know the exhaust gas mass flow rate.  For the determination of the exhaust mass flow rate, either of the methods described in paragraphs 8.3.1.3 to 8.3.1.6 may be used.

8.3.1.2.	Response time

For the purpose of emissions calculation, the response time of either method described in paragraphs 8.3.1.3 to 8.3.1.6 shall be equal to or less than the analyzer response time of  ≤  10 s, as required in paragraph 9.3.5.

For the purpose of controlling of a partial flow dilution system, a faster response is required.  For partial flow dilution systems with online control, the response time shall be ≤  0.3 s.  For partial flow dilution systems with look ahead control based on a pre-recorded test run, the response time of the exhaust flow measurement system shall be ≤  5 s with a rise time of ≤  1 s.  The system response time shall be specified by the instrument manufacturer.  The combined response time requirements for the exhaust gas flow and partial flow dilution system are indicated in paragraph 9.4.6.1.

8.3.1.3.	Direct measurement method

Direct measurement of the instantaneous exhaust flow shall be done by systems, such as:

(a)	pressure differential devices, like flow nozzle, (details see ISO 5167)

(b)	ultrasonic flowmeter

(c)	vortex flowmeter

Precautions shall be taken to avoid measurement errors which will impact emission value errors.  Such precautions include the careful installation of the device in the engine exhaust system according to the instrument manufacturers' recommendations and to good engineering practice.  Especially, engine performance and emissions shall not be affected by the installation of the device.

The flowmeters shall meet the linearity requirements of paragraph 9.2.

8.3.1.4.	Air and fuel measurement method

This involves measurement of the airflow and the fuel flow with suitable flowmeters.  The calculation of the instantaneous exhaust gas flow shall be as follows:

qmew,i  = qmaw,i + qmf,i 	(25)

where:

qmew,i	is the instantaneous exhaust mass flow rate, kg/s

qmaw,i	is the instantaneous intake air mass flow rate, kg/s

qmf,i	is the instantaneous fuel mass flow rate, kg/s

The flowmeters shall meet the linearity requirements of paragraph 9.2., but shall be accurate enough to also meet the linearity requirements for the exhaust gas flow.

8.3.1.5.	Tracer measurement method

This involves measurement of the concentration of a tracer gas in the exhaust.

A known amount of an inert gas (e.g. pure helium) shall be injected into the exhaust gas flow as a tracer.  The gas is mixed and diluted by the exhaust gas, but shall not react in the exhaust pipe.  The concentration of the gas shall then be measured in the exhaust gas sample.

In order to ensure complete mixing of the tracer gas, the exhaust gas sampling probe shall be located at least 1 m or 30 times the diameter of the exhaust pipe, whichever is larger, downstream of the tracer gas injection point.  The sampling probe may be located closer to the injection point if complete mixing is verified by comparing the tracer gas concentration with the reference concentration when the tracer gas is injected upstream of the engine.

The tracer gas flow rate shall be set so that the tracer gas concentration at engine idle speed after mixing becomes lower than the full scale of the trace gas analyzer.

The calculation of the exhaust gas flow shall be as follows:

(26)

where:

qmew,i	is the instantaneous exhaust mass flow rate, kg/s

qvt	is tracer gas flow rate,  cm³/min

cmix,i	is the instantaneous concentration of the tracer gas after mixing, ppm

e	is the density of the exhaust gas, kg/m³ (cf. table 4)

cb	is the background concentration of the tracer gas in the intake air, ppm

The background concentration of the tracer gas (cb) may be determined by averaging the background concentration measured immediately before the test run and after the test run.

When the background concentration is less than 1 per cent of the concentration of the tracer gas after mixing (cmix.i) at maximum exhaust flow, the background concentration may be neglected.

The total system shall meet the linearity requirements for the exhaust gas flow of paragraph 9.2.

8.3.1.6.	Airflow and air to fuel ratio measurement method

This involves exhaust mass calculation from the airflow and the air to fuel ratio.  The calculation of the instantaneous exhaust gas mass flow is as follows:

(27)

with

(28)

(29)

where:

qmew,i	is the instantaneous exhaust mass flow rate, kg/s

qmaw,i	is the instantaneous intake air mass flow rate, kg/s

A/Fst	is the stoichiometric air to fuel ratio, kg/kg

i	is the instantaneous excess air ratio

cCO2d	is the dry CO2 concentration, per cent

cCOd	is the dry CO concentration, ppm

cHCw	is the wet HC concentration, ppm

Airflowmeter and analyzers shall meet the linearity requirements of paragraph 9.2., and the total system shall meet the linearity requirements for the exhaust gas flow of paragraph 9.2.

If an air to fuel ratio measurement equipment such as a zirconia type sensor is used for the measurement of the excess air ratio, it shall meet the specifications of paragraph 9.3.2.7.

8.3.2.	Determination of the gaseous components

8.3.2.1.	Introduction

The gaseous components in the raw exhaust gas emitted by the engine submitted for testing shall be measured with the measurement and sampling systems described in paragraph 9.3. and Annex 3.  The data evaluation is described in paragraph 8.3.2.2.

Two calculation procedures are described in paragraphs 8.3.2.3. and 8.3.2.4., which are equivalent for the reference fuels of Annex 2.  The procedure in paragraph 8.3.2.3. is more straightforward, since it uses tabulated u values for the ratio between component and exhaust gas density.  The procedure in paragraph 8.3.2.4. is more accurate for fuel qualities that deviate from the specifications in Annex 2, but requires elementary analysis of the fuel composition.

8.3.2.2.	Data evaluation

For the evaluation of the gaseous emissions, the raw emission concentrations (HC, CO and NOx) and the exhaust gas mass flow rate shall be recorded and stored with at least 2 Hz on a computer system.  All other data shall be recorded with a sample rate of at least 1 Hz.  For analogue analyzers, the response shall be recorded, and the calibration data may be applied online or offline during the data evaluation.

For calculation of the mass emission of the gaseous components, the traces of the recorded concentrations and the trace of the exhaust gas mass flow rate shall be time aligned by the transformation time as defined in paragraph 3.1.28.  Therefore, the response time of each gaseous emissions analyzer and of the exhaust gas mass flow system shall be determined according to paragraphs 8.3.1.2. and 9.3.5., respectively, and recorded.

8.3.2.3.	Calculation of mass emission based on tabulated values

The mass of the pollutants (g/test) shall be determined by calculating the instantaneous mass emissions from the raw concentrations of the pollutants and the exhaust gas mass flow, aligned for the transformation time as determined in accordance with paragraph 8.3.2.2., integrating the instantaneous values over the cycle, and multiplying the integrated values with the u values from table 4.  If measured on a dry basis, the dry/wet correction according to paragraph 8.1. shall be applied to the instantaneous concentration values before any further calculation is done.

For the calculation of NOx, the mass emission shall be multiplied, where applicable, with the humidity correction factor kh,D, or kh,G, as determined according to paragraph 8.2.

The following equation shall be applied:

mgas =		(in g/test)	(30)

where:

ugas	is the ratio between density of exhaust component and density of exhaust gas

cgas,i	is the instantaneous concentration of the component in the exhaust gas, ppm

qmew,i	is the instantaneous exhaust mass flow, kg/s

f	is the data sampling rate, Hz

n	is the number of measurements

An example of the calculation procedures is given in Annex 6.

| Fuel | e |  |  | Gas |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NOx | CO | HC | CO2 | O2 | CH4 |
|  |  |  |  | gas [kg/m3] |  |  |  |
|  |  | 2.053 | 1.250 | a) | 1.9636 | 1.4277 | 0.716 |
|  |  |  |  | ugasb) |  |  |  |
| Diesel | 1.2943 | 0.001586 | 0.000966 | 0.000479 | 0.001517 | 0.001103 | 0.000553 |
| Ethanol | 1.2757 | 0.001609 | 0.000980 | 0.000805 | 0.001539 | 0.001119 | 0.000561 |
| CNGc) | 1.2661 | 0.001621 | 0.000987 | 0.000558d) | 0.001551 | 0.001128 | 0.000565 |
| Propane | 1.2805 | 0.001603 | 0.000976 | 0.000512 | 0.001533 | 0.001115 | 0.000559 |
| Butane | 1.2832 | 0.001600 | 0.000974 | 0.000505 | 0.001530 | 0.001113 | 0.000558 |
| LPGe) | 1.2811 | 0.001602 | 0.000976 | 0.000510 | 0.001533 | 0.001115 | 0.000559 |
| a) depending on fuel b) at  = 2, dry air, 273 K, 101.3 kPa c) u accurate within 0.2 % for mass composition of: C = 66 - 76 %; H = 22 - 25 %; N = 0 - 12 % d) NMHC on the basis of CH2.93 (for total HC the ugas coefficient of CH4 shall be used) e) u accurate within 0.2 % for mass composition of: C3 = 70 - 90 %; C4 = 10 - 30 % |  |  |  |  |  |  |  |

Table 4:

Raw exhaust gas u values and component densities

8.3.2.4.		Calculation of mass emission based on exact equations

The mass of the pollutants (g/test) shall be determined by calculating the instantaneous mass emissions from the raw concentrations of the pollutants, the u values and the exhaust gas mass flow, aligned for the transformation time as determined in accordance with paragraph 8.3.2.2. and integrating the instantaneous values over the cycle.  If measured on a dry basis, the dry/wet correction according to paragraph 8.1. shall be applied to the instantaneous concentration values before any further calculation is done.

For the calculation of NOx, the mass emission shall be multiplied with the humidity correction factor kh,D, or kh,G, as determined according to paragraph 8.2.

The following equation shall be applied:

mgas  =		(in g/test)	(31)

where:

ugas,i	is the instantaneous density ratio of exhaust component and exhaust gas

cgas,i	is the instantaneous concentration of the component in the exhaust gas, ppm

qmew,i	is the instantaneous exhaust mass flow, kg/s

f	is the data sampling rate, Hz

n	is the number of measurements

The instantaneous u values shall be calculated as follows:

ugas,i  = Mgas / (Me,i x 1,000)	(32)

or

ugas,i =gas / (e,i x 1,000)	(33)

with

gas = Mgas / 22.414 	(34)

where:

Mgas	is the molar mass of the gas component, g/mol (cf. Annex 6)

Me,i	is the instantaneous molar mass of the exhaust gas, g/mol

gas	is the density of the gas component, kg/m3

e,i	is the instantaneous density of the exhaust gas, kg/m3

The molar mass of the exhaust, Me, shall be derived for a general fuel composition CHONSunder the assumption of complete combustion, as follows:

(35)

where:

qmaw,i	is the instantaneous intake air mass flow rate on wet basis, kg/s

qmf,i	is the instantaneous fuel mass flow rate, kg/s

Ha	is the intake air humidity, g water per kg dry air

Ma	is the molar mass of the dry intake air = 28.965 g/mol

The exhaust density e shall be derived, as follows:

(36)

where:

qmad,i	is the instantaneous intake air mass flow rate on dry basis, kg/s

qmf,i	is the instantaneous fuel mass flow rate, kg/s

Ha	is the intake air humidity, g water per kg dry air

kf	is the fuel specific factor according to equation 11 in paragraph 8.1.1.

8.3.3.	Particulate determination

8.3.3.1.	Data evaluation

The tare weight of the filter, as determined according to paragraph 7.8.2.2., shall be subtracted from the gross weight of the filter, as determined according to paragraph 7.8.4.5., which results in the particulate sample mass mf.  For the evaluation of the particulate concentration, the total sample mass (msep) through the filter over the test cycle shall be recorded.

With the prior approval of the type approval or certification authority, the particulate mass may be corrected for the particulate level of the dilution air, as determined in paragraph 7.8.2.5., in line with good engineering practice and the specific design features of the particulate measurement system used.

8.3.3.2.	Calculation of mass emission

Depending on system design, the mass of particulates (g/test) shall be calculated by either of the methods in paragraphs 8.3.3.2.1. or 8.3.3.2.2 after buoyancy correction of the particulate sample filter according to paragraph 8.5.3.  An example of the calculation procedures is given in Annex 6.

8.3.3.2.1.	Calculation based on sample ratio

mPM = mf / (rs x 1,000)	(37)

where:

mf	is the particulate mass sampled over the cycle, mg

rs	is the average sample ratio over the test cycle

with

rs	=		(38)

where:

mse	is the sample mass over the cycle, kg

mew	is the total exhaust mass flow over the cycle, kg

msep	is the mass of diluted exhaust gas passing the particulate collection filters, kg

msed	is the mass of diluted exhaust gas passing the dilution tunnel, kg

In case of the total sampling type system, msep and msed are identical.

8.3.3.2.2.	Calculation based on dilution ratio

mPM =  	(39)

where:

mf	is the particulate mass sampled over the cycle, mg

msep	is the mass of diluted exhaust gas passing the particulate collection filters, kg

medf	is the mass of equivalent diluted exhaust gas over the cycle, kg

The total mass of equivalent diluted exhaust gas mass over the cycle shall be determined as follows:

medf	= 	(40)

qmedf,I	= qmew,i x rd,i	(41)

rd,i	= 	(42)

where:

qmedf,i	is the instantaneous equivalent diluted exhaust mass flow rate, kg/s

qmew,i	is the instantaneous exhaust mass flow rate, kg/s

rd,i	is the instantaneous dilution ratio

qmdew,i	is the instantaneous diluted exhaust mass flow rate, kg/s

qmdw,i	is the instantaneous dilution air mass flow rate, kg/s

f	is the data sampling rate, Hz

n	is the number of measurements

8.4.	Full flow dilution measurement (CVS)

The concentration signals, either by integration over the cycle or by bag sampling, of the gaseous components shall be used for the calculation of the mass emissions by multiplication with the diluted exhaust mass flow rate.  The exhaust mass flow rate shall be measured with a constant volume sampling (CVS) system, which may use a positive displacement pump (PDP), a critical flow venturi (CFV) or a subsonic venturi (SSV) with or without flow compensation.

For bag sampling and particulate sampling, a proportional sample shall be taken from the diluted exhaust gas of the CVS system.  For a system without flow compensation, the ratio of sample flow to CVS flow must not vary by more than ± 2.5 per cent from the set point of the test.  For a system with flow compensation, each individual flow rate shall be constant within ± 2.5 per cent of its respective target flow rate.

The complete test set up is schematically shown in figure 7.

Figure 7:

Scheme of full flow measurement system

8.4.1.	Determination of the diluted exhaust gas flow

8.4.1.1.	Introduction

For calculation of the emissions in the diluted exhaust gas, it is necessary to know the diluted exhaust gas mass flow rate.  The total diluted exhaust gas flow over the cycle (kg/test) shall be calculated from the measurement values over the cycle and the corresponding calibration data of the flow measurement device (V0 for PDP, KV for CFV, Cd for SSV) by either of the methods described in paragraphs 8.4.1.2. to 8.4.1.4.  If the total sample flow of particulates (msep) exceeds 0.5 per cent of the total CVS flow (med), the CVS flow shall be corrected for msep or the particulate sample flow shall be returned to the CVS prior to the flow measuring device.

8.4.1.2.	PDP-CVS system

The calculation of the mass flow over the cycle is as follows, if the temperature of the diluted exhaust is kept within ± 6 K over the cycle by using a heat exchanger:

med	= 1.293 x V0 x nP x pp x 273 / (101.3 x T)	(43)

where:

V0	is the volume of gas pumped per revolution under test conditions, m³/rev

nP	is the total revolutions of pump per test

pp	is the absolute pressure at pump inlet, kPa

T	is the average temperature of the diluted exhaust gas at pump inlet, K

If a system with flow compensation is used (i.e. without heat exchanger), the instantaneous mass emissions shall be calculated and integrated over the cycle.  In this case, the instantaneous mass of the diluted exhaust gas shall be calculated as follows:

med,i	= 	1.293 x V0 x nP,i x pp  x 273 / (101.3 x T)	(44)

where:

nP,i	is the total revolutions of pump per time interval

8.4.1.3.	CFV-CVS system

The calculation of the mass flow over the cycle is as follows, if the temperature of the diluted exhaust is kept within ± 11 K over the cycle by using a heat exchanger:

med	=	1.293 x t x Kv x pp / T 0.5	(45)

where:

t	is the cycle time, s

KV	is the calibration coefficient of the critical flow venturi for standard conditions,

pp	is the absolute pressure at venturi inlet, kPa

T	is the absolute temperature at venturi inlet, K

If a system with flow compensation is used (i.e. without heat exchanger), the instantaneous mass emissions shall be calculated and integrated over the cycle.  In this case, the instantaneous mass of the diluted exhaust gas shall be calculated as follows:

med,i	=	1.293 x ti x KV x pp / T 0.5	(46)

where:

ti	is the time interval, s

8.4.1.4.	SSV-CVS system

The calculation of the mass flow over the cycle shall be as follows, if the temperature of the diluted exhaust is kept within ± 11 K over the cycle by using a heat exchanger:

med	=	1.293 x QSSV	(47)

with

(48)

where:

A0	is 0.006111 in SI units of

dV	is the diameter of the SSV throat, m

Cd	is the discharge coefficient of the SSV

pp	is the absolute pressure at venturi inlet, kPa

T	is the temperature at the venturi inlet, K

rp	is the ratio of the SSV throat to inlet absolute static pressure,

rD	is the ratio of the SSV throat diameter, d, to the inlet pipe inner diameter D

If a system with flow compensation is used (i.e. without heat exchanger), the instantaneous mass emissions shall be calculated and integrated over the cycle.  In this case, the instantaneous mass of the diluted exhaust gas shall be calculated as follows:

med	=	1.293 x QSSV x ti	(49)

where:

ti	is the time interval, s

The real time calculation shall be initialized with either a reasonable value for Cd, such as 0.98, or a reasonable value of Qssv.  If the calculation is initialized with Qssv, the initial value of Qssv shall be used to evaluate the Reynolds number.

During all emissions tests, the Reynolds number at the SSV throat shall be in the range of Reynolds numbers used to derive the calibration curve developed in paragraph 9.5.4.

8.4.2.	Determination of the gaseous components

8.4.2.1.	Introduction

The gaseous components in the diluted exhaust gas emitted by the engine submitted for testing shall be measured by the methods described in Annex 3.  Dilution of the exhaust shall be done with filtered ambient air, synthetic air or nitrogen.  The flow capacity of the full flow system shall be large enough to completely eliminate water condensation in the dilution and sampling systems.  Data evaluation and calculation procedures are described in paragraphs 8.4.2.2. and 8.4.2.3.

8.4.2.2.	Data evaluation

For continuous sampling, the emission concentrations (HC, CO and NOx) shall be recorded and stored with at least 1 Hz on a computer system, for bag sampling one mean value per test is required.  The diluted exhaust gas mass flow rate and all other data shall be recorded with a sample rate of at least 1 Hz.  For analogue analyzers the response will be recorded, and the calibration data may be applied online or offline during the data evaluation.

8.4.2.3.	Calculation of mass emission

8.4.2.3.1.	Systems with constant mass flow

For systems with heat exchanger, the mass of the pollutants shall be determined from the following equation:

mgas	=	ugas x cgas x med 	(in g/test)	(50)

where:

ugas	is the ratio between density of exhaust component and density of air

cgas	is the average background corrected concentration of the component, ppm

med	is the total diluted exhaust mass over the cycle, kg

If measured on a dry basis, the dry/wet correction according to paragraph 8.1. shall be applied.

For the calculation of NOx, the mass emission shall be multiplied, if applicable, with the humidity correction factor kh,D, or kh,G, as determined according to paragraph 8.2.

The u values are given in table 5.  For calculating the ugas values, the density of the diluted exhaust gas has been assumed to be equal to air density.  Therefore, the ugas values are identical for single gas components, but different for HC.

| Fuel | de |  |  | Gas |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NOx | CO | HC | CO2 | O2 | CH4 |
|  |  |  |  | gas [kg/m3] |  |  |  |
|  |  | 2.053 | 1.250 | a) | 1.9636 | 1.4277 | 0.716 |
|  |  |  |  | ugasb) |  |  |  |
| Diesel | 1.293 | 0.001588 | 0.000967 | 0.000480 | 0.001519 | 0.001104 | 0.000553 |
| Ethanol | 1.293 | 0.001588 | 0.000967 | 0.000795 | 0.001519 | 0.001104 | 0.000553 |
| CNGc) | 1.293 | 0.001588 | 0.000967 | 0.000584d) | 0.001519 | 0.001104 | 0.000553 |
| Propane | 1.293 | 0.001588 | 0.000967 | 0.000507 | 0.001519 | 0.001104 | 0.000553 |
| Butane | 1.293 | 0.001588 | 0.000967 | 0.000501 | 0.001519 | 0.001104 | 0.000553 |
| LPGe) | 1.293 | 0.001588 | 0.000967 | 0.000505 | 0.001519 | 0.001104 | 0.000553 |
| a) depending on fuel b) at  = 2, dry air, 273 K, 101.3 kPa c) u accurate within 0.2 % for mass composition of: C = 66 - 76 %; H = 22 - 25 %; N = 0 - 12 % d) NMHC on the basis of CH2.93 (for total HC the ugas coefficient of CH4 shall be used) e) u accurate within 0.2 % for mass composition of: C3 = 70 - 90 %; C4 = 10 - 30 % |  |  |  |  |  |  |  |

Table 5:

Diluted exhaust gas u values and component densities

Alternatively, the exact calculation method described in paragraph 8.3.2.4., equations 32 or 33, may be used for calculating the u values.

8.4.2.3.2.	Determination of the background corrected concentrations

The average background concentration of the gaseous pollutants in the dilution air shall be subtracted from the measured concentrations to get the net concentrations of the pollutants.  The average values of the background concentrations can be determined by the sample bag method or by continuous measurement with integration.  The following equation shall be used:

cgas	=	cgas,e - cd x (1 - (1/D))	(51)

where:

cgas,e	is the concentration of the component measured in the diluted exhaust gas, ppm

cd	is the concentration of the component measured in the dilution air, ppm

D	is the dilution factor

The dilution factor shall be calculated as follows:

a)	for diesel and LPG fueled gas engines

D	=		(52)

b)	for NG fueled gas engines

D	=		(53)

where:

cCO2,e	is the wet concentration of CO2 in the diluted exhaust gas, per cent vol

cHC,e	is the wet concentration of HC in the diluted exhaust gas, ppm C1

cNMHC,e	is the wet concentration of NMHC in the diluted exhaust gas, ppm C1

cCO,e	is the wet concentration of CO in the diluted exhaust gas, ppm

FS	is the stoichiometric factor

The stoichiometric factor shall be calculated as follows:

FS	=		(54)

where:

	is the molar hydrogen ratio of the fuel (H/C)

Alternatively, if the fuel composition is not known, the following stoichiometric factors may be used:

FS (diesel)	=	13.4

FS (LPG)	=	11.6

FS (NG)	=	  9.5

8.4.2.3.3	Systems with flow compensation

For systems without heat exchanger, the mass of the pollutants (g/test) shall be determined by calculating the instantaneous mass emissions and integrating the instantaneous values over the cycle.  Also, the background correction shall be applied directly to the instantaneous concentration value.  The following equation shall be applied:

mgas 	= 		(55)

where:

cgas,e	is the concentration of the component measured in the diluted exhaust gas, ppm

cd	is the concentration of the component measured in the dilution air, ppm

med,i	is the instantaneous mass of the diluted exhaust gas, kg

med	is the total mass of diluted exhaust gas over the cycle, kg

ugas	is the tabulated value from table 5

D	is the dilution factor

8.4.3.	Particulate determination

8.4.3.1.	Calculation of mass emission

The particulate mass (g/test) shall be calculated after buoyancy correction of the particulate sample filter according to paragraph 8.5.3., as follows:

mPM =  	(56)

where:

mf	is the particulate mass sampled over the cycle and buoyancy corrected, mg

msep 	is the mass of diluted exhaust gas passing the particulate collection filters, kg

med 	is the mass of diluted exhaust gas over the cycle, kg

with

msep	=	mset - mssd	(57)

where:

mset 	is the mass of double diluted exhaust gas through particulate filter, kg

mssd	is the mass of secondary dilution air, kg

If the particulate background level of the dilution air is determined in accordance with paragraph 7.8.2.5, the particulate mass may be background corrected.  In this case, the particulate mass (g/test) shall be calculated as follows:

mPM = 	(58)

where:

msep	is the mass of diluted exhaust gas passing the particulate collection filters, kg

med	is the mass of diluted exhaust gas over the cycle, kg

msd	is the mass of dilution air sampled by background particulate sampler, kg

mb	is the mass of the collected background particulates of the dilution air, mg

D	is the dilution factor as determined in paragraph 8.4.2.4.2.

8.5.	General calculations

8.5.1.	Drift correction

If drift correction is applied in accordance with paragraph 7.8.4., the corrected concentration value shall be calculated as follows:

(59)

where:

cref,z 	is the reference concentration of the zero gas (usually zero), ppm

cref,s 	is the reference concentration of the span gas, ppm

cpre,z 	is the pre-test analyzer concentration of the zero gas, ppm

cpre,s 	is the pre-test analyzer concentration of the span gas, ppm,

cpost,z is the post-test analyzer concentration of the zero gas, ppm,

cpost,s is the post-test analyzer concentration of the span gas, ppm

Two sets of brake-specific emission results shall be calculated for each component in accordance with paragraphs 8.3. and/or 8.4. One set shall be calculated using uncorrected concentrations and another set shall be calculated using the concentrations corrected for drift according to equation 59.

Depending on the measurement system and calculation method used, the uncorrected emissions results shall be calculated with equations 30, 31 50, 51 or 55, respectively.  For calculation of the corrected emissions, cgas in equations 30, 31 50, 51 or 55, respectively, shall be replaced with ccor of equation 59.  If instantaneous concentration values cgas,i are used in the respective equation, the corrected value shall also be applied as instantaneous value ccor,i.  In equation 51, the correction shall be applied to both the measured and the background concentration.

The comparison shall be made as a percentage of the uncorrected results.  The difference between the uncorrected and the corrected brake-specific emission values shall be within ± 4 per cent of the uncorrected brake-specific emission values.  If the drift is greater than 4 per cent, the test shall be voided.

If drift correction is applied, only the drift-corrected emission results shall be used when reporting  emissions.

8.5.2.	Calculation of NMHC and CH4 with the non-methane cutter

The concentration of NMHC and CH4 shall be calculated as follows:

(60)

(61)

where:

cHC(w/NMC)	is the HC concentration with sample gas flowing through the NMC, ppm

cHC(w/oNMC)	is the HC concentration with sample gas bypassing the NMC, ppm

EM	is the methane efficiency as determined per paragraph 9.3.8.1.

EE	is the ethane efficiency as determined per paragraph 9.3.8.2.

8.5.3.	Particulate filter buoyancy correction (old para 9.4.3.5.)

The sampling filter shall be corrected for its buoyancy in air.  The buoyancy correction depends on sampling filter density, air density and the density of the balance calibration weight, and does not account for the buoyancy of the PM itself.

If the density of the filter material is not known, the following densities shall be used:

(a)	teflon coated glass fiber filter: 2,300 kg/m3

(b)	teflon membrane filter: 2,144 kg/m3

(c)	teflon membrane filter with polymethylpentene support ring: 920 kg/m3

For stainless steel calibration weights, a density of 8,000 kg/m3 shall be used.  If the material of the calibration weight is different, its density must be known.

The following equation shall be used:

(62)

with

(63)

where:

muncor	is the uncorrected particulate sample mass, mg

ρa	is the density of the air, kg/m3

ρw	is the density of balance calibration weight, kg/m3

ρf	is the density of the particulate sampling filter, kg/m3

pb	is the total atmospheric pressure, kPa

Ta	is the air temperature in the balance environment, K

28.836	is the molar mass of the air at reference humidity (282.5 K), g/mol

8.3144	is the molar gas constant

8.5.4.	Calculation of the specific emissions

The specific emissions egas or ePM (g/kWh) shall be calculated for each individual component in the following ways depending on the type of test cycle.

8.5.4.1.	Test result

For the WHSC, hot WHTC, or cold WHTC, the following formula shall be applied:

(64)

where:

m	is the mass emission of the component, g/test

Wact	is the	actual cycle work as determined according to paragraph 7.7.1., kWh

For the WHTC, the final test result shall be a weighted average from cold start test and hot start test by using either of the following options:

(65a)

(65b)

The option shall be selected by the Contracting Parties.

9.	EQUIPMENT SPECIFICATION AND VERIFICATION

This gtr does not contain details of flow, pressure, and temperature measuring equipment or systems.  Instead, only the linearity requirements of such equipment or systems necessary for conducting an emissions test are given in paragraph 9.2.

9.1.	Dynamometer specification

An engine dynamometer with adequate characteristics to perform the appropriate test cycle described in paragraphs 7.2. and 7.3. shall be used.

The instrumentation for torque and speed measurement shall allow the measurement accuracy of the shaft power as needed to comply with the cycle validation criteria.  Additional calculations may be necessary.  The accuracy of the measuring equipment shall be such that the linearity requirements given in paragraph 9.2., table 6 are not exceeded.

9.2.	Linearity requirements

The calibration of all measuring instruments and systems shall be traceable to national (international) standards.  The measuring instruments and systems shall comply with the linearity requirements given in table 6.  The linearity verification according to paragraph 9.2.1. shall be performed for the gas analyzers at least every 3 months or whenever a system repair or change is made that could influence calibration.  For the other instruments and systems, the linearity verification shall be done as required by internal audit procedures, by the instrument manufacturer or in accordance with ISO 9000 requirements.

| Measurement system |  | Slope a1 | Standard error SEE | Coefficient of determination r2 |
| --- | --- | --- | --- | --- |
| Engine speed | ≤ 0.05 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Engine torque | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Fuel flow | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Airflow | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Exhaust gas flow | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Dilution air flow | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Diluted exhaust gas flow | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Sample flow | ≤ 1 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Gas analyzers | ≤ 0.5 % max | 0.99 - 1.01 | ≤ 1 % max | ≥ 0.998 |
| Gas dividers | ≤ 0.5 % max | 0.98 - 1.02 | ≤ 2 % max | ≥ 0.990 |
| Temperatures | ≤ 1 % max | 0.99 - 1.01 | ≤ 1 % max | ≥ 0.998 |
| Pressures | ≤ 1 % max | 0.99 - 1.01 | ≤ 1 % max | ≥ 0.998 |
| PM balance | ≤ 1 % max | 0.99 - 1.01 | ≤ 1 % max | ≥ 0.998 |

Table 6:

Linearity requirements of instruments and measurement systems

9.2.1.	Linearity verification

9.2.1.1.	Introduction

A linearity verification shall be performed for each measurement system listed in table 6. At least 10 reference values, or as specified otherwise, shall be introduced to the measurement system, and the measured values shall be compared to the reference values by using a least squares linear regression in accordance with equation 11.  The maximum limits in table 6 refer to the maximum values expected during testing.

9.2.1.2.	General requirements

The measurement systems shall be warmed up according to the recommendations of the instrument manufacturer.  The measurement systems shall be operated at their specified temperatures, pressures and flows.

9.2.1.3.	Procedure

The linearity verification shall be run for each normally used operating range with the following steps.

(a)	The instrument shall be set at zero by introducing a zero signal.  For gas analyzers, purified synthetic air (or nitrogen) shall be introduced directly to the analyzer port.

(b)	The instrument shall be spanned by introducing a span signal.  For gas analyzers, an appropriate span gas shall be introduced directly to the analyzer port.

(c)	The zero procedure of (a) shall be repeated.

(d)	The verification shall be established by introducing at least 10 reference values (including zero) that are within the range from zero to the highest values expected during emission testing.  For gas analyzers, known gas concentrations shall be introduced directly to the analyzer port.

(e)	At a recording frequency of at least 1 Hz, the reference values shall be measured and the measured values recorded for 30 s.

(f)	The arithmetic mean values over the 30 s period shall be used to calculate the least squares linear regression parameters according to equation 6 in paragraph 7.7.2.

(g)	The linear regression parameters shall meet the requirements of paragraph 9.2., table 6.

(h)	The zero setting shall be rechecked and the verification procedure repeated, if necessary.

9.3.	Gaseous emissions measurement and sampling system

9.3.1.	Analyzer specifications

9.3.1.1.	General

The analyzers shall have a measuring range and response time appropriate for the accuracy required to measure the concentrations of the exhaust gas components under transient and steady state conditions.

The electromagnetic compatibility (EMC) of the equipment shall be on a level as to minimize additional errors.

9.3.1.2.	Accuracy

The accuracy, defined as the deviation of the analyzer reading from the reference value, shall not exceed ± 2 per cent of the reading or ± 0.3 per cent of full scale whichever is larger.

9.3.1.3.	Precision

The precision, defined as 2.5 times the standard deviation of 10 repetitive responses to a given calibration or span gas, shall be no greater than 1 per cent of full scale concentration for each range used above 155 ppm (or ppm C) or 2 per cent of each range used below 155 ppm (or ppm C).

9.3.1.4.	Noise

The analyzer peak-to-peak response to zero and calibration or span gases over any 10 seconds period shall not exceed 2 per cent of full scale on all ranges used.

9.3.1.5.	Zero drift

The drift of the zero response shall be less than 2 per cent of full scale on the range used, as determined in accordance with paragraph 7.8.4.  Zero response is defined as the mean response, including noise, to a zero gas during a 30 s time interval.  If the zero drift is greater than 2 per cent, drift correction may be applied in accordance with paragraph 8.5.1.

9.3.1.6.	Span drift

The drift of the span response shall be less than 2 per cent of full scale on the range used, as determined in accordance with paragraph 7.8.4.  Span response is defined as the mean response, including noise, to a span gas during a 30 s time interval.  If the span drift is greater than 2 per cent, drift correction may be applied in accordance with paragraph 8.5.1.

9.3.1.7.	Rise time

The rise time of the analyzer installed in the measurement system shall not exceed 2.5 s.

9.3.1.8.	Gas drying

Exhaust gases may be measured wet or dry.  A gas-drying device, if used, shall have a minimal effect on the composition of the measured gases.  Chemical dryers are not an acceptable method of removing water from the sample.

9.3.2.	Gas analyzers

9.3.2.1.	Introduction

Paragraphs 9.3.2.2 to 9.2.3.7 describe the measurement principles to be used.  A detailed description of the measurement systems is given in Annex 3.  The gases to be measured shall be analyzed with the following instruments.  For non-linear analyzers, the use of linearizing circuits is permitted.

9.3.2.2.	Carbon monoxide (CO) analysis

The carbon monoxide analyzer shall be of the non-dispersive infrared (NDIR) absorption type.

9.3.2.3.	Carbon dioxide (CO2) analysis

The carbon dioxide analyzer shall be of the non-dispersive infrared (NDIR) absorption type.

9.3.2.4.	Hydrocarbon (HC) analysis

The hydrocarbon analyzer shall be of the heated flame ionization detector (HFID) type with detector, valves, pipework, etc. heated so as to maintain a gas temperature of 463 K ± 10 K (190 ± 10 °C).  Optionally, for NG fueled and PI engines, the hydrocarbon analyzer may be of the non-heated flame ionization detector (FID) type depending upon the method used (see Annex 3, paragraph A.3.1.3.).

9.3.2.5.	Non-methane hydrocarbon (NMHC) analysis

The determination of the non-methane hydrocarbon fraction shall be performed with a heated non-methane cutter (NMC) operated in line with an FID as per Annex 3, paragraph A.3.1.4. by subtraction of the methane from the hydrocarbons.

9.3.2.6.	Oxides of nitrogen (NOx) analysis

Two measurement instruments are specified for NOx measurement and either instrument may be used provided it meets the criteria specified in 9.3.2.6.1. or 9.3.2.6.2., respectively. For determination of system equivalency of an alternate measurement procedure in accordance with paragraph 5.1.1., only the CLD is permitted.

9.3.2.6.1.	Chemiluminescent detector (CLD)

If measured on a dry basis, the oxides of nitrogen analyzer shall be of the chemiluminescent detector (CLD) or heated chemiluminescent detector (HCLD) type with a NO2/NO converter.  If measured on a wet basis, a HCLD with converter maintained above 328 K (55 °C) shall be used, provided the water quench check (see paragraph 9.3.9.2.2.) is satisfied.  For both CLD and HCLD, the sampling path shall be maintained at a wall temperature of 328 K to 473 K (55 °C to 200 °C) up to the converter for dry measurement, and up to the analyzer for wet measurement.

9.3.2.6.2.	Non-dispersive ultraviolet detector (NDUV)

A nondispersive ultraviolet (NDUV) analyzer shall be used to measure NOx concentration.  If the NDUV analyzer measures only NO, a NO2-to-NO converter that meets the verification in paragraph 9.3.6. shall be placed upstream of the NDUV analyzer. The NDUV temperature shall be maintained to prevent aqueous condensation, unless one of the following configurations is used:

(a) An NDUV shall be connected downstream of any dryer or chiller that is downstream of an NO2-to-NO converter that meets the verification in paragraph 9.3.6.

(b) An NDUV shall be connected downstream of any dryer or chiller that meets the verification in paragraph 9.3.9.2.4.

9.3.2.7.	Air to fuel measurement

The air to fuel measurement equipment used to determine the exhaust gas flow as specified in paragraph 8.3.1.6. shall be a wide range air to fuel ratio sensor or lambda sensor of Zirconia type.  The sensor shall be mounted directly on the exhaust pipe where the exhaust gas temperature is high enough to eliminate water condensation.

The accuracy of the sensor with incorporated electronics shall be within:

 3 per cent of reading	for 	 < 2

 5 per cent of reading	for	2   < 5

 10 per cent of reading	for	5  

To fulfill the accuracy specified above, the sensor shall be calibrated as specified by the instrument manufacturer.

9.3.3.	Calibration gases

The shelf life of all calibration gases shall be respected.  The expiration date of the calibration gases stated by the manufacturer shall be recorded.

9.3.3.1.	Pure gases

The required purity of the gases is defined by the contamination limits given below.  The following gases shall be available for operation:

a)	For raw exhaust gas

Purified nitrogen

(Contamination  1 ppm C1,   1 ppm CO,   400 ppm CO2,  0.1 ppm NO)

Purified oxygen

(Purity  99.5 per cent vol O2)

Hydrogen-helium mixture

(40 ± 2 per cent hydrogen, balance helium)

(Contamination  1 ppm C1,  400 ppm CO2)

Purified synthetic air

(Contamination  1 ppm C1,  1 ppm CO,  400 ppm CO2,  0.1 ppm NO)

(Oxygen content between 18-21 per cent vol.)

b)	For dilute exhaust gas (optionally for raw exhaust gas)

Purified nitrogen

(Contamination  0.05 ppm C1,   1 ppm CO,   10 ppm CO2,  0.02 ppm NO)

Purified oxygen

(Purity  99.5 per cent vol O2)

Hydrogen-helium mixture

(40 ± 2 per cent hydrogen, balance helium)

(Contamination  0.05 ppm C1,  10 ppm CO2)

Purified synthetic air

(Contamination  0.05 ppm C1,   1 ppm CO,   10 ppm CO2,  0.02 ppm NO)

(Oxygen content between 20.5 - 21.5 per cent vol.)

9.3.3.2.	Calibration and span gases

Mixtures of gases having the following chemical compositions shall be available.  Other gas combinations are allowed provided the gases do not react with one another.

C3H8 and purified synthetic air (see paragraph 9.3.3.1.);

CO and purified nitrogen;

NOx and purified nitrogen (the amount of NO2 contained in this calibration gas shall not exceed 5 per cent of the NO content);

NO2 and purified synthetic air (for NDUV analyzer, if applicable)

CO2 and purified nitrogen;

CH4 and purified synthetic air;

C2H6 and purified synthetic air

The true concentration of a calibration and span gas shall be within ± 1 per cent of the nominal value, and shall be traceable to national or international standards.  All concentrations of calibration gas shall be given on a volume basis (volume percent or volume ppm).

9.3.3.3.	Gas dividers

The gases used for calibration and span may also be obtained by means of gas dividers (precision blending devices), diluting with purified N2 or with purified synthetic air.  The accuracy of the gas divider shall be such that the concentration of the blended calibration gases is accurate to within ± 2 per cent.  This accuracy implies that primary gases used for blending shall be known to an accuracy of at least  1 per cent, traceable to national or international gas standards.  The verification shall be performed at between 15 and 50 per cent of full scale for each calibration incorporating a gas divider. An additional verification may be performed using another calibration gas, if the first verification has failed.

Optionally, the blending device may be checked with an instrument which by nature is linear, e.g. using NO gas with a CLD.  The span value of the instrument shall be adjusted with the span gas directly connected to the instrument.  The gas divider shall be checked at the settings used and the nominal value shall be compared to the measured concentration of the instrument.  This difference shall in each point be within ± 1 per cent of the nominal value.

For conducting the linearity verification according to paragraph 9.2.1., the gas divider shall be accurate to within  1 per cent.

9.3.3.4.	Oxygen interference check gases

Oxygen interference check gases are a blend of propane, oxygen and nitrogen.  They shall contain propane with 350 ppm C  75 ppm C hydrocarbon.  The concentration value shall be determined to calibration gas tolerances by chromatographic analysis of total hydrocarbons plus impurities or by dynamic blending.  The oxygen concentrations required for positive ignition and compression ignition engine testing are listed in table 7 with the remainder being purified nitrogen.

| Type of engine | O2 concentration (per cent) |
| --- | --- |
| Compression ignition | 21 (20 to 22) |
| Compression and positive ignition | 10 (9 to 11) |
| Compression and positive ignition | 5 (4 to 6) |
| Positive ignition | 0 (0 to 1) |

Table 7:

Oxygen interference check gases

9.3.4.	Leakage test

A system leakage test shall be performed.  The probe shall be disconnected from the exhaust system and the end plugged.  The analyzer pump shall be switched on.  After an initial stabilization period all flowmeters will read approximately zero in the absence of a leak.  If not, the sampling lines shall be checked and the fault corrected.

The maximum allowable leakage rate on the vacuum side shall be 0.5 per cent of the in-use flow rate for the portion of the system being checked.  The analyzer flows and bypass flows may be used to estimate the in-use flow rates.

Alternatively, the system may be evacuated to a pressure of at least 20 kPa vacuum (80 kPa absolute).  After an initial stabilization period the pressure increase p (kPa/min) in the system shall not exceed:

p = p / Vs  x 0.005 x qvs	(66)

where:

Vs	is the system volume, l

qvs	is the system flow rate, l/min

Another method is the introduction of a concentration step change at the beginning of the sampling line by switching from zero to span gas.  If for a correctly calibrated analyzer after an adequate period of time the reading is ≤  99 per cent compared to the introduced concentration, this points to a leakage problem that shall be corrected.

9.3.5.	Response time check of the analytical system

The system settings for the response time evaluation shall be exactly the same as during measurement of the test run (i.e. pressure, flow rates, filter settings on the analyzers and all other response time influences).  The response time determination shall be done with gas switching directly at the inlet of the sample probe.  The gas switching shall be done in less than 0.1 s.  The gases used for the test shall cause a concentration change of at least 60 per cent full scale (FS).

The concentration trace of each single gas component shall be recorded.  The response time is defined to be the difference in time between the gas switching and the appropriate change of the recorded concentration.  The system response time (t90) consists of the delay time to the measuring detector and the rise time of the detector.  The delay time is defined as the time from the change (t0) until the response is 10 per cent of the final reading (t10).  The rise time is defined as the time between 10 per cent and 90 per cent response of the final reading (t90 – t10).

For time alignment of the analyzer and exhaust flow signals, the transformation time is defined as the time from the change (t0) until the response is 50 per cent of the final reading (t50).

The system response time shall be  10 s with a rise time of   2.5 s in accordance with paragraph 9.3.1.7. for all limited components (CO, NOx, HC or NMHC) and all ranges used.  When using a NMC for the measurement of NMHC, the system response time may exceed 10 s.

9.3.6.	Efficiency test of NOx converter

The efficiency of the converter used for the conversion of NO2 into NO is tested as given in paragraphs 9.3.6.1 to 9.3.6.8 (see figure 8).

Figure 8:

Scheme of NO2 converter efficiency device

9.3.6.1.	Test setup

Using the test setup as schematically shown in figure 8 and the procedure below, the efficiency of the converter shall be tested by means of an ozonator.

9.3.6.2.	Calibration

The CLD and the HCLD shall be calibrated in the most common operating range following the manufacturer's specifications using zero and span gas (the NO content of which shall amount to about 80 per cent of the operating range and the NO2 concentration of the gas mixture to less than 5 per cent of the NO concentration).  The NOx analyzer shall be in the NO mode so that the span gas does not pass through the converter.  The indicated concentration has to be recorded.

9.3.6.3.	Calculation

The per cent efficiency of the converter shall be calculated as follows:

(67)

where:

a	is the NOx concentration according to paragraph 9.3.6.6.

b	is the NOx concentration according to paragraph 9.3.6.7.

c	is the NO concentration according to paragraph 9.3.6.4.

d	is the NO concentration according to paragraph 9.3.6.5.

9.3.6.4.	Adding of oxygen

Via a T-fitting, oxygen or zero air shall be added continuously to the gas flow until the concentration indicated is about 20 per cent less than the indicated calibration concentration given in paragraph 9.3.6.2. (the analyzer is in the NO mode).

The indicated concentration (c) shall be recorded.  The ozonator is kept deactivated throughout the process.

9.3.6.5.	Activation of the ozonator

The ozonator shall be activated to generate enough ozone to bring the NO concentration down to about 20 per cent (minimum 10 per cent) of the calibration concentration given in paragraph 9.3.6.2.  The indicated concentration (d) shall be recorded (the analyzer is in the NO mode).

9.3.6.6.	NOx mode

The NO analyzer shall be switched to the NOx mode so that the gas mixture (consisting of NO, NO2, O2 and N2) now passes through the converter.  The indicated concentration (a) shall be recorded (the analyzer is in the NOx mode).

9.3.6.7.	Deactivation of the ozonator

The ozonator is now deactivated.  The mixture of gases described in paragraph 9.3.6.6. passes through the converter into the detector.  The indicated concentration (b) shall be recorded (the analyzer is in the NOx mode).

9.3.6.8.	NO mode

Switched to NO mode with the ozonator deactivated, the flow of oxygen or synthetic air shall be shut off.  The NOx reading of the analyzer shall not deviate by more than ± 5 per cent from the value measured according to paragraph 9.3.6.2. (the analyzer is in the NO mode).

9.3.6.9.	Test interval

The efficiency of the converter shall be tested at least once per month.

9.3.6.10.	Efficiency requirement

The efficiency of the converter ENOx shall not be less than 95 per cent.

If, with the analyzer in the most common range, the ozonator cannot give a reduction from 80 per cent to 20 per cent according to paragraph 9.3.6.5., the highest range which will give the reduction shall be used.

9.3.7.	Adjustment of the FID

9.3.7.1.	Optimization of the detector response

The FID shall be adjusted as specified by the instrument manufacturer.  A propane in air span gas shall be used to optimize the response on the most common operating range.

With the fuel and airflow rates set at the manufacturer's recommendations, a 350 ± 75 ppm C span gas shall be introduced to the analyzer.  The response at a given fuel flow shall be determined from the difference between the span gas response and the zero gas response.  The fuel flow shall be incrementally adjusted above and below the manufacturer's specification.  The span and zero response at these fuel flows shall be recorded.  The difference between the span and zero response shall be plotted and the fuel flow adjusted to the rich side of the curve.  This is the initial flow rate setting which may need further optimization depending on the results of the hydrocarbon response factors and the oxygen interference check according to paragraphs 9.3.7.2. and 9.3.7.3.  If the oxygen interference or the hydrocarbon response factors do not meet the following specifications, the airflow shall be incrementally adjusted above and below the manufacturer's specifications, repeating paragraphs 9.3.7.2. and 9.3.7.3. for each flow.

The optimization may optionally be conducted using the procedures outlined in SAE paper No. 770141.

9.3.7.2.	Hydrocarbon response factors

A linearity verification of the analyzer shall be performed using propane in air and purified synthetic air according to paragraph 9.2.1.3.

Response factors shall be determined when introducing an analyzer into service and after major service intervals.  The response factor (rh) for a particular hydrocarbon species is the ratio of the FID C1 reading to the gas concentration in the cylinder expressed by ppm C1.

The concentration of the test gas shall be at a level to give a response of approximately 80 per cent of full scale.  The concentration shall be known to an accuracy of ± 2 per cent in reference to a gravimetric standard expressed in volume.  In addition, the gas cylinder shall be preconditioned for 24 hours at a temperature of 298 K ± 5 K (25 °C ± 5 °C).

The test gases to be used and the relative response factor ranges are as follows:

(a)	Methane and purified synthetic air	1.00  rh  1.15

(b)	Propylene and purified synthetic air	0.90  rh  1.1

(c)	Toluene and purified synthetic air	0.90  rh  1.1

These values are relative to a rh of 1 for propane and purified synthetic air.

9.3.7.3.	Oxygen interference check

For raw exhaust gas analyzers only, the oxygen interference check shall be performed when introducing an analyzer into service and after major service intervals.

A measuring range shall be chosen where the oxygen interference check gases will fall in the upper 50 per cent.  The test shall be conducted with the oven temperature set as required. Oxygen interference check gas specifications are found in paragraph 9.3.3.4.

(a)	The analyzer shall be set at zero.

(b)	The analyzer shall be spanned with the 0 per cent oxygen blend for positive ignition engines.  Compression ignition engine instruments shall be spanned with the 21 per cent oxygen blend.

(c)	The zero response shall be rechecked.  If it has changed by more than 0.5 per cent of full scale, steps (a) and (b) of this paragraph shall be repeated.

(d)	The 5 per cent and 10 per cent oxygen interference check gases shall be introduced.

(e)	The zero response shall be rechecked.  If it has changed by more than  1 per cent of full scale, the test shall be repeated.

(f)	The oxygen interference EO2 shall be calculated for each mixture in step (d) as follows:

EO2	=	(cref,d - c) x 100 / cref,d	(68)

with the analyzer response being

c	=		(69)

where:

cref,b is the reference HC concentration in step (b), ppm C

cref,d is the reference HC concentration in step (d), ppm C

cFS,b is the full scale HC concentration in step (b), ppm C

cFS,d is the full scale HC concentration in step (d), ppm C

cm,b is the measured HC concentration in step (b), ppm C

cm,d is the measured HC concentration in step (d), ppm C

(g)	The oxygen interference EO2 shall be less than  1.5 per cent for all required oxygen interference check gases prior to testing.

(h)	If the oxygen interference EO2 is greater than  1.5 per cent, corrective action may be taken by incrementally adjusting the airflow above and below the manufacturer's specifications, the fuel flow and the sample flow.

(i)	The oxygen interference shall be repeated for each new setting.

9.3.8.	Efficiency of the non-methane cutter (NMC)

The NMC is used for the removal of the non-methane hydrocarbons from the sample gas by oxidizing all hydrocarbons except methane.  Ideally, the conversion for methane is 0 per cent, and for the other hydrocarbons represented by ethane is 100 per cent.  For the accurate measurement of NMHC, the two efficiencies shall be determined and used for the calculation of the NMHC emission mass flow rate (see paragraph 8.5.1.).

9.3.8.1.	Methane Efficiency

Methane calibration gas shall be flown through the FID with and without bypassing the NMC and the two concentrations recorded.  The efficiency shall be determined as follows:

(70)

where:

cHC(w/NMC)	is the HC concentration with CH4 flowing through the NMC, ppm C

cHC(w/o NMC)	is the HC concentration with CH4 bypassing the NMC, ppm C

9.3.8.2.	Ethane Efficiency

Ethane calibration gas shall be flown through the FID with and without bypassing the NMC and the two concentrations recorded.  The efficiency shall be determined as follows:

(71)

where:

cHC(w/NMC)	is the HC concentration with C2H6 flowing through the NMC, ppm C

cHC(w/o NMC)	is the HC concentration with C2H6 bypassing the NMC, ppm C

9.3.9.	Interference effects

Other gases than the one being analyzed can interfere with the reading in several ways. Positive interference occurs in NDIR instruments where the interfering gas gives the same effect as the gas being measured, but to a lesser degree.  Negative interference occurs in NDIR instruments by the interfering gas broadening the absorption band of the measured gas, and in CLD instruments by the interfering gas quenching the reaction. The interference checks in paragraphs 9.3.9.1. and 9.3.9.2. shall be performed prior to an analyzer's initial use and after major service intervals.

9.3.9.1.	CO analyzer interference check

Water and CO2 can interfere with the CO analyzer performance.  Therefore, a CO2 span gas having a concentration of 80 to 100 per cent of full scale of the maximum operating range used during testing shall be bubbled through water at room temperature and the analyzer response recorded.  The analyzer response shall not be more than 1 per cent of full scale for ranges equal to or above 300 ppm or more than 3 ppm for ranges below 300 ppm.

9.3.9.2.	NOx analyzer quench checks

The two gases of concern for CLD (and HCLD) analyzers are CO2 and water vapour. Quench responses to these gases are proportional to their concentrations, and therefore require test techniques to determine the quench at the highest expected concentrations experienced during testing.

9.3.9.2.1.	CO2 quench check

A CO2 span gas having a concentration of 80 to 100 per cent of full scale of the maximum operating range shall be passed through the NDIR analyzer and the CO2 value recorded as A.  It shall then be diluted approximately 50 per cent with NO span gas and passed through the NDIR and CLD, with the CO2 and NO values recorded as B and C, respectively.  The CO2 shall then be shut off and only the NO span gas be passed through the (H)CLD and the NO value recorded as D.

The per cent quench shall be calculated as follows:

(72)

where:

A	is the undiluted CO2 concentration measured with NDIR, per cent

B	is the diluted CO2 concentration measured with NDIR, per cent

C	is the diluted NO concentration measured with (H)CLD, ppm

D	is the undiluted NO concentration measured with (H)CLD, ppm

Alternative methods of diluting and quantifying of CO2 and NO span gas values such as dynamic mixing/blending are permitted with the approval of the type approval or certification authority.

9.3.9.2.2.	Water quench check

This check applies to wet gas concentration measurements only.  Calculation of water quench shall consider dilution of the NO span gas with water vapour and scaling of water vapour concentration of the mixture to that expected during testing.

A NO span gas having a concentration of 80 per cent to 100 per cent of full scale of the normal operating range shall be passed through the (H) CLD and the NO value recorded as D.  The NO span gas shall then be bubbled through water at room temperature and passed through the (H) CLD and the NO value recorded as C.  The water temperature shall be determined and recorded as F.  The mixture's saturation vapour pressure that corresponds to the bubbler water temperature (F) shall be determined and recorded as G.

The water vapour concentration (in per cent) of the mixture shall be calculated as follows:

H = 100 x (G / pb)	(73)

and recorded as H.  The expected diluted NO span gas (in water vapour) concentration shall be calculated as follows:

De = D x ( 1- H / 100 )	(74)

and recorded as De.  For diesel exhaust, the maximum exhaust water vapour concentration (in per cent) expected during testing shall be estimated, under the assumption of a fuel H/C ratio of 1.8/1, from the maximum CO2 concentration in the exhaust gas A as follows:

Hm = 0.9 x A	(75)

and recorded as Hm

The per cent water quench shall be calculated as follows:

EH2O = 100 x ( ( De - C ) / De) x (Hm / H)	(76)

where:

De	is the expected diluted NO concentration, ppm

C	is the measured diluted NO concentration, ppm

Hm	is the maximum water vapour concentration, per cent

H	is the actual water vapour concentration, per cent

9.3.9.2.3.	Maximum allowable quench

The combined CO2 and water quench shall not exceed 2 per cent of full scale.

9.3.9.2.4.	Efficiency of the cooling bath

For dry CLD analyzers, it shall be demonstrated that for the highest expected water vapour concentration Hm (see paragraph 9.3.9.2.2.), the water removal technique maintains CLD humidity at ≤ 5 g water/kg dry air (or about 0.008 per cent H2O), which is 100 per cent relative humidity at 3.9 °C and 101.3 kPa.  This humidity specification is also equivalent to about 25 per cent relative humidity at 25 °C and 101.3 kPa.  This may be demonstrated by measuring the temperature at the outlet of a thermal dehumidifier, or by measuring humidity at a point just upstream of the CLD.  Humidity of the CLD exhaust might also be measured as long as the only flow into the CLD is the flow from the dehumidifier.

9.3.10.	Sampling for raw gaseous emissions, if applicable (old para 8.3.2.2.)

The gaseous emissions sampling probes shall be fitted at least 0.5 m or 3 times the diameter of the exhaust pipe - whichever is the larger - upstream of the exit of the exhaust gas system but sufficiently close to the engine as to ensure an exhaust gas temperature of at least 343 K (70 °C) at the probe.

In the case of a multi-cylinder engine with a branched exhaust manifold, the inlet of the probe shall be located sufficiently far downstream so as to ensure that the sample is representative of the average exhaust emissions from all cylinders.  In multi-cylinder engines having distinct groups of manifolds, such as in a "Vee" engine configuration, it is recommended to combine the manifolds upstream of the sampling probe.  If this is not practical, it is permissible to acquire a sample from the group with the highest CO2 emission.  For exhaust emission calculation the total exhaust mass flow shall be used.

If the engine is equipped with an exhaust after-treatment system, the exhaust sample shall be taken downstream of the exhaust after-treatment system.

9.3.11.	Sampling for dilute gaseous emissions, if applicable (old para 8.4.2.2.)

The exhaust pipe between the engine and the full flow dilution system shall conform to the requirements laid down in Annex 3.  The gaseous emissions sample probe(s) shall be installed in the dilution tunnel at a point where the dilution air and exhaust gas are well mixed, and in close proximity to the particulates sampling probe.

Sampling can generally be done in two ways:

(a)	the emissions are sampled into a sampling bag over the cycle and measured after completion of the test; for HC, the sample bag shall be heated to 464  11 K (191  11°C), for NOx, the sample bag temperature shall be above the dew point temperature;

(b)	the emissions are sampled continuously and integrated over the cycle.

The background concentrations shall be sampled upstream of the dilution tunnel into a sampling bag, and shall be subtracted from the emissions concentration according to paragraph 8.4.2.4.2.

9.4.	Particulate measurement and sampling system

9.4.1.	General specifications

To determine the mass of the particulates, a particulate dilution and sampling system, a particulate sampling filter, a microgram balance, and a temperature and humidity controlled weighing chamber, are required.  The particulate sampling system shall be designed to ensure a representative sample of the particulates proportional to the exhaust flow.

9.4.2.	General requirements of the dilution system (old paras 8.3.3.1. and 8.4.3.1.)

The determination of the particulates requires dilution of the sample with filtered ambient air, synthetic air or nitrogen.  (the diluent).  The dilution system shall be set as follows:

(a) 	completely eliminate water condensation in the dilution and sampling systems,

(b) 	maintain the temperature of the diluted exhaust gas between 315 K (42 °C) and 325 K (52 °C) within 20 cm upstream or downstream of the filter holder(s),

(c) 	the diluent temperature shall be between 293 K and 325 K (20 °C to 52 °C) in close proximity to the entrance into the dilution tunnel

(d) 	the minimum dilution ratio shall be within the range of 5:1 to 7:1 and at least 2:1 for the primary dilution stage based on the maximum engine exhaust flow rate,

(e) 	for a partial flow dilution system, the residence time in the system from the point of diluent introduction to the filter holder(s) shall be between 0.5 and 5 seconds,

(f)	for a full flow dilution system, 	the overall residence time in the system from the point of diluent introduction to the filter holder(s) shall be between 1 and 5 seconds, and 	the residence time in the secondary dilution system, if used, from the point of secondary diluent introduction to the filter holder(s) shall be at least 0.5 seconds.

Dehumidifying the diluent before entering the dilution system is permitted, and especially useful if diluent humidity is high.

9.4.3.	Particulate sampling (new heading)

9.4.3.1.	Partial flow dilution system (old para 8.3.3.2.)

The particulate sampling probe shall be installed in close proximity to the gaseous emissions sampling probe, but sufficiently distant as to not cause interference. Therefore, the installation provisions of paragraph 8.3.2.2. also apply to particulate sampling.  The sampling line shall conform to the requirements laid down in Annex 3.

In the case of a multi-cylinder engine with a branched exhaust manifold, the inlet of the probe shall be located sufficiently far downstream so as to ensure that the sample is representative of the average exhaust emissions from all cylinders.  In multi-cylinder engines having distinct groups of manifolds, such as in a "Vee" engine configuration, it is recommended to combine the manifolds upstream of the sampling probe.  If this is not practical, it is permissible to acquire a sample from the group with the highest particulate emission.  For exhaust emission calculation the total exhaust mass flow of the manifold shall be used.

9.4.3.2.	Full flow dilution system (old para 8.4.3.2.)

The particulate sampling probe shall be installed in close proximity to the gaseous emissions sampling probe, but sufficiently distant as to not cause interference, in the dilution tunnel.  Therefore, the installation provisions of paragraph 8.3.2.2. also apply to particulate sampling.  The sampling line shall conform to the requirements laid down in Annex 3.

9.4.4.	Particulate sampling filters

The diluted exhaust shall be sampled by a filter that meets the requirements of paragraphs 9.4.4.1. to 9.4.4.3. during the test sequence.

9.4.4.1.	Filter specification

All filter types shall have a 0.3 µm DOP (di-octylphthalate) collection efficiency of at least 99 per cent.  The filter material shall be either:

(a)	fluorocarbon (PTFE) coated glass fiber, or

(b)	fluorocarbon (PTFE) membrane.

9.4.4.2.	Filter size

The filter shall be circular with a nominal diameter of 47 mm (tolerance of 46.50  0.6 mm) and an exposed diameter (filter stain diameter) of at least 38 mm.

9.4.4.3.	Filter face velocity

The face velocity through the filter shall be between 0.80 and 1.00 m/s with less than 5 per cent of the recorded flow values exceeding this range.  If the total PM mass on the filter exceeds 400 µg, the filter face velocity may be reduced to 0.50 m/s. The face velocity shall be calculated as the volumetric flow rate of the sample at the pressure upstream of the filter and temperature of the filter face, divided by the filter's exposed area.

9.4.5.	Weighing chamber and analytical balance specifications

The chamber (or room) environment shall be free of any ambient contaminants (such as dust, aerosol, or semi-volatile material) that could contaminate the particulate filters.  The weighing room shall meet the required specifications for at least 60 min before weighing filters.

9.4.5.1.	Weighing chamber conditions

The temperature of the chamber (or room) in which the particulate filters are conditioned and weighed shall be maintained to within 295 K ± 1 K (22 °C ± 1 °C) during all filter conditioning and weighing.  The humidity shall be maintained to a dew point of 282.5 K ± 1 K  (9.5 °C ± 1 °C).

If the stabilization and weighing environments are separate, the temperature of the stabilization environment shall be maintained at a tolerance of 295 K ± 3 K (22 °C ± 3 °C), but the dew point requirement remains at 282.5 K ± 1 K  (9.5 °C ± 1 °C).

Humidity and ambient temperature shall be recorded.

9.4.5.2.	Reference filter weighing

At least two unused reference filters shall be weighed within 12 hours of, but preferably at the same time as the sample filter weighing.  They shall be the same material as the sample filters.

If the average weight of the reference filters changes between sample filter weighings by more than 10 µg, all sample filters shall be discarded and the emissions test repeated.

The reference filters shall be replaced at least once per year, or if the filter mass changes by more than 10 per cent.

9.4.5.3.	Analytical balance

The analytical balance used to determine the filter weight shall meet the linearity verification criterion of paragraph 9.2., table 6.  This implies a precision (standard deviation) of at least 2 µg and a resolution of at least 1 µg (1 digit = 1 µg).

The balance shall be installed as follows:

(a) 	installed on a vibration-isolation platform to isolate it from external noise and vibration.

(b) 	shielded from convective airflow with a static-dissipating draft shield that is electrically grounded.

9.4.5.4.	Elimination of static electricity effects

The filter shall be neutralized prior to weighing, e.g. by a Polonium neutralizer or a device of similar effect.  If a PTFE membrane filter is used, the static electricity shall be measured and is recommended to be within  2.0 V of neutral.

In addition, static electric charge shall be minimized in the balance environment, as follows:

(a) 	The balance shall be electrically grounded.

(b) 	Stainless steel tweezers shall be used if PM samples are handled manually.

(c) 	Tweezers shall be grounded with a grounding strap, or a grounding strap shall be provided for the operator such that the grounding strap shares a common ground with the balance. Grounding straps shall have an appropriate resistor to protect operators from accidental shock.

9.4.5.5.	Additional specifications (old para 9.4.5.)

All parts of the dilution system and the sampling system from the exhaust pipe up to the filter holder, which are in contact with raw and diluted exhaust gas, shall be designed to minimize deposition or alteration of the particulates.  All parts shall be made of electrically conductive materials that do not react with exhaust gas components, and shall be electrically grounded to prevent electrostatic effects.

9.4.5.6.	Calibration of the flow measurement instrumentation (old para 9.4.6.)

Each flowmeter used in a  particulate sampling and partial flow dilution system shall be subjected to the linearity verification, as  described in paragraph 9.2.1., as often as necessary to fulfil the accuracy requirements of this gtr.  For the flow reference values, an accurate flowmeter traceable to international and/or national standards shall be used. For differential flow measurement calibration see paragraph 9.4.6.2.

9.4.6.	Special requirements for the partial flow dilution system (old para 9.4.7.)

The partial flow dilution system has to be designed to extract a proportional raw exhaust sample from the engine exhaust stream, thus responding to excursions in the exhaust stream flow rate.  For this it is essential that the dilution ratio or the sampling ratio rd or rs be determined such that the accuracy requirements of paragraph 9.4.6.2. are fulfilled.

9.4.6.1.	System response time (old paragraph 8.3.3.3.)

For the control of a partial flow dilution system, a fast system response is required.  The transformation time for the system shall be determined by the procedure in paragraph 9.4.7.3.  If the combined transformation time of the exhaust flow measurement (see paragraph 8.3.1.2.) and the partial flow system is  ≤  0.3 s, online control shall be used.  If the transformation time exceeds 0.3 s, look ahead control based on a pre-recorded test run shall be used.  In this case, the combined rise time shall be    1 s and the combined delay time    10 s.

The total system response shall be designed as to ensure a representative sample of the particulates, qmp,i, proportional to the exhaust mass flow.  To determine the proportionality, a regression analysis of qmp,i versus qmew,i shall be conducted on a minimum 5 Hz data acquisition rate, and the following criteria shall be met:

(a)	The coefficient of determination r2 of the linear regression between qmp,i and qmew,i shall not be less than 0.95.

(b)	The standard error of estimate of qmp,i on qmew,i shall not exceed 5 per cent of qmp maximum.

(c)	qmp intercept of the regression line shall not exceed  2 per cent of qmp maximum.

Look-ahead control is required if the combined transformation times of the particulate system, t50,P and of the exhaust mass flow signal, t50,F are  >  0.3 s.  In this case, a pre-test shall be run, and the exhaust mass flow signal of the pre-test be used for controlling the sample flow into the particulate system.  A correct control of the partial dilution system is obtained, if the time trace of qmew,pre of the pre-test, which controls qmp, is shifted by a "look-ahead" time of t50,P + t50,F.

For establishing the correlation between qmp,i and qmew,i the data taken during the actual test shall be used, with qmew,i time aligned by t50,F relative to qmp,i (no contribution from t50,P to the time alignment).  That is, the time shift between qmew and qmp is the difference in their transformation times that were determined in paragraph 9.4.7.3.

9.4.6.2.	Specifications for differential flow measurement (old para 9.4.4.)

For partial flow dilution systems, the accuracy of the sample flow qmp is of special concern, if not measured directly, but determined by differential flow measurement:

qmp = qmdew – qmdw	(77)

In this case, the maximum error of the difference shall be such that the accuracy of qmp is within 5 per cent when the dilution ratio is less than 15.  It can be calculated by taking root-mean-square of the errors of each instrument.

Acceptable accuracies of qmp can be obtained by either of the following methods:

(a)	The absolute accuracies of qmdew and qmdw are  0.2 per cent which guarantees an accuracy of qmp of  5 per cent at a dilution ratio of 15.  However, greater errors will occur at higher dilution ratios.

(b)	Calibration of qmdw relative to qmdew is carried out such that the same accuracies for qmp as in (a) are obtained.  For details see paragraph 9.4.6.2.

(c)	The accuracy of qmp is determined indirectly from the accuracy of the dilution ratio as determined by a tracer gas, e.g. CO2.  Accuracies equivalent to method (a) for qmp are required.

(d)	The absolute accuracy of qmdew and qmdw is within  2 per cent of full scale, the maximum error of the difference between qmdew and qmdw is within 0.2 per cent, and the linearity error is within  0.2 per cent of the highest qmdew observed during the test.

9.4.6.3.	Calibration of differential flow measurement

The flowmeter or the flow measurement instrumentation shall be calibrated in one of the following procedures, such that the probe flow qmp into the tunnel shall fulfil the accuracy requirements of paragraph 9.4.4.:

(a)	The flowmeter for qmdw shall be connected in series to the flowmeter for qmdew, the difference between the two flowmeters shall be calibrated for at least 5 set points with flow values equally spaced between the lowest qmdw value used during the test and the value of qmdew used during the test.  The dilution tunnel may be bypassed.

(b)	A calibrated flow device shall be connected in series to the flowmeter for qmdew and the accuracy shall be checked for the value used for the test.  The calibrated flow device shall be connected in series to the flowmeter for qmdw, and the accuracy shall be checked for at least 5 settings corresponding to dilution ratio between 3 and 50, relative to qmdew used during the test.

(c)	The transfer tube (TT) shall be disconnected from the exhaust, and a calibrated flow-measuring device with a suitable range to measure qmp shall be connected to the transfer tube.  qmdew shall be set to the value used during the test, and qmdw shall be sequentially set to at least 5 values corresponding to dilution ratios between 3 and 50.  Alternatively, a special calibration flow path may be provided, in which the tunnel is bypassed, but the total and dilution airflow through the corresponding meters as in the actual test.

(d)	A tracer gas shall be fed into the exhaust transfer tube TT.  This tracer gas may be a component of the exhaust gas, like CO2 or NOx.  After dilution in the tunnel the tracer gas component shall be measured.  This shall be carried out for 5 dilution ratios between 3 and 50.  The accuracy of the sample flow shall be determined from the dilution ratio rd:

qmp = qmdew /rd	(78)

The accuracies of the gas analyzers shall be taken into account to guarantee the accuracy of qmp.

9.4.6.4.	Carbon flow check

A carbon flow check using actual exhaust is strongly recommended for detecting measurement and control problems and verifying the proper operation of the partial flow system.  The carbon flow check should be run at least each time a new engine is installed, or something significant is changed in the test cell configuration.

The engine shall be operated at peak torque load and speed or any other steady state mode that produces 5 per cent or more of CO2.  The partial flow sampling system shall be operated with a dilution factor of about 15 to 1.

If a carbon flow check is conducted, the procedure given in Annex 5 shall be applied. The carbon flow rates shall be calculated according to equations 80 to 82 in Annex 5. All carbon flow rates should agree to within 3 per cent.

9.4.6.5.	Pre-test check

A pre-test check shall be performed within 2 hours before the test run in the following way.

The accuracy of the flowmeters shall be checked by the same method as used for calibration (see paragraph 9.4.6.2.) for at least two points, including flow values of qmdw that correspond to dilution ratios between 5 and 15 for the qmdew value used during the test.

If it can be demonstrated by records of the calibration procedure under paragraph 9.4.6.2. that the flowmeter calibration is stable over a longer period of time, the pre-test check may be omitted.

9.4.6.6.	Determination of the transformation time

The system settings for the transformation time evaluation shall be exactly the same as during measurement of the test run.  The transformation time shall be determined by the following method.

An independent reference flowmeter with a measurement range appropriate for the probe flow shall be put in series with and closely coupled to the probe.  This flowmeter shall have a transformation time of less than 100 ms for the flow step size used in the response time measurement, with flow restriction sufficiently low as to not affect the dynamic performance of the partial flow dilution system, and consistent with good engineering practice.

A step change shall be introduced to the exhaust flow (or airflow if exhaust flow is calculated) input of the partial flow dilution system, from a low flow to at least 90 per cent of full scale.  The trigger for the step change shall be the same one used to start the look-ahead control in actual testing.  The exhaust flow step stimulus and the flowmeter response shall be recorded at a sample rate of at least 10 Hz.

From this data, the transformation time shall be determined for the partial flow dilution system, which is the time from the initiation of the step stimulus to the 50 per cent point of the flowmeter response.  In a similar manner, the transformation times of the qmp signal of the partial flow dilution system and of the qmew,i signal of the exhaust flowmeter shall be determined.  These signals are used in the regression checks performed after each test (see paragraph 9.4.6.1.)

The calculation shall be repeated for at least 5 rise and fall stimuli, and the results shall be averaged.  The internal transformation time (< 100 ms) of the reference flowmeter shall be subtracted from this value.  This is the "look-ahead" value of the partial flow dilution system, which shall be applied in accordance with paragraph 9.4.6.1.

9.5.	Calibration of the CVS system

9.5.1.	General

The CVS system shall be calibrated by using an accurate flowmeter and a restricting device.  The flow through the system shall be measured at different restriction settings, and the control parameters of the system shall be measured and related to the flow.

Various types of flowmeters may be used, e.g. calibrated venturi, calibrated laminar flowmeter, calibrated turbine meter.

9.5.2.	Calibration of the positive displacement pump (PDP)

All the parameters related to the pump shall be simultaneously measured along with the parameters related to a calibration venturi which is connected in series with the pump. The calculated flow rate (in m3/s at pump inlet, absolute pressure and temperature) shall be plotted versus a correlation function which is the value of a specific combination of pump parameters.  The linear equation which relates the pump flow and the correlation function shall be determined.  If a CVS has a multiple speed drive, the calibration shall be performed for each range used.

Temperature stability shall be maintained during calibration.

Leaks in all the connections and ducting between the calibration venturi and the CVS pump shall be maintained lower than 0.3 per cent of the lowest flow point (highest restriction and lowest PDP speed point).

9.5.2.1.	Data analysis

The airflow rate (qvCVS) at each restriction setting (minimum 6 settings) shall be calculated in standard m3/s from the flowmeter data using the manufacturer's prescribed method.  The airflow rate shall then be converted to pump flow (V0) in m3/rev at absolute pump inlet temperature and pressure as follows:

(79)

where:

qvCVS is the airflow rate at standard conditions (101.3 kPa, 273 K), m3/s

T	is the temperature at pump inlet, K

pp	is the absolute pressure at pump inlet, kPa

n	is the pump speed, rev/s

To account for the interaction of pressure variations at the pump and the pump slip rate, the correlation function (X0) between pump speed, pressure differential from pump inlet to pump outlet and absolute pump outlet pressure shall be calculated as follows:

(80)

where:

pp	is the pressure differential from pump inlet to pump outlet, kPa

pp	is the absolute outlet pressure at pump outlet, kPa

A linear least-square fit shall be performed to generate the calibration equation as follows:

(81)

D0 and m are the intercept and slope, respectively, describing the regression lines.

For a CVS system with multiple speeds, the calibration curves generated for the different pump flow ranges shall be approximately parallel, and the intercept values (D0) shall increase as the pump flow range decreases.

The calculated values from the equation shall be within ± 0.5 per cent of the measured value of V0. Values of m will vary from one pump to another.  Particulate influx over time will cause the pump slip to decrease, as reflected by lower values for m.  Therefore, calibration shall be performed at pump start-up, after major maintenance, and if the total system verification indicates a change of the slip rate.

9.5.3.	Calibration of the critical flow venturi (CFV)

Calibration of the CFV is based upon the flow equation for a critical venturi.  Gas flow is a function of venturi inlet pressure and temperature.

To determine the range of critical flow, Kv  shall be plotted as a function of venturi inlet pressure.  For critical (choked) flow, Kv will have a relatively constant value.  As pressure decreases (vacuum increases), the venturi becomes unchoked and Kv decreases, which indicates that the CFV is operated outside the permissible range.

9.5.3.1.	Data analysis

The airflow rate (qvCVS) at each restriction setting (minimum 8 settings) shall be calculated in standard m3/s from the flowmeter data using the manufacturer's prescribed method.  The calibration coefficient shall be calculated from the calibration data for each setting as follows:

(82)

where:

qvCVS	is the airflow rate at standard conditions (101.3 kPa, 273 K), m3/s

T	is the temperature at the venturi inlet, K

pp	is the absolute pressure at venturi inlet, kPa

The average KV and the standard deviation shall be calculated.  The standard deviation shall not exceed ± 0.3 per cent of the average KV.

9.5.4.	Calibration of the subsonic venturi (SSV)

Calibration of the SSV is based upon the flow equation for a subsonic venturi.  Gas flow is a function of inlet pressure and temperature, pressure drop between the SSV inlet and throat, as shown in equation 43 (see paragraph 8.4.1.4.).

9.5.4.1.	Data analysis

The airflow rate (QSSV) at each restriction setting (minimum 16 settings) shall be calculated in standard m3/s from the flowmeter data using the manufacturer's prescribed method.  The discharge coefficient shall be calculated from the calibration data for each setting as follows:

(83)

where:

QSSV	is the airflow rate at standard conditions (101.3 kPa, 273 K), m3/s

T	is the temperature at the venturi inlet, K

dV	is the diameter of the SSV throat, m

rp	is the ratio of the SSV throat to inlet absolute static pressure =

rD	is the ratio of the SSV throat diameter, dV, to the inlet pipe inner diameter D

To determine the range of subsonic flow, Cd shall be plotted as a function of Reynolds number Re, at the SSV throat.  The Re at the SSV throat shall be calculated with the following equation:

(84)

with

(85)

where:

A1	is 25.55152 in SI units of

QSSV	is the airflow rate at standard conditions (101.3 kPa, 273 K), m3/s

dV	is the diameter of the SSV throat, m

μ	is the absolute or dynamic viscosity of the gas, kg/ms

b	is 1.458 x 106 (empirical constant), kg/ms K0.5

S	is 110.4 (empirical constant), K

Because QSSV is an input to the Re equation, the calculations must be started with an initial guess for QSSV or Cd of the calibration venturi, and repeated until QSSV converges. The convergence method shall be accurate to 0.1 per cent of point or better.

For a minimum of sixteen points in the region of subsonic flow, the calculated values of Cd from the resulting calibration curve fit equation must be within ± 0.5 per cent of the measured Cd for each calibration point.

9.5.5.	Total system verification

The total accuracy of the CVS sampling system and analytical system shall be determined by introducing a known mass of a pollutant gas into the system while it is being operated in the normal manner.  The pollutant is analyzed, and the mass calculated according to paragraph 8.4.2.4. except in the case of propane where a u factor of 0.000472 is used in place of 0.000480 for HC.  Either of the following two techniques shall be used.

9.5.5.1.	Metering with a critical flow orifice

A known quantity of pure gas (carbon monoxide or propane) shall be fed into the CVS system through a calibrated critical orifice.  If the inlet pressure is high enough, the flow rate, which is adjusted by means of the critical flow orifice, is independent of the orifice outlet pressure (critical flow).  The CVS system shall be operated as in a normal exhaust emission test for about 5 to 10 minutes.  A gas sample shall be analyzed with the usual equipment (sampling bag or integrating method), and the mass of the gas calculated. The mass so determined shall be within ± 3 per cent of the known mass of the gas injected.

9.5.5.2.	Metering by means of a gravimetric technique

The mass of a small cylinder filled with carbon monoxide or propane shall be determined with a precision of ± 0.01 g.  For about 5 to 10 minutes, the CVS system shall be operated as in a normal exhaust emission test, while carbon monoxide or propane is injected into the system.  The quantity of pure gas discharged shall be determined by means of differential weighing.  A gas sample shall be analyzed with the usual equipment (sampling bag or integrating method), and the mass of the gas calculated.  The mass so determined shall be within ± 3 per cent of the known mass of the gas injected.

Annex 1

WHTC ENGINE DYNAMOMETER SCHEDULE

| Time | Norm. | Norm. |  | Time | Norm. | Norm. |  | Time | Norm. | Norm. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Speed | Torque |  |  | Speed | Torque |  |  | Speed | Torque |
| s | per cent | per cent |  | s | per cent | per cent |  | s | per cent | per cent |
| 1 | 0.0 | 0.0 |  | 47 | 0.0 | 0.0 |  | 93 | 32.8 | 32.7 |
| 2 | 0.0 | 0.0 |  | 48 | 0.0 | 0.0 |  | 94 | 33.7 | 32.5 |
| 3 | 0.0 | 0.0 |  | 49 | 0.0 | 0.0 |  | 95 | 34.4 | 29.5 |
| 4 | 0.0 | 0.0 |  | 50 | 0.0 | 13.1 |  | 96 | 34.3 | 26.5 |
| 5 | 0.0 | 0.0 |  | 51 | 13.1 | 30.1 |  | 97 | 34.4 | 24.7 |
| 6 | 0.0 | 0.0 |  | 52 | 26.3 | 25.5 |  | 98 | 35.0 | 24.9 |
| 7 | 1.5 | 8.9 |  | 53 | 35.0 | 32.2 |  | 99 | 35.6 | 25.2 |
| 8 | 15.8 | 30.9 |  | 54 | 41.7 | 14.3 |  | 100 | 36.1 | 24.8 |
| 9 | 27.4 | 1.3 |  | 55 | 42.2 | 0.0 |  | 101 | 36.3 | 24.0 |
| 10 | 32.6 | 0.7 |  | 56 | 42.8 | 11.6 |  | 102 | 36.2 | 23.6 |
| 11 | 34.8 | 1.2 |  | 57 | 51.0 | 20.9 |  | 103 | 36.2 | 23.5 |
| 12 | 36.2 | 7.4 |  | 58 | 60.0 | 9.6 |  | 104 | 36.8 | 22.7 |
| 13 | 37.1 | 6.2 |  | 59 | 49.4 | 0.0 |  | 105 | 37.2 | 20.9 |
| 14 | 37.9 | 10.2 |  | 60 | 38.9 | 16.6 |  | 106 | 37.0 | 19.2 |
| 15 | 39.6 | 12.3 |  | 61 | 43.4 | 30.8 |  | 107 | 36.3 | 18.4 |
| 16 | 42.3 | 12.5 |  | 62 | 49.4 | 14.2 |  | 108 | 35.4 | 17.6 |
| 17 | 45.3 | 12.6 |  | 63 | 40.5 | 0.0 |  | 109 | 35.2 | 14.9 |
| 18 | 48.6 | 6.0 |  | 64 | 31.5 | 43.5 |  | 110 | 35.4 | 9.9 |
| 19 | 40.8 | 0.0 |  | 65 | 36.6 | 78.2 |  | 111 | 35.5 | 4.3 |
| 20 | 33.0 | 16.3 |  | 66 | 40.8 | 67.6 |  | 112 | 35.2 | 6.6 |
| 21 | 42.5 | 27.4 |  | 67 | 44.7 | 59.1 |  | 113 | 34.9 | 10.0 |
| 22 | 49.3 | 26.7 |  | 68 | 48.3 | 52.0 |  | 114 | 34.7 | 25.1 |
| 23 | 54.0 | 18.0 |  | 69 | 51.9 | 63.8 |  | 115 | 34.4 | 29.3 |
| 24 | 57.1 | 12.9 |  | 70 | 54.7 | 27.9 |  | 116 | 34.5 | 20.7 |
| 25 | 58.9 | 8.6 |  | 71 | 55.3 | 18.3 |  | 117 | 35.2 | 16.6 |
| 26 | 59.3 | 6.0 |  | 72 | 55.1 | 16.3 |  | 118 | 35.8 | 16.2 |
| 27 | 59.0 | 4.9 |  | 73 | 54.8 | 11.1 |  | 119 | 35.6 | 20.3 |
| 28 | 57.9 | m |  | 74 | 54.7 | 11.5 |  | 120 | 35.3 | 22.5 |
| 29 | 55.7 | m |  | 75 | 54.8 | 17.5 |  | 121 | 35.3 | 23.4 |
| 30 | 52.1 | m |  | 76 | 55.6 | 18.0 |  | 122 | 34.7 | 11.9 |
| 31 | 46.4 | m |  | 77 | 57.0 | 14.1 |  | 123 | 45.5 | 0.0 |
| 32 | 38.6 | m |  | 78 | 58.1 | 7.0 |  | 124 | 56.3 | m |
| 33 | 29.0 | m |  | 79 | 43.3 | 0.0 |  | 125 | 46.2 | m |
| 34 | 20.8 | m |  | 80 | 28.5 | 25.0 |  | 126 | 50.1 | 0.0 |
| 35 | 16.9 | m |  | 81 | 30.4 | 47.8 |  | 127 | 54.0 | m |
| 36 | 16.9 | 42.5 |  | 82 | 32.1 | 39.2 |  | 128 | 40.5 | m |
| 37 | 18.8 | 38.4 |  | 83 | 32.7 | 39.3 |  | 129 | 27.0 | m |
| 38 | 20.7 | 32.9 |  | 84 | 32.4 | 17.3 |  | 130 | 13.5 | m |
| 39 | 21.0 | 0.0 |  | 85 | 31.6 | 11.4 |  | 131 | 0.0 | 0.0 |
| 40 | 19.1 | 0.0 |  | 86 | 31.1 | 10.2 |  | 132 | 0.0 | 0.0 |
| 41 | 13.7 | 0.0 |  | 87 | 31.1 | 19.5 |  | 133 | 0.0 | 0.0 |
| 42 | 2.2 | 0.0 |  | 88 | 31.4 | 22.5 |  | 134 | 0.0 | 0.0 |
| 43 | 0.0 | 0.0 |  | 89 | 31.6 | 22.9 |  | 135 | 0.0 | 0.0 |
| 44 | 0.0 | 0.0 |  | 90 | 31.6 | 24.3 |  | 136 | 0.0 | 0.0 |
| 45 | 0.0 | 0.0 |  | 91 | 31.9 | 26.9 |  | 137 | 0.0 | 0.0 |
| 46 | 0.0 | 0.0 |  | 92 | 32.4 | 30.6 |  | 138 | 0.0 | 0.0 |
| 139 | 0.0 | 0.0 |  | 189 | 0.0 | 5.9 |  | 239 | 0.0 | 0.0 |
| 140 | 0.0 | 0.0 |  | 190 | 0.0 | 0.0 |  | 240 | 0.0 | 0.0 |
| 141 | 0.0 | 0.0 |  | 191 | 0.0 | 0.0 |  | 241 | 0.0 | 0.0 |
| 142 | 0.0 | 4.9 |  | 192 | 0.0 | 0.0 |  | 242 | 0.0 | 0.0 |
| 143 | 0.0 | 7.3 |  | 193 | 0.0 | 0.0 |  | 243 | 0.0 | 0.0 |
| 144 | 4.4 | 28.7 |  | 194 | 0.0 | 0.0 |  | 244 | 0.0 | 0.0 |
| 145 | 11.1 | 26.4 |  | 195 | 0.0 | 0.0 |  | 245 | 0.0 | 0.0 |
| 146 | 15.0 | 9.4 |  | 196 | 0.0 | 0.0 |  | 246 | 0.0 | 0.0 |
| 147 | 15.9 | 0.0 |  | 197 | 0.0 | 0.0 |  | 247 | 0.0 | 0.0 |
| 148 | 15.3 | 0.0 |  | 198 | 0.0 | 0.0 |  | 248 | 0.0 | 0.0 |
| 149 | 14.2 | 0.0 |  | 199 | 0.0 | 0.0 |  | 249 | 0.0 | 0.0 |
| 150 | 13.2 | 0.0 |  | 200 | 0.0 | 0.0 |  | 250 | 0.0 | 0.0 |
| 151 | 11.6 | 0.0 |  | 201 | 0.0 | 0.0 |  | 251 | 0.0 | 0.0 |
| 152 | 8.4 | 0.0 |  | 202 | 0.0 | 0.0 |  | 252 | 0.0 | 0.0 |
| 153 | 5.4 | 0.0 |  | 203 | 0.0 | 0.0 |  | 253 | 0.0 | 31.6 |
| 154 | 4.3 | 5.6 |  | 204 | 0.0 | 0.0 |  | 254 | 9.4 | 13.6 |
| 155 | 5.8 | 24.4 |  | 205 | 0.0 | 0.0 |  | 255 | 22.2 | 16.9 |
| 156 | 9.7 | 20.7 |  | 206 | 0.0 | 0.0 |  | 256 | 33.0 | 53.5 |
| 157 | 13.6 | 21.1 |  | 207 | 0.0 | 0.0 |  | 257 | 43.7 | 22.1 |
| 158 | 15.6 | 21.5 |  | 208 | 0.0 | 0.0 |  | 258 | 39.8 | 0.0 |
| 159 | 16.5 | 21.9 |  | 209 | 0.0 | 0.0 |  | 259 | 36.0 | 45.7 |
| 160 | 18.0 | 22.3 |  | 210 | 0.0 | 0.0 |  | 260 | 47.6 | 75.9 |
| 161 | 21.1 | 46.9 |  | 211 | 0.0 | 0.0 |  | 261 | 61.2 | 70.4 |
| 162 | 25.2 | 33.6 |  | 212 | 0.0 | 0.0 |  | 262 | 72.3 | 70.4 |
| 163 | 28.1 | 16.6 |  | 213 | 0.0 | 0.0 |  | 263 | 76.0 | m |
| 164 | 28.8 | 7.0 |  | 214 | 0.0 | 0.0 |  | 264 | 74.3 | m |
| 165 | 27.5 | 5.0 |  | 215 | 0.0 | 0.0 |  | 265 | 68.5 | m |
| 166 | 23.1 | 3.0 |  | 216 | 0.0 | 0.0 |  | 266 | 61.0 | m |
| 167 | 16.9 | 1.9 |  | 217 | 0.0 | 0.0 |  | 267 | 56.0 | m |
| 168 | 12.2 | 2.6 |  | 218 | 0.0 | 0.0 |  | 268 | 54.0 | m |
| 169 | 9.9 | 3.2 |  | 219 | 0.0 | 0.0 |  | 269 | 53.0 | m |
| 170 | 9.1 | 4.0 |  | 220 | 0.0 | 0.0 |  | 270 | 50.8 | m |
| 171 | 8.8 | 3.8 |  | 221 | 0.0 | 0.0 |  | 271 | 46.8 | m |
| 172 | 8.5 | 12.2 |  | 222 | 0.0 | 0.0 |  | 272 | 41.7 | m |
| 173 | 8.2 | 29.4 |  | 223 | 0.0 | 0.0 |  | 273 | 35.9 | m |
| 174 | 9.6 | 20.1 |  | 224 | 0.0 | 0.0 |  | 274 | 29.2 | m |
| 175 | 14.7 | 16.3 |  | 225 | 0.0 | 0.0 |  | 275 | 20.7 | m |
| 176 | 24.5 | 8.7 |  | 226 | 0.0 | 0.0 |  | 276 | 10.1 | m |
| 177 | 39.4 | 3.3 |  | 227 | 0.0 | 0.0 |  | 277 | 0.0 | m |
| 178 | 39.0 | 2.9 |  | 228 | 0.0 | 0.0 |  | 278 | 0.0 | 0.0 |
| 179 | 38.5 | 5.9 |  | 229 | 0.0 | 0.0 |  | 279 | 0.0 | 0.0 |
| 180 | 42.4 | 8.0 |  | 230 | 0.0 | 0.0 |  | 280 | 0.0 | 0.0 |
| 181 | 38.2 | 6.0 |  | 231 | 0.0 | 0.0 |  | 281 | 0.0 | 0.0 |
| 182 | 41.4 | 3.8 |  | 232 | 0.0 | 0.0 |  | 282 | 0.0 | 0.0 |
| 183 | 44.6 | 5.4 |  | 233 | 0.0 | 0.0 |  | 283 | 0.0 | 0.0 |
| 184 | 38.8 | 8.2 |  | 234 | 0.0 | 0.0 |  | 284 | 0.0 | 0.0 |
| 185 | 37.5 | 8.9 |  | 235 | 0.0 | 0.0 |  | 285 | 0.0 | 0.0 |
| 186 | 35.4 | 7.3 |  | 236 | 0.0 | 0.0 |  | 286 | 0.0 | 0.0 |
| 187 | 28.4 | 7.0 |  | 237 | 0.0 | 0.0 |  | 287 | 0.0 | 0.0 |
| 188 | 14.8 | 7.0 |  | 238 | 0.0 | 0.0 |  | 288 | 0.0 | 0.0 |
| 289 | 0.0 | 0.0 |  | 339 | 0.0 | 0.0 |  | 389 | 25.2 | 14.7 |
| 290 | 0.0 | 0.0 |  | 340 | 0.0 | 0.0 |  | 390 | 28.6 | 28.4 |
| 291 | 0.0 | 0.0 |  | 341 | 0.0 | 0.0 |  | 391 | 35.5 | 65.0 |
| 292 | 0.0 | 0.0 |  | 342 | 0.0 | 0.0 |  | 392 | 43.8 | 75.3 |
| 293 | 0.0 | 0.0 |  | 343 | 0.0 | 0.0 |  | 393 | 51.2 | 34.2 |
| 294 | 0.0 | 0.0 |  | 344 | 0.0 | 0.0 |  | 394 | 40.7 | 0.0 |
| 295 | 0.0 | 0.0 |  | 345 | 0.0 | 0.0 |  | 395 | 30.3 | 45.4 |
| 296 | 0.0 | 0.0 |  | 346 | 0.0 | 0.0 |  | 396 | 34.2 | 83.1 |
| 297 | 0.0 | 0.0 |  | 347 | 0.0 | 0.0 |  | 397 | 37.6 | 85.3 |
| 298 | 0.0 | 0.0 |  | 348 | 0.0 | 0.0 |  | 398 | 40.8 | 87.5 |
| 299 | 0.0 | 0.0 |  | 349 | 0.0 | 0.0 |  | 399 | 44.8 | 89.7 |
| 300 | 0.0 | 0.0 |  | 350 | 0.0 | 0.0 |  | 400 | 50.6 | 91.9 |
| 301 | 0.0 | 0.0 |  | 351 | 0.0 | 0.0 |  | 401 | 57.6 | 94.1 |
| 302 | 0.0 | 0.0 |  | 352 | 0.0 | 0.0 |  | 402 | 64.6 | 44.6 |
| 303 | 0.0 | 0.0 |  | 353 | 0.0 | 0.0 |  | 403 | 51.6 | 0.0 |
| 304 | 0.0 | 0.0 |  | 354 | 0.0 | 0.5 |  | 404 | 38.7 | 37.4 |
| 305 | 0.0 | 0.0 |  | 355 | 0.0 | 4.9 |  | 405 | 42.4 | 70.3 |
| 306 | 0.0 | 0.0 |  | 356 | 9.2 | 61.3 |  | 406 | 46.5 | 89.1 |
| 307 | 0.0 | 0.0 |  | 357 | 22.4 | 40.4 |  | 407 | 50.6 | 93.9 |
| 308 | 0.0 | 0.0 |  | 358 | 36.5 | 50.1 |  | 408 | 53.8 | 33.0 |
| 309 | 0.0 | 0.0 |  | 359 | 47.7 | 21.0 |  | 409 | 55.5 | 20.3 |
| 310 | 0.0 | 0.0 |  | 360 | 38.8 | 0.0 |  | 410 | 55.8 | 5.2 |
| 311 | 0.0 | 0.0 |  | 361 | 30.0 | 37.0 |  | 411 | 55.4 | m |
| 312 | 0.0 | 0.0 |  | 362 | 37.0 | 63.6 |  | 412 | 54.4 | m |
| 313 | 0.0 | 0.0 |  | 363 | 45.5 | 90.8 |  | 413 | 53.1 | m |
| 314 | 0.0 | 0.0 |  | 364 | 54.5 | 40.9 |  | 414 | 51.8 | m |
| 315 | 0.0 | 0.0 |  | 365 | 45.9 | 0.0 |  | 415 | 50.3 | m |
| 316 | 0.0 | 0.0 |  | 366 | 37.2 | 47.5 |  | 416 | 48.4 | m |
| 317 | 0.0 | 0.0 |  | 367 | 44.5 | 84.4 |  | 417 | 45.9 | m |
| 318 | 0.0 | 0.0 |  | 368 | 51.7 | 32.4 |  | 418 | 43.1 | m |
| 319 | 0.0 | 0.0 |  | 369 | 58.1 | 15.2 |  | 419 | 40.1 | m |
| 320 | 0.0 | 0.0 |  | 370 | 45.9 | 0.0 |  | 420 | 37.4 | m |
| 321 | 0.0 | 0.0 |  | 371 | 33.6 | 35.8 |  | 421 | 35.1 | m |
| 322 | 0.0 | 0.0 |  | 372 | 36.9 | 67.0 |  | 422 | 32.8 | m |
| 323 | 0.0 | 0.0 |  | 373 | 40.2 | 84.7 |  | 423 | 45.3 | 0.0 |
| 324 | 4.5 | 41.0 |  | 374 | 43.4 | 84.3 |  | 424 | 57.8 | m |
| 325 | 17.2 | 38.9 |  | 375 | 45.7 | 84.3 |  | 425 | 50.6 | m |
| 326 | 30.1 | 36.8 |  | 376 | 46.5 | m |  | 426 | 41.6 | m |
| 327 | 41.0 | 34.7 |  | 377 | 46.1 | m |  | 427 | 47.9 | 0.0 |
| 328 | 50.0 | 32.6 |  | 378 | 43.9 | m |  | 428 | 54.2 | m |
| 329 | 51.4 | 0.1 |  | 379 | 39.3 | m |  | 429 | 48.1 | m |
| 330 | 47.8 | m |  | 380 | 47.0 | m |  | 430 | 47.0 | 31.3 |
| 331 | 40.2 | m |  | 381 | 54.6 | m |  | 431 | 49.0 | 38.3 |
| 332 | 32.0 | m |  | 382 | 62.0 | m |  | 432 | 52.0 | 40.1 |
| 333 | 24.4 | m |  | 383 | 52.0 | m |  | 433 | 53.3 | 14.5 |
| 334 | 16.8 | m |  | 384 | 43.0 | m |  | 434 | 52.6 | 0.8 |
| 335 | 8.1 | m |  | 385 | 33.9 | m |  | 435 | 49.8 | m |
| 336 | 0.0 | m |  | 386 | 28.4 | m |  | 436 | 51.0 | 18.6 |
| 337 | 0.0 | 0.0 |  | 387 | 25.5 | m |  | 437 | 56.9 | 38.9 |
| 338 | 0.0 | 0.0 |  | 388 | 24.6 | 11.0 |  | 438 | 67.2 | 45.0 |
| 439 | 78.6 | 21.5 |  | 489 | 45.5 | m |  | 539 | 56.7 | m |
| 440 | 65.5 | 0.0 |  | 490 | 40.4 | m |  | 540 | 46.9 | m |
| 441 | 52.4 | 31.3 |  | 491 | 49.7 | 0.0 |  | 541 | 37.5 | m |
| 442 | 56.4 | 60.1 |  | 492 | 59.0 | m |  | 542 | 30.3 | m |
| 443 | 59.7 | 29.2 |  | 493 | 48.9 | m |  | 543 | 27.3 | 32.3 |
| 444 | 45.1 | 0.0 |  | 494 | 40.0 | m |  | 544 | 30.8 | 60.3 |
| 445 | 30.6 | 4.2 |  | 495 | 33.5 | m |  | 545 | 41.2 | 62.3 |
| 446 | 30.9 | 8.4 |  | 496 | 30.0 | m |  | 546 | 36.0 | 0.0 |
| 447 | 30.5 | 4.3 |  | 497 | 29.1 | 12.0 |  | 547 | 30.8 | 32.3 |
| 448 | 44.6 | 0.0 |  | 498 | 29.3 | 40.4 |  | 548 | 33.9 | 60.3 |
| 449 | 58.8 | m |  | 499 | 30.4 | 29.3 |  | 549 | 34.6 | 38.4 |
| 450 | 55.1 | m |  | 500 | 32.2 | 15.4 |  | 550 | 37.0 | 16.6 |
| 451 | 50.6 | m |  | 501 | 33.9 | 15.8 |  | 551 | 42.7 | 62.3 |
| 452 | 45.3 | m |  | 502 | 35.3 | 14.9 |  | 552 | 50.4 | 28.1 |
| 453 | 39.3 | m |  | 503 | 36.4 | 15.1 |  | 553 | 40.1 | 0.0 |
| 454 | 49.1 | 0.0 |  | 504 | 38.0 | 15.3 |  | 554 | 29.9 | 8.0 |
| 455 | 58.8 | m |  | 505 | 40.3 | 50.9 |  | 555 | 32.5 | 15.0 |
| 456 | 50.7 | m |  | 506 | 43.0 | 39.7 |  | 556 | 34.6 | 63.1 |
| 457 | 42.4 | m |  | 507 | 45.5 | 20.6 |  | 557 | 36.7 | 58.0 |
| 458 | 44.1 | 0.0 |  | 508 | 47.3 | 20.6 |  | 558 | 39.4 | 52.9 |
| 459 | 45.7 | m |  | 509 | 48.8 | 22.1 |  | 559 | 42.8 | 47.8 |
| 460 | 32.5 | m |  | 510 | 50.1 | 22.1 |  | 560 | 46.8 | 42.7 |
| 461 | 20.7 | m |  | 511 | 51.4 | 42.4 |  | 561 | 50.7 | 27.5 |
| 462 | 10.0 | m |  | 512 | 52.5 | 31.9 |  | 562 | 53.4 | 20.7 |
| 463 | 0.0 | 0.0 |  | 513 | 53.7 | 21.6 |  | 563 | 54.2 | 13.1 |
| 464 | 0.0 | 1.5 |  | 514 | 55.1 | 11.6 |  | 564 | 54.2 | 0.4 |
| 465 | 0.9 | 41.1 |  | 515 | 56.8 | 5.7 |  | 565 | 53.4 | 0.0 |
| 466 | 7.0 | 46.3 |  | 516 | 42.4 | 0.0 |  | 566 | 51.4 | m |
| 467 | 12.8 | 48.5 |  | 517 | 27.9 | 8.2 |  | 567 | 48.7 | m |
| 468 | 17.0 | 50.7 |  | 518 | 29.0 | 15.9 |  | 568 | 45.6 | m |
| 469 | 20.9 | 52.9 |  | 519 | 30.4 | 25.1 |  | 569 | 42.4 | m |
| 470 | 26.7 | 55.0 |  | 520 | 32.6 | 60.5 |  | 570 | 40.4 | m |
| 471 | 35.5 | 57.2 |  | 521 | 35.4 | 72.7 |  | 571 | 39.8 | 5.8 |
| 472 | 46.9 | 23.8 |  | 522 | 38.4 | 88.2 |  | 572 | 40.7 | 39.7 |
| 473 | 44.5 | 0.0 |  | 523 | 41.0 | 65.1 |  | 573 | 43.8 | 37.1 |
| 474 | 42.1 | 45.7 |  | 524 | 42.9 | 25.6 |  | 574 | 48.1 | 39.1 |
| 475 | 55.6 | 77.4 |  | 525 | 44.2 | 15.8 |  | 575 | 52.0 | 22.0 |
| 476 | 68.8 | 100.0 |  | 526 | 44.9 | 2.9 |  | 576 | 54.7 | 13.2 |
| 477 | 81.7 | 47.9 |  | 527 | 45.1 | m |  | 577 | 56.4 | 13.2 |
| 478 | 71.2 | 0.0 |  | 528 | 44.8 | m |  | 578 | 57.5 | 6.6 |
| 479 | 60.7 | 38.3 |  | 529 | 43.9 | m |  | 579 | 42.6 | 0.0 |
| 480 | 68.8 | 72.7 |  | 530 | 42.4 | m |  | 580 | 27.7 | 10.9 |
| 481 | 75.0 | m |  | 531 | 40.2 | m |  | 581 | 28.5 | 21.3 |
| 482 | 61.3 | m |  | 532 | 37.1 | m |  | 582 | 29.2 | 23.9 |
| 483 | 53.5 | m |  | 533 | 47.0 | 0.0 |  | 583 | 29.5 | 15.2 |
| 484 | 45.9 | 58.0 |  | 534 | 57.0 | m |  | 584 | 29.7 | 8.8 |
| 485 | 48.1 | 80.0 |  | 535 | 45.1 | m |  | 585 | 30.4 | 20.8 |
| 486 | 49.4 | 97.9 |  | 536 | 32.6 | m |  | 586 | 31.9 | 22.9 |
| 487 | 49.7 | m |  | 537 | 46.8 | 0.0 |  | 587 | 34.3 | 61.4 |
| 488 | 48.7 | m |  | 538 | 61.5 | m |  | 588 | 37.2 | 76.6 |
| 589 | 40.1 | 27.5 |  | 639 | 39.8 | m |  | 689 | 46.6 | 0.0 |
| 590 | 42.3 | 25.4 |  | 640 | 36.0 | m |  | 690 | 32.3 | 34.6 |
| 591 | 43.5 | 32.0 |  | 641 | 29.7 | m |  | 691 | 32.7 | 68.6 |
| 592 | 43.8 | 6.0 |  | 642 | 21.5 | m |  | 692 | 32.6 | 67.0 |
| 593 | 43.5 | m |  | 643 | 14.1 | m |  | 693 | 31.3 | m |
| 594 | 42.8 | m |  | 644 | 0.0 | 0.0 |  | 694 | 28.1 | m |
| 595 | 41.7 | m |  | 645 | 0.0 | 0.0 |  | 695 | 43.0 | 0.0 |
| 596 | 40.4 | m |  | 646 | 0.0 | 0.0 |  | 696 | 58.0 | m |
| 597 | 39.3 | m |  | 647 | 0.0 | 0.0 |  | 697 | 58.9 | m |
| 598 | 38.9 | 12.9 |  | 648 | 0.0 | 0.0 |  | 698 | 49.4 | m |
| 599 | 39.0 | 18.4 |  | 649 | 0.0 | 0.0 |  | 699 | 41.5 | m |
| 600 | 39.7 | 39.2 |  | 650 | 0.0 | 0.0 |  | 700 | 48.4 | 0.0 |
| 601 | 41.4 | 60.0 |  | 651 | 0.0 | 0.0 |  | 701 | 55.3 | m |
| 602 | 43.7 | 54.5 |  | 652 | 0.0 | 0.0 |  | 702 | 41.8 | m |
| 603 | 46.2 | 64.2 |  | 653 | 0.0 | 0.0 |  | 703 | 31.6 | m |
| 604 | 48.8 | 73.3 |  | 654 | 0.0 | 0.0 |  | 704 | 24.6 | m |
| 605 | 51.0 | 82.3 |  | 655 | 0.0 | 0.0 |  | 705 | 15.2 | m |
| 606 | 52.1 | 0.0 |  | 656 | 0.0 | 3.4 |  | 706 | 7.0 | m |
| 607 | 52.0 | m |  | 657 | 1.4 | 22.0 |  | 707 | 0.0 | 0.0 |
| 608 | 50.9 | m |  | 658 | 10.1 | 45.3 |  | 708 | 0.0 | 0.0 |
| 609 | 49.4 | m |  | 659 | 21.5 | 10.0 |  | 709 | 0.0 | 0.0 |
| 610 | 47.8 | m |  | 660 | 32.2 | 0.0 |  | 710 | 0.0 | 0.0 |
| 611 | 46.6 | m |  | 661 | 42.3 | 46.0 |  | 711 | 0.0 | 0.0 |
| 612 | 47.3 | 35.3 |  | 662 | 57.1 | 74.1 |  | 712 | 0.0 | 0.0 |
| 613 | 49.2 | 74.1 |  | 663 | 72.1 | 34.2 |  | 713 | 0.0 | 0.0 |
| 614 | 51.1 | 95.2 |  | 664 | 66.9 | 0.0 |  | 714 | 0.0 | 0.0 |
| 615 | 51.7 | m |  | 665 | 60.4 | 41.8 |  | 715 | 0.0 | 0.0 |
| 616 | 50.8 | m |  | 666 | 69.1 | 79.0 |  | 716 | 0.0 | 0.0 |
| 617 | 47.3 | m |  | 667 | 77.1 | 38.3 |  | 717 | 0.0 | 0.0 |
| 618 | 41.8 | m |  | 668 | 63.1 | 0.0 |  | 718 | 0.0 | 0.0 |
| 619 | 36.4 | m |  | 669 | 49.1 | 47.9 |  | 719 | 0.0 | 0.0 |
| 620 | 30.9 | m |  | 670 | 53.4 | 91.3 |  | 720 | 0.0 | 0.0 |
| 621 | 25.5 | 37.1 |  | 671 | 57.5 | 85.7 |  | 721 | 0.0 | 0.0 |
| 622 | 33.8 | 38.4 |  | 672 | 61.5 | 89.2 |  | 722 | 0.0 | 0.0 |
| 623 | 42.1 | m |  | 673 | 65.5 | 85.9 |  | 723 | 0.0 | 0.0 |
| 624 | 34.1 | m |  | 674 | 69.5 | 89.5 |  | 724 | 0.0 | 0.0 |
| 625 | 33.0 | 37.1 |  | 675 | 73.1 | 75.5 |  | 725 | 0.0 | 0.0 |
| 626 | 36.4 | 38.4 |  | 676 | 76.2 | 73.6 |  | 726 | 0.0 | 0.0 |
| 627 | 43.3 | 17.1 |  | 677 | 79.1 | 75.6 |  | 727 | 0.0 | 0.0 |
| 628 | 35.7 | 0.0 |  | 678 | 81.8 | 78.2 |  | 728 | 0.0 | 0.0 |
| 629 | 28.1 | 11.6 |  | 679 | 84.1 | 39.0 |  | 729 | 0.0 | 0.0 |
| 630 | 36.5 | 19.2 |  | 680 | 69.6 | 0.0 |  | 730 | 0.0 | 0.0 |
| 631 | 45.2 | 8.3 |  | 681 | 55.0 | 25.2 |  | 731 | 0.0 | 0.0 |
| 632 | 36.5 | 0.0 |  | 682 | 55.8 | 49.9 |  | 732 | 0.0 | 0.0 |
| 633 | 27.9 | 32.6 |  | 683 | 56.7 | 46.4 |  | 733 | 0.0 | 0.0 |
| 634 | 31.5 | 59.6 |  | 684 | 57.6 | 76.3 |  | 734 | 0.0 | 0.0 |
| 635 | 34.4 | 65.2 |  | 685 | 58.4 | 92.7 |  | 735 | 0.0 | 0.0 |
| 636 | 37.0 | 59.6 |  | 686 | 59.3 | 99.9 |  | 736 | 0.0 | 0.0 |
| 637 | 39.0 | 49.0 |  | 687 | 60.1 | 95.0 |  | 737 | 0.0 | 0.0 |
| 638 | 40.2 | m |  | 688 | 61.0 | 46.7 |  | 738 | 0.0 | 0.0 |
| 739 | 0.0 | 0.0 |  | 789 | 17.2 | m |  | 839 | 38.1 | m |
| 740 | 0.0 | 0.0 |  | 790 | 14.0 | 37.6 |  | 840 | 37.2 | 42.7 |
| 741 | 0.0 | 0.0 |  | 791 | 18.4 | 25.0 |  | 841 | 37.5 | 70.8 |
| 742 | 0.0 | 0.0 |  | 792 | 27.6 | 17.7 |  | 842 | 39.1 | 48.6 |
| 743 | 0.0 | 0.0 |  | 793 | 39.8 | 6.8 |  | 843 | 41.3 | 0.1 |
| 744 | 0.0 | 0.0 |  | 794 | 34.3 | 0.0 |  | 844 | 42.3 | m |
| 745 | 0.0 | 0.0 |  | 795 | 28.7 | 26.5 |  | 845 | 42.0 | m |
| 746 | 0.0 | 0.0 |  | 796 | 41.5 | 40.9 |  | 846 | 40.8 | m |
| 747 | 0.0 | 0.0 |  | 797 | 53.7 | 17.5 |  | 847 | 38.6 | m |
| 748 | 0.0 | 0.0 |  | 798 | 42.4 | 0.0 |  | 848 | 35.5 | m |
| 749 | 0.0 | 0.0 |  | 799 | 31.2 | 27.3 |  | 849 | 32.1 | m |
| 750 | 0.0 | 0.0 |  | 800 | 32.3 | 53.2 |  | 850 | 29.6 | m |
| 751 | 0.0 | 0.0 |  | 801 | 34.5 | 60.6 |  | 851 | 28.8 | 39.9 |
| 752 | 0.0 | 0.0 |  | 802 | 37.6 | 68.0 |  | 852 | 29.2 | 52.9 |
| 753 | 0.0 | 0.0 |  | 803 | 41.2 | 75.4 |  | 853 | 30.9 | 76.1 |
| 754 | 0.0 | 0.0 |  | 804 | 45.8 | 82.8 |  | 854 | 34.3 | 76.5 |
| 755 | 0.0 | 0.0 |  | 805 | 52.3 | 38.2 |  | 855 | 38.3 | 75.5 |
| 756 | 0.0 | 0.0 |  | 806 | 42.5 | 0.0 |  | 856 | 42.5 | 74.8 |
| 757 | 0.0 | 0.0 |  | 807 | 32.6 | 30.5 |  | 857 | 46.6 | 74.2 |
| 758 | 0.0 | 0.0 |  | 808 | 35.0 | 57.9 |  | 858 | 50.7 | 76.2 |
| 759 | 0.0 | 0.0 |  | 809 | 36.0 | 77.3 |  | 859 | 54.8 | 75.1 |
| 760 | 0.0 | 0.0 |  | 810 | 37.1 | 96.8 |  | 860 | 58.7 | 36.3 |
| 761 | 0.0 | 0.0 |  | 811 | 39.6 | 80.8 |  | 861 | 45.2 | 0.0 |
| 762 | 0.0 | 0.0 |  | 812 | 43.4 | 78.3 |  | 862 | 31.8 | 37.2 |
| 763 | 0.0 | 0.0 |  | 813 | 47.2 | 73.4 |  | 863 | 33.8 | 71.2 |
| 764 | 0.0 | 0.0 |  | 814 | 49.6 | 66.9 |  | 864 | 35.5 | 46.4 |
| 765 | 0.0 | 0.0 |  | 815 | 50.2 | 62.0 |  | 865 | 36.6 | 33.6 |
| 766 | 0.0 | 0.0 |  | 816 | 50.2 | 57.7 |  | 866 | 37.2 | 20.0 |
| 767 | 0.0 | 0.0 |  | 817 | 50.6 | 62.1 |  | 867 | 37.2 | m |
| 768 | 0.0 | 0.0 |  | 818 | 52.3 | 62.9 |  | 868 | 37.0 | m |
| 769 | 0.0 | 0.0 |  | 819 | 54.8 | 37.5 |  | 869 | 36.6 | m |
| 770 | 0.0 | 0.0 |  | 820 | 57.0 | 18.3 |  | 870 | 36.0 | m |
| 771 | 0.0 | 22.0 |  | 821 | 42.3 | 0.0 |  | 871 | 35.4 | m |
| 772 | 4.5 | 25.8 |  | 822 | 27.6 | 29.1 |  | 872 | 34.7 | m |
| 773 | 15.5 | 42.8 |  | 823 | 28.4 | 57.0 |  | 873 | 34.1 | m |
| 774 | 30.5 | 46.8 |  | 824 | 29.1 | 51.8 |  | 874 | 33.6 | m |
| 775 | 45.5 | 29.3 |  | 825 | 29.6 | 35.3 |  | 875 | 33.3 | m |
| 776 | 49.2 | 13.6 |  | 826 | 29.7 | 33.3 |  | 876 | 33.1 | m |
| 777 | 39.5 | 0.0 |  | 827 | 29.8 | 17.7 |  | 877 | 32.7 | m |
| 778 | 29.7 | 15.1 |  | 828 | 29.5 | m |  | 878 | 31.4 | m |
| 779 | 34.8 | 26.9 |  | 829 | 28.9 | m |  | 879 | 45.0 | 0.0 |
| 780 | 40.0 | 13.6 |  | 830 | 43.0 | 0.0 |  | 880 | 58.5 | m |
| 781 | 42.2 | m |  | 831 | 57.1 | m |  | 881 | 53.7 | m |
| 782 | 42.1 | m |  | 832 | 57.7 | m |  | 882 | 47.5 | m |
| 783 | 40.8 | m |  | 833 | 56.0 | m |  | 883 | 40.6 | m |
| 784 | 37.7 | 37.6 |  | 834 | 53.8 | m |  | 884 | 34.1 | m |
| 785 | 47.0 | 35.0 |  | 835 | 51.2 | m |  | 885 | 45.3 | 0.0 |
| 786 | 48.8 | 33.4 |  | 836 | 48.1 | m |  | 886 | 56.4 | m |
| 787 | 41.7 | m |  | 837 | 44.5 | m |  | 887 | 51.0 | m |
| 788 | 27.7 | m |  | 838 | 40.9 | m |  | 888 | 44.5 | m |
| 889 | 36.4 | m |  | 939 | 32.7 | 56.5 |  | 989 | 32.6 | m |
| 890 | 26.6 | m |  | 940 | 33.4 | 62.8 |  | 990 | 30.9 | m |
| 891 | 20.0 | m |  | 941 | 34.6 | 68.2 |  | 991 | 29.9 | m |
| 892 | 13.3 | m |  | 942 | 35.8 | 68.6 |  | 992 | 29.2 | m |
| 893 | 6.7 | m |  | 943 | 38.6 | 65.0 |  | 993 | 44.1 | 0.0 |
| 894 | 0.0 | 0.0 |  | 944 | 42.3 | 61.9 |  | 994 | 59.1 | m |
| 895 | 0.0 | 0.0 |  | 945 | 44.1 | 65.3 |  | 995 | 56.8 | m |
| 896 | 0.0 | 0.0 |  | 946 | 45.3 | 63.2 |  | 996 | 53.5 | m |
| 897 | 0.0 | 0.0 |  | 947 | 46.5 | 30.6 |  | 997 | 47.8 | m |
| 898 | 0.0 | 0.0 |  | 948 | 46.7 | 11.1 |  | 998 | 41.9 | m |
| 899 | 0.0 | 0.0 |  | 949 | 45.9 | 16.1 |  | 999 | 35.9 | m |
| 900 | 0.0 | 0.0 |  | 950 | 45.6 | 21.8 |  | 1000 | 44.3 | 0.0 |
| 901 | 0.0 | 5.8 |  | 951 | 45.9 | 24.2 |  | 1001 | 52.6 | m |
| 902 | 2.5 | 27.9 |  | 952 | 46.5 | 24.7 |  | 1002 | 43.4 | m |
| 903 | 12.4 | 29.0 |  | 953 | 46.7 | 24.7 |  | 1003 | 50.6 | 0.0 |
| 904 | 19.4 | 30.1 |  | 954 | 46.8 | 28.2 |  | 1004 | 57.8 | m |
| 905 | 29.3 | 31.2 |  | 955 | 47.2 | 31.2 |  | 1005 | 51.6 | m |
| 906 | 37.1 | 10.4 |  | 956 | 47.6 | 29.6 |  | 1006 | 44.8 | m |
| 907 | 40.6 | 4.9 |  | 957 | 48.2 | 31.2 |  | 1007 | 48.6 | 0.0 |
| 908 | 35.8 | 0.0 |  | 958 | 48.6 | 33.5 |  | 1008 | 52.4 | m |
| 909 | 30.9 | 7.6 |  | 959 | 48.8 | m |  | 1009 | 45.4 | m |
| 910 | 35.4 | 13.8 |  | 960 | 47.6 | m |  | 1010 | 37.2 | m |
| 911 | 36.5 | 11.1 |  | 961 | 46.3 | m |  | 1011 | 26.3 | m |
| 912 | 40.8 | 48.5 |  | 962 | 45.2 | m |  | 1012 | 17.9 | m |
| 913 | 49.8 | 3.7 |  | 963 | 43.5 | m |  | 1013 | 16.2 | 1.9 |
| 914 | 41.2 | 0.0 |  | 964 | 41.4 | m |  | 1014 | 17.8 | 7.5 |
| 915 | 32.7 | 29.7 |  | 965 | 40.3 | m |  | 1015 | 25.2 | 18.0 |
| 916 | 39.4 | 52.1 |  | 966 | 39.4 | m |  | 1016 | 39.7 | 6.5 |
| 917 | 48.8 | 22.7 |  | 967 | 38.0 | m |  | 1017 | 38.6 | 0.0 |
| 918 | 41.6 | 0.0 |  | 968 | 36.3 | m |  | 1018 | 37.4 | 5.4 |
| 919 | 34.5 | 46.6 |  | 969 | 35.3 | 5.8 |  | 1019 | 43.4 | 9.7 |
| 920 | 39.7 | 84.4 |  | 970 | 35.4 | 30.2 |  | 1020 | 46.9 | 15.7 |
| 921 | 44.7 | 83.2 |  | 971 | 36.6 | 55.6 |  | 1021 | 52.5 | 13.1 |
| 922 | 49.5 | 78.9 |  | 972 | 38.6 | 48.5 |  | 1022 | 56.2 | 6.3 |
| 923 | 52.3 | 83.8 |  | 973 | 39.9 | 41.8 |  | 1023 | 44.0 | 0.0 |
| 924 | 53.4 | 77.7 |  | 974 | 40.3 | 38.2 |  | 1024 | 31.8 | 20.9 |
| 925 | 52.1 | 69.6 |  | 975 | 40.8 | 35.0 |  | 1025 | 38.7 | 36.3 |
| 926 | 47.9 | 63.6 |  | 976 | 41.9 | 32.4 |  | 1026 | 47.7 | 47.5 |
| 927 | 46.4 | 55.2 |  | 977 | 43.2 | 26.4 |  | 1027 | 54.5 | 22.0 |
| 928 | 46.5 | 53.6 |  | 978 | 43.5 | m |  | 1028 | 41.3 | 0.0 |
| 929 | 46.4 | 62.3 |  | 979 | 42.9 | m |  | 1029 | 28.1 | 26.8 |
| 930 | 46.1 | 58.2 |  | 980 | 41.5 | m |  | 1030 | 31.6 | 49.2 |
| 931 | 46.2 | 61.8 |  | 981 | 40.9 | m |  | 1031 | 34.5 | 39.5 |
| 932 | 47.3 | 62.3 |  | 982 | 40.5 | m |  | 1032 | 36.4 | 24.0 |
| 933 | 49.3 | 57.1 |  | 983 | 39.5 | m |  | 1033 | 36.7 | m |
| 934 | 52.6 | 58.1 |  | 984 | 38.3 | m |  | 1034 | 35.5 | m |
| 935 | 56.3 | 56.0 |  | 985 | 36.9 | m |  | 1035 | 33.8 | m |
| 936 | 59.9 | 27.2 |  | 986 | 35.4 | m |  | 1036 | 33.7 | 19.8 |
| 937 | 45.8 | 0.0 |  | 987 | 34.5 | m |  | 1037 | 35.3 | 35.1 |
| 938 | 31.8 | 28.8 |  | 988 | 33.9 | m |  | 1038 | 38.0 | 33.9 |
| 1039 | 40.1 | 34.5 |  | 1,089 | 46.3 | 24.0 |  | 1139 | 51.7 | 0.0 |
| 1040 | 42.2 | 40.4 |  | 1,090 | 47.8 | 20.6 |  | 1140 | 59.2 | m |
| 1041 | 45.2 | 44.0 |  | 1,091 | 47.2 | 3.8 |  | 1141 | 47.2 | m |
| 1042 | 48.3 | 35.9 |  | 1,092 | 45.6 | 4.4 |  | 1142 | 35.1 | 0.0 |
| 1043 | 50.1 | 29.6 |  | 1,093 | 44.6 | 4.1 |  | 1143 | 23.1 | m |
| 1044 | 52.3 | 38.5 |  | 1,094 | 44.1 | m |  | 1144 | 13.1 | m |
| 1045 | 55.3 | 57.7 |  | 1,095 | 42.9 | m |  | 1145 | 5.0 | m |
| 1046 | 57.0 | 50.7 |  | 1,096 | 40.9 | m |  | 1146 | 0.0 | 0.0 |
| 1047 | 57.7 | 25.2 |  | 1,097 | 39.2 | m |  | 1147 | 0.0 | 0.0 |
| 1048 | 42.9 | 0.0 |  | 1,098 | 37.0 | m |  | 1148 | 0.0 | 0.0 |
| 1049 | 28.2 | 15.7 |  | 1,099 | 35.1 | 2.0 |  | 1149 | 0.0 | 0.0 |
| 1050 | 29.2 | 30.5 |  | 1,100 | 35.6 | 43.3 |  | 1150 | 0.0 | 0.0 |
| 1051 | 31.1 | 52.6 |  | 1,101 | 38.7 | 47.6 |  | 1151 | 0.0 | 0.0 |
| 1052 | 33.4 | 60.7 |  | 1,102 | 41.3 | 40.4 |  | 1152 | 0.0 | 0.0 |
| 1053 | 35.0 | 61.4 |  | 1,103 | 42.6 | 45.7 |  | 1153 | 0.0 | 0.0 |
| 1054 | 35.3 | 18.2 |  | 1,104 | 43.9 | 43.3 |  | 1154 | 0.0 | 0.0 |
| 1055 | 35.2 | 14.9 |  | 1,105 | 46.9 | 41.2 |  | 1155 | 0.0 | 0.0 |
| 1056 | 34.9 | 11.7 |  | 1,106 | 52.4 | 40.1 |  | 1156 | 0.0 | 0.0 |
| 1057 | 34.5 | 12.9 |  | 1,107 | 56.3 | 39.3 |  | 1157 | 0.0 | 0.0 |
| 1058 | 34.1 | 15.5 |  | 1108 | 57.4 | 25.5 |  | 1158 | 0.0 | 0.0 |
| 1059 | 33.5 | m |  | 1109 | 57.2 | 25.4 |  | 1159 | 0.0 | 0.0 |
| 1060 | 31.8 | m |  | 1110 | 57.0 | 25.4 |  | 1160 | 0.0 | 0.0 |
| 1061 | 30.1 | m |  | 1111 | 56.8 | 25.3 |  | 1161 | 0.0 | 0.0 |
| 1062 | 29.6 | 10.3 |  | 1112 | 56.3 | 25.3 |  | 1162 | 0.0 | 0.0 |
| 1063 | 30.0 | 26.5 |  | 1113 | 55.6 | 25.2 |  | 1163 | 0.0 | 0.0 |
| 1064 | 31.0 | 18.8 |  | 1114 | 56.2 | 25.2 |  | 1164 | 0.0 | 0.0 |
| 1065 | 31.5 | 26.5 |  | 1115 | 58.0 | 12.4 |  | 1165 | 0.0 | 0.0 |
| 1066 | 31.7 | m |  | 1116 | 43.4 | 0.0 |  | 1166 | 0.0 | 0.0 |
| 1067 | 31.5 | m |  | 1117 | 28.8 | 26.2 |  | 1167 | 0.0 | 0.0 |
| 1068 | 30.6 | m |  | 1118 | 30.9 | 49.9 |  | 1168 | 0.0 | 0.0 |
| 1069 | 30.0 | m |  | 1119 | 32.3 | 40.5 |  | 1169 | 0.0 | 0.0 |
| 1070 | 30.0 | m |  | 1120 | 32.5 | 12.4 |  | 1170 | 0.0 | 0.0 |
| 1071 | 29.4 | m |  | 1121 | 32.4 | 12.2 |  | 1171 | 0.0 | 0.0 |
| 1072 | 44.3 | 0.0 |  | 1122 | 32.1 | 6.4 |  | 1172 | 0.0 | 0.0 |
| 1073 | 59.2 | m |  | 1123 | 31.0 | 12.4 |  | 1173 | 0.0 | 0.0 |
| 1074 | 58.3 | m |  | 1124 | 30.1 | 18.5 |  | 1174 | 0.0 | 0.0 |
| 1075 | 57.1 | m |  | 1125 | 30.4 | 35.6 |  | 1175 | 0.0 | 0.0 |
| 1076 | 55.4 | m |  | 1126 | 31.2 | 30.1 |  | 1176 | 0.0 | 0.0 |
| 1077 | 53.5 | m |  | 1127 | 31.5 | 30.8 |  | 1177 | 0.0 | 0.0 |
| 1078 | 51.5 | m |  | 1128 | 31.5 | 26.9 |  | 1178 | 0.0 | 0.0 |
| 1079 | 49.7 | m |  | 1129 | 31.7 | 33.9 |  | 1179 | 0.0 | 0.0 |
| 1080 | 47.9 | m |  | 1130 | 32.0 | 29.9 |  | 1180 | 0.0 | 0.0 |
| 1081 | 46.4 | m |  | 1131 | 32.1 | m |  | 1181 | 0.0 | 0.0 |
| 1082 | 45.5 | m |  | 1132 | 31.4 | m |  | 1182 | 0.0 | 0.0 |
| 1083 | 45.2 | m |  | 1133 | 30.3 | m |  | 1183 | 0.0 | 0.0 |
| 1084 | 44.3 | m |  | 1134 | 29.8 | m |  | 1184 | 0.0 | 0.0 |
| 1085 | 43.6 | m |  | 1135 | 44.3 | 0.0 |  | 1185 | 0.0 | 0.0 |
| 1086 | 43.1 | m |  | 1136 | 58.9 | m |  | 1186 | 0.0 | 0.0 |
| 1087 | 42.5 | 25.6 |  | 1137 | 52.1 | m |  | 1187 | 0.0 | 0.0 |
| 1088 | 43.3 | 25.7 |  | 1138 | 44.1 | m |  | 1188 | 0.0 | 0.0 |
| 1189 | 0.0 | 0.0 |  | 1239 | 58.5 | 85.4 |  | 1289 | 61.9 | 76.1 |
| 1190 | 0.0 | 0.0 |  | 1240 | 59.5 | 85.6 |  | 1290 | 65.6 | 73.7 |
| 1191 | 0.0 | 0.0 |  | 1241 | 61.0 | 86.6 |  | 1291 | 69.9 | 79.3 |
| 1192 | 0.0 | 0.0 |  | 1242 | 62.6 | 86.8 |  | 1292 | 74.1 | 81.3 |
| 1193 | 0.0 | 0.0 |  | 1243 | 64.1 | 87.6 |  | 1293 | 78.3 | 83.2 |
| 1194 | 0.0 | 0.0 |  | 1244 | 65.4 | 87.5 |  | 1294 | 82.6 | 86.0 |
| 1195 | 0.0 | 0.0 |  | 1245 | 66.7 | 87.8 |  | 1295 | 87.0 | 89.5 |
| 1196 | 0.0 | 20.4 |  | 1246 | 68.1 | 43.5 |  | 1296 | 91.2 | 90.8 |
| 1197 | 12.6 | 41.2 |  | 1247 | 55.2 | 0.0 |  | 1297 | 95.3 | 45.9 |
| 1198 | 27.3 | 20.4 |  | 1248 | 42.3 | 37.2 |  | 1298 | 81.0 | 0.0 |
| 1199 | 40.4 | 7.6 |  | 1249 | 43.0 | 73.6 |  | 1299 | 66.6 | 38.2 |
| 1200 | 46.1 | m |  | 1250 | 43.5 | 65.1 |  | 1300 | 67.9 | 75.5 |
| 1201 | 44.6 | m |  | 1251 | 43.8 | 53.1 |  | 1301 | 68.4 | 80.5 |
| 1202 | 42.7 | 14.7 |  | 1252 | 43.9 | 54.6 |  | 1302 | 69.0 | 85.5 |
| 1203 | 42.9 | 7.3 |  | 1253 | 43.9 | 41.2 |  | 1303 | 70.0 | 85.2 |
| 1204 | 36.1 | 0.0 |  | 1254 | 43.8 | 34.8 |  | 1304 | 71.6 | 85.9 |
| 1205 | 29.3 | 15.0 |  | 1255 | 43.6 | 30.3 |  | 1305 | 73.3 | 86.2 |
| 1206 | 43.8 | 22.6 |  | 1256 | 43.3 | 21.9 |  | 1306 | 74.8 | 86.5 |
| 1207 | 54.9 | 9.9 |  | 1257 | 42.8 | 19.9 |  | 1307 | 76.3 | 42.9 |
| 1208 | 44.9 | 0.0 |  | 1258 | 42.3 | m |  | 1308 | 63.3 | 0.0 |
| 1209 | 34.9 | 47.4 |  | 1259 | 41.4 | m |  | 1309 | 50.4 | 21.2 |
| 1210 | 42.7 | 82.7 |  | 1260 | 40.2 | m |  | 1310 | 50.6 | 42.3 |
| 1211 | 52.0 | 81.2 |  | 1261 | 38.7 | m |  | 1311 | 50.6 | 53.7 |
| 1212 | 61.8 | 82.7 |  | 1262 | 37.1 | m |  | 1312 | 50.4 | 90.1 |
| 1213 | 71.3 | 39.1 |  | 1263 | 35.6 | m |  | 1313 | 50.5 | 97.1 |
| 1214 | 58.1 | 0.0 |  | 1264 | 34.2 | m |  | 1314 | 51.0 | 100.0 |
| 1215 | 44.9 | 42.5 |  | 1265 | 32.9 | m |  | 1315 | 51.9 | 100.0 |
| 1216 | 46.3 | 83.3 |  | 1266 | 31.8 | m |  | 1316 | 52.6 | 100.0 |
| 1217 | 46.8 | 74.1 |  | 1267 | 30.7 | m |  | 1317 | 52.8 | 32.4 |
| 1218 | 48.1 | 75.7 |  | 1268 | 29.6 | m |  | 1318 | 47.7 | 0.0 |
| 1219 | 50.5 | 75.8 |  | 1269 | 40.4 | 0.0 |  | 1319 | 42.6 | 27.4 |
| 1220 | 53.6 | 76.7 |  | 1270 | 51.2 | m |  | 1320 | 42.1 | 53.5 |
| 1221 | 56.9 | 77.1 |  | 1271 | 49.6 | m |  | 1321 | 41.8 | 44.5 |
| 1222 | 60.2 | 78.7 |  | 1272 | 48.0 | m |  | 1322 | 41.4 | 41.1 |
| 1223 | 63.7 | 78.0 |  | 1273 | 46.4 | m |  | 1323 | 41.0 | 21.0 |
| 1224 | 67.2 | 79.6 |  | 1274 | 45.0 | m |  | 1324 | 40.3 | 0.0 |
| 1225 | 70.7 | 80.9 |  | 1275 | 43.6 | m |  | 1325 | 39.3 | 1.0 |
| 1226 | 74.1 | 81.1 |  | 1276 | 42.3 | m |  | 1326 | 38.3 | 15.2 |
| 1227 | 77.5 | 83.6 |  | 1277 | 41.0 | m |  | 1327 | 37.6 | 57.8 |
| 1228 | 80.8 | 85.6 |  | 1278 | 39.6 | m |  | 1328 | 37.3 | 73.2 |
| 1229 | 84.1 | 81.6 |  | 1279 | 38.3 | m |  | 1329 | 37.3 | 59.8 |
| 1230 | 87.4 | 88.3 |  | 1280 | 37.1 | m |  | 1330 | 37.4 | 52.2 |
| 1231 | 90.5 | 91.9 |  | 1281 | 35.9 | m |  | 1331 | 37.4 | 16.9 |
| 1232 | 93.5 | 94.1 |  | 1282 | 34.6 | m |  | 1332 | 37.1 | 34.3 |
| 1233 | 96.8 | 96.6 |  | 1283 | 33.0 | m |  | 1333 | 36.7 | 51.9 |
| 1234 | 100.0 | m |  | 1284 | 31.1 | m |  | 1334 | 36.2 | 25.3 |
| 1235 | 96.0 | m |  | 1285 | 29.2 | m |  | 1335 | 35.6 | m |
| 1236 | 81.9 | m |  | 1286 | 43.3 | 0.0 |  | 1336 | 34.6 | m |
| 1237 | 68.1 | m |  | 1287 | 57.4 | 32.8 |  | 1337 | 33.2 | m |
| 1238 | 58.1 | 84.7 |  | 1288 | 59.9 | 65.4 |  | 1338 | 31.6 | m |
| 1339 | 30.1 | m |  | 1389 | 50.4 | 50.2 |  | 1439 | 36.3 | 98.8 |
| 1340 | 28.8 | m |  | 1390 | 53.0 | 26.1 |  | 1440 | 37.7 | 100.0 |
| 1341 | 28.0 | 29.5 |  | 1391 | 59.5 | 0.0 |  | 1441 | 39.2 | 100.0 |
| 1342 | 28.6 | 100.0 |  | 1392 | 66.2 | 38.4 |  | 1442 | 40.9 | 100.0 |
| 1343 | 28.8 | 97.3 |  | 1393 | 66.4 | 76.7 |  | 1443 | 42.4 | 99.5 |
| 1344 | 28.8 | 73.4 |  | 1394 | 67.6 | 100.0 |  | 1444 | 43.8 | 98.7 |
| 1345 | 29.6 | 56.9 |  | 1395 | 68.4 | 76.6 |  | 1445 | 45.4 | 97.3 |
| 1346 | 30.3 | 91.7 |  | 1396 | 68.2 | 47.2 |  | 1446 | 47.0 | 96.6 |
| 1347 | 31.0 | 90.5 |  | 1397 | 69.0 | 81.4 |  | 1447 | 47.8 | 96.2 |
| 1348 | 31.8 | 81.7 |  | 1398 | 69.7 | 40.6 |  | 1448 | 48.8 | 96.3 |
| 1349 | 32.6 | 79.5 |  | 1399 | 54.7 | 0.0 |  | 1449 | 50.5 | 95.1 |
| 1350 | 33.5 | 86.9 |  | 1400 | 39.8 | 19.9 |  | 1450 | 51.0 | 95.9 |
| 1351 | 34.6 | 100.0 |  | 1401 | 36.3 | 40.0 |  | 1451 | 52.0 | 94.3 |
| 1352 | 35.6 | 78.7 |  | 1402 | 36.7 | 59.4 |  | 1452 | 52.6 | 94.6 |
| 1353 | 36.4 | 50.5 |  | 1403 | 36.6 | 77.5 |  | 1453 | 53.0 | 65.5 |
| 1354 | 37.0 | 57.0 |  | 1404 | 36.8 | 94.3 |  | 1454 | 53.2 | 0.0 |
| 1355 | 37.3 | 69.1 |  | 1405 | 36.8 | 100.0 |  | 1455 | 53.2 | m |
| 1356 | 37.6 | 49.5 |  | 1406 | 36.4 | 100.0 |  | 1456 | 52.6 | m |
| 1357 | 37.8 | 44.4 |  | 1407 | 36.3 | 79.7 |  | 1457 | 52.1 | m |
| 1358 | 37.8 | 43.4 |  | 1408 | 36.7 | 49.5 |  | 1458 | 51.8 | m |
| 1359 | 37.8 | 34.8 |  | 1409 | 36.6 | 39.3 |  | 1459 | 51.3 | m |
| 1360 | 37.6 | 24.0 |  | 1410 | 37.3 | 62.8 |  | 1460 | 50.7 | m |
| 1361 | 37.2 | m |  | 1411 | 38.1 | 73.4 |  | 1461 | 50.7 | m |
| 1362 | 36.3 | m |  | 1412 | 39.0 | 72.9 |  | 1462 | 49.8 | m |
| 1363 | 35.1 | m |  | 1413 | 40.2 | 72.0 |  | 1463 | 49.4 | m |
| 1364 | 33.7 | m |  | 1414 | 41.5 | 71.2 |  | 1464 | 49.3 | m |
| 1365 | 32.4 | m |  | 1415 | 42.9 | 77.3 |  | 1465 | 49.1 | m |
| 1366 | 31.1 | m |  | 1416 | 44.4 | 76.6 |  | 1466 | 49.1 | m |
| 1367 | 29.9 | m |  | 1417 | 45.4 | 43.1 |  | 1467 | 49.1 | 8.3 |
| 1368 | 28.7 | m |  | 1418 | 45.3 | 53.9 |  | 1468 | 48.9 | 16.8 |
| 1369 | 29.0 | 58.6 |  | 1419 | 45.1 | 64.8 |  | 1469 | 48.8 | 21.3 |
| 1370 | 29.7 | 88.5 |  | 1420 | 46.5 | 74.2 |  | 1470 | 49.1 | 22.1 |
| 1371 | 31.0 | 86.3 |  | 1421 | 47.7 | 75.2 |  | 1471 | 49.4 | 26.3 |
| 1372 | 31.8 | 43.4 |  | 1422 | 48.1 | 75.5 |  | 1472 | 49.8 | 39.2 |
| 1373 | 31.7 | m |  | 1423 | 48.6 | 75.8 |  | 1473 | 50.4 | 83.4 |
| 1374 | 29.9 | m |  | 1424 | 48.9 | 76.3 |  | 1474 | 51.4 | 90.6 |
| 1375 | 40.2 | 0.0 |  | 1425 | 49.9 | 75.5 |  | 1475 | 52.3 | 93.8 |
| 1376 | 50.4 | m |  | 1426 | 50.4 | 75.2 |  | 1476 | 53.3 | 94.0 |
| 1377 | 47.9 | m |  | 1427 | 51.1 | 74.6 |  | 1477 | 54.2 | 94.1 |
| 1378 | 45.0 | m |  | 1428 | 51.9 | 75.0 |  | 1478 | 54.9 | 94.3 |
| 1379 | 43.0 | m |  | 1429 | 52.7 | 37.2 |  | 1479 | 55.7 | 94.6 |
| 1380 | 40.6 | m |  | 1430 | 41.6 | 0.0 |  | 1480 | 56.1 | 94.9 |
| 1381 | 55.5 | 0.0 |  | 1431 | 30.4 | 36.6 |  | 1481 | 56.3 | 86.2 |
| 1382 | 70.4 | 41.7 |  | 1432 | 30.5 | 73.2 |  | 1482 | 56.2 | 64.1 |
| 1383 | 73.4 | 83.2 |  | 1433 | 30.3 | 81.6 |  | 1483 | 56.0 | 46.1 |
| 1384 | 74.0 | 83.7 |  | 1434 | 30.4 | 89.3 |  | 1484 | 56.2 | 33.4 |
| 1385 | 74.9 | 41.7 |  | 1435 | 31.5 | 90.4 |  | 1485 | 56.5 | 23.6 |
| 1386 | 60.0 | 0.0 |  | 1436 | 32.7 | 88.5 |  | 1486 | 56.3 | 18.6 |
| 1387 | 45.1 | 41.6 |  | 1437 | 33.7 | 97.2 |  | 1487 | 55.7 | 16.2 |
| 1388 | 47.7 | 84.2 |  | 1438 | 35.2 | 99.7 |  | 1488 | 56.0 | 15.9 |
| 1489 | 55.9 | 21.8 |  | 1539 | 57.0 | 59.5 |  | 1589 | 56.8 | 42.9 |
| 1490 | 55.8 | 20.9 |  | 1540 | 56.7 | 57.0 |  | 1590 | 56.5 | 42.8 |
| 1491 | 55.4 | 18.4 |  | 1541 | 56.7 | 69.8 |  | 1591 | 56.7 | 43.2 |
| 1492 | 55.7 | 25.1 |  | 1542 | 56.8 | 58.5 |  | 1592 | 56.5 | 42.8 |
| 1493 | 56.0 | 27.7 |  | 1543 | 56.8 | 47.2 |  | 1593 | 56.9 | 42.2 |
| 1494 | 55.8 | 22.4 |  | 1544 | 57.0 | 38.5 |  | 1594 | 56.5 | 43.1 |
| 1495 | 56.1 | 20.0 |  | 1545 | 57.0 | 32.8 |  | 1595 | 56.5 | 42.9 |
| 1496 | 55.7 | 17.4 |  | 1546 | 56.8 | 30.2 |  | 1596 | 56.7 | 42.7 |
| 1497 | 55.9 | 20.9 |  | 1547 | 57.0 | 27.0 |  | 1597 | 56.6 | 41.5 |
| 1498 | 56.0 | 22.9 |  | 1548 | 56.9 | 26.2 |  | 1598 | 56.9 | 41.8 |
| 1499 | 56.0 | 21.1 |  | 1549 | 56.7 | 26.2 |  | 1599 | 56.6 | 41.9 |
| 1500 | 55.1 | 19.2 |  | 1550 | 57.0 | 26.6 |  | 1600 | 56.7 | 42.6 |
| 1501 | 55.6 | 24.2 |  | 1551 | 56.7 | 27.8 |  | 1601 | 56.7 | 42.6 |
| 1502 | 55.4 | 25.6 |  | 1552 | 56.7 | 29.7 |  | 1602 | 56.7 | 41.5 |
| 1503 | 55.7 | 24.7 |  | 1553 | 56.8 | 32.1 |  | 1603 | 56.7 | 42.2 |
| 1504 | 55.9 | 24.0 |  | 1554 | 56.5 | 34.9 |  | 1604 | 56.5 | 42.2 |
| 1505 | 55.4 | 23.5 |  | 1555 | 56.6 | 34.9 |  | 1605 | 56.8 | 41.9 |
| 1506 | 55.7 | 30.9 |  | 1556 | 56.3 | 35.8 |  | 1606 | 56.5 | 42.0 |
| 1507 | 55.4 | 42.5 |  | 1557 | 56.6 | 36.6 |  | 1607 | 56.7 | 42.1 |
| 1508 | 55.3 | 25.8 |  | 1558 | 56.2 | 37.6 |  | 1608 | 56.4 | 41.9 |
| 1509 | 55.4 | 1.3 |  | 1559 | 56.6 | 38.2 |  | 1609 | 56.7 | 42.9 |
| 1510 | 55.0 | m |  | 1560 | 56.2 | 37.9 |  | 1610 | 56.7 | 41.8 |
| 1511 | 54.4 | m |  | 1561 | 56.6 | 37.5 |  | 1611 | 56.7 | 41.9 |
| 1512 | 54.2 | m |  | 1562 | 56.4 | 36.7 |  | 1612 | 56.8 | 42.0 |
| 1513 | 53.5 | m |  | 1563 | 56.5 | 34.8 |  | 1613 | 56.7 | 41.5 |
| 1514 | 52.4 | m |  | 1564 | 56.5 | 35.8 |  | 1614 | 56.6 | 41.9 |
| 1515 | 51.8 | m |  | 1565 | 56.5 | 36.2 |  | 1615 | 56.8 | 41.6 |
| 1516 | 50.7 | m |  | 1566 | 56.5 | 36.7 |  | 1616 | 56.6 | 41.6 |
| 1517 | 49.9 | m |  | 1567 | 56.7 | 37.8 |  | 1617 | 56.9 | 42.0 |
| 1518 | 49.1 | m |  | 1568 | 56.7 | 37.8 |  | 1618 | 56.7 | 40.7 |
| 1519 | 47.7 | m |  | 1569 | 56.6 | 36.6 |  | 1619 | 56.7 | 39.3 |
| 1520 | 47.3 | m |  | 1570 | 56.8 | 36.1 |  | 1620 | 56.5 | 41.4 |
| 1521 | 46.9 | m |  | 1571 | 56.5 | 36.8 |  | 1621 | 56.4 | 44.9 |
| 1522 | 46.9 | m |  | 1572 | 56.9 | 35.9 |  | 1622 | 56.8 | 45.2 |
| 1523 | 47.2 | m |  | 1573 | 56.7 | 35.0 |  | 1623 | 56.6 | 43.6 |
| 1524 | 47.8 | m |  | 1574 | 56.5 | 36.0 |  | 1624 | 56.8 | 42.2 |
| 1525 | 48.2 | 0.0 |  | 1575 | 56.4 | 36.5 |  | 1625 | 56.5 | 42.3 |
| 1526 | 48.8 | 23.0 |  | 1576 | 56.5 | 38.0 |  | 1626 | 56.5 | 44.4 |
| 1527 | 49.1 | 67.9 |  | 1577 | 56.5 | 39.9 |  | 1627 | 56.9 | 45.1 |
| 1528 | 49.4 | 73.7 |  | 1578 | 56.4 | 42.1 |  | 1628 | 56.4 | 45.0 |
| 1529 | 49.8 | 75.0 |  | 1579 | 56.5 | 47.0 |  | 1629 | 56.7 | 46.3 |
| 1530 | 50.4 | 75.8 |  | 1580 | 56.4 | 48.0 |  | 1630 | 56.7 | 45.5 |
| 1531 | 51.4 | 73.9 |  | 1581 | 56.1 | 49.1 |  | 1631 | 56.8 | 45.0 |
| 1532 | 52.3 | 72.2 |  | 1582 | 56.4 | 48.9 |  | 1632 | 56.7 | 44.9 |
| 1533 | 53.3 | 71.2 |  | 1583 | 56.4 | 48.2 |  | 1633 | 56.6 | 45.2 |
| 1534 | 54.6 | 71.2 |  | 1584 | 56.5 | 48.3 |  | 1634 | 56.8 | 46.0 |
| 1535 | 55.4 | 68.7 |  | 1585 | 56.5 | 47.9 |  | 1635 | 56.5 | 46.6 |
| 1536 | 56.7 | 67.0 |  | 1586 | 56.6 | 46.8 |  | 1636 | 56.6 | 48.3 |
| 1537 | 57.2 | 64.6 |  | 1587 | 56.6 | 46.2 |  | 1637 | 56.4 | 48.6 |
| 1538 | 57.3 | 61.9 |  | 1588 | 56.5 | 44.4 |  | 1638 | 56.6 | 50.3 |
| 1639 | 56.3 | 51.9 |  | 1689 | 57.6 | 8.9 |  | 1739 | 56.1 | 46.8 |
| 1640 | 56.5 | 54.1 |  | 1690 | 57.5 | 8.0 |  | 1740 | 56.1 | 45.8 |
| 1641 | 56.3 | 54.9 |  | 1691 | 57.5 | 5.8 |  | 1741 | 56.2 | 46.0 |
| 1642 | 56.4 | 55.0 |  | 1692 | 57.3 | 5.8 |  | 1742 | 56.3 | 45.9 |
| 1643 | 56.4 | 56.2 |  | 1693 | 57.6 | 5.5 |  | 1743 | 56.3 | 45.9 |
| 1644 | 56.2 | 58.6 |  | 1694 | 57.3 | 4.5 |  | 1744 | 56.2 | 44.6 |
| 1645 | 56.2 | 59.1 |  | 1695 | 57.2 | 3.2 |  | 1745 | 56.2 | 46.0 |
| 1646 | 56.2 | 62.5 |  | 1696 | 57.2 | 3.1 |  | 1746 | 56.4 | 46.2 |
| 1647 | 56.4 | 62.8 |  | 1697 | 57.3 | 4.9 |  | 1747 | 55.8 | m |
| 1648 | 56.0 | 64.7 |  | 1698 | 57.3 | 4.2 |  | 1748 | 55.5 | m |
| 1649 | 56.4 | 65.6 |  | 1699 | 56.9 | 5.5 |  | 1749 | 55.0 | m |
| 1650 | 56.2 | 67.7 |  | 1700 | 57.1 | 5.1 |  | 1750 | 54.1 | m |
| 1651 | 55.9 | 68.9 |  | 1701 | 57.0 | 5.2 |  | 1751 | 54.0 | m |
| 1652 | 56.1 | 68.9 |  | 1702 | 56.9 | 5.5 |  | 1752 | 53.3 | m |
| 1653 | 55.8 | 69.5 |  | 1703 | 56.6 | 5.4 |  | 1753 | 52.6 | m |
| 1654 | 56.0 | 69.8 |  | 1704 | 57.1 | 6.1 |  | 1754 | 51.8 | m |
| 1655 | 56.2 | 69.3 |  | 1705 | 56.7 | 5.7 |  | 1755 | 50.7 | m |
| 1656 | 56.2 | 69.8 |  | 1706 | 56.8 | 5.8 |  | 1756 | 49.9 | m |
| 1657 | 56.4 | 69.2 |  | 1707 | 57.0 | 6.1 |  | 1757 | 49.1 | m |
| 1658 | 56.3 | 68.7 |  | 1708 | 56.7 | 5.9 |  | 1758 | 47.7 | m |
| 1659 | 56.2 | 69.4 |  | 1709 | 57.0 | 6.6 |  | 1759 | 46.8 | m |
| 1660 | 56.2 | 69.5 |  | 1710 | 56.9 | 6.4 |  | 1760 | 45.7 | m |
| 1661 | 56.2 | 70.0 |  | 1711 | 56.7 | 6.7 |  | 1761 | 44.8 | m |
| 1662 | 56.4 | 69.7 |  | 1712 | 56.9 | 6.9 |  | 1762 | 43.9 | m |
| 1663 | 56.2 | 70.2 |  | 1713 | 56.8 | 5.6 |  | 1763 | 42.9 | m |
| 1664 | 56.4 | 70.5 |  | 1714 | 56.6 | 5.1 |  | 1764 | 41.5 | m |
| 1665 | 56.1 | 70.5 |  | 1715 | 56.6 | 6.5 |  | 1765 | 39.5 | m |
| 1666 | 56.5 | 69.7 |  | 1716 | 56.5 | 10.0 |  | 1766 | 36.7 | m |
| 1667 | 56.2 | 69.3 |  | 1717 | 56.6 | 12.4 |  | 1767 | 33.8 | m |
| 1668 | 56.5 | 70.9 |  | 1718 | 56.5 | 14.5 |  | 1768 | 31.0 | m |
| 1669 | 56.4 | 70.8 |  | 1719 | 56.6 | 16.3 |  | 1769 | 40.0 | 0.0 |
| 1670 | 56.3 | 71.1 |  | 1720 | 56.3 | 18.1 |  | 1770 | 49.1 | m |
| 1671 | 56.4 | 71.0 |  | 1721 | 56.6 | 20.7 |  | 1771 | 46.2 | m |
| 1672 | 56.7 | 68.6 |  | 1722 | 56.1 | 22.6 |  | 1772 | 43.1 | m |
| 1673 | 56.8 | 68.6 |  | 1723 | 56.3 | 25.8 |  | 1773 | 39.9 | m |
| 1674 | 56.6 | 68.0 |  | 1724 | 56.4 | 27.7 |  | 1774 | 36.6 | m |
| 1675 | 56.8 | 65.1 |  | 1725 | 56.0 | 29.7 |  | 1775 | 33.6 | m |
| 1676 | 56.9 | 60.9 |  | 1726 | 56.1 | 32.6 |  | 1776 | 30.5 | m |
| 1677 | 57.1 | 57.4 |  | 1727 | 55.9 | 34.9 |  | 1777 | 42.8 | 0.0 |
| 1678 | 57.1 | 54.3 |  | 1728 | 55.9 | 36.4 |  | 1778 | 55.2 | m |
| 1679 | 57.0 | 48.6 |  | 1729 | 56.0 | 39.2 |  | 1779 | 49.9 | m |
| 1680 | 57.4 | 44.1 |  | 1730 | 55.9 | 41.4 |  | 1780 | 44.0 | m |
| 1681 | 57.4 | 40.2 |  | 1731 | 55.5 | 44.2 |  | 1781 | 37.6 | m |
| 1682 | 57.6 | 36.9 |  | 1732 | 55.9 | 46.4 |  | 1782 | 47.2 | 0.0 |
| 1683 | 57.5 | 34.2 |  | 1733 | 55.8 | 48.3 |  | 1783 | 56.8 | m |
| 1684 | 57.4 | 31.1 |  | 1734 | 55.6 | 49.1 |  | 1784 | 47.5 | m |
| 1685 | 57.5 | 25.9 |  | 1735 | 55.8 | 49.3 |  | 1785 | 42.9 | m |
| 1686 | 57.5 | 20.7 |  | 1736 | 55.9 | 47.7 |  | 1786 | 31.6 | m |
| 1687 | 57.6 | 16.4 |  | 1737 | 55.9 | 47.4 |  | 1787 | 25.8 | m |
| 1688 | 57.6 | 12.4 |  | 1738 | 55.8 | 46.9 |  | 1788 | 19.9 | m |
| 1789 | 14.0 | m |  |  |  |  |  |  |  |  |
| 1790 | 8.1 | m |  |  |  |  |  |  |  |  |
| 1791 | 2.2 | m |  |  |  |  |  |  |  |  |
| 1792 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1793 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1794 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1795 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1796 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1797 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1798 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1799 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |
| 1800 | 0.0 | 0.0 |  |  |  |  |  |  |  |  |

m = motoring

Annex 2

DIESEL REFERENCE FUEL

| Parameter | Unit | Limits 1/ |  | Test method 5/ |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Minimum | Maximum |  |  |
| Cetene number |  | 45 | 55 | ISO 5165 |  |
| Density at 15 °C | kg/m3 | 835 | 845 | ISO 3675 |  |
| Distillation: |  |  |  |  |  |
| - 50 per cent vol. | °C | 245 | 295 | ISO 3405 |  |
| - final boiling point | C | 321 | 366 |  |  |
| Flash point | °C | 55 |  | ISO 2719 |  |
| Kinematic viscosity at 40 °C | mm2/s | 2.0 | 4.0 | ISO 3104 |  |
| Polycylic aromatic hydrocarbons | per cent m/m | 2.0 | 6.0 | EN 12916 |  |
| Water content | per cent m/m |  | 0.02 | EN-ISO 12937 |  |
| Sulphur content | mg/kg |  | 15 | EN-ISO 14596 |  |
| Total aromatics | per cent m/m | 10 | 25 | EN 12916 |  |
| Lubricity (HFRR at 60 °C) | µm |  | 400 | CEC F-06-A-96 |  |
| Oxidation stability @ 110°C 2/3/ | h | 20 |  | EN 14112 |  |
| FAME 4/ | per cent v/v | 4.5 | 5.5 | EN 14078 |  |

1/	The values quoted in the specification are "true values".  In establishment of their limit values the terms of ISO 4259 "Petroleum products - Determination and application of precision data in relation to methods of test.' have been applied and in fixing a minimum value, a minimum difference of 2R above zero has been taken into account; in fixing a maximum and minimum value, the minimum difference is 4R (R = reproducibility).

Notwithstanding this measure, which is necessary for statistical reasons, the manufacturer of fuels should nevertheless aim at a zero value where the stipulated maximum value is 2R and at the mean value in the case of quotations of maximum and minimum limits.  Should it be necessary to clarify the question as to whether a fuel meets the requirements of the specifications, the terms of ISO 4259 should be applied.

2/	Even though oxidation stability is controlled, it is likely that shelf life will be limited. Advice shall be sought from the supplier as to storage conditions and life

3/	Oxidation stability can be demonstrated by EN-ISO 12205 or by EN 14112. This requirement shall be revised based on CEN/TC19 evaluations of oxidative stability performance and test limits.

4/	FAME quality according EN 14214 (ASTM D 6751)

5/	The latest version of the respective test method applies

Annex 3

MEASUREMENT EQUIPMENT

A.3.1.	Analytical system

A.3.1.1.	Introduction

This annex contains the basic requirements and the general descriptions of the sampling and analyzing systems.  Since various configurations can produce equivalent results, exact conformance with figures 9 and 10 is not required.  However, conformance with the basic requirements such as sampling line dimensions, heating and design is mandatory.  Components such as instruments, valves, solenoids, pumps, flow devices and switches may be used to provide additional information and coordinate the functions of the component systems.  Other components which are not needed to maintain the accuracy on some systems, may be excluded if their exclusion is based upon good engineering judgement.

A.3.1.2.	Description of the analytical system

Analytical system for the determination of the gaseous emissions in the raw exhaust gas (figure 9) or in the diluted exhaust gas (figure 10) are described based on the use of:

(a)	HFID or FID analyzer for the measurement of hydrocarbons;

(b)	NDIR analyzers for the measurement of carbon monoxide and carbon dioxide;

(c)	HCLD or CLD analyzer for the measurement of the oxides of nitrogen.

The sample for all components should be taken with one sampling probe and internally split to the different analyzers.  Optionally, two sampling probes located in close proximity may be used.  Care must be taken that no unintended condensation of exhaust components (including water and sulphuric acid) occurs at any point of the analytical system.

a = vent	b = zero, span gas	c = exhaust pipe	d = optional

Figure 9

Schematic flow diagram of raw exhaust gas analysis system for CO, CO2, NOx, HC

a = vent		b = zero, span gas	c = dilution tunnel	d = optional

Figure 10

Schematic flow diagram of diluted exhaust gas analysis system for CO, CO2, NOx, HC

A.3.1.3.	Components of figures 9 and 10

EP	Exhaust pipe

SP	Raw exhaust gas sampling probe (figure 9 only)

A stainless steel straight closed end multi-hole probe is recommended.  The inside diameter shall not be greater than the inside diameter of the sampling line.  The wall thickness of the probe shall not be greater than 1 mm.  There shall be a minimum of 3 holes in 3 different radial planes sized to sample approximately the same flow.  The probe shall extend across at least 80 per cent of the diameter of the exhaust pipe.  One or two sampling probes may be used.

SP2	Dilute exhaust gas HC sampling probe (figure 10 only)

The probe shall:

(a)	be defined as the first 254 mm to 762 mm of the heated sampling line HSL1

(b)	have a 5 mm minimum inside diameter

(c)	be installed in the dilution tunnel DT (figure 15) at a point where the dilution air and exhaust gas are well mixed (i.e. approximately 10 tunnel diameters downstream of the point where the exhaust enters the dilution tunnel)

(d)	be sufficiently distant (radially) from other probes and the tunnel wall so as to be free from the influence of any wakes or eddies

(e)	be heated so as to increase the gas stream temperature to 463 K  10 K (190 °C  10 °C) at the exit of the probe, or to 385 K  10 K (112 °C  10 °C) for positive ignition engines

(f)	non-heated in case of FID measurement (cold)

SP3	Dilute exhaust gas CO, CO2, NOx sampling probe (figure 10 only)

The probe shall:

(a)	be in the same plane as SP2

(b)	be sufficiently distant (radially) from other probes and the tunnel wall so as to be free from the influence of any wakes or eddies

(c)	be heated and insulated over its entire length to a minimum temperature of 328 K (55 °C) to prevent water condensation

HF1	Heated pre-filter (optional)

The temperature shall be the same as HSL1.

HF2	Heated filter

The filter shall extract any solid particles from the gas sample prior to the analyzer.  The temperature shall be the same as HSL1.  The filter shall be changed as needed.

HSL1	Heated sampling line

The sampling line provides a gas sample from a single probe to the split point(s) and the HC analyzer.

The sampling line shall:

(a)	have a 4 mm minimum and a 13.5 mm maximum inside diameter

(b)	be made of stainless steel or PTFE

(c)	maintain a wall temperature of 463 K ± 10 K (190 °C ± 10 °C) as measured at every separately controlled heated section, if the temperature of the exhaust gas at the sampling probe is equal to or below 463 K (190 °C)

(d)	maintain a wall temperature greater than 453 K (180 °C), if the temperature of the exhaust gas at the sampling probe is above 463 K (190 °C)

(e)	maintain a gas temperature of 463 K ± 10 K (190 °C ± 10 °C) immediately before the heated filter HF2 and the HFID

HSL2	Heated NOx sampling line

The sampling line shall:

(a)	maintain a wall temperature of 328 K to 473 K (55 °C to 200 °C), up to the converter for dry measurement, and up to the analyzer for wet measurement

(b)	be made of stainless steel or PTFE

HP	Heated sampling pump

The pump shall be heated to the temperature of HSL.

SL	Sampling line for CO and CO2

The line shall be made of PTFE or stainless steel. It may be heated or unheated.

HC	HFID analyzer

Heated flame ionization detector (HFID) or flame ionization detector (FID) for the determination of the hydrocarbons.  The temperature of the HFID shall be kept at 453 K to 473 K (180 °C to 200 °C).

CO, CO2	NDIR analyzer

NDIR analyzers for the determination of carbon monoxide and carbon dioxide (optional for the determination of the dilution ratio for PT measurement).

NOx	CLD analyzer

CLD or HCLD analyzer for the determination of the oxides of nitrogen.  If a HCLD is used it shall be kept at a temperature of 328 K to 473 K (55 °C to 200 °C).

B	Cooling bath (optional for NO measurement)

To cool and condense water from the exhaust sample.  It is optional if the analyzer is free from water vapour interference as determined in paragraph 9.3.9.2.2.  If water is removed by condensation, the sample gas temperature or dew point shall be monitored either within the water trap or downstream.  The sample gas temperature or dew point shall not exceed 280 K (7 °C).  Chemical dryers are not allowed for removing water from the sample.

BK	Background bag (optional; figure 10 only)

For the measurement of the background concentrations.

BG	Sample bag (optional; figure 10 only)

For the measurement of the sample concentrations.

A.3.1.4.	Non-methane cutter method (NMC)

The cutter oxidizes all hydrocarbons except CH4 to CO2 and H2O, so that by passing the sample through the NMC only CH4 is detected by the HFID.  In addition to the usual HC sampling train (see figures 9 and 10), a second HC sampling train shall be installed equipped with a cutter as laid out in figure 11.  This allows simultaneous measurement of total HC and NMHC.

The cutter shall be characterized at or above 600 K (327°C) prior to test work with respect to its catalytic effect on CH4 and C2H6 at H2O values representative of exhaust stream conditions.  The dew point and O2 level of the sampled exhaust stream must be known.  The relative response of the FID to CH4 and C2H6 shall be determined in accordance with paragraph 9.3.8.

Figure 11

Schematic flow diagram of methane analysis with the NMC

A.3.1.5.	Components of figure 11

NMC	Non-methane cutter

To oxidize all hydrocarbons except methane

HC

Heated flame ionization detector (HFID) or flame ionization detector (FID) to measure the HC and CH4 concentrations.  The temperature of the HFID shall be kept at 453 K to 473 K (180 °C to 200 °C).

V1	Selector valve

To select zero and span gas

R	Pressure regulator

To control the pressure in the sampling line and the flow to the HFID

A.3.2.	Dilution and particulate sampling system

A.3.2.1.	Introduction

This annex contains the basic requirements and the general descriptions of the dilution and particulate sampling systems.  Since various configurations can produce equivalent results, exact conformance with figures 12 to 17 is not required.  However, conformance with the basic requirements such as sampling line dimensions, heating and design is mandatory.  Additional components such as instruments, valves, solenoids, pumps, and switches may be used to provide additional information and coordinate the functions of the component systems.  Other components which are not needed to maintain the accuracy on some systems, may be excluded if their exclusion is based upon good engineering judgment.

A.3.2.2.	Description of partial flow system

A dilution system is described based upon the dilution of a part of the exhaust stream. Splitting of the exhaust stream and the following dilution process may be done by different dilution system types.  For subsequent collection of the particulates, the entire dilute exhaust gas or only a portion of the dilute exhaust gas is passed to the particulate sampling system.  The first method is referred to as total sampling type, the second method as fractional sampling type.  The calculation of the dilution ratio depends upon the type of system used.

With the total sampling system as shown in figure 12, raw exhaust gas is transferred from the exhaust pipe (EP) to the dilution tunnel (DT) through the sampling probe (SP) and the transfer tube (TT).  The total flow through the tunnel is adjusted with the flow controller FC2 and the sampling pump (P) of the particulate sampling system (see figure 16).  The dilution airflow is controlled by the flow controller FC1, which may use qmew or qmaw and qmf as command signals, for the desired exhaust split.  The sample flow into DT is the difference of the total flow and the dilution airflow.  The dilution airflow rate is measured with the flow measurement device FM1, the total flow rate with the flow measurement device FM3 of the particulate sampling system (see figure 16).  The dilution ratio is calculated from these two flow rates.

a = exhaust		b = optional	c = details see Figure 16

Figure 12

Scheme of partial flow dilution system (total sampling type)

With the fractional sampling system as shown in figure 13, raw exhaust gas is transferred from the exhaust pipe EP to the dilution tunnel DT through the sampling probe SP and the transfer tube TT.  The total flow through the tunnel is adjusted with the flow controller FC1 connected either to the dilution airflow or to the suction blower for the total tunnel flow.  The flow controller FC1 may use qmew or qmaw and qmf as command signals for the desired exhaust split.  The sample flow into DT is the difference of the total flow and the dilution airflow.  The dilution airflow rate is measured with the flow measurement device FM1, the total flow rate with the flow measurement device FM2.  The dilution ratio is calculated from these two flow rates.  From DT, a particulate sample is taken with the particulate sampling system (see figure 16).

a = exhaust  b = to PB or SB  c = details see Figure 16  d = to particulate sampling system  e = vent

Figure 13

Scheme of partial flow dilution system (fractional sampling type)

A.3.2.3.	Components of figures 12 and 13

EP	Exhaust pipe

The exhaust pipe may be insulated.  To reduce the thermal inertia of the exhaust pipe a thickness to diameter ratio of 0.015 or less is recommended.  The use of flexible sections shall be limited to a length to diameter ratio of 12 or less.  Bends shall be minimized to reduce inertial deposition.  If the system includes a test bed silencer the silencer may also be insulated.  It is recommended to have a straight pipe of 6 pipe diameters upstream and 3 pipe diameters downstream of the tip of the probe.

SP	Sampling probe

The type of probe shall be either of the following

(a)	open tube facing upstream on the exhaust pipe centreline

(b)	open tube facing downstream on the exhaust pipe centreline

(c)	multiple hole probe as described under SP in paragraph A.3.1.3.

(d)	hatted probe facing upstream on the exhaust pipe centreline as shown in figure 14

The minimum inside diameter of the probe tip shall be 4 mm.  The minimum diameter ratio between exhaust pipe and probe shall be 4.

When using probe type (a), an inertial pre-classifier (cyclone or impactor) with at 50 per cent cut point between 2.5 and 10 µm shall be installed immediately upstream of the filter holder.

Figure 14

Scheme of hatted probe

TT	Exhaust transfer tube

The transfer tube shall be:

(a)	as short as possible, but not more than 0.26 m in length, if insulated for 80 per cent of the total length, as measured from the entrance into the dilution tunnel;

(b)	not more than 1 m in length, if heated above 150 °C for 90 per cent of the total length, as measured from the entrance into the dilution tunnel;

(c)	equal to or greater than the probe diameter, but not more than 25 mm in diameter;

(d)	exiting on the centreline of the dilution tunnel and pointing downstream.

With respect to (a), insulation shall be done with material with a maximum thermal conductivity of 0.05 W/mK with a radial insulation thickness corresponding to the diameter of the probe.

FC1	Flow controller

A flow controller shall be used to control the dilution airflow through the pressure blower PB and/or the suction blower SB.  It may be connected to the exhaust flow sensor signals specified in paragraph 8.3.1.  The flow controller may be installed upstream or downstream of the respective blower.  When using a pressurized air supply, FC1 directly controls the airflow.

FM1	Flow measurement device

Gas meter or other flow instrumentation to measure the dilution airflow.  FM1 is optional if the pressure blower PB is calibrated to measure the flow.

DAF	Dilution air filter

The dilution air (ambient air, synthetic air, or nitrogen) shall be filtered with a high-efficiency (HEPA) filter in line with ISO 14644 that has an initial minimum collection efficiency of 99.97 per cent.

FM2	Flow measurement device (fractional sampling type, figure 13 only)

Gas meter or other flow instrumentation to measure the diluted exhaust gas flow.  FM2 is optional if the suction blower SB is calibrated to measure the flow.

PB	Pressure blower (fractional sampling type, figure 13 only)

To control the dilution airflow rate, PB may be connected to the flow controllers FC1 or FC2.  PB is not required when using a butterfly valve.  PB may be used to measure the dilution airflow, if calibrated.

SB	Suction blower (fractional sampling type, figure 13 only)

SB may be used to measure the diluted exhaust gas flow, if calibrated.

DT	Dilution tunnel

The dilution tunnel:

(a)	shall be of a sufficient length to cause complete mixing of the exhaust and dilution air under turbulent flow conditions (Reynolds number, Re, greater than 4000, where Re is based on the inside diameter of the dilution tunnel) for a fractional sampling system, i.e. complete mixing is not required for a total sampling system;

(b)	shall be constructed of stainless steel;

(c)	may be heated to no greater than 325 K (52 °C) wall temperature;

(d)	may be insulated.

PSP	Particulate sampling probe (fractional sampling type, figure 13 only)

The particulate sampling probe is the leading section of the particulate transfer tube PTT (see paragraph A.3.2.7.) and:

(a)	shall be installed facing upstream at a point where the dilution air and exhaust gas are well mixed, i.e. on the dilution tunnel DT centreline approximately 10 tunnel diameters downstream of the point where the exhaust enters the dilution tunnel;

(b)	shall be 12 mm in minimum inside diameter;

(c)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the dilution air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust into the dilution tunnel;

(d)	may be insulated.

A.3.2.4.	Description of full flow dilution system

A dilution system is described based upon the dilution of the total amount of raw exhaust gas in the dilution tunnel DT using the CVS (constant volume sampling) concept, and is shown in figure 15.

The diluted exhaust gas flow rate shall be measured either with a positive displacement pump (PDP), with a critical flow venturi (CFV) or with a subsonic venturi (SSV).  A heat exchanger (HE) or electronic flow compensation (EFC) may be used for proportional particulate sampling and for flow determination.  Since particulate mass determination is based on the total diluted exhaust gas flow, it is not necessary to calculate the dilution ratio.

For subsequent collection of the particulates, a sample of the dilute exhaust gas shall be passed to the double dilution particulate sampling system (see figure 17).  Although partly a dilution system, the double dilution system is described as a modification of a particulate sampling system, since it shares most of the parts with a typical particulate sampling system.

a = analyzer system  b = background air  c = exhaust  d = details see Figure 17

e = to double dilution system  f = if EFC is used  i = vent  g = optional  h = or

Figure 15

Scheme of full flow dilution system (CVS)

A.3.2.5.	Components of figure 15

EP	Exhaust pipe

The exhaust pipe length from the exit of the engine exhaust manifold, turbocharger outlet or after-treatment device to the dilution tunnel shall be not more than 10 m.  If the system exceeds 4 m in length, then all tubing in excess of 4 m shall be insulated, except for an in-line smoke meter, if used.  The radial thickness of the insulation shall be at least 25 mm.  The thermal conductivity of the insulating material shall have a value no greater than 0.1 W/mK measured at 673 K.  To reduce the thermal inertia of the exhaust pipe a thickness-to-diameter ratio of 0.015 or less is recommended.  The use of flexible sections shall be limited to a length-to-diameter ratio of 12 or less.

PDP	Positive displacement pump

The PDP meters total diluted exhaust flow from the number of the pump revolutions and the pump displacement.  The exhaust system backpressure shall not be artificially lowered by the PDP or dilution air inlet system.  Static exhaust backpressure measured with the PDP system operating shall remain within  1.5 kPa of the static pressure measured without connection to the PDP at identical engine speed and load.  The gas mixture temperature immediately ahead of the PDP shall be within  6 K of the average operating temperature observed during the test, when no flow compensation (EFC) is used.  Flow compensation is only permitted, if the temperature at the inlet to the PDP does not exceed 323 K (50 °C).

CFV	Critical flow venturi

CFV measures total diluted exhaust flow by maintaining the flow at chocked conditions (critical flow).  Static exhaust backpressure measured with the CFV system operating shall remain within  1.5 kPa of the static pressure measured without connection to the CFV at identical engine speed and load.  The gas mixture temperature immediately ahead of the CFV shall be within  11 K of the average operating temperature observed during the test, when no flow compensation (EFC) is used.

SSV	Subsonic venturi

SSV measures total diluted exhaust flow by using the gas flow function of a subsonic venturi in dependence of inlet pressure and temperature and pressure drop between venturi inlet and throat.  Static exhaust backpressure measured with the SSV system operating shall remain within  1.5 kPa of the static pressure measured without connection to the SSV at identical engine speed and load.  The gas mixture temperature immediately ahead of the SSV shall be within  11 K of the average operating temperature observed during the test, when no flow compensation (EFC) is used.

HE	Heat exchanger (optional)

The heat exchanger shall be of sufficient capacity to maintain the temperature within the limits required above.  If EFC is used, the heat exchanger is not required.

EFC	Electronic flow compensation (optional)

If the temperature at the inlet to the PDP, CFV or SSV is not kept within the limits stated above, a flow compensation system is required for continuous measurement of the flow rate and control of the proportional sampling into the double dilution system.  For that purpose, the continuously measured flow rate signals are used to maintain the proportionality of the sample flow rate through the particulate filters of the double dilution system (see figure 17) within  2.5 per cent.

DT	Dilution tunnel

The dilution tunnel

(a)	shall be small enough in diameter to cause turbulent flow (Reynolds number, Re, greater than 4000, where Re is based on the inside diameter of the dilution tunnel) and of sufficient length to cause complete mixing of the exhaust and dilution air;

(b)	may be insulated.

The engine exhaust shall be directed downstream at the point where it is introduced into the dilution tunnel, and thoroughly mixed.  A mixing orifice may be used.

For the double dilution system, a sample from the dilution tunnel is transferred to the secondary dilution tunnel where it is further diluted, and then passed through the sampling filters (figure 17).  The flow capacity of the PDP or CFV must be sufficient to maintain the diluted exhaust stream in the DT at a temperature of less than or equal to 464 K (191 °C) at the sampling zone.  The secondary dilution system shall provide sufficient secondary dilution air to maintain the doubly diluted exhaust stream at a temperature between 315 K (42 °C) and 325 K (52 °C) immediately before the particulate filter.

DAF	Dilution air filter

The dilution air (ambient air, synthetic air, or nitrogen) shall be filtered with a high-efficiency (HEPA) filter in line with ISO 14644 that has an initial minimum collection efficiency of 99.97 per cent.

PSP	Particulate sampling probe

The probe is the leading section of PTT and

(a)	shall be installed facing upstream at a point where the dilution air and exhaust gases are well mixed, i.e. on the dilution tunnel DT centreline of the dilution systems, approximately 10 tunnel diameters downstream of the point where the exhaust enters the dilution tunnel;

(b)	shall be of 12 mm minimum inside diameter;

(c)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre-heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust in the dilution tunnel;

(d)	may be insulated.

A.3.2.6.	Description of particulate sampling system

The particulate sampling system is required for collecting the particulates on the particulate filter and is shown in figures 16 and 17.  In the case of total sampling partial flow dilution, which consists of passing the entire diluted exhaust sample through the filters, the dilution and sampling systems usually form an integral unit (see figure 12).  In the case of fractional sampling partial flow dilution or full flow dilution, which consists of passing through the filters only a portion of the diluted exhaust, the dilution and sampling systems usually form different units.

For a partial flow dilution system, a sample of the diluted exhaust gas is taken from the dilution tunnel DT through the particulate sampling probe PSP and the particulate transfer tube PTT by means of the sampling pump P, as shown in figure 16.  The sample is passed through the filter holder(s) FH that contain the particulate sampling filters.  The sample flow rate is controlled by the flow controller FC3.

For of full flow dilution system, a double dilution particulate sampling system shall be used, as shown in figure 17.  A sample of the diluted exhaust gas is transferred from the dilution tunnel DT through the particulate sampling probe PSP and the particulate transfer tube PTT to the secondary dilution tunnel SDT, where it is diluted once more.  The sample is then passed through the filter holder(s) FH that contain the particulate sampling filters.  The dilution airflow rate is usually constant whereas the sample flow rate is controlled by the flow controller FC3.  If electronic flow compensation EFC (see figure 15) is used, the total diluted exhaust gas flow is used as command signal for FC3.

a = from dilution tunnel

#### Figure 16

Scheme of particulate sampling system

a =  diluted exhaust from DT  b = optional  c = vent  d = secondary dilution air

Figure 17

Scheme of double dilution particulate sampling system

A.3.2.7.	Components of figures 16 (partial flow system only) and 17 (full flow system only)

PTT	Particulate transfer tube

The particulate transfer tube shall not exceed 1,020 mm in length, and shall be minimized in length whenever possible.

The dimensions are valid for:

(a)	the partial flow dilution fractional sampling system from the probe tip to the filter holder;

(b)	the partial flow dilution total sampling system from the end of the dilution tunnel to the filter holder;

(c)	the full flow double dilution system from the probe tip to the entrance of the secondary dilution tunnel.

The transfer tube:

(a)	may be heated to no greater than 325 K (52 °C) wall temperature;

(b)	may be insulated.

SDT	Secondary dilution tunnel (figure 17 only)

The secondary dilution tunnel should have a minimum diameter of 75 mm, and should be of sufficient length so as to provide a residence time of at least 0.25 s for the doubly diluted sample.  The filter holder FH shall be located within 300 mm of the exit of the SDT.

The secondary dilution tunnel:

(a)	may be heated to no greater than 325 K (52 °C) wall temperature by direct heating or by dilution air pre heating, provided the air temperature does not exceed 325 K (52 °C) prior to the introduction of the exhaust into the dilution tunnel;

(b)	may be insulated.

FH	Filter holder

The filter holder:

(a)	shall have a 12.5° (from center) divergent cone angle to transition from the transfer line diameter to the exposed diameter of the filter face;

(b)	may be heated to no greater than 325 K (52 °C) wall temperature;

(c)	may be insulated.

An inertial pre-classifier with a 50 per cent cut point between 2.5 µm and 10 µm shall be installed immediately upstream of the filter holder, if an open tube sampling probe facing upstream is used.

P	Sampling pump

FC2	Flow controller

A flow controller shall be used for controlling the particulate sample flow rate.

FM3	Flow measurement device

Gas meter or flow instrumentation to determine the particulate sample flow through the particulate filter.  It may be installed upstream or downstream of the sampling pump P.

FM4	Flow measurement device

Gas meter or flow instrumentation to determine the secondary dilution airflow through the particulate filter.

BV	Ball valve (optional)

The ball valve shall have an inside diameter not less than the inside diameter of the particulate transfer tube PTT, and a switching time of less than 0.5 s.

Annex 4

DETERMINATION OF SYSTEM EQUIVALENCE

The determination of system equivalency according to paragraph 5.1.1. shall be based on a 7 sample pair (or larger) correlation study between the candidate system and one of the accepted reference systems of this gtr using the appropriate test cycle(s).  The equivalency criteria to be applied shall be the F-test and the two-sided Student t-test.

This statistical method examines the hypothesis that the sample standard deviation and sample mean value for an emission measured with the candidate system do not differ from the sample standard deviation and sample mean value for that emission measured with the reference system.  The hypothesis shall be tested on the basis of a 10 per cent significance level of the F and t values.  The critical F and t values for 7 to 10 sample pairs are given in table 8.  If the F and t values calculated according to the equation below are greater than the critical F and t values, the candidate system is not equivalent.

The following procedure shall be followed.  The subscripts R and C refer to the reference and candidate system, respectively:

(a)	Conduct at least 7 tests with the candidate and reference systems operated in parallel.  The number of tests is referred to as nR and nC.

(b)	Calculate the mean values  and  and the standard deviations sR and sC.

(c)	Calculate the F value, as follows:

(86)

(the greater of the two standard deviations sR or sC must be in the numerator)

(d)	Calculate the t value, as follows:

(87)

(e)	Compare the calculated F and t values with the critical F and t values corresponding to the respective number of tests indicated in table 8.  If larger sample sizes are selected, consult statistical tables for 10 per cent significance (90 per cent confidence) level.

(f)	Determine the degrees of freedom (df), as follows:

for the F-test:	df = nR –1 /  nC –1	(88)

for the t-test:	df = nC + nR –2	(89)

(g)	Determine the equivalency, as follows:

(i)	if F < Fcrit and t < tcrit, then the candidate system is equivalent to the reference system of this gtr

(ii)	if F  Fcrit or t  tcrit , then the candidate system is different from the reference system of this gtr

| Sample Size | F-test |  | t-test |  |
| --- | --- | --- | --- | --- |
|  | df | Fcrit | df | tcrit |
| 7 | 6/6 | 3.055 | 12 | 1.782 |
| 8 | 7/7 | 2.785 | 14 | 1.761 |
| 9 | 8/8 | 2.589 | 16 | 1.746 |
| 10 | 9/9 | 2.440 | 18 | 1.734 |

Table 8

t and F values for selected sample sizes

Annex 5

CARBON FLOW CHECK

### A.5.1.	Introduction

All but a tiny part of the carbon in the exhaust comes from the fuel, and all but a minimal part of this is manifest in the exhaust gas as CO2.  This is the basis for a system verification check based on CO2 measurements.

The flow of carbon into the exhaust measurement systems is determined from the fuel flow rate.  The flow of carbon at various sampling points in the emissions and particulate sampling systems is determined from the CO2 concentrations and gas flow rates at those points.

In this sense, the engine provides a known source of carbon flow, and observing the same carbon flow in the exhaust pipe and at the outlet of the partial flow PM sampling system verifies leak integrity and flow measurement accuracy.  This check has the advantage that the components are operating under actual engine test conditions of temperature and flow.

Figure 18 shows the sampling points at which the carbon flows shall be checked.  The specific equations for the carbon flows at each of the sample points are given below.

Figure 18

Measuring points for carbon flow check

A.5.2.	Carbon flow rate into the engine (location 1)

The carbon mass flow rate into the engine for a fuel CHO is given by:

(90)

where:

qmf	is the fuel mass flow rate, kg/s

A.5.3.	Carbon flow rate in the raw exhaust (location 2)

The carbon mass flow rate in the exhaust pipe of the engine shall be determined from the raw CO2 concentration and the exhaust gas mass flow rate:

(91)

where:

cCO2,r	is the wet CO2 concentration in the raw exhaust gas, per cent

cCO2,a	is the wet CO2 concentration in the ambient air, per cent

qmew	is the exhaust gas mass flow rate on wet basis, kg/s

Me	is the molar mass of exhaust gas, g/mol

If CO2 is measured on a dry basis it shall be converted to a wet basis according to paragraph 8.1.

A.5.4.	Carbon flow rate in the dilution system (location 3)

For the partial flow dilution system, the splitting ratio also needs to be taken into account.  The carbon flow rate shall be determined from the dilute CO2 concentration, the exhaust gas mass flow rate and the sample flow rate:

(92)

where:

cCO2,d	is the wet CO2 concentration in the dilute exhaust gas at the outlet of the dilution tunnel, per cent

cCO2,a	is the wet CO2 concentration in the ambient air, per cent

qmew	is the exhaust gas mass flow rate on wet basis, kg/s

qmp	is the sample flow of exhaust gas into partial flow dilution system, kg/s

Me	is the molar mass of exhaust gas, g/mol

If CO2 is measured on a dry basis, it shall be converted to wet basis according to paragraph 8.1.

A.5.5.	Calculation of the molar mass of the exhaust gas

The molar mass of the exhaust gas shall be calculated according to equation 30 (see paragraph 8.3.2.5.)

Alternatively, the following exhaust gas molar masses may be used:

Me (diesel)	=	28.9 g/mol

Me (LPG)	=	28.6 g/mol

Me (NG)	=	28.3 g/mol

Annex 6

EXAMPLE OF CALCULATION PROCEDURE

A.6.1.	Speed and torque denormalization procedure

As an example, the following test point shall be denormalized:

per cent speed	 =	 43 per cent

per cent torque 	=	 82 per cent

Given the following values:

nlo	=	 1,015 min-1

nhi	=	 2,200 min-1

npref	=	 1,300 min-1

nidle	=	   600 min-1

results in:

actual speed	=

= 1,178 min-1

With the maximum torque of 700 Nm observed from the mapping curve at 1,178 min-1

actual torque 	=	  	=	 574 Nm

A.6.2.	Basic data for stoichiometric calculations

Atomic mass of hydrogen	1.00794 g/atom

Atomic mass of carbon	12.011 g/atom

Atomic mass of sulphur	32.065 g/atom

Atomic mass of nitrogen	14.0067 g/atom

Atomic mass of oxygen	15.9994 g/atom

Atomic mass of argon	39.9 g/atom

Molar mass of water	18.01534 g/mol

Molar mass of carbon dioxide	44.01 g/mol

Molar mass of carbon monoxide	28.011 g/mol

Molar mass of oxygen	31.9988 g/mol

Molar mass of nitrogen	28.011 g/mol

Molar mass of nitric oxide	30.008 g/mol

Molar mass of nitrogen dioxide	46.01 g/mol

Molar mass of sulphur dioxide	64.066 g/mol

Molar mass of dry air	28.965 g/mol

Assuming no compressibility effects, all gases involved in the engine intake/combustion/exhaust process can be considered to be ideal and any volumetric calculations shall therefore be based on a molar volume of 22.414 l/mol according to Avogadro’s hypothesis.

A.6.3.	Gaseous emissions (diesel fuel)

The measurement data of an individual point of the test cycle (data sampling rate of 1 Hz) for the calculation of the instantaneous mass emission are shown below.  In this example, CO and NOx are measured on a dry basis, HC on a wet basis.  The HC concentration is given in propane equivalent (C3) and has to be multiplied by 3 to result in the C1 equivalent.  The calculation procedure is identical for the other points of the cycle.

The calculation example shows the rounded intermediate results of the different steps for better illustration.  It should be noted that for actual calculation, rounding of intermediate results is not permitted (see paragraph 8.).

| Ta,i (K) | Ha,i (g/kg) | Wact kWh | qmew,i (kg/s) | qmaw,i (kg/s) | qmf,i (kg/s) | cHC,i (ppm) | cCO,i (ppm) | cNOx,i (ppm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 295 | 8.0 | 40 | 0.155 | 0.150 | 0.005 | 10 | 40 | 500 |

The following fuel composition is considered:

| Component | Molar ratio | per cent mass |
| --- | --- | --- |
| H |  = 1.8529 | wALF = 13.45 |
| C |  = 1.0000 | wBET = 86.50 |
| S |  = 0.0002 | wGAM = 0.050 |
| N |  = 0.0000 | wDEL = 0.000 |
| O |  = 0.0000 | wEPS = 0.000 |

Step 1: Dry/wet correction (paragraph 8.1.):

Equation (11):	kf = 0.055584 x 13.45 - 0.0001083 x 86.5 - 0.0001562 x 0.05 = 0.7382

Equation (8):	kw,a=  = 0.9331

Equation (7):	cCO,i (wet) 	=	40 x 0.9331	= 37.3 ppm

cNOx,i (wet)	=	500 x 0.9331	= 466.6 ppm

Step 2: NOx correction for temperature and humidity (paragraph 8.2.1.):

Equation (18):				= 0.9576

Step 3: Calculation of the instantaneous emission of each individual point of the cycle (paragraph 8.3.2.4.):

Equation (25):	mHC,i	=	10 x 3 x 0.155	= 4.650

mCO,i	=	37.3 x 0.155	= 5.782

mNox,I	=	466.6 x 0.9576 x 0.155	= 69.26

Step 4: Calculation of the mass emission over the cycle by integration of the instantaneous emission values and the u values from table 4 (paragraph 8.3.2.4.):

The following calculation is assumed for the WHTC cycle (1,800 s) and the same emission in each point of the cycle.

Equation (25):	mHC	=	0.000479 x	= 4.01 g/test

mCO	=	0.000966 x	= 10.05 g/test

mNOx	=	0.001586 x	= 197.72 g/test

Step 5: Calculation of the specific emissions (paragraph 8.5.2.1.):

Equation (56):	eHC	=	4.01 / 40	= 0.10 g/kWh

eCO	=	10.05 / 40	= 0.25 g/kWh

eNOx	=	197.72 / 40	= 4.94 g/kWh

A.6.4.	Particulate Emission (diesel fuel)

| pb (kPa) | Wact (kWh) | qmew,i (kg/s) | qmf,i (kg/s) | qmdw,i (kg/s) | qmdew,i (kg/s) | muncor (mg) | msep (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 99 | 40 | 0.155 | 0.005 | 0.0015 | 0.0020 | 1.7000 | 1.515 |

Step 1: Calculation of medf (paragraph 8.3.3.5.2.):

Equation (37):	rd,i	=		= 4

Equation (36):	qmedf,i	=	0.155 x 4	= 0.620 kg/s

Equation (35):	medf 	=		= 1,116 kg/test

Step 2: Buoyancy correction of the particulate mass (paragraph 9.4.3.5.)

Equation (72):	ρa	=		= 1.164 kg/m3

Equation (71):	mf	=		= 1.7006 mg

Step 3: Calculation of the particulate mass emission (paragraph 8.3.3.5.2.):

Equation (34):	mPM	=		= 1.253 g/test

Step 4: Calculation of the specific emission (paragraph 8.5.2.1.):

Equation (56):	ePM	=	1.253 / 40	= 0.031 g/kWh

Annex 7

INSTALLATION OF AUXILIARIES FOR EMISSIONS TEST

| Number | Auxiliaries | Fitted for emission test |
| --- | --- | --- |
| 1 | Inlet system |  |
|  | Inlet manifold | Yes |
|  | Crankcase emission control system | Yes |
|  | Control devices for dual induction inlet manifold system | Yes |
|  | Air flow meter | Yes |
|  | Air inlet duct work | Yes, or test cell equipment |
|  | Air filter | Yes, or test cell equipment |
|  | Inlet silencer | Yes, or test cell equipment |
|  | Speed-limiting device | Yes |
| 2 | Induction-heating device of inlet manifold | Yes, if possible to be set in the most favourable condition |
| 3 | Exhaust system |  |
|  | Exhaust manifold | Yes |
|  | Connecting pipes | Yes |
|  | Silencer | Yes |
|  | Tail pipe | Yes |
|  | Exhaust brake | No, or fully open |
|  | Pressure charging device | Yes |
| 4 | Fuel supply pump | Yes |
| 5 | Equipment for gas engines |  |
|  | Electronic control system, air flow meter, etc. | Yes |
|  | Pressure reducer | Yes |
|  | Evaporator | Yes |
|  | Mixer | Yes |
| 6 | Fuel injection equipment |  |
|  | Prefilter | Yes |
|  | Filter | Yes |
|  | Pump | Yes |
|  | High-pressure pipe | Yes |
|  | Injector | Yes |
|  | Air inlet valve | Yes |
|  | Electronic control system, sensors, etc. | Yes |
|  | Governor/control system | Yes |
|  | Automatic full-load stop for the control rack depending on atmospheric conditions | Yes |
| 7 | Liquid-cooling equipment |  |
|  | Radiator | No |
|  | Fan | No |
|  | Fan cowl | No |
|  | Water pump | Yes |
|  | Thermostat | Yes, may be fixed fully open |
| 8 | Air cooling |  |
|  | Cowl | No |
|  | Fan or Blower | No |
|  | Temperature-regulating device | No |
| 9 | Electrical equipment |  |
|  | Generator | No |
|  | Coil or coils | Yes |
|  | Wiring | Yes |
|  | Electronic control system | Yes |
| 10 | Intake air charging equipment |  |
|  | Compressor driven either directly by the engine and/or by the exhaust gases | Yes |
|  | Charge air cooler | Yes, or test cell system |
|  | Coolant pump or fan (engine-driven) | No |
|  | Coolant flow control device | Yes |
| 11 | Anti-pollution device (exhaust after-treatment system) | Yes |
| 12 | Starting equipment | Yes, or test cell system |
| 13 | Lubricating oil pump | Yes |

- - - - -