| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRRF/2009/28 3 July 2009 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Sixty-sixth session

Geneva, 15-17 September 2009

Item 3(c) of the provisional agenda

REGULATIONS Nos. 13 AND 13-H

(Braking)

Trailer braking

Proposal for amendments to Regulation No. 13

#### Submitted by the expert from Germany /

The text reproduced below was prepared by the expert from Germany to allow O3 trailers to be type-approved using successful Type-III test results.  The modifications to the existing text of the Regulation are marked in bold and strikethrough characters.

A.	PROPOSAL

Annex 4, items 3.1.2.4. and 3.1.2.5., amend to read:

"3.1.2.4.	In addition, the vehicles shall undergo the Type-I test or alternatively a Type-III test in the case of an O3 trailer.

3.1.2.5.	In the Type-I or the Type-III test of a semi-trailer, the mass braked by the latter's axle(s) must correspond to the maximum axle load(s) (not including the king pin load)."

Annex 11, Appendix 3, item 2.3.2., amend to read:

"2.3.2.	In the case of vehicles of categories O3 and O4…"

B	JUSTIFICATION

The proposal provides a simplified procedure for the type approval for O3 trailer fitted with an axle type approved according to Type III tests.

Currently, if the same axle is used in both O3 and O4 trailers (depending on the number of axles used), it will be tested to both the Type-I (O3 trailer) and the Type-III (O4 trailer) requirements. The Type-III test is the more stringent test and a braking system meeting this requirement will also meet the Type-I test requirement. Therefore, it is proposed to allow that Type-III test results be also used for the type approval of a O3 trailer.

The use of O4 test results for O3 trailers is already used in Regulation No. 13 (Annex 19, paragraph 5.1.2.).

- - - - -

/	In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles.  The present document is submitted in conformity with that mandate.