UN/SCEGHS/19/INF.13

Committee of Experts on the Transport of Dangerous Goods and on the Globally Harmonized System of Classification and Labelling of Chemicals Sub-Committee of Experts on the Globally Harmonized 7 June 2010 System of Classification and Labelling of Chemicals Nineteenth session Geneva, 30 June–2 July 2010 Item 2 (d) of the provisional agenda Updating of the third revised edition of the Globally Harmonized System of Classification and Labelling of Chemicals (GHS): Annexes

Revision of Annexes 1, 2 and 3 of the GHS – Precautionary statements

Transmitted by the expert from the United Kingdom on behalf of the informal correspondence group

A.	Introduction

1.	At its sixteenth session (December 2008), the Sub-Committee approved the continuing work plan for the informal correspondence group on the improvement of Annexes 1, 2, and 3 of the GHS (see document ST/SG/AC.10/C.4/2008/CRP.3/Add.5).  This stated that the group would:

“Pursue work to further improve Annexes 1, 2 and 3 of the GHS. The correspondence group will work on the following work streams, with the first given priority:

Workstream 1: to develop proposals to rationalize and improve the usability of hazard and precautionary statements, including proposals to eliminate redundancies among these statements

Workstream 2: to improve the presentation of Annexes 1 to 3 of the GHS, taking into account the intended audiences, uses and purposes of the GHS.”

2.	In accordance with Workstream 1 of this work programme, this paper presents a set of preliminary proposals for rationalising the precautionary statements given in Annex 3 of the GHS.

B.	Background – GHS precautionary statements

3.	Presently there are a number of factors limiting the usability of the GHS precautionary statements:

(a)	A high number of precautionary statements may be triggered for a substance or mixture, even with a relatively simple classification.  Including all the statements triggered for a given chemical would arguably confuse the user and reduce the effectiveness of hazard and safety communication;

(b)	Selection of appropriate precautionary statements by manufacturers and suppliers can be difficult when a large number are triggered;

(c)	Different hazard classifications trigger similar but different precautionary statements, leading to arguably unnecessary small differences between precautionary measures recommended in different circumstances.

4.	To illustrate these problems, three examples of substances are given in Annex II to this paper, together with the list of precautionary statements that are triggered. (The classifications are based on those given in the European Union harmonised classification list given in Part 3.1 of Annex VI of EC Regulation 1272/2008). It can be seen that a large number of PS are triggered in each case, to the extent that hazard communication is likely to be obscured if all are included on a chemical label.

5.	For these reasons, the informal correspondence group has been considering a number of approaches to rationalising and improving the usability of the GHS precautionary statements. These approaches have been outlined in more detail in previous INF papers (UN/SCEGHS/15/INF.26, UN/SCEGHS/16/INF.24, UN/SCEGHS/18/INF.18).  However, they include:

(a)	Editorial amendments to rationalise/reduce/shorten the existing precautionary statements;

(b)	Development of new combinations of precautionary statements;

(c)	Re-grouping of precautionary statements;

(d)	Provision of further guidance for selection of precautionary statements, in particular using the “conditions for use” column in the tables in Annex 3 part 3 of the GHS

(e)	Forming an “order of preference” for precautionary statements, allowing some to be prioritised over others;

(f)	Use of “signpost statements” which direct the user of a chemical to alternative places where safety information can be given, such as a safety data sheet or instructions for use.

C.	Progress on rationalisation of precautionary statements since the eighteenth session of the Sub-Committee

6.	Since the eighteenth session the informal correspondence group has held three telephone conferences, and has provisionally agreed a set of amendments to help rationalise and in some cases clarify the existing precautionary statements. The proposed amendments mostly fall within approach (a) above (editorial amendments/rationalisations), though in some cases they also draw on the other approaches.

7.	The proposals are outlined and explained in Annex I to this paper.  In Annex III, the changes required by the proposals to the tables of precautionary statements given in Annex 3 of the GHS are shown.  By way of illustration, Annex II also contains three examples of substances which trigger a relatively large number of precautionary statements, and shows the effect of the proposals for rationalisation on these statements.

8.	It can be seen that although the proposals made in the paper do help to reduce the number/length of statements, their overall impact in these examples is relatively small (and is likely to be small in other cases too).  Therefore it is likely that more work on approaches (b) – (f) (outlined in para. 5) will be required in order to find a full solution to the issues identified above with the precautionary statements.  Work on these further approaches will therefore be continued by the group as soon as an agreed proposal is reached on the proposals presented here.

D.	Progress on other work within the group

P410

9.	At the eighteenth session of the GHS Sub-Committee, the Responsible Packaging Management Association of southern Africa (RPMASA) presented the informal document UN/SCEGHS/18/INF.9, proposing to amend the GHS so that the precautionary statement P410 “Protect from sunlight” is no longer triggered for the hazard class, Gases under Pressure.  Agreement was not reached on the proposal at this session, but it was agreed that the issue would be further discussed within the Annexes 1-3 correspondence group.

10.	This issue has since been discussed at two teleconferences within the group, and RPMASA will present a revised version of their proposal in a separate INF paper.

Physical hazard precautionary statements

11.	The informal correspondence group has also agreed to consider a set of proposals from CEFIC members to make some revisions to clarify and rationalise the conditions of application for the precautionary statements for physical hazards.  A number of further proposals have also been received from EU physical hazard experts for amendment to the physical hazard statements.

12.	A consolidated document is currently being produced which combines the various amendments which have been proposed, for circulation to the informal correspondence group and to the Sub-Committee.  However, the proposals in their totality have not yet been considered by the correspondence group. The Sub-Committee will be updated as soon as there is further progress within this workstream.

Workstream 2 – improving the presentation of Annexes 1-3

13.	It was agreed at the December 2008 meeting of the Sub-Committee that the correspondence group should also take forward work on Workstream 2 of the group’s work plan, to improve the presentation of Annexes 1 to 3 of the GHS, though Workstream 1 (rationalisation of precautionary statements) should be given priority.  This has not yet been discussed within the group, however it will be taken forward as soon as satisfactory progress has been made within Workstream 1.

E.	Action requested of the Subcommittee and next steps

14.	The Sub-Committee’s preliminary views are sought on the proposals to amend the GHS precautionary statements outlined in Annexes I and III of this INF document.  Subject to the feedback received from the Sub-Committee, the correspondence group will then aim to present the changes in a working document for formal agreement at the December 2010 Sub-Committee meeting.

F.	Summary of annexes

Annex I (page 5):

Summary of proposed changes to the precautionary statements with associated explanations

Annex II (page 15)

Examples of substances and their associated precautionary statements, together with the amendments resulting from the proposals in this document.

Annex III (page 27)

Marked up version of all changes to Annex 3 of GHS as a result of proposed amendments to the precautionary statements.

Annex I

List of proposed amendments to precautionary statements with explanations

1.	P223 Keep away from any possible contact with water, because of violent reaction and possible flash fire.

Current situation:

The precautionary statement above applies to substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) – categories 1 and 2.

Proposed change:

Delete text “because of violent reaction and possible flash fire” so that precautionary statement reads simply, “Keep away from any possible contact with water”.

Justification:

Explanation of a required action is not normally given in precautionary statements.  Text after comma is redundant because the hazard statement for this class gives the required information. For category 1 the hazard statement is: “In contact with water releases flammable gases, which may ignite spontaneously”, and for category 2 the hazard statement is “In contact with water releases flammable gas”.

2.	P244 Keep reduction valves free from grease and oil

Current situation:

The above PS applies to oxidizing gases (chapter 2.4) category 1.

Proposed change:

Rephrase as “Keep valves and fittings free from oil and grease”.

Justification:

This precaution should not be restricted to the equipment connected to the pressure receptacles, but should be more general.  The proposed text is also currently widely in use on a voluntary basis.

3.	P260 Do not breathe dust/fume/gas/mist/vapours spray
P261 Avoid breathing dust/fume/gas/mist/vapours/spray

Current situation

P260 applies to acute toxicity, categories 1 and 2, STOT single and repeat exposure categories 1 and 2, skin corrosion categories 1A, 1B, 1C and reproductive toxicity, effects on or via lactation.

P261 applies to acute toxicity, categories 3 and 4, respiratory sensitization categories 1, 1A and 1B, skin sensitization categories 1, 1A and 1B and STOT single exposure category 3 (narcotic effects/respiratory tract irritation).

Proposed change

Include a note in the conditions for use for P261 stating that P261 need not be present when P260 is already on the label.

Justification

Clearly, P260 is a stronger precaution than P261 and renders P261 redundant.

4.	P280 Wear protective gloves/protective clothing/eye protection/face protection
P281 Use personal protective equipment as required

Current situation

P280 applies to various hazard categories within 15 different health and physical hazard classes.

P281 applies to carcinogenic, mutagenic and toxic to reproduction (CMRs) substances and mixtures and unstable explosives.

Proposal

Delete P281 and extend P280 to cover CMRs and unstable explosives, adding a note for “manufacturer/supplier or the competent authority to specify the type of equipment”.

Justification

P281 has the same intent as P280, but P280 gives the option for more precision as to the specific personal protective equipment (PPE) required.  Therefore, there seems no need to have two different statements covering the general need to use PPE – it is preferable to have the single statement P280 cover all cases.

5.	P284 Wear respiratory protection
P285 In case of inadequate ventilation wear respiratory protection

Current situation

P284 applies to acute toxicity, inhalation, categories 1 and 2.

P285 applies to respiratory sensitisers categories 1, 1A and 1B.

Proposed change

Consolidate into a single statement, which would apply to both acute toxicity categories and respiratory sensitisers.  This would read:

P284 [In case of inadequate ventilation] wear respiratory protection.

With the condition for use:

“- text in square brackets may be used where additional safety information regarding adequate ventilation is provided.”

Justification

The possibility of using chemicals without respiratory protection in conditions of good ventilation exists for both respiratory sensitisers and acutely toxic chemicals.  However, in the absence of further information users may find it difficult to judge what is “adequate ventilation”.  Therefore if the phrase “in case of inadequate ventilation” is used, additional information should be required with the chemical to explain what ventilation conditions would be adequate.

6.	P307 IF exposed:
P308 IF exposed or concerned:
P309 IF exposed or if you feel unwell:

Current situation

These occur only in combination with statements advising the user to seeking medical advice.

P307 + P311
IF exposed: Call a POISON CENTER or doctor/physician

assigned to specific target organ toxicity, single exposure, category 1.

P308 + P313
IF exposed or concerned: Get medical advice/attention

assigned to all CMR categories.

P309 + P311
IF exposed or if you feel unwell: Call a POISON CENTER or doctor/physician

assigned to specific target organ toxicity, single exposure, category 2.

Proposal

Delete P307 and P309 and replace them with P308, so that P308 is used in all three combined precautionary statements above.  They would now read:

P307 P308 + P311
IF exposed or concerned: Call a POISON CENTER or doctor/physician

P308 + P313
IF exposed or concerned: Get medical advice/attention

P309 P308 + P311
IF exposed or concerned if you feel unwell: Call a POISON CENTER or doctor/physician

Justification

P307, P308 and P309 are intended to advise seeking medical attention in a number of scenarios: exposure without symptoms or with delayed symptoms, symptoms without known exposure, or exposure with immediate symptoms.  However, all these eventualities are covered by P308 so rationalisation can be achieved without loss of protection by using P308 throughout.  This also has the advantage of simplifying this set of precautionary statements by using a single formulation instead of several slightly different ones.

7.	P310 Immediately call a POISON CENTER or doctor/physician
P311 Call a POISON CENTER or doctor/physician
P312 Call a POISON CENTER or doctor/physician if you feel unwell

Current situation

The statements above are used for numerous health hazard classes and categories in conjunction with a prefix, e.g. “IF INHALED:”, “IF SWALLOWED:”, “if exposed or concerned:” etc.  They are generally used to advise seeking advice where there will be acute effects (acute toxicity, skin corrosion, severe eye damage, STOT single exposure and other classes).

Proposal

Edit the statements to read “call a POISON CENTER/doctor/ ….”.  In full they would read:

P310 Immediately call a POISON CENTER or /doctor/physician/…

P311 Call a POISON CENTER or doctor/physician/….

P312 Call a POISON CENTER or doctor/physician/…. if you feel unwell

Add a note in conditions for use:

“Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice.”

Justification

The word “physician” is not in current usage in either US or British English so there seems no need to offer this as an option.  Countries have differing arrangements for delivery of first-line medical advice: in some countries members of the public should call Poison Centres, whereas in others (e.g. UK) this is not the case and Poison Centres only deal with medical professionals.  The proposed amendment retains the reference to Poison Centres but gives some flexibility for the appropriate advice to be given in each country.

8.	P321 Specific treatment (see … on this label)
P322 Specific measures (see … on this label)

Current situation

P321 is given for various categories within acute toxicity (oral and inhalation), skin corrosion, skin irritation and skin sensitization, and specific target organ toxicity.  The type of treatment is specified in conditions for use and may be a specific antidote (for acute toxicity oral 1, 2), a cleaning agent (for skin corrosion/irritation/sensitization categories) or other unspecified measures.

P322 is only assigned to acute toxicity, dermal categories 1, 2, 3 and 4, and is given the condition for use:

“...  Reference to supplemental first aid instruction.

- if immediate measures such as specific cleansing agent is advised.”

Proposal

Delete P322 and use P321 instead for the categories to which it applies.  Therefore, acute toxicity, dermal, categories 1, 2, 3 and 4 would all receive the statement:

P321 Specific treatment (see … on this label)

With the condition for use:

“...  Reference to supplemental first aid instruction.

- if immediate measures such as specific cleansing agent is advised.”

Justification

Rationalisation.  The GHS currently draws no clear distinction between ‘treatment’ and ‘measures’ – i.e. between cases where P321 and P322 applies.  E.g. while the conditions for use for P322 refer to ‘measures’ and use of cleansing agents, the conditions for use for P321 (specific treatment) also refer to both measures (e.g. for acute toxicity, inhalation category 3), and to cleansing agents (e.g. for skin corrosion 1A, 1B and 1C).  Therefore it would be simpler just to use the one formulation (specific treatment) to refer to all cases.

9.	P332 If skin irritation occurs:
P333 If skin irritation or rash occurs:

Current situation

The statements only occur in combination with references to seeking medical attention.

P332 + P313
If skin irritation occurs: Get medical advice/attention

applies to skin irritation categories 2 and 3.

P333 + P313 
If skin irritation or rash occurs: Get medical advice/attention

applies to skin sensitizers, 1, 1A and 1B.

Proposal

Add a condition for use to P332 + P313: “- may be omitted if P333 + P313 appears on the label”

Justification

Evident redundancy where both statements appear, P313 + P333 is the stronger of the two so should take precedence.

10.	P340 Remove victim to fresh air and keep at rest in a position comfortable for breathing.
P341 If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing.
P342 If experiencing respiratory symptoms:

Current situation

Respiratory sensitisers are assigned P304 + P341:

IF INHALED: If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing.

Chemicals classified as acutely toxic by inhalation, categories 1, 2, 3 and 4 are assigned P304 + P340:

IF INHALED: If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing.

Proposal

For respiratory sensitisers use P304 + P340 instead of P304 + P341.

Also, edit P340 to read: “Remove person to fresh air and ensure comfortable breathing”.

Respiratory sensitisers would now be assigned:

P304 + P341 P340

IF INHALED: If breathing is difficult, remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing.

Chemicals classified as acutely toxic by inhalation, categories 1, 2, 3 and 4 would be assigned:

P304 + P340
IF INHALED: remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing.

Justification

The double conditional in P304 + P341 “IF INHALED: If breathing is difficult…” is confusing and seems unnecessary.  It seems simpler to require “IF INHALED: remove person to fresh air…”.

The phrasing “remove victim [person] to fresh air and keep at rest in a position comfortable for breathing” is rather long and complex.  The same essential precaution could be conveyed using the shorter wording “remove victim [person] to fresh air and ensure comfortable breathing”.

“Victim” seems rather an emotive term, so the more neutral “person” is preferable.

11.	P350 Gently wash with plenty of soap and water
P352 Wash with plenty of soap and water

Current situation

P350 and P352 both occur only in combination with P302 “IF ON SKIN:”.

Chemicals classified as acutely toxic, dermal, categories 1 and 2 are assigned:

P302 + P350
IF ON SKIN: Gently wash with plenty of soap and water

Chemicals classified as acutely toxic, dermal category 3 and 4, skin irritant, category 2 and skin sensitisation 1, 1A and 1B, are assigned:

P302 + P352
IF ON SKIN: wash with plenty of soap and water

Proposal

Delete P350 and use P302 + P352 instead for chemicals classified as acutely toxic 1 and 2.

Amend P352 to read “wash with plenty of water/…”, and add the condition for use, “Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate”.

Chemicals classified as acutely toxic 1, 2, 3 and 4, skin irritant 2, and skin sensitizer 1, 1A and 1B, would therefore receive:

P302 + P352
IF ON SKIN: wash with plenty of soap and water/….

Justification

“Gently” seems inappropriate for more severe categories of dermal acute toxicity, quick action is paramount and this phrasing may discourage it.

The most important precaution for all the above hazard categories is to wash the skin in plenty of water as soon as possible, advising use of soap could cause unnecessary delay.

There should be flexibility to require a cleansing agent other than water, but only in exceptional cases where water is clearly not appropriate.

12.	P361 Remove/take off immediately all contaminated clothing
P362 Take off contaminated clothing and wash before reuse
P363 Wash contaminated clothing before reuse

Current situation

These statements are assigned in different combinations to a number of different hazard categories.  The current assignment is summarised in the tables below.  (The tables include changes proposed in this document to precautionary statements other than P361-P363.)

| Precautionary statements |  | Hazard class/category |  |
| --- | --- | --- | --- |
| P361 | Remove/take off immediately all contaminated clothing | Flammable liquids | 1, 2, 3 |
|  |  | Acute toxicity, dermal | 1, 2, 3 |
|  |  | Skin corrosion | 1A, 1B, 1C |
| P362 | Take off contaminated clothing and wash before reuse | Skin irritation | 2 |
| P363 | Wash contaminated clothing before reuse | Acute toxicity, dermal | 1, 2, 3 |
|  |  | Acute toxicity, dermal | 4 |
|  |  | Skin corrosion | 1A, 1B, 1C |
|  |  | Skin sensitization | 1, 1A, 1B |

| Hazard class/category | Precautionary statements |
| --- | --- |
| Flammable liquids 1, 2, 3 | P303 + P361 + P353 IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower |
| Acute toxicity, dermal 1, 2, 3 | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/…. P361 Remove/Take off immediately all contaminated clothing P363 Wash contaminated clothing before reuse |
| Acute toxicity, dermal 4 | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/… P363 Wash contaminated clothing before reuse |
| Skin corrosion 1A, 1B, 1C | P303 + P361 + P353 IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower P363 Wash contaminated clothing before reuse |
| Skin irritation 2 | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/… P362 Take off contaminated clothing and wash before re-use |
| Skin sensitization 1, 1A, 1B | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/… P363 Wash contaminated clothing before reuse |

Proposal

Include a statement advising removal of contaminated clothing for acute toxicity, dermal 4 and skin sensitization 1, 1A, 1B (currently they only require “wash contaminated clothing before reuse”).

Delete “and wash before reuse” from P362.  Introduce a further (partial) precautionary statement, “and wash it before reuse”, to enable combinations to be formed with P361 and P362, e.g. P361 + P364 “Remove/Take off immediately all contaminated clothing and wash it before reuse”

Delete “Remove” from “Remove/Take off…”

The revised assignment of precautionary statements to the hazard categories above would be:

| Precautionary statements |  | Hazard class/category |  |
| --- | --- | --- | --- |
| P361 | Remove/tTake off immediately all contaminated clothing | Flammable liquids | 1, 2, 3 |
|  |  | Acute toxicity, dermal | 1, 2, 3 |
|  |  | Skin corrosion | 1A, 1B, 1C |
| P362 | Take off contaminated clothing and wash before reuse. | Skin irritation | 2 |
|  |  | Acute toxicity, dermal | 4 |
|  |  | Skin sensitization | 1A, 1B, 2 |
| P363 | Wash contaminated clothing before reuse | Acute toxicity, dermal | 1, 2, 3 |
|  |  | Skin corrosion | 1A, 1B, 1C |
|  |  | Acute toxicity, dermal | 4 |
|  |  | Skin sensitization | 1A, 1B, 2 |
| P364 | and wash it before reuse | Acute toxicity, dermal | 1, 2, 3, 4 |
|  |  | Skin irritation | 2 |
|  |  | Skin sensitization | 1A, 1B |

| Hazard class/category | Precautionary statements |
| --- | --- |
| Flammable liquids 1, 2, 3 | P303 + P361 + P353 IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower |
| Acute toxicity, dermal 1, 2, 3 | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/…. P361 + P364 Remove/Take off immediately all contaminated clothing and wash it before reuse P363 Wash contaminated clothing before reuse. |
| Acute toxicity, dermal 4 | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/…. P363 P362 + P364 Take off contaminated clothing and wash it before reuse |
| Skin corrosion 1A, 1B, 1C | P303 + P361 + P353 IF ON SKIN (or hair): Remove/tTake off immediately all contaminated clothing. Rinse skin with water/shower P363 Wash contaminated clothing before reuse |
| Skin irritation 2 | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/…. P362 + P364 Take off contaminated clothing and wash before reuse and wash it before reuse |
| Skin sensitization 1, 1A, 1B | P302 + P352 IF ON SKIN: Wash with plenty of soap and water/…. P363 Wash contaminated clothing before reuse. P362 + P364 Take off contaminated clothing and wash it before reuse |

Justification

Acute toxicity, dermal category 4 and skin sensitization 1, 1A and 1B should also be assigned an instruction to remove contaminated clothing since this will occur within the scope of ‘IF ON SKIN:’ – removal of contaminated clothing is advisable if contaminant has also got into contact with skin.

Currently there is a mixture of precautionary statements advising single actions (e.g. “Remove/Take off immediately all contaminated clothing” and “Wash contaminated clothing before reuse”) and what are in effect combination statements (e.g. “Take off contaminated clothing and wash before reuse”).  Adding the partial statement ‘and wash it before reuse’ makes the situation simpler by allowing all the statements to give single instructions.  It also enables combination statements to be formed when appropriate, e.g. P361 + P364 “Remove/Take off immediately all contaminated clothing and wash it before reuse”.

P363 is preserved as a single statement since for Skin corrosion and flammable liquids it is not possible to combine it with P361 “Remove/Take off immediately all contaminated clothing”, as P361 is followed by P353, “Rinse skin with water/shower”.

Adding ‘it’ in ‘“Remove/Take off immediately all contaminated clothing and wash it before reuse” etc. makes clear that it is the clothing (not the skin) that needs to be washed after clothing is removed.

“Remove” means the same as “Take off”, so is redundant.

Annex II

Examples of substances with associated precautionary statements with effect of rationalisation proposals

Substances classifications are those listed in Table 3.1 of part 3 of Annex 3 of Regulation (EC) No 1272/2008 on classification, labelling and packaging of substances and mixtures.  The unofficial consolidated table available at http://ecb.jrc.ec.europa.eu/classification-labelling has been used.

Example 1: Hydrazine bis(3-carboxy-4-hydroxybenzensulfonate)

Classification

Carc 1B

Acute tox 4

Skin Corr 1B

Skin Sens 1

Aquatic Chronic 3

Hazard statements

| H350 | May cause cancer |
| --- | --- |
| H302 | Harmful if swallowed |
| H314 | Causes severe skin burns and eye damage |
| H317 | May cause an allergic skin reaction |
| H412 | Harmful to aquatic life with long lasting effects |

| Precautionary statements |  |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P260 | Do not breathe dusts or mists |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray |
| P264 | Wash… thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection |
| P281 | Use personal protective equipment as required |
| P301 + P312 | IF swallowed: call a POISON CENTER or doctor/physician if you feel unwell |
| P301 + P330 + P331 | IF swallowed: Rinse mouth. Do NOT induce vomiting |
| P302 + P352 | IF ON SKIN: Wash with plenty of soap and water |
| P303 + P361 + P353 | IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. |
| P304 + P340 | IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. |
| P310 | Immediately call a POISON CENTER or doctor/physician |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P308 + P313 | If exposed or concerned: Get medical advice/attention |
| P321 | Specific treatment (see… on this label) |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention |
| P363 | Wash contaminated clothing before reuse |
| P405 | Store locked up |
| P501 | Dispose of contents/container to… |

| Precautionary statements after rationalisation proposals applied |  |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P260 | Do not breathe dusts or mists |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray |
| P264 | Wash… thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection |
| P281 | Use personal protective equipment as required |
| P301 + P312 | IF SWALLOWED: call a POISON CENTER or/doctor/physician… if you feel unwell |
| P301 + P330 + P331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting |
| P302 + P352 | IF ON SKIN: Wash with plenty of soap and water/… |
| P303 + P361 + P353 | IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. |
| P304 + P340 | IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing |
| P310 | Immediately call a POISON CENTER or/doctor/physician…. |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P308 + P313 | If exposed or concerned: Get medical advice/attention |
| P321 | Specific treatment (see… on this label) |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention. |
| P362 + P364 | Take off contaminated clothing and wash it before re-use |
| P363 | Wash contaminated clothing before reuse |
| P405 | Store locked up |
| P501 | Dispose of contents/container to… |

| Precautionary statements after rationalisation proposals applied (clean) |  |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P260 | Do not breathe dusts or mists |
| P264 | Wash… thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection [comment: appropriate PPE would need to be specified to cover all hazards of substance] |
| P301 + P312 | IF SWALLOWED: call a POISON CENTER/doctor… if you feel unwell |
| P301 + P330 + P331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting |
| P302 + P352 | IF ON SKIN: Wash with plenty of water/… |
| P303 + P361 + P353 | IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower. |
| P304 + P340 | IF INHALED: Remove person to fresh air and ensure comfortable breathing |
| P310 | Immediately call a POISON CENTER doctor …. |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P308 + P313 | If exposed or concerned: Get medical advice/attention |
| P321 | Specific treatment (see… on this label) |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention. |
| P362 + P364 | Take off contaminated clothing and wash it before re-use |
| P405 | Store locked up |
| P501 | Dispose of contents/container to… |

Example 2: Hexyl acrylate (CAS no: 2499-95-8)

Classification

Eye irritancy 2

STOT SE 3

Skin irritancy 2

Skin sensitisation 1

Aquatic chronic 2

Hazard statements

| H319 | Causes serious eye irritation |
| --- | --- |
| H335 | May cause respiratory irritation |
| H315 | Causes skin irritation |
| H317 | May cause an allergic skin reaction |
| H411 | Toxic to aquatic life with long lasting effects |

| Precautionary statements |  |
| --- | --- |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray. |
| P264 | Wash … thoroughly after handling |
| P271 | Use only outdoors or in a well-ventilated area. |
| P272 | Contaminated work clothing should not be allowed out of the workplace. |
| P273 | Avoid release to the environment. |
| P280 | Wear eye protection/face protection. |
| P302 + P352 | IF ON SKIN: Wash with plenty of soap and water |
| P304 + P340 | IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P312 | Call a POISON CENTER or doctor/physician if you feel unwell. |
| P321 | Specific treatment (see ... on this label) |
| P332 + P313 | If skin irritation occurs: Get medical advice/attention. |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention. |
| P337 + P313 | If eye irritation persists: Get medical advice/attention |
| P362 | Take off contaminated clothing and wash before reuse |
| P363 | Wash contaminated clothing before reuse |
| P403 + P233 | Store in a well-ventilated place. Keep container tightly closed |
| P405 | Store locked up |
| P501 | Dispose of contents/container to... |

| Precautionary statements after rationalisation proposals applied |  |
| --- | --- |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray. |
| P264 | Wash … thoroughly after handling |
| P271 | Use only outdoors or in a well-ventilated area. |
| P272 | Contaminated work clothing should not be allowed out of the workplace. |
| P273 | Avoid release to the environment. |
| P280 | Wear eye protection/face protection. |
| P302 + P352 | IF ON SKIN: Wash with plenty of soap and water/… |
| P304 + P340 | IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P312 | Call a POISON CENTER or/doctor/physician… if you feel unwell. |
| P321 | Specific treatment (see ... on this label) |
| P332 + P313 | If skin irritation occurs: Get medical advice/attention. |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention. |
| P337 + P313 | If eye irritation persists: Get medical advice/attention |
| P362 | Take off contaminated clothing and wash before reuse |
| P363 | Wash contaminated clothing before reuse |
| P362 + P364 | Take off contaminated clothing and wash it before reuse |
| P403 + P233 | Store in a well-ventilated place. Keep container tightly closed |
| P405 | Store locked up |
| P501 | Dispose of contents/container to... |

| Precautionary statements after rationalisation proposals applied (clean) |  |
| --- | --- |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray. |
| P264 | Wash … thoroughly after handling |
| P271 | Use only outdoors or in a well-ventilated area. |
| P272 | Contaminated work clothing should not be allowed out of the workplace. |
| P273 | Avoid release to the environment. |
| P280 | Wear eye protection/face protection. |
| P302 + P352 | IF ON SKIN: Wash with plenty of water/… |
| P304 + P340 | IF INHALED: Remove person to fresh air and ensure comfortable breathing |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P312 | Call a POISON CENTER/doctor … if you feel unwell. |
| P321 | Specific treatment (see ... on this label) |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention. |
| P337 + P313 | If eye irritation persists: Get medical advice/attention |
| P362 + P364 | Take off contaminated clothing and wash it before reuse |
| P403 + P233 | Store in a well-ventilated place. Keep container tightly closed |
| P405 | Store locked up |
| P501 | Dispose of contents/container to... |

Example 3: 2,2-dibromo-2-nitroethanol (CAS no: 69094-18-4)

Classification

Explosive 1.1

Carcinogenic 2

Acute Tox. 4

STOT RE 2

Skin Corr. 1A

Skin Sens. 1

Aquatic Acute 1

Aquatic Chronic 1

Hazard statements

| H201 | Explosive; mass explosion hazard |
| --- | --- |
| H302 | Harmful if swallowed |
| H314 | Causes severe burns and eye damage |
| H317 | May cause an allergic skin reaction |
| H351 | Suspected of causing cancer |
| H373 | May cause damage to organs through prolonged or repeated exposure |
| H410 | Very toxic to aquatic life with long lasting effects |

| Precautionary statements |  |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P210 | Keep away from heat/sparks/open flames/hot surfaces. — No smoking. |
| P230 | Keep wetted with … |
| P240 | Ground/bond container and receiving equipment |
| P250 | Do not subject to grinding/shock/…/friction. |
| P260 | Do not breathe dust/fume/gas/mist/vapours/spray |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray |
| P264 | Wash … thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection |
| P281 | Use personal protective equipment as required |
| P301+P312 | IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell |
| P301+P330+P331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting |
| P302+P352 | IF ON SKIN: Wash with plenty of soap and water |
| P303+P361+P353 | IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower |
| P304+P340 | IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing |
| P310 | Immediately call a POISON CENTER or doctor/physician |
| P305+P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing |
| P308+P313 | IF exposed or concerned: Get medical advice/attention |
| P333+P313 | If skin irritation or rash occurs: Get medical advice/attention |
| P370+380 | In case of fire: Evacuate area |
| P314 | Get medical advice/attention if you feel unwell |
| P321 | Specific treatment (see … on this label) |
| P330 | Rinse mouth |
| P363 | Wash contaminated clothing before reuse |
| P372 | Explosion risk in case of fire |
| P373 | DO NOT fight fire when fire reaches explosives |
| P391 | Collect spillage |
| P401 | Store …in accordance with local/regional/national/international regulations (to be specified) |
| P405 | Store locked up |
| P501 | Dispose of contents/container to …. |

| Precautionary statements after rationalisation proposals applied |  |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P210 | Keep away from heat/sparks/open flames/hot surfaces. — No smoking. |
| P230 | Keep wetted with … |
| P240 | Ground/bond container and receiving equipment |
| P250 | Do not subject to grinding/shock/…/friction. |
| P260 | Do not breathe dust/fume/gas/mist/vapours/spray |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray |
| P264 | Wash … thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection |
| P281 | Use personal protective equipment as required |
| P301+P312 | IF SWALLOWED: Call a POISON CENTER or/doctor/physician… if you feel unwell |
| P301+P330+P331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting |
| P302+P352 | IF ON SKIN: Wash with plenty of soap and water/… |
| P303+P361+P353 | IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower |
| P304+P340 | IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing |
| P310 | Immediately a POISON CENTER or/doctor/physician… |
| P305+P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing |
| P308+P313 | IF exposed or concerned: Get medical advice/attention |
| P333+P313 | If skin irritation or rash occurs: Get medical advice/attention |
| P370+380 | In case of fire: Evacuate area |
| P314 | Get medical advice/attention if you feel unwell |
| P321 | Specific treatment (see … on this label) |
| P330 | Rinse mouth |
| P362 + P364 | Take off contaminated clothing and wash it before re-use |
| P363 | Wash contaminated clothing before reuse |
| P372 | Explosion risk in case of fire |
| P373 | DO NOT fight fire when fire reaches explosives |
| P391 | Collect spillage |
| P401 | Store …in accordance with local/regional/national/international regulations (to be specified) |
| P405 | Store locked up |
| P501 | Dispose of contents/container to …. |

| Precautionary statements after rationalisation proposals applied (clean) |  |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P210 | Keep away from heat/sparks/open flames/hot surfaces. — No smoking. |
| P230 | Keep wetted with … |
| P240 | Ground/bond container and receiving equipment |
| P250 | Do not subject to grinding/shock/…/friction. |
| P260 | Do not breathe dust/fume/gas/mist/vapours/spray |
| P264 | Wash … thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection |
| P301+P312 | IF SWALLOWED: Call a POISON CENTER/doctor … if you feel unwell |
| P301+P330+P331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting |
| P302+P352 | IF ON SKIN: Wash with plenty of water/… |
| P303+P361+P353 | IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower |
| P304+P340 | IF INHALED: Remove person to fresh air and ensure comfortable breathing |
| P310 | Immediately a POISON CENTER/doctor … |
| P305+P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing |
| P308+P313 | IF exposed or concerned: Get medical advice/attention |
| P333+P313 | If skin irritation or rash occurs: Get medical advice/attention |
| P370+380 | In case of fire: Evacuate area |
| P314 | Get medical advice/attention if you feel unwell |
| P321 | Specific treatment (see … on this label) |
| P330 | Rinse mouth |
| P362 + P364 | Take off contaminated clothing and wash it before re-use |
| P372 | Explosion risk in case of fire |
| P373 | DO NOT fight fire when fire reaches explosives |
| P391 | Collect spillage |
| P401 | Store …in accordance with local/regional/national/international regulations (to be specified) |
| P405 | Store locked up |
| P501 | Dispose of contents/container to …. |

Annex III

Changes to Annex 3 of GHS resulting from rationalisation proposals

(Note: inserted text is underlined, deleted text is indicated in strikethrough)

1.	Amend Annex 3, section 2, paragraph A3.2.3.7 to read:

“To facilitate translation into the languages of users, precautionary statements have been broken down into individual sentences or parts of sentences in the tables in this section.  In a number of instances the text that appears on a GHS label requires that these be added back together.  This is indicated in this annex by codes conjoined with a plus sign  “+”.  For example P305 + P351 + P338 indicates that the text to appear on the label is “IF IN EYES:  Rinse cautiously with water for several minutes.  Remove contact lenses, if present and easy to do.  Continue rinsing”. These additive precautionary statements can also be found at the end of each of the precautionary statement tables in this section.  Translation of only the single precautionary statements is required, as this will enable the compilation of the additive precautionary statements.”

2. 	Amend Tables 3.2.1, 3.2.2, 3.2.3, 3.2.4 and 3.2.5 to read as follows:

“Table A3.2.1:  Codification of general precautionary statements

| Code | General precautionary statements | Hazard class | Hazard category | Conditions for use |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| P101 | If medical advice is needed, have product container or label at hand. | as appropriate |  | Consumer products |
| P102 | Keep out of reach of children. | as appropriate |  | Consumer products |
| P103 | Read label before use. | as appropriate |  | Consumer products |

Table A3.2.2:  Codification of prevention precautionary statements

| Code | Prevention precautionary statements | Hazard class | Hazard category | Conditions for use |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| P201 | Obtain special instructions before use. | Explosives (chapter 2.1) | Unstable explosive |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity, effects on or via lactation (chapter 3.7) | Additional category |  |
| P202 | Do not handle until all safety precautions have been read and understood. | Explosives (chapter 2.1) | Unstable explosive |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
| P210 | Keep away from heat/sparks/open flames/hot surfaces. – No smoking. | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | Manufacturer/supplier or the competent authority to specify applicable ignition source(s). |
|  |  | Flammable gases (chapter 2.2) | 1, 2 |  |
|  |  | Flammable aerosols (chapter 2.3) | 1, 2 |  |
|  |  | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
|  |  | Flammable solids (chapter 2.7) | 1, 2 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
|  |  | Flammable liquids (chapter 2.6) | 4 | - specify to keep away from flames and hot surfaces. |
|  |  | Oxidizing liquids (chapter 2.13) | 1, 2, 3 | - specify to keep away from heat. |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
| P211 | Do not spray on an open flame or other ignition source. | Flammable aerosols (chapter 2.3) | 1, 2 |  |
| P220 | Keep/Store away from clothing/.../combustible materials. | Oxidizing gases (chapter 2.4) | 1 | ... Manufacturer/supplier or the competent authority to specify other incompatible materials. |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Oxidizing liquids (chapter 2.13) | 2, 3 |  |
|  |  | Oxidizing solids (chapter 2.14) | 2, 3 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1 | - specify to keep away from clothing and other combustible materials. |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P221 | Take any precaution to avoid mixing with combustibles/... | Oxidizing liquids (chapter 2.13) | 1, 2, 3 | ... Manufacturer/supplier or the competent authority to specify other incompatible materials. |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
| P222 | Do not allow contact with air. | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
| P223 | Keep away from any possible contact with water, because of violent reaction and possible flash fire. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2 |  |
| P230 | Keep wetted with ... | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.5 | ... Manufacturer/supplier or the competent authority to specify appropriate material. - if drying out increases explosion hazard, except as needed for manufacturing or operating processes (e.g. nitrocellulose). |
| P231 | Handle under inert gas. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P232 | Protect from moisture. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P233 | Keep container tightly closed. | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3 | - if product is volatile so as to generate hazardous atmosphere. |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P234 | Keep only in original container. | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
|  |  | Substances and mixtures corrosive to metals (chapter 2.16) | 1 |  |
| P235 | Keep cool. | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Self-heating substances and mixtures (chapter 2.11) | 1, 2 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
| P240 | Ground/bond container and receiving equipment. | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | - if the explosive is electrostatically sensitive. |
|  |  | Flammable liquids (chapter 2.6) | 1, 2, 3 | - if electrostatically sensitive material is for reloading. - if product is volatile so as to generate hazardous atmosphere. |
|  |  | Flammable solids (chapter 2.7) | 1, 2 | - if electrostatically sensitive material is for reloading. |
| P241 | Use explosion-proof electrical/ventilating/lighting/.../ equipment. | Flammable liquids (chapter 2.6) | 1, 2, 3 | … Manufacturer/supplier or the competent authority to specify other equipment. |
|  |  | Flammable solids (chapter 2.7) | 1, 2 | ... Manufacturer/supplier or the competent authority to specify other equipment. - if dust clouds can occur. |
| P242 | Use only non-sparking tools. | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
| P243 | Take precautionary measures against static discharge. | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
| P244 | Keep reduction valves and fittings free from grease and oil oil and grease. | Oxidizing gases (chapter 2.4) | 1 |  |
| P250 | Do not subject to grinding/shock/…/friction. | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | ... Manufacturer/supplier or the competent authority to specify applicable rough handling. |
| P251 | Pressurized container: Do not pierce or burn, even after use. | Flammable aerosols (chapter 2.3) | 1, 2 |  |
| P260 | Do not breathe dust/fume/gas/mist/vapours/spray. | Acute toxicity – inhalation (chapter 3.1) | 1, 2 | Manufacturer/supplier or the competent authority to specify applicable conditions |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
|  |  | Specific target organ toxicity, repeated exposure (chapter 3.9) | 1, 2 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C | - specify do not breathe dusts or mists - if inhalable particles of dusts or mists may occur during use. |
|  |  | Reproductive toxicity – effects on or via lactation (chapter 3.7) | Additional category |  |
| P261 | Avoid breathing dust/fume/gas/mist/vapours/spray. | Acute toxicity – inhalation (chapter 3.1) | 3, 4 | Manufacturer/supplier or the competent authority to specify applicable conditions. -may be omitted if P260 is given on label |
|  |  | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P262 | Do not get in eyes, on skin, or on clothing. | Acute toxicity – dermal (chapter 3.1) | 1, 2 |  |
| P263 | Avoid contact during pregnancy/while nursing. | Reproductive toxicity – effects on or via lactation (chapter 3.7) | Additional category |  |
| P264 | Wash ... thoroughly after handling. | Acute toxicity – oral (chapter 3.1) | 1, 2, 3, 4 | ... Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. |
|  |  | Acute toxicity – dermal (chapter 3.1) | 1, 2 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Eye irritation (chapter 3.3) | 2A, 2B |  |
|  |  | Reproductive toxicity – effects on or via lactation (chapter 3.7) | Additional category |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
|  |  | Specific target organ toxicity, repeated exposure (chapter 3.9) | 1 |  |
| P270 | Do not eat, drink or smoke when using this product. | Acute toxicity – oral (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Acute toxicity – dermal (chapter 3.1) | 1, 2 |  |
|  |  | Reproductive toxicity – effects on or via lactation (chapter 3.7) | Additional category |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
|  |  | Specific target organ toxicity, repeated exposure (chapter 3.9) | 1 |  |
| P271 | Use only outdoors or in a well-ventilated area. | Acute toxicity – inhalation (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P272 | Contaminated work clothing should not be allowed out of the workplace. | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P273 | Avoid release to the environment. | Hazardous to the aquatic environment, acute hazard (chapter 4.1) | 1, 2, 3 | - if this is not the intended use. |
|  |  | Hazardous to the aquatic environment, long-term hazard (chapter 4.1) | 1, 2, 3, 4 |  |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection. | Explosives (chapter 2.1) | Unstable explosive | Manufacturer/supplier or the competent authority to specify type of equipment |
|  |  | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | - Specify face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |
|  |  | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 | - Specify protective gloves and eye/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |
|  |  | Flammable solids (chapter 2.7) | 1, 2 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Self-heating substances and mixtures (chapter 2.11) | 1, 2 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1, 2, 3 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
|  |  | Acute toxicity – dermal (chapter 3.1) | 1, 2, 3, 4 | - Specify protective gloves/clothing. Manufacturer/supplier or the competent authority to specify type of equipment. |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C | - Specify protective gloves/clothing and eye/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |
|  |  | Skin irritation (chapter 3.2) | 2 | - Specify protective gloves. Manufacturer/supplier or the competent authority to specify type of equipment. |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Severe eye damage (chapter 3.3) | 1 | - Specify eye/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |
|  |  | Eye irritation (chapter 3.3) | 2A |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 | Manufacturer or the competent authority to specify type of equipment |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
| P281 | Use personal protective equipment as required. | Explosives (chapter 2.1) | Unstable explosive |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
| P282 | Wear cold insulating gloves/face shield/eye protection. | Gases under pressure (chapter 2.5) | Refrigerated liquefied gas |  |
| P283 | Wear fire/flame resistant/retardant clothing. | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P284 | [In case of inadequate ventilation] wear respiratory protection. | Acute toxicity, inhalation (chapter 3.1) | 1, 2 | Manufacturer/supplier or the competent authority to specify equipment. - text in square brackets may be used where additional safety information regarding adequate ventilation is provided |
|  |  | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P285 | In case of inadequate ventilation wear respiratory protection. | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B | Manufacturer/supplier or the competent authority to specify equipment. |
| P231 + P232 | Handle under inert gas. Protect from moisture. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P235 + P410 | Keep cool. Protect from sunlight. | Self-heating substances and mixtures (chapter 2.11) | 1, 2 |  |

Table A3.2.3:  Codification of response precautionary statements

| Code | Response precautionary statements | Hazard class | Hazard category | Conditions for use |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| P301 | IF SWALLOWED: | Acute toxicity, oral (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Aspiration hazard (chapter 3.10) | 1, 2 |  |
| P302 | IF ON SKIN: | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P303 | IF ON SKIN (or hair): | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
| P304 | IF INHALED: | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3, 4, 5 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P305 | IF IN EYES: | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Severe eye damage (chapter 3.3) | 1 |  |
|  |  | Eye irritation (chapter 3.3) | 2A, 2B |  |
| P306 | IF ON CLOTHING: | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P307 | IF exposed: | Specific target organ toxicity, single exposure (chapter 3.8) | 1 |  |
| P308 | IF exposed or concerned: | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity, effects on or via lactation (chapter 3.7) | Additional category |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
| P309 | IF exposed or if you feel unwell: | Specific target organ toxicity, single exposure (chapter 3.8) | 2 |  |
| P310 | Immediately call a POISON CENTER or/ doctor/physician/…. | Acute toxicity, oral (chapter 3.1) | 1, 2, 3 | Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |
|  |  | Acute toxicity, dermal (chapter 3.1) | 1, 2 |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 1, 2 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Severe eye damage (chapter 3.3) | 1 |  |
|  |  | Aspiration hazard (chapter 3.10) | 1, 2 |  |
| P311 | Call a POISON CENTER or/ doctor/physician/…. | Acute toxicity, inhalation (chapter 3.1) | 3 | Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |
|  |  | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
| P312 | Call a POISON CENTER or /doctor/physician/… if you feel unwell. | Acute toxicity, oral (chapter 3.1) | 4 | Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |
|  |  | Acute toxicity, oral (chapter 3.1) | 5 |  |
|  |  | Acute toxicity, dermal (chapter 3.1) | 3, 4, 5 |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 4 |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 5 |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P313 | Get medical advice/attention. | Skin irritation (chapter 3.2) | 2, 3 |  |
|  |  | Eye irritation (chapter 3.3) | 2A, 2B |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity, effects on or via lactation (chapter 3.7) | Additional category |  |
| P314 | Get medical advice/attention if you feel unwell. | Specific target organ toxicity, repeated exposure (chapter 3.9) | 1, 2 |  |
| P315 | Get immediate medical advice/attention. | Gases under pressure (chapter 2.5) | Refrigerated liquefied gas |  |
| P320 | Specific treatment is urgent (see ... on this label). | Acute toxicity, inhalation (chapter 3.1) | 1, 2 | ... Reference to supplemental first aid instruction. - if immediate administration of antidote is required. |
| P321 | Specific treatment (see ... on this label). | Acute toxicity, oral (chapter 3.1) | 1, 2, 3 | ... Reference to supplemental first aid instruction. - if immediate administration of antidote is required. |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 3 | ... Reference to supplemental first aid instruction. - if immediate specific measures are required. |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C | ... Reference to supplemental first aid instruction. - manufacturer/supplier or competent authority may specify a cleansing agent if appropriate. |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1 | ... Reference to supplemental first aid instruction. - if immediate measures are required. |
|  |  | Acute toxicity, dermal (chapter 3.1) | 1, 2 | ... Reference to supplemental first aid instruction. - if immediate measures such as specific cleansing agent is advised. |
|  |  | Acute toxicity, dermal (chapter 3.1) | 3, 4 |  |
| P322 | Specific measures (see ... on this label). | Acute toxicity, dermal (chapter 3.1) | 1, 2 | ... Reference to supplemental first aid instruction. - if immediate measures such as specific cleansing agent is advised. |
|  |  | Acute toxicity, dermal (chapter 3.1) | 3, 4 |  |
| P330 | Rinse mouth. | Acute toxicity, oral (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
| P331 | Do NOT induce vomiting. | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Aspiration hazard (chapter 3.10) | 1, 2 |  |
| P332 | If skin irritation occurs: | Skin irritation (chapter 3.2) | 2, 3 |  |
| P333 | If skin irritation or rash occurs: | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P334 | Immerse in cool water/wrap in wet bandages. | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2 |  |
| P335 | Brush off loose particles from skin. | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2 |  |
| P336 | Thaw frosted parts with lukewarm water. Do not rub affected area. | Gases under pressure (chapter 2.5) | Refrigerated liquefied gas |  |
| P337 | If eye irritation persists: | Eye irritation (chapter 3.3) | 2A, 2B |  |
| P338 | Remove contact lenses, if present and easy to do. Continue rinsing. | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Severe eye damage (chapter 3.3) | 1 |  |
|  |  | Eye irritation (chapter 3.3) | 2A, 2B |  |
| P340 | Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P341 | If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing. | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P342 | If experiencing respiratory symptoms: | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P350 | Gently wash with plenty of soap and water. | Acute toxicity, dermal (chapter 3.1) | 1, 2 |  |
| P351 | Rinse cautiously with water for several minutes. | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Severe eye damage (chapter 3.3) | 1 |  |
|  |  | Eye irritation (chapter 3.3) | 2A, 2B |  |
| P352 | Wash with plenty of water/… soap and water. | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3, 4 | Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate. |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P353 | Rinse skin with water/shower. | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
| P360 | Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P361 | Remove/Take off immediately all contaminated clothing. | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
|  |  | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
| P362 | Take off contaminated clothing and wash before reuse. | Acute toxicity, dermal (chapter 3.1) | 4 |  |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P363 | Wash contaminated clothing before reuse. | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3 |  |
|  |  | Acute toxicity, dermal (chapter 3.1) | 4 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P364 | and wash it before reuse | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitization (chapter 3.4) | 1A, 1B |  |
| P370 | In case of fire: | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 |  |
|  |  | Oxidizing gases (chapter 2.4) | 1 |  |
|  |  | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 |  |
|  |  | Flammable solids (chapter 2.7) | 1, 2 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P370 (cont’d) | In case of fire: (cont’d) | Oxidizing liquids (chapter 2.13) | 1, 2, 3 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
| P371 | In case of major fire and large quantities: | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P372 | Explosion risk in case of fire. | Explosives (chapter 2.1) | Unstable explosives and Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | - except if explosives are 1.4S AMMUNITION AND COMPONENTS THEREOF. |
| P373 | DO NOT fight fire when fire reaches explosives. | Explosives (chapter 2.1) | Unstable explosives and Divisions 1.1, 1.2, 1.3, 1.4, 1.5 |  |
| P374 | Fight fire with normal precautions from a reasonable distance. | Explosives (chapter 2.1) | Division 1.4 | - if explosives are 1.4S AMMUNITION AND COMPONENTS THEREOF. |
| P375 | Fight fire remotely due to the risk of explosion. | Self-reactive substances and mixtures (chapter 2.8) | Types A, B |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P376 | Stop leak if safe to do so. | Oxidizing gases (chapter 2.4) | 1 |  |
| P377 | Leaking gas fire: Do not extinguish, unless leak can be stopped safely. | Flammable gases (chapter 2.2) | 1, 2 |  |
| P378 | Use ... for extinction. | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 | ... Manufacturer/supplier or the competent authority to specify appropriate media - if water increases risk. |
|  |  | Flammable solids (chapter 2.7) | 1, 2 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1, 2, 3 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
| P380 | Evacuate area. | Explosives (chapter 2.1) | Unstable explosives |  |
|  |  | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P381 | Eliminate all ignition sources if safe to do so. | Flammable gases (chapter 2.2) | 1, 2 |  |
| P390 | Absorb spillage to prevent material damage. | Substances and mixtures corrosive to metals (chapter 2.16) | 1 |  |
| P391 | Collect spillage. | Hazardous to the aquatic environment, acute hazard (chapter 4.1) | 1 |  |
|  |  | Hazardous to the aquatic environment, long-term hazard (chapter 4.1) | 1, 2 |  |
| P301 + P310 | IF SWALLOWED: Immediately call a POISON CENTER or/ doctor/physician/…. | Acute toxicity, oral (chapter 3.1) | 1, 2, 3 |  |
|  |  | Aspiration hazard (chapter 3.10) | 1, 2 |  |
| P301 + P312 | IF SWALLOWED: Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. | Acute toxicity, oral (chapter 3.1) | 4 |  |
| P301 + P330 + P331 | IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
| P302 + P334 | IF ON SKIN: Immerse in cool water/wrap in wet bandages. | Pyrophoric liquids (chapter 2.9) | 1 |  |
| P302 + P350 | IF ON SKIN: Gently wash with plenty of soap and water. | Acute toxicity, dermal (chapter 3.1) | 1, 2 |  |
| P302 + P352 | IF ON SKIN: Wash with plenty of soap and water/… | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3, 4 | Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate. |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P303 + P361 + P353 | IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. | Flammable liquids (chapter 2.6) | 1, 2, 3 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
| P304 + P312 | IF INHALED: Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. | Acute toxicity, inhalation (chapter 3.1) | 5 |  |
| P304 + P340 | Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Respiratory sensitization (Chapter 3.4) | 1, 1A, 1B |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P304 + P341 | IF INHALED: If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing. | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Severe eye damage (chapter 3.3) | 1 |  |
|  |  | Eye irritation (chapter 3.3) | 2A, 2B |  |
| P306 + P360 | IF ON CLOTHING: Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |
| P307+ P308 + P311 | IF exposed or concerned: Call a POISON CENTER or/ doctor/physician/….. | Specific target organ toxicity, single exposure (chapter 3.8) | 1 |  |
| P308 + P313 | IF exposed or concerned: Get medical advice/ attention. | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity, effects on or via lactation (chapter 3.7) | Additional category |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
| P309 P308 + P311 | IF exposed or concerned if you feel unwell: Call a POISON CENTER or/ doctor/physician/…. | Specific target organ toxicity, single exposure (chapter 3.8) | 2 |  |
| P332 + P313 | If skin irritation occurs: Get medical advice/ attention. | Skin irritation (chapter 3.2) | 2, 3 | - may be omitted when P333 + P313 appears on the label |
| P333 + P313 | If skin irritation or rash occurs: Get medical advice/attention. | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P335 + P334 | Brush off loose particles from skin. Immerse in cool water/wrap in wet bandages. | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2 |  |
| P337 + P313 | If eye irritation persists: Get medical advice/attention. | Eye irritation (chapter 3.3) | 2A, 2B |  |
| P342 + P311 | If experiencing respiratory symptoms: Call a POISON CENTER or/ doctor/physician/…. | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
| P361 + P364 | Take off immediately all contaminated clothing and wash it before reuse. | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3 |  |
| P362 + P364 | Take off contaminated clothing and wash it before reuse. | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3 |  |
|  |  | Skin irritation (chapter 3.2) | 2 |  |
|  |  | Skin sensitisation (chapter 3.4) | 1A, 1B |  |
| P370 + P376 | In case of fire: Stop leak if safe to do so. | Oxidizing gases (chapter 2.4) | 1 |  |
| P370 + P378 | In case of fire: Use ... for extinction. | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 | ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |
|  |  | Flammable solids (chapter 2.7) | 1, 2 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Pyrophoric liquids (chapter 2.9) | 1 |  |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1, 2, 3 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
| P370 + P380 | In case of fire: Evacuate area. | Explosives (chapter 2.1) | Divisions 1.1, 1.2, 1.3, 1.4, 1.5 |  |
| P370 + P380 + P375 | In case of fire: Evacuate area. Fight fire remotely due to the risk of explosion. | Self-reactive substances and mixtures (chapter 2.8) | Types A, B |  |
| P371 + P380 + P375 | In case of major fire and large quantities: Evacuate area. Fight fire remotely due to the risk of explosion. | Oxidizing liquids (chapter 2.13) | 1 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1 |  |

Table A3.2.4:  Codification of storage precautionary statements

| Code | Storage precautionary statements | Hazard class | Hazard category | Conditions for use |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| P401 | Store … | Explosives (chapter 2.1) | Unstable explosives and Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | ... in accordance with local/regional/ national/international regulations (to be specified). |
| P402 | Store in a dry place. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P403 | Store in a well-ventilated place. | Flammable gases (chapter 2.2) | 1, 2 |  |
|  |  | Oxidizing gases (chapter 2.4) | 1 |  |
|  |  | Gases under pressure (chapter 2.5) | Compressed gas |  |
|  |  |  | Liquefied gas |  |
|  |  |  | Refrigerated Liquefied gas |  |
|  |  |  | Dissolved gas |  |
|  |  | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3 | - if product is volatile so as to generate hazardous atmosphere. |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P404 | Store in a closed container. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P405 | Store locked up. | Acute toxicity, oral (chapter 3.1) | 1, 2, 3 |  |
|  |  | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3 |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
|  |  | Aspiration hazard (chapter 3.10) | 1, 2 |  |
| P406 | Store in corrosive resistant/... container with a resistant inner liner. | Substances and mixtures corrosive to metals (chapter 2.16) | 1 | ... Manufacturer/supplier or the competent authority to specify other compatible materials. |
| P407 | Maintain air gap between stacks/pallets. | Self-heating substances and mixtures (chapter 2.11) | 1, 2 |  |
| P410 | Protect from sunlight. | Flammable aerosols (chapter 2.3) | 1, 2 |  |
|  |  | Gases under pressure (chapter 2.5) | Compressed gas |  |
|  |  |  | Liquefied gas |  |
|  |  |  | Dissolved gas |  |
|  |  | Self-heating substances and mixtures (chapter 2.11) | 1, 2 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
| P411 | Store at temperatures not exceeding …°C/…°F. | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F | ... Manufacturer/supplier or the competent authority to specify temperature. |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
| P412 | Do not expose to temperatures exceeding 50 °C/ 122 °F. | Flammable aerosols (chapter 2.3) | 1, 2 |  |
| P413 | Store bulk masses greater than … kg/…lbs at temperatures not exceeding …°C/…°F. | Self-heating substances and mixtures (chapter 2.11) | 1, 2 | ... Manufacturer/supplier or the competent authority to specify mass and temperature. |
| P420 | Store away from other materials. | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Self-heating substances and mixtures (chapter 2.11) | 1, 2 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
| P422 | Store contents under ... | Pyrophoric liquids (chapter 2.9) | 1 | ... Manufacturer/supplier or the competent authority to specify appropriate liquid or inert gas. |
|  |  | Pyrophoric solids (chapter 2.10) | 1 |  |
| P402 + P404 | Store in a dry place. Store in a closed container. | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
| P403 + P233 | Store in a well-ventilated place. Keep container tightly closed. | Acute toxicity, inhalation (chapter 3.1) | 1, 2, 3 | - if product is volatile so as to generate hazardous atmosphere. |
|  |  | Specific target organ toxicity, single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
| P403 + P235 | Store in a well-ventilated place. Keep cool. | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
| P410 + P403 | Protect from sunlight. Store in a well-ventilated place. | Gases under pressure (chapter 2.5) | Compressed gas |  |
|  |  |  | Liquefied gas |  |
|  |  |  | Dissolved gas |  |
| P410 + P412 | Protect from sunlight. Do not expose to temperatures exceeding 50 °C/122 °F. | Flammable aerosols (chapter 2.3) | 1, 2 |  |
| P411 + P235 | Store at temperatures not exceeding …°C/…°F. Keep cool. | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F | ... Manufacturer/supplier or the competent authority to specify temperature. |

Table A3.2.5:  Codification of disposal precautionary statements

| Code | Disposal precautionary statements | Hazard class | Hazard category | Conditions for use |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| P501 | Dispose of contents/container to ... | Explosives (chapter 2.1) | Unstable explosives and Divisions 1.1, 1.2, 1.3, 1.4, 1.5 | ... in accordance with local/regional/ national/international regulation (to be specified). |
|  |  | Flammable liquids (chapter 2.6) | 1, 2, 3, 4 |  |
|  |  | Self-reactive substances and mixtures (chapter 2.8) | Types A, B, C, D, E, F |  |
|  |  | Substances and mixtures which, in contact with water, emit flammable gases (chapter 2.12) | 1, 2, 3 |  |
|  |  | Oxidizing liquids (chapter 2.13) | 1, 2, 3 |  |
|  |  | Oxidizing solids (chapter 2.14) | 1, 2, 3 |  |
|  |  | Organic peroxides (chapter 2.15) | Types A, B, C, D, E, F |  |
|  |  | Acute toxicity, oral (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Acute toxicity, dermal (chapter 3.1) | 1, 2, 3, 4 |  |
|  |  | Acute toxicity, inhalation (chapter 3.1) | 1, 2 |  |
|  |  | Skin corrosion (chapter 3.2) | 1A, 1B, 1C |  |
|  |  | Respiratory sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Skin sensitization (chapter 3.4) | 1, 1A, 1B |  |
|  |  | Germ cell mutagenicity (chapter 3.5) | 1A, 1B, 2 |  |
|  |  | Carcinogenicity (chapter 3.6) | 1A, 1B, 2 |  |
|  |  | Reproductive toxicity (chapter 3.7) | 1A, 1B, 2 |  |
|  |  | Specific target organ toxicity, single exposure (chapter 3.8) | 1, 2 |  |
| P501 (cont’d) | Dispose of contents/container to ... (cont’d) | Specific target organ toxicity , single exposure; respiratory tract irritation (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, single exposure; narcotic effects (chapter 3.8) | 3 |  |
|  |  | Specific target organ toxicity, repeated exposure (chapter 3.9) | 1, 2 |  |
|  |  | Aspiration hazard (chapter 3.10) | 1, 2 |  |
|  |  | Hazardous to the aquatic environment, acute hazard (chapter 4.1) | 1, 2, 3 |  |
|  |  | Hazardous to the aquatic environment, long-term hazard (chapter 4.1) | 1, 2, 3, 4 |  |
| P502 | Refer to manufacturer/supplier for information on recovery/recycling | Hazardous to the ozone layer (chapter 4.2) | 1 |  |

3. 	Amend the table in A3.3.5.1 of part 3 of Annex 3 of GHS as follows:

| EXPLOSIVES (Chapter 2.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exploding bomb |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Unstable explosive | Danger |  | H200 | Unstable explosive |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P201 Obtain special instructions before use. P202 Do not handle until all safety precautions have been read and understood. P281 Use personal protective equipment as required. P280 Wear protective gloves/protective clothing/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment |  | P372 Explosion risk in case of fire. P373 DO NOT fight fire when fire reaches explosives. P380 Evacuate area. |  |  | P401 Store ... …in accordance with local/regional/ national/international regulations (to be specified). | P501 Dispose of contents/container to ... …in accordance with local/regional/ national/international regulations (to be specified). |  |

| EXPLOSIVES (Chapter 2.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exploding bomb |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Division 1.1 | Danger |  | H201 | Explosive; mass explosion hazard |  |  |  |
| Division 1.2 | Danger |  | H202 | Explosive; severe projection hazard |  |  |  |
| Division 1.3 | Danger |  | H203 | Explosive; fire, blast or projection hazard |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P230 Keep wetted with... … Manufacturer/supplier or the competent authority to specify appropriate material. - if drying out increases explosion hazard, except as needed for manufacturing or operating processes (e.g. nitrocellulose). P240 Ground/bond container and receiving equipment. - if the explosive is electrostatically sensitive. P250 Do not subject to grinding/shock/…/friction. …Manufacturer/supplier or the competent authority to specify applicable rough handling. P280 Wear face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 +P380 In case of fire: evacuate area. P372 Explosion risk in case of fire. P373 DO NOT fight fire when fire reaches explosives. |  |  | P401 Store ... …in accordance with local/regional/national/international regulations (to be specified). | P501 Dispose of contents/container to ... … in accordance with local/ regional/national/international regulations (to be specified). |  |

| ## EXPLOSIVES ## (Chapter 2.1) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ### Symbol Exploding bomb |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |  |
| Division 1.4 | Warning |  | H204 |  | Fire or projection hazard |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P240 Ground/bond container and receiving equipment. - if the explosive is electrostatically sensitive. P250 Do not subject to grinding/shock/…/friction. Manufacturer/supplier or the competent authority to specify applicable rough handling. P280 Wear face protection. Manufacturer/supplier or competent authority to specify type of equipment. |  | P370+P380 In case of fire: Evacuate area. P372 Explosion risk in case of fire. - except if explosives are 1.4S AMMUNITION AND COMPONENTS THEREOF. P373 DO NOT fight fire when fire reaches explosives. P374 Fight fire with normal precautions from a reasonable distance. - If explosives are 1.4S AMMUNITION AND COMPONENTS THEREOF. |  |  |  | P401 Store ... …in accordance with local/regional/ national/international regulations (to be specified). | P501 Dispose of contents/container to... … in accordance with local/regional/national/international regulations (to be specified). |  |

| ## EXPLOSIVES ## (Chapter 2.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Division 1.5 | Danger |  | H205 | May mass explode in fire |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P230 Keep wetted with... … Manufacturer/supplier or the competent authority to specify appropriate material. - if drying out increases explosion hazard, except as needed for manufacturing or operating processes (e.g. nitrocellulose). P240 Ground/bond container and receiving equipment - if the explosive is electrostatically sensitive. P250 Do not subject to grinding/shock/…/friction. …Manufacturer/supplier or the competent authority to specify applicable rough handling. P280 Wear face protection. Manufacturer/supplier or competent authority to specify type of equipment. |  | P370 + P380 In case of fire: Evacuate area. P372 Explosion risk in case of fire. P373 DO NOT fight fire when fire reaches explosives. |  |  | P401 Store ... …in accordance with local/regional/ national/international regulations (to be specified). | P501 Dispose of contents/container to ... … in accordance with local/regional/ national/international regulations (to be specified). |  |

| ## FLAMMABLE GASES ## (Chapter 2.2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H220 | Extremely flammable gas |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. -No smoking. Manufacturer/supplier or competent authority to specify applicable ignition source(s). |  | P377 Leaking gas fire: Do not extinguish, unless leak can be stopped safely. P381 Eliminate all ignition sources if safe to do so. |  |  | P403 Store in well-ventilated place. |  |  |

| FLAMMABLE GASES (Chapter 2.2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | Warning |  | H221 | Flammable gas |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. -No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition sources(s). |  | P377 Leaking gas fire: Do not extinguish, unless leak can be stopped safely. P381 Eliminate all ignition sources if safe to do so. |  |  | P403 Store in well-ventilated place. |  |  |

| FLAMMABLE AEROSOLS (Chapter 2.3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H222 |  | Extremely flammable aerosol |  |  |
| 2 | Warning |  | H223 |  | Flammable aerosol |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  | Storage |  | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. -No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition sources(s). P211 Do not spray on an open flame or other ignition source. P251 Pressurized container: Do not pierce or burn, even after use. |  |  |  | P410 + P412 Protect from sunlight. Do not expose to temperatures exceeding 50 ºC/122 ºF. |  |  |  |

| OXIDIZING GASES (Chapter 2.4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame over circle |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H270 | May cause or intensify fire; oxidizer |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P220 Keep/Store away from clothing/…/combustible materials. …Manufacturer/supplier or the competent authority to specify other incompatible materials. P244 Keep reduction valves and fittings free from grease and oil oil and grease. |  | P370 + P376 In case of fire: Stop leak if safe to do so. |  |  | P403 Store in well-ventilated place. |  |  |

| GASES UNDER PRESSURE (Chapter 2.5) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Gas cylinder |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Compressed gas | Warning |  | H280 | Contains gas under pressure; may explode if heated |  |  |  |
| Liquefied gas | Warning |  | H280 | Contains gas under pressure; may explode if heated |  |  |  |
| Dissolved gas | Warning |  | H280 | Contains gas under pressure; may explode if heated |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  |  |  |  | P410 + P403 Protect from sunlight. Store in a well-ventilated place. |  |  |

| GASES UNDER PRESSURE (Chapter 2.5) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Gas cylinder |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Refrigerated liquefied gas | Warning |  | H281 | Contains refrigerated gas; may cause cryogenic burns or injury |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P282 Wear cold insulating gloves/face shield/eye protection. |  | P336 Thaw frosted parts with lukewarm water. Do not rub affected area. P315 Get immediate medical advice/attention |  |  | P403 Store in well-ventilated place. |  |  |

| FLAMMABLE LIQUIDS (Chapter 2.6) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H224 | Extremely flammable liquid and vapour |  |  |  |
| 2 | Danger |  | H225 | Highly flammable liquid and vapour |  |  |  |
| 3 | Warning |  | H226 | Flammable liquid and vapour |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces.– No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P233 Keep container tightly closed. P240 Ground/Bond container and receiving equipment - if electrostatically sensitive material is for reloading. - if product is volatile so as to generate hazardous atmosphere. P241 Use explosion-proof electrical/ventilating/ lighting/.../equipment. ... Manufacturer/supplier or the competent authority to specify other equipment. P242 Use only non-sparking tools. P243 Take precautionary measures against static discharge. P280 Wear protective gloves/eye protection/face protection Manufacturer/supplier or the competent authority to specify type of equipment. |  | P303 + P361 + P353 IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P403 + P235 Store in a well-ventilated place. Keep cool. | P501 Dispose of contents/container to... … in accordance with local/regional/national/international regulations (to be specified). |  |

| FLAMMABLE LIQUIDS (Chapter 2.6) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 4 | Warning |  | H227 | Combustible liquid |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from flames and hot surfaces. – No smoking. P280 Wear protective gloves/eye protection/face protection Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P403 + P235 Store in a well-ventilated place. Keep cool. | P501 Dispose of contents/container to... in accordance with local/regional/ national/international regulations (to be specified). |  |

| FLAMMABLE SOLIDS (Chapter 2.7) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H228 | Flammable solid |  |  |  |
| 2 | Warning |  | H228 | Flammable solid |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P240 Ground/Bond container and receiving equipment. - if electrostatically sensitive material is for reloading. P241 Use explosion-proof electrical/ventilating/ lighting/... /equipment. ... Manufacturer/supplier or the competent authority to specify other equipment. - if dust clouds can occur. P280 Wear protective gloves/eye protection/face protection Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  |  |  |  |

| SELF-REACTIVE SUBSTANCES AND MIXTURES (Chapter 2.8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exploding bomb |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Type A | Danger |  | H240 | Heating may cause an explosion |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P220 Keep/Store away from clothing/…/combustible materials. ... Manufacturer/supplier or the competent authority to specify other incompatible materials. P234 Keep only in original container. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. P370 + P380 + P375 In case of fire: Evacuate area. Fight fire remotely due to the risk of explosion. |  |  | P403 + P235 Store in a well-ventilated place. Keep cool. P411 Store at temperatures not exceeding …°C/…°F. ... Manufacturer/supplier or the competent authority to specify temperature. P420 Store away from other materials. | P501 Dispose of contents/container to... … in accordance with local/regional/national/international regulations (to be specified). |  |

| SELF-REACTIVE SUBSTANCES AND MIXTURES (Chapter 2.8) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exploding bomb and flame |  |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |  |
| Type B | Danger |  | H241 | Heating may cause a fire or explosion |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P220 Keep/Store away from clothing/.../combustible materials. ... Manufacturer/supplier or the competent authority to specify other incompatible materials. P234 Keep only in original container. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk P370 + P380 + P375 In case of fire: Evacuate area. Fight fire remotely due to the risk of explosion. |  |  | P403 + P235 Store in a well-ventilated place. Keep cool. P411 Store at temperatures not exceeding …°C/…°F. ... Manufacturer/supplier or the competent authority to specify temperature. P420 Store away from other materials. | P501 Dispose of contents/container to... …in accordance with local/regional/national/international regulations (to be specified). |  |  |

| SELF-REACTIVE SUBSTANCES AND MIXTURES (Chapter 2.8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Type C | Danger |  | H242 | Heating may cause a fire |  |  |  |
| Type D | Danger |  | H242 | Heating may cause a fire |  |  |  |
| Type E | Warning |  | H242 | Heating may cause a fire |  |  |  |
| Type F | Warning |  | H242 | Heating may cause a fire |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P220 Keep/Store away from clothing/…/combustible materials. …Manufacturer/supplier or the competent authority to specify other incompatible materials. P234 Keep only in original container. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P403 + P235 Store in a well-ventilated place. Keep cool. P411 Store at temperatures not exceeding …°C/…°F. ...Manufacturer/supplier or the competent authority to specify temperature. P420 Store away from other materials. | P501 Dispose of contents/container to... …in accordance with local/regional/national/international regulations (to be specified). |  |

| PYROPHORIC LIQUIDS (Chapter 2.9) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H250 | Catches fire spontaneously if exposed to air |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition sources(s). P222 Do not allow contact with air. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P302 + P334 IF ON SKIN: Immerse in cool water/wrap with wet bandages P370 + P378 In case of fire: Use ... for extinction ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P422 Store contents under ... ... Manufacturer/supplier or the competent authority to specify appropriate liquid or inert gas. |  |  |

| PYROPHORIC SOLIDS (Chapter 2.10) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H250 | Catches fire spontaneously if exposed to air |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P222 Do not allow contact with air. P280 Wear protective gloves/eye protection/face protection Manufacturer/supplier or the competent authority to specify type of equipment. |  | P335 + P334 Brush off loose particles from skin. Immerse in cool water/wrap in wet bandages. P370 + P378 In case of fire: Use ... for extinction ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P422 Store contents under ... …Manufacturer/supplier or the competent authority to specify appropriate liquid or inert gas. |  |  |

| SELF-HEATING SUBSTANCES AND MIXTURES (Chapter 2.11) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H251 | Self-heating; may catch fire |  |  |  |
| 2 | Warning |  | H252 | Self-heating in large quantities; may catch fire |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P235 + P410 Keep cool. Protect from sunlight. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  |  |  |  | P407 Maintain air gap between stacks/pallets. P413 Store bulk masses greater than … kg/…lbs at temperatures not exceeding …°C/…°F. ... Manufacturer/supplier or the competent authority to specify mass and temperature. P420 Store away from other materials. |  |  |

| SUBSTANCES AND MIXTURES WHICH, IN CONTACT WITH WATER, EMIT FLAMMABLE GASES (Chapter 2.12) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H260 | In contact with water releases flammable gases, which may ignite spontaneously |  |  |  |
| 2 | Danger |  | H261 | In contact with water releases flammable gas |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P223 Keep away from any possible contact with water, because of violent reaction and possible flash fire. P231 + P232 Handle under inert gas. Protect from moisture. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P335 + P334 Brush off loose particles from skin and immerse in cool water/wrap in wet bandages. P370 + P378 In case of fire: Use ... for extinction ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P402 + P404 Store in a dry place. Store in a closed container. | P501 Dispose of contents/container to... …in accordance with local/regional/national/ international regulations (to be specified). |  |

| SUBSTANCES AND MIXTURES WHICH, IN CONTACT WITH WATER, EMIT FLAMMABLE GASES (Chapter 2.12) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | Warning |  | H261 | In contact with water releases flammable gas |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P231 + P232 Handle under inert gas. Protect from moisture. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  | P402 + P404 Store in a dry place. Store in a closed container. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| OXIDIZING LIQUIDS (Chapter 2.13) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame over circle |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H271 | May cause fire or explosion; strong oxidizer |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat. P220 Keep/Store away from clothing and other combustible materials. P221 Take any precaution to avoid mixing with combustibles/... ... Manufacturer/supplier or the competent authority to specify other incompatible materials. P280 Wear protective gloves /eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. P283 Wear fire/flame resistant/retardant clothing. |  | P306 + P360 IF ON CLOTHING: Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. P371 + P380 + P375 In case of major fire and large quantities: Evacuate area. Fight fire remotely due to the risk of explosion. P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  |  | P501 Dispose of contents/container to... …in accordance with local/regional/ national/international regulations (to be specified). |  |

| OXIDIZING LIQUIDS (Chapter 2.13) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame over circle |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | Danger |  | H272 | May intensify fire; oxidizer |  |  |  |
| 3 | Warning |  | H272 | May intensify fire; oxidizer |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat. P220 Keep/Store away from clothing/…/combustible materials. …Manufacturer/supplier or the competent authority to specify other incompatible materials. P221 Take any precaution to avoid mixing with combustibles/... ... Manufacturer/supplier or the competent authority to specify other incompatible materials. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  |  | P501 Dispose of contents/container to... …in accordance with local/regional/ national/international regulations (to be specified). |  |

| OXIDIZING SOLIDS (Chapter 2.14) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame over circle |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H271 | May cause fire or explosion; strong oxidizer |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat. P220 Keep away from clothing and other combustible materials. P221 Take any precaution to avoid mixing with combustibles/... …Manufacturer/supplier or the competent authority to specify other incompatible materials. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. P283 Wear fire/flame resistant/retardant clothing. |  | P306 + P360 IF ON CLOTHING: Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. P371 + P380 + P375 In case of major fire and large quantities: Evacuate area. Fight fire remotely due to the risk of explosion. P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  |  | P501 Dispose of contents/container to... …in accordance with local/regional/ national/international regulations (to be specified). |  |

| OXIDIZING SOLIDS (Chapter 2.14) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame over circle |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | Danger |  | H272 | May intensify fire; oxidizer |  |  |  |
| 3 | Warning |  | H272 | May intensify fire; oxidizer |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat. P220 Keep/Store away from clothing/…/ combustible materials. … Manufacturer/supplier or the competent authority to specify incompatible materials. P221 Take any precaution to avoid mixing with combustibles/... …Manufacturer/supplier or the competent authority to specify other incompatible materials. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P370 + P378 In case of fire: Use ... for extinction. ... Manufacturer/supplier or the competent authority to specify appropriate media. - if water increases risk. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ORGANIC PEROXIDES (Chapter 2.15) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exploding bomb |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Type A | Danger |  | H240 | Heating may cause an explosion |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces.- No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P220 Keep/Store away from clothing/…/combustible materials. ... Manufacturer/supplier or the competent authority to specify incompatible materials. P234 Keep only in original container. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  |  |  |  | P411 + P235 Store at temperatures not exceeding …°C/…°F. Keep cool. ... Manufacturer/supplier or the competent authority to specify temperature. P410 Protect from sunlight. P420 Store away from other materials. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ORGANIC PEROXIDES (Chapter 2.15) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exploding bomb and flame |  |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |  |
| Type B | Danger |  | H241 | Heating may cause a fire or explosion |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P220 Keep /Store away from clothing/.../combustible materials. ... Manufacturer/supplier or the competent authority to specify incompatible materials. P234 Keep only in original container. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  |  |  |  | P411 + P235 Store at temperatures not exceeding …°C/…°F. Keep cool. Manufacturer/supplier or the competent authority to specify temperature. P410 Protect from sunlight. P420 Store away from other materials. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |  |

| ORGANIC PEROXIDES (Chapter 2.15) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Flame |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| Type C | Danger |  | H242 | Heating may cause a fire |  |  |  |
| Type D | Danger |  | H242 | Heating may cause a fire |  |  |  |
| Type E | Warning |  | H242 | Heating may cause a fire |  |  |  |
| Type F | Warning |  | H242 | Heating may cause a fire |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. Manufacturer/supplier or the competent authority to specify applicable ignition source(s). P220 Keep/Store away from clothing/.../ combustible materials ... Manufacturer/supplier or the competent authority to specify incompatible materials. P234 Keep only in original container. P280 Wear protective gloves/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  |  |  |  | P411 + P235 Store at temperatures not exceeding …°C/…°F. Keep cool. ... Manufacturer/supplier or the competent authority to specify temperature. P410 Protect from sunlight. P420 Store away from other materials. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| CORROSIVE TO METALS (Chapter 2.16) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Corrosion |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Warning |  | H290 | May be corrosive to metals |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P234 Keep only in original container. |  | P390 Absorb spillage to prevent material damage. |  |  | P406 Store in corrosive resistant/... container with a resistant inner liner. ... Manufacturer/supplier or the competent authority to specify other compatible materials. |  |  |

| ACUTE TOXICITY - ORAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Skull and crossbones |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H300 | Fatal if swallowed |  |  |  |
| 2 | Danger |  | H300 | Fatal if swallowed |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P264 Wash …thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER or/ doctor/physician/… Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - if immediate administration of antidote is required. P330 Rinse mouth. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - ORAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Skull and crossbones |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | Danger |  | H301 | Toxic if swallowed |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - if immediate administration of antidote is required. P330 Rinse mouth. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - ORAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 4 | Warning |  | H302 | Harmful if swallowed |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P301 + P312 IF SWALLOWED: Call a POISON CENTER or/ doctor/physician/… if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P330 Rinse mouth. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - ORAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 5 | Warning |  | H303 | May be harmful if swallowed |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  | P312 Call a POISON CENTER or/ doctor/physician/… if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  |  |  |  |

| ACUTE TOXICITY - DERMAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Skull and crossbones |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H310 | Fatal in contact with skin |  |  |  |
| 2 | Danger |  | H310 | Fatal in contact with skin |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P262 Do not get in eyes, on skin, or on clothing. P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. P280 Wear protective gloves/protective clothing. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P302 + P350 P352 IF ON SKIN: Gently wash with plenty of soap and water/…. Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate P310 Immediately call a POISON CENTER or/ doctor/physician/… Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P322 P321 Specific measures treatment (see ... on this label) ... Reference to supplemental first aid instruction. - if immediate measures such as specific cleansing agent is advised. P361 + P364 Remove/Take off immediately all contaminated clothing and wash it before reuse. P363 Wash contaminated clothing before reuse. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - DERMAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Skull and crossbones |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | Danger |  | H311 | Toxic in contact with skin |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P280 Wear protective gloves/protective clothing. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P302 + P350 P352 IF ON SKIN: Gently wash with plenty of soap and water/…. Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate P312 Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P322 P321 Specific measures treatment (see ... on this label)... Reference to supplemental first aid instruction. - if measures such as specific cleansing agent is advised. P361 + P364 Remove/Take off immediately all contaminated clothing and wash it before reuse. P363 Wash contaminated clothing before reuse. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - DERMAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 4 | Warning |  | H312 | Harmful in contact with skin |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P280 Wear protective gloves/protective clothing Manufacturer/supplier or the competent authority to specify type of equipment. |  | P302 + P350 P352 IF ON SKIN: Gently wash with plenty of soap and water/…. Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate P312 Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P322 P321 Specific measures treatment (see ... on this label) ... Reference to supplemental first aid instruction. - if measures such as specific cleansing agent is advised. P363 Wash contaminated clothing before reuse. P362 + P364 Take off contaminated clothing and wash it before reuse. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - DERMAL (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 5 | Warning |  | H313 | May be harmful in contact with skin |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  | P312 Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  |  |  |  |

| ACUTE TOXICITY - INHALATION (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Skull and crossbones |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H330 | Fatal if inhaled |  |  |  |
| 2 | Danger |  | H330 | Fatal if inhaled |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P260 Do not breathe dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. P271 Use only outdoors or in a well-ventilated area. P284 [In case of inadequate ventilation] wear respiratory protection. Manufacturer/supplier or the competent authority to specify equipment. - text in square brackets may be used where additional safety information regarding adequate ventilation is provided |  | P304 + P340 IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. P310 Immediately call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P320 Specific treatment is urgent (see ... on this label) ... Reference to supplemental first aid instruction. - if immediate administration of antidote is required. |  |  | P403 + P233 Store in a well-ventilated place. Keep container tightly closed. - if product is volatile as to generate hazardous atmosphere. P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - INHALATION (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Skull and crossbones |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | Danger |  | H331 | Toxic if inhaled |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P261 Avoid breathing dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. -may be omitted if P260 is given on label P271 Use only outdoors or in a well-ventilated area. |  | P304 + P340 IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. P311 Call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - if immediate specific measures are required. |  |  | P403 + P233 Store in a well-ventilated place. Keep container tightly closed. - if product is volatile so as to generate hazardous atmosphere. P405 Store locked up. | P501 Dispose of content/container to… … in accordance with local/regional/national/international regulations (to be specified). |  |

| ACUTE TOXICITY - INHALATION (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 4 | Warning |  | H332 | Harmful if inhaled |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P261 Avoid breathing dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions -may be omitted if P260 is given on label P271 Use only outdoors or in a well-ventilated area. |  | P304 + P340 IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. P312 Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  |  |  |  |

| ACUTE TOXICITY - INHALATION (Chapter 3.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 5 | Warning |  | H333 | May be harmful if inhaled |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  | P304 + P312 IF INHALED: Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  |  |  |  |

| SKIN CORROSION/IRRITATION (Chapter 3.2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Corrosion |
| Hazard category |  | Signal word | Hazard statement |  |  |  |  |
| 1A to 1C |  | Danger | H314 | Causes severe skin burns and eye damage |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention | Response |  |  |  | Storage | Disposal |  |
| P260 Do not breathe dusts or mists. - if inhalable particles of dusts or mists may occur during use. P264 Wash …thoroughly after handling. …Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P280 Wear protective gloves/protective clothing/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. | P301 + P330 + P331 IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. P303 + P361 + P353 IF ON SKIN (or hair): Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower. P363 Wash contaminated clothing before reuse. P304 + P340 IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. P310 Immediately call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate. P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |  |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| SKIN CORROSION/IRRITATION (Chapter 3.2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | Warning |  | H315 | Causes skin irritation |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P280 Wear protective gloves. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P302 + P350 P352 IF ON SKIN: Wash with plenty of soap and water/…. Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate. P332 + P313 If skin irritation occurs: Get medical advice/attention. P362 + P364 Take off contaminated clothing and wash before reuse and wash it before reuse. |  |  |  |  |  |

| SKIN CORROSION/IRRITATION (Chapter 3.2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | Warning |  | H316 | Causes mild skin irritation |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  | P332 + P313 If skin irritation occurs: Get medical advice/attention. |  |  |  |  |  |

| EYE DAMAGE/IRRITATION (Chapter 3.3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Corrosion |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H318 | Causes serious eye damage |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P280 Wear eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P310 Immediately call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  |  |  |  |

| EYE DAMAGE/IRRITATION (Chapter 3.3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2A | Warning |  | H319 | Causes serious eye irritation |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P280 Wear eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337 + P313 If eye irritation persists: Get medical advice/attention. |  |  |  |  |  |

| EYE DAMAGE/IRRITATION (Chapter 3.3) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2B | Warning |  | H320 | Causes eye irritation |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. |  | P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337 + P313 If eye irritation persists: Get medical advice/attention. |  |  |  |  |  |

| SENSITIZATION - RESPIRATORY (Chapter 3.4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1, 1A, 1B | Danger |  | H334 | May cause allergy or asthma symptoms or breathing difficulties if inhaled |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P261 Avoid breathing dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. - may be omitted if P260 is given on label P285 P284 [In case of inadequate ventilation] wear respiratory protection. Manufacturer/supplier or the competent authority to specify equipment - text in square brackets may be used where additional safety information regarding adequate ventilation is provided |  | P304 + P341 P340 IF INHALED: If breathing is difficult, Rremove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. P342 + P311 If experiencing respiratory symptoms: Call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| SENSITIZATION - SKIN (Chapter 3.4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1, 1A, 1B | Warning |  | H317 | May cause an allergic skin reaction |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P261 Avoid breathing dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. -may be omitted if P260 is given on label P272 Contaminated work clothing should not be allowed out of the workplace. P280 Wear protective gloves. Manufacturer/supplier or the competent authority to specify type of equipment. |  | P302 + P350 P352 IF ON SKIN: Gently wash with plenty of soap and water/…. Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate, or may recommend an alternative agent in exceptional cases if water is clearly inappropriate P333 + P313 If skin irritation or rash occurs: Get medical advice/attention. P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - Manufacturer/supplier or the competent authority may specify a cleansing agent if appropriate. P362 + P364 Take off contaminated clothing and wash it before reuse. P363 Wash contaminated clothing before reuse. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| GERM CELL MUTAGENICITY (Chapter 3.5) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H340 | May cause genetic defects <...> |  |  |  |
| 2 | Warning |  | H341 | Suspected of causing genetic defects <...> |  |  |  |
|  |  |  | <...> (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P201 Obtain special instructions before use. P202 Do not handle until all safety precautions have been read and understood. P281 Use personal protective equipment as required. P280 Wear protective gloves/protective clothing/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment |  | P308 + P313 IF exposed or concerned: Get medical advice/attention. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| CARCINOGENICITY (Chapter 3.6) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H350 | May cause cancer <...> |  |  |  |
| 2 | Warning |  | H351 | Suspected of causing cancer <...> |  |  |  |
|  |  |  | <...> (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard). |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P201 Obtain special instructions before use. P202 Do not handle until all safety precautions have been read and understood. P281 Use personal protective equipment as required. P280 Wear protective gloves/protective clothing/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment |  | P308 + P313 IF exposed or concerned: Get medical advice/attention. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| TOXIC TO REPRODUCTION (Chapter 3.7) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H360 | May damage fertility or the unborn child <...> <<...>> |  |  |  |
| 2 | Warning |  | H361 | Suspected of damaging fertility or the unborn child <...> <<...>> |  |  |  |
|  |  |  | <...> (state specific effect if known) |  |  |  |  |
|  |  |  | <<...>> (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P201 Obtain special instructions before use. P202 Do not handle until all safety precautions have been read and understood. P281 Use personal protective equipment as required. P280 Wear protective gloves/protective clothing/eye protection/face protection. Manufacturer/supplier or the competent authority to specify type of equipment |  | P308 + P313 IF exposed or concerned: Get medical advice/attention. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| TOXIC TO REPRODUCTION (Chapter 3.7) (effects on or via lactation) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| (additional) | No signal word |  | H362 | May cause harm to breast-fed children |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P201 Obtain special instructions before use. P260 Do not breathe dusts or mists. - if inhalable particles of dusts or mists may occur during use. P263 Avoid contact during pregnancy/while nursing. P264 Wash … thoroughly after handling. …Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P308 + P313 IF exposed or concerned: Get medical advice/attention. |  |  |  |  |  |

| SPECIFIC TARGET ORGAN TOXICITY (Single Exposure) (Chapter 3.8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H370 | Causes damage to organs <...> <<...>> |  |  |  |
|  |  |  | <...> (or state all organs affected if known) |  |  |  |  |
|  |  |  | <<...>> (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P260 Do not breathe dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. P264 Wash …thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P307P308 + P311 IF exposed or concerned: Call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P321 Specific treatment (see ... on this label) ... Reference to supplemental first aid instruction. - if immediate measures are required. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| SPECIFIC TARGET ORGAN TOXICITY (Single exposure) (Chapter 3.8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | Warning |  | H371 | May cause damage to organs <...> <<...>> |  |  |  |
|  |  |  | <...> (or state all organs affected, if known) |  |  |  |  |
|  |  |  | <<...>> (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P260 Do not breathe dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. P264 Wash … thoroughly after handling. … Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P309P308 + P311 IF exposed or concerned or if you feel unwell: Call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| SPECIFIC TARGET ORGAN TOXICITY (Single Exposure) (Chapter 3.8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | Warning |  | H335 | May cause respiratory irritation; or |  |  |  |
|  |  |  | H336 | May cause drowsiness or dizziness |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P261 Avoid breathing dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. -may be omitted if P260 is given on label P271 Use only outdoors or in a well-ventilated area. |  | P304 + P340 IF INHALED: Remove victim person to fresh air and keep at rest in a position ensure comfortable for breathing. P312 Call a POISON CENTER or/ doctor/physician/…. if you feel unwell. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. |  |  | P403 + P233 Store in a well-ventilated place. Keep container tightly closed. - if product is volatile so as to generate hazardous atmosphere. P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| SPECIFIC TARGET ORGAN TOXICITY (Repeated exposure) (Chapter 3.9) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H372 | Causes damage to organs <...> through prolonged or repeated exposure <<...>> |  |  |  |
|  |  |  | <...> (state all organs affected, if known) |  |  |  |  |
|  |  |  | <<...>> (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P260 Do not breathe dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. P264 Wash … thoroughly after handling. …Manufacturer/supplier or the competent authority to specify parts of the body to be washed after handling. P270 Do not eat, drink or smoke when using this product. |  | P314 Get medical advice/attention if you feel unwell. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| SPECIFIC TARGET ORGAN TOXICITY (Repeated Exposure) (Chapter 3.9) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | Warning |  | H373 | May cause damage to organs <...> through prolonged or repeated exposure <<...>> |  |  |  |
|  |  |  | <...> | (state all organs affected, if known) |  |  |  |
|  |  |  | <<...>> | (state route of exposure if it is conclusively proven that no other routes of exposure cause the hazard) |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P260 Do not breathe dust/fume/gas/mist/ vapours/spray. Manufacturer/supplier or the competent authority to specify applicable conditions. |  | P314 Get medical advice/attention if you feel unwell. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| ASPIRATION HAZARD (Chapter 3.10) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Health hazard |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Danger |  | H304 | May be fatal if swallowed and enters airways |  |  |  |
| 2 | Warning |  | H305 | May be harmful if swallowed and enters airways |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  | P301 + P310 IF SWALLOWED: Call a POISON CENTER or/ doctor/physician/…. Manufacturer/supplier or the competent authority to specify the appropriate source of emergency medical advice. P331 Do NOT induce vomiting. |  |  | P405 Store locked up. | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| HAZARDOUS TO THE AQUATIC ENVIRONMENT - ACUTE HAZARD (Chapter 4.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Environment |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Warning |  | H400 | Very toxic to aquatic life |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P273 Avoid release to the environment. - if this is not the intended use. |  | P391 Collect spillage. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| HAZARDOUS TO THE AQUATIC ENVIRONMENT - ACUTE HAZARD (Chapter 4.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 2 | No signal word |  | H401 | Toxic to aquatic life |  |  |  |
| 3 | No signal word |  | H402 | Harmful to aquatic life |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P273 Avoid release to the environment. - if this is not the intended use. |  |  |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| HAZARDOUS TO THE AQUATIC ENVIRONMENT - LONG-TERM HAZARD (Chapter 4.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Environment |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Warning |  | H410 | Very toxic to aquatic life with long lasting effects |  |  |  |
| 2 | No signal word |  | H411 | Toxic to aquatic life with long lasting effects |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P273 Avoid release to the environment. - if this is not the intended use. |  | P391 Collect spillage. |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| HAZARDOUS TO THE AQUATIC ENVIRONMENT - LONG-TERM HAZARD (Chapter 4.1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol No symbol |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 3 | No signal word |  | H412 | Harmful to aquatic life with long lasting effects |  |  |  |
| 4 | No signal word |  | H413 | May cause long lasting harmful effects to aquatic life |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
| P273 Avoid release to the environment. - if this is not the intended use. |  |  |  |  |  | P501 Dispose of contents/container to... ... in accordance with local/regional/national/international regulations (to be specified). |  |

| HAZARDOUS TO THE OZONE LAYER (Chapter 4.2) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ### Symbol Exclamation mark |
| Hazard category | Signal word |  | Hazard statement |  |  |  |  |
| 1 | Warning |  | H420 | Harms public health and the environment by destroying ozone in the upper atmosphere |  |  |  |
| Precautionary statements |  |  |  |  |  |  |  |
| Prevention |  | Response |  |  | Storage | Disposal |  |
|  |  |  |  |  |  | P502 Refer to manufacturer/supplier for information on recovery/recycling |  |