|  | United Nations | ECE/TRANS/WP.11/2009/11/Rev.1/Add.1 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 18 August 2010 Original: English |

Economic Commission for Europe

Inland Transport Committee

Working Party on the Transport of Perishable Foodstuffs

Sixty-sixth session

Geneva, 9-12 November 2010

Item 5 (a) of the provisional agenda

Proposals of amendments to the ATP: Pending proposals

Introduction of type examination certificates as a means of 
certifying compliance of designs and tests carried out in 
accordance with the ATP

Transmitted by the Government of France

Addendum

1.	The certificates to be annexed to document ECE/TRANS/WP.11/2009/11/Rev.1 are reproduced below.

MODEL No. l A –Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of:  Wagon  lorry  trailer  semi-trailer  container  Other: …………

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Principal dimensions:

Outside: length 		m, width …………… m, height………………… m

Inside: length 		m, width …………… m, height………………… m

Total floor area of body		m2

Usable internal volume of body 	m3

Total inside surface area Si of body 	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the body walls, structural peculiarities of body and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, the equipment mentioned above is assignable to the following category (3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |
| --- | --- |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue			Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2)State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No………..

Page 2/2

General view of the equipment

Include here a picture of the body

Specifications of the body walls

| Unit (mm) | Outside coating | Thermal insulation | Internal coating | Total | Density kg/m3 |
| --- | --- | --- | --- | --- | --- |
| Top |  |  |  |  |  |
| Lateral sides |  |  |  |  |  |
| Rear door/wall |  |  |  |  |  |
| Front face |  |  |  |  |  |
| Bottom |  |  |  |  |  |

Abbreviations: GC = Gel coat / PU = Polyurethane

Structural peculiarities of the body

| Accessories | No. | Position in the body | Type (No. of door flaps) | Height (mm) | Length (mm) | Thickness (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Doors |  | Right side / Left side |  |  |  |  |
| Vents |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

This type examination certificate is composed of 1 page and 1 page of annex and must be reproduced in full.

MODEL No. l B –Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: Tanks for the carriage of liquid foodstuffs

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Principal dimensions:

Outside: length of cylinder 		m, major axis …………… m, minor axis…………………	 m

Inside: length of cylinder		m, major axis …………… m, minor axis…………………	 m

Usable internal volume		m3

Internal volume of each compartment 	m3

Inside surface area of each compartment Si1 	, iS2	………………..,	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the tank walls, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, the equipment mentioned above is assignable to the following category(3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |
| --- | --- |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue			Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/2

General view of the equipment

Include here a picture of the body

Specifications of the tank walls

Structural peculiarities of the body

Number, dimensions and description of manholes

Description of manhole covers

Number, dimensions and description of discharge piping

Number and description of tank cradles

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

This type examination certificate is composed of 1 page and 1 page of annex and must be reproduced in full

MODEL No. 4 A –Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: Refrigerated equipment using ice or dry ice ( Wagon  lorry  trailer  semi-trailer  container  Other: 	)

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Description of cooling appliance:

Manufacturer

Type, serial number

Year of manufacture 	………………………………………………………………………………

Specifications of the equipment, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, and the performance of the cooling appliance, the equipment mentioned above is assignable to the following category(3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |  A |
| --- | --- | --- |
|  |  |  B |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |  A |
|  |  |  B |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue					Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/2

General view of the equipment

Include here a picture of the equipment

Principal dimensions of the body:

Outside: length 		m, width …………… m, height………………… m

Inside: length 		m, width …………… m, height………………… m

Total floor area of body		m2

Usable internal volume of body 	m3

Total inside surface area Si of body 	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the body walls

| Unit (mm) | Outside coating | Thermal insulation | Internal coating | Total | Density kg/m3 |
| --- | --- | --- | --- | --- | --- |
| Top |  |  |  |  |  |
| Lateral sides |  |  |  |  |  |
| Rear door/wall |  |  |  |  |  |
| Front face |  |  |  |  |  |
| Bottom |  |  |  |  |  |

Abbreviations: GC = Gel coat / PU = Polyurethane

Structural peculiarities of the body

| Accessories | No. | Position in the body | Type (No. of door flaps) | Height (mm) | Length (mm) | Thickness (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Doors |  | Right side / Left side |  |  |  |  |
| Vents |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

Description of cooling appliance:

Nature of refrigerant

Nominal refrigerant filling capacity specified by manufacturer 	kg

Actual filling of refrigerant used for test kg

 Drive independent  dependent  mains-operated

Cooling appliance  removable  not removable

Filling device

Inside ventilation appliances:

Description (number of appliances, etc.)

Power of electric fans 	W

Delivery rate 	m3/h

Dimensions of ducts: cross-section ………… m2, length 	m

Air intake screen; description1

This type examination certificate is composed of 1 page and 1 page of annex and must be reproduced in full.

MODEL No. 4 B - Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: Refrigerated equipment using eutectic plates ( Wagon  lorry  trailer  semi-trailer  container  Other: 	)

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Description of cooling appliance:

Manufacturer

Type, serial number

Year of manufacture

Specifications of the equipment, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, and the performance of the cooling appliance, the equipment mentioned above is assignable to the following category(3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |  A |
| --- | --- | --- |
|  |  |  B |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |  A |
|  |  |  B |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue					Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/2

General view of the equipment

Include here a picture of the equipment

Principal dimensions of the body:

Outside: length 		m, width …………… m, height………………… m

Inside: length 		m, width …………… m, height………………… m

Total floor area of body		m2

Usable internal volume of body 	m3

Total inside surface area Si of body 	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the body walls

| Unit (mm) | Outside coating | Thermal insulation | Internal coating | Total | Density kg/m3 |
| --- | --- | --- | --- | --- | --- |
| Top |  |  |  |  |  |
| Lateral sides |  |  |  |  |  |
| Rear door/wall |  |  |  |  |  |
| Front face |  |  |  |  |  |
| Bottom |  |  |  |  |  |

Abbreviations: GC = Gel coat / PU = Polyurethane

Structural peculiarities of the body

| Accessories | No. | Position in the body | Type (No. of door flaps) | Height (mm) | Length (mm) | Thickness (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Doors |  | Right side / Left side |  |  |  |  |
| Vents |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

Description of cooling appliance:

Description

Nature of eutectic solution

Nominal eutectic solution filling capacity specified by manufacturer 	kg

Latent heat at freezing temperature stated by manufacturer 	 	kJ/kg at 	°C

 Drive independent  dependent  mains-operated1

Cooling appliance  removable  not removable1

Eutectic plates: Make 	 Type

Dimensions and number of plates, where situated; distance from walls (attach drawing)

Total cold reserve stated by manufacturer for freezing temperature of	 kJ to 	°C

Inside ventilation appliances (if any):

Description

Automatic devices

Mechanical refrigerator (if any) (if any):

Make 	 Type 	 No.

Where situated

Compressor: Make 	 Type

Type of drive

Nature of refrigerant

Condenser

Refrigerating capacity stated by the manufacturer for the specified freezing temperature and an outside temperature of +30 °C

Automatic devices:

Make 	 Type

Defrosting (if any)

Thermostat

LP pressostat

HP pressostat

Relief valve

Others

Accessory devices:

Electrical heating devices of the door joint:

Capacity by linear metre of the resistor 	W/m

Linear length of the resistor

This type examination certificate is composed of 1 page and 1 page of annex and must be reproduced in full

MODEL No. 4 C - Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: Refrigerated equipment using liquefied gases ( Wagon  lorry  trailer  semi-trailer  container  Other: 	)

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Description of cooling appliance:

Manufacturer

Type, serial number

Year of manufacture

Specifications of the equipment, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, and the performance of the cooling appliance, the equipment mentioned above is assignable to the following category(3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |  A |
| --- | --- | --- |
|  |  |  B |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |  A |
|  |  |  B |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue			Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/2

General view of the equipment

Include here a picture of the equipment

Principal dimensions of the body:

Outside: length 		m, width …………… m, height………………… m

Inside: length 		m, width …………… m, height………………… m

Total floor area of body		m2

Usable internal volume of body 	m3

Total inside surface area Si of body 	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the body walls

| Unit (mm) | Outside coating | Thermal insulation | Internal coating | Total | Density kg/m3 |
| --- | --- | --- | --- | --- | --- |
| Top |  |  |  |  |  |
| Lateral sides |  |  |  |  |  |
| Rear door/wall |  |  |  |  |  |
| Front face |  |  |  |  |  |
| Bottom |  |  |  |  |  |

Abbreviations: GC = Gel coat / PU = Polyurethane

Structural peculiarities of the body

| Accessories | No. | Position in the body | Type (No. of door flaps) | Height (mm) | Length (mm) | Thickness (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Doors |  | Right side / Left side |  |  |  |  |
| Vents |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

Description of cooling appliance:

Description

Drive independent/dependent/mains-operated1

Cooling appliance removable/not removable1

Manufacturer

Type, serial number

Year of manufacture

Nature of refrigerant

Nominal refrigerant filling capacity specified by manufacturer 	kg

Actual filling of refrigerant used for test 	kg

Description of tank

Filling device (description, where situated)

Inside ventilation appliances:

Description (number, etc.)

Power of electric fans 	W

Delivery rate 	m3/h

Dimensions of ducts: cross-section 	 m2, length 	 m

Automatic devices: 	Mechanical refrigerator (if any):

Make 	 Type

Where situated

Compressor: Make 	 Type 	 No.

Type of drive

Nature of refrigerant

Condenser

Refrigerating capacity stated by the manufacturer for the specified freezing temperature and an outside temperature of +30 °C 	W

Automatic devices:

Make 	 Type

Defrosting (if any)

Thermostat

LP pressostat

HP pressostat

Relief valve

Others

Accessory devices:

Electrical heating devices of the door joint:

Capacity by linear metre of the resistor 	W/m

Linear length of the resistor

This type examination certificate is composed of 1 page and 1 page of annex and must be reproduced in full

MODEL No. 5 - Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: Mechanically refrigerated equipment ( Wagon  lorry  trailer  semi-trailer  container  Other: 	)

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Description of cooling appliance:

Manufacturer

Type, serial number

Year of manufacture

Specifications of the equipment, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, and the performance of the cooling appliance, the equipment mentioned above is assignable to the following category(3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |  A |
| --- | --- | --- |
|  |  |  B |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |  C |
|  |  |  D |
|  |  |  E |
|  |  |  F |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue					Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/3

General view of the equipment

Include here a picture of the equipment

Principal dimensions of the body:

Outside: length 		m, width …………… m, height………………… m

Inside: length 		m, width …………… m, height………………… m

Total floor area of body		m2

Usable internal volume of body 	m3

Total inside surface area Si of body 	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the body walls

| Unit (mm) | Outside coating | Thermal insulation | Internal coating | Total | Density kg/m3 |
| --- | --- | --- | --- | --- | --- |
| Top |  |  |  |  |  |
| Lateral sides |  |  |  |  |  |
| Rear door/wall |  |  |  |  |  |
| Front face |  |  |  |  |  |
| Bottom |  |  |  |  |  |

Abbreviations: GC = Gel coat / PU = Polyurethane

Structural peculiarities of the body

| Accessories | No. | Position in the body | Type (No. of door flaps) | Height (mm) | Length (mm) | Thickness (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Doors |  | Right side / Left side |  |  |  |  |
| Vents |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

Description of mechanical refrigerating appliance:

Drive  independent  dependent  mains-operated1

Mechanical refrigerating appliances  removable  not removable

Nature of refrigerant and filling capacity

Effective refrigerating capacity stated by manufacturer for an outside temperature of + 30 °C and an inside temperature of:

0 °C

-10 °C

-20 ° C

Compressor:

Make 	 Type

Drive:  electric  thermal  hydraulic

Description

Make 	 Type 	power	kW	at

Condenser and evaporator

Motor element of fan(s):	 make 	type	number

power:	 kW 	at	rpm

This type examination certificate is composed of 1 page and 2 pages of annex and must be reproduced in full

Annex to Type Examination Certificate No.………..

Page 3/3

Inside ventilation appliances:

Description (number of appliances, etc.)

Power of electric fans 	W

Delivery rate 	m3/h

Dimensions of ducts: cross-section	 m2 length	m

Automatic devices:

Make 	 Type

Defrosting (if any)

Thermostat

LP pressostat

HP pressostat

Relief valve

Others

This type examination certificate is composed of 1 page and 2 pages of annex and must be reproduced in full

MODEL No. 6 - Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: Heated equipment ( Wagon  lorry  trailer  semi-trailer  container  Other: 		)

Technical specifications:

Brand:		Registration number: 	 Serial number:

Date of first entry into service:	Tare(2) kg		Carrying capacity (2):	kg

Body description:

Brand and type:			Identification number:

Built by:			Date of construction:

Description of heating appliance:

Manufacturer

Type, serial number 	………………………………………………………………………………

Year of manufacture 	………………………………………………………………………………

Specifications of the equipment, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: On the basis of the result contained in the test report N° XXXX issued by [NAME] on [DD/MM/YYYY], especially the K coefficient value established to be equal to ………… W/m²K, and the performance of the cooling appliance, the equipment mentioned above is assignable to the following category(3):

| I |  N (Normally insulated equipment characterized by a K coefficient equal to or less than 0.70 W/m²K;) |  A |
| --- | --- | --- |
|  |  R (Heavily insulated equipment characterized by a K coefficient equal to or less than 0.40 W/m²K and by sidewalls with a thickness of at least 45 mm for transport equipment of a width greater than 2.50 m.) |  B |
|  |  |  C |

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue					Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/2

General view of the equipment

Include here a picture of the equipment

Principal dimensions of the body:

Outside: length 		m, width …………… m, height………………… m

Inside: length 		m, width …………… m, height………………… m

Total floor area of body		m2

Usable internal volume of body 	m3

Total inside surface area Si of body 	m2

Total outside surface area Se of body	m2

Mean surface area: S =  	m2

Specifications of the body walls

| Unit (mm) | Outside coating | Thermal insulation | Internal coating | Total | Density kg/m3 |
| --- | --- | --- | --- | --- | --- |
| Top |  |  |  |  |  |
| Lateral sides |  |  |  |  |  |
| Rear door/wall |  |  |  |  |  |
| Front face |  |  |  |  |  |
| Bottom |  |  |  |  |  |

Abbreviations: GC = Gel coat / PU = Polyurethane

Structural peculiarities of the body

| Accessories | No. | Position in the body | Type (No. of door flaps) | Height (mm) | Length (mm) | Thickness (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Doors |  | Right side / Left side |  |  |  |  |
| Vents |  |  |  |  |  |  |
| Other: |  |  |  |  |  |  |

Supplementary devices and accessories:

- List here the supplementary devices or accessories like lighting, meat rails, meat rail runner stops, kick-strips…

- List only the ones which have an impact on the equipment's K value.

Description of mechanical refrigerating appliance:

Description

Drive  independent  dependent  mains-operated1

Heating appliance removable/not removable1

Manufacturer

Where situated

Overall area of heat exchange surfaces 	m2

Effective power rating as specified by manufacturer 	kW

Inside ventilation appliances:

Description (number of appliances, etc.)

Power of electric fans 	W

Delivery rate 	m3/h

Dimensions of ducts: cross-section 	m2 length 	m

This type examination certificate is composed of 1 page and 1 page of annex and must be reproduced in full

MODEL No. 10 - Type Examination Certificate

Type Examination Certificate

No………..

Issued by approved testing station/expert:(1)	Name

Address

In accordance with: the provisions of the Agreement on the International Carriage of Perishable Foodstuffs and on the Special Equipment to be Used for such Carriage (ATP)

Manufacturer:	Name

Address

Authorized representative:	Name

Address

In respect of: a Refrigeration Unit

 Self-contained  not self-contained  Removable  not removable  Single unit  assembled

Technical specifications:

Date of manufacture:	 Make:

Type: 	Serial No:

Description of the unit:

Compressor:	, Brand: 	…………… type…………………

Number of cylinders:	, Cubic capacity: 	…………… Nominal speed of rotation: …………… rpm

Methods of drive:

 electric motor   separate internal combustion engine  vehicle engine  vehicle motion

Compressor drive motor:

Electrical: Make 	 Type 	Power	kW	at	rpm

Supply voltage:	V, Supply frequency: 	…………… Hz

Internal combustion engine: Make 	 Type 		Number of cylinders

Cubic capacity: 		Power	kW	at	rpm

Fuel:

Hydraulic motor: Make 	 Type

Method of drive:

Alternator: Make 	 Type

Speed of rotation given by the manufacturer: Nominal 		rpm,     Minimum 	rpm

Speed of rotation given by the manufacturer: Nominal 		rpm,     Minimum 	rpm

Refrigerant fluid: 	Nominal capacity of refrigerant:

Specifications of the equipment, structural peculiarities and supplementary accessories are listed in the annex to this certificate.

Declaration of conformity: Transport equipment equipped with a refrigeration unit corresponding to this certificate may be accepted as mechanically refrigerated equipment without undergoing an efficiency test if the effective refrigerating capacity determined in the annex of the present certificate in continuous operation exceeds the heat loss through the walls for the class of ATP under consideration, multiplied by the factor 1.75.

Valid until: This certificate is valid for a period of 6 years from its date of issue

Date of issue					Signature of the issuing authority representative

Important: The principal characteristics and approval conditions are set out in the annex hereto, which forms part of the approval documents and consists of   page(s). All documents are registered by the approved testing station which issued this certificate. The manufacturer shall inform the approved test station that holds the technical documentation concerning this Type Examination Certificate of all modifications to the equipment that may affect its conformity with the requirements or the conditions for validity of this certificate. Such modifications require additional approval in the form of an addition to this original Type Examination Certificate.

(1)Delete as necessary (experts only in the case of tests carried out under ATP annex 1, appendix 2, para. 27 or 49).(2) State source of information.(3) Chose by marking the applicable category.

Annex to Type Examination Certificate No.………..

Page 2/2

Description of the type of refrigeration unit

Specifications of the unit

| Heat exchangers |  | Condenser | Evaporator |
| --- | --- | --- | --- |
| Make-type |  |  |  |
| Number of tubes |  |  |  |
| Fan pitch (mm) |  |  |  |
| Tube: nature and diameter (mm) |  |  |  |
| Exchange surface area (m2) |  |  |  |
| Frontal area (m2) |  |  |  |
| FANS | Number |  |  |
|  | Number of blades per fan |  |  |
|  | Diameter (mm) |  |  |
|  | Nominal power (W) |  |  |
|  | Total nominal output at a pressure of ………… Pa (m3/h) |  |  |
|  | Method of drive |  |  |

Expansion valve: Make: 	 Model: 	 Adjustable 	 Not adjustable

Defrosting device:

Automatic device:

Security device:

| Mean temperature inlet to evaporator | Refrigerating capacity (Wo) W |
| --- | --- |
| with engine driven: -20 °C -10 °C 0 °C | Compressor nominal speed ( rpm) |
| with electric motor driven -20 °C -10 °C 0 °C | Compressor nominal speed ( rpm) |

This type examination certificate is composed of 2 pages and must be reproduced in full

?	Submitted in accordance with the programme of work of the Inland Transport Committee for 
2010–2014 (ECE/TRANS/208, para. 106; ECE/TRANS/2010/8, programme activity 02.11).