| INF.7 | .15/88/INF.3 | INF.4 |  |
| --- | --- | --- | --- |
| Economic Commission for Europe Inland Transport Committee Working Party on the Transport of Perishable Foodstuffs 5 November 2010 Sixty-sixth session Geneva, 9-12 November 2010 Item 5 (b) of the provisional agenda Proposals for amendments to the ATP: New issues. |  |  |  |

K coefficient for equipment in service

Transmitted by the Government of Germany

Initial situation

Section 5 of Annex 1, Appendix 2 to the ATP specifies the two different testing procedures that the competent authority may apply to test equipment in service:

Measurement of the K coefficient in accordance with paragraphs 2.1.1 to 2.3.2 
or 
the assessment of the equipment by an expert on the basis of a visual inspection and technical expertise. Further details are given in sub-section 5.1.

Sub-section 5.3 “Decisions” states that the results of the measurement of the K coefficient must be “satisfactory“. The equipment may then be kept in service for a further period of six years in its initial class. If the expert’s conclusions after the visual inspection are favourable, the equipment may be kept in service for a further period of three years in its initial class.

At present, the ATP contains no further definition of the term “satisfactory“. Moreover, the initial K coefficient at the time of the equipment’s entry into service may not be used as a reference value given that the insulation of the equipment is subject to a normal physical aging process and that currently, for the majority of semi-trailers, the initial K coefficient lies between 0.35 W/m2K and 0.40 W/m2K.

In order to ensure equal conditions between transport operators in ATP member countries, we suggest that the following wording be added to sub-section 5.3:

Results shall be deemed to be satisfactory if the following values for the K coefficient are attained by equipment in service:

| Initial class | IR | IN |
| --- | --- | --- |
| Years in service | K coefficient (≤) | K coefficient (≤) |
| 6 | 0.5 | 0.9 |
| 9 | 0.6 | 1.1 |

_____________