Proposal for Corrigendum 2  to Supplement 3 to the 04 Series to Regulation No. 48 (ECE/TRANS/WP.29/2009/22)

A.	PROPOSAL

Paragraph 5.7.2.1 amend to read:

5.7.2.1. 	Either the total area of the projection of the distinct parts on a plane tangent to the exterior surface of the outer lens and perpendicular to the reference axis shall occupy not less than 60 per cent of the smallest quadrilateral circumscribing the said projection, or the distance between two adjacent/tangential distinct parts shall not exceed 15 mm when measured perpendicularly to the reference axis. This requirement shall not apply to a retro-reflector"

B.	JUSTIFICATION

The wording of paragraph 5.7.2.1 was amended by ECE/TRANS/WP29/2008/86 (Supplement 2 to the 04 Series of Regulation 48) with the addition of the words "This requirement shall not apply to a retro-reflector."

In parallel the proposal in ECE/TRANS/WP29/2009/22 (Supplement 3 to the 04 Series of Regulation 48) was developed, also referencing paragraph 5.7.2., but the amendment to be introduced by supplement 2 was not taken into consideration and the obsolete version of the text was inadvertently reinstated.

This proposal reinstates the correct wording of paragraph 5.7.2.1.