| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2010/4 14 January 2010 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Sixty-second session

Geneva, 29 - 31 March 2010

Item 7 of the provisional agenda

REGULATION No. 19

(Front fog lamps)

Proposal for Supplement 1 to the 04 series of amendments to

Regulation No. 19

Submitted by the expert from the Working Party "Brussels 1952" /

The text reproduced below was prepared by the expert from the Working Party "Brussels 1952" (GTB) in order to simplify the photometric testing requirements for the class "B" front fog lamps.  The modifications to the current text, including the draft 04 series of amendments to Regulation No. 19, are marked in bold or strikethrough characters.

A.	PROPOSAL

Paragraph 6.3.5., amend the table to read:

"6.3.5.	The illumination (see Annex 4, paragraph 2.1.) shall meet the following requirements:

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| Zone D | 1.75°D to 3.5°D | 12°L to 12°R | 1700 cd min 11500 cd max | At least one point on each vertical line |
|  |  |  | 11500 cd max | Whole zone |
| Zone E | 1.75°D to 3.5°D | 12°L to 22°L and 12°R to 22°R | 810 cd min 11500 cd max | At least one point on each vertical line |
|  |  |  | 11500 cd max | Whole zone |
| … |  |  |  |  |

…."

B. 	JUSTIFICATION

The current provisions in paragraph 6.3.5. relating to the class B front fog lamps require that compliance with the minimum intensity requirements in zones D and E is confirmed by "at least one point on each vertical line" and this is necessary to ensure that a minimum performance is guaranteed throughout the zone.  However, the same requirement relating to the verification of compliance of the maximum intensity provision involves much unnecessary testing time when it is sufficient to specify a maximum value for the complete zone.

This proposal does not involve any change to the maximum and minimum photometric requirements and follows the practice already adopted for the class F3 front fog lamp.

- - - - - -

/  In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles.  The present document is submitted in conformity with that mandate.