Proposal for draft amendments to global technical regulation No. 2

A. PROPOSAL

Part A (Statement of technical rationale and justification)

Paragraph 4(e), amend to read:

“(e) Reference Fuel

The principal performance requirements introduced in paragraph 5.2. of this global technical regulation are originally based on the use of Japanese fuel. It is observed that the Japanese fuel shows similar emission characteristics as the reference fuels specified in Annex 2 (A.2.1.), therefore, the use of this standardized reference fuel for determining compliance with the emission limits set out in paragraph 5.2. is considered as an ideal condition for ensuring the reproducibility of regulatory emission testing, and Contracting Parties are encouraged to use such fuel in their compliance testing.”

Part B (TEXT OF THE REGULATION)

Under Annex 2 (A.2.1), add a footnote:

The Japanese fuel can be used as an alternative fuel for paragraph 6.4. for compliance with the principal emission limits in reference to Part A paragraph 4(e).

B. JUSTIFICATION

A comparison study was performed using three motorcycles in order to check the emission tendency of CO, THC, and NOx.  The test results showed no significant difference in emission characteristics between the cases where the Japanese fuel and the fuels in Annex 2 (A.2.1.) were used.  The details of the test results are shown in Figures 1-3 and Table 1 below.

**********