GFV-10-04

Some issues on Dual-fuel for GFV-10 meeting 8 June 2010

Submitted by Volvo & TNO

GRPE guidance needed for work priorities and work process

- Huge number of issues for GFV. Major technology differences between HD (Diesel –

gas) and LD (several DF emerging technologies, incl. gasoline – gas) vehicles. Would a

separate Task-Force for HD-DF be a solution?

- Follow-up of section (17) of GFV-09-02 report : "A fundamental decision must be

made if certification requirements will be recommended in Reg.49, aimed at

OEMs."

- validation of section (23): Approaching OEM and retrofit simultaneously could save

time. Amendments for regulation 49 (HDV) are needed for drafting an

amendment in R.115 (concerning the emission test procedures to R49).

- Is it OK that new R49 rules be as harmonized as possible with other rules already

existing or under elaboration throughout the world?

Work-priority could then be given to the already regulated dual-fuel technologies in order

to facilitate their early introduction ("Low hanging fruits")

Types of HD-DF (Diesel – gas)

- DF Type 1 engines: Diesel / gas Dual-fuel engines with a very high level of gas usage,

for example the CAL "HD pilot ignition engines"

- DF Type 2 engines: Diesel / gas Dual-fuel engines with a balanced usage of Diesel fuel

- DF Type 3 engines: Diesel / gas Dual-fuel engines with a very low level of gas usage

- The split between the types needs further work (e.g. 10% diesel for Type1 in CAL)

Basic principles to be respected

- Dual-fuel (Diesel /gas) engines capable of running with pure Diesel-fuel should comply

with all the Diesel-fuel requirements.

However a limp-home operation in a pure Diesel mode should be considered as an

exception to this rule, but mobility restriction should apply (e.g. vehicle speed limitation).

- DF type1 engines should be considered as PI mono-fuel gas engines (they may be

considered like in the USA as pilot ignited engines). Note: PN and NH3 to be addressed

in the proper manner.

- DF type3 engines (e.g. less than 10% gas usage) should be considered as diesel-fuelled

engines (in order to ensure sufficient robustness to the Regulation by avoiding "pseudo"

Dual-fuels engines)

- DF type 2 engines should comply with the all the requirements applicable to Diesel mono-fuelled

engines and to all the requirements applicable to mono-fuelled gas engines in a

manner that is proportionate to the actual usage of respectively Diesel fuel and gas.

This concerns emissions on the engine test-bed during certification and on the road in

applying the PEMS method.

This also concerns CoP requirements, Durability requirements, OBD, NOx control

requirements, CO2 and FC recording. These requirements will need however more

careful investigation regarding the proportionate principle.

- - - - -