| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL Working Paper No. HDH-01-03 (1st HDH meeting, 19 May 2010) Original: ENGLISH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-fifty-first session, 22-25 June 2010,

agenda item 16.4)

PROPOSAL FOR AN EMISSIONS TEST PROCEDURE FOR HEAVY DUTY HYBRID VEHICLES (HD-HV'S)

Transmitted by the representative of the European Community (EC)

Note:	This document contains a proposal to develop a new annex to Global Technical Regulation n°4 (WHDC) concerning the emissions of pollutants and CO2 from heavy duty hybrid vehicles under the 1998 Agreement. It is based on the text of informal document No. GRPE-59-02 distributed during the fifty-ninth session of GRPE (ECE/TRANS/WP.29/GRPE/59, paras. 12 and 13).

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

A.	OBJECTIVE OF THIS PROPOSAL

The objective of this proposal is to establish an amendment to Global Technical Regulation (gtr) n° 4 (WHDC) with respect to pollutant emissions and CO2 emission from heavy duty hybrid vehicles under the 1998 Global Agreement. Greater fuel efficiency and the reduction of CO2 emissions are becoming an increasingly urgent issue in view of global warming and surging petroleum prices. Hybrid  vehicles (HV's) are recognized as one solution for achieving lower emissions and increased fuel efficiency. Consequently, a widespread introduction of HV's has taken place during the last years, primarily for passenger cars. But also commercial vehicle manufacturers have introduced, or announced the introduction, of several hybrid concepts for urban, delivery and extra-urban operation. While testing of passenger car hybrids is covered by ECE R 83, no provisions exist today within the ECE framework for heavy duty hybrids.

With gtr n°4, a globally harmonized emissions testing procedure for conventional commercial vehicles has been established. Traditionally, emissions testing of conventional heavy duty vehicles involves engine testing, and the certified engine can then be installed in any vehicle independent of its application. Contrary to conventional vehicles, emissions testing and certification of HV's disregarding the vehicle application is not the optimal technical solution. Since engine speed and load cycles of HV's are indeed different from those of conventional powertrains, it is necessary to incorporate vehicle and mission related elements into the certification procedure.

If such an approach is used, engine technologies and engine calibrations can be tailored to the hybrid applications allowing engine technology to be optimized for HV operation, keeping the overall emissions performance equal to conventional engines and fully optimizing fuel consumption, CO2 emissions and product costs.

B.	DESCRIPTION OF THE PROPOSED REGULATION

The proposal aims to provide an engine based test procedure and harmonized technical requirements for pollutant emissions and CO2 certification of HV's. The test procedure will focus around the HILS (Hardware-in-the-Loop) approach, which starts from a vehicle cycle and simulates powertrain and vehicle components to result in a HV specific engine cycle for emissions testing and measurement. This allows using the test cell environment, data evaluation procedures and emissions calculations already specified in gtr n°4. The proposal is intended to cover a wide range of HV technologies including electric hybrids, hydraulic hybrids, plug-in hybrids, range extenders and start/stop solutions.

As a possible future task, a chassis dynamometer based test procedure, as with passenger cars, might be investigated as an alternative to HILS. If used for type approval, such an approach would need specifying new requirements with respect to test cell environment, data evaluation procedures and emissions calculations, since they are currently not in the scope of gtr n°4.

It is proposed to use the World Harmonized Vehicle Cycle (WHVC) developed under the WHDC mandate as base cycle for the HILS method. Similar to the original WHDC approach, where a standard gearbox model was used for converting the WHVC into the standard engine cycle WHTC, HILS uses individual powertrain components (e.g. engine, transmission, electric motor, battery, accumulator), vehicle parameters (mass, inertia) and a driver model for creating the individual HV engine cycle. This HV engine cycle is then used for emissions testing. A certain HV vehicle standardization will be incorporated to accommodate a powertrain system in a range of similar vehicles.

HILS includes the following models:

-	The vehicle model covers running and acceleration resistance, taking into account rolling and air resistance coefficients, vehicle mass, rotating equivalent mass, speed and acceleration, etc.;

-	The MG (motor-generator) model represents the electric motor, the generator or other regenerative braking system whose input data are generated from component testing;

-	The transmission model represents clutch and gearbox, the gear ratios and efficiencies;

-	The battery, capacitor and accumulator models express the conditions of the battery/capacitor/accumulator, state of charge (SOC), capacity, resistance, charge and discharge power, etc.

In order to take the overall vehicle mission into account, application for the emissions test of subsets, or combinations of subsets, of the WHVC (urban, rural, motorway) in combination with appropriate weighting factors will be investigated. Testing requirements such as engine preconditioning, test fuels and general emissions testing and measurement provisions will be based on gtr n°4 (WHDC).

The following ambitious timetable is proposed:

| Item | Time |
| --- | --- |
| 1st WG meeting | 19/05/2010 |
| GRPE & WP.29 approval | 06/2010 |
| 2nd WG meeting (timing & cost) | 10/2010 |
| Report to GRPE | 01/2011 |
| 2 years work program |  |
| WG final report to GRPE | 01/2013 |
| GRPE adoption | 01/2014 |
| WP29 adoption | 06/2014 |

C.	EXISTING REGULATIONS AND INTERNATIONAL STANDARDS

Japanese Regulation:

Kokujikan No.60 of 30 June 2004, “Measurement Procedure for Exhaust Emission from Electric Hybrid Heavy-Duty Motor Vehicles”;

Kokujikan No.281 of 16 March 2007, “Measurement Procedure for Fuel Consumption Rate and Exhaust Emissions of Heavy-Duty Hybrid Electric Vehicles using Hardware-In-the-Loop Simulator System”

SAE Standards:

SAE J 2711 "Recommended Practice for Measuring Fuel Economy amd Emissions of Hybrid-Electric and Conventional Heavy-Duty Vehicles"

- - - - -