| WLTP DHC subgroup |  |
| --- | --- |
| Date | 22 DEC 2009 |
| Title | In-use data collection plan |
| Author | JAPAN |
| Working paper number | WLTP-DHC-02-10 |

# 1. Introduction

This in-use data collection plan was developed by Japanese experts based on the agreement during DHC meeting.

# 2. Schedule

After second meeting of DHC, start preparing for testing and expect to complete in-use data collection by the end of Feb.

Table 1 Data collecting schedule

# 3. Contents

## 3.1. Review currently available data

a.	JC08 cycle development

1)	Research agency : Suuri-Keikaku Corporation

2)	Requested by : Ministry of Environment

3)	Year : in 2000 and 2001

4)	Test vehicle : Total 10 vehicles

Passenger car : 4 vehicles, Light duty commercial vehicle : 6 vehicles,

( see Table2. and Fig1. for more detail)

5)	Test condition : In case of LDCVs, the data was collected under three loading conditions (empty/ half load/ full load) . Some data was collected from engine on (i.e. Cold start)

6)	Test road : Non-motorway in the two biggest cities (Tokyo metropolitan area, Osaka) and Motorway

7)	Total number of data files : 163 files

8)	Total driving time : Approx. 300hrs

9)	Total driving distance : Approx. 9, 000 km

11)	Collected data : Time, Vehicle speed, Engine speed, Coolant temperature and Altitude

10)	Sampling frequency : 2 Hz

11)	Driver’s age/gender : Age: Unknown, Gender: All of them are male.

b.	Research for JC08 shift point

1)	Research agency : JARI

2)	Requested by : JAMA, MLIT

3)	Year : in 1999, 2001 and 2003

4)	Test vehicle : Total 12 vehicles

Passenger car : 6 vehicles, Light duty commercial vehicle : 6 vehicles

( see Table2. and Fig1. for more detail)

5)	Test condition : No load（in 1999: 2 drivers, in 2001 and 2003: 1 driver）for LDCV.

6)	Location :

- Tokyo, Osaka, Kyoto, Nagoya, Gifu, Shizuoka, Numazu in 1999

- Metropolitan area（Non-motorway and Motorways）in 2001/2003

7)	Total number of data files : 3,800files

8)	Total driving time : Approx. 1,400 hours

9)	Total driving distance : Approx. 39,000 km

10)	Collected data : Time, Vehicle speed, Engine speed and Clutch signal

11)	Sampling frequency : 10 Hz

Table 2 Test vehicles

Fig..1 Vehicle specification distribution

Table 3 Detail of currently available data

## 3.2. Test Vehicle

One (1) passenger car and one (1) light duty commercial vehicle

Since Japan have already collected the in-use driving data on variety of vehicle specifications and most of these data meet the criteria instructed by DHC guideline, the additional data collection will be conducted on average specification vehicle focusing on the driving conditions in where enough data is not available..

3.2.1. Vehicle selection

Plan to procure the test vehicles which possess the average specification.

(normalized vehicle weight and power to mass ratio, taking into account sales volume)

No. of production (vehicle units)

Normalized vehicle weight (-)

Power to mass ratio (kW/t)

Fig. 2 Normalized vehicle weight, Power to mass ratio and vehicle production

Source : FY2006, FY2008 Vehicle specification (JSAE) and JAMA data

Table 4 Weighted average value of normalized vehicle weight (Wn)

and the weighted average power to mass ratio (PMR) by vehicle type

Average spec. of PCs

Average spec. of LDCVs

Fig. 3 Vehicle specification distribution

3.2.2. Procurement

Rental car would be better considering road accident insurance and vehicle availability. However, in case of need for ECU information, it would be better to procure a test vehicle with ECU set from auto manufacturers.

3.2.3. Transmission

Sales volume ratio in 2008 : AT(CTV) 97.5％ (passenger car), 67.9％ (light duty commercial vehicle)

Therefore, the vehicles with automatic transmission will be first choice.

Table 5 AT sales volume ratio

3.2.4. Fuel type for LDCV

No big difference is observed between petrol LDCV(1.9million) and diesel LDVC (2.14million). Therefore, test vehicles are selected from rental car based on availability.

## 3.3. Test location and route selection

3.3.1. Road type definition

Table 6 Definition of road type

|  | Urban | Rural | Motorway |
| --- | --- | --- | --- |
| Suggested WLTP | Paved roads in urban areas with a speed limit ≤50 km/hour (exclude mountain areas) | Paved non-motorways outside and inside urban areas with a speed limit between 50 and 80 km/hour | Paved motorways (multi-lane roads specifically constructed and controlled for fast traffic) |
| JAPAN | ・Densely Inhabited District (DID) ・Speed limit≦60km/h ・exclude mountain areas | ・Non-Densely Inhabited District ・Non motorways ・exclude mountain areas | ・Motorways （within City and between Cities） ・exclude mountain areas |

(*) Photographic/video graphic evidence of roads are also provided.

(*) DID (Densely Inhabited District): neighboring area with population density 4,000/km² or more, and population of 5,000 or more.

■: DID ― : Motorway  ― : National road  ― : main district road

R25km

R25km

Tokyo area

Osaka area

Fig. 4 DID and Main roads in Tokyo and Osaka area

### 3.3.2. Driving routes and conditions

After re-categorized the current data into new defined road type and time period, the following points for route selection on each road type / time period need to be considered.

1)  Urban:

A plenty of data is available. In spite of lack of the weekend data in both vehicle categories compared with “Peak / Off peak” data, the distribution of average speed during weekend is fairly close to off-peak. No plan to collect the additional in-use data.

2)  Rural:

Data for LDCV with automatic transmission need to be collected to meet DHC guideline. Mostly the existing data was collected in rural area located close to DID region. New data collection needs to be conducted in more suburban area.

3)  Motorway:

A plenty of data is available. However, most of data was collected in metropolitan motorway. The additional data plans to be collected in inter-city motorway.

## 3.4. Driving condition

3.4.1. Driving behavior

No instruction is made other than to follow overall traffic flow.

3.4.2. Driver selection

Most existing data was collected in 30･40’s and very few in 20’s and 50･60’s.

- Needs consideration when collecting new data

- Driver’s age: 3 generation（20’s, 30 & 40’s and 50 & 60’s）

3.4.3. Season/weather conditions

Data collection will be terminated only when speed limit is intentionally reduced due to weather conditions (snow, heavy rain, etc)

3.4.4. Time of data collection

Data collection will be conducted on each test vehicle during the following period.

Table 7 Data collection plan

| Road type | Weekday On-peak | Weekday Off-peak | Weekend |
| --- | --- | --- | --- |
| Urban | No | No | No |
| Rural | Yes | Yes | Yes |
| Motorway | Yes | Yes | Yes |

- On-peak : 7 AM to 9 AM and 5 PM to 7 PM on weekdays

- Off-peak : rest of above hours on weekdays

- Weekend : Weekends (Sat & Sun) and Holidays

## 3.5. Amount of data to be collect

Table 8 shows the target amount of data to be collected on each test vehicle.

Test vehicles : One Passenger car and One Light duty commercial vehicle

Test period : Approx. 6 days per each vehicle category

Table 8 Detail for additional data collection

## 3.6. Measurement item

Table 9 Measurement items

| No. | Measurement items | Measurement methods | Notes |
| --- | --- | --- | --- |
| 1 | Vehicle speed | ・Drive shaft speed ・ECU ・ABS signal ・GPS | Must |
| 2 | Engine speed | ・Photoelectric Pick Up ・ECU | MT Must |
| 3 | Latitude | ・GPS |  |
| 4 | Longitude | ・GPS |  |
| 5 | Altitude | ・GPS ・GPS & Geographic information |  |
| 6 | Slope | ・Calculate from altitude data |  |
| 7 | Atmospheric pressure | ・Pressure sensor |  |
| 8 | Atmosphere temperature | ・Thermo couple |  |
| 9 | Water temperature | ・Thermo couple |  |
| 10 | Oil temperature | ・Thermo couple |  |
| 11 | Event Maker |  |  |
| - | Boost / Throttle angle | ・Pressure sensor / potential meter |  |
| - | Brake signal | ・Contact switch ・ECU（？） |  |
| - | Engine torque | ・ECU？（If measureable） |  |
| - | Clutch signal | ・ECU ・Contact switch |  |
| - | Gear position | ・Position sensor |  |

# 4. Statistical information

Japan will generate the weighting factors by using the traffic census data.

The following matrix will be filled out.

Table 10