| WLTP DTP subgroup |  |
| --- | --- |
| Date | 11 June 2010 |
| Title | Draft Nitrogen Dioxide (NO2) measurement procedure |
| Working paper number | WLTP-DTP-02-06 |

- Introduction

- These guidelines were developed following in response to discussions at the 1st DTP (Develop Test Procedure) meeting held in Ann Arbor in April 2010.  It was identified that a test procedure is required NO2.

- Once this proposal concept has been accepted by the DTP team, OICA will draft a proposal for the legislative text.

- Background

  - Reason for the requirement

Revised and challenging air quality standards for NO2 are in force since the beginning of 2010 in the European Union.  As road transport contributes around half of the ambient NO2, it is anticipated that a comparable means of determining the ratio of NO2 to the total NOx emissions will be required for the ongoing discussions.  It is therefore proposed to develop such a procedure during the WLTP project in order to ensure worldwide harmonisation from the outset.

  - Existing legislation

In developing this draft test procedure, it was appropriate to review existing legislation that measures NO2.  Although no test procedure is known for the measurement of vehicle emissions, there is an existing practice for air quality measurements and this should form the basis for a proposal, at least to ensure possible correlation of results.

- Measurement principle

- Two alternative analyser concepts are proposed, these being Chemi-Luminescent Analyser (CLA) or Ultra-Violet Resonance Absorption (UV-RAS).

- Contrary to the majority of today’s light duty measurement procedures, and due to the uncertainty of NO to NO2 ratios over time and the solubility of NO2 in water, it is proposed to measure NO2 directly rather than from CVS bags.

- As NO2 is seen as a pollutant emission, and therefore is expected to have the same dimension as NOx (unit mass per unit of distance), it is recommended to adopt the modal measurement process that is currently applied to FID hydrocarbon measurements in ECE R83.