| WLTP DTP subgroup |  |
| --- | --- |
| Date | 11 June 2010 |
| Title | Draft Ammonia (NH3) measurement procedure |
| Working paper number | WLTP-DTP-02-07 |

- Introduction

- These guidelines were developed following in response to discussions at the 1st DTP (Develop Test Procedure) meeting held in Ann Arbor in April 2010.  It was identified that a test procedure is required for the measurement of NH3.  Once this proposal concept has been accepted by the DTP team, OICA will draft a proposal for the legislative text.

- Background

  - Reason for the requirement

The measurement of NH3 in the exhaust gas of vehicles with SCR systems is a possible indicator that the reagent is not being overdosed and hence that the system is in control.

  - Existing legislation

In developing this draft test procedure, it was appropriate to review existing legislation that measures NH3.  Although not yet adopted as legislation, the coming Euro VI regulation (for heavy duty vehicles and engines) from the EU will contain a test procedure for measurement of NH3 and this has formed the basis for this proposal.

- Measurement principle

- Two alternative analyser concepts are proposed as anticipated in the Euro VI legislation, these being Laser Diode Spectrometer (LDS) or Fourier Transform Infra-Red (FTIR).

- Contrary to the majority of today’s light duty measurement procedures, and due to the high reactivity and solubility of NH3 with water, it is proposed to measure NH3 directly rather than from CVS bags.

- As NH3 is seen as a control measure rather than a pollutant emission, and therefore is expected to have a volumetric concentration limit, it is recommended to adopt a simple averaging process as foreseen in the Euro VI legislation.