OICA proposal for an example of a phase-in process of AEBS

| Step (NT/AT) | Technical requirement | Target | Subject vehicle speed | Target vehicle speed | Minimum speed reduction for the subject vehicle | Latest time for 1st warning before EB start | Latest time for 2nd warning before EB start |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Step 1 (2013/ 2015) | A (M3/N3) | Moving | [80±2km/h] | [30]＋4/–0km/h | Not less than [50]km/h | [1.4] s | [0.8] s |
| Step2 [?/?] | B (M3/N3) | Moving | [80±2km/h] | [10]＋4/–0km/h | Not less than [70]km/h | [1.4/2.0] s | [0.8] s |
|  | C (M3/N3) | Stationary | [80±2km/h] | Stationary | Not less than [20]km/h | [0.8] s | [0.8] s |
| Step3 [?/?] | D (M2/N2) | Moving | [80±2km/h] | [10]＋4/–0km/h | [8/10/12] Km/h | [0.8] s | [0.8] s |
|  | E (M2/N2) | Stationary | [80±2km/h] | Stationary | [8/10] Km/h | [0.8] s | [0.8] s |

Note :

- This table shows an example of a phase-in introduction of AEBS

- Figures and values are subject to revision by OICA

- Introduction steps and technical requirements are linked together.