|  | United Nations | ECE/TRANS/WP.29/GRRF/2010/20 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 12 July 2010 Original: English English and French only |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Sixty-eighth session

Geneva, 21–23 September 2010

Item 5 of the provisional agenda

Regulation No. 90 (Replacement brake linings)

Proposal for amendments to document ECE/TRANS/WP.29/GRRF/2009/23/Rev.2

Submitted by the experts from the European Association of Automotive Suppliers and the International Organization of Motor Vehicle Manufacturers *

The text reproduced below was prepared by the experts from the European Association of Automotive Suppliers (CLEPA) and the International Organization of Motor Vehicle Manufacturers (OICA) to clarify the provisions of ECE/TRANS/WP.29/GRRF/2009/23/Rev.2. This document is based on Informal document No. GRRF-67-24 distributed at the sixty-seventh session of the Working Party on Brakes and Running Gear (GRRF) (see report ECE/TRANS/WP.29/GRRF/67, para. 23). Modifications to ECE/TRANS/WP.29/2009/23/Rev.2 are marked in bold for new characters.

I.	Proposal

Paragraph 5.3.6.2.2.1., amend to read:

“5.3.6.2.2.1.	Test group relating to the tests stipulated in paragraphs 1 to 4 of Annex 11 or Annex 12.

Brake drums may be grouped together as test groups on the basis that each permitted test group range is from the smallest inside diameter to the smallest inside diameter +10 per cent and by no more than 40mm on the rubbed width of the drum.”

Paragraph 6.2.1., amend to read:

“6.2.1.	Every unit sold shall be provided in some form with at least the following information:”

Paragraph 6.2.1.2., amend to read:

“6.2.1.2.	in the case of motor vehicles:

the principal make, type and trade name of the vehicle, the axle intended to be fitted and period of manufacturing of the vehicle; should the period of manufacture not be readily available a reference to the original part number/identification code may be used;”

Annex 11

The heading of paragraph  4.1.1.1.1., amend to read:

“4.1.1.1.1.	Test conditions (brake disc thermal fatigue test)”

Paragraph 4.1.1.1.2., amend to read:

“4.1.1.1.2.	Test program (brake disc thermal fatigue test)”

Table A11/4.1.1.1.2.

| Test provision | Thermal fatigue test |
| --- | --- |
| Vehicle categories | M1 / N1 |
| Type of braking | Sequential brake applications |
| Braking interval (= ttotal) | 70 s |
| Number of brake applications per cycle | 2 |
| Brake torque in accordance with a deceleration of | 5.0 m/s² |
| Total number of braking cycles | 100 or 150 (see 4.1.1.1.3.) |
| Brake applications from to | vmax 20 km/h |
| Initial temperature of the 1st brake application in each cycle |  100 °C |

vmax	the vmax to be used to test the replacement part is that corresponding to the vehicle which has the highest ratio of kinetic energy to disc mass

tbra	actual braking period during the application

tacc	minimum acceleration time in accordance with the accelerating power of the respective vehicle

trest	rest period

ttotal	Braking interval (tbra + tacc + trest)”

The heading of paragraphs 4.1.1.2.1., 4.1.1.2.2. and 4.1.1.2.3., amend to read:

“4.1.1.2.1.	Test conditions (brake disc thermal fatigue test)

4.1.1.2.2.	Test program (brake disc thermal fatigue test)

4.1.1.2.3.	Test result (brake disc thermal fatigue test)”

Paragraph 4.1.2.1.2., amend to read:

“4.1.2.1.2.	Test program (brake disc high load test)

The test has to be carried out according to the following table:

Table A11/4.1.2.1.2.

| Test provision | High load test |
| --- | --- |
| Vehicle categories | M1 / N1 |
| Type of braking | Single brake applications |
| Number of brake applications | 70 |
| Initial temperature at the beginning of braking |  100 °C |
| Brake torque in accordance with a deceleration of | 10.0 m/s² (however, brake pressure p ≤ 16000 kPa) |
| Brake applications from to | vmax 10 km/h |

vmax	the vmax to be used to test the replacement part is that corresponding to the vehicle which has the highest ratio of kinetic energy to disc mass”

Paragraphs  4.1.2.2.2., amend to read:

“4.1.2.2.2.	Test program (brake disc high load test)

500 brake applications are carried out from a speed of 50 km/h to 10 km/h with a brake torque corresponding to a deceleration of 6 m/s2.

Initial temperature: ≤ 200 °C”

The heading of paragraphs 4.2.1.1.1., 4.2.1.1.2., 4.2.1.1.3. and 4.2.1.2.1., amend to read:

“4.2.1.1.1.	Test conditions (brake drum thermal fatigue test)

4.2.1.1.2.	Test program (brake drum thermal fatigue test)

4.2.1.1.3.	Test result (brake drum thermal fatigue test)

4.2.1.2.1.	Test conditions (brake drum thermal fatigue test)”

Paragraph 4.2.1.2.2., amend to read:

“4.2.1.2.2.	Test program (brake drum thermal fatigue test)

Table A11/4.2.1.2.2.

| Test provision | Thermal fatigue test |
| --- | --- |
| Type of braking | Sequential brake applications |
| Number of brake applications | 250 or 300 (whichever is applicable) - see paragraph 4.2.1.2.3. NB: The test is interrupted when a through crack appears. |
| Brake torque in accordance with a deceleration of | 3.0 m/s2 |
| Brake applications from to | 130 80 km/h |
| Initial temperature of each brake application |  50 °C |
| Cooling pursuant to paragraph 3.2.3. | permitted |

”               The heading of paragraph 4.2.1.2.3., amend to read:

“4.2.1.2.3.	Test result (brake drum thermal fatigue test)”

Paragraph 4.2.2.1.2., amend to read:

“4.2.2.1.2.	Test program (brake drum high load and thermal fatigue test)

The test has to be carried out according to the following table:

Table A11/4.2.2.1.2.

| "Bedding in" procedure | Make 100 consecutive snub applications with v1 = 80 km/h and v2 = 10 km/h and an initial temperature of ≤ 100 °C. The deceleration of the first application shall be constant 1.5 m/s2. From the second up to the last application the pressure shall be constant and equivalent to the average of the first application. The bedding should be continued until a minimum of 80 per cent lining to drum contact is achieved. |
| --- | --- |
| Test provision | Brake drum high load test |
| Type of braking | Single brake applications |
| Number of brake applications | 100 |
| Initial temperature at the beginning of braking |  100 °C |
| Brake torque in accordance with a deceleration of | 10.0 m/s² (however, brake pressure p  16000 kPa) |
| Brake applications from to | vmax 10 km/h |

vmax	the vmax to be used to test the replacement part is that corresponding to the vehicle which has the highest ratio of kinetic energy to disc mass”

Paragraph 4.2.2.2.2., amend to read:

“4.2.2.2.2.	Test program (brake drum high load test)

Table A11/4.2.2.2.2.

| Test provision | High load test |
| --- | --- |
| Type of braking | Braking to less than 5 km/h |
| Total number of brake applications | 150 |
| Initial brake drum temperature at each brake application |  100 °C |
| Brake applications from to | 60 km/h  5 km/h |
| Brake torque in accordance with a deceleration of | 6 m/s² |
| Cooling (also deviating from paragraph 3.2.3. of this annex) | Permitted |

”

Annex 12

Paragraph 4.1.1.1.2., amend to read:

“4.1.1.1.2.	Test program (brake disc thermal fatigue test)”

Table A12/4.1.1.1.2.

| Test provision | Thermal fatigue test |
| --- | --- |
| Vehicle categories | O1 / O2 |
| Type of braking | Sequential brake applications |
| Braking interval (= ttotal) | 70 s |
| Number of brake applications per cycle | 2 |
| Brake torque in accordance with a deceleration of | 5.0 m/s2 |
| Total number of braking cycles | 100 or 150 (see paragraph 4.1.1.1.3.) |
| Brake applications from to | 80 km/h 20 km/h |
| Initial temperature of the 1st brake application in each cycle |  100 °C |

vmax	maximum design speed (as per its range of use)

tbra	actual braking period during the application

tacc	minimum acceleration time in accordance with the accelerating power of the respective vehicle

trest	rest period

ttotal	Braking interval (tbra + tacc + trest)”

The heading of paragraphs 4.1.1.1.3., 4.1.1.2.1, 4.1.1.2.2. and 4.1.1.2.3., amend to read:

“4.1.1.1.3.	Test result (brake disc thermal fatigue test)

4.1.1.2.1.	Test condtions (brake disc thermal fatigue test)

4.1.1.2.2.	Test program (brake disc thermal fatigue test)

4.1.1.2.3.	Test result (brake disc thermal fatigue test)”

Paragraph 4.1.2.1., amend to read:

“4.1.2.1.	Vehicles of categories O1 and O2

This test is conducted using a new disc, an original brake caliper of the vehicle(s) concerned and new brake lining assemblies of the vehicle(s) concerned which have been type approved according to Regulations No. 13, 13-H or 90 (in the condition as mounted on the vehicle, e.g. protective grease removed).

Worn brake linings may be replaced during the test if necessary.”

Insert new paragraphs 4.1.2.1.1. to 4.1.2.1.3., to read:

“4.1.2.1.1.	Test conditions (brake disc high load test)

See paragraph 4.1.1.1.1. above.

4.1.2.1.2.	Test program (brake disc high load test)

The test has to be carried out according to the following table:

Table A12/4.1.2.1.2.

| Test provision | High load test |
| --- | --- |
| Vehicle categories | O1/O2 |
| Type of braking | Single brake applications |
| Number of brake applications | 70 |
| Initial temperature at the beginning of braking |  100 °C |
| Brake torque in accordance with a deceleration of | 10.0 m/s² (however, brake pressure p  16 000 kPa) |
| Brake applications from to | 80 10 km/h |

4.1.2.1.3.	Test result (brake disc high load test)

The test is regarded as having been passed if 70 or more brake applications are completed without damage or failure.

If less than 70 brake applications are completed before damage or failure then a test should be conducted on the original part and the results compared.  If the damage or failure point is no worse than the number of cycles of the original part -10 per cent then the test is regarded as having been passed.

Damage in this context means:

(a)	Radial cracks on the friction surfaces which are longer than two thirds of the radial height of the friction surface.

(b)	Cracks on the friction surface which reach the inner or outer diameter of the friction surface.

(c)	Through-cracking of any friction ring.

(d)	Any type of structural damage or cracks in any area outside the friction surface.”

Paragraph 4.1.2.2.2., amend to read:

“4.1.2.2.2.	Test program (brake disc high load test)

500 brake applications are carried out from a speed of 50 km/h to 10 km/h with a brake torque corresponding to a deceleration of 6 m/s2.

Initial temperature: ≤ 200 °C”

Paragraph 4.2.1.1.1., amend to read:

“4.2.1.1.1.	Test conditions (brake drum thermal fatigue test)”

Paragraph 4.2.1.1.2., amend to read:

“4.2.1.1.2.	Test program (brake drum thermal fatigue test)

Table A12/4.2.1.1.2.

| Test provision | Thermal fatigue test |
| --- | --- |
| Type of braking | Sequential brake applications |
| Number of brake applications | 250 or 300 (whichever is applicable) - see 4.2.1.1.3. NB: The test is interrupted when a through crack appears. |
| Brake torque in accordance with a deceleration of | 3.0 m/s2 |
| Brake applications from to | 130 80 km/h |
| Initial temperature of each brake application |  50 °C |
| Cooling pursuant to paragraph 3.2.3. | Permitted |

”

Paragraph 4.2.1.1.3., amend to read:

“4.2.1.1.3.	Test result (brake drum thermal fatigue test)

The test is regarded as having been passed if 300 or more brake applications are completed without damage or failure.

If less than 300 brake applications but more than 250 brake applications are completed without damage or failure then the Technical Service must repeat the test on a new replacement part.  Under these circumstances both tests must complete more than 250 brake applications without damage or failure for the part to have passed the test.

If less than 300 brake applications are completed before damage or failure then a test should be conducted on the original part and the results compared. If the damage or failure point is no worse than the original part then the test is regarded as having been passed.

Damage in this context means:

(a)	Cracks on the friction surface which are longer than two-thirds of the axial width of the friction surface.

(b)	Cracks on the friction surface which reach the axial outer end of the drum.

(c)	Through-cracking of the drum.

(d)	Any type of structural damage or cracks in any area outside the friction surface.”

Paragraph 4.2.1.2.1., amend to read:

“4.2.1.2.1.	Test conditions (brake drum thermal fatigue test)”

Paragraph 4.2.1.2.2., amend to read:

“4.2.1.2.2.	Test program (brake drum thermal fatigue test)

Table A12/4.2.1.2.2.

| Test provision | Thermal fatigue test |
| --- | --- |
| Type of braking | Sequential brake applications |
| Number of brake applications | 250 or 300 (whichever is applicable) - see 4.2.1.2.3. NB: The test is interrupted when a through crack appears. |
| Brake torque in accordance with a deceleration of | 3.0 m/s2 |
| Brake applications from to | 130 80 km/h |
| Initial temperature of each brake application |  50 °C |
| Cooling pursuant to paragraph 3.2.3. | Permitted |

”

The heading of paragraph 4.2.1.2.3., amend to read:

“4.2.1.2.3.	Test result (brake drum thermal fatigue test)”

Paragraph 4.2.2.1.1., amend to read:

“4.2.2.1.1.	Test conditions (brake drum high load test)

The inertia mass of the inertia dynamometer shall be determined in accordance with the requirements laid down in paragraphs 3.2.1., 3.2.1.1. and 3.2.1.2. of Annex 12.

The rotational speed of the dynamometer shall correspond to the linear test speed of the vehicle based on the mean of the largest and smallest dynamic rolling radius of the tyres authorized for that vehicle.”

Paragraph 4.2.2.2.2., amend to read:

“4.2.2.2.2.	Test program (brake drum high load test)

Table A12/4.2.2.2.2.

| Test provision | High load test |
| --- | --- |
| Type of braking | Braking to less than 5 km/h |
| Total number of brake applications | 150 |
| Initial brake drum temperature at each brake application |  100 °C |
| Brake applications from to | 60 km/h 0 km/h |
| Brake torque in accordance with a deceleration of | 6 m/s² |
| Cooling (also deviating from paragraph 3.2.3. of this annex | Permitted |

”

Annex 13, add the items., to read */:

“Approval number 2/:

Approval number 2/:		”

II.	Justification

1.	A number of changes in the document are editorial to improve the consistency of wording and to bring clarity throughout the document.

2.	A change in wording has been made to paragraph 5.3.6.2.2.1 to clarify what is permitted as a brake drum test group.

3.	In paragraph 6.2.1, the wording has been changed to allow some latitude in how the required information is provided at the point of sale as the current wording does not deliver a practical “real world” solution for brake discs and drums.

4.	In Annex 12, paragraph 4.1.2, a test is added for the high load testing of brake discs for O1 and O2 vehicles; this was missing in the first draft as these applications are currently few in number.

5.	In Annex 12, paragraph 4.2.1.1.3, changes are made to the brake drum thermal fatigue test to bring the test in line with the same type of testing on other categories of vehicles.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2006–2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

*	*/	Note by the secretariat: A number should be assigned to these items.