Comments from India on the GTR for Passenger Vehicle Tyres under formulation by the Working Group on Tyres

The following are the comments on Tyre Document gtr-09-02e Draft, 10th June 2010 document.

| Clause 3. Definition |  |
| --- | --- |
| Proposed change | It is necessary to define the "Maximum Inflation Pressure" for tyre which has significance similar to Maximum application Load capacity & Maximum Load Rating. The proposed definition is, "Maximum Inflation Pressure : This is the maximum cold inflation pressure permitted in tyre and it is recommended that the cold inflation pressures be limited to 320 kPa for sizes having a speed symbol upto T, to 350 kPa for sizes having speed symbol H, V, W or Y, Reinforced tyres and ZR marked tyres.” |
| Justification | Vehicle camber angles, especially under severe driving conditions, have an influence on tyre performance. In order to counteract this influence, it may be necessary to increase the inflation pressure by a factor Ks (camber factor) as defined in Engineering Design Information of ETRTO or reduce the load. For a given size, the calculated inflation pressure shall not exceed the maximum inflation pressure under such situation. |
| Clause 3. Definition – Principal grooves |  |
| Comments | Does this definition limit the positioning of TWIs only in principal grooves ? |
| Clause 3. Definition – PSI Index |  |
| Comments | Table reference not given. Add "as per appendix 4" |
| Justification | It will be easy to locate the PSI index . |
| Clause 3. Definition – Tread Wear Indicators (TWI) |  |
| Proposed change | May be included at the end of note - "Beyond which tyre is rendered un-usable and shall be removed or retreaded" |
| Clause 4. Requirements |  |
| Comments on Clause 4.1 | For plant code registration the detailed procedure & registering site address to be provided. |
| Justification | For example - The current system calls for presence of an agent in US for non US companies. |
| Comments on Clause 4.2.2.9 | The inscription of word “XL” or “Reinforce” or “Extra Load” should be kept |
| Justification | To provide an option for wide range of products already using these words. |
| Comments on Clause 4.2.2.16 | “The suffix “CP” after the rim diameter marking referred to in paragraph Appendix 3…” is not reflecting anywhere. |
| Justification | “CP” as per ETRTO is – Commercial Vehicle tires for service on motor caravans |
| Clause 4.4 Physical Dimensions of Radial Pneumatic Tyres |  |
| Comments on Clause 4.4.4.2 | Instead of “adjust the pressure to that specified by the manufacturer”, it should be specified , for example, as specified for tyre strength testing in tabular format for standard & reinforced tyres. (eg Section 3.3 of ETRTO PC tyres) |
| Justification | Otherwise the dimensional consistency would not be there. |
| Comments on Clause 4.4.5.4 | Instead of including “Bureau of Indian Standards (BIS)” , please include Indian Tyre Technical Advisory Committee (ITTAC)” . |
| Justification | BIS is a regulatory standard and is governed by government bodies, whereas ITTAC is committee for Indian tyre standards like ETRTO, JATMA,T&RA etc. |
| Clause 4.5 Strength Test |  |
| Comments on Clause 4.5.1 and 4.5.2 | The pressures specified in the tables against size designation & “Test Type” are referring to which pressure? Also it is not including the pressure like 250 & 290 kPa marked on tyre (in line with ETRTO) |

| Strength Test | Passenger Tyres |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | kPa |  |  |  |  |
| Inflation pressure marked on the tyre | 240/250 | 280/290 | 300 | 340 | 350 |
| Test inflation | 180 | 220 | 180 | 220 | 180 |

| Justification | It is very essential to ‘clearly identify’ what is marked on tyre and what needs to be followed. |
| --- | --- |
| Clause 4.6 Tubeless Tyre Bead Unseating Resistance Test |  |
| Comments on Clause 4.6.2.2 | The pressures specified in the tables against “Test Type” are referring to which pressure ? Also it is not including the pressure like 250 & 290 kPa marked on tyre |

| Bead Unseat Test |  | Passenger Tyres |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | kPa |  |  |  |  |
| Inflation pressure marked on the tyre |  | 240/250 | 280/290 | 300 | 340 | 350 |
| Test inflation |  | 180 | 220 | 180 | 220 | 180 |
| Justification | It is very essential to clarify between what is “marked on tyre” and what needs to be followed. |  |  |  |  |  |
| Section II Clause 4.8, 4.9 & 4.10 Endurance Test, Low Inflation Pressure Performance Test, Hi-speed Performance Test |  |  |  |  |  |  |
| Comments on Clause 4.8.3.1, 4.9.3.2 & 4.10.3.1 | Instead of removing testing on 2.0 m drum it should be kept along with 1.7m as an additional option |  |  |  |  |  |
| Justification | Many manufacturers have 2.0 m drums for testing and it would be better to have optional standards for 2.0 m drums also. |  |  |  |  |  |
| Comment on clause 4.8.3.2, 4.9.3.5 & 4.10.3.6 | During the test the ambient temperature shall be at least 38 deg C. No tolerance or max. limit specified. It may be 38±3°C. |  |  |  |  |  |
| Justification | It is always better to have tolerances to avoid inconsistency in the results. It can be specified similar to clause 4.12.1.7 |  |  |  |  |  |
| Clause 4.11 Tyre Wet Grip Test |  |  |  |  |  |  |
| Comment on clause 4.11.2.1 | In the formula for measured value corrected for effects of temperature pbfc = pbfc (measure)+0.003 . 5(t-20) The “Dot” shown between 0.003 & 5 is not clear, as whether it means a multiplication or else. |  |  |  |  |  |
| Comments on clause 4.11.2.2 | Clause number typographical error as 14.11.2.2, instead of 14 it should be 4 |  |  |  |  |  |