|  | United Nations | ECE/TRANS/WP.29/GRSG/2010/31 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 27 July 2010 Original: English English and French only |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on General Safety Provisions

Ninety-ninth session

Geneva, 19–22 October 2010

Item 12(b) of the provisional agenda

Global technical regulations under the 1998 Agreement - 
Proposal to amend gtr No. 6 (Safety glazing)

Proposal for Amendment 1 to global technical regulation No. 6 (Safety glazing)

Submitted by the expert from the European Commission*

The text reproduced below was prepared by the expert from the European Commission to amend global technical regulation (gtr) No. 6 with regards to markings following the request of the Administrative Committee of the 1998 Agreement (ECE/TRANS/WP.29/AC.3/27). The modifications to the current text of the gtr are marked in bold for new or strikethrough for deleted characters.

Proposal

Paragraphs 4.1.2.1. to 4.1.2.3.3, amend to read:

“4.1.2.1.	Identification marks for windscreens.

4.1.2.1.1.	“II” for laminated-glass.

4.1.2.1.2.	“III” “IV” for glass-plastics.

4.1.2.2.	Identification marks for panes.

4.1.2.2.1.	“I” No symbol is required for uniformly toughened glass.

4.1.2.2.2.	“IV” “XI” for laminated glass.

4.1.2.2.3.	“V” “VI” for a double glazed unit.

4.1.2.2.4.	“VI” for glass-plastics.

4.1.2.3.	Additional identification marks.

4.1.2.3.1.	Glazing faced with plastic shall be marked with “/P” after the mark required by paragraph 4.1.2.1. or 4.1.2.2., e.g., II/P.

4.1.2.3.2.	Glazing with a light transmission of less than 70 per cent shall be marked with "/RLT" “/V” after the mark required by paragraph 4.1.2.2., e.g. I/RLT. *

4.1.2.3.3.	If glazing requires both the "/P" and the "/RLT" “/V”, the mark required by paragraph 4.1.2.3.1 shall precede the mark required by paragraph 4.1.2.3.2."

*	*	In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.

*	*	Note by the secretariat: The marking proposed for glazing with a light transmission of less than 70 per cent does not seem to be aligned with that of Regulation No. 43. Furthermore, the example is not consistent with the marking proposed.