Head restraint gtr Phase2 Status and Open issues

Hiroyuki Asada,　JASIC/Japan

Status and Schedule of Gtr No.7 Phase2 Informal WG & BioRID II TEG

|  | Dummy Inspection | Component Testing | Sled Testing | Deliverable timing |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Step1 by May 2011 | Step2 by TBD |
| Head restraint height | A new method was proposed by Netherlands, who are studying a new concept in response to comments received from the informal group. |  |  | Aug. 2010 |  |
| Sled Pulse | Delta V=16 or 18km/h was proposed by Japan |  |  | Sept. 2010 |  |
| Evaluation indicators | - NIC, Upper & Lower FX, FZ, MY were proposed by Japan. - Dynamic backset was proposed by EEVC |  |  | Sept. 2010 |  |
| Reference value | - Dynamic backset criteria proposed by Japan |  |  | Sept. 2010 |  |
| Drawing Package For HRMD | New HRMD and calibration tool were proposed by VDA but further discussions are taking place between VDA and SAE |  |  | Sept. 2010 |  |
| Drawing Package for BioRID II | Drawing package (2D & 3D) are being prepared by the BioRid manufacturer(s). |  |  | Draft by July 2010 | Upright version |
| Certification Procedure | Proposal Completed | Proposal is modifying | Under Discussion | Draft by Sept. 2010 | Upright version |
| Durability | Korea experienced neck bumper issues when trialing the revised calibration procedure. FORD identified a need for a body block on the calibration sled to avoid dummy damage. |  |  | By July 2010 |  |
| Repeatability & Reproducibility |  |  | Japan reported new result | Understand issue by Sept. 2010 | Upright version |
| Biofidelity | Japan & EEVC: Completed U.S: Under Discussion | Japan & EEVC: Completed U.S: Under Discussion | Japan & EEVC: Completed U.S: Under Discussion | Standard position be Completed If US result will be different by Oct. 2010, keep HIII as Contracting Party option. | Upright version |
| PADI | Proposal is being prepared by dummy manufacturer. |  |  | Complete Draft by Sept. 2010 | Upright version |
| Seating procedure for standard seat | - Japan proposed procedure with design torso angle about [20] degree and higher, will study R&R. - Phase1 request fix 25 degree. |  |  | Review for use of limit and R&R with design torso angel by Sept. 2010 |  |
| Seating procedure for upright seat | . PDB and Japan are studying upright torso tool and have identified some issues. . Biomechanical reference for the alternative setting comb has to be established. |  |  | Review volume of production by Sept. 2010. | Maybe postpone |
| Mid or High Speed Dynamic test | . US studies of candidate dummies are ongoing. . US are studying moderate delta V (around [24km/h]). |  |  |  | Propose end of Oct. 2010. |

Next TEG web meeting on June 22, and informal group meeting Sept [20th], 21st and 22nd  at Berlin, Germany.