Proposed amendments to GRSP/2010/8:

Paragraph 5.2.4.5., amend to read:

“5.2.4.5.	The ISOFIX top tether anchorage shall have dimensions to permit the attachment of an ISOFIX top tether hook as specified in figure 3.

Clearance shall be provided around each ISOFIX top tether anchorage to allow latching and unlatching to it.

All tether anchorages located rearward of any ISOFIX low anchorage shall be designed to prevent misuse by one or more of the following measures

(a)	Designing all anchorages in the ISOFIX top tether zone area as ISOFIX top tether anchorages or

(b)	Marking only the ISOFIX top tether anchorage points using one of the symbols, or its mirror image, as set out in figure 13 of Annex 9 or

(c)	Marking anchorage points not in accordance with (a) or (b) above to provide a clear indication that these anchorages should not be used in combination with ISOFIX lower anchorages.

For each ISOFIX top tether anchorage under a cover, the cover shall be identified by for example one of the symbols or the mirror image of one of the symbols set out in figure 13 of Annex 9; the cover shall be removable without the use of tools.