|  | United Nations | ECE/TRANS/WP.11/2011/9 |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 27 July 2011 Original: English |

Economic Commission for Europe

Inland Transport Committee

Working Party on the Transport of Perishable Foodstuffs

Sixty-seventh session

Geneva, 25-28 October 2011

Item 5 (a) of the provisional agenda

Proposals of amendments to the ATP: Pending proposals

K coefficient of equipment in service

Transmitted by the German Government

I.	Initial situation/Justification

1.	Section 5 of Annex 1, Appendix 2 of the ATP specifies the two different testing procedures that the competent authority may apply to test equipment in service: measurement of the K coefficient in accordance with paragraphs 2.1.1 to 2.3.2 or assessment of the equipment by an expert on the basis of a visual inspection and technical expertise.

2.	Further details are given in subsection 5.1. Subsection 5.3 "Decisions" states that the results of the measurement of the K coefficient must be "satisfactory". The equipment may then be kept in service for a further period of six years in its initial class. If the expert’s conclusions after the visual inspection are favourable, the equipment may be kept in service for a further period of three years in its initial class. At present, the ATP contains no further definition of the term "satisfactory".

3.	Moreover, the initial K coefficient (e.g. from the type test) of 0.40 W/m2.K or 0.70 W/m2.K at the time of the equipment’s entry into service may not be used as a reference value given that the insulation of the equipment is subject to a normal physical aging process. For the majority of semi-trailers, the initial K coefficient of the type test lies between 0.35 W/m2.K and 0.40 W/m2.K.

4.	In order to ensure equitable conditions between transport operators and their safe operation in ATP member countries, it is suggested that the K values should be fixed after 6 and 9 years.

II.	Proposal

5.	It is proposed to add the following text and table at the end of subsection 5.3 (i):

"The results shall be deemed to be satisfactory if the following values for the K coefficient are not exceeded by equipment in service:

| Initial class | IR | IN |
| --- | --- | --- |
| Years in service | K coefficient (≤) [W/m².K] | K coefficient (≤) [W/m².K] |
| 6 | 0.50 | 0.90 |
| 9 | 0.60 | 1.10 |

?	Submitted in accordance with the programme of work of the Inland Transport Committee for 
2010–2014 (ECE/TRANS/208, para. 106; ECE/TRANS/2010/8, programme activity 02.11).