Amending ECE/TRANS/WP.29/GRB/2011/11

I.	Proposal

Annex 6

Paragraph 3.5., amend to read:

"3.5.	Duration and speed.

When the deceleration method is selected, the following requirements apply:

(a)	The deceleration j shall be determined in exact dω/dt or approximate Δω/Δt form, where ω is angular velocity, t – time

(b)	For duration Δt, the time increments shall not exceed 0.5 s;

(c)	Any variation of the test drum speed shall not exceed 1 km/h within one time increment."

Paragraph 4.6.2., amend to read:

"4.6.2.	Deceleration method

The deceleration method follows the procedure below:

(a)	Remove the tyre from the test surface;

(b)	Record the deceleration of the test drum Do/ t and that of the unloaded tyre T0/ t3 or record the deceleration of the test drum jD0 and that of the unloaded tyre jT0 in exact or approximate form in accordance with paragraph 3.5."

Paragraph 5.1.5., amend to read:

"5.1.5.	Deceleration method

Calculate the parasitic losses Fpl, in newton.

| Where: |  |  |
| --- | --- | --- |
| ID |  | is the test drum inertia in rotation, in kilogram meter squared, |
| R |  | is the test drum surface radius, in meter, |
| D0 |  | is the test drum angular speed, without tyre, in radians per second, |
| t0 |  | is the time increment chosen for the measurement of the parasitic losses without tyre, in second, |
| IT |  | is the spindle, tyre and wheel inertia in rotation, in kilogram meter squared, |
| R |  | is the tyre rolling radius, in metre, |
| T0 |  | is the tyre angular speed, unloaded tyre, in radian per second. |

Or

Fpl=IDRjD0+ITRrjT0

where:

| ID |  | is the test drum inertia in rotation, in kilogram meter squared; |
| --- | --- | --- |
| R |  | is the test drum surface radius, in meter; |
| jD0 |  | is the deceleration of the test drum, without tyre, in radians per second squared; |
| IT |  | is the spindle, tyre and wheel inertia in rotation, in kilogram meter squared; |
| Rr |  | is the tyre rolling radius, in metre; |
| jT0 |  | is the deceleration of unloaded tyre, in radians per second squared." |

Paragraph 5.2.5., amend to read:

"5.2.5.	Deceleration method

The rolling resistance Fr, in newton, is calculated using the equation:

| Where: |  |  |
| --- | --- | --- |
| ID |  | is the test drum inertia in rotation, in kilogram metre squared, |
| R |  | is the test drum surface radius, in meter, |
| Fpl |  | represents the parasitic losses as calculated in paragraph 5.1.5., |
| tv |  | is the time increment chosen for measurement, in second, |
| Δv |  | is the test drum angular speed increment, without tyre, in radian per second, |
| IT |  | is the spindle, tyre and wheel inertia in rotation, in kilogram metre squared, |
| Rr |  | is the tyre rolling radius, in metre, |
| Fr |  | is the rolling resistance, in newton. |

Or

Fr=IDRjV+RITRr2jV−Fpl

where:

| Where: |  |  |
| --- | --- | --- |
| ID |  | is the test drum inertia in rotation, in kilogram metre squared, |
| R |  | is the test drum surface radius, in meter, |
| Fpl |  | represents the parasitic losses as calculated in paragraph 5.1.5., |
| jV |  | is the deceleration of the test drum, in radians per second squared, |
| IT |  | is the spindle, tyre and wheel inertia in rotation, in kilogram metre squared, |
| Rr |  | is the tyre rolling radius, in metre, |
| Fr |  | is the rolling resistance, in newton." |

Annex 6, Appendix 1

Paragraph 4, amend to read:

"4.	Control accuracy

…

(d)	time: +/- 0.02 s 0.5 ms

…"