LNG TF-03-02

File:input.doc

## DRAFT

## GUIDELINE  for  Regulation No. 110

Uniform provisions concerning the approval of:

I.	approval of Specific components of motor vehicles using CNG/LNG /2. in their propulsion system;

II.	approval of Vehicles with regard to the installation of specific components of an approved type for the use OF CNG/LNG /2. in their propulsion system

CONTENTS

REGULATION

1.	Scope (CNG/LNG /2.)

2.	Definition and classification of  CNG/LNG /2. components

2.3	CNG components definitions

2.30	LNG components definitions

START

In contact with gas

Cryogenic gas

-40°C

450<p<3000

Safety valve

Class 1

Class 3

Class 2

Class 5

Class 4

Class 0

Not subject to this Regulation

P0<26000

NO

NO

NO

NO

NO

NO

YES

YES

NO

YES

YES

YES

YES

YES

P>3000

20<p<450

Fig. 1-1  Flow scheme for CNG and LNG components classification

(Pressure in KPa)

| Performance Test | Over-pressure Strength Test | Leakage Test (external) | Leakage Test (internal) | Continued Operation Durability Test | Corrosion Resistance | Ozone Ageing | CNG Compatibility | Vibration Resistance | Dry - Heat Resistance | Low Temperature test (-40°C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Annex 5A | Annex 5B | Annex 5C | Annex 5L | Annex 5E | Annex 5G | Annex 5D | Annex 5N | Annex 5F | Annex 5P |
| Class 0 | X | X | A | A | X | X | X | X | X | O |
| Class 1 | X | X | A | A | X | X | X | X | X | O |
| Class 2 | X | X | A | A | X | A | X | X | A | O |
| Class 3 | X | X | A | A | X | X | X | X | X | O |
| Class 4 | O | O | O | O | X | A | X | O | A | O |
| Class 5 | X | X | X | X | X | X | X | X | X | X |

X   =	Applicable

O   =	Not applicable

A   =	As applicable

Fig. 1-2 - Tests  applicable  to  specific  classes  of  components

(excluding CNG cylinders and LNG cryogenic tank)

PART I

3.	Application for approval	 14

4.	Markings	 14

5.	Approval	 15

6.	Specifications regarding CNG components	 16

7.	Modifications of a type of CNG component and extension of approval	 18

8.	Specifications regarding LNG components	18

8.1     Modifications of a type of LNG components and extension of approval	18

9.	Conformity of production	 19

10.	Penalties for non-conformity of production	 19

11.	(Not allocated)	 19

12.	Production definitely discontinued	 19

13.	Names and addresses of Technical Services

responsible for conducting approval tests,

and of Administrative Departments	 20

PART II

14.	Definitions	 21

15.	Application for approval	 21

16.	Approval	 22

17.	Requirements for the installation of specific components for the use of compressed natural gas (CNG) in the propulsion system of a vehicle	24

17.1	Requirements for the installation of specific components for the use of liquefied natural gas (LNG) in the propulsion system of a vehicle	24

18.	Conformity of production	 30

19.	Penalties for non-conformity of production	 30

20.	Modification and extension of approval

of a vehicle type	 30

21.	Production definitely discontinued	 31

22.	Names and addresses of Technical Services responsible for conducting approval tests, and of Administrative Departments 	31

ANNEXES

Annex 1A	-	Essential characteristics of the CNG/LNG /2. component

Annex 1B	-	Essential characteristics of the vehicle, engine and CNG/LNG /2. -related system

Annex 2A	-	Arrangement of the CNG/LNG /2. component type-approval mark

Annex 2B	-	Communication concerning the approval or extension or refusal or withdrawal of approval or production definitely discontinued of a type of CNG/LNG /2. component pursuant to Regulation No. 110

Addendum	-	Additional information concerning the type-approval of a type of CNG/LNG /2. components pursuant to Regulation No. 110

Annex 2C	-	CNG/LNG /2. arrangement of approval marks

Annex 2D	-	Communication concerning the approval or extension or refusal or withdrawal of approval or production definitely discontinued of a vehicle type with regard to the installation of CNG/LNG /2. system pursuant to Regulation No. 110

Annex 3	-	Gas cylinders - High pressure cylinder for the on-board storage of natural gas as a fuel for automotive vehicles

Annex 3A	-	Cryogenic tanks for the on board storage of liquefied natural gas (LNG) as a fuel automotive vehicles, including their accessories

Annex 4A	-	Automatic valve, non-return valve, the pressure relief valve, pressure relief device and the excess flow valve, only for CNG applications

Annex 4B	-	Flexible fuel lines or hoses

Annex 4C	-	CNG filter

Annex 4D	-	CNG Pressure regulator

Annex 4E	-	CNG Pressure and temperature sensors

Annex 4F	-	CNG Filling unit

Annex 4G	-	CNG Gas flow adjuster and gas/air mixer or gas injector

Annex 4H	-	CNG/LNG /2. Electronic control unit

Annex 4I	-	LNG vaporizer

Annex 4L	-	LNG Filling unit

Annex 5	-	Test procedures

Annex 5A	-	CNG Overpressure test (Strength test)

Annex 5B	-	CNG External leakage test

Annex 5C	-	CNG Internal leakage test

Annex 5D	-	NG compatibility test

Annex 5E	-	CNG Corrosion resistance test

Annex 5F	-	CNG Resistance to dry heat

Annex 5G	-	CNG Ozone ageing

Annex 5H	-	CNG Temperature cycle test

Annex 5I	-	Pressure cycle test applicable only to cylinders (see Annex 3)

Annex 5L	-	CNG Durability test (Continued operation)

Annex 5M	-	Burst/destructive test applicable only to cylinders (see Annex 3)

Annex 5N	-	CNG Vibration resistance test

Annex 5O	-	Ambient operating temperatures

Annex 5P	-	LNG - low temperature test (-40°C)

Annex 6	-	Identification mark for public service CNG

Annex 7	-	Identification mark for public service LNG