Complete OICA proposal for a phase-in process of AEBS

This document summarizes the OICA position toward the phased-in introduction of the Advanced Emergency Braking System into an UNECE Draft Regulation.

- OICA supports the establishment of an unique Regulation for covering the different existing types of AEBS and defining harmonized requirements, in order to lead to type-approvals which are mutually recognised by each Contracting Party. This permits to lower barriers to trade and consequently make the new technologies more affordable to the final users.

- Table of phased-in introduction of AEBS.

Notes:

- It is to be stressed that this table represents a non divisible OICA position, where the performances proposed to be required are closely linked to their application time. The dates and performances proposed below represent the state of the art for the automobile Industry.

- OICA proposes a phased-in introduction of AEBS for the vehicles of categories M2/N2 and M3/N3 fitted with a Vehicle Stability Function in accordance with the performance requirements of Annex 21 of Regulation No.13.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step (NT/AT) | Technical requirement addresses vehicles of | Scope | Target | Subject vehicle speed | Target vehicle speed | Minimum speed reduction for the subject vehicle | Latest time for 1st warning before EB start |
| A | Step1 2013/2015 | M3/N3/M2/N2 | Pneumatic braking and pneumatic rear suspension | Moving | 80±2km/h | 30＋4/–0km/h | Not less than 50km/h | 1.4 s |
| B |  |  |  | Stationary | 80±2km/h | Stationary | Not less than 10km/h |  |
| C | Step 2 2016/2020 |  | Pneumatic braking and pneumatic rear suspension | Moving | 80±2km/h | 10＋4/–0km/h | Not less than 70km/h | 2.0 s |
| D |  |  |  | Stationary | 80±2km/h | Stationary | Not less than 20 km/h | 1.4 s |
| E |  |  | Air over hydraulic braking AND pneumatic rear suspension | Moving | 80±2km/h | [30＋4/–0] km/h | Not less than [50km/h] | 1.4 s |
| F |  |  |  | Stationary | 80±2km/h | [Stationary] | Not less than [10km/h] | 1.4 s |
| G | Step3 [2020/2022] |  | Hydraulic braking | Moving | [80]±2km/h | [30]＋4/–0km/h | [15] km/h | [0.8] s |
| H |  |  |  | Stationary |  | Stationary | [8] km/h | [0.8] s |

- Exemptions:

- Special purpose vehicles (as defined in RE.3: “A vehicle of category M, N or O for conveying passengers or goods and for performing a special function for which special body arrangements and/or equipment are necessary.”)

- Vehicles which comply with the definition of CATEGORY G – Off-road vehicles as defined in RE.3

- Vehicles of category M3 Classes I, II, A and articulated busses because the chassis manufacturer has no control about the category of the vehicle at the end of the multi-stage approval process.