|  | United Nations | AEBS/LDWS-11-03 20110105 ECE/TRANS/WP.29/GRRF/2011/XX AEBS-M+A 01 Series |  |
| --- | --- | --- | --- |
|  | Economic and Social Council |  | Distr.: General 22 November 2010 Original: English English and French only |

Economic Commission for Europe

Inland Transport Committee

World Forum for Harmonization of Vehicle Regulations

Working Party on Brakes and Running Gear

Sixty-ninth session

Geneva, 1–4 February 2011

Item 2 of the provisional agenda

Automatic Emergency Braking and Lane
Departure Warning Systems (AEBS/LDWS)

Proposal for a Draft 01 Series of Amendments to the Regulation on advanced emergency braking system

Submitted by the Informal group on Automatic Emergency Braking and Lane Departure Warning Systems (AEBS/LDWS) *

The text reproduced below was prepared by the Informal group on Automatic Emergency Braking and Lane Departure Warning Systems (AEBS/LDWS) to introduce a phased approach of AEBS implementation via a 01 Series of Amendments to the draft Regulation on advanced emergency braking system.

I.	Proposal

Paragraph.1., amend to read (reference to footnote 1 and footnote 1 remain unchanged):

1.		Scope and purpose

This Regulation applies to the approval of vehicles of category M2, N2, M3 and N31 equipped with a pneumatic braking system and pneumatic rear suspension with regard to an on-board system to mitigate or avoid a rear-end in lane collision.

Paragraph 6.5.3., amend to read:

6.5.3.	Moving target

The moving target shall be moving on the axis centre of the test course at a constant speed of

(a)	 30+4/-0 10+4/-0 km/h for the vehicles of categories M3 and N3, and

(b)	30+4/-0 km/h for the vehicles of categories M2 and N2.

Paragraph 6.6.3., amend to read (with the addition of two paragraphs):

6.6.3.	The speed reduction of the subject vehicle at the time of the impact with the stationary target shall be:

6.6.3.1.	not less than 10 20 km/h for a vehicle of Category N3,

6.6.3.2.	not less than 10 20 km/h for a vehicle of Category M3,

6.6.3.3.	not less than 10 km/h for a vehicle of Category N2,

6.6.3.4.	not less than 10 km/h for a vehicle of Category M2.

Paragraph 6.7.2.1., amend to read:

6.7.2.1.	At least one haptic or acoustic warning mode shall be provided,

(a)	in the case of a subject vehicle of categories M3 and N3 not later than [2.0s] before the start of the Emergency Braking phase.

(b)	in the case of a subject vehicle of categories M2 or N2 not later than [1.4s] before the start of the Emergency Braking phase.

6.7.2.1.	At least one haptic or acoustic warning mode shall be provided not later than [1.4] s.before the start of the Emergency Braking phase.

Insert new Paragraph 12, to read:

12.	Transitional provisions

12.1.	As from the official date of entry into force of the 01 series of amendments, no Contracting Party applying this Regulation shall refuse to grant UNECE type approvals under this Regulation as amended by the 01 series of amendments.

12.2.	Contracting Parties applying this Regulation shall not refuse to grant UNECE type approvals and extensions of UNECE type approvals to the 00 series of amendments to this Regulation.

12.3.	No Contracting Party applying this Regulation shall refuse national or regional type approval of a vehicle type approved to the 01 series of amendments to this Regulation.

12.4.	No Contracting Party applying this Regulation shall refuse first national or regional registration of a vehicle which complies with the requirements of the 01 series of amendments to this Regulation.

12.5.	Until [01/11/2016], no Contracting Party applying this Regulation shall refuse national or regional type approval of a vehicle type approved to the 00 series of amendments to this Regulation.

12.6.	Until [01/11/2020], no Contracting Party applying this Regulation shall refuse first national or regional registration of a vehicle which complies with the requirements of the 00 series of amendments to this Regulation.

Annex 2, amend to read:

Annex 2

Arrangements of approval marks

(see paragraphs 4.4. to 4.4.2. of this Regulation))

XXXR - 01185

a = 8 mm min

The above approval mark affixed to a vehicle shows that the vehicle type concerned has been approved in Belgium (E6) with regard to the AEBS pursuant to Regulation No. XXX. The first two digits of the approval number indicate that the approval was granted in accordance with the requirements of the 01 Series of Amendments to the Regulation No. XXX in its original form.

*	*	In accordance with the programme of work of the Inland Transport Committee for 2006–2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance the performance of vehicles. The present document is submitted in conformity with that mandate.